�����������������������������ͻ

�    Cardem     v1.000.414    �

�����������������������������͹�

�    ISO 7816   SDL Sender    �

�����������������������������ͼ

IMPORTANT:

----------

All rights reserved. No part of the executable file Cardem.exe and code

therein and/or this document may be reproduced, altered or transmitted in any

form or by any means - electronic, electro static, mechanical, photocopying,

recording or otherwise, either for profit or non-profit, without the prior

permission of the author. Copyright of the executable file Cardem.exe and

code therein remains the property of the author. Usage of Cardem.exe is

limited by license to one-user. Additional copies of the executable file

cannot be made with the authors prior consent.

Any breach of the above terms may result in legal action against the user.

In short, if you did not receive this copy directly from the author you are

in violation of the above terms. Legal action could be taken against you by

the author.

Please send email to : jc@eurosat.com,  to receive a free, registered copy.

The author does not guarantee the safety of the code and is not liable for

any damage the use of this program may cause - use at your own risk. If in

doubt, email the author at :  jc@eurosat.com

Introduction

------------

Cardem is an MS-DOS based utility to send Smartcard Data Log (SDL) files to

an ISO 7816 Asyncronous Contact Smartcard. The SDL file format is detailed

in SDLNOTES.TXT, included in this archive.

The purpose of this utility is to aid in the design of smartcards, by

allowing the developer to test the card in a pseudo operating environment.

It might not always be economically viable or possible to test the normal

operation of the developing smartcard in its true application so a bridging

application such as Cardem would be used to emulate the host for the

smartcard.

Using Cardem

------------

Cardem requires an interface between the PC and the smartcard. Such interfaces

are available from PayTV enthusiasts' internet sites. Contact the author for

further details. Full assembly instructions for a compatible interface are

included in this archive should you wish to do so, see ADAPTER.TXT.

Various options are passed to Cardem by either the SDL file or by command line

options. If given, command line options override SDL file settings (see

examples of usage below).

Once communication is established, each line of the SDL file is sent to the

smartcard, one by one. A processing delay is available by command line option

'd<n>' specified in milliseconds, which introduces a delay inbetween SDL

file line processing. The program can be terminated during this delay by

pressing the ESC key. Please note that some command structures in smartcards

require the host to return within a designated time. This processing delay

can sometimes cause an error in such communications so it is suggested that

the output file be used for debugging purposes, rather than using the PC

screen.

Known Card Acceptance

---------------------

Cardem has been fully tested working on the following smartcards :

VideoCrypt1

Series 03

Series 04

Series 06

Series 07

Series 09

Series 10

Series 11

09->D2MAC

COP8 Smartcards

v3.10   Bigmac

v3.11

v3.16   Millenium 12+

v3.17

v4.04   Arkansas

v4.05   California

Amiga A0 Smartcards

CardMate 3 Smartcards

If you have luck with successful communication on smartcards not listed above,

please send email to   jc@eurosat.com   with full details, including the SDL

file used.

Interfaces tested

-----------------

Cardem has been fully tested on the following interfaces :

.  Mk11 by Paul Maxwell-King

information available at :  http://www.maxking.demon.co.uk

.  Interesting Devices Smartcard Reader/Writer

information available at :  http://users.aol.com/devices232

.  Clanzers Smartcard Reader/Writer

information available at :  http://www.wedzboyz.demon.co.uk

CARDEM.EXE: usage     for v1.000.414

------------------------------------

Syntax:

CARDEM [<com>] [wa<n>] [wb<n>] [b<n>] [d<n>] [i<name>] [o<name>] [r<n>]

<com>    select port         use com port <com>

wa<n>    byte delay          specify in uS

wb<n>    procedure delay     specify in uS

b<n>     operating baud      operating baud rate

d<n>     process delay       optional processing delay in ms

i<name>  input SDL file      input SDL file

o<name>  write file          screen output is written to named file

r<n>     reset baud          reset baud rate

defaults are:

com: 2

wa:  1000

wa:  15000

d:   0

r:   09600

b:   09600

interface: Phoenix type

Example Usage

-------------

All command line options are interchangeable - they are not set in order.

1.  Cardem idata.sdl

This will use the SDL file named 'data.sdl'

2.  Cardem 1 idata.sdl

This will change the comport used to COM 1, and use the SDL file

named 'data.sdl'

3.  Cardem idata.sdl d1000

This will use the SDL file named 'data.sdl' and set a processing

delay of 1000ms ( 1 second ) inbetween each line of commands to

the smartcard.

4.  Cardem idata.sdl r19200

This will use the SDL file named 'data.sdl' and reset the

smartcard at 19200bps. Please note that the 'r' command line option

will override the reset baud rate specified in the SDL file.

5.  Cardem idata.sdl b19200

This will use the SDL file named 'data.sdl' and operate the

smartcard at 19200bps. Please note that the 'b' command line option

will override the operating baud rate specified in the SDL file.

6.  Cardem 1 idata.sdl d2000 r19200 b19200

This will change the comport used to COM 1 and use the SDL file

named 'data.sdl'. It will set a processing delay of 2000ms ( 2

seconds ) and reset the smartcard at 19200bps. Lastly, it will

operate the smartcard at 19200bps. Please note that the 'r' and

'b' command line options will override the relevant baud rates

specified in the SDL file.

The wa<n> delays are as per alternative software making use of such delays.

Cardem will autodetect whether a Phoenix or SmartMouse type interface is

being used.

Cardem Version History

----------------------

Version     Date         Notes

1.000.414   11-Mar-97    : . improvements made to outputfile

: . bug fixes

1.000.401    2-Mar-97    : . 2nd evaluation release

: . bug fixes

1.000.384   15-Feb-97    : . 1st evaluation release