Contents of This Document

1) Overview

2) N1231A Hardware Driver Installation Instructions

- For Microsoft Windows NT Systems

- For Microsoft Windows 2000 Systems

- For Microsoft Windows XP Systems

3) N1231A Software Installation Instructions

- General Information

- API Library: OEM Version Installation

- API Library: Development Version Installation

- Monitor Program Installation

- Older Version Information

Overview

The Agilent Technologies N1231A is a PCI 3-Axis Board designed for use as part of an Agilent laser interferometry position measurement system. Installation of an Agilent Technologies N1231A Three-Axis Laser Board involves several steps:

1) Physical installation of the board into the PC.

2) Installation of the N1231A Driver software.

3) Installation of the N1231A API Library (either the OEM or the Development version).

4) Optionally: Installation of the N1231A Monitor Program.

These steps are discussed in the Getting Started section of the "User's Guide" for the N1231A.  The following is a supplement to information provided in the User's Guide and includes information specific to each of the three supported Windows Operating Systems (NT, 2000, and XP).

- The physical installation of the board in the PC is the same regardless of the specific operating system.

- For Windows NT, the Driver software is installed by the API Library installation program, so no separate installation step is required.  For Windows 2000 and Windows XP, see the N1231A Hardware Driver Installation Instructions, below.

- The installation program for the API Library detects which operating system is in use and installs the appropriate files.  See N1231A Software Installation Instructions, below.

- Installation of the Monitor Program is the same regardless of the operating system.  Note that although documentation for the Monitor Program refers only to Windows NT, the program will also work under Windows 2000 or XP.

N1231A Hardware Driver Installation Instructions

For Microsoft Windows NT Systems

Hardware Drivers for the N1231A are installed by the N1231A Software Installation program (see below).

For Microsoft Windows 2000 Systems

1) When a Windows 2000 system first boots up following installation of an N1231A card it should detect the new hardware and automatically run the "Found New Hardware Wizard." The N1231A will be listed as an "Other PCI Bridge Device."

2) Insert the installation disk into the CD-ROM drive and follow the instructions in the wizard.

- Select "Search for a suitable driver for my device"

- Select "Specify location" or "Have disk" and then navigate to the "\Drivers\XP_and_2000" directory on the installation CD.

3) The wizard should report "Windows found a driver."

If a driver has been previously installed (for example, if multiple N1231A boards are installed) the system may find the driver automatically, but may still ask for the installation disk.

For Microsoft Windows XP Systems

1) When a Windows XP system first boots up following installation of an N1231A card it should detect the new hardware and automatically run the "Found New Hardware Wizard." The N1231A will be listed as an "Other PCI Bridge Device."

2) Insert the installation disk into the CD-ROM drive and follow the instructions in the wizard.

- Select "Search for the best driver in these locations"

- Select "Search removable media"

3) The wizard should report that software has been installed for "Agilent Laser Axis Board"

If a driver has been previously installed (for example, if multiple N1231A boards are installed) the system may find the driver automatically.

N1231A Software Installation Instructions

General Information:

This software is intended for use under the following operating systems:

- Microsoft Windows NT 4.0

- Microsoft Windows 2000

- Microsoft XP

Description of setup programs:

N1231A API OEM Version 1.1:

- Installs the API Library DLL.

- If running Windows NT: Installs the Device Drivers and modifies the Registry (for Windows 2000 or XP, see N1231A Hardware Driver Installation Instructions, above).

- Does not install any standalone executables.

- Intended for installation of the library on an end-user machine where the executable program is supplied by a separate setup program.

N1231A API Development Version 1.1:

- Includes everything installed by the OEM version.

- Installs additional files useful by those developing software using the API library.

- If the API Development Version is installed, it is not necessary to install the API OEM Version.

N1231A Monitor

- Installs the N1231A Monitor program which provides a graphical interface to the N1231A card.

- The N1231A API (either the OEM version or the Development version) should be installed prior to installing the N1231A Monitor.

Note: This CD also contains an earlier version (1.0) of the API for possible use in cases where a conflict may be detected with existing programs. See: "Older Version of API", below.

To Install the OEM version of the N1231A API Library from the CD:

- Exit from all applications.

- Insert the CD in an appropriate drive.

- Navigate to the directory: "\N1231A API OEM"

- Run "Setup.exe"

- Follow the instructions on the screen.

- When asked to choose a Setup Type,  the default choice of "OEM" is recommended.

- If running under Windows NT: If this is the first time the software has been installed, or if the software has been uninstalled prior to this installation, reboot the computer.

To Install the Development version of the N1231A API Library from the CD:

- Exit from all applications.

- Insert the CD in an appropriate drive.

- Navigate to the directory: "\N1231A API Development"

- Run "Setup.exe"

- Follow the instructions on the screen.  When asked to choose a Setup Type,  the default choice of "Typical"  is recommended. Other choices are "Compact" and "Custom."

-- Typical installs all files.

-- Compact installs all files except those in the "User Files" directory.

-- Custom allows selecting the components to be installed.

- If running under Windows NT: If this is the first time the software has been installed, or if the software has been uninstalled prior to this installation, reboot the computer.

To Install the N1231A Monitor program from the CD:

Note: If the N1231A API Library is not yet installed, follow the instructions above to install it before proceeding with this installation (either the OEM or development version may be used).

- Exit from all applications.

- Insert the CD in an appropriate drive.

- Navigate to the directory: "\N1231A Monitor"

- Run "Setup.exe"

- Follow the instructions on the screen

Note: The N1231A Monitor program will run under Windows NT, Windows 2000, and Windows XP, although its Readme.txt file indicates it is for Windows NT.

Older Version of  API:

Versions 1.1 and 1.0 of the N1231A API Library are functionally the same.  However, Version 1.1 has been re-compiled to operate with a newer driver (Pci9030.sys) and DLL (PlxApi.dll) supplied by PLX Technology, Inc.  The original Version 1.0 is only available for use under Windows NT 4.0.

Installation of Version 1.1 will overwrite existing Pci9030.sys and PlxApi.dll files.  Therefore, other programs dependent on these files may no longer operate.  If the installation program detects these older files it will issue a warning.  In this case, if operating under Windows NT, consider installing Version 1.0 of this program (only available for Windows NT 4.0).

To install the older Version 1.0 of the API:

- Exit from all applications.

- Insert the CD in an appropriate drive.

- Navigate to one of  these directories:

"\Version 1_0\N1231A API Development"

"\Version 1_0\N1231A API OEM"

- Run "Setup.exe"

- Follow the instructions on the screen.  When asked to choose a Setup Type,  the default choice of "Typical" or "OEM" is recommended. Other choices (for the Development version) are "Compact" and "Custom."

-- Typical installs all files.

-- Compact installs all files except those in the "User Files" directory.

-- Custom allows selecting the components to be installed.

- If this is the first time the software has been installed, or if the software has been uninstalled prior to this installation, reboot the computer.

N1231A Monitor is compatible with either version (1.0 or 1.1) of the N1231A API.