Benchtop Memory Erasure

HP 1650-series Benchtop Logic Analyzers 
HP 1660-series Benchtop Logic Analyzers 
HP 1670-series Benchtop Logic Analyzers

Top of Form 1 Bottom of Form 1

When the following products are used in secure applications, all data storage devices must be cleared prior to removal from the secured environment.The memory erasure matrix tabulates the size, usage, and clearing methods of all storage elements designed into each HP Logic Analyzer and associated products.

* For best viewing results, please maximize your program window

- 1650 series

- 1660 series

- 1670 series

| Product |  |  | Mem Type | Mem Size | Memory Usage | Method to Clear |
| --- | --- | --- | --- | --- | --- | --- |
| Model # | Description |  |  |  |  |
| 1650A | Benchtop Logic Analyzer | DRAM DRAM EPROM EPROM VRAM | 10K 256Kx4 256K 256K 64Kx4 | Acquisition memory Program/Data Storage Boot ROM Boot ROM DataWaveform storage | V;Cycle power V;Cycle power NV; No user data NV; No user data V;Cycle power |
| 1651A | Benchtop Logic Analyzer | DRAM DRAM EPROM EPROM VRAM | 2K 256Kx4 256K 256K 64Kx4 | Acquisition memory Program/Data Storage Boot ROM Boot ROM DataWaveform storage | V;Cycle power V;Cycle power NV; No user data NV; No user data V;Cycle power |
| 1650B | Benchtop Logic Analyzer | DRAM DRAM EPROM EPROM VRAM | 10K 256Kx4 256K 256K 64Kx4 | Acquisition memory Program/Data Storage Boot ROM Boot ROM DataWaveform storage | V;Cycle power V;Cycle power NV; No user data NV; No user data V;Cycle power |
| 1651B | Benchtop Logic Analyzer | DRAM DRAM EPROM EPROM VRAM | 2K 256Kx4 256K 256K 64Kx4 | Acquisition memory Program/Data Storage Boot ROM Boot ROM DataWaveform storage | V;Cycle power V;Cycle power NV; No user data NV; No user data V;Cycle power |
| 1652B | Benchtop Logic Analyzer | DRAM EEPROM EPROM EPROM FISO RAM VRAM | 10K 8K 256K 256K 4K 256Kx4 64Kx4 | Acquisition memory Cal factor storage Boot ROM Boot ROM Scope acq memory Program/Data Storage DataWaveform storage | V;Cycle power NV; No user data NV; No user data NV; No user data NV; No user data V;Cycle power V;Cycle power |
| 1653B | Benchtop Logic Analyzer | DRAM EEPROM EPROM EPROM FISO RAM VRAM | 10K 8K 256K 256K 4K 256Kx4 64Kx4 | Acquisition memory Cal factor storage Boot ROM Boot ROM Scope acq memory Program/Data Storage DataWaveform storage | V;Cycle power NV; No user data NV; No user data NV; No user data NV; No user data V;Cycle power V;Cycle power |
| 1660C | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 136x4K 512Kx8 256Kx8 540Mb | Acquisition memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1660CS | Benchtop Logic Analyzer with Scope | DRAM DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 136x4K 2x8K 512Kx8 256Kx8 540Mb | Acquisition memory Scope acq memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1660A | Benchtop Logic Analyzer | DRAM DRAM EEPROM EPROM NVRAM VRAM | 136x4K 1Mbx4 1Mb 128K 4K 256Kx8 | Acquisition memory Program memory Program storage Boot ROM Cal factors Data/Waveform | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; No user data V; Cycle power |
| 1660AS | Benchtop Logic Analyzer with Scope | DRAM DRAM DRAM EEPROM EPROM NVRAM VRAM | 136x4K 2x8K 1Mbx4 1Mb 128K 4K 256Kx8 | Acquisition memory Scope acq memory Program memory Program storage Boot ROM Cal factors Data/Waveform | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; No user data V; Cycle power V; Cycle power |
| 1661C | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 102x4K 512Kx8 256Kx8 540Mb | Acquisition memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1661CS | Benchtop Logic Analyzer with Scope | DRAM DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 102x4K 2x8K 512Kx8 256Kx8 540Mb | Acquisition memory Scope acq memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1661A | Benchtop Logic Analyzer | DRAM DRAM EEPROM EPROM NVRAM VRAM | 102x4K 1Mbx4 1Mb 128K 4K 256Kx8 | Acquisition memory Program memory Program storage Boot ROM Cal factors Data/Waveform | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; No user data V; Cycle power |
| 1661AS | Benchtop Logic Analyzer with Scope | DRAM DRAM DRAM EEPROM EPROM NVRAM VRAM | 102x4K 2x8K 1Mbx4 1Mb 128K 4K 256Kx8 | Acquisition memory Scope acq memory Program memory Program storage Boot ROM Cal factors Data/Waveform | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; No user data V; Cycle power V; Cycle power |
| 1662C | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 68x4K 512Kx8 256Kx8 540Mb | Acquisition memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1662CS | Benchtop Logic Analyzer with Scope | DRAM DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 68x4K 2x8K 512Kx8 256Kx8 540Mb | Acquisition memory Scope acq memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1662A | Benchtop Logic Analyzer | DRAM DRAM EEPROM EPROM NVRAM VRAM | 68x4K 1Mbx4 1Mb 128K 4K 256Kx8 | Acquisition memory Program memory Program storage Boot ROM Cal factors Data/Waveform | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; No user data V; Cycle power |
| 1662AS | Benchtop Logic Analyzer with Scope | DRAM DRAM DRAM EEPROM EPROM NVRAM VRAM | 68x4K 2x8K 1Mbx4 1Mb 128K 4K 256Kx8 | Acquisition memory Scope acq memory Program memory Program storage Boot ROM Cal factors Data/Waveform | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; No user data V; Cycle power |
| 1663C | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 34x4K 512Kx8 256Kx8 540Mb | Acquisition memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1663CS | Benchtop Logic Analyzer with Scope | DRAM DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 34x4K 2x8K 512Kx8 256Kx8 540Mb | Acquisition memory Scope acq memory Cal factors/Pgm data Boot ROM LAN address/config Expandable acqmem Data/Waveform Data storage | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; Reset w/ random values V; Cycle power V; Cycle power NV; Use HP Disk Erase OPT_011 |
| 1663A | Benchtop Logic Analyzer | DRAM DRAM EEPROM EPROM NVRAM VRAM | 34x4K 1Mbx4 1Mb 128K 4K 256Kx8 | Acquisition memory Program memory Program storage Boot ROM Cal factors Data/Waveform | V; Cycle power V; Cycle power NV; No user data NV; No user data NV; No user data V; Cycle power |
| 1664A | Benchtop Logic Analyzer | DRAM DRAM EPROM VRAM | 34x4K 1Mbx4 128K 256Kx8 | Acquisition memory Program memory Boot ROM Data/Waveform | V; Cycle power V; Cycle power NV; No user data V; Cycle power |
| 1670A | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 136x4K 512Kx8 128Kx16 540Mb | Acquisition memory Cal factors/Pgm. storage Boot ROM LAN address/config Expandable acq. memory Data/Waveform storage Data storage | V; Cycle main power NV; No user data stored NV; No user data stored NV; Reset w/ random values V; Cycle main power V; Cycle main power NV; Use HP Disk Erase OPT_011 |
| 1671A | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 102x4K 512Kx8 128Kx16 540Mb | Acquisition memory Cal factors/Pgm. storage Boot ROM LAN address/config Expandable acq. memory Data/Waveform storage Data storage | V; Cycle main power NV; No user data stored NV; No user data stored NV; Reset w/ random values V; Cycle main power V; Cycle main power NV; Use HP Disk Erase OPT_011 |
| 1672A | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 68x4K 512Kx8 128Kx16 540Mb | Acquisition memory Cal factors/Pgm. storage Boot ROM LAN address/config Expandable acq. memory Data/Waveform storage Data storage | V; Cycle main power NV; No user data stored NV; No user data stored NV; Reset w/ random values V; Cycle main power V; Cycle main power NV; Use HP Disk Erase OPT_011 |
| 1670D | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 136x4K 512Kx8 256Kx16 540Mb | Acquisition memory Cal factors/Pgm. storage Boot ROM LAN address/config Expandable acq. memory Data/Waveform storage Data storage | V; Cycle main power NV; No user data stored NV; No user data stored NV; Reset w/ random values V; Cycle main power V; Cycle main power NV; Use HP Disk Erase OPT_011 |
| 1671D | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 102x4K 512Kx8 256Kx16 540Mb | Acquisition memory Cal factors/Pgm. storage Boot ROM LAN address/config Expandable acq. memory Data/Waveform storage Data storage | V; Cycle main power NV; No user data stored NV; No user data stored NV; Reset w/ random values V; Cycle main power V; Cycle main power NV; Use HP Disk Erase OPT_011 |
| 1672D | Benchtop Logic Analyzer | DRAM EEPROM EPROM NVRAM RAM VRAM Fixed Disk | 68x4K 512Kx8 256Kx16 540Mb | Acquisition memory Cal factors/Pgm. storage Boot ROM LAN address/config Expandable acq. memory Data/Waveform storage Data storage | V; Cycle main power NV; No user data stored NV; No user data stored NV; Reset w/ random values V; Cycle main power V; Cycle main power NV; Use HP Disk Erase OPT_011 |

Guide to Key Terms

| Mem Type | Description |  | Other | Description |
| --- | --- | --- | --- | --- |
| DRAM | Dynamic Random Access Memory |  | V | Volatile |
| EEPROM | Electronically Erasable Programmable Read Only Memory |  | NV | Non-Volatile |
| EPROM | Electronically Programmable Read Only Memory |  | Cycle Main Power | Turn machine off then on |
| FISO | Fast In Slow Out |  | Cal factors | Calibration factors |
| NVRAM | Non-Volatile Random Access Memory |  | OPT_003/OPT_011 | Downloadable utility for fixed disk erasure |
| RAM | Random Access Memory |  |  |  |
| ROM | Read Only Memory |  |  |  |
| SRAM | Static Random Access Memory |  |  |  |
| VRAM | Video Random Access Memory |  |  |  |

Option 011 Disk Erasure Utility

The OPT_011 utility is used for the benchtop logic analyzers fixed disks respectively. The utility erases and makes unrecoverable all user data on the 540Mb fixed disk by first writing 0's to the entire disk, then 1's, and then alternately 0's and 1's. This is not a supported utility.

- Insert a formatted 1.44MB DOS flexible disk in the flexible disk drive of your PC

- Do a [ SHIFT ][ MOUSE CLICK ] on the following link:

- OPTC_011 Utility for the 1660/70 series of Benchtop Analyzers

- Netscape will then prompt you for a destination. Specify a: as the destination Drive.

- Insert the flexible disk in the logic analyzer flexible disk drive, then apply power to the logic analyzer. The utility will automatically begin to erase the hard disk drive.

Top of Form 2 Bottom of Form 2