MHL™
(Mobile High-definition Link)

Compliance Test Specification

Main Required Methods

Confidential information of:

Nokia Corporation,

Samsung Electronics Corporation,

Silicon Image, Inc.,

Sony Corporation,

Toshiba Corporation,

each, an “MHL Promoter” and collectively, the “MHL Promoters”.

Version 3.0

Revision 1.0

Release Candidate fc07

MHL, LLC

October 2013

Notice

THIS SPECIFICATION IS PROVIDED “AS IS” WITH NO WARRANTIES WHATSOEVER, EXPRESS OR IMPLIED, INCLUDING, WITHOUT LIMITATION, NO WARRANTIES OF MERCHANTABILITY, NONINFRINGEMENT, FITNESS FOR ANY PARTICULAR PURPOSE, OR ANY WARRANTY OTHERWISE ARISING OUT OF ANY PROPOSAL, SPECIFICATION, OR SAMPLE.

Nokia Corporation, Samsung Corporation, Silicon Image, Inc., Sony Corporation, Toshiba Corporation and MHL, LLC disclaim all liability, including liability for infringement of any proprietary rights, relating to use of information in this specification.

Copyright © 2001-2013 by one or more of the MHL Promoters. All rights reserved. No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted herein. Unauthorized use or duplication prohibited. “MHL” and all associated logos are trademarks of MHL, LLC. Third-party trademarks and servicemarks are property of their respective owners.

Document Revision History

| 2013-10 | Distributed Revision 3.0. |
| --- | --- |

Please refer also to the Revision Highlights section, beginning on Section 2.6 on page 25.

# Introduction

This document contains a version of the MHL Compliance Test Specification, as such term is defined in the Mobile High-Definition Link Specification Adopter Agreement.  A product’s satisfaction of the criteria and tests set forth herein do not guarantee that such product is Fully Compliant, as such term is defined in the Mobile High-Definition Link Specification Adopter Agreement.  This version of the MHL Compliance Test Specification shall be effective until a superseding version is made available by the MHL Promoters to the MHL Adopters. In the event that an Authorized Testing Center has tested and certified a product by performing the criteria and tests set forth herein and the product is later reported to the Agent as not being Fully Compliant, then the respective Promoter or Adopter shall, in a timely manner following the date it is notified by the Agent of such report, resubmit the product to an Authorized Testing Center for re-testing and re-certification.

Table of Contents

1	Introduction	3

2	Background	8

2.1	Formatting of this Document	8

2.1.1	Formatting of Tests	8

2.1.2	Symbolic Terms Used in Test Definitions	8

2.1.3	Test Descriptions	8

2.1.4	Capability Declaration Form and Test Results Form	9

2.1.5	Usages and Conventions	9

2.1.6	Terminology in this CTS	9

2.2	Test Results	9

2.2.1	Definitions of Test Result Terminology	9

2.2.2	Requirement for Successful Completion of Compliance Testing	10

2.2.3	Test Flow Usage of PASS, FAIL and PASS (SKIP)	10

2.3	Test Equipment	10

2.3.1	TMDS Test Equipment	11

2.3.2	MHL System-level Equipment	14

2.3.3	CBUS Equipment	15

2.3.4	HDCP Equipment	17

2.3.5	Adapters	18

2.3.6	Miscellaneous	21

2.4	Required Video and Audio Patterns	21

2.4.1	Required Video Patterns	21

2.4.2	Required Audio Patterns	23

2.5	Measurement Methods	24

2.5.1	CBUS Electrical Measurements	24

2.6	Revision History	25

2.7	Normative References	26

2.8	Organization of this Document	26

2.9	CDF Field Names	26

3	Source Test	33

3.1	Electrical Tests	33

3.2	System Tests	33

3.3	CBUS Tests	34

3.4	eCBUS oCBUS / MHL 3 Protocol Tests	35

3.5	eCBUS MHL 3 Active Mode Source Tests	38

3.6	eCBUS MHL 3 Robustness Mode Source Tests	40

3.7	MHL 3 Electrical Tests	41

3.7.2	TMDS and eCBUS Electrical Test Procedures	45

3.8	MHL 3 System Tests	70

4	Sink Test	85

4.1	Electrical Tests	85

4.2	System Tests	86

4.3	CBUS Tests	86

4.4	eCBUS oCBUS / MHL 3 Legacy Protocol Tests	88

4.5	eCBUS MHL 3 Active Mode Sink Tests	91

4.6	eCBUS MHL 3 Robustness Mode Sink Tests	92

4.7	MHL 3 Electrical Tests	93

4.8	MHL 3 System Tests	125

5	Dongle Test	138

5.1	Electrical Tests	138

5.2	System Tests	138

5.3	CBUS Tests	139

5.4	eCBUS oCBUS / MHL 3 Legacy Protocol Tests	140

5.5	eCBUS MHL 3 Active Mode Dongle Tests	143

5.6	eCBUS MHL 3 Robustness Mode Dongle Tests	144

5.7	MHL 3 Electrical Tests	145

5.8	MHL 3 System Tests	173

6	Tests Common to Sources, Sinks and Dongles	187

6.1	Electrical Tests	187

6.2	System Tests	187

6.3	CBUS Tests	187

6.4	eCBUS MHL 3 Active Mode Tests	187

6.5	eCBUS MHL 3 Robustness Tests	216

7	Cable Test	224

7.1	Mechanical tests	224

7.2	Electrical Tests	224

8	Appendices	233

8.1	Capability Declaration Form (CDF)	233

8.2	Test Results Form (TRF)	239

List of Figures

Figure 2-1. Flowchart for CTS Test Flow	10

Figure 2-2. Relay/Echo Cancel/Level Converter Termination Board	13

Figure 2-3. Recommended MHL3 Signal Generator and Adapter for Dongle Test.	15

Figure 2-4. Recommended MHL3 Signal Generator and Adapter for Sink Test.	15

Figure 2-5. CBUS Source Board	16

Figure 2-6. CBUS Sink Board	16

Figure 2-7. MHL HDCP Analyzer	17

Figure 2-8. MHL HDCP Emulator	17

Figure 2-9. TPA Board for DUT with eCBUS-S (a) and for DUT with eCBUS-D (b)	18

Figure 2-10. TPA Board for Source RxSense Test (Source TPA-RSEN)	20

Figure 2-11. TPA Board for Sink and Dongle RxSense Test (Sink/Dongle TPA-RSEN)	20

Figure 2-12. Gray Ramp Color Derivation	22

Figure 2-13. Representative Gray Ramp Video Patterns	22

Figure 2-14. Chess Board Video Pattern	23

Figure 2-15. CBUS Voltage Level Waveform	24

Figure 3-1. Test configuration at TP1 for source TMDS output and eCBUS-S FWD output	41

Figure 3-2. Test configuration at TP2 for source TMDS output and eCBUS-S FWD output	41

Figure 3-3. Test configuration at TP1 for source TMDS output and eCBUS-D FWD output	42

Figure 3-4. Test configuration at TP2 for source TMDS output and eCBUS-D FWD output	42

Figure 3-5. Test configuration at TP1 for source eCBUS-S BWD input	43

Figure 3-6. Test configuration at TP1 for source eCBUS-S BWD input jitter tolerance	43

Figure 3-7. Test configuration at TP1 for source eCBUS-D BWD input	44

Figure 3-8. Test configuration at TP1 for source eCBUS-D BWD input intra-pair skew and input jitter tolerance	44

Figure 3-9. Test configuration at TP1 for source TMDS and eCBUS-S impedance	45

Figure 3-10. Test configuration at TP1 for source TMDS and eCBUS-D impedance	45

Figure 3-11. Set reference point for differential mode	66

Figure 3-12. Set reference point for differential mode	67

Figure 3-13. Set reference point for single-ended signal	69

Figure 4-1. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Sink Test	93

Figure 4-2. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Sink Jitter Tolerance Test	93

Figure 4-3. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Sink Test	94

Figure 4-4. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Sink Jitter Tolerance Test	94

Figure 4-5. Test configuration at TP2 for Sink TMDS input and eCBUS-S FWD input	95

Figure 4-6. Test configuration at TP2 for Sink TMDS and eCBUS Tests	95

Figure 4-7. Test configuration at TP2 for Sink TMDS input and eCBUS-D FWD input	96

Figure 4-8. Test configuration at TP2 for Sink TMDS and eCBUS FWD Tests	96

Figure 4-9. Test configuration at TP2 for Sink eCBUS-S BWD output	97

Figure 4-10. Test configuration at TP1 for Sink eCBUS-S BWD output	97

Figure 4-11. Test configuration at TP2 for Sink eCBUS-D BWD output	98

Figure 4-12. Test configuration at TP1 for Sink eCBUS-D BWD output	99

Figure 4-13. Test configuration at TP2 for sink TMDS and eCBUS-S impedance	99

Figure 4-14. Test configuration at TP2 for sink TMDS and eCBUS-D impedance	100

Figure 4-15. Set reference point for differential mode	118

Figure 4-16. Direct Attach Sink – Differential Impedance Test Symbology	119

Figure 4-17. Set reference point for differential mode	120

Figure 4-18. Direct Attach Sink – Differential Impedance Test Symbology	121

Figure 4-19. Set reference point for single-ended signal	123

Figure 4-20. Direct Attach Sink – Single-ended Impedance Test Symbology	123

Figure 5-1. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Dongle Test	145

Figure 5-2. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Dongle Jitter Tolerance Test	146

Figure 5-3. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Dongle Test	146

Figure 5-4. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Dongle Jitter Tolerance Test	147

Figure 5-5. Test configuration at TP3 for Dongle TMDS input and eCBUS-S FWD input	147

Figure 5-6. Test configuration at TP3 for Dongle TMDS input and eCBUS-D FWD input	147

Figure 5-7. Test configuration at TP3 for Dongle eCBUS-S BWD output	148

Figure 5-8. Test configuration at TP3 for Dongle eCBUS-D BWD output	148

Figure 5-9. Test configuration at TP2 for Dongle TMDS and eCBUS-S impedance	149

Figure 5-10. Test configuration at TP2 for Dongle TMDS and eCBUS-D impedance	149

Figure 5-11. Set reference point for differential mode	167

Figure 5-12. Dongle – Differential Impedance Test Symbology	167

Figure 5-13. Set reference point for differential mode	169

Figure 5-14. Dongle – Differential Impedance Test Symbology	169

Figure 5-15. Set reference point for single-ended signal	171

Figure 5-16. Dongle – Single-ended Impedance Test Symbology	171

Figure 7-1. Single-Ended Characteristic Impedance Test Setup	225

Figure 7-2. Set reference point for Single-Ended	225

Figure 7-3. Single-Ended eCBUS Insertion Loss Test Setup	227

Figure 7-4. Near-End Crosstalk Test Setup	228

Figure 7-5. Differential Intra-Pair Skew Test Setup	229

Figure 7-6. Illustration of Cable Length for the Impedance Test	230

Figure 7-7. Differential Characteristic Impedance Test Setup	230

Figure 7-8. Set reference point for differential mode	231

Figure 7-9. Near-End Crosstalk Test Setup	232

List of Tables

Table 2-1. MHL3 TPA Boards Description	19

Table 3-1. AVLINK MHL2 Source Test Sequence	33

Table 3-2. CBUS MHL 2 Source Test Sequence	34

Table 3-3. eCBUS oCBUS / MHL 3 Suite Normal Tests	36

Table 3-4. eCBUS oCBUS  / MHL 3 Suite Robustness Tests	37

Table 3-5. eCBUS MHL 3 Active Mode Source Tests	39

Table 3-6. eCBUS MHL 3 Robustness Mode Source Tests	40

Table 3-7. AVLINK Test Sequence	71

Table 3-8. High-End Video Format List	73

Table 3-9. 3D and Multi-View Testing Configurations	83

Table 4-1. Sink AVLINK Test Sequence	86

Table 4-2. eCBUS oCBUS Suite CBUS Tests	87

Table 4-3. eCBUS oCBUS / MHL 3 Suite Normal Tests	88

Table 4-4. eCBUS oCBUS / MHL 3 Suite Robustness Tests	89

Table 4-5. eCBUS MHL 3 Active Mode Sink Tests	91

Table 4-6. eCBUS MHL 3 Robustness Mode Sink Tests	92

Table 4-7. Sink AVLINK Test Sequence	126

Table 4-8. High-End Video Format Support Requirement	128

Table 4-9. RR Insertion Alignment	133

Table 4-10. 3D and Multi-View Testing Configurations	135

Table 5-1. Dongle AVLINK Test Sequence	138

Table 5-2. eCBUS oCBUS Suite CBUS Tests	139

Table 5-3. eCBUS oCBUS / MHL 3 Suite Normal Tests	140

Table 5-4. eCBUS oCBUS / MHL 3 Suite Robustness Tests	142

Table 5-5. eCBUS MHL 3 Active Mode Dongle Tests	143

Table 5-6. eCBUS MHL 3 Robustness Mode Dongle Tests	145

Table 5-7. Dongle AVLINK Test Sequence	173

Table 5-8. RR Insertion Alignment	182

Table 5-9. 3D and Multi-View Testing Configurations	184

Table 6-1. Tester Sink Emulation	212

Table 8-1. CDF Field Definitions	233

# Background

## Formatting of this Document

### Formatting of Tests

Each test description uses a sequence of headings as shown in the following table:

| Level | Style | Designation |
| --- | --- | --- |
| 1 | Heading 1 | Device Type Section: Source, Sink, Dongle, Cable |
| 2 | Heading 2 | Test Category: Electrical, System, etc. |
| 3 | Heading 3 | Test Group: Video, Audio, HDCP, etc. |
| 4 | Heading 4 | Individual Major Test (common setup) |

### Symbolic Terms Used in Test Definitions

A number of symbolic terms – variable names – are used in the description of tests. These terms indicate aspects of the device under test, as described in the device’s Capability Declaration Form (CDF); and aspects of the test equipment, as described in the methodologies.

Variables containing information about the device under test from the CDF document use the prefix CDF_.

Variables containing information about the device under test from execution of tests use the prefix MEAS_.

A variable name which uses a Greek letter in its name in the MHL Specification may be represented here with the Greek name spelled out. This facilitates use of the variable name in the test equipment. For example: ΔTRF is represented in this document as DELTA TRF.

### Test Descriptions

#### Use of Subsections

Each test is described in a series of sections:

XXX.1	Test Objective

XXX.2	References

XXX.3	Required Test Equipment

XXX.4	Required Methodology

#### “Required” and “Recommended” Test Methodologies

Each test is described first by a required methodology, which explains the methodology for proper measurement of the parameters or performance to PASS, FAIL or PASS (SKIP) the test. This method refers to one or more of the required test equipment.

The test is then described by one or more recommended methodology, using specific fixtures and test equipment, to guide the test to a compliant test result in actual practice. These recommended methodologies are included in one or more separate documents, usually one per vendor of test equipment. Contact the MHL, LLC for details on the documents available.

### Capability Declaration Form and Test Results Form

Each device under test (DUT) arrives at the test center with a Capabilities Declaration Form (CDF). This form lists the features of the DUT, as well as providing directions on how to setup the DUT for each feature and input or output mode.  Through the use of the details in the CDF the test engineer is able to setup each test and properly exercise the DUT.

A test results form (TRF) is generated by the test center as the tests are performed.  This TRF is delivered back to the Adopter to show the PASS, FAIL or PASS (SKIP) result of each test.

### Usages and Conventions

In addition to the Usages and Conventions defined in the MHL Specification (q.v.), the following are used in this Compliance Test Specification:

0x#_##_###	Symbolizes a hexadecimal value, with the first field (#) representing exactly two bits, the second field (##) representing exactly one bit, and the third field (###) representing exactly 8 bits. This syntax is used throughout the tests covering CBUS. The 2 bits in the first field are the two header bits in a CBUS packet; the 1 bit in the second field is the control bit; the 8 bits in the third field are the eight payload bits. For example, 0x2_0_E3. Refer to Section 7.2.3 in the MHL Specification.

### Terminology in this CTS

| Acronym or Term | Definition |
| --- | --- |
| oCBUS | 1 MHz (or 1 Megabit per second) CBUS used on MHL 1 and MHL 2 devices. The CTS tests this CBUS mode for each MHL 3 device. |

## Test Results

Each test in this Compliance Test Specification ends in a PASS, FAIL or PASS (SKIP) result.

### Definitions of Test Result Terminology

PASS	Indicates that the device under test completed the recommended methodology with no FAIL or SKIP results.

FAIL	Indicates that the device under test failed to meet the requirements of one (or more) steps in the recommended methodology. Usually this means that the device's performance does not fall within the bounds prescribed by the methodology, such as the minimum and maximum limits of an electrical test. It may also mean that the device did not implement a required feature, and the lack of such implementation was determined by one or more steps in the methodology.

PASS (SKIP)	Indicates that the device under test did not complete the recommended methodology because the conditions defined in the device's Capability Declaration Form, when checked in the first step of the methodology, mean that the test does not apply to this device under test. An example is the implementation of driving VBUS output, which is not required for Source devices.

### Requirement for Successful Completion of Compliance Testing

A device under test passes the compliance test suite if it completes the CTS with no FAIL results.

### Test Flow Usage of PASS, FAIL and PASS (SKIP)

Figure 2-1. Flowchart for CTS Test Flow

## Test Equipment

A variety of equipment is needed for testing MHL products completely.  Each piece is authorized and included by name in the separate documents associated with the Main Required Methods of the Compliance Test Specification. This section describes the technical requirements of the major pieces of equipment used in testing.

NOTE: The proper configuration of each piece of test equipment is criticial to the proper functioning of the test. Compliance testing shall follow closely the instructions in each Recommended Methodology to perform each test. Settings on equipment shall not be varied outside of the Recommended Methodology in the performance of compliance testing.

NOTE: Care shall be taken by the test engineer in eliminating from the proximity of the device under test any equipment or other device which may interfere with the accuracy of the tests. Some DUTs use wireless technologies which, when active, may affect the accuracy of MHL-related tests. Adopters shall indicate in the CDF scenarios which affect their DUT, as instructional to the test engineer.

### TMDS Test Equipment

Refer also to equipment in Section 2.3.3 for electrical tests which require CBUS Test Equipment assistance.

#### High-bandwidth Digital Oscilloscope for MHL 3

TMDS measurements require a High-bandwidth Digital Oscilloscope:

- -3dB Bandwidth: DC to 16GHz.

- Sampling rate > 40Gsample/sec, when 2 or more channels are simultaneously sampling

- Sample memory: more than 20M samples per channel

#### High-Bandwidth Probe for MHL 3

The High-Bandwidth Probe shall have the following performance characteristics:

- -3dB Bandwidth:

  - Differential Probe for differential signal and single-ended signal: 13GHz or higher

  - Active Probe for common-mode signal: 2GHz or higher

#### Low-bandwidth Digital Oscilloscope

RxSense measurements require a Low-bandwidth Digital Oscilloscope:

- -3dB Bandwidth: DC to 100MHz or greater

- Sampling rate > 500Msample/sec, when 3 or more channels are simultaneously sampling.

- Sample memory: more than 1M samples per channel.

#### Passive Probe

The Passive Probe shall have the following performance characteristics to measure RSEN:

- -3dB Bandwidth: DC to 100MHz or greater

- Input resistance > 1M ohms

- Input capacitance < 200pF.

- More than 1 mega samples per second per channel.

#### Jitter and Eye Analyzer for MHL 3

The Jitter and Eye Analyzer shall function and process the MHL clock and data signals following the MHL Clock Recovery Unit and the Reference Cable Equalizer specified in MHL 3.0 specification 16.3.1.2 to 16.3.1.5. These are usually implemented in Digital Oscilloscope software.

#### MHL Signal Generator for MHL 3

The MHL 3 Signal Generator creates content streams at all modes supported by MHL 3.

- AV Data Signal Generator

  - Generate AV data for all MHL 3 defined format

  - Output data bit rates: 1.5Gbps, 3.0Gbps, 6Gbps

  - Output amplitude: 150mV – 1000mV differential

  - Jitter Generation:

    - Jitter frequency: 100 kHz to 20 MHz

    - Jitter amplitude: maximum 1 UI with 0.05 UI granularities

- eCBUS-D Signal Generator

  - Generate eCBUS-D clock and data

  - Output data bit rates: 1.5Gbps

  - Output amplitude: 150mV – 700mV differential

  - Jitter Generation:

    - Jitter frequency: 100 kHz to 20 MHz

    - Jitter amplitude: maximum 1 UI with 0.05 UI granularities

- eCBUS-S Signal Generator

  - Generate eCBUS-S clock and data

  - Output data bit rates: 750Mbps

  - Output amplitude: 150mV – 370mV differential

  - Jitter Generation:

    - Jitter frequency: 100 kHz to 20 MHz

    - Jitter amplitude: maximum 4 UI with 0.05 UI granularities

#### MHL Cable Emulator for MHL 3

The MHL 3 Cable Emulator is defined in the MHL 3 Specification, Section 16.3.1.5.

#### Transition Time Converter (TTC) for MHL 3

The transition time converter shall slow down the rise and fall times of output signals of MHL Signal Generator to the following values:

- TMDS: 60ps (differential 20-80%)

- eCBUS-D:  250ps (differential 20-80%)

- eCBUS-S: 1000ps (single-ended 20-80%)

#### Combiner for MHL 3

- DC to 18 GHz

- Rise time 17 picoseconds typical

- DC insertion loss 6 dB ± 0.05 dB maximum

#### AC insertion loss 6 dB ± 0.5 dB maximumBias Tee for MHL 3

- Wideband, 0.1 – 12 GHz

- Low insertion loss, 0.6 dB typical

- Good isolation, 40 dB typical

#### Network Analyzer

- 4 ports

- 300 kHz to 8GHz

- Dynamic accuracy over the frequency range 300kHz – 8GHz

  - Magnitude:  <= (+/-)0.5dB from 0 to - 50dBm

  - Phase:  <= (+/-)4 degrees from 0 to - 50dBm

#### TDR/TDT Oscilloscope

For TDR measurement:

- Bandwidth:  >= 18GHz

- Pulse rise time: <= 75ps (10-90%)

- 2 port

- Rise time adjustment capability to 200ps (20-80% differential) and 600ps (20-80% common-mode)

For TDT measurement:

- Bandwidth:  >= 18GHz

- Pulse rise time: <= 75ps (10-90%)

- 4 port

#### Relay/Echo Cancel/Level Converter Termination  (RELT) Board

Figure 2-2. Relay/Echo Cancel/Level Converter Termination Board

- Relay

  - Bandwidth: >= 18GHz

  - Switch time: < 50ms

- Level Converter

  - Max speed: >= 3GHz

  - Input : TMDS

  - Output amplitude:  200mV – 500mV for 50ohm

#### DummyTermination Board

Dummy Termination Board is required for Sink, Dongle and Direct Attach Sink testing. The Dummy Termination Board shall be prepared by the DUT vendor and submitted along with the DUT. The Dummy Termination Board can be the DUT itself, set to BIST IMPEDANCE_MODE to represent the impedance characteristics of the DUT and is used for subtracting eCBUS-S FWD or eCBUS-D FWD  signal from the bi-directional eCBUS-S or eCBUS-D signal as shown in Sections 4.7.1 and 5.7.1.

#### EMI Source Device

EMI Source device is used for Cable EMI test. It shall generate 6Gbps AV signal of PRBS pattern and 480 mode as defined in MHL3 BIST specification. It shall generate eCBUS-S FWD clock and PRBS data pattern, or eCBUS-D FWD clock and PRBS data pattern, as defined in MHL3 BIST specification. The Source device shall be enclosed in a metal shielding box.

#### EMI Sink Device

EMI Sink device is used for Cable EMI test. It shall receive 6Gbps AV signal of PRBS pattern and 480 mode as defined in MHL3 BIST specification. It shall generate eCBUS-S BWD PRBS data pattern, or eCBUS-D BWD PRBS data pattern, as defined in MHL3 BIST specification. The Sink device shall be enclosed in a metal shielding box.

### MHL System-level Equipment

MHL System Tests also referred to AVLINK tests in MHL3 consist of 2 levels of testing:

- MHL3 DUT tested as an MHL3 device using MHL3 test equipment

- MHL 3 DUT tested as a legacy MHL2.x/ 1.x device using MHL 2.x/1.x test equipment.

The MHL 3 testing is used to verify compliance in MHL3 operation modes and legacy (MHL 2.x/1.x ) testing is used to verify compliance in legacy support mode. The specific system configurations for both MHL3 and legacy are described in the vendor specific MOI sections.

There is a table at the beginning of sections 3.2, 3.8, 4.2, 4.8, 5.2, 5.8 which lists the tests numbers and the modes the device is tested under.

#### MHL3 AVLink Protocol Analyzer

The MHL3 AVLink Protocol Analyzer is able to take in a complete stream of content with discovery, authentication, etc.

- MHL3 Pseudo Sink: Receive MHL3 stream and display on HDMI capable TV.

  - Receive MHL3 A/V stream, decode and re-transmit in HDMI format.

  - Detect A/V format, pixel encoding, pixel repetition.

  - Measure A/V timing.

  - Analyze A/V data and AVI InfoFrame.

  - Monitor EDID access.

  - Verify RBP and other MHL 3 system commands.

#### MHL3 AVLink Signal Generator

The MHL3 AVLink Signal Generator creates content streams at all modes supported by MHL3.

- Generate MHL3 clock and data for all MHL3 defined format

- Maximum output data bit rate: > 6Gbps

  - granularitiesGenerate A/V format.

  - Generate A/V test pattern.

  - EDID access

  - Verify RBP and other MHL 3 system commands.

Figure 2-3 and Figure 2-4 show the connection from MHL3 Signal Generator to the Sink DUT and to the Dongle DUT. The Dongle DUT requires a downstream Sink device to show the signal. A Sink DUT which has no display requires a downstream display device to show the signal.

Figure 2-3. Recommended MHL3 Signal Generator and Adapter for Dongle Test.

Figure 2-4. Recommended MHL3 Signal Generator and Adapter for Sink Test.

### CBUS Equipment

#### MHL3-CBUS Protocol Generator / Analyzer

The MHL3 CBUS Protocol Generator / Analyzer is a test tool for analyzing and exercising Mobile High-Definition Link (MHL3) CBUS protocol.The tool comprising custom hardware and software is capable of generating and running CBUS sequences for limits testing of the MHL3 Certification Test Specification, and generating validation reports for in spec and out of spec conditions.

The tool:

- Generates CBUS (oCBUS, eCBUS-S, eCBUS-D) sequences for emulating either Source or Sink MHL3 devices.

- Captures CBUS, VBUS and MHL3 activity to a file, decodes the CBUS stream and flags errors.

#### Digital  Oscilloscope

The Digital Oscilloscope is a digital waveform viewing instrument that allows observation of constantly varying signal voltages.

- -3dB Bandwidth: DC to 500MHz or greater

- Sampling rate > 1Gsample/sec, when 2 or more channels are simultaneously sampling.

- Minimum 2 channel, 4 channel preferred

#### CBUS Source Board

A CBUS Source  board provides CBUS connection and discovery to an MHL3 Sink or MHL3 Dongle DUT. The CBUS Source Board can be an MHL3 Source device or an MHL3 CBUS Protocol Generator / Analyzer or an MHL3 RX/TX Analyzer.

Figure 2-5. CBUS Source Board

#### CBUS Sink Board

A CBUS Sink board provides CBUS connection and discovery to MHL3 Source DUT. The CBUS Sink board can be an MHL3 Sink device or an MHL3 CBUS Protocol Generator / Analyzer or an MHL3 RX/TX Analyzer.

Figure 2-6. CBUS Sink Board

### HDCP Equipment

#### HDCP Analyzer

The HDCP Analyzer is used for testing HDCP Transmitters by checking the stream of TMDS and the DDC traffic (via CBUS) for proper authentication and encryption.

Figure 2-7. MHL HDCP Analyzer

#### HDCP Emulator

The HDCP Emulator is used for testing HDCP Receivers, by creating a TMDS stream and managing the DDC transactions (via CBUS) in parallel for HDCP authentication. Responses from the Receiver are checked for compliance.  The output of the Receiver-under-test is checked to show proper decryption.

Figure 2-8. MHL HDCP Emulator

### Adapters

#### Test Point Access (TPA) Boards for MHL 3

A TPA board provides connection between a DUT and test equipment. It includes a mating connector (Source side Plug, Source side Receptacle, Sink side Plug, or Sink side Receptacle) and connection ports (TMDS+, TMDS-, VBus, eCBUS-S, eCBUS-D+, eCBUS-D- and GND).

Figure 2-9. TPA Board for DUT with eCBUS-S (a) and for DUT with eCBUS-D (b)

The following types of MHL TPA boards are used in the subsequent CTS testing procedures for MHL3. They are differentiated by the mating connector type.

Table 2-1. MHL3 TPA Boards Description

| MHL3 TPA | Description | MHL3 Testing | Notes |
| --- | --- | --- | --- |
| M3-S TPA SRC-P | Source side Plug connector with eCBUS-S | Source DUT Testing Dongle or DA Sink Test Signal Calibration |  |
| M3-D TPA SRC-P | Source side Plug connector with eCBUS-D | Source DUT Testing Dongle or DA Sink Test Signal Calibration |  |
| M3-S TPA SRC-R | Source side Receptacle connector with eCBUS-S | Source Test Signal Calibration Dongle or DA Sink DUT Testing Cable DUT Testing (Source side) |  |
| M3-D TPA SRC-R | Source side Receptacle connector with eCBUS-D | Source Test Signal Calibration Dongle or DA Sink DUT Testing Cable DUT Testing (Source side) |  |
| M3-S TPA SNK-P M3-S TPA DirSRC-P | Sink side Plug connector with eCBUS-S | Sink DUT Testing DA Source Test Signal Calibration | 1 |
| M3-D TPA SNK-P M3-D TPA DirSRC-P | Sink side Plug connector with eCBUS-D | Sink DUT Testing DA Source Test Signal Calibration | 1 |
| M3-S TPA SNK-R M3-S TPA DirSRC-R | Sink side Receptacle connector with eCBUS-S | Sink Test Signal Calibration DA Source DUT Testing Cable DUT Testing (Sink side) | 1 |
| M3-D TPA SNK-R M3-D TPA DirSRC-R | Sink side Receptacle connector with eCBUS-D | Sink Test Signal Calibration DA Source DUT Testing Cable DUT Testing (Sink side) | 1 |

Note on Table 2-1:

- Direct Attach Source, with part number including 'DirSRC', uses Sink side connector.

#### RxSense (RSEN) Test Point Access (TPA) Boards

The following two types of TPA boards will be used for RxSense Impedance testing on Source, Sink, and Dongle DUTs:

- Source TPA-RSEN: Used for Source DUT RxSense Impedance detection test

- Sink/Dongle TPA-RSEN: Used for Sink and Dongle DUTs RxSense Impedance test

Figure 2-10. TPA Board for Source RxSense Test (Source TPA-RSEN)

Figure 2-11. TPA Board for Sink and Dongle RxSense Test (Sink/Dongle TPA-RSEN)

#### MHL3 Pass-Through Test Point Adapter (TPA)

The MHL3 Pass-Through Test Point Adapter (TPA) is a passive adapter board that allows MHL3 signals (MHL+, MHL–, eCBUSD+, eCBUS-D-, eCBUS-S, oCBUS, Cable Detect) to be probed while the link is in use.

### Miscellaneous

#### SMA Coaxial Cable

- -3dB Bandwidth: 10GHz or greater

- 50 ohm impedance

#### DC Source-Meter

- ISVM: Can measure the voltage (0 – 10V, 0.1mV resolution and accuracy or better) with controlling the drain current (0 – 1A, 0.1uA resolution and accuracy or better)

#### Digital Multi-meter

A DC Multi-meter is needed to measure DC voltages at various test points.

- DC Voltage

  - DC voltage resolution <= 1uV when range is 0 – 1mV

  - DC voltage accuracy <= +/- 10uV when range is 0 – 1mV

- DC Resistance

  - DC resistance resolution is more than 3 digits

  - DC resistance accuracy <= 1%

  - Maximum measurable resistance >= 1M ohms

#### DC Power Supply

A DC Power Supply is needed to drive the TPA boards. A dual mode power supply would be preferred.

- Voltage range: 0 – 10V

- Voltage accuracy: <= +/- 1%

- Current: maximum 1A

#### DC Block

- -3dB BW: 18GHz or greater

## Required Video and Audio Patterns

Some tests in this specification require the generation or measurement of specific video patterns or audio content streams.

There may be new patterns included to support specific tests which are TBD at this time.

### Required Video Patterns

#### Gray Ramp Pattern

The Gray Ramp pattern creates a full video field or frame with a changing color, ranging from black on the left to white on the right, repeating across the width of the screen. The pixel data values for the range from 'black' to 'white' depend on the color space used on the link.  A formula for 'gray' in RGB and YCbCr color spaces is shown in Figure 2-12.

{Red=Green=Blue=Modi+Black,Whitefor i=0 to LineWidth-1 step 1 } for each line in frame

{Luma=Modi+Black,White, Chroma=128for i=0 to LineWidth-1 step 1

Figure 2-12. Gray Ramp Color Derivation

The limited range and full range values for R, G, B, Y, Cb and Cr for 'black' and 'white' are defined in the CEA-861E Specification, Section 5.4.

The Gray Ramp pattern for PackedPixel mode is the corresponding pattern varying from the minimum luma on the left to the maximum luma on the right.

Figure 2-13. Representative Gray Ramp Video Patterns

Representative Gray Ramp patterns are shown in Figure 2-13, with a 1920x1080 pattern on the left, and a 720x480 pattern on the right.

MHL Source DUTs are required to output this Gray Ramp pattern in some tests. MHL Sink and Dongle DUTs are required to render, or pass downstream this Gray Ramp pattern in some tests. Refer to each test’s required methodology for specific video timings, normal versus PackedPixel mode, etc.

Some test equipment is also required to output this Gray Ramp pattern in some tests. Refer to each test's required methodology for specific video timings, normal versus PackedPixel mode, etc.

#### Chess Board Pattern

The fully saturated black and white Chess Board pattern is required in the Source pixel encoding test and the Source full/limited range test. MHL Source DUTs are required to output this Chess Board pattern in some tests.

For the Chess Board pattern, the Source device creates forty-eight blocks of color, either black or white, as shown in Figure 2-14.  Each rectangular region thus created alternates from black in one region to white in the neighboring regions, beginning with a white rectangle in the upper-left portion of the screen, and ending with a white rectangle in the lower-right portion of the screen.

Note that exact dimensions in pixels and rows of each rectangular region will vary according to the video format, but the color will change seven times moving left-to-right across each row, and change five times moving top-to-bottom on each column.

The limited range and full range values for R, G, B, Y, Cb and Cr are defined in the CEA-861E Specification, Section 5.4.

Figure 2-14. Chess Board Video Pattern

#### 3D Video Patterns

No specific 3D video pattern is required, but when testing 3D video mode support the pattern output from the Source DUT should distinctly show the desired 3D effect.

### Required Audio Patterns

#### 1 Kilohertz Audio Sinusoidal Tone

An MHL Source DUT, while being tested, shall be capable to generate a 1 kHz sinusoidal tone at a 32kHz, 44.1kHz , and all other sampling rates supported by the device up to 192kHz on its MHL output.

An MHL Signal Generator shall be capable to generate a 1kHz sinusoidal tone at 32kHz, 44.1kHz, 48kHz and other sampling rates up to 192kHz.

## Measurement Methods

### CBUS Electrical Measurements

#### CBUS Voltage Level Measurement Technique

Various measurement points for determining the HIGH and LOW logic levels on CBUS are shown in Figure 2-15.

##### Measuring Falling Edge Rate

The measurement is the time from when CBUS voltage falls below VOH_CBUS{min} [Point A in the figure] until the voltage falls below VOL_CBUS{max} [Point B in the figure].

##### Measuring Rising Edge Rate

The measurement is the time from when CBUS voltage rises above VOL_CBUS{max} [Point D in the figure] until the voltage rises above VOH_CBUS{min} [Point E in the figure].

##### Measuring Drive High Time

The measurement is the time from when the CBUS voltage starts rising above its stable LOW value [Point C in the figure] until the time the voltage starts dropping after the drive is removed [Point F in the figure].

Figure 2-15. CBUS Voltage Level Waveform

## Revision History

Each released version of this specification shall record the highlights of its changes in this section. Cross-references to the pertinent pages may be included as appropriate.

Revision	 Highlights

MHL Compliance Test Specification Revision 3.0

Add tests for eCBUS modifications and all new features of MHL 3.0.

## Normative References

| Abbreviation | Document |
| --- | --- |
| CTS 1.x | MHL Compliance Test Specification 1.3. |
| CTS 2.x | MHL Compliance Test Specification 2.1. |

## Organization of this Document

The advent of MHL 3 has created the requirement to test for backward-compatibility with MHL 1 and MHL 2 devices when connected to the MHL 3 device-under-test. In addition, since MHL 3 supports many of the AV Link modes and CBUS protocols from MHL 1 and MHL 2, tests are needed to check for those "MHL 1" and "MHL 2" modes across a connection between two MHL 3 devices.

In order to assess compliance to previous CTS documents, this CTS includes sections which make normative reference to the MHL 1 CTS and MHL 2 CTS.

CTS 1 and CTS 2 each included a Section 6.3, with tests to execute over one or more device category. This required the reader to examine not only Section 3 for Source device testing, but also Section 6.3; for Sinks (Sections 4 and 6.3); and for Dongles (Sections 5 and 6.3).

In CTS 3 several new sub-sections have been added to each of Sections 3, 4 and 5:

- All tests required for Sources are listed within Section 3.

- All tests for Sinks are listed within Section 4.

- All tests for Dongles are listed within Section 5.

| Source | Sink | Dongle | DUT demonstrates compliance to requirements of … |
| --- | --- | --- | --- |
| 3.1 | 4.1 | 5.1 | legacy device Electrical tests, each a normative reference to CTS 2. |
| 3.2 | 4.2 | 5.2 | legacy device System tests, each a normative reference to CTS 2. |
| 3.3 | 4.3 | 5.3 | legacy device CBUS protocol tests, each a normative reference to CTS 2. |
| 3.4 | 4.4 | 5.4 | legacy device CBUS behavioral tests, from Section 6.3. |
| 3.5 | 4.5 | 5.5 | MHL 3 device eCBUS normal tests, from Section 6.4. |
| 3.6 | 4.6 | 5.6 | MHL 3 device eCBUS robustness tests, from Section 6.5. |
| 3.7 | 4.7 | 5.7 | MHL 3 device Electrical tests. |
| 3.8 | 4.8 | 5.8 | MHL 3 device System tests. |
| 6.3 | Common tests to check oCBUS operation across Legacy, eCBUS-S and eCBUS-D. |  |  |
| 6.4 | Common tests to check eCBUS normal operation across eCBUS-S and eCBUS-D. |  |  |
| 6.5 | Common tests to check eCBUS robustness operation across eCBUS-S and eCBUS-D. |  |  |
| 7 | Cable tests for MHL 3 cable. |  |  |

## CDF Field Names

Individual tests include in their definitions various field values obtained from the Adopter's Capability Declaration form (CDF). Aside from the symbols used in tests, the CDF provides additional information, the field names for which are listed in the following subsections.

Informational CDF Fields

The following CDF field names provide general information about the DUT, and are not cited in individual tests.

CDF_PRIMARY_CONTACT_NAME

CDF_CONTACT_NAME

CDF_DATE

CDF_MFR_NAME

CDF_MFR_PRIMARY_CONTACT_NAME

CDF_MFR_EMAIL

CDF_MFR_PHONE

CDF_MFR_ADDRESS_1

CDF_MFR_ADDRESS_2

CDF_MFR_CITY

CDF_MFR_STATE_PROVINCE

CDF_MFR_POSTAL_CODE

CDF_MFR_COUNTRY

CDF_CONTACT_NAME

CDF_CONTACT_EMAIL

CDF_CONTACT_PHONE

CDF_ADOPTER_NAME

CDF_ADOPTER_COMPANY_NAME

CDF_ADOPTER_EMAIL

CDF_ADOPTER_NAME

CDF_ADOPTER_ADDRESS_1

CDF_ADOPTER_ADDRESS_2

CDF_ADOPTER_CITY

CDF_ADOPTER_STATE_PROVINCE

CDF_ADOPTER_POSTAL_CODE

CDF_ADOPTER_COUNTRY

CDF_MFR_POWER_SUPPLY_VOLTS

CDF_MODEL_NUMBER

CDF_PRODUCT_TYPE

CDF_PRODUCT_BRAND_NAME

CDF_FAMILY_MODELS

CDF_OP_SYS_NAME

CDF_OP_SYS_VER

CDF_APPL_NAME

CDF_APPL_VER

Capability Register and Extended Capability Register CDF Fields

The following CDF field names form the components of the Device Capability and Extended Device Capability registers in the MHL DUTs. These registers' values are checked in various tests in the CTS, but these individual field names are not cited in the tests for brevity. They are included here to coordinate the use of field names between the CTS and the CDF.

CDF_CR_MHL_VER_MAJOR

CDF_CR_MHL_VER_MINOR

CDF_CR_DEV_TYPE

CDF_CR_PLIM

CDF_CR_POW

CDF_CR_ADOPTER_ID_H

CDF_CR_ADOPTER_ID_L

CDF_CR_SUPP_RGB444

CDF_CR_SUPP_YCBCR444

CDF_CR_SUPP_PPIXEL

CDF_CR_SUPP_ISLANDS

CDF_CR_SUPP_VGA

CDF_CR_SUPP_16BPP

CDF_CR_AUD_2CH

CDF_CR_AUD_8CH

CDF_CR_AUD_HBR

CDF_CR_VT_GRAPHICS

CDF_CR_VT_PHOTO

CDF_CR_VT_CINEMA

CDF_CR_VT_GAME

CDF_CR_SUPP_VT

CDF_CR_LD_DISPLAY

CDF_CR_LD_VIDEO

CDF_CR_LD_AUDIO

CDF_CR_LD_MEDIA

CDF_CR_LD_TUNER

CDF_CR_LD_RECORD

CDF_CR_LD_SPEAKER

CDF_CR_LD_GUI

CDF_CR_BANDWIDTH

CDF_CR_RCP_SUPPORT

CDF_CR_RAP_SUPPORT

CDF_CR_SP_SUPPORT

CDF_CR_DEVICE_ID_H

CDF_CR_DEVICE_ID_L

CDF_CR_SCRATCHPAD_SIZE

CDF_CR_INT_SIZE

CDF_CR_STAT_SIZE

CDF_CR_UCP_RECV_SUPPORT

CDF_CR_UCP_SEND_SUPPORT

CDF_CR_XVYCC_SUPPORT

CDF_CR_RBP_SUPPORT

CDF_CR_ECBUS_S_075

CDF_CR_ECBUS_S_8BIT

CDF_CR_ECBUS_S_12BIT

CDF_CR_ECBUS_D_150

CDF_CR_ECBUS_D_8BIT

CDF_CR_TMDS_150

CDF_CR_TMDS_300

CDF_CR_TMDS_600

CDF_CR_DEV_HOST

CDF_CR_DEV_DEVICE

CDF_CR_DEV_CHARGER

CDF_CR_HID_HOST

CDF_CR_HID_DEVICE

CDF_CR_LD_PHONE

RCP Fields

The following CDF fields are used in checking RCP sending and receiving behavior in the MHL DUT. These fields are used in the CDF, but are not listed individually in the test descriptions, for brevity.

CDF_RCP_RCV_BEHAVIOR_00

CDF_RCP_RCV_BEHAVIOR_01

CDF_RCP_RCV_BEHAVIOR_02

CDF_RCP_RCV_BEHAVIOR_03

CDF_RCP_RCV_BEHAVIOR_04

CDF_RCP_RCV_BEHAVIOR_05

CDF_RCP_RCV_BEHAVIOR_06

CDF_RCP_RCV_BEHAVIOR_07

CDF_RCP_RCV_BEHAVIOR_08

CDF_RCP_RCV_BEHAVIOR_09

CDF_RCP_RCV_BEHAVIOR_0A

CDF_RCP_RCV_BEHAVIOR_0B

CDF_RCP_RCV_BEHAVIOR_0C

CDF_RCP_RCV_BEHAVIOR_0D

CDF_RCP_RCV_BEHAVIOR_20

CDF_RCP_RCV_BEHAVIOR_21

CDF_RCP_RCV_BEHAVIOR_22

CDF_RCP_RCV_BEHAVIOR_23

CDF_RCP_RCV_BEHAVIOR_24

CDF_RCP_RCV_BEHAVIOR_25

CDF_RCP_RCV_BEHAVIOR_26

CDF_RCP_RCV_BEHAVIOR_27

CDF_RCP_RCV_BEHAVIOR_28

CDF_RCP_RCV_BEHAVIOR_29

CDF_RCP_RCV_BEHAVIOR_2A

CDF_RCP_RCV_BEHAVIOR_2B

CDF_RCP_RCV_BEHAVIOR_2C

CDF_RCP_RCV_BEHAVIOR_30

CDF_RCP_RCV_BEHAVIOR_31

CDF_RCP_RCV_BEHAVIOR_32

CDF_RCP_RCV_BEHAVIOR_33

CDF_RCP_RCV_BEHAVIOR_34

CDF_RCP_RCV_BEHAVIOR_35

CDF_RCP_RCV_BEHAVIOR_36

CDF_RCP_RCV_BEHAVIOR_37

CDF_RCP_RCV_BEHAVIOR_38

CDF_RCP_RCV_BEHAVIOR_41

CDF_RCP_RCV_BEHAVIOR_42

CDF_RCP_RCV_BEHAVIOR_43

CDF_RCP_RCV_BEHAVIOR_44

CDF_RCP_RCV_BEHAVIOR_45

CDF_RCP_RCV_BEHAVIOR_46

CDF_RCP_RCV_BEHAVIOR_47

CDF_RCP_RCV_BEHAVIOR_48

CDF_RCP_RCV_BEHAVIOR_49

CDF_RCP_RCV_BEHAVIOR_4A

CDF_RCP_RCV_BEHAVIOR_4B

CDF_RCP_RCV_BEHAVIOR_4C

CDF_RCP_RCV_BEHAVIOR_50

CDF_RCP_RCV_BEHAVIOR_51

CDF_RCP_RCV_BEHAVIOR_60

CDF_RCP_RCV_BEHAVIOR_61

CDF_RCP_RCV_BEHAVIOR_62

CDF_RCP_RCV_BEHAVIOR_63

CDF_RCP_RCV_BEHAVIOR_64

CDF_RCP_RCV_BEHAVIOR_65

CDF_RCP_RCV_BEHAVIOR_66

CDF_RCP_RCV_BEHAVIOR_67

CDF_RCP_RCV_BEHAVIOR_68

CDF_RCP_RCV_BEHAVIOR_71

CDF_RCP_RCV_BEHAVIOR_72

CDF_RCP_RCV_BEHAVIOR_73

CDF_RCP_RCV_BEHAVIOR_74

CDF_RCP_RCV_BEHAVIOR_75

CDF_RCP_RCV_BEHAVIOR_7E

CDF_RCP_SEND_PROCEDURE_00

CDF_RCP_SEND_PROCEDURE_01

CDF_RCP_SEND_PROCEDURE_02

CDF_RCP_SEND_PROCEDURE_03

CDF_RCP_SEND_PROCEDURE_04

CDF_RCP_SEND_PROCEDURE_05

CDF_RCP_SEND_PROCEDURE_06

CDF_RCP_SEND_PROCEDURE_07

CDF_RCP_SEND_PROCEDURE_08

CDF_RCP_SEND_PROCEDURE_09

CDF_RCP_SEND_PROCEDURE_0A

CDF_RCP_SEND_PROCEDURE_0B

CDF_RCP_SEND_PROCEDURE_0C

CDF_RCP_SEND_PROCEDURE_0D

CDF_RCP_SEND_PROCEDURE_20

CDF_RCP_SEND_PROCEDURE_21

CDF_RCP_SEND_PROCEDURE_22

CDF_RCP_SEND_PROCEDURE_23

CDF_RCP_SEND_PROCEDURE_24

CDF_RCP_SEND_PROCEDURE_25

CDF_RCP_SEND_PROCEDURE_26

CDF_RCP_SEND_PROCEDURE_27

CDF_RCP_SEND_PROCEDURE_28

CDF_RCP_SEND_PROCEDURE_29

CDF_RCP_SEND_PROCEDURE_2A

CDF_RCP_SEND_PROCEDURE_2B

CDF_RCP_SEND_PROCEDURE_2C

CDF_RCP_SEND_PROCEDURE_30

CDF_RCP_SEND_PROCEDURE_31

CDF_RCP_SEND_PROCEDURE_32

CDF_RCP_SEND_PROCEDURE_33

CDF_RCP_SEND_PROCEDURE_34

CDF_RCP_SEND_PROCEDURE_35

CDF_RCP_SEND_PROCEDURE_36

CDF_RCP_SEND_PROCEDURE_37

CDF_RCP_SEND_PROCEDURE_38

CDF_RCP_SEND_PROCEDURE_41

CDF_RCP_SEND_PROCEDURE_42

CDF_RCP_SEND_PROCEDURE_43

CDF_RCP_SEND_PROCEDURE_44

CDF_RCP_SEND_PROCEDURE_45

CDF_RCP_SEND_PROCEDURE_46

CDF_RCP_SEND_PROCEDURE_47

CDF_RCP_SEND_PROCEDURE_48

CDF_RCP_SEND_PROCEDURE_49

CDF_RCP_SEND_PROCEDURE_4A

CDF_RCP_SEND_PROCEDURE_4B

CDF_RCP_SEND_PROCEDURE_4C

CDF_RCP_SEND_PROCEDURE_50

CDF_RCP_SEND_PROCEDURE_51

CDF_RCP_SEND_PROCEDURE_60

CDF_RCP_SEND_PROCEDURE_61

CDF_RCP_SEND_PROCEDURE_62

CDF_RCP_SEND_PROCEDURE_63

CDF_RCP_SEND_PROCEDURE_64

CDF_RCP_SEND_PROCEDURE_65

CDF_RCP_SEND_PROCEDURE_66

CDF_RCP_SEND_PROCEDURE_67

CDF_RCP_SEND_PROCEDURE_68

CDF_RCP_SEND_PROCEDURE_71

CDF_RCP_SEND_PROCEDURE_72

CDF_RCP_SEND_PROCEDURE_73

CDF_RCP_SEND_PROCEDURE_74

CDF_RCP_SEND_PROCEDURE_75

CDF_RCP_SEND_PROCEDURE_7E

RBP Fields

The following CDF fields are used in checking RBP sending and receiving behavior in the MHL DUT. These fields are used in the CDF, but are not listed individually in the test descriptions, for brevity.

CDF_RBP_RCV_BEHAVIOR_01

CDF_RBP_RCV_BEHAVIOR_02

CDF_RBP_RCV_BEHAVIOR_03

CDF_RBP_RCV_BEHAVIOR_04

CDF_RBP_RCV_BEHAVIOR_05

CDF_RBP_RCV_BEHAVIOR_06

CDF_RBP_RCV_BEHAVIOR_07

CDF_RBP_RCV_BEHAVIOR_20

CDF_RBP_RCV_BEHAVIOR_21

CDF_RBP_RCV_BEHAVIOR_30

CDF_RBP_RCV_BEHAVIOR_31

CDF_RBP_RCV_BEHAVIOR_32

CDF_RBP_RCV_BEHAVIOR_33

CDF_RBP_RCV_BEHAVIOR_34

CDF_RBP_RCV_BEHAVIOR_35

CDF_RBP_SEND_PROCEDURE_01

CDF_RBP_SEND_PROCEDURE_02

CDF_RBP_SEND_PROCEDURE_03

CDF_RBP_SEND_PROCEDURE_04

CDF_RBP_SEND_PROCEDURE_05

CDF_RBP_SEND_PROCEDURE_06

CDF_RBP_SEND_PROCEDURE_07

CDF_RBP_SEND_PROCEDURE_20

CDF_RBP_SEND_PROCEDURE_21

CDF_RBP_SEND_PROCEDURE_30

CDF_RBP_SEND_PROCEDURE_31

CDF_RBP_SEND_PROCEDURE_32

CDF_RBP_SEND_PROCEDURE_33

CDF_RBP_SEND_PROCEDURE_34

CDF_RBP_SEND_PROCEDURE_35

# Source Test

Source devices shall be tested with HDCP disabled, with the exception of tests in Section 3.8.4.

## Electrical Tests

TMDS Electrical Tests

All tests in the CTS 2, Section 3.1.1, shall be run against the Source DUT.

## System Tests

General Test Setups

This test group defines the MHL3 legacy compliance testing in MHL 1 / MHL 2 mode.

All MHL3 devices have an MHL 1 / MHL 2 compatibility mode test requirement.  Refer to MHL CTS 2 test procedures for the specific test steps for legacy tests.  The general Required Methodology for all Source DUT tests with normative references to CTS 2.1 are tested in oCBUS mode. Test results for legacy mode tests shall be recorded in the TRF by their test ID as listed in Table 3-1 below.

Table 3-1. AVLINK MHL2 Source Test Sequence

| Test ID | MHL 2 oCBUS Compatibility mode |
| --- | --- |
| 3.2.2.1 | X |
| 3.2.2.2 | X |
| 3.2.2.3 | X |
| 3.2.2.4 | X |
| 3.2.2.5 | X |
| 3.2.2.6 | X |
| 3.2.3.1 | X |
| 3.2.3.2 | X |
| 3.2.3.3 | X |
| 3.2.3.4 | X |
| 3.2.3.5 | X |
| 3.2.3.6 | X |
| 3.2.3.7 | X |
| 3.2.4.1 | X |
| 3.2.4.2 | X |
| 3.2.4.3 | X |
| 3.2.5.1 | X |
| 3.2.6.1 | X |
| 3.2.6.2 | X |
| 3.2.6.3 | X |
| 3.2.6.4 | X |
| 3.2.7.1 | X |
| 3.2.7.2 | X |
| 3.2.8.1 | X |
| 3.2.9.1 | X |
| 3.2.9.2 | X |
| 3.2.9.3 | X |
| 3.2.9.4 | X |
| 3.2.10.1 | X |
| 3.2.10.2 | X |

## CBUS Tests

Tests in Section 3.3 of the released MHL 2 Compliance Test Specification are executed using the varieties of CBUS as noted in Table 3-2. As each test is executed in Section 3.3, the test number beginning with "3.3", together with the CBUS mode as invoked from Table 3-2, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 3-2, according to which columns of Table 3-2 are marked with an 'X'.

Table 3-2. CBUS MHL 2 Source Test Sequence

| Test ID | MHL 2 oCBUS Compatibility mode |
| --- | --- |
| 3.3.3.2 | X |
| 3.3.3.3 | X |
| 3.3.4.1 | X |
| 3.3.5.1 | X |
| 3.3.5.2 | X |
| 3.3.5.3 | X |
| 3.3.5.4 | X |
| 3.3.5.5 | X |
| 3.3.6.1 | X |
| 3.3.6.2 | X |
| 3.3.6.3 | X |
| 3.3.6.4 | X |
| 3.3.7.1 | X |
| 3.3.7.2 | X |
| 3.3.7.3 | X |
| 3.3.7.4 | X |
| 3.3.7.5 | X |
| 3.3.8.1 | X |
| 3.3.8.2 | X |
| 3.3.9.1 | X |
| 3.3.9.2 | X |
| 3.3.10.1 | X |
| 3.3.11.1 | X |
| 3.3.11.2 | X |
| 3.3.12.1 | X |
| 3.3.12.2 | X |
| 3.3.12.3 | X |
| 3.3.13.1 | X |
| 3.3.13.2 | X |
| 3.3.14.1 | X |
| 3.3.14.2 | X |
| 3.3.14.3 | X |
| 3.3.15.1 | X |
| 3.3.16.1 | X |
| 3.3.16.2 | X |
| 3.3.17.1 | X |
| 3.3.18.1 | X |
| 3.3.19.1 | X |
| 3.3.20.1 | X |
| 3.3.21.1 | X |
| 3.3.22.1 | X |
| 3.3.22.2 | X |
| 3.3.22.3 | X |
| 3.3.23.2 | X |
| 3.3.23.3 | X |
| 3.3.23.4 | X |
| 3.3.23.5 | X |

## eCBUS oCBUS / MHL 3 Protocol Tests

Tests in Section 6.3 of the released MHL 2 Compliance Test Specification are executed using the varieties of CBUS as noted in Table 3-3. As each test is executed in Section 6.3, the test number beginning with "6.3", together with the CBUS mode as invoked from Table 3-3, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 3-3, according to which columns of Table 3-3 are marked with an 'X'.

Table 3-3. eCBUS oCBUS / MHL 3 Suite Normal Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.3.1.1 | X | X | If DUT Supports |
| 6.3.2.1 | X | X | If DUT Supports |
| 6.3.3.1 | X | X | If DUT Supports |
| 6.3.3.2 | X | X | If DUT Supports |
| 6.3.3.3 | X | X | If DUT Supports |
| 6.3.3.4 | X | X | If DUT Supports |
| 6.3.3.5 | X | X | If DUT Supports |
| 6.3.3.6 | X | X | If DUT Supports |
| 6.3.3.7 | X | X | If DUT Supports |
| 6.3.5.1 | X | X | If DUT Supports |
| 6.3.5.2 | X | X | If DUT Supports |
| 6.3.5.3 | X | X | If DUT Supports |
| 6.3.5.4 | X | X | If DUT Supports |
| 6.3.5.5 | X | X | If DUT Supports |
| 6.3.5.6 | X | X | If DUT Supports |
| 6.3.5.7 | X | X | If DUT Supports |
| 6.3.5.8 | X | X | If DUT Supports |
| 6.3.8.1 | X | X | If DUT Supports |
| 6.3.8.2 | X | X | If DUT Supports |
| 6.3.10.1 | X | X | If DUT Supports |
| 6.3.10.2 | X | X | If DUT Supports |
| 6.3.10.3 | X | X | If DUT Supports |
| 6.3.10.4 | X | X | If DUT Supports |
| 6.3.10.5 | X | X | If DUT Supports |
| 6.3.10.6 | X | X | If DUT Supports |
| 6.3.10.7 | X | X | If DUT Supports |
| 6.3.10.8 | X | X | If DUT Supports |
| 6.3.11.20 | X | X | If DUT Supports |
| 6.3.11.21 | X | X | If DUT Supports |
| 6.3.13.1 | X | X | If DUT Supports |
| 6.3.13.2 | X | X | If DUT Supports |
| 6.3.14.1 | X | X | If DUT Supports |
| 6.3.14.2 | X | X | If DUT Supports |
| 6.3.15.1 |  |  |  |
| 6.3.15.2 |  |  |  |
| 6.3.17.2 | X | X | If DUT Supports |
| 6.3.17.3 | X | X | If DUT Supports |
| 6.3.18.1 | X | X | If DUT Supports |
| 6.3.18.2 | X | X | If DUT Supports |
| 6.3.18.3 | X | X | If DUT Supports |
| 6.3.18.4 | X | X | If DUT Supports |
| 6.3.20.2 |  |  |  |
| 6.3.20.3 |  |  |  |
| 6.3.21.1 |  |  |  |
| 6.3.21.2 |  |  |  |
| 6.3.21.3 |  |  |  |
| 6.3.21.4 |  |  |  |
| 6.3.21.5 |  |  |  |

Notes on Table 3-3:

- The Required Methodology text in MHL 2.x CTS calls out oCBUS parameter values which must be replaced with MHL 3 parameter values during the test of an MHL 3 DUT.

Table 3-4. eCBUS oCBUS  / MHL 3 Suite Robustness Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.3.4.1 | X | X | If DUT Supports |
| 6.3.6.1 | X | X | If DUT Supports |
| 6.3.6.2 | X | X | If DUT Supports |
| 6.3.6.3 | X | X | If DUT Supports |
| 6.3.6.4 | X | X | If DUT Supports |
| 6.3.6.5 | X | X | If DUT Supports |
| 6.3.7.1 | X | X | If DUT Supports |
| 6.3.9.1 | X | X | If DUT Supports |
| 6.3.11.1 | X | X | If DUT Supports |
| 6.3.11.2 | X | X | If DUT Supports |
| 6.3.11.3 | X | X | If DUT Supports |
| 6.3.11.4 | X | X | If DUT Supports |
| 6.3.11.5 | X | X | If DUT Supports |
| 6.3.11.6 | X | X | If DUT Supports |
| 6.3.11.7 | X | X | If DUT Supports |
| 6.3.11.8 | X | X | If DUT Supports |
| 6.3.11.9 | X | X | If DUT Supports |
| 6.3.11.10 | X | X | If DUT Supports |
| 6.3.11.11 | X | X | If DUT Supports |
| 6.3.11.12 | X | X | If DUT Supports |
| 6.3.11.13 | X | X | If DUT Supports |
| 6.3.11.14 | X | X | If DUT Supports |
| 6.3.11.15 | X | X | If DUT Supports |
| 6.3.11.16 | X | X | If DUT Supports |
| 6.3.11.17 | X | X | If DUT Supports |
| 6.3.11.19 | X | X | If DUT Supports |
| 6.3.11.22 | X | X | If DUT Supports |
| 6.3.11.23 | X | X | If DUT Supports |
| 6.3.11.24 | X | X | If DUT Supports |
| 6.3.12.1 | X | X | If DUT Supports |
| 6.3.12.2 | X | X | If DUT Supports |
| 6.3.12.3 | X | X | If DUT Supports |
| 6.3.12.4 | X | X | If DUT Supports |
| 6.3.12.5 | X | X | If DUT Supports |
| 6.3.12.6 | X | X | If DUT Supports |
| 6.3.12.7 | X | X | If DUT Supports |
| 6.3.12.8 | X | X | If DUT Supports |
| 6.3.12.9 | X | X | If DUT Supports |
| 6.3.19.1 | X | X | If DUT Supports |
| 6.3.19.2 | X | X | If DUT Supports |
| 6.3.19.3 | X | X | If DUT Supports |
| 6.3.19.4 | X | X | If DUT Supports |
| 6.3.22.1 |  |  |  |
| 6.3.22.2 |  |  |  |
| 6.3.22.3 |  |  |  |

## eCBUS MHL 3 Active Mode Source Tests

Tests in Section 6.4 of this Compliance Test Specification are executed using the varieties of CBUS as noted in Table 3-5. As each test is executed in Section 6.4, the test number beginning with "6.4", together with the CBUS mode as invoked from Table 3-5, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 3-5, according to which columns of Table 3-5 are marked with an 'X'.

Table 3-5. eCBUS MHL 3 Active Mode Source Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.4.3.1 | X |  |  |
| 6.4.3.1 | X |  |  |
| 6.4.3.2 | X |  |  |
| 6.4.3.3 | X |  |  |
| 6.4.3.4 | X |  |  |
| 6.4.3.5 | X |  |  |
| 6.4.3.6 | X |  |  |
| 6.4.3.8 | X |  |  |
| 6.4.4.1 | X |  |  |
| 6.4.5.1 | X |  |  |
| 6.4.5.2 | X |  |  |
| 6.4.5.3 | X |  |  |
| 6.4.5.4 | X |  |  |
| 6.4.7.1 |  | X |  |
| 6.4.8.1 |  |  | If DUT Supports |
| 6.4.9.1 |  | X | If DUT Supports |
| 6.4.10.1 |  | X | If DUT Supports |
| 6.4.10.2 |  | X | If DUT Supports |
| 6.4.10.3 |  | X | If DUT Supports |
| 6.4.10.4 |  | X | If DUT Supports |
| 6.4.10.5 |  | X | If DUT Supports |
| 6.4.11.1 |  | X | If DUT Supports |
| 6.4.11.2 |  | X | If DUT Supports |
| 6.4.11.3 |  | X | If DUT Supports |
| 6.4.12.1 |  | X | If DUT Supports |
| 6.4.12.2 |  | X | If DUT Supports |
| 6.4.12.3 |  | X | If DUT Supports |
| 6.4.13.1 |  | X | If DUT Supports |
| 6.4.13.2 |  | X | If DUT Supports |
| 6.4.13.3 |  | X | If DUT Supports |
| 6.4.14.1 |  | X | If DUT Supports |
| 6.4.15.1 |  | X | If DUT Supports |
| 6.4.16.1 |  | X | If DUT Supports |
| 6.4.16.3 |  | X | If DUT Supports |
| 6.4.16.4 |  | X | If DUT Supports |
| 6.4.16.5 |  | X | If DUT Supports |
| 6.4.17.1 |  | X | If DUT Supports |
| 6.4.18.1 |  | X | If DUT Supports |

## eCBUS MHL 3 Robustness Mode Source Tests

Tests in Section 6.5 of this Compliance Test Specification are executed using the varieties of CBUS as noted in Table 3-6. As each test is executed in Section 6.5, the test number beginning with "6.5", together with the CBUS mode as invoked from Table 3-6, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 3-6, according to which columns of Table 3-6 are marked with an 'X'.

Table 3-6. eCBUS MHL 3 Robustness Mode Source Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.5.1.1 |  | X | If DUT Supports |
| 6.5.1.1 |  | X | If DUT Supports |
| 6.5.2.1 |  | X | If DUT Supports |
| 6.5.2.2 |  | X | If DUT Supports |
| 6.5.2.3 |  | X | If DUT Supports |
| 6.5.3.2 |  | X | If DUT Supports |
| 6.5.3.3 |  | X | If DUT Supports |
| 6.5.3.4 |  | X | If DUT Supports |
| 6.5.4.1 |  | X | If DUT Supports |
| 6.5.5.1 |  | X | If DUT Supports |
| 6.5.5.2 |  | X | If DUT Supports |
| 6.5.5.3 |  | X | If DUT Supports |

## MHL 3 Electrical Tests

TMDS and eCBUS Electrical Test Setups

The following figures provide details on various test setups used in the MHL 3 Electrical Tests.

Figure 3-1. Test configuration at TP1 for source TMDS output and eCBUS-S FWD output

Figure 3-2. Test configuration at TP2 for source TMDS output and eCBUS-S FWD output

Figure 3-3. Test configuration at TP1 for source TMDS output and eCBUS-D FWD output

Figure 3-4. Test configuration at TP2 for source TMDS output and eCBUS-D FWD output

Figure 3-5. Test configuration at TP1 for source eCBUS-S BWD input

Figure 3-6. Test configuration at TP1 for source eCBUS-S BWD input jitter tolerance

Figure 3-7. Test configuration at TP1 for source eCBUS-D BWD input

Figure 3-8. Test configuration at TP1 for source eCBUS-D BWD input intra-pair skew and input jitter tolerance

Figure 3-9. Test configuration at TP1 for source TMDS and eCBUS-S impedance

Figure 3-10. Test configuration at TP1 for source TMDS and eCBUS-D impedance

### TMDS and eCBUS Electrical Test Procedures

Single-Ended High Level Output Voltage of Differential TMDS Data+/-: VSE_HIGH_DF_TMDS_DATA

This test confirms that the single-ended high level voltage of the differential TMDS data output is within the specified limits.

For a Source DUT with eCBUS-S, make connections shown in Figure 3-1. For a Source DUT with eCBUS-D, make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode and eCBUS TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

  - eCBUS PATTERN: PRBS

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the TMDS Data waveform, measure the mean value (VSE_HIGH_DF_TMDS_DATA) of the voltage histogram for the bits 4 and 5 of the high period.

If VTERM - 1200 mV <= VSE_HIGH_DF_TMDS_DATA  <= VTERM + 20 mV, then PASS; else FAIL.

Single-Ended Low Level Output Voltage of Differential TMDS Data+/-: VSE_LOW_DF_TMDS_DATA

This test confirms that the single-ended low level voltage of the differential TMDS data is within the specified limits.

- For a Source DUT with eCBUS-S, make connections shown in Figure 3-1. For a Source DUT with eCBUS-D, make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode and eCBUS TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

  - eCBUS PATTERN: PRBS

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the TMDS Data waveform, measure the mean value (VSE_LOW_DF_TMDS_DATA) of the voltage histogram for the bits 4 and 5 of the low period.

If VTERM - 1500 mV <= VSE_LOW_DF_TMDS_DATA  <= VTERM - 380 mV, then PASS; else FAIL.

Single-Ended High Level Output Voltage of Differential MHL CLK+/- and eCBUS-D FWD+/-: VSE_HIGH_DF_MHL_CLK, VSE_HIGH_DF_eCBUS_FWD

This test confirms that the single-ended high level voltages of the differential MHL CLK and eCBUS-D FWD are within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following eCBUS TX BIST mode and AVLINK TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low every cycle

  - eCBUS DURATION: 32.0 seconds

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the eCBUS-D FWD waveform, measure the mean value (VSE_HIGH_DF_MHL_CLK) of the voltage histogram for middle portion of Differential MHL Clock Back Porch period.

From the eCBUS-D FWD waveform, measure the mean value (VSE_HIGH_DF_eCBUS_FWD) of the voltage histogram for middle portion of eCBUS-D data high period.

If VTERM - 200 mV <= VSE_HIGH_DF_MHL_CLK  <= VTERM + 20 mV AND VTERM - 200 mV <= VSE_HIGH_DF_eCBUS_FWD  <= VTERM + 20 mV, then PASS; else FAIL.

Single-Ended Low Level Output Voltage of Differential MHL CLK+/- and eCBUS-D+/-: VSE_LOW_DF_MHL_CLK, VSE_LOW_DF_eCBUS_FWD

This test confirms that the single-ended low level voltages of the differential MHL CLK and eCBUS-D FWD are within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following eCBUS TX BIST mode and AVLINK TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low every cycle

  - eCBUS DURATION: 32.0 seconds

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the eCBUS-D FWD waveform, measure the mean value (VSE_LOW_DF_MHL_CLK) of the voltage histogram for middle portion of Differential MHL Clock Front Porch period.

From the eCBUS-D FWD waveform, measure the mean value (VSE_LOW_DF_eCBUS_FWD) of the voltage histogram for middle portion of eCBUS-D data low period.

If VTERM - 550 mV <= VSE_LOW_DF_MHL_CLK  <= VTERM - 180 mV AND VTERM - 550 mV <= VSE_LOW_DF_eCBUS_FWD  <= VTERM - 180 mV, then PASS; else FAIL.

Differential Output Swing Voltage of Differential TMDS Data: VDF_SWING_DF_TMDS_DATA

This test confirms that the differential swing voltage of the differential TMDS data is within the specified limits.

- For a Source DUT with eCBUS-S, make connections shown in Figure 3-1. For a Source DUT with eCBUS-D, make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode and eCBUS TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

  - eCBUS PATTERN: PRBS

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the TMDS Data waveform, measure the differential swing voltage (VDF_SWING_DF_TMDS_DATA).

  - Obtain differential waveform using the scope’s math function.

  - Obtain the mean values of the voltage histograms for the bits 4 and 5 of the high and low periods.

  - Obtain the differential swing voltage (VDF_SWING_DF_TMDS_DATA) by subtracting the two mean values  in step b.

If 350 mV <= VDF_SWING_DF_TMDS_DATA <=1000 mV, then PASS; else FAIL.

Differential Output Swing Voltage of Differential MHL Clock and eCBUS-D FWD Data: VDF_SWING_DF_MHL_CLK, VDF_SWING_DF_eCBUS_FWD

This test confirms that the differential swing voltages of the differential MHL Clock and eCBUS-D FWD Data are within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following eCBUS TX BIST mode and AVLINK TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low every cycle

  - eCBUS DURATION: 32.0 seconds

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the Differential MHL Clock and eCBUS-D FWD Data waveform, measure the differential swing voltage (VDF_SWING_DF_MHL_CLK, VDF_SWING_DF_eCBUS_FWD).

  - Obtain differential waveform using the scope’s math function.

  - VDF_SWING_DF_MHL_CLK:

    - Obtain the mean values of the voltage histograms for the middle one TBIT_DF_eCBUS_FWD periods of the Differential MHL Clock Front Porch and Back Porch.

    - Obtain the differential swing voltage (VDF_SWING_DF_MHL_CLK) by subtracting the two mean values  in the previous step.

  - VDF_SWING_DF_eCBUS_FWD:

    - Obtain the mean values of the voltage histograms for the middle three TBIT_DF_eCBUS_FWD periods of the 5 High and 5 Low bits of eCBUS-D FWD Data.

    - Obtain the differential swing voltage (VDF_SWING_DF_eCBUS_FWD) by subtracting the two mean values  in the previous step.

If 400mV <= VDF_SWING_DF_MHL_CLK  <= 700mV AND 400mV <= VDF_SWING_DF_eCBUS_FWD <= 700mV, then PASS; else FAIL.

Single-Ended High Level Output Voltage of Single-Ended MHL CLK and eCBUS-S FWD Data: VSE_HIGH_SE_MHL_CLK, VSE_HIGH_SE_eCBUS_FWD

This test confirms that the single-ended high level voltages of the single-ended MHL CLK and eCBUS-S FWD Data are within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following eCBUS TX BIST mode and AVLINK TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform and measure the mean value (VSE_HIGH_SE_MHL_CLK) of the voltage histogram for middle one TBIT_SE_eCBUS_FWD period of Single-Ended MHL Clock Back Porch period.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform and measure the mean value (VSE_HIGH_SE_eCBUS_FWD) of the voltage histogram for middle half TBIT_SE_eCBUS_FWD period of the high eCBUS-S FWD Data bits.

If 220 mV <= VSE_HIGH_SE_MHL_CLK <= 400 mV AND 220 mV <= VSE_HIGH_SE_eCBUS_FWD <= 400 mV, then PASS; else FAIL.

Single-Ended Low Level Output Voltage of Single-Ended MHL CLK and eCBUS-S FWD Data: VSE_LOW_SE_MHL_CLK, VSE_LOW_SE_eCBUS_FWD

This test confirms that the single-ended low level voltages of the single-ended MHL CLK and eCBUS-S FWD Data are within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following eCBUS TX BIST mode and AVLINK TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform and measure the mean value (VSE_LOW_SE_MHL_CLK) of the voltage histogram for middle one TBIT_SE_eCBUS_FWD period of Single-Ended MHL Clock Front Porch period.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform and measure the mean value (VSE_LOW_SE_eCBUS_FWD) of the voltage histogram for middle half TBIT_SE_eCBUS_FWD period of the low eCBUS-S FWD Data bits.

If -30 mV <= VSE_LOW_SE_MHL_CLK <= 30 mV AND -30 mV <= VSE_LOW_SE_eCBUS_FWD <= 30 mV, then PASS; else FAIL.

Single-Ended Output Swing Voltage of Single-Ended MHL CLK and eCBUS-S FWD Data: VSE_SWING_SE_MHL_CLK, VSE_SWING_SE_eCBUS_FWD

This test confirms that the single-ended output swing voltages of the Single-Ended MHL CLK and eCBUS-S FWD Data are within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Obtain VSE_SWING_SE_MHL_CLK by subtracting VSE_LOW_SE_MHL_CLK from VSE_HIGH_SE_MHL_CLK obtained in Sections 3.7.2.7 and 3.7.2.8.

Obtain VSE_SWING_SE_eCBUS_FWD by subtracting VSE_LOW_SE_eCBUS_FWD from VSE_HIGH_SE_eCBUS_FWD obtained in Sections 3.7.2.7 and 3.7.2.8.

If 250 mV <= VSE_SWING_SE_MHL_CLK <= 370 mV AND 250 mV <= VSE_SWING_SE_eCBUS_FWD <= 370 mV, then PASS; else FAIL.

Differential MHL Clock Frequency: FDF_MHL_CLK

This test confirms that the Differential MHL Clock Frequency is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the Differential MHL Clock waveform, obtain Differential MHL Clock Frequency (FDF_MHL_CLK) by measuring the mean frequency over 500 microsecondsec.

If 149.925 MHz <= FDF_MHL_CLK <= 150.075 MHz, then PASS; else FAIL.

Differential MHL Clock Front Porch: TCFP_DF_MHL_CLK

This test confirms that the Differential MHL Clock Front Porch time is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the Differential MHL Clock and eCBUS-D FWD Data waveform, obtain Differential MHL Clock Front Porch time (TCFP_DF_MHL_CLK) by measuring the time between the falling edge of the dummy bit of eCBUS-D FWD Data and the subsequent rising edge of Differential MHL Clock.

If 2.4*TBIT_DF_eCBUS_FWD <= TCFP_DF_MHL_CLK <= 2.6*TBIT_DF_eCBUS_FWD, then PASS; else FAIL.

Differential MHL Clock Back Porch: TCBP_DF_MHL_CLK

This test confirms that the Differential MHL Clock Back Porch time is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the Differential MHL Clock and eCBUS-D FWD Data waveform, obtain Differential MHL Clock Back Porch time (TCBP_DF_MHL_CLK) by measuring the time between the rising edge of Differential MHL Clock and the falling edge of the subsequent first bit of eCBUS-D FWD Data.

If 1.4*TBIT_DF_eCBUS_FWD <= TCBP_DF_MHL_CLK <= 1.6*TBIT_DF_eCBUS_FWD, then PASS; else FAIL.

Rise Time of Differential TMDS Data: TR_DF_TMDS_DATA

This test confirms that the rise time of Differential TMDS Data is within the specified limits.

- For a Source DUT with eCBUS-S, make connections shown in Figure 3-1. For a Source DUT with eCBUS-D, make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the Differential TMDS Data waveform, obtain 20% - 80% Rise Time of Differential TMDS Data (TR_DF_TMDS_DATA).

If 45 ps <= TR_DF_TMDS_DATA, then PASS; else FAIL.

Fall Time of Differential TMDS Data: TF_DF_TMDS_DATA

This test confirms that the fall time of Differential TMDS Data is within the specified limits.

- For a Source DUT with eCBUS-S, make connections shown in Figure 3-1. For a Source DUT with eCBUS-D, make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the Differential TMDS Data waveform, obtain 20% - 80% Fall Time of Differential TMDS Data (TF_DF_TMDS_DATA).

If 45 ps <= TF_DF_TMDS_DATA, then PASS; else FAIL.

Rise Time of Differential MHL Clock and eCBUS-D FWD Data: TR_DF_MHL_CLK, TR_DF_eCBUS_FWD

This test confirms that the rise time of Differential MHL Clock and eCBUS-D FWD Data are within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: Fixed 10 pattern, 0011100111 every cycle

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the Differential MHL Clock and eCBUS-D Data waveform, obtain 20% - 80% Rise Time of Differential MHL Clock (TR_DF_MHL_CLK).

From the Differential MHL Clock and eCBUS-D Data waveform, obtain 20% - 80% Rise Time of eCBUS-D FWD Data (TR_DF_eCBUS_FWD) at the first high bit rising edge after the MHL Clock rising edge.

If 100 ps <= TR_DF_MHL_CLK <= 250 ps AND 100 ps <= TR_DF_eCBUS_FWD <= 250 ps, then PASS; else FAIL.

Fall Time of Differential MHL Clock and eCBUS-D FWD Data: TF_DF_MHL_CLK, TF_DF_eCBUS_FWD

This test confirms that the fall time of Differential MHL Clock and eCBUS-D FWD Data are within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 5 High/5 Low every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: Fixed 10 pattern, 0011100111

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the Differential MHL Clock and eCBUS-D Data waveform, obtain 20% - 80% Fall Time at the falling edge of the dummy bit (TF_DF_MHL_CLK).

From the Differential MHL Clock and eCBUS-D Data waveform, obtain 20% - 80% Fall Time of eCBUS-D FWD Data (TF_DF_eCBUS_FWD) at the first low bit falling edge after the MHL Clock rising edge.

If 100 ps <= TF_DF_MHL_CLK <= 250 ps AND 100 ps <= TF_DF_eCBUS_FWD <= 250 ps, then PASS; else FAIL.

Peak-Peak Amplitude of Differential TMDS Data: VPP_TP1_DF_TMDS_DATA

This test confirms that the Peak-Peak Amplitude of Differential TMDS Data is within the specified limits.

- For a Source DUT with eCBUS-S, make connections shown in Figure 3-1. For a Source DUT with eCBUS-D, make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

From the Differential TMDS Data waveform of 500s long, measure the Peak-Peak Amplitude of Differential TMDS Data (VPP_TP1_DF_TMDS_DATA).

If VPP_TP1_DF_TMDS_DATA <= 1560 mV, then PASS; else FAIL.

Peak-Peak Amplitude of eCBUS-D FWD Data: VPP_TP1_DF_eCBUS_FWD

This test confirms that the Peak-Peak Amplitude of eCBUS-D FWD Data is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-3.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the eCBUS-D FWD Data waveform of 500s long, measure the Peak-Peak Amplitude of eCBUS-D FWD Data (VPP_TP1_DF_eCBUS_FWD).

If VPP_TP1_DF_eCBUS_FWD <= 910 mV, then PASS; else FAIL.

Differential MHL Clock Jitter at TP2: TCLOCK_JITTER_TP2_DF_MHL_CLK

This test confirms that Differential MHL Clock Jitter at TP2 is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-4.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the Differential MHL Clock waveform of 500s long, measure the Differential MHL Clock Jitter at TP2 (TCLOCK_JITTER_TP2_DF_MHL_CLK).

  - Apply the Worst Case Cable Emulator to the Differential MHL Clock waveform.

  - Apply MHL 2.1 Reference Cable Equalizer after the Worst Case Cable Emulator.

  - Apply MHL Clock Recovery Unit after MHL 2.1 Reference Cable Equalizer.

  - Measure the Differential MHL Clock Jitter (TCLOCK_JITTER_TP2_DF_MHL_CLK) using the recoverd clock in step c.

If TCLOCK_JITTER_TP2_DF_MHL_CLK <= 67 ps, then PASS; else FAIL.

Single-Ended MHL Clock Frequency: FSE_MHL_CLK

This test confirms that the Single-Ended MHL Clock Frequency is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform to remove the noise on the waveform.

From the Differential MHL Clock waveform, obtain Single-Ended MHL Clock Frequency (FSE_MHL_CLK) by measuring the mean frequency over 500 microsecondsec.

If 74.9625 MHz <= FSE_MHL_CLK <= 75.0375 MHz, then PASS; else FAIL.

Single-Ended MHL Clock Front Porch: TCFP_SE_MHL_CLK

This test confirms that the Single-Ended MHL Clock Front Porch time is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform to remove the noise on the waveform.

From the Single-Ended MHL Clock and eCBUS-S FWD Data waveform, obtain Single-Ended MHL Clock Front Porch time (TCFP_SE_MHL_CLK) by measuring the time between the falling edge of ‘1’ bit of eCBUS-S FWD Data and the subsequent rising edge of Single-Ended MHL Clock.

If 5000 ps <= TCFP_SE_MHL_CLK <= 5666.67 ps, then PASS; else FAIL.

Single-Ended MHL Clock Back Porch: TCBP_SE_MHL_CLK

This test confirms that the Single-Ended MHL Clock Back Porch time is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform to remove the noise on the waveform.

From the Single-Ended MHL Clock and eCBUS-S FWD Data waveform, obtain Single-Ended MHL Clock Back Porch time (TCBP_SE_MHL_CLK) by measuring the time between the the rising edge of Single-Ended MHL Clock and the falling edge of ‘0’ bit of eCBUS-S FWD Data.

If 5000 ps <= TCBP_SE_MHL_CLK <= 5666.67 ps, then PASS; else FAIL.

Rise Time of Single-Ended MHL Clock: TR_SE_MHL_CLK

This test confirms that the rise time of Single-Ended MHL Clock is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform to remove the noise on the waveform.

From the Single-Ended Clock waveform, obtain 20% - 80% Rise Time of Single-Ended MHL Clock (TR_SE_MHL_CLK).

If 500 ps <= TR_SE_MHL_CLK <= 1000 ps, then PASS; else FAIL.

Fall Time of Single-Ended MHL Clock and eCBUS-S FWD Data: TF_SE_MHL_CLK, TF_SE_eCBUS_FWD

This test confirms that the fall times of Single-Ended MHL Clock and eCBUS-S FWD Data are within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 10 High every cycle

  - AVLINK DATA RATE: supported lowest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: 32.0 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

Apply 1GHz low pass filter to the Single-Ended MHL Clock and eCBUS-S FWD Data waveform to remove the noise on the waveform.

From the Single-Ended MHL Clock and eCBUS-S Data waveform, obtain 20% - 80% Fall Time of eCBUS-S FWD Data (TF_SE_eCBUS_FWD) at the falling edge of eCBUS-S Data.

The Fall Time of Single-Ended MHL Clock (TF_SE_MHL_CLK) is the same as the Fall Time of eCBUS-S FWD Data (TF_SE_eCBUS_FWD).

If 500 ps <= TF_SE_MHL_CLK (TF_SE_eCBUS_FWD) <= 1000 ps, then PASS; else FAIL.

Peak-Peak Amplitude of eCBUS-S FWD Data: VPP_TP1_SE_eCBUS_FWD

This test confirms that the Peak-Peak Amplitude of eCBUS-S FWD Data is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-1.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

From the eCBUS-S FWD Data waveform of 500s long, measure the Peak-Peak Amplitude of eCBUS-S FWD Data (VPP_TP1_SE_eCBUS_FWD).

If VPP_TP1_SE_eCBUS_FWD <= 500 mV, then PASS; else FAIL.

Single-Ended MHL Clock Jitter at TP2: TCLOCK_JITTER_TP2_SE_MHL_CLK

This test confirms that Single-Ended MHL Clock Jitter at TP2 is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-2.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

From the Single-Ended MHL Clock waveform of 500s long, measure the Single-Ended MHL Clock Jitter at TP2 (TCLOCK_JITTER_TP2_SE_MHL_CLK).

  - Apply the Worst Case Cable Emulator to the Single-Ended MHL Clock waveform.

  - Apply the Low Pass Filter defined MHL 3.0 Specification Section 16.3.1.4 after the Worst Case Cable Emulator.

  - Apply MHL 2.1 Reference Cable Equalizer after the Low Pass Filter.

  - Apply MHL Clock Recovery Unit after MHL 2.1 Reference Cable Equalizer.

  - Measure the Single-Ended MHL Clock Jitter (TCLOCK_JITTER_TP2_SE_MHL_CLK) using the recoverd clock in step d.

If TCLOCK_JITTER_TP2_SE_MHL_CLK <= 150 ps, then PASS; else FAIL.

Differential TMDS Data Eye Diagram at TP2

This test confirms that the Differential TMDS Data Eye Diagram is within the specified limits.

- For a Source DUT with eCBUS-S, make connections shown in Figure 3-2. For a Source DUT with eCBUS-D, make connections shown in Figure 3-4.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: highest supported data rates among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S or eCBUS-D connection from the CBUS Sink to the scope.

From the Differential TMDS Data waveform, measure the eye diagram.

  - Capture concurrent Differential TMDS Data waveform and MHL clock waveform of 500 microseconds long.

  - Apply the Worst Case Cable Emulator to the Differential TMDS Data waveform. Skip this step if a hardware cable emulator is used for the Worst Case Cable Emulator.

  - Apply 6 Gbps Reference Cable Equalizer to the Differential TMDS Data waveform for 6 Gbps data rate and apply MHL 2.1 Reference Cable Equalizer to the Differential TMDS Data waveform for 3 Gbps and 1.5 Gbps data rates.

  - Measure the eye diagram using the MHL clock

    - If the MHL clock is differential, process the clock as described in Section3.7.2.19.

    - If the MHL clock is single-ended, process the clock as described in Section 3.7.2.26.

Perform the eye diagram test at all other supported TMDS data rates.

If the eye diagram passes eye mask test shown in Figure 16-17 in MHL 3.0 specification at all data rates, then PASS; else FAIL.

eCBUS-D FWD Data Eye Diagram at TP2

This test confirms that the eCBUS-D FWD Data Eye Diagram is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 3-4.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the scope.

From the eCBUS-D FWD Data waveform, measure the eye diagram.

  - Capture eCBUS-D FWD Data and MHL clock waveform of 500 microseconds long.

  - Apply the Worst Case Cable Emulator to the eCBUS-D FWD Data waveform. Skip this step if a hardware cable emulator is used for the Worst Case Cable Emulator.

  - Apply the MHL 2.1 Reference Cable Equalizer to the eCBUS-D FWD Data waveform.

  - Measure the eye diagram using the MHL clock. Process the clock as described in Section 3.7.2.19.

If the eye diagram passes eye mask test shown in Figure 16-18 in MHL 3.0 specification, then PASS; else FAIL.

eCBUS-S FWD Data Eye Diagram at TP2

This test confirms that the eCBUS-S FWD Data Eye Diagram is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 3-2.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: PRBS pattern with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 2400 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the scope.

From the eCBUS-S FWD Data waveform, measure the eye diagram.

  - Capture eCBUS-S FWD Data and MHL clock waveform of 500 microseconds long.

  - Apply the Worst Case Cable Emulator to the eCBUS-S FWD Data waveform. Skip this step if a hardware cable emulator is used for the Worst Case Cable Emulator.

  - Apply the Low Pass Filter defined MHL 3.0 Specification Section 16.3.1.4 after the Worst Case Cable Emulator.

  - Measure the eye diagram using the MHL clock. Process the clock as described in Section 3.7.2.26.

If the eye diagram passes eye mask test shown in Figure 16-19 in MHL 3.0 specification, then PASS; else FAIL.

Input DC Voltage Tolerance of eCBUS-D BWD Data: VIDC_DF_eCBUS_BWD

This test confirms that the Source DUT tolerates the input DC voltage levels of eCBUS-D BWD Data specified in the specification.

- Make connections shown in Figure 3-7.

Calibrate the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data.

  - Perform CBUS discovery between the DUT and the CBUS Sink.

  - Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

    - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

    - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

    - AVLINK DURATION: Indefinite

  - Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

    - eCBUS PATTERN: PRBS pattern

    - eCBUS DURATION: 32.0 seconds

  - Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

  - Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the MHL3 Signal Generator.

  - Disconnect eCBUS-D connection from the DUT and connect it to eCBUS-D BWD Calibration Path shown in Figure 3-7.

  - Adjust the MHL3 Signal Generator output by measuring it at TP1 in the eCBUS-D BWD Calibration Path

    - eCBUS-D BWD PRBS pattern and eCBUS training patterns

    - Differential swing voltage: 550mV

    - Differential DC level: VTERM – 80 mV

  - Connect the DUT eCBUS-D to the RELT board and switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 Signal Generator to the CBUS Sink.

  - Reset or restart the DUT.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: Indefinite

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Set the DUT in the following eCBUS RX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to MHL3 eCBUS-D Signal Generator.

Finish eCBUS training between the DUT and the MHL3 eCBUS-D Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 eCBUS-D Signal Generator to the CBUS Sink.

Check the eCBUS RX BIST Error Count.

Calibrate the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data with the following setting.

  - Differential swing voltage: 550mV.

  - Differential DC level is VTERM: – 375 mV.

Repeat steps 3 to 11.

If the Error Count is zero in steps 11 and 13, then PASS; else FAIL.

Differential Input Swing Voltage Tolerance of eCBUS-D BWD Data: VIDF_SWING_DF_eCBUS_BWD

This test confirms that the Source DUT tolerates the input swing voltage ranges of eCBUS-D BWD Data specified in the specification.

- Make connections shown in Figure 3-7.

Calibrate the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data.

  - Perform CBUS discovery between the DUT and the CBUS Sink.

  - Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

    - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

    - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

    - AVLINK DURATION: Indefinite

  - Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

    - eCBUS PATTERN: PRBS pattern

    - eCBUS DURATION: 32.0 seconds

  - Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

  - Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the MHL3 Signal Generator.

  - Disconnect eCBUS-D connection from the DUT and connect it to eCBUS-D BWD Calibration Path shown in Figure 3-7.

  - Adjust the MHL3 Signal Generator output by measuring it at TP1 in the eCBUS-D BWD Calibration Path

    - eCBUS-D BWD PRBS pattern and eCBUS training patterns

    - Differential swing voltage: 700mV

    - Differential DC level: VTERM – 228 mV

  - Connect the DUT eCBUS-D to the RELT board and switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 Signal Generator to the CBUS Sink.

  - Reset or restart the DUT.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: Indefinite

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Set the DUT in the following eCBUS RX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to MHL3 eCBUS-D Signal Generator.

Finish eCBUS training between the DUT and the MHL3 eCBUS-D Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 eCBUS-D Signal Generator to the CBUS Sink.

Check the eCBUS RX BIST Error Count.

Calibrate the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data with the following setting.

  - Differential swing voltage: 150 mV.

  - Differential DC level: VTERM – 228 mV.

Repeat steps 3 to 11.

If the Error Count is zero in steps 11 and 13, then PASS; else FAIL.

Input DC Voltage Tolerance of eCBUS-S BWD Data: VIDC_SE_eCBUS_BWD

This test confirms that the Source DUT tolerates the input DC voltage levels of eCBUS-S BWD Data specified in the specification.

- Make connections shown in Figure 3-5.

Calibrate the MHL3 eCBUS-S Signal Generator for eCBUS-S BWD Data.

  - Perform CBUS discovery between the DUT and the CBUS Sink.

  - Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

    - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

    - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

    - AVLINK DURATION: Indefinite

  - Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

    - eCBUS PATTERN: PRBS pattern

    - eCBUS DURATION: 32.0 seconds

  - Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

  - Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the MHL3 Signal Generator.

  - Disconnect eCBUS-S connection from the DUT and connect it to eCBUS-S BWD Calibration Path shown in Figure 3-5.

  - Adjust the MHL3 Signal Generator output by measuring it at TP1 in the eCBUS-S BWD Calibration Path

  - eCBUS-S BWD PRBS pattern and eCBUS training patterns

    - Single-ended swing voltage: 270mV

    - Single-ended DC level: 215 mV

    - Connect the DUT eCBUS-S to the RELT board and switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the MHL3 Signal Generator to the CBUS Sink.

  - Reset or restart the DUT.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: Indefinite

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Set the DUT in the following eCBUS RX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to MHL3 eCBUS-S Signal Generator.

Finish eCBUS training between the DUT and the MHL3 eCBUS-S Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the MHL3 eCBUS-S Signal Generator to the CBUS Sink.

Check the eCBUS RX BIST Error Count.

Calibrate the MHL3 eCBUS-S Signal Generator for eCBUS-S BWD Data with the following setting.

  - Single-ended swing voltage: 270mV.

  - Single-ended input DC level: 95 mV.

Repeat steps 3 to 11.

If the Error Count is zero in steps 11 and 13, then PASS; else FAIL.

Single-Ended Input Swing Voltage Tolerance of eCBUS-S BWD Data: VISE_SWING_SE_eCBUS_BWD

This test confirms that the Source DUT tolerates the input swing voltage ranges of eCBUS-S BWD Data specified in the specification.

- Make connections shown in Figure 3-5.

Calibrate the MHL3 eCBUS-S Signal Generator for eCBUS-S BWD Data.

  - Perform CBUS discovery between the DUT and the CBUS Sink.

  - Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

    - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

    - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

    - AVLINK DURATION: Indefinite

  - Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

    - eCBUS PATTERN: PRBS pattern

    - eCBUS DURATION: 32.0 seconds

  - Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

  - Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the MHL3 Signal Generator.

  - Disconnect eCBUS-S connection from the DUT and connect it to eCBUS-S BWD Calibration Path shown in Figure 3-5.

  - Adjust the MHL3 Signal Generator output by measuring it at TP1 in the eCBUS-S BWD Calibration Path

    - eCBUS-S BWD PRBS pattern and eCBUS training patterns

    - Single-ended swing voltage: 370mV

    - Single-ended DC level: 155 mV

  - Connect the DUT eCBUS-S to the RELT board and switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the MHL3 Signal Generator to the CBUS Sink.

  - Reset or restart the DUT.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: Indefinite

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Set the DUT in the following eCBUS RX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to MHL3 eCBUS-S Signal Generator.

Finish eCBUS training between the DUT and the MHL3 eCBUS-S Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the MHL3 eCBUS-S Signal Generator to the CBUS Sink.

Check the eCBUS RX BIST Error Count.

Calibrate the MHL3 eCBUS-S Signal Generator for eCBUS-S BWD Data with the following setting.

  - Single-ended swing voltage: 170 mV

  - Single-ended DC level: 155 mV.

Repeat steps 3 to 11.

If the Error Count is zero in steps 11 and 13, then PASS; else FAIL.

Intra-Pair Skew Tolerance of eCBUS-D BWD Data:TSKEW_TP1_DF_eCBUS_BWD

This test confirms that the Source DUT tolerates the intra-pair skew of input eCBUS-D BWD Data specified in the specification.

- Make connections shown in Figure 3-7.

Calibrate the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data.

  - Perform CBUS discovery between the DUT and the CBUS Sink.

  - Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

    - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

    - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

    - AVLINK DURATION: Indefinite

  - Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

    - eCBUS PATTERN: PRBS pattern

    - eCBUS DURATION: 32.0 seconds

  - Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

  - Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the MHL3 Signal Generator.

  - Disconnect eCBUS-D connection from the DUT and connect it to eCBUS-D BWD Calibration Path shown in Figure 3-7.

  - Adjust the MHL3 Signal Generator output by measuring it at TP1 in the eCBUS-D BWD Calibration Path

    - eCBUS-D BWD PRBS pattern and eCBUS training patterns

    - Differential swing voltage: 550mV

    - Differential DC level: VTERM – 228 mV

    - Intra-pair skew: positive 50ps (the positive signal leads the negative signal)

  - Connect the DUT eCBUS-D to the RELT board and switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 Signal Generator to the CBUS Sink.

  - Reset or restart the DUT.

Make connections shown in Figure 3-8.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: Indefinite

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Set the DUT in the following eCBUS RX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to MHL3 eCBUS-D Signal Generator.

Finish eCBUS training between the DUT and the MHL3 eCBUS-D Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 eCBUS-D Signal Generator to the CBUS Sink.

Check the eCBUS RX BIST Error Count.

Make connections shown in Figure 3-7.

Calibrate the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data with the following setting.

  - Differential swing voltage: 550mV

  - Differential DC level: VTERM – 228 mV

  - Intra-pair skew: negative 50ps (the negative signal leads the positive signal)

Make connections shown in Figure 3-8.

Repeat steps 4 to 12.

Repeat steps 1 to 16 with swapped connection of the eCBUS-D positive and negative signals between the MHL3 eCBUS-D Signal Generator and the MHL3 cable emulator.

If the Error Count is zero in steps 12, 16 and 17, then PASS; else FAIL.

Jitter Tolerance of eCBUS-D BWD Data

This test confirms that the Source DUT tolerates the maximum jitter of input eCBUS-D BWD Data specified in the specification.

- Make connections shown in Figure 3-7.

Calibrate the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data.

  - Perform CBUS discovery between the DUT and the CBUS Sink.

  - Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

    - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

    - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

    - AVLINK DURATION: Indefinite

  - Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

    - eCBUS PATTERN: PRBS pattern

    - eCBUS DURATION: 32.0 seconds

  - Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

  - Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to the MHL3 Signal Generator.

  - Disconnect eCBUS-D connection from the DUT and connect it to eCBUS-D BWD Calibration Path shown in Figure 3-7.

  - Adjust the MHL3 Signal Generator output by measuring it at TP1 in the eCBUS-D BWD Calibration Path

    - eCBUS-D BWD PRBS pattern and eCBUS training patterns

    - Differential swing voltage: 400mV

    - Differential DC level: VTERM – 228 mV

    - Differential rise/fall times (20% - 80%): 250ps

  - Reset or restart the DUT.

Make connections shown in Figure 3-8 and repeat steps a - f.

  - Calibrate 500kHz jitter from the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data until a marginally pass eCBUS-D BWD Data eye is obtained at TP1. If necessary, increase the swing amplitude of the MHL3 eCBUS-D Signal Generator output and adjust the jiiter amount until a marginally pass eCBUS-D BWD Data eye is obtained at TP1.

  - Connect the DUT eCBUS-D to the RELT board and switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 Signal Generator to the CBUS Sink.

  - Reset or restart the DUT.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: Indefinite

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Set the DUT in the following eCBUS RX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to MHL3 eCBUS-D Signal Generator.

Finish eCBUS training between the DUT and the MHL3 eCBUS-D Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 eCBUS-D Signal Generator to the CBUS Sink.

Check the eCBUS RX BIST Error Count.

Go to steps 1 and 2, and calibrate 1MHz jitter from the MHL3 eCBUS-D Signal Generator for eCBUS-D BWD Data until a marginally pass eCBUS-D BWD Data eye is obtained at TP1. If necessary, increase the swing amplitude of the MHL3 eCBUS-D Signal Generator output and adjust the jiiter amount until a marginally pass eCBUS-D BWD Data eye is obtained at TP1.

Repeat steps 3 to 11.

Repeat steps 1 to 13 with swapped connection of the eCBUS-D positive and negative signals between the MHL3 eCBUS-D Signal Generator and the MHL3 cable emulator.

If the Error Count is zero in steps 11, 13 and 14, then PASS; else FAIL.

Jitter Tolerance of eCBUS-S BWD Data

This test confirms that the Source DUT tolerates the maximum jitter of input eCBUS-S BWD Data specified in the specification.

- Make connections shown in Figure 3-5.

Calibrate the MHL3 eCBUS-S Signal Generator for eCBUS-S BWD Data.

  - Perform CBUS discovery between the DUT and the CBUS Sink.

  - Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

    - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

    - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

    - AVLINK DURATION: Indefinite

  - Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

    - eCBUS PATTERN: PRBS pattern

    - eCBUS DURATION: 32.0 seconds

  - Trigger the BIST and finish eCBUS training between the DUT and the CBUS Sink.

  - Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to the MHL3 Signal Generator.

  - Disconnect eCBUS-S connection from the DUT and connect it to eCBUS-S BWD Calibration Path shown in Figure 3-5.

  - Adjust the MHL3 Signal Generator output by measuring it at TP1 in the eCBUS-S BWD Calibration Path

    - eCBUS-S BWD PRBS pattern and eCBUS training patterns

    - Single-ended swing voltage: 250mV

    - Differential DC level: 155 mV

    - Differential rise/fall times (20% - 80%): 1000ps

  - Reset or restart the DUT.

Make connections shown in Figure 3-6 and repeat steps a - f.

  - Calibrate 500kHz jitter from the MHL3 eCBUS-S Signal Generator for eCBUS-S BWD Data until a marginally pass eCBUS-S BWD Data eye is obtained at TP1. If necessary, increase the swing amplitude of the MHL3 eCBUS-S Signal Generator output and adjust the jiiter amount until a marginally pass eCBUS-S BWD Data eye is obtained at TP1.

  - Connect the DUT eCBUS-D to the RELT board and switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the MHL3 Signal Generator to the CBUS Sink.

  - Reset or restart the DUT.

Perform CBUS discovery between the DUT and the CBUS Sink.

Set the DUT in the following AVLINK TX BIST mode using the CBUS Sink.

  - AVLINK PATTERN: Fixed 10 pattern without randomizing, 1010101010 every cycle

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK DURATION: Indefinite

Set the DUT in the following eCBUS TX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Set the DUT in the following eCBUS RX BIST mode using the CBUS Sink.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to MHL3 eCBUS-S Signal Generator.

Finish eCBUS training between the DUT and the MHL3 eCBUS-S Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the MHL3 eCBUS-S Signal Generator to the CBUS Sink.

Check the eCBUS RX BIST Error Count.

Go to steps 1 and 2, and calibrate 1MHz jitter from the MHL3 eCBUS-S Signal Generator for eCBUS-S BWD Data until a marginally pass eCBUS-S BWD Data eye is obtained at TP1. If necessary, increase the swing amplitude of the MHL3 eCBUS-S Signal Generator output and adjust the jitter amount until a marginally pass eCBUS-D BWD Data eye is obtained at TP1.

Repeat steps 3 to 11.

If the Error Count is zero in steps 11 and 13, then PASS; else FAIL.

Differential Impedance of TMDS: ZD_THRU_TMDS_SRC, ZDT_ TMDS_SRC

This test confirms that the differential impedance of the source device is within the range allowed by the specification.

Before testing, compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables).

Figure 3-11. Set reference point for differential mode

- Connect M3-S TPA SRC-P or M3-S TPA DirSRC-R for eCBUS-S or M3-D TPA SRC-P or M3-D TPA DirSRC-R for eCBUS-D to the TDR oscilloscope through a DC Block.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%).

Record the time at the peak of the change point in Figure 3-11. This is the start point of the Source DUT input connector (TIN).

Calculate TTERM = TIN + CDF_SRC_DF_TERM_DISTANCE. TTERM is the source termination point for differential signals. CDF_SRC_DF_TERM_DISTANCE is obtained from CDF of the Source DUT.

Connect the equipment as shown in Figure 3-9 for eCBUS-S or in Figure 3-10 for eCBUS-D.

Turn on the DUT and enable MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Sink board.

Set the DUT BIST Impedance_Mode to AV LINK TX HIGH using the CBUS Sink.

Trigger the BIST

Measure the differential impedance along the MHL differential signal path from TIN to TTERM + 1 ns.

  - The differential impedance from TIN to TTERM is ZDTHROUGH_HIGH and the differential impedance from TTERM to TTERM + 1ns is ZDT_HIGH.

Set the DUT BIST Impedance_Mode to AV LINK TX LOW using the CBUS Sink.

Trigger the BIST

Measure the differential impedance along the MHL differential signal path from TIN to TTERM + 1 ns.

  - The differential impedance from TIN to TTERM is ZDTHROUGH_LOW and the differential impedance from TTERM to TTERM + 1ns is ZDT_LOW.

Calculate ZDTHROUGH and ZDT

  - ZDTHROUGH= (ZDTHROUGH_HIGH+ ZDTHROUGH_LOW)/2

  - ZDT= (ZDT_HIGH+ ZDT_LOW)/2

Evaluate ZDTHROUGH and ZDT

  - If (70 ohms <= ZDTHROUGH <= 115 ohms) AND (80 ohms <= ZDT <= 120 ohms), then PASS. Else if (60 ohms <= ZDTHROUGH < 70 ohms) OR (115 ohms < ZDTHROUGH <= 125 ohms), the impedance is in the excursion range. If the excursion occurs single time AND the duration of the single excursion is less than 600ps AND (80 ohms <= ZDT <= 120 ohms), then PASS.

  - For all other cases, FAIL.

If preceding steps pass, then PASS, else FAIL.

Differential Impedance of eCBUS-D: ZD_THRU_eCBUS_SRC, ZDT_eCBUS_SRC

This test confirms that the differential impedance of the source device is within the range allowed by the specification. Before testing, compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables).

Figure 3-12. Set reference point for differential mode

- Connect M3-D TPA SRC-P or M3-D TPA DirSRC-R to the TDR oscilloscope through a DC Block.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%).

Record the time at the peak of the change point in Figure 3-12. This is the start point of the Source DUT input connector (TIN).

Calculate TTERM = TIN + CDF_SRC_DF_TERM_DISTANCE. TTERM is the source termination point for differential signals. CDF_SRC_DF_TERM_DISTANCE is obtained from CDF of the Source DUT.

Connect the equipment as shown in Figure 3-10.

Turn on the DUT and enable MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Sink board.

Set the DUT BIST Impedance_Mode to eCBUS-D TX HIGH and eCBUS-D Rx using the CBUS Sink.

Trigger the BIST

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to TDR Scope or VNA-base TDR.

Measure the differential impedance along the MHL differential signal path from TIN to TTERM + 1 ns.

  - The differential impedance from TIN to TTERM is ZDTHROUGH_HIGH and the differential impedance from TTERM to TTERM + 1ns is ZDT_HIGH.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from TDR Scope or VNA-base TDR to the CBUS Sink.

Set the DUT BIST Impedance_Mode to eCBUS-D TX LOW and eCBUS-D Rx using the CBUS Sink.

Trigger the BIST

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to TDR Scope or VNA-base TDR.

Measure the differential impedance along the MHL differential signal path from TIN to TTERM + 1 ns.

  - The differential impedance from TIN to TTERM is ZDTHROUGH_LOW and the differential impedance from TTERM to TTERM + 1ns is ZDT_LOW.

Calculate ZDTHROUGH and ZDT.

  - ZDTHROUGH= (ZDTHROUGH_HIGH+ ZDTHROUGH_LOW)/2

  - ZDT= (ZDT_HIGH+ ZDT_LOW)/2

Evaluate ZDTHROUGH and ZDT.

  - If (75 ohms <= ZDTHROUGH <= 115 ohms) AND (80 ohms <= ZDT <= 120 ohms), then PASS. Else if (70 ohms <= ZDTHROUGH < 75 ohms) OR (115 ohms < ZDTHROUGH <= 125 ohms), the impedance is in the excursion range. If the excursion occurs single time AND the duration of the single excursion is less than 600ps AND (80 ohms <= ZDT <= 120 ohms), then PASS.

  - For all other cases, FAIL.

If preceding steps pass, then PASS, else FAIL.

Single-Ended Impedance of eCBUS-S: ZS_THRU_eCBUS_SRC, ZST_eCBUS_SRC

This test confirms that the single-ended impedance of the source device is within the range allowed by the specification.

Figure 3-13. Set reference point for single-ended signal

- Connect M3-S TPA SRC-P or M3-S TPA DirSRC-R to the TDR oscilloscope through a DC block.

Set the effective rise time of the single-ended TDR pulse to 500 ps (20 – 80%).

Record the time at the peak of change point in Figure 3-13. This is the start point of the Source DUT input connector (TIN).

Calculate TTERM = TIN + CDF_SRC_SE_TERM_DISTANCE. TTERM is the sink termination point for Single-ended signals. CDF_SRC_SE_TERM_DISTANCE is obtained from CDF of the Source DUT.

Connect the equipment as shown in Figure 3-9.

Turn on the DUT and enable the MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Sink board.

Set the DUT BIST Impedance_Mode to eCBUS-S TX HIGH and eCBUS-S Rx using the CBUS Sink.

Trigger the BIST

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to TDR Scope or VNA-base TDR.

Measure the single-ended impedance along the MHL single-ended signal path from TIN to TTERM + 3 ns.

  - The single-ended impedance from TIN to TTERM is ZSTHROUGH_HIGH and the single-ended impedance from TTERM to TTERM + 3ns is ZST_HIGH.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from TDR Scope or VNA-base TDR to the CBUS Sink.

Set the DUT BIST Impedance_Mode to eCBUS-S TX LOW and eCBUS-S Rx using the CBUS Sink.

Trigger the BIST

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Sink to TDR Scope or VNA-base TDR.

Measure the single-ended impedance along the MHL single-ended signal path from TIN to TTERM + 3 ns.

  - The single-ended impedance from TIN to TTERM is ZSTHROUGH_LOW and the single-ended impedance from TTERM to TTERM + 3ns is ZST_LOW.

Calculate ZDTHROUGH and ZDT.

  - ZSTHROUGH= (ZSTHROUGH_HIGH+ ZSTHROUGH_LOW)/2

  - ZST= (ZST_HIGH+ ZST_LOW)/2

Evaluate ZSTHROUGH and ZST.

  - If (35 ohms <= ZSTHROUGH <= 57.5 ohms) AND (40 ohms <= ZST <= 60 ohms), then PASS. Else if (30 ohms <= ZSTHROUGH < 35 ohms) OR (57.5 ohms < ZSTHROUGH <= 62.5 ohms), the impedance is in the excursion range. If the excursion occurs single time AND the duration of the single excursion is less than 1000ps AND (40 ohms <= ZST <= 60 ohms), then PASS.

  - For all other cases, FAIL.

If preceding steps pass, then PASS, else FAIL.

## MHL 3 System Tests

This section defines tests unique to MHL 3 devices connected in eCBUS-S or eCBUS-D mode, using AVLink or CBUS to transfer data in protocols not supported by MHL 1 / MHL 2 devices.

The equipment setups shown in Section 2.3.2 are used throughout Section 3.8.

All MHL 3 devices have a legacy compatibility mode test requirement.  Refer to MHL CTS 2.1 test procedures for the specific test steps for legacy tests.  New tests for MHL 3 are defined in Section 3.8.  The general Required Methodology for all Source DUT tests with normative references to CTS 2.1 are tested in oCBUS mode, in MHL 3 mode;  eCBUS-S and, if supported by the DUT, eCBUS-D. Test results for each mode sequence will be recorded as TESTxxxx- S for eCBUS-S mode, and TESTxxxx-D for eCBUS-D mode (if supported).  If eCBUS-D is not supported, then the result for this test sequence will be PASS (SKIP).  The general Required Methodology are defined below:

General Required Methodology

- Connect DUT to test MHL 3 Source test equipment using MHL 3 cable in eCBUS-S mode.  Execute tests listed in Section 3.8.

Execute TESTxxxx and record results as PASS or FAIL for TESTxxxx-S.

Connect DUT to test MHL 3 Source test equipment using MHL 3 cable in eCBUS-D mode if supported.  Repeat execution of tests in Section 3.8.

If eCBUS-D mode is not supported, result is PASS (SKIP).  If eCBUS-D mode is supported,  execute TESTxxxx and record results as PASS or FAIL for TESTxxxx-D.

Connect DUT to test MHL 2 Source test equipment using MHL 2 cable in oCBUS mode.  Execute tests in section 3.2 of MHL1/ MHL2 CTS.

Execute TESTxxxx and record results as PASS or FAIL for TESTxxxx.

When required,tester must follow proper WRITE_BURST protocol including but not limit to having correct BURST_IDs, WRT_REQ, GRT_WRT handshaking, and their timings in both MHL3 and MHL1/ MHL2 tests

If  all test sequences pass, overall test result is PASS, else FAIL.

Table 3-7. AVLINK Test Sequence

| Test ID | MHL3 Mode eCBUS-S | MHL3 Mode eCBUS-D |
| --- | --- | --- |
| 3.8.1.1 | X | If DUT Supports |
| 3.8.2.1 | X | If DUT Supports |
| 3.8.2.2 | X | If DUT Supports |
| 3.8.2.3 | X | If DUT Supports |
| 3.8.3.1 | X | If DUT Supports |
| 3.8.3.2 | X | If DUT Supports |
| 3.8.3.3 | X | If DUT Supports |
| 3.8.3.4 | X | If DUT Supports |
| 3.8.3.5 | X | If DUT Supports |
| 3.8.3.6 | X | If DUT Supports |
| 3.8.3.7 | X | If DUT Supports |
| 3.8.3.8 | X | If DUT Supports |
| 3.8.3.9 | X | If DUT Supports |
| 3.8.3.10 | X | If DUT Supports |
| 3.8.3.11 | X | If DUT Supports |
| 3.8.4.1 | X | If DUT Supports |
| 3.8.5.1 | X | If DUT Supports |
| 3.8.5.2 | X | If DUT Supports |
| 3.8.6.1 | X | If DUT Supports |
| 3.8.7.1 | X | If DUT Supports |
| 3.8.7.2 | X | If DUT Supports |
| 3.8.7.3 | X | If DUT Supports |
| 3.8.7.4 | X | If DUT Supports |
| 3.8.7.5 | X | If DUT Supports |
| 3.8.8.1 | X | If DUT Supports |
| 3.8.8.2 | X | If DUT Supports |
| 3.8.9.1 | X | If DUT Supports |

TX BIST

AVLink TX BIST Test

Confirm that the Source DUT supports required TX BIST and check functionality.

- Connect DUT to MHL AVLINK Analyzer and initiate discovery.

Sink Tester to issue BIST_SETUP Command with the following values:

  - AV Link_DATA_RATE: Use the highest AV Link data rate supported by Source DUT.

  - AV Link Pattern: Fixed8.

  - AV Link_VIDEO MODE:  using the highest video mode supported by Source DUT that is indicated in the CDF

  - AV Link Duration = 0x00  (duration control by Tester)

  - AV Link Fixed H  8’b10000010  (for bitwire inverse for Fixed10 pattern)

  - AV Link Fixed L:

    - 8’b11110101  (for Fixed10 pattern)

    - 8'b00000000 (for Fixed8 pattern)

  - AV Link Randomizer:

    - 0x01 (for Fixed8 pattern)

    - 0x00 (otherwise)

  - All other parameters are programed to 0x00 for this command (offset: 0x03 to 0x07, and 0x0F)

Fail if BIST_READY command is not received by Sink Tester before TBIST_SETUP[MAX)

Sink Tester to examine the data of the received BIST_READY command.

If AV LINK ERROR bit is equal to 1 then Pass[SKIP]  (Tester Error).

Fail if AV LINK Ready bit is not 1.

Sink Tester to send BIST_TRIGGER command with only AV LINK TX BIST enabled (other bits = 0).

Sink Tester waits and captures at  least 15 Frames of Video data.

Sink Tester to issue BIST_STOP command.

Sink Tester to examine the Test Pattern from Source DUT. Fail If the pattern is not as expected (as in 2).

Continue step 2 with Fixed10 pattern, and with PRBS pattern if CDF_AVLINK_BIST_PRBS_SUPP field is YES.

If all BIST patterns Pass, then PASS, else FAIL.

Video Modes

Video Formats

Verify that Source supports the required video formats and the optional video formats listed in the CDF in 24-Bit and 16-bits-per-pixel mode and are compliant with the EIA/CEA-861F specification.

- If CDF_VIDEO_480p_60 field in the CDF is NO and if CDF_VIDEO_576p_50 field in the CDF is NO, then FAIL, else continue to test.

- Configure DUT to send out one of the video formats indicated in the CDF which uses 24-bits-per-pixel mode.

- Measure:

  - H-front porch, H-sync pulse width, H-back porch,  H-active, H-sync polarity.

  - V-front porch, V-sync pulse width, V-back porch,  V-active, V-sync polarity.

  - Clock frequency, offset of the active edge of V-sync from the active edge of H-sync.

- Compare the measurement values with the video format timings defined in the EIA/CEA-861F specification. If any value does not equal the proper values for the select video format, then FAIL.

- If CDF_AVI_SUPPORT is YES, then:

  - If AVI InfoFrame is not sent by Source DUT at least once every 2 fields then FAIL.

  - Tester to examine the content of AVI InfoFrame, FAIL if  it does not match with what being sent in step 2.

- Repeat from step 2 until all video formats indicated in the CDF which use 24-bits-per-pixel mode are tested.

- If all video formats specified in the CDF are verified, then PASS.

Pixel Encoding

Verify that Source is capable of supporting RGB pixel encoding, YCbCr 4:4:4,  and YCbCr 4:2:2 Encoding.

- Verify that Source is capable of supporting RGB pixel encoding, YCbCr 4:4:4,  and YCbCr 4:2:2 Encoding. If CDF_VIDEO_RGB field in the CDF is YES, then continue to test, else FAIL.

- Configure DUT’s output mode as 720x480p 59.94/60Hz or 720x576p 50Hz.

- Configure DUT’s output mode as RGB pixel encoding.

- Configure DUT to send out fully saturated black and white chess board pattern.

- If video data of the fully saturated black and white chess board pattern is detected as RGB pixel encoding, then continue to test, else FAIL.

- If CDF_VIDEO_YCBCR_444 and CDF_VIDEO_YCBCR_422 field in the CDF is NO and if source is capable of sending out color-difference color space across any other analog or digital video interface, then FAIL, else continue to test.

- If CDF_VIDEO_YCBCR_422 field in the CDF is YES, then configure DUT’s output mode as YCbCr 4:2:2 pixel encoding.

- Configure DUT to send out fully saturated black and white chess board pattern.

- If video data of the fully saturated black and white chess board pattern is detected as YCbCr 4:2:2 encoding, then continue to test, else FAIL.

- If CDF_VIDEO_YCBCR_444 field in the CDF is YES, then repeat the test from step 8 with CDF_VIDEO_YCBCR_444.

- If all Pixel Encodings indicated in the CDF is verified, then PASS.

High-End Video Mode Support Requirement

Verify that Source is capable of supporting/ generating  required High-End video resolutions, as defined  in MHL3 requirements.

Table 3-8. High-End Video Format List

| Index | Mode | Refresh |
| --- | --- | --- |
| 93 | 3840x2160 | 24 |
| 94 | 3840x2160 | 25 |
| 95 | 3840x2160 | 30 |

- If CDF_PRODUCT_HEV_CAPABLE is NO then PASS(SKIP).

- If CDF_VIDEO_HEV is NO then FAIL.  Else, continue to test.

- If the CDF does not indicate support for at least one of the modes in Table 3-8, then FAIL.

- Tester to wait for SET_INT(FEAT_REQ) from Source DUT.  Fail if Source does not send this command within TSRC_FEAT_REQ{max}.

- Tester, using one of the two methods below, one at a time, to inform to Source DUT of the Tester’s HEV formats support as shown in Table 3-8:

  - WRITE_BURST only method.

  - EDID only method.

- Configure DUT to send out each of the HEV formats indicated in both the CDF and in Table 3-8, one at a time until all are tested.

- If CDF_AVI_SUPPORT is YES, then:

  - If the AVI and VSIF are not coming from the DUT at least once every 2 fields, then FAIL.

  - Examine the AVI and VSIF data. If they do not match the video mode being sent, then FAIL.

- Tester to measure:

  - H-front porch, H-sync pulse width, H-back porch,  H-active, H-sync polarity.

  - V-front porch, V-sync pulse width, V-back porch,  V-active, V-sync polarity.

  - Offset of the active edge of V-sync from the active edge of H-sync.

- Compare the measurement values with the video format timings defined in the EIA/CEA-861F specification. If any value does not equal the proper values for the select video format, then FAIL.

- If all HEV formats specified in the CDF pass, then PASS, else FAIL.

Audio Modes

IEC 60958 / IEC61937 Support

Verify that Source is capable of audio transmission of Audio Sampling Frequency that is complied with either IEC60958 format or IEC61937 format.

- If CDF_AUDIO_2CH_32kHz and CDF_AUDIO_2CH_44kHz and CDF_AUDIO_2CH_48kHz fields in the CDF are all three set to NO, and if the DUT has no other audio output, then end test with PASS (SKIP).

- If CDF_AUDIO_2CH_32kHz and CDF_AUDIO_2CH_44kHz and CDF_AUDIO_2CH_48kHz fields in the CDF are all three set to NO, and if the DUT has any other audio output, then FAIL.

Configure DUT to output one of the of the audio sampling frequency such as PCM_2CH_192kHz as indicated in step 1 with an appropriate video format that can carry the Audio-mode.

Count audio samples by using Layout and sample_present bits in the header, and get Channel Status Bits.

  - If Layout Value is 1, then FAIL, else continue to test.

  - If repetition period of B bit is 192 Frames, then continue to test, else FAIL.

  - If Channel Status Bit 24, 25, 26, 27 matches sample frequency in the table below, then continue to test, else FAIL.

  - If Sample Size matches the CDF_AUDIO_SAMPLE_SIZE field in the CDF, then continue to test, else FAIL.

Repeat the test from step 3 for other audio sampling as indicated in the CDF.

If any of Audio formats fails, then FAIL, else PASS.

| Channel Status Bit Number |  |  |  | Sample Frequency |
| --- | --- | --- | --- | --- |
| 24 | 25 | 26 | 27 |  |
| 1 | 1 | 0 | 0 | 32 kHz |
| 0 | 0 | 0 | 0 | 44.1 kHz |
| 0 | 1 | 0 | 0 | 48 kHz |
| 0 | 0 | 0 | 1 | 88.2 kHz |
| 0 | 0 | 1 | 1 | 176.4 kHz |
| 0 | 1 | 0 | 1 | 96 kHz |
| 0 | 1 | 1 | 1 | 192 kHz |

Audio Clock Regeneration

Verify that N, CTS and audio sample frequency have correct correlation.

- If one of the CDF_AUDIO_2CH_32kHz,  CDF_AUDIO_2CH_44kHz,  CDF_AUDIO_2CH_48kHz fields in the CDF is set to NO, then FAIL, else continue.

Configure DUT to output one of the of the audio sampling frequency such as PCM_2CH_192kHz as indicated in step 1 with an appropriate video format that can carry the Audio-mode.

Examine the Audio Clock Regeneration Packet.

  - If N value is greater than or equal to {128 x FS}/1500, and is less than or equal to {128 x FS}/300, then continue to test, else FAIL.

  - If Bits 28 and 29 of the Channel Status Bits in the audio stream are set to ‘1’, and ‘0’, respectively, then the expected accuracy is 50ppm:

    - If the average CTS value over more than 2 seconds of measurement meets the limitations of {PixelClock x N}/{128 x FS} within an accuracy of plus or minus 50 ppm, then PASS, else FAIL.

    - NOTE: PixelClock must be multiplied by the Pixel Repetition Factor.

  - Otherwise the expected accuracy is 100ppm:

    - If the average CTS value over more than 2 seconds of measurement meets the limitations of {PixelClock x N}/{128 x actual audio sample rate} within an accuracy of plus or minus 100 ppm, then PASS, else FAIL.

    - NOTE: PixelClock must be multiplied by the Pixel Repetition Factor.

Repeat the test from step 2 for other audio sampling frequency as indicated in the CDF.

If any of Audio mode fails the test, then FAIL, else PASS.

Audio InfoFrame

Verify for audio that whenever an active audio stream is being transmitted an accurate Audio InfoFrame is transmitted at least as often as TAIF.

- If one of the CDF_AUDIO_2CH_32kHz,  CDF_AUDIO_2CH_44kHz,  CDF_AUDIO_2CH_48kHz fields in the CDF is set to NO, then FAIL, else continue.

Configure DUT to output one of the of the audio sampling frequency such as PCM_2CH_192kHz as indicated in step 1 with an appropriate video format that can carry the Audio-mode.

If Audio InfoFrame of which HB0 is 0x84 is detected at least as often as TAIF, then continue to test, else FAIL.

If HB1 – HB2 of the Audio InfoFrame Packet are 0x01 and 0xA (decimal 10) respectively, then continue test, else FAIL.

If CC2, CC1 and CC0 match the number of Audio Channel, then continue to test, else FAIL.

Examine CC2-CC0 and CA7-CA0. If they have valid combination, then continue to test, else FAIL.

If SF2, SF1 and SF0 are 0,0,0, then continue to test, else FAIL.

If LFEPBL0 is 0x01 and LFEPBL1 is 0x01 then FAIL, else continue to test.

If sum of all values is 0x00, then continue to test, else FAIL.

Repeat the test from step 2 for other audio sampling frequency as indicated in the CDF.

If any of Audio mode fails the test, then FAIL, else PASS.

MultiChannel Audio

Verify that Source is capable of transmitting multi-channel audio that is indicated in the CDF that is complied with either IEC60958 format or IEC61937 format.

- If CDF_AUDIO_PCM_CHANNELS fields in the CDF is 2 or less, end test with PASS (SKIP).

Configure DUT to output the highest available sampling rate according to CDF_AUDIO_MAX_FS_MULTI_CH field with the greatest number of channel according to CDF_AUDIO_PCM_CHANNELS.

Count audio samples by using Layout and sample_present bits in the header, and get Channel Status Bits.

  - If Layout Value is 0, then FAIL, else continue to test.

  - If repetition period of B bit is 192 “Frames”, then continue to test, else FAIL.

  - If Channel Status Bit 24, 25, 26, 27 matches sample frequency in the table below, then continue to test, else FAIL.

  - If Sample Size matches the CDF_AUDIO_SAMPLE_SIZE field in the CDF, then continue to test, else FAIL.

If any step above fails, then FAIL, else PASS.

| Channel Status Bit Number |  |  |  | Sample Frequency |
| --- | --- | --- | --- | --- |
| 24 | 25 | 26 | 27 |  |
| 1 | 1 | 0 | 0 | 32 kHz |
| 0 | 0 | 0 | 0 | 44.1 kHz |
| 0 | 0 | 0 | 1 | 88.2 kHz |
| 0 | 0 | 1 | 1 | 176.4 kHz |
| 0 | 1 | 0 | 0 | 48 kHz |
| 0 | 1 | 0 | 1 | 96 kHz |
| 0 | 1 | 1 | 1 | 192 kHz |

IEC 60958 / IEC61937  While in 16-bits-per-pixel Mode

Verify that Source, while transmitting video in 16-bits-per-pixel mode, is capable of audio transmission that is complied with either IEC60958 format or IEC61937 format.

- If CDF_VIDEO_16_BIT field in the CDF indicates no support for 16-bit then end test with PASS (SKIP).

Configure DUT to output one of the PCM_2CH_32kHz, PCM_2CH_44.1kHz ,PCM_2CH_48kHz and as  indicated in the CDF with a 16-bit video formats that can carry the Audio-mode.

Count audio samples by using Layout and sample_present bits in the header, and get Channel Status Bits.

  - If Layout Value is 1, then FAIL, else continue to test.

  - If repetition period of B bit is 192 Frames, then continue to test, else FAIL.

  - If Channel Status Bit 24, 25, 26, 27 matches sample frequency in the table below, then continue to test, else FAIL.

  - If Sample Size matches the CDF_AUDIO_SAMPLE_SIZE field in the CDF, then continue to test, else FAIL.

Repeat the test from step 3 for PCM_2CH_32kHz, PCM_2CH_44.1kHz,PCM_2CH_48kHz and other sampling frequency if it is indicated in the CDF.

If any of Audio formats fails, then FAIL, else PASS.

| Channel Status Bit Number |  |  |  | Sample Frequency |
| --- | --- | --- | --- | --- |
| 24 | 25 | 26 | 27 |  |
| 1 | 1 | 0 | 0 | 32 kHz |
| 0 | 0 | 0 | 0 | 44.1 kHz |
| 0 | 0 | 0 | 1 | 88.2 kHz |
| 0 | 0 | 1 | 1 | 176.4 kHz |
| 0 | 1 | 0 | 0 | 48 kHz |
| 0 | 1 | 0 | 1 | 96 kHz |
| 0 | 1 | 1 | 1 | 192 kHz |

Audio Clock Regeneration While in 16-bits-per-pixel Mode

Verify that N, CTS and audio sample frequency have correct correlation while transmitting 16-bit Video format.

- If CDF_VIDEO_16_BIT field in the CDF indicates no support for 16-bit then end test with PASS (SKIP).

Configure DUT to output one of the PCM_2CH_32kHz, PCM_2CH_44.1kHz, PCM_2CH_48kHz and as indicated in the CDF with a 16-bit video formats that can carry the Audio-mode

Examine the Audio Clock Regeneration Packet.

  - If N value is greater than or equal to {128 x FS}/1500, and is less than or equal to {128 x FS}/300, then continue to test, else FAIL.

  - If Bits 28 and 29 of the Channel Status Bits in the audio stream are set to ‘1’, and ‘0’, respectively, then the expected accuracy is 50ppm:

    - If the average CTS value over more than 2 seconds of measurement meets the limitations of {PixelClock x N}/{128 x FS} within an accuracy of plus or minus 50 ppm, then PASS, else FAIL.

    - NOTE: PixelClock must be multiplied by the Pixel Repetition Factor.

  - Otherwise the expected accuracy is 100ppm:

    - If the average CTS value over more than 2 seconds of measurement meets the limitations of {PixelClock x N}/{128 x actual audio sample rate} within an accuracy of plus or minus 100 ppm, then PASS, else FAIL.

NOTE: PixelClock must be multiplied by the Pixel Repetition Factor.

Repeat the test from step 2 for PCM_2CH_32kHz, PCM_2CH_44.1kHz, PCM_2CH_48kHz and as indicated in the CDF.

If any of Audio mode fails the test, then FAIL, else PASS.

Audio InfoFrame While in 16-bits-per-pixel Mode.

Verify that, while 16-bits-per-pixel mode, whenever an active audio stream is being transmitted an accurate Audio InfoFrame is transmitted at least as often as TAIF.

- If CDF_VIDEO_16_BIT field in the CDF indicates no support for 16-bit then end test with PASS (SKIP).

Configure DUT to output one of the PCM_2CH_32kHz, PCM_2CH_44.1kHz, PCM_2CH_48kHz and as indicated in the CDF with a 16-bit Video format that can carry such Audio format.

If Audio InfoFrame of which HB0 is 0x84 is detected at least as often as TAIF, then continue to test, else FAIL.

If HB1 – HB2 of the Audio InfoFrame Packet are 0x01 and 0xA (decimal 10) respectively, then continue test, else FAIL.

If CC2, CC1 and CC0 match the number of Audio Channel, then continue to test, else FAIL.

Examine CC2-CC0 and CA7-CA0. If they have valid combination, then continue to test, else FAIL.

If SF2, SF1 and SF0 are 0,0,0, then continue to test, else FAIL.

If LFEPBL0 is 0x01 and LFEPBL1 is 0x01 then FAIL, else continue to test.

If sum of all values is 0x00, then continue to test, else FAIL.

Repeat the test from step 2 for PCM_2CH_32kHz, PCM_2CH_44.1kHz, PCM_2CH_48kHz and as indicated in the CDF.

If all Audio modes indicated in the CDF are verified, then PASS.

MultiChannel Audio in 16-bits-per-pixel Mode

Verify that Source, while transmitting 16-bit video mode, is capable of transmitting multi-channel audio that is indicated in the CDF that is complied with either IEC60958 format or IEC61937 format.

- If CDF_VIDEO_16_BIT field in the CDF indicates no support for 16-bit then end test with PASS (SKIP).

If CDF_AUDIO_PCM_CHANNELS fields in the CDF is 2 or less, end test with PASS (SKIP).

Configure DUT to output a 16-bits-per-pixel Video format with the highest available sampling rate according to CDF_AUDIO_MAX_FS_MULTI_CH field and with the greatest number of channel according to CDF_AUDIO_PCM_CHANNELS.

Count audio samples by using Layout and sample_present bits in the header, and get Channel Status Bits.

  - If Layout Value is 0, then FAIL, else continue to test.

  - If repetition period of B bit is 192 “Frames”, then continue to test, else FAIL.

  - If Channel Status Bit 24, 25, 26, 27 matches sample frequency in the table below, then continue to test, else FAIL.

  - If Sample Size matches the CDF_AUDIO_SAMPLE_SIZE field in the CDF, then continue to test, else FAIL.

If any step above fails, then FAIL, else PASS.

| Channel Status Bit Number |  |  |  | Sample Frequency |
| --- | --- | --- | --- | --- |
| 24 | 25 | 26 | 27 |  |
| 1 | 1 | 0 | 0 | 32 kHz |
| 0 | 0 | 0 | 0 | 44.1 kHz |
| 0 | 0 | 0 | 1 | 88.2 kHz |
| 0 | 0 | 1 | 1 | 176.4 kHz |
| 0 | 1 | 0 | 0 | 48 kHz |
| 0 | 1 | 0 | 1 | 96 kHz |
| 0 | 1 | 1 | 1 | 192 kHz |

High-Bitrate Audio

Verify that Source is capable of transmitting High-Bitrate Audio Stream Packets.

- If CDF_AUDIO_HBR field in the CDF indicates no support for High-Bitrate audio then end test with PASS (SKIP).

Configure DUT to output a video format that can carry an HBR audio signal (e.g. Dolby TrueHD MAT or DTS-HD Master Audio)

If any packet type (HB0) 0x02 that has its packet specific data HB1[7:0] NOT equal 0xF0 then FAIL

Repeat the test from step 2 for other  HBR audio signal (e.g Dolby TrueHD or DTS-HD).

If any of Audio formats fails, then FAIL, else PASS.

High-Bitrate Audio While in 16-bits-per-pixel Mode

Verify that Source, while transmitting 16-bit Video Mode, is capable of transmitting High-Bitrate Audio Stream Packets.

- If CDF_VIDEO_16_BIT field in the CDF indicates no support for 16-bit then end test with PASS (SKIP).

If CDF_AUDIO_HBR field in the CDF indicates no support for High-Bitrate audio then end test with PASS (SKIP).

Configure DUT to output a video format in 16-bits-per-pixel Mode that can carry an HBR audio signal (e.g. Dolby TrueHD MAT or DTS-HD Master Audio)

If any packet type (HB0) 0x02 that has its packet specific data HB1[7:0] NOT equal 0xF0 then FAIL

Repeat the test from step 2 for other  HBR audio signal (e.g Dolby TrueHD or DTS-HD).

If any of HBR Audio formats fails, then FAIL, else PASS.

Audio Data Tunneling Receiving Support

Verify that Source is capable of receiving Audio via  Audio data tunneling.

- If the CDF_ADT_RECEIVE_SUPPORT is set to YES, then continue else end test with PASS (SKIP).

Tester Sink sends SET_INT(FEAT_REQ) to Source DUT.

Tester Sink allows Source DUT to execute WRITE_BURST command by responding to REQ_WRT interrupt with GRT_WRT interrupt.

If WRITE_BURST command from Source DUT is not arriving within  TGRT_BURST(max) then FAIL.

Examine the WRITE_BURST data.  If basic audio (32khz, 44.1khz and 48khz) is not supported then FAIL.

Tester to reject (by responding with 0x01,0x02,0x03) any attempt to do TDM Allocation by DUT.

Tester uses Section 13.10 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation that has Feature_ID=0x02 and has enough bandwidth to carry Audio data as indicated in the CDF.

FAIL if DUT does not respond by indicating 0x00 (Accepted).

Configure Sink Tester to output basic ADT Audio ( 32khz, 44.1khz and 48khz) formats, plus the corresponding audio InfoFrame packet, audio regeneration packet.  Note that Audio InfoFrame and Audio Clock Regeneration packets transmission shall be sent at least once each TAIF0.

If CDF_ADT_PLAYING_SUPPORT is YES, perform listening test based on CDF_ADT_RECEIVE_PROC, else skip to step 12.

If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

If any of Audio formats fails, then FAIL, else PASS.

Audio Data Tunneling Transmitting Support

Verify that Source is capable of Transmitting Audio via  Audio data tunneling.

- If CDF_ADT_SEND_SUPPORT is set to YES, then continue else end test with PASS (SKIP).

- Tester to wait for SET_INT(FEAT_REQ) from Source DUT.  Fail if this command is not sent within TSRC_FEAT_REQ{max}.

- Tester , using WRITE_BURST, to inform DUT of the support of basic audio(32khz, 44.1khz and 48khz) and any audio format in the CDF.

- Tester to reject (by responding with 0x01,0x02,0x03) any attempt to do TDM Allocation by DUT.

- Tester uses section 13.10 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation that has Feature_ID=0x02 and has enough bandwidth to carry Audio data as indicated in the CDF.

- FAIL if DUT does not respond by indicating 0x00 (Accepted).

- Configure DUT to enable its TMDS (such as sending any video format).

- Configure DUT to send out all Basic ADT Audio ( 32khz, 44.1khz and 48khz) formats, based on CDF_ADT_SEND_PROC.

- If the Audio InfoFrame and VSIF are not coming from the DUT at least once every 2 fields, then FAIL.

- Examine the Audio InfoFrame and VSIF data (including but not limit to Channel Status, Channel Count, AV_DELAY_SYNC…). If they do not match the Audio mode being sent, then FAIL.

- If Audio Regeneration packet is also sent by the DUT, verify the N and CTS value and their relationship with the link clock of the current audio format according to section 14.2.1 (Audio Data Tunneling and Audio Clock Regeneration).  FAIL if the relationship and the restriction are incorrect.

- Perform listening test if Tester has the ability to output audio which it receives from Source DUT, else skip to step 14.

- If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

- Configure DUT to disable TMDS output.  Then repeat the test from Step 8.

- If all ADT formats specified in the CDF pass, then PASS, else FAIL.

HDCP 2.2 Test

HDCP 2.2

Verify that Source complies with HDCP2.2 Specification.

- If the CDF_HDCP_SUPPORT field in the CDF is YES then continue to test, else PASS (SKIP).

Follow the HDCP2.2-ON-MHL CTS procedure. If the CDF_HDCP_REPEATER field in the CDF is YES, then include all HDCP Repeater tests.

If all tests in the HDCP2.2-ON-MHL CTS procedure result in pass, then PASS; else FAIL.

RBP Sub-Command Tests

The test implementation shall include UTF character sequences for all lengths of characters (1 to 4 8-bit bytes or octets), with both invalid and valid values in each of those lengths.

RBP Sub-Commands Receiving Test

Verify that DUT responds to valid RBP sub-commands.

- If CDF indicates CDF_RBP_RECV_SUPPORT is YES then continue test, else PASS (SKIP).

Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence

Determine supported button codes from CDF and test each for reception of proper code and that RBPK is received after each command from the DUT.

Tester asks operator to confirm the operation was performed in response to the code received by the DUT.

Repeat step 3 for each code identified in the CDF.

From the CDF determine if any unsupported codes exist and send that code to force and RBPE opcode 0x01 (ineffective code)

If DUT passes all iterations of the test, then PASS. Else FAIL.

RBP Sub-Commands Transmitting Test

Verify that DUT sends valid RBP sub-commands.

- If CDF indicates CDF_RBP_SEND_SUPPORT is YES then continue test, else PASS (SKIP).

Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence

Determine supported button codes from CDF and test each for transmission and correct reception of the code by the test equipment and reception of  RBPK is received after each command from the DUT.

Repeat step 3 for each code identified in the CDF

From the CDF list of supported codes, the tester picks one to not support so that an unsupported code condition exists and tester responds with RBPE opcode 0x01 (ineffective code).  Ask test operator to confirm the DUT exhibits the proper error handling such as displying an error message.

If DUT passes all iterations of the test, then PASS. Else FAIL.

ATT Sub-command

The ATT MSC subcommand is new for MHL 3 and is used in conjunction with Multi-View support.

ATT Subcommand Test

Verify functionality and support of ATT MSC subcommand used in Multi-View  support.

- Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence.

Upon Receiving SET_INT(FEAT_REQ) from Source DUT, Tester, following WRITE_BURST protocol, to send to Source DUT VDI data with VDI_L= 0xFF (indicating support of all Multi-View Mode).

Tester to send SET_INT(FEAT_REQ).

FAIL if WRITE_BURST from Source DUT does not arrive within TGRT_BURST(max).

FAIL if SET_INT(FEAT_COMPLETE) does not arrive within TSOURCE_FEAT_COMPL(max).

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT. It is necessary for the tester to record in a variable the support status for Multi-View indicated by the DUT in the VDI field of its WRITE_BURSTs of HEV video modes. If any VDI field indicates support for Multi-View, then the ATT command shall be accurately supported. If no VDI field indicates support for Multi-View, then the ATT command shall not be sent to the DUT by a connected Source. (Refer to MHL 3 Specification, Sections 5.11, and 7.9.1.)

  - If DUT's VDI_L[5:0] are not all '0', then DUT is indicating some supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to YES.

  - If DUT's VDI_L[5:0] are all '0', then DUT is indicating no supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to NO.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and MEAS_VIDEO_MULTIVIEW is NO; then PASS (SKIP), else continue.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is YES, and MEAS_VIDEO_MULTIVIEW is YES, then continue, else FAIL.

Invoke DUT to send ATT command.

Invoke DUT to send ATT command.

If DUT ATT command is successful,  test equipment shall respond  with ATTK.

If  TATT_WAIT{max} elapses and no further commands are sent then FAIL, else continue.

If transaction completes without error then continue, else FAIL.

Tester to send ATT command.

FAIL if DUT does not respond with ATTK before TATT_WAIT{max} elapses.

If transaction completes without error then PASS, else FAIL.

AVLINK Packetization Tests

The AVLINK packetization tests are specific to MHL3 and are defined to verify low level AVLINK link compliance.  These tests are not defined for legacy MHL compatibility mode.

AVLINK Speed Test

Verify functionality and link speeds supported by source DUT.

- The Source DUT is connected to the MHL 3 tester and run through the test for each speed indicated in the CDF.  The Tester advertises each supported CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600 in its TMDS_SPEEDS register in numerical order starting with TMDS_150.  At least one must be supported but if any other is not supported as indicated in the CDF, these test iterations will receive a PASS(SKIP).

Verify that Source DUT TMDS_SPEEDS register matches the CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600  information in the CDF.

Check the Tester AVLINK_MODE_CONTROL: LINK_RATE field to verify that Source DUT has connected at the correct speed.

Examine the captured AV Symbol Stream with its associated timestamps.

Verify that the Symbol rate is correct over 2 frames.

Verify that the captured AV symbols are valid TMDS symbols.

Verify that the TMDS stream contains TMDS symbols which correctly manage disparity.

AVLINK Packetization Basic Protocol

Verify functionality and basic protocol of the source DUT AVLINK.

- The Tester Sink connects to the Source DUT using TMDS_150.

Command Source DUT to send a 720x480p 59.94/60Hz or 720x576p 50Hz video.

Examine the captured AV Symbol Stream with its associated timestamps.

Verify that the 3 Symbols following each SOP Symbol are identical (after removing any bursts of 4 RR Symbols).

Verify that the Source DUT does not send Extended Content packets.

Verify that the Source DUT does not send packets using Reserved header values.

Verify that RR symbols occur in groups of exactly 4 symbols.

Verify that TMDS Maximized RR Symbol groups occur with a periodicity of T_RR{max} or less.
(If the capture window is smaller than 2 * T_RR{max}, it may be necessary to perform several AV Data captures in order to see 2 bursts of RR Symbols.)

Verify that the Randomizer makes a valid randomizer sequence following each RR Symbol burst by de-randomizing  GAP symbols. GAP symbols occur between an EOP symbol and a following SOP symbol (excluding RR symbols).  GAP symbols also occur after a Content packet containing 252 bytes of data, until a following SOP symbol (excluding RR symbols).

Verify that each sequence of GAP symbols ends with a SOP symbol (excluding RR symbols).

If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol Timing

Verify functionality and basic protocol of the source DUT AVLINK.

- The Tester Sink connects to the Source DUT using TMDS_150.

Command Source DUT to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

Examine the captured AV Symbol Stream with its associated timestamps.

Locate the beginning and ending of AV Data sections.

Verify that the start of AV Data sections meets the T_TMDS_PACKET_JITTER Spec.

Verify that each byte of Control and AV Data is transferred within +- 256 Symbol times of its ideal time.

If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Extended Content Protocol

Verify functionality when Extended Content packets are sent by the Source DUT AVLINK.

- If CDF_EXTENDED_CONTENT_SUPPORT is NO, then end test with PASS (SKIP).

- If CDF_PROC_SRC_EXTENDED_CONTENT_SELECT is not empty, have operator execute the procedure.

- The Tester Sink connects to the Source DUT using TMDS_600.

Command Source DUT to send AV Link data with the format specified in CDF_EXTENDED_CONTENT_SUPPORT.

Examine the captured AV Symbol Stream with its associated timestamps.

Verify that the Source DUT sends Extended Content packets and no Content packets.

Verify that the Source DUT does not send packets using Reserved header values.

Verify that each Extended Content packet ends with an EOP.

Verify that the following Headers all use the same PKT_STREAM_ID.

Locate the beginning and ending of Blanking Period Data sections.

Locate the beginning and ending of AV Data sections.

Verify that Blanking Period sections which are in the Vertical Blanking period are terminated by EOP Symbols so that no “Extended Content” Packet is longer than the equivalent period of one Video Line.

If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Extended Content Protocol Timing

Verify functionality when Extended Content packets are sent on the DUT AVLINK.

- If CDF_EXTENDED_CONTENT_SUPPORT is NO, then end test with PASS (SKIP).

- If CDF_PROC_SRC_EXTENDED_CONTENT_SELECT is not empty, have operator execute the procedure.

- The Tester Sink connects to the Source DUT using the TMDS Link speed needed to force Extended Content Packetization.

Command Source DUT to send AV data with the format specified in CDF_EXTENDED_CONTENT_SUPPORT.Examine the captured AV Symbol Stream with its associated timestamps.

Locate the beginning and ending of AV Data sections.

Verify that the start of AV Data sections meets the T_TMDS_PACKET_JITTER Spec.

Verify that each byte of Control and AV Data is transferred within +- 256 Video Content Symbol times of its ideal time.

If no failures are detected, then PASS. Else FAIL.

3D and Multi-View

Tests a Source device ability to support and properly evaluate a sink devices 3D, Multi-view and dual 3D mode capability.  Refer to section 5.11.2.1 MHL Source Multi-View Format Support Requirements and Table 5-23 MHL Source Multi-View Support Requirements in MHL 3.0 spec for minimum support requirements.

Table 3-9. 3D and Multi-View Testing Configurations

| Configuration | VDI_L Settings |  |  | Description (3D/Multiview mode support) |
| --- | --- | --- | --- | --- |
|  | VDI_L[6] | VDI_L[5:3] | VDI_L[2:0] |  |
| 1 | 0 | 0 | 0 | No Multi-View or 3D support |
| 2 | 0 | 0 | Non-zero | 3D |
| 3 | 0 | Non-zero | 0 | 4-View |
| 4 | 0 | Non-zero | Non-zero | 4-View, 2-View, 3D |
| 5 | 1 | 0 | 0 | None (invalid setting) |
| 6 | 1 | 0 | Non-zero | 2-View, 3D |
| 7 | 1 | Non-zero | 0 | 4-View, Dual3D |
| 8 | 1 | Non-zero | Non-zero | 4-View, 2-View, Dual3D, 3D |
| - Note1: VDI_L[7] = 1 to indicate separate audio for multiview. |  |  |  |  |
| - Note2: VDI_L[2:0] is for LR,TB,FS_SUPP respectively. |  |  |  |  |
| - Note3: VDI_L[5:3] is for TBLR,FSLR, FSTB respectively. |  |  |  |  |

3D Support

Verify functionality and support of 3D and Multi-View modes.

- If CDF_PRODUCT_3D_CAPABLE field is NO then PASS[SKIP].

- FAIL if CDF_VIDEO_3D field is NO.

- FAIL if CDF does not indicate a minimun support for the 3D modes as indicated in MHL spec Table 5-9 (MHL Source 3D mode support Requirements).

- Tester to construct the VDI_L byte as follow:

  - VDI_L[0]=’1’ if CDF_VIDEO_FS_SUPP field is not empty, else = ‘0’.

  - VDI_L[1]=’1’ if CDF_VIDEO_TB_SUPP field is not empty, else = ‘0’.

  - VDI_L[2]=’1’ if CDF_VIDEO_LR_SUPP field is not empty, else = ‘0’.

  - VDI_L[7:3]=’0’.  (Since only 3D is tested, these fields become do not care).

- Tester to wait for Source DUT to send SET_INT(FEAT_REQ).  If this does not arrive within TSRC_FEAT_REQ{max}   then FAIL.

- Tester, following WRITE_BURST protocol, to send to Source DUT VDI data as constructed in step 4.

- Configure DUT to send each supported video modes as indicated in the CDF.

- If either AVI infoframe or VSIF is not sent at least once every 2 video fields, then FAIL.

- Tester to verify AVI infoframe and VSIF data (VIC, MHL_VID_FMT, MHL_3D_FMT_TYPE…).  If data does not match the correct video mode being sent then FAIL.

- Tester to verify video stream correctly for each mode that was sent.

- Repeat from Step 7 for all supported video modes that is indicated in the CDF.

- If DUT passes all iterations of the test, then PASS. Else FAIL.

Multi-View Support

Verify Multiview/Dual-3D functionality and support of Source DUT.

- If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO then PASS[SKIP].

- FAIL If CDF_VIDEO_MULTIVIEW field is NO.

- FAIL if CDF does not indicate support for one of the Multiview modes as indicated in MHL spec Table 5-23 (MHL Source Multi-View Support Requirements).

- Tester to construct the VDI_L byte as follow:

  - VDI_L[0]=’1’ if CDF_VIDEO_FS_SUPP field is not empty, else = ‘0’.

  - VDI_L[1]=’1’ if CDF_VIDEO_TB_SUPP field is not empty, else = ‘0’.

  - VDI_L[2]=’1’ if CDF_VIDEO_LR_SUPP field is not empty, else = ‘0’.

  - VDI_L[3]=’1’ if CDF_VIDEO_FSTB_SUPP field is not empty, else = ‘0’.

  - VDI_L[4]=’1’ if CDF_VIDEO_FSLR_SUPP field is not empty, else = ‘0’.

  - VDI_L[5]=’1’ if CDF_VIDEO_TBLR_SUPP field is not empty, else = ‘0’.

  - VDI_L[6]=’1’ if CDF_VIDEO_DUAL3D field is not empty, else = ‘0’.

  - VDI_L[7]=’1’ if CDF_VIDEO_SEP_AUD field is YES, else = ‘0’.

- Tester to wait for Source DUT to send SET_INT(FEAT_REQ).  If this does not arrive within TSRC_FEAT_REQ{max}   then FAIL.

- Tester , following WRITE_BURST protocol, to send to Source DUT VDI data as constructed in step 4.

- Configure DUT to send each supported video mode, starting with a 2View mode if supported, as indicated inthe CDF  with separate audio for each views If constructed VDI_L[7] = ‘1’ (else, one audio stream for the whole multi-view).

- If either AVI infoframe or VSIF is not sent at least once every 2 video fields, then FAIL.

- Tester to verify AVI infoframe and VSIF data (VIC, MHL_VID_FMT, MHL_3D_FMT_TYPE…).  If data does not match with the correct video mode being sent, then FAIL.

- Tester to verify video stream correctly for each mode that was sent.Repeat from Step 7 for all supported video modes that is indicated in the CDF in the order below.

  - All 2View modes.

  - All 4View modes

  - All Dual3D modes.

- If DUT passes all iterations of the test, then PASS. Else FAIL.

AV SYNC

Source AV Sync Test

Verify functionality of Source DUT's AV Sync function.

- If CDF_AV_SYNC_SUPPORTED is YES  then continue to test, else PASS(SKIP)

Connect DUT and perform Discovery

Configure the DUT to output 720x480p 59.94/60Hz or 720x576p 50Hz and PCM_2CH_44kHz audio with 1kHz sine wave test tone. Source VSIF indicates no audio delay.

Verify VSIF indicates no audio delay, if no delay then continue, else FAIL.

Tester sends Write Burst command to request audio delay by by setting value of DELAY_H= 0xAA, DELAY_M=0xAA, DELAY_L=0xAA.

Wait for DUT to respond through VSIF indicating the audio delay now being sent.

If new DUT audio delay value matches the requested value,  then PASS.

EDID

Tests the Source device ability to read and evaluate the EDID contents in MHL3 mode using eCBUS protocol.

EDID Read Test

Verify that Source DUT properly reads EDID.

- Configure  a Pseudo MHL sink device to have a valid 2 block EDID with CEA timing extension ver3.

Connect DUT to the Pseudo MHL sink device.

If DUT reads block 0 and block 1 while MHL link is being established, then continue to test, else FAIL.

Update EDID in the Pseudo MHL sink device, and indicate to the DUT by EDID_CHG interrupt bit in DCHANGE_INT register that EDID has changed.

If DUT reads block 0 and block 1 again, then continue to test, else FAIL.

Send CLR_HPD command to the DUT, wait THPD_WIDTH{min}, then send SET_HPD to the DUT.

If DUT reads block 0 and block 1 again, then continue to test, else FAIL.

Disconnect Pseudo MHL sink device from the DUT.

Configure a Pseudo MHL sink device to have a valid 4 block EDID with extension map and CEA timing extension ver3.

Connect DUT to the Pseudo MHL sink device.

If DUT reads block 0,1,2 and block 3 while MHL link is being established, then continue to test, else FAIL.

Update EDID in the Pseudo MHL sink device, and indicate to the DUT by EDID_CHG interrupt bit in DCHANGE_INT register (using SET_INT command) that EDID has changed.

If DUT reads block 0,1,2 and block 3 again, then PASS, else FAIL.

# Sink Test

## Electrical Tests

TMDS Electrical Tests

All tests in the CTS 2, Section 4.1.1, shall be run against the Sink DUT.

## System Tests

This test group defines the MHL3 legacy compliance testing in MHL 1 / MHL 2 mode.

The equipment setups shown in Section 2.3.2 are used throughout Section 4.2.

All MHL3 devices have an MHL 1 / MHL 2 compatibility mode test requirement.  Refer to MHL CTS 2 test procedures for the specific test steps for legacy tests.  New tests for MHL 3 are defined in Section 4.8.  The general Required Methodology for all MHL3 Sink DUT tests based on normative references to CTS 2.1 are tested in oCBUS mode.  Test results for each legacy mode test shall be recorded in the TRF by their test ID as listed in Table 4-1below.

Table 4-1. Sink AVLINK Test Sequence

| Test ID | MHL 2 oCBUS Compatibility mode |
| --- | --- |
| 4.2.1.1 | X |
| 4.2.1.2 | X |
| 4.2.1.3 | X |
| 4.2.1.4 | X |
| 4.2.2.1 | X |
| 4.2.2.2 | X |
| 4.2.2.3 | X |
| 4.2.2.4 | X |
| 4.2.2.5 | X |
| 4.2.3.1 | X |
| 4.2.3.2 | X |
| 4.2.4.1 | X |
| 4.2.5.1 | X |
| 4.2.5.2 | X |
| 4.2.5.3 | X |
| 4.2.6.1 | X |
| 4.2.6.2 | X |
| 4.2.7.1 | X |
| 4.2.8.1 | X |
| 4.2.8.2 | X |
| 4.2.9.1 | X |
| 4.2.9.2 | X |

## CBUS Tests

Tests in Section 4.3 of the released MHL 2 Compliance Test Specification are executed using the varieties of CBUS as noted in Table 4-2. As each test is executed in Section 4.3, the test number beginning with "4.3", together with the CBUS mode as invoked from Table 4-2, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 4-2, according to which columns of Table 4-2 are marked with an 'X'.

Table 4-2. eCBUS oCBUS Suite CBUS Tests

| TEST | MHL 2 oCBUS Compatibility Mode |
| --- | --- |
| 4.3.3.2 | X |
| 4.3.3.3 | X |
| 4.3.4.1 | X |
| 4.3.5.1 | X |
| 4.3.6.1 | X |
| 4.3.6.2 | X |
| 4.3.6.3 | X |
| 4.3.6.4 | X |
| 4.3.7.1 | X |
| 4.3.7.2 | X |
| 4.3.8.1 | X |
| 4.3.8.2 | X |
| 4.3.9.1 | X |
| 4.3.10.1 | X |
| 4.3.10.2 | X |
| 4.3.11.1 | X |
| 4.3.11.2 | X |
| 4.3.11.3 | X |
| 4.3.11.4 | X |
| 4.3.12.1 | X |
| 4.3.12.2 | X |
| 4.3.13.1 | X |
| 4.3.14.1 | X |
| 4.3.14.2 | X |
| 4.3.14.3 | X |
| 4.3.15.1 | X |
| 4.3.15.2 | X |
| 4.3.16.1 | X |
| 4.3.17.1 | X |
| 4.3.17.2 | X |
| 4.3.18.1 | X |
| 4.3.19.1 | X |
| 4.3.20.1 | X |
| 4.3.21.1 | X |
| 4.3.22.1 | X |
| 4.3.23.1 | X |
| 4.3.23.2 | X |
| 4.3.24.1 | X |
| 4.3.25.1 | X |

## eCBUS oCBUS / MHL 3 Legacy Protocol Tests

Tests in Section 6.3 of the released MHL 2 Compliance Test Specification are executed using the varieties of CBUS as noted in Table 4-3. As each test is executed in Section 6.3, the test number beginning with "6.3", together with the CBUS mode as invoked from Table 4-3, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 4-3, according to which columns of Table 4-3 are marked with an 'X'.

Table 4-3. eCBUS oCBUS / MHL 3 Suite Normal Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.3.1.1 | X | X | if DUT Supports |
| 6.3.2.1 | X | X | if DUT Supports |
| 6.3.3.1 | X | X | if DUT Supports |
| 6.3.3.2 | X | X | if DUT Supports |
| 6.3.3.3 | X | X | if DUT Supports |
| 6.3.3.4 | X | X | if DUT Supports |
| 6.3.3.5 | X | X | if DUT Supports |
| 6.3.3.6 | X | X | if DUT Supports |
| 6.3.3.7 | X | X | if DUT Supports |
| 6.3.5.1 | X | X | if DUT Supports |
| 6.3.5.2 | X | X | if DUT Supports |
| 6.3.5.3 | X | X | if DUT Supports |
| 6.3.5.4 | X | X | if DUT Supports |
| 6.3.5.5 | X | X | if DUT Supports |
| 6.3.5.6 | X | X | if DUT Supports |
| 6.3.5.7 | X | X | if DUT Supports |
| 6.3.5.8 | X | X | if DUT Supports |
| 6.3.8.1 | X | X | if DUT Supports |
| 6.3.8.2 | X | X | if DUT Supports |
| 6.3.10.1 | X | X | if DUT Supports |
| 6.3.10.2 | X | X | if DUT Supports |
| 6.3.10.3 | X | X | if DUT Supports |
| 6.3.10.4 | X | X | if DUT Supports |
| 6.3.10.5 | X | X | if DUT Supports |
| 6.3.10.6 | X | X | if DUT Supports |
| 6.3.10.7 | X | X | if DUT Supports |
| 6.3.10.8 | X | X | if DUT Supports |
| 6.3.11.20 | X | X | if DUT Supports |
| 6.3.11.21 |  |  |  |
| 6.3.13.1 |  |  |  |
| 6.3.13.2 |  |  |  |
| 6.3.14.1 |  |  |  |
| 6.3.14.2 |  |  |  |
| 6.3.15.1 | X | X | if DUT Supports |
| 6.3.15.2 | X | X | if DUT Supports |
| 6.3.16.1 | X | X | if DUT Supports |
| 6.3.16.2 | X | X | if DUT Supports |
| 6.3.17.2 |  |  |  |
| 6.3.17.3 |  |  |  |
| 6.3.18.1 |  |  |  |
| 6.3.18.2 |  |  |  |
| 6.3.18.3 |  |  |  |
| 6.3.18.4 |  |  |  |
| 6.3.20.2 | X | X | if DUT Supports |
| 6.3.20.3 | X | X | if DUT Supports |
| 6.3.21.1 | X | X | if DUT Supports |
| 6.3.21.2 | X | X | if DUT Supports |
| 6.3.21.3 | X | X | if DUT Supports |
| 6.3.21.4 | X | X | if DUT Supports |
| 6.3.21.5 | X | X | if DUT Supports |

Notes on Table 4-3:

- The Required Methodology text in MHL 2.x CTS calls out oCBUS parameter values which must be replaced with MHL 3 parameter values during the test of an MHL 3 DUT.

Table 4-4. eCBUS oCBUS / MHL 3 Suite Robustness Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.3.4.1 |  |  |  |
| 6.3.6.1 | X | X | if DUT Supports |
| 6.3.6.2 | X | X | if DUT Supports |
| 6.3.6.3 | X | X | if DUT Supports |
| 6.3.6.4 | X | X | if DUT Supports |
| 6.3.6.5 | X | X | if DUT Supports |
| 6.3.7.1 | X | X | if DUT Supports |
| 6.3.9.1 | X | X | if DUT Supports |
| 6.3.11.1 | X | X | if DUT Supports |
| 6.3.11.2 | X | X | if DUT Supports |
| 6.3.11.3 | X | X | if DUT Supports |
| 6.3.11.4 | X | X | if DUT Supports |
| 6.3.11.5 | X | X | if DUT Supports |
| 6.3.11.6 | X | X | if DUT Supports |
| 6.3.11.7 | X | X | if DUT Supports |
| 6.3.11.8 | X | X | if DUT Supports |
| 6.3.11.9 | X | X | if DUT Supports |
| 6.3.11.10 | X | X | if DUT Supports |
| 6.3.11.11 | X | X | if DUT Supports |
| 6.3.11.12 | X | X | if DUT Supports |
| 6.3.11.13 | X | X | if DUT Supports |
| 6.3.11.14 | X | X | if DUT Supports |
| 6.3.11.15 | X | X | if DUT Supports |
| 6.3.11.16 | X | X | if DUT Supports |
| 6.3.11.17 | X | X | if DUT Supports |
| 6.3.11.19 | X | X | if DUT Supports |
| 6.3.11.22 | X | X | if DUT Supports |
| 6.3.11.23 | X | X | if DUT Supports |
| 6.3.11.24 | X | X | if DUT Supports |
| 6.3.12.1 | X | X | if DUT Supports |
| 6.3.12.2 | X | X | if DUT Supports |
| 6.3.12.3 | X | X | if DUT Supports |
| 6.3.12.4 | X | X | if DUT Supports |
| 6.3.12.5 | X | X | if DUT Supports |
| 6.3.12.6 | X | X | if DUT Supports |
| 6.3.12.7 | X | X | if DUT Supports |
| 6.3.12.8 | X | X | if DUT Supports |
| 6.3.12.9 | X | X | if DUT Supports |
| 6.3.16.1 | X | X | if DUT Supports |
| 6.3.16.2 | X | X | if DUT Supports |
| 6.3.19.1 |  |  |  |
| 6.3.19.2 |  |  |  |
| 6.3.19.3 |  |  |  |
| 6.3.19.4 |  |  |  |
| 6.3.22.1 | X | X | if DUT Supports |
| 6.3.22.2 | X | X | if DUT Supports |
| 6.3.22.3 | X | X | if DUT Supports |

## eCBUS MHL 3 Active Mode Sink Tests

Tests in Section 6.4 of this Compliance Test Specification are executed using the varieties of CBUS as noted in Table 4-5. As each test is executed in Section 6.4, the test number beginning with "6.4", together with the CBUS mode as invoked from Table 4-5, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 4-5, according to which columns of Table 4-5 are marked with an 'X'.

Table 4-5. eCBUS MHL 3 Active Mode Sink Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.4.3.1 | X |  |  |
| 6.4.3.1 | X |  |  |
| 6.4.3.2 | X |  |  |
| 6.4.3.3 | X |  |  |
| 6.4.3.4 | X |  |  |
| 6.4.3.7 | X |  |  |
| 6.4.3.8 | X |  |  |
| 6.4.4.2 | X |  |  |
| 6.4.6.1 | X |  |  |
| 6.4.6.2 | X |  |  |
| 6.4.7.2 |  | X |  |
| 6.4.8.2 |  |  | If DUT Supports |
| 6.4.9.1 |  | X | If DUT Supports |
| 6.4.10.1 |  | X | If DUT Supports |
| 6.4.10.2 |  | X | If DUT Supports |
| 6.4.10.3 |  | X | If DUT Supports |
| 6.4.10.4 |  | X | If DUT Supports |
| 6.4.10.5 |  | X | If DUT Supports |
| 6.4.11.1 |  | X | If DUT Supports |
| 6.4.11.2 |  | X | If DUT Supports |
| 6.4.11.3 |  | X | If DUT Supports |
| 6.4.12.1 |  | X | If DUT Supports |
| 6.4.12.2 |  | X | If DUT Supports |
| 6.4.12.3 |  | X | If DUT Supports |
| 6.4.13.1 |  | X | If DUT Supports |
| 6.4.13.2 |  | X | If DUT Supports |
| 6.4.13.3 |  | X | If DUT Supports |
| 6.4.14.1 |  | X | If DUT Supports |
| 6.4.15.1 |  | X | If DUT Supports |
| 6.4.16.2 |  | X | If DUT Supports |
| 6.4.16.3 |  | X | If DUT Supports |
| 6.4.16.4 |  | X | If DUT Supports |
| 6.4.16.5 |  | X | If DUT Supports |
| 6.4.17.2 |  | X | If DUT Supports |
| 6.4.18.2 |  | X | If DUT Supports |
| 6.4.18.3 |  | X | If DUT Supports |

## eCBUS MHL 3 Robustness Mode Sink Tests

Tests in Section 6.5 of this Compliance Test Specification are executed using the varieties of CBUS as noted in Table 4-6. As each test is executed in Section 6.5, the test number beginning with "6.5", together with the CBUS mode as invoked from Table 4-6, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 4-6, according to which columns of Table 4-6 are marked with an 'X'.

Table 4-6. eCBUS MHL 3 Robustness Mode Sink Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.5.1.1 |  | X | if DUT Supports |
| 6.5.1.1 |  | X | If DUT Supports |
| 6.5.1.2 |  | X | If DUT Supports |
| 6.5.2.1 |  | X | If DUT Supports |
| 6.5.2.2 |  | X | If DUT Supports |
| 6.5.2.3 |  | X | If DUT Supports |
| 6.5.3.2 |  | X | If DUT Supports |
| 6.5.3.3 |  | X | If DUT Supports |
| 6.5.3.4 |  | X | If DUT Supports |
| 6.5.4.1 |  | X | If DUT Supports |
| 6.5.5.1 |  | X | If DUT Supports |
| 6.5.5.2 |  | X | If DUT Supports |
| 6.5.5.3 |  | X | If DUT Supports |

## MHL 3 Electrical Tests

TMDS and eCBUS Electrical Test Setups

Figure 4-1. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Sink Test

Figure 4-2. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Sink Jitter Tolerance Test

Figure 4-3. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Sink Test

Figure 4-4. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Sink Jitter Tolerance Test

Figure 4-5. Test configuration at TP2 for Sink TMDS input and eCBUS-S FWD input

Figure 4-6. Test configuration at TP2 for Sink TMDS and eCBUS Tests

Figure 4-7. Test configuration at TP2 for Sink TMDS input and eCBUS-D FWD input

Figure 4-8. Test configuration at TP2 for Sink TMDS and eCBUS FWD Tests

Figure 4-9. Test configuration at TP2 for Sink eCBUS-S BWD output

Figure 4-10. Test configuration at TP1 for Sink eCBUS-S BWD output

Figure 4-11. Test configuration at TP2 for Sink eCBUS-D BWD output

Figure 4-12. Test configuration at TP1 for Sink eCBUS-D BWD output

Figure 4-13. Test configuration at TP2 for sink TMDS and eCBUS-S impedance

Figure 4-14. Test configuration at TP2 for sink TMDS and eCBUS-D impedance

TMDS and eCBUS Electrical Test Procedures

Input DC Voltage Tolerance of Differential TMDS Data: VIDC_DF_TMDS_DATA

This test confirms that the sink device supports the DC voltage level of the differential TMDS data input signal allowed by the specification.

Test Signal Calibration

- Make connections shown in Figure 4-1 for DUT with eCBUS-S and Figure 4-3 for DUT with eCBUS-D and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS

      - eCBUS-S: 750ps

      - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS

      - eCBUS-S single-ended swing: 310mV

      - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS

      - Max DC Level: VTERM – 180mV

      - Min DC level: VTERM – 1350mV

    - eCBUS

      - eCBUS-S: 155mV

      - eCBUS-D: VTERM – 228mV

Tolerance Test

- Make connections shown Figure 4-5 for DUT with eCBUS-S and Figure 4-7 for DUT with eCBUS-D. Use Max DC Level setting in vi.1.a of Test Signal Calibration described above for TMDS signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count.

Change the TMDS signal output DC level to the Min DC Level setting in vi.1.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Input DC Voltage Tolerance of Differential MHL CLK and eCBUS-D FWD Data: VIDC_DF_MHL_CLK, VIDC_DF_eCBUS_FWD

This test confirms that the sink device supports the DC voltage level of the differential MHL CLK and eCBUS-D FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown Figure 4-3 and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D

      - Max DC Level: VTERM – 80mV

      - Min DC level: VTERM – 375mV

Tolerance Test

- Make connections shown in Figure 4-7. Use Max DC Level setting in vi.2.a of Test Signal Calibration described above for eCBUS-D FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-D FWD signal output DC level to the Min DC Level setting in vi.2.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Differential input swing voltage Tolerance of Differential TMDS Data: VIDF_SWING_DF_TMDS_DATA

This test confirms that the sink device supports the differential swing voltage of the differential TMDS data input signal allowed by the specification.

Test Signal Calibration

- Make connections shown Figure 4-1 for DUT with eCBUS-S and Figure 4-3 for DUT with eCBUS-D and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS

      - eCBUS-S: 1000ps

      - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing:

      - Max swing: 1000mV

      - Min swing: 150mV

    - eCBUS

      - eCBUS-S single-ended swing: 310mV

      - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS

      - eCBUS-S: 155mV

      - eCBUS-D: VTERM – 228mV

Tolerance Test

- Make connections shown Figure 4-5 for DUT with eCBUS-S and Figure 4-7 for DUT with eCBUS-D. Use Max swing setting in v.1.a of Test Signal Calibration described above for TMDS signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count.

Change the TMDS signal output swing to the Min swing setting in v.1.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Differential Input Swing Voltage Tolerance of Differential MHL CLK and eCBUS-D FWD Data: VIDF_SWING_DF_MHL_CLK, VIDF_SWING_DF_eCBUS_FWD

This test confirms that the sink device supports the differential swing voltage of the differential MHL CLK and eCBUS-D FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 4-3 and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-D differential swing

      - Max swing: 700mV

      - Min swing: 150mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D: VTERM – 228mV

Tolerance Test

- Make connections shown in Figure 4-7. Use Max swing setting in v.2.a of Test Signal Calibration described above for eCBUS-D FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-D FWD signal output swing to the Min swing setting in v.2.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Input DC Voltage Tolerance of Single-Ended MHL CLK and eCBUS-S FWD Data: VIDC_SE_MHL_CLK, VIDC_SE_eCBUS_FWD

ngle-ended MHL CLK and eCBUS-S FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-S.

Test Signal Calibration

- Make connections shown in Figure 4-1 and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-S: 1000ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-S single-ended swing: 310mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-S

      - Max DC Level: 215mV

      - Min DC level: 95mV

Tolerance Test

- Make connections shown in Figure 4-5. Use Max DC Level setting in vi.2.a of Test Signal Calibration described above for eCBUS-S FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-S FWD signal output DC level to the Min DC Level setting in vi.2.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Single-Ended Input Swing Voltage Tolerance of Single-Ended MHL CLK and eCBUS-S FWD Data: VISE_SWING_SE_MHL_CLK, VISE_SWING_SE_eCBUS_FWD

This test confirms that the sink device supports the single-ended swing voltage of the single-ended MHL CLK and eCBUS-S FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-S.

Test Signal Calibration

- Make connections shown in Figure 4-1 and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-S: 1000ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-S single-ended swing

      - Max swing: 370mV

      - Min swing: 170mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-S: 155mV

Tolerance Test

- Make connections shown in Figure 4-5. Use Max swing setting in v.2.a of Test Signal Calibration described above for eCBUS-S FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-S FWD signal output swing to the Min swing setting in v.2.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Differential Intra-Pair Skew Tolerance of Differential TMDS Data: TSKEW_TP2_DF_TMDS_DATA

This test confirms that the sink device tolerates the differential intra-pair skew of the differential TMDS data input signal allowed by the specification.

Test Signal Calibration

- Make connections shown in Figure 4-1 for DUT with eCBUS-S and Figure 4-3 for DUT with eCBUS-D and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS

      - eCBUS-S: 1000ps

      - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS

      - eCBUS-S single-ended swing: 310mV

      - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS

      - eCBUS-S: 155mV

      - eCBUS-D: VTERM – 228mV

  - TMDS intra-pair skew

    - Positive 25ps: positive output leads negative output by 25ps.

    - Negative 25ps: negative output leads positive output by 25ps.

Tolerance Test

- Make connections shown in Figure 4-6 for DUT with eCBUS-S and Figure 4-8 for DUT with eCBUS-D. Use Positive 25ps skew setting in vii.1 of Test Signal Calibration described above for TMDS intra-pair skew.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count.

Change the TMDS signal intra-pair skew to the Negative 25ps setting in vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

Swap the TMDS signal connection between the MHL3 Cable Emulator and MHL3 Signal Generator.

Repeat steps 1 to 11.

If the Error Count is zero in steps 9, 11 and 13, then PASS; else FAIL.

Differential Intra-Pair Skew Tolerance of Differential MHL CLK and eCBUS-D FWD Data: TSKEW_TP2_DF_MHL_CLK, TSKEW_TP2_DF_eCBUS_FWD

This test confirms that the sink device tolerates the differential intra-pair skew of the differential MHL CLK and eCBUS-D FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 4-3 and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D: VTERM – 228mV

  - Differential MHL CLK and eCBUS-D FWD intra-pair skew

    - Positive 50ps: positive output leads negative output by 50ps.

    - Negative 50ps: negative output leads positive output by 50ps.

Tolerance Test

- Make connections shown in Figure 4-8. Use Positive 50ps skew setting in vii.1 of Test Signal Calibration described above for MHL CLK and eCBUS-D FWD intra-pair skew.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the MHL CLK and eCBUS-D signal intra-pair skew to the Negative 50ps setting in vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

Swap the eCBUS-D signal connection between the MHL3 Cable Emulator and MHL3 Signal Generator.

Repeat steps 1 to 11.

If the Error Count is zero in steps 9, 11 and 13, then PASS; else FAIL.

Jitter Tolerance of Differential MHL CLK, eCBUS-D FWD Data and Differential TMDS Data: TCLOCK_JITTER_TP2_DF_MHL_CLK, TMDS Eye Diagram at TP2, eCBUS-D FWD Eye Diagram at TP2

This test confirms that the sink device tolerates the differential MHL CLK jitter and eCBUS-D FWD data eye diagram impairments and differential TMDS data eye diagram impairments allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 4-3 and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 350mV

    - eCBUS-D differential swing: 400mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D: VTERM – 228mV

  - TMDS intra-pair skew, MHL CLK/eCBUS-D intra-pair skew

    - Positive skew: positive output leads negative output by 25ps for TMDS and by 50ps for MHLCLK/eCBUS-D.

    - Negative skew: negative output leads positive output by 25ps for TMDS and by 50ps for MHL CLK/eCBUS-D.

Make connections shown in Figure 4-4 and continue calibration of the test signal for the following condition.

  - Differential MHL CLK and eCBUS-D FWD data jitter

    - 7MHz jitter

      - Add 67ps pk-pk jitter of 7MHz jitter frequency to the MHL CLK and eCBUS-D FWD data.

      - Confirm that eCBUS-D FWD data eye diagram requirement is met. If not, increase the eCBUS FWD data swing until the eye diagram passes.

    - 10MHz jitter

      - Add 67ps pk-pk jitter of 10MHz jitter frequency to the MHL CLK and eCBUS-D FWD data.

      - Confirm that eCBUS-D FWD data eye diagram requirement is met. If not, increase the eCBUS FWD data swing until the eye diagram passes.

  - Differential TMDS data eye diagram

    - 1MHz jitter

      - With 7MHz jitter above in 2.i.1, add 1MHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 7MHz MHL CLK jitter and TMDS data swing voltage.

    - 500kHz jitter

      - With 10MHz jitter above in 2.i.2, add 500kHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 10MHz MHL CLK jitter and TMDS data swing voltage.

Tolerance Test

- Make connections shown in Figure 4-8.

Use Positive skew setting in 1.vii.1, 7MHz jitter in 2.i.1 and 1MHz jitter in 2.ii.1 of Test Signal Calibration described above.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test and eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count.

Change the calibration setting to Positive skew setting in 1.vii.1, 10MHz jitter in 2.i.2 and 500kHz jitter in 2.ii.2 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in 1.vii.2, 7MHz jitter in 2.i.1 and 1MHz jitter in 2.ii.1 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in 1.vii.2, 10MHz jitter in 2.i.2 and 500kHz jitter in 2.ii.2 of Test Signal Calibration described above

Repeat steps 2 to 10.

Swap the TMDS and eCBUS-D signals connection between the MHL3 Cable Emulator and MHL3 Signal Generator.

Repeat steps 1 to 16

Repeat steps 1 to 18 for all other supported TMDS data rates.

If the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count are zero in all cases, then PASS; else FAIL.

Jitter Tolerance of Single-Ended MHL CLK, eCBUS-S FWD Data and Differential TMDS Data: TCLOCK_JITTER_TP2_SE_MHL_CLK, TMDS Eye Diagram at TP2, eCBUS-S FWD Eye Diagram at TP2

This test confirms that the sink device tolerates the single-ended MHL CLK jitter and eCBUS-S FWD data eye diagram impairments and differential TMDS data eye diagram impairments allowed by the specification. This test is applied only to the DUT with eCBUS-S.

Test Signal Calibration

- Make connections shown in Figure 4-1 and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-S: 1000ps

  - Swing voltage:

    - TMDS differential swing: 350mV

    - eCBUS-S single-ended swing: 250mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-S: 155mV

  - TMDS intra-pair skew

    - Positive skew: positive output leads negative output by 25ps for TMDS.

    - Negative skew: negative output leads positive output by 25ps for TMDS.

Make connections shown in Figure 4-2 and continue calibration of the test signal for the following condition.

  - Single-Ended MHL CLK and eCBUS-S FWD data jitter

    - 7MHz jitter

      - Add 150ps pk-pk jitter of 7MHz jitter frequency to the MHL CLK and eCBUS-S FWD data.

      - Confirm that eCBUS-S FWD data eye diagram requirement is met. If not, increase the eCBUS-S FWD data swing until the eye diagram passes.

    - 10MHz jitter

      - Add 150ps pk-pk jitter of 10MHz jitter frequency to the MHL CLK and eCBUS-S FWD data.

      - Confirm that eCBUS-S FWD data eye diagram requirement is met. If not, increase the eCBUS-S FWD data swing until the eye diagram passes.

  - Differential TMDS data eye diagram

    - 1MHz jitter

      - With 7MHz jitter above in 2.i.1, add 1MHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 7MHz MHL CLK jitter and TMDS data swing voltage.

    - 500kHz jitter

      - With 10MHz jitter above in 2.i.2, add 500kHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 10MHz MHL CLK jitter and TMDS data swing voltage.

Tolerance Test

- Make connections shown in Figure 4-6.

Use Positive skew setting in 1.vii.1, 7MHz jitter in 2.i.1 and 1MHz jitter in 2.ii.1 of Test Signal Calibration described above.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test and eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count.

Change the calibration setting to Positive skew setting in 1.vii.1, 10MHz jitter in 2.i.2 and 500kHz jitter in 2.ii.2 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in 1.vii.2, 7MHz jitter in 2.i.1 and 1MHz jitter in 2.ii.1 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in 1.vii.2, 10MHz jitter in 2.i.2 and 500kHz jitter in 2.ii.2 of Test Signal Calibration described above

Repeat steps 2 to 10.

Swap the TMDS signal connection between the MHL3 Cable Emulator and MHL3 Signal Generator.

Repeat steps 1 to 16

Repeat steps 1 to 18 for all other supported TMDS data rates.

If the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count are zero in all cases, then PASS; else FAIL.

Single-Ended High Level Output Voltage of eCBUS-D BWD+/-: VSE_HIGH_DF_eCBUS_BWD

This test confirms that the single-ended high level voltage of eCBUS-D BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 4-11.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.4 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD waveform, measure the mean value (VSE_HIGH_DF_eCBUS_BWD) of the voltage histogram for middle portion of eCBUS-D data high period.

If VTERM – 200 mV <= VSE_HIGH_DF_eCBUS_BWD  <= VTERM + 20 mV, then PASS; else FAIL.

Single-Ended Low Level Output Voltage of eCBUS-D BWD +/-: VSE_LOW_DF_eCBUS_BWD

This test confirms that the single-ended low level voltage of eCBUS-D BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 4-24.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.4 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD waveform, measure the mean value (VSE_LOW_DF_eCBUS_BWD) of the voltage histogram for middle portion of eCBUS-D data low period.

If VTERM – 550 mV <= VSE_HIGH_DF_eCBUS_BWD  <= VTERM – 180 mV, then PASS; else FAIL.

Differential Output Swing Voltage of eCBUS-D BWD Data: VDF_SWING_DF_eCBUS_BWD

This test confirms that the differential swing voltage of eCBUS-D BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 4-11.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.4 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD Data waveform, measure the differential swing voltage (VDF_SWING_DF_eCBUS_BWD).

Obtain differential waveform using the scope’s math function.

Measure VDF_SWING_DF_eCBUS_BWD:

  - Obtain the mean values of the voltage histograms for the middle three TBIT_DF_eCBUS_BWD  periods of the 5 High and 5 Low bits of eCBUS-D BWD Data.

  - Obtain the differential swing voltage (VDF_SWING_DF_eCBUS_BWD) by subtracting the two mean values  in the previous step.

If 400mV <= VDF_SWING_DF_eCBUS_BWD <= 700mV, then PASS; else FAIL.

Single-Ended High Level Output Voltage of eCBUS-S BWD Data: VSE_HIGH_SE_eCBUS_BWD

This test confirms that the single-ended high level voltage of eCBUS-S BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 4-9.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.6 with eCBUS-S FWD swing 310mV on eCBUS-S_SG port. eCBUS-S_EC port duplicates the eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD waveform, measure the mean value (VSE_HIGH_SE_eCBUS_BWD) of the voltage histogram for middle portion of eCBUS-S data high period.

If 220 mV <= VSE_HIGH_SE_eCBUS_BWD  <= 400 mV, then PASS; else FAIL.

Single-Ended Low Level Output Voltage of eCBUS-S BWD Data: VSE_LOW_SE_eCBUS_BWD

This test confirms that the single-ended low level voltage of eCBUS-S BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 4-9.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.6 with eCBUS-S FWD swing 310mV on eCBUS-S_SG port. eCBUS-S_EC port duplicates the eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD waveform, measure the mean value (VSE_LOW_SE_eCBUS_BWD) of the voltage histogram for middle portion of eCBUS-S data low period.

If -30 mV <= VSE_LOW_SE_eCBUS_BWD  <= 30 mV, then PASS; else FAIL.

Single-Ended Output Swing Voltage of eCBUS-S BWD Data: VSE_SWING_SE_eCBUS_BWD

This test confirms that the single-ended output swing voltage of eCBUS-S BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Obtain VSE_SWING_SE_eCBUS_BWD by subtracting VSE_LOW_SE_eCBUS_BWD from VSE_HIGH_SE_eCBUS_BWD obtained in Sections 4.7.2.14 and 4.7.2.15.

If 250 mV <= VSE_SWING_SE_eCBUS_BWD <= 370 mV, then PASS; else FAIL.

Rise Time of eCBUS-D BWD Data: TR_DF_eCBUS_BWD

This test confirms that the rise time of eCBUS-D BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 4-11.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.4 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD Data waveform, obtain 20% - 80% Rise Time of eCBUS-D BWD Data (TR_DF_eCBUS_BWD) at the rising edge of the high period.

If 100 ps <= TR_DF_eCBUS_BWD <= 250 ps, then PASS; else FAIL.

Fall Time of eCBUS-D BWD Data: TF_DF_eCBUS_BWD

This test confirms that the fall time of eCBUS-D BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 4-11.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.4 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD Data waveform, obtain 20% - 80% Fall Time of eCBUS-D BWD Data (TF_DF_eCBUS_BWD) at the falling edge of the low period.

If 100 ps <= TF_DF_eCBUS_BWD <= 250 ps, then PASS; else FAIL.

Peak-Peak Amplitude of eCBUS-D BWD Data: VPP_TP2_DF_eCBUS_BWD

This test confirms that the Peak-Peak Amplitude of eCBUS-D BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 4-11.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.4 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD Data waveform of 500 microseconds long, measure the Peak-Peak Amplitude of eCBUS-D BWD Data (VPP_TP2_DF_eCBUS_BWD).

If VPP_TP2_DF_eCBUS_BWD <= 910 mV, then PASS; else FAIL.

Rise Time of eCBUS-S BWD Data: TR_SE_eCBUS_BWD

This test confirms that the rise time of eCBUS-S BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 4-9.

Set the MHL3 Signal Generator outputs as calibrated in 4.7.2.6 with eCBUS-S FWD swing 310mV on eCBUS-S_SG port. eCBUS-S_EC port duplicates the eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD Data waveform, obtain 20% - 80% Rise Time of eCBUS-S BWD Data (TR_SE_eCBUS_BWD).

  - Apply 1GHz Low Pass Filter to the eCBUS waveform before rise time test to remove noise on the waveform.

If 500 ps <= TR_SE_eCBUS_BWD <= 1000 ps, then PASS; else FAIL.

Fall Time of eCBUS-S BWD Data: TF_SE_eCBUS_BWD

This test confirms that the fall time of eCBUS-S BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 4-9.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.6 with eCBUS-S FWD swing 310mV on eCBUS-S_SG port. eCBUS-S_EC port duplicates the eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD Data waveform, obtain 20% - 80% Fall Time of eCBUS-S BWD Data (TF_SE_eCBUS_BWD).

  - Apply 1GHz Low Pass Filter to the eCBUS waveform before fall time test to remove noise on the waveform.

If 500 ps <= TF_SE_eCBUS_BWD <= 1000 ps, then PASS; else FAIL.

Peak-Peak Amplitude of eCBUS-S BWD Data: VPP_TP2_SE_eCBUS_BWD

This test confirms that the Peak-Peak Amplitude of eCBUS-S BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 4-9.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.6 with eCBUS-S FWD swing 310mV on eCBUS-S_SG port. eCBUS-S_EC port duplicates the eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD Data waveform of 500 microseconds long, measure the Peak-Peak Amplitude of eCBUS-S BWD Data (VPP_TP2_SE_eCBUS_BWD).

If VPP_TP2_SE_eCBUS_BWD <= 500 mV, then PASS; else FAIL.

eCBUS-D BWD Data Eye Diagram at TP1

This test confirms that the eCBUS-D BWD Data Eye Diagram at TP1 is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 4-12.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.9. Use Positive skew, 10MHz jitter for MHL CLK and eCBUS-D FWD Data, and 500kHz jitter for TMDS Data. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD Data waveform, measure the eye diagram.

  - Capture eCBUS-D BWD Data waveform of 500 microseconds long.

  - Apply the Worst Case Cable Emulator to the eCBUS-D FWD Data waveform.

  - Apply the MHL 2.1 Reference Cable Equalizer to the eCBUS-D FWD Data waveform.

  - Measure the eye diagram using the MHL clock from the MHL3 Signal Generator. Process the clock as described in Section 3.7.2.19.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.9. Use Positive skew, 7MHz jitter for MHL CLK and eCBUS-D FWD Data, and 1MHz jitter for TMDS Data.

Repeat steps 3-8.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.9. Use Negative skew, 10MHz jitter for MHL CLK and eCBUS-D FWD Data, and 500kHz jitter for TMDS Data.

Repeat steps 3-8.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.9. Use Negative skew, 7MHz jitter for MHL CLK and eCBUS-D FWD Data, and 1MHz jitter for TMDS Data.

Repeat steps 3-8.

If the eye diagrams in steps 8, 10, 12, and 14 pass eye mask test shown in Figure 16-21 in MHL 3.0 specification, then PASS; else FAIL.

eCBUS-S BWD Data Eye Diagram at TP1

This test confirms that the eCBUS-S BWD Data Eye Diagram at TP1 is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 4-10.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.10. Use Positive skew, 10MHz jitter for MHL CLK and eCBUS-S FWD Data, and 500kHz jitter for TMDS Data. eCBUS-S_EC port duplicates eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD Data waveform, measure the eye diagram.

  - Capture eCBUS-S BWD Data waveform of 500 microseconds long.

  - Apply the Worst Case Cable Emulator to the eCBUS-S BWD Data waveform.

  - Apply the Low Pass Filter defined MHL 3.0 Specification Section 16.3.1.4 after the Worst Case Cable Emulator.

  - Measure the eye diagram using the MHL clock from the MHL3 Signal Generator. Process the clock as described in Section 3.7.2.26.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.10. Use Positive skew, 7MHz jitter for MHL CLK and eCBUS-S FWD Data, and 1MHz jitter for TMDS Data.

Repeat steps 3 – 8.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.10. Use Negative skew, 10MHz jitter for MHL CLK and eCBUS-S FWD Data, and 500kHz jitter for TMDS Data.

Repeat steps 3 – 8.

Set the MHL3 Signal Generator outputs as calibrated in Section 4.7.2.10. Use Negative skew, 7MHz jitter for MHL CLK and eCBUS-S FWD Data, and 1MHz jitter for TMDS Data.

Repeat steps 3 – 8.

If the eye diagrams in steps 8, 10, 12 and 14 pass eye mask test shown in Figure 16-22 in MHL 3.0 specification, then PASS; else FAIL.

Differential Impedance of TMDS: ZD_THRU_TMDS_SNK, ZDT_ TMDS_SNK

This test confirms that the differential impedance of the sink device is within the range allowed by the specification.

Figure 4-15. Set reference point for differential mode

Figure 4-16. Direct Attach Sink – Differential Impedance Test Symbology

Before testing, compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables).

- Connect M3-S TPA SNK-P or M3-S TPA DirSNK-R for eCBUS-S or M3-D TPA SNK-P or M3-D TPA DirSNK-R for eCBUS-D to the TDR oscilloscope through a DC Block.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%).

Record the time at the peak of the change point in Figure 4-15. This is the start point of the Sink DUT input connector (TIN).

Calculate TTERM = TIN + CDF_SINK_DF_TERM_DISTANCE. TTERM is the sink termination point for differential signals. CDF_SINK_DF_TERM_DISTANCE is obtained from CDF of the Sink DUT.

Connect the equipment as shown in Figure 4-13 for eCBUS-S or in Figure 4-14 for eCBUS-D.

Turn on the DUT and enable MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Source board.

Set the DUT BIST Impedance_Mode to AV LINK RX using the CBUS Source.

Trigger the BIST.

If DUT is Direct Attach Sink, go to step 11. (Steps 11-18 are the same as the Dongle differential impedance measurement procedure in Section 5.7.2.20.)

Measure the differential impedance along the MHL differential signal path from TIN to TTERM + 1 ns.

  - The differential impedance from TIN to TTERM is ZDTHROUGH and the differential impedance from TTERM to TTERM + 1ns is ZDT.

  - If (85 ohms <= ZDTHROUGH <= 115 ohms) AND (90 ohms <= ZDT <= 110 ohms), then PASS. Else if (75 ohms <= ZDTHROUGH < 85 ohms) OR (115 ohms < ZDTHROUGH <= 125 ohms), the impedance is in the excursion range. If the excursion occurs single time AND the duration of the single excursion is less than 250ps AND (90 ohms <= ZDT <= 110 ohms), then PASS.

  - For all other cases, FAIL.

Calculate TCABLESTART and TCABLEEND. TCABLESTART and TCABLEEND are the start and end points of the cable portion in Direct Attach Sink for differential signals.

  - If (TTERM <= TIN + 6ns), TCABLESTART = TCABLEEND = TIN + (TTERM – TIN) / 2.

  - If (TTERM > TIN + 6ns), TCABLESTART = TIN + CDF_DirSINK_DF_CABLE_START_DISTANCE and TCABLEEND = TIN + CDF_DirSINK_DF_CABLE_END_DISTANCE . CDF_DirSINK_DF_CABLE_START_DISTANCE and CDF_DirSINK_DF_CABLE_END_DISTANCE are obtained from CDF of the Direct Attach Sink DUT.

Record the differential impedance increment ZDINCREMENT_A from TCABLESTART to TCABLEEND.

Calculate ZDINCREMENT_B.

  - If (ZDINCREMENT_A < 0), ZDINCREMENT_B = 0.

  - If (ZDINCREMENT_A >= 0), ZDINCREMENT_B = ZDINCREMENT_A.

Measure the differential impedance ZDTHROUGH1 from TIN to TCABLESTART and ZDTHROUGH2_WITHCABLE from TCABLEEND to TTERM.

Calculate ZDTHROUGH2 = ZDTHROUGH2_WITHCABLE – ZDINCREMENT_B.

Measure the differential impedance ZDT_WITHCABLE from TTERM to TTERM + 1ns.

Calculate ZDT = ZDT_WITHCABLE – ZDINCREMENT_B.

Evaluate ZDTHROUGH1, ZDTHROUGH2, and ZDT.

  - If (85 ohms <= [ZDTHROUGH1, ZDTHROUGH2] <= 115 ohms) AND (90 ohms <= ZDT <= 110 ohms), it is PASS

  - If (65 ohms <= [ZDTHROUGH1, ZDTHROUGH2] < 85 ohms) OR (115 ohms < [ZDTHROUGH1, ZDTHROUGH2] <= 125 ohms), the impedance is in the excursion range. If an excursion occurs no more than one time for  ZDTHROUGH1 and no more than one time for ZDTHROUGH2 AND the duration of each excursion is less than 350ps AND (90 ohms <= ZDT <= 110 ohms), it is PASS.

  - All other cases are FAIL.

If preceding steps pass, then PASS, else FAIL.

Differential Impedance of eCBUS-D: ZD_THRU_eCBUS_SNK, ZDT_eCBUS_SNK

This test confirms that the differential impedance of the sink device is within the range allowed by the specification.

Figure 4-17. Set reference point for differential mode

Figure 4-18. Direct Attach Sink – Differential Impedance Test Symbology

Before testing, compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables).

- Connect M3-D TPA SNK-P or M3-D TPA DirSNK-R to the TDR oscilloscope through a DC Block.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%).

Record the time at the peak of the change point in Figure 4-17. This is the start point of the Sink DUT input connector (TIN).

Calculate TTERM = TIN + CDF_SINK_DF_TERM_DISTANCE. TTERM is the sink termination point for differential signals. CDF_SINK_DF_TERM_DISTANCE is obtained from CDF of the Sink DUT.

Connect the equipment as shown in Figure 4-14.

Turn on the DUT and enable MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Source board.

If DUT is Direct Attach Sink, go to step 18. (Steps 18-36 are the same as the Dongle differential impedance measurement procedure in Section 5.7.2.21.)

Set the DUT BIST Impedance_Mode to eCBUS-D TX HIGH and eCBUS-D Rx using the CBUS Source.

Trigger the BIST

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Source to TDR Scope or VNA-base TDR.

  - Measure the differential impedance along the MHL differential signal path from TIN to TTERM + 1 ns.

The differential impedance from TIN to TTERM is ZDTHROUGH_HIGH and the differential impedance from TTERM to TTERM + 1ns is ZDT_HIGH.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from TDR Scope or VNA-base TDR to the CBUS Source.

Set the DUT BIST Impedance_Mode to eCBUS-D TX LOW and eCBUS-D Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Sink to TDR Scope or VNA-base TDR.

Measure the differential impedance along the MHL differential signal path from TIN to TTERM + 1 ns.

  - The differential impedance from TIN to TTERM is ZDTHROUGH_LOW and the differential impedance from TTERM to TTERM + 1ns is ZDT_LOW.

Calculate ZDTHROUGH and ZDT.

  - ZDTHROUGH= (ZDTHROUGH_HIGH+ ZDTHROUGH_LOW)/2

  - ZDT= (ZDT_HIGH+ ZDT_LOW)/2

Evaluate ZDTHROUGH and ZDT.

  - If (85 ohms <= ZDTHROUGH <= 115 ohms) AND (90 ohms <= ZDT <= 110 ohms), then PASS. Else if (75 ohms <= ZDTHROUGH < 85 ohms) OR (115 ohms < ZDTHROUGH <= 125 ohms), the impedance is in the excursion range. If the excursion occurs single time AND the duration of the single excursion is less than 250ps AND (90 ohms <= ZDT <= 110 ohms), then PASS.

  - For all other cases, FAIL.

  - Proceed to step #37.c.

Calculate TCABLESTART and TCABLEEND. TCABLESTART and TCABLEEND are the start and end points of the cable portion in Direct Attach Sink for differential signals.

  - If (TTERM <= TIN + 6ns), TCABLESTART = TCABLEEND = TIN + (TTERM – TIN) / 2.

  - If (TTERM > TIN + 6ns), TCABLESTART = TIN + CDF_DirSINK_DF_CABLE_START_DISTANCE and TCABLEEND = TIN + CDF_DirSINK_DF_CABLE_END_DISTANCE . CDF_DirSINK_DF_CABLE_START_DISTANCE and CDF_DirSINK_DF_CABLE_END_DISTANCE are obtained from CDF of the Direct Attach Sink DUT.

Record the differential impedance increment ZDINCREMENT_A from TCABLESTART to TCABLEEND.

Calculate ZDINCREMENT_B.

  - If (ZDINCREMENT_A < 0), ZDINCREMENT_B = 0.

  - If (ZDINCREMENT_A >= 0), ZDINCREMENT_B = ZDINCREMENT_A.

Set the DUT BIST Impedance_Mode to eCBUS-D TX HIGH and eCBUS-D Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the differential impedance ZDTHROUGH1_HIGH from TIN to TCABLESTART and ZDTHROUGH2_WITHCABLE_HIGH from TCABLEEND to TTERM.

Calculate ZDTHROUGH2_HIGH = ZDTHROUGH2_WITHCABLE_HIGH – ZDINCREMENT_B.

Measure the differential impedance ZDT_WITHCABLE_HIGH from TTERM to TTERM + 1ns.

Calculate ZDT_HIGH = ZDT_WITHCABLE_HIGH – ZDINCREMENT_B.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from TDR Scope or VNA-base TDR to the CBUS Source.

Set the DUT BIST Impedance_Mode to eCBUS-D TX LOW and eCBUS-D Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the differential impedance ZDTHROUGH1_LOW from TIN to TCABLESTART and ZDTHROUGH2_WITHCABLE_LOW from TCABLEEND to TTERM.

Calculate ZDTHROUGH2_LOW = ZDTHROUGH2_WITHCABLE_LOW – ZDINCREMENT_B.

Measure the differential impedance ZDT_WITHCABLE_LOW from TTERM to TTERM + 1ns.

Calculate ZDT_LOW = ZDT_WITHCABLE_LOW – ZDINCREMENT_B.

Calculate ZDTHROUGH and ZDT.

  - ZDTHROUGH= (ZDTHROUGH_HIGH+ ZDTHROUGH_LOW)/2

  - ZDT= (ZDT_HIGH+ ZDT_LOW)/2

Evaluate ZDTHROUGH1, ZDTHROUGH2, and ZDT.

  - If (85 ohms <= [ZDTHROUGH1, ZDTHROUGH2] <= 115 ohms) AND (90 ohms <= ZDT <= 110 ohms), it is PASS

  - If (65 ohms <= [ZDTHROUGH1, ZDTHROUGH2] < 85 ohms) OR (115 ohms < [ZDTHROUGH1, ZDTHROUGH2] <= 125 ohms), the impedance is in the excursion range. If an excursion occurs no more than one time for  ZDTHROUGH1 and no more than one time for ZDTHROUGH2 AND the duration of each excursion is less than 350ps AND (90 ohms <= ZDT <= 110 ohms), it is PASS.

  - All other cases are FAIL.

If preceding steps pass, then PASS, else FAIL.

Single-Ended Impedance of eCBUS-S: ZS_THRU_eCBUS_SNK, ZST_eCBUS_SNK

This test confirms that the single-ended impedance of the sink device is within the range allowed by the specification.

Figure 4-19. Set reference point for single-ended signal

Figure 4-20. Direct Attach Sink – Single-ended Impedance Test Symbology

- Connect M3-S TPA SNK-P or M3-S TPA DirSNK-R to the TDR oscilloscope through a DC block.

Set the effective rise time of the single-ended TDR pulse to 500 ps (20 – 80%).

Record the time at the peak of change point in Figure 4-19. This is the start point of the Sink DUT input connector (TIN).

Calculate TTERM = TIN + CDF_SINK_SE_TERM_DISTANCE. TTERM is the sink termination point for single-ended signals. CDF_SINK_SE_TERM_DISTANCE is obtained from CDF of the Sink DUT.

Connect the equipment as shown in Figure 4-13.

Turn on the DUT and enable the MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Source board.

If DUT is Direct Attach Sink, go to step 18. (Steps 18-36 are the same as the Dongle single-ended impedance measurement procedure in Section 5.7.2.22.)

Set the DUT BIST Impedance_Mode to eCBUS-S TX HIGH and eCBUS-S Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the single-ended impedance along the MHL single-ended signal path from TIN to TTERM + 3 ns.

  - The single-ended impedance from TIN to TTERM is ZSTHROUGH_HIGH and the single-ended impedance from TTERM to TTERM + 3ns is ZST_HIGH.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the TDR Scope or VNA-base TDR to CBUS Source.

Set the DUT BIST Impedance_Mode to eCBUS-S TX LOW and eCBUS-S Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the single-ended impedance along the MHL single-ended signal path from TIN to TTERM + 3 ns.

  - The single-ended impedance from TIN to TTERM is ZSTHROUGH_LOW and the single-ended impedance from TTERM to TTERM + 3ns is ZST_LOW.

Calculate ZSTHROUGH and ZST.

  - ZSTHROUGH= (ZSTHROUGH_HIGH+ ZSTHROUGH_LOW)/2

  - ZST= (ZST_HIGH+ ZST_LOW)/2

Evaluate ZSTHROUGH and ZST.

  - If (35 ohms <= ZSTHROUGH <= 57.5 ohms) AND (42.5 ohms <= ZST <= 57.5 ohms), then PASS. Else if (30 ohms <= ZSTHROUGH < 35 ohms) OR (57.5 ohms < ZSTHROUGH <= 62.5 ohms), the impedance is in the excursion range. If the excursion occurs single time AND the duration of the single excursion is less than 1000ps AND (42.5 ohms <= ZST <= 57.5 ohms), then PASS.

  - For all other cases, FAIL.

  - Proceed to Step #37.c.

For Direct Attach Sink, calculate TCABLESTART and TCABLEEND. TCABLESTART and TCABLEEND are the start and end points of the cable portion in Direct Attach Sink for single-ended signals.

  - If (TTERM <= TIN + 10ns), TCABLESTART = TCABLEEND = TIN + (TTERM – TIN) / 2.

  - If (TTERM > TIN + 10ns), TCABLESTART = TIN + CDF_DirSINK_SE_CABLE_START_DISTANCE and TCABLEEND = TIN + CDF_DirSINK_SE_CABLE_END_DISTANCE. CDF_DirSINK_SE_CABLE_START_DISTANCE and CDF_DirSINK_SE_CABLE_END_DISTANCE are obtained from CDF of the Direct Sink DUT.

Record the single-ended impedance increment ZSINCREMENT_A from TCABLESTART to TCABLEEND.

Calculate ZSINCREMENT_B.

  - If (ZSINCREMENT_A < 0), ZSINCREMENT_B = 0.

  - If (ZSINCREMENT_A >= 0), ZSINCREMENT_B = ZSINCREMENT_A.

Set the DUT BIST Impedance_Mode to eCBUS-S TX HIGH and eCBUS-S Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the single-ended impedance ZSTHROUGH1_HIGH from TIN to TCABLESTART and ZSTHROUGH2_WITHCABLE_HIGH from TCABLEEND to TTERM.

Calculate ZSTHROUGH2 = ZSTHROUGH2_WITHCABLE_HIGH – ZSINCREMENT_B.

Measure the single-ended impedance ZST_WITHCABLE_HIGH from TTERM to TTERM + 3 ns.

Calculate ZST_HIGH = ZST_WITHCABLE_HIGH – ZSINCREMENT_B.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from TDR Scope or VNA-base TDR to the CBUS Source.

Set the DUT BIST Impedance_Mode to eCBUS-S TX LOW and eCBUS-S Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the single-ended impedance ZSTHROUGH1_LOW from TIN to TCABLESTART and ZSTHROUGH2_WITHCABLE_LOW from TCABLEEND to TTERM.

Calculate ZSTHROUGH2 = ZSTHROUGH2_WITHCABLE_LOW – ZSINCREMENT_B.

Measure the single-ended impedance ZST_WITHCABLE_LOW from TTERM to TTERM + 3 ns.

Calculate ZST_LOW = ZST_WITHCABLE_LOW – ZSINCREMENT_B.

Calculate ZSTHROUGH and ZST.

  - ZSTHROUGH= (ZSTHROUGH_HIGH+ ZSTHROUGH_LOW)/2

  - ZST= (ZST_HIGH+ ZST_LOW)/2

Evaluate ZSTHROUGH1, ZSTHROUGH2, and ZST.

  - If (35 ohms <= [ZSTHROUGH1, ZSTHROUGH2 ]<= 57.5 ohms) AND (42.5 ohms <= ZST <= 57.5 ohms), it is PASS

  - If (30 ohms <= [ZSTHROUGH1, ZSTHROUGH2] < 35 ohms) OR (57.5 ohms < [ZSTHROUGH1, ZSTHROUGH2] <= 62.5 ohms), the impedance is in the excursion range. If an excursion occurs no more than one time for  ZSTHROUGH1 and no more than one time for ZSTHROUGH2 AND the duration of each excursion is less than 1000ps AND (42.5 ohms <= ZST <= 57.5 ohms), it is PASS.

  - All other cases are FAIL.

If preceding steps pass, then PASS, else FAIL.

## MHL 3 System Tests

This section defines tests unique to MHL 3 devices connected in eCBUS-S or eCBUS-D mode, using AVLink or CBUS to transfer data in protocols not supported by MHL1 / MHL2 devices.

All MHL 3 devices have a legacy MHL1/ MHL2 compatibility mode test requirement as defined in Section 4.2.  Refer to MHL CTS 2 test procedures for the specific test steps.  New tests for MHL 3 are defined in this  section.  The general Required Methodology for all Sink DUT tests with normative references to CTS 2.1 are tested in oCBUS mode, in MHL 3 mode;  eCBUS-S and, if supported by the DUT, eCBUS-D. Test results for each mode sequence will be recorded TESTxxxx- S for eCBUS-S mode, and TESTxxxx-D for eCBUS-D mode (if supported).  If eCBUS-D is not supported, then the result for this test sequence will be PASS (SKIP).  The general Required Methodology are defined below.

General Required Methodology

- Connect DUT to test MHL 3 Source test equipment using MHL 3 cable in eCBUS-S mode.  Execute tests listed in Section 4.8.

Execute TESTxxxx and record results as PASS or FAIL for TESTxxxx-S.

Connect DUT to test MHL 3 Sink test equipment using MHL 3 cable in eCBUS-D mode if supported.  Repeat execution of tests in Section 4.8.

If eCBUS-D mode is not supported, result is PASS (SKIP).  If eCBUS-D mode is supported,  execute TESTxxxx and record results as PASS or FAIL for TESTxxxx-D.

Connect DUT to test MHL 2 Source test equipment using MHL 2 cable in oCBUS mode.  Execute tests in Section 4.2 of MHL1/ MHL 2 CTS.

Execute TESTxxxx and record results as PASS or FAIL for TESTxxxx.

When required, tester must follow proper WRITE_BURST protocol including but not limit to having correct BURST_IDs, WRT_REQ, GRT_WRT handshaking, and their timings in both MHL3 and MHL1 /MHL2 tests.

If  all test sequences pass, overall test result is PASS, else FAIL.

Table 4-7. Sink AVLINK Test Sequence

| Test ID | MHL3 Mode eCBUS-S | MHL3 Mode eCBUS-D |
| --- | --- | --- |
| 4.8.1.1 | X | If DUT Supports |
| 4.8.2.1 | X | If DUT Supports |
| 4.8.2.2 | X | If DUT Supports |
| 4.8.2.3 | X | If DUT Supports |
| 4.8.3.1 | X | If DUT Supports |
| 4.8.3.2 | X | If DUT Supports |
| 4.8.3.3 | X | If DUT Supports |
| 4.8.3.4 | X | If DUT Supports |
| 4.8.3.5 | X | If DUT Supports |
| 4.8.3.6 | X | If DUT Supports |
| 4.8.3.7 | X | If DUT Supports |
| 4.8.4.1 | X | If DUT Supports |
| 4.8.5.1 | X | If DUT Supports |
| 4.8.5.2 | X | If DUT Supports |
| 4.8.6.1 | X | If DUT Supports |
| 4.8.7.1 | X | If DUT Supports |
| 4.8.7.2 | X | If DUT Supports |
| 4.8.7.3 | X | If DUT Supports |
| 4.8.7.4 | X | If DUT Supports |
| 4.8.7.5 | X | If DUT Supports |
| 4.8.7.6 | X | If DUT Supports |
| 4.8.7.7 | X | If DUT Supports |
| 4.8.7.8 | X | If DUT Supports |
| 4.8.8.1 | X | If DUT Supports |
| 4.8.8.2 | X | If DUT Supports |
| 4.8.9.1 | X | If DUT Supports |
| 4.8.10.1 | X | If DUT Supports |

RX BIST

AVLink RX BIST Test

Confirm that the Sink DUT supports required RX BIST and check functionality.

- Connect DUT to MHL AVLINK Generator and initiate discovery.

Source Tester to issue BIST_SETUP Command with the following values:

  - AV Link_DATA_RATE: Use the highest AV Link data rate supported by Sink DUT.

  - AV Link Pattern : Fixed8.

  - AV Link_VIDEO MODE:  using the highest video mode supported by Sink DUT that is indicated in the CDF.

  - AV Link Duration = 0x01  (Minimum # of Frames/ Time).

  - AV Link Fixed H  8’b10000010  (for bitwire inverse for Fixed10 pattern).

  - AV Link Fixed L:

    - 8’b11110101 (for Fixed10 pattern)

    - 8'b00000000 (for Fixed8 pattern)

  - AV Link Randomizer

    - 0x01 (for Fixed8 pattern)

    - 0x00 (otherwise)

  - All other parameters are programed to 0x00 for this command (offset: 0x03 to 0x07, and 0x0F).

Fail if BIST_READY command is not received by Source Tester before TBIST_SETUP[MAX).

Source Tester to examine the received BIST_READY command.

If AV LINK ERROR bit is equal to 1 then Pass[SKIP]  (Tester Error).

Fail if AV LINK Ready bit is not 1.

Source Tester to send BIST_TRIGGER command with only AV LINK RX BIST enabled (other bits = 0).

Source Tester to send 33 Frames  or 1.1 second of Video data (depending on BIST pattern).

Source Tester to issue BIST_REQUEST_STAT command.

Source Testet to examine BIST_RETURN_STAT data.  Fail if AV_LINK_STAT_H and AV_LINK_STAT_L are not equal to 0x00.

Continue step 2 with Fixed10 pattern, and with PRBS pattern if CDF_AVLINK_BIST_PRBS_SUPP field is YES.

If BIST patterns Pass, then PASS, else FAIL.

Video Test

Additional Video Formats in 24-bits-per-pixel Mode

Verify that sink DUT supports the video formats with no distortion.

Perform tests using video modes specified in CDF. Use Ramp pattern.

- If any of the following video formats are supported CDF_VIDEO_1080p_50,  CDF_VIDEO_1080p_60, CDF_VIDEO_1080p_48w, CDF_VIDEO_1080p_50w, CDF_VIDEO_1080p_24w, CDF_VIDEO_1080p_25w,  continue with test, else PASS(SKIP).

Configure MHL Signal Generator to send out test patterns in one of the video formats in the CDF.

If DUT displays the patterns with no distortion, then continue to test, else FAIL.

Repeat the test from step 2 for all video formats indicated in the CDF.

If all of the video modes listed in the CDF pass, then PASS, else FAIL.

Additional Video Formats in 16-bits-per-pixel Mode

Verify that sink DUT supports the video formats with no distortion.

Perform tests using video modes specified in CDF. Use Ramp pattern.

- If any of the following formats are supported,  CDF_VIDEO_16BIT_1080p_100, CDF_VIDEO_16BIT_1080p_120,  CDF_VIDEO_16BIT_2160p_30, CDF_VIDEO_16BIT_2160p_25 CDF_VIDEO_16BIT_2160p_24, CDF_VIDEO_16BIT_2160p_25w,  CDF_VIDEO_16BIT_2160p_24w,  CDF_VIDEO_16BIT_1080p_96w ,  continue with test, else PASS(SKIP).

Configure MHL Signal Generator to send out test patterns in one of the video formats in the CDF.

If DUT displays the patterns with no distortion, then continue to test, else FAIL.

Repeat the test from step 2for all video formats indicated in the CDF.

If all of the 16-bit video modes listed in the CDF pass, then PASS, else FAIL.

High End  Video Formats

Verify that sink DUT supports the  minimum set of High-End video formats with no distortion.

Table 4-8. High-End Video Format Support Requirement

| Index | Mode | Refresh |
| --- | --- | --- |
| 93, 103 | 3840x2160 | 24 |
| 94, 104 | 3840x2160 | 25 |
| 95, 105 | 3840x2160 | 30 |

- {max}{max}If CDF_PRODUCT_HEV_CAPABLE is NO then PASS(SKIP).

If CDF_VIDEO_HEV is NO thenFAIL.  Else, continue to test.

Tester to read Sink DUT’s EDID.

Tester to send SET_INT(FEAT_REQ).

If WRITE_BURST from Sink is not received within TGRT_BURST{max} then FAIL.

If SET_INT(FEAT_COMPLETE) from Sink is not received within TSINK_FEAT_COMPL{max} then FAIL.

Tester to compare Sink DUT’s EDID and Sink DUT WRITE_BURST data for HEV support modes.

FAIL if both EDID and WRITE_BURST data have identical HEV mode support information (e.g. EDID and WRITE_BURST both have VIC value of 95 for 2160p30).

Compare list of supported modes (either in EDID or in the WRITE_BURST data) to CDF. If there is a mismatch, then FAIL. Or if the list does not have all the modes as indicated in table 4-8 then FAIL.

In sequential order of  HEVD, configure test equipment to send out the corresponding video mode that are in table 4-8, High-End Format Support Requirement.  If DUT displays the video pattern with no distortion, then continue to test, else FAIL.

Repeat the test from step 9 for all video formats indicated.

If all video formats in in the list pass, then PASS, else FAIL.

Audio Test

IEC 60958 / IEC61937 Support

Verify that Sink supports the Audio formats and reproduces audio properly.

- If one of the CDF_AUDIO_2CH_32kHz,  CDF_AUDIO_2CH_44kHz,  CDF_AUDIO_2CH_48kHz fields in the CDF is set to NO, then FAIL, else continue.

Configure the MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and one of audio format such as PCM_2CH_192kHz, as indicated in the CDF.

Configure the MHL Signal Generator to output 1 kHz sine wave.

If DUT regenerates the 1 kHz sound with no distortion, then continue the test, else FAIL.

Repeat the test from step 2 for other sampling frequency that is indicated in the CDF as in step 1.

If any of the Audio formats fails the test, then FAIL, else PASS.

Audio Clock Regeneration Support

Verify that Sink properly regenerates audio when different Audio Clock Regeneration parameters are used.

- If one of the CDF_AUDIO_2CH_32kHz,  CDF_AUDIO_2CH_44kHz,  CDF_AUDIO_2CH_48kHz fields in the CDF is set to NO, then FAIL, else continue.

Configure the MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and one of audio format such as PCM_2CH_192kHz, as indicated in the CDF.

Configure MHL Signal Generator to send out the test patterns with 1kHz sine wave using minimum N parameter.

If DUT reproduces 1kHz sine wave with no distortion, then continue to test, else FAIL.

Configure MHL Signal Generator to send out test patterns with 1kHz sine wave using maximum N parameter.

If DUT reproduces 1kHz since wave with no distortion, then continue to test, else FAIL.

Repeat the test from step 2 for other sampling frequency that is indicated in the CDF as in step 1.

If any of the Audio-mode fails the test, then FAIL, else PASS.

IEC 60958 / IEC61937 While in 16-bits-per-pixel Mode

Verify that Sink supports the required Audio formats and reproduces audio properly while in 16-bits-per-pixel mode.

- If CDF_VIDEO_16BIT field in the CDF indicates no support for 16-bits-per-pixel mode, then end test with PASS (SKIP).

If any of CDF_AUDIO_2CH_32kHz, CDF_AUDIO_2CH_44kHz and CDF_AUDIO_2CH_48kHz field in the CDF is NO, and if the DUT has any other audio input, then FAIL, else continue to test.

Configure the MHL Signal Generator to output one of the 16-bits-per-pixel Video formats and one of PCM_2CH_32kHz, PCM_2CH_44.1kHz, PCM_2CH_48kHz and any as indicated in the CDF.

Configure the MHL Signal Generator to output 1 kHz sine wave.

If DUT regenerates the 1 kHz sound with no distortion, then continue the test, else FAIL.

Repeat the test from step 5 for PCM_2CH_32kHz, PCM_2CH_44.1kH, PCM_2CH_48kHz and other audio sampling frequency if it is indicated in the CDF.

If any of the Audio formats fails the test, then FAIL, else PASS.

Audio Clock Regeneration While in 16-bits-per-pixel Mode

Verify that Sink properly regenerates audio when different Audio Clock Regeneration parameters are used while in 16-bits-per-pixel mode.

- If CDF_VIDEO_16BIT field in the CDF indicates no support for 16-bits-per-pixel mode then end test with PASS (SKIP).

If any of CDF_AUDIO_2CH_32kHz, CDF_AUDIO_2CH_44kHz and CDF_AUDIO_2CH_48kHz field in the CDF is YES, and if the DUT has any other audio input then FAIL, else continue to test.

Configure MHL Signal Generator to output one of the 16-bits-per-pixel Video formats and one of PCM_2CH_32kHz, PCM_2CH_44.1kHz, PCM_2CH_48kHz and any as indicated in the CDF.

Configure MHL Signal Generator to send out the test patterns with 1kHz sine wave using minimum N parameter.

  - If DUT reproduces 1kHz sine wave with no distortion, then continue to test, else FAIL.

Configure MHL Signal Generator to send out test patterns with 1kHz sine wave using maximum N parameter.

  - If DUT reproduces 1kHz since wave with no distortion, then continue to test, else FAIL.

Repeat the test from step 3 for PCM_2CH_32kHz, PCM_2CH_44.1kHz, PCM_2CH_48kHz and other as indicated in the CDF.

If any of the Audio-mode fails the test, then FAIL, else PASS.

High-Bitrate Audio

Verify that Sink supports the High-Bitrate Audio formats and reproduces audio properly.

- If CDF_AUDIO_HBR field in the CDF is NO then end test with PASS (SKIP).

Configure the MHL Signal Generator to output a Video format, such as 720p60, that carries HBR signal such as Dolby TrueHD that Sink DUT can support.

Perform listening test

If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

Repeat the test from step 2 for other HBR format if it is indicated in the CDF.

If any of the Audio formats fails the test, then FAIL, else PASS.

High-Bitrate in 16-bits-per-pixel

Verify that Sink supports the High-Bitrate Audio formats and reproduces audio properly while in 16-bits-per-pixel Mode.

- If CDF_VIDEO_16BIT in the CDF is NO then end test with PASS (SKIP).

If CDF_AUDIO_2CH_32kHz fields in the CDF is NO then end test with PASS (SKIP).

Configure the MHL Signal Generator to output a 16-bits-per-pixel mode Video format,  that carries HBR signal such as Dolby TrueHD Sink DUT can support.

Perform listening test.

If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

Repeat the test from step 3 for other HBR format if it is indicated in the CDF.

If any of the Audio formats fails the test, then FAIL, else PASS.

Audio Data Tunneling Receiving Support

Verify that Sink DUT is capable of receiving Audio via  Audio data tunneling.

- If the CDF_ADT_RECEIVE_SUPPORT  is set to YES, then continue else end test with PASS (SKIP).

Tester Source sends SET_INT(FEAT_REQ) to Sink DUT.

Tester Source allows Sink DUT to execute WRITE_BURST command by responding to REQ_WRT interrupt with GRT_WRT interrupt.

If WRITE_BURST command from Sink DUT is not arriving within TGRT_BURST(max) then FAIL.

Examine the WRITE_BURST data.  If basic audio (32khz, 44.1khz and 48khz) is not supported then FAIL.

Tester to reject (by responding with 0x01,0x02,0x03) any attempt to do TDM Allocation by DUT.

Tester uses section 13.10 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation that has Feature_ID=0x02 and has enough bandwidth to carry ADT data as indicated in the CDF.

FAIL if DUT does not respond by indicating 0x00 (Accepted).

Configure Source Tester to output basic ADT Audio ( 32khz, 44.1khz and 48khz) formats, plus the corresponding audio InfoFrame packet, audio regeneration packet.  Note that Audio InfoFrame and Audio Clock Regeneration packets transmission shall be sent at least once each TAIF0.

If CDF_ADT_PLAYING_SUPPORT is YES, perform listening test based on CDF_ADT_RECEIVE_PROC, else skip to step 12.

If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

If any of Audio formats fails, then FAIL, else PASS.

Audio Data Tunneling Transmitting Support

Verify that Sink is capable of Transmitting Audio via  Audio data tunneling.

- If CDF_ADT_SEND_SUPPORT is set to YES, then continue else end test with PASS (SKIP).

- Tester to wait for SET_INT(FEAT_REQ) from Sink DUT.  Fail if this command is not sent within TSRC_FEAT_REQ{max}.

- Tester , using WRITE_BURST, to inform DUT of the support of basic audio(32khz, 44.1khz and 48khz) and any audio format in the CDF.

- Tester to reject (by responding with 0x01,0x02,0x03) any attempt to do TDM Allocation by DUT.

- Tester uses section 13.10 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation that has Feature_ID=0x02 and has enough bandwidth to carry Audio data as indicated in the CDF.

- FAIL if DUT does not respond by indicating 0x00 (Accepted).

- Configure Tester to enable its TMDS (such as sending any video format).

- Configure DUT to send out all Basic ADT Audio ( 32khz, 44.1khz and 48khz) formats, based on CDF_ADT_SEND_PROC.

- If the Audio InfoFrame and VSIF are not coming from the DUT at least once every 2 fields, then FAIL.

- Examine the Audio InfoFrame and VSIF data (including but not limit to Channel Status, Channel Count, AV_DELAY_SYNC…). If they do not match the Audio mode being sent, then FAIL.

- If Audio Regeneration packet is also sent by the DUT, verify the N and CTS value and their relationship with the link clock of the current audio format according to section 14.2.1 (Audio Data Tunneling and Audio Clock Regeneration).  FAIL if the relationship and the restriction are incorrect.

- If CDF_ADT_PLAYING_SUPPORT is YES, perform listening test, else skip to step 14.

- If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

- Configure Tester to disable TMDS output.  Then repeat the test from Step 8.

- If all ADT formats specified in the CDF pass, then PASS, else FAIL.

HDCP2.2 Test

HDCP2.2

Verify that Sink complies with HDCP2.2 Specification.

- If the CDF_HDCP_SUPPORT field in CDF is YES, then continue to test, else end test with PASS (SKIP).

Follow the HDCP2.2-ON-MHL CTS procedure. If the CDF_HDCP_REPEATER field in the CDF is YES, then include all HDCP Repeater tests.

If all tests in the HDCP2.2-ON-MHL CTS procedure pass, then PASS; else FAIL.

*Note: HDCP test tool must support clock stretch.

RBP Sub-Command Tests

The test implementation shall include UTF character sequences for all lengths of characters (1 to 4 8-bit bytes or octets), with both invalid and valid values in each of those lengths.

RBP Sub-Commands Receiving Test

Verify that DUT responds to valid RBP sub-commands.

- If CDF indicates CDF_RBP_RECV_SUPPORT is YES then continue test, else PASS (SKIP).

Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence

Determine supported button codes from CDF and test each for reception of proper code and that RBPK is received after each command from the DUT.

Tester asks operator to confirm the operation was performed in response to the code received by the DUT.

Repeat step 3 for each code identified in the CDF.

From the CDF determine if any unsupported codes exist and send that code to force and RBPE opcode 0x01 (ineffective code)

If DUT passes all iterations of the test, then PASS. Else FAIL.

RBP Sub-Commands Transmitting Test

Verify that DUT sends valid RBP sub-commands.

- If CDF indicates CDF_RBP_SEND_SUPPORT is YES then continue test, else PASS (SKIP).

Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence

Determine supported button codes from CDF and test each for transmission and correct reception of the code by the test equipment and reception of  RBPK is received after each command from the DUT.

Repeat step 3 for each code identified in the CDF

From the CDF list of supported codes, the tester picks one to not support so that an unsupported code condition exists and tester responds with RBPE opcode 0x01 (ineffective code).  Ask test operator to confirm the DUT exhibits the proper error handling such as displying an error message.

If DUT passes all iterations of the test, then PASS. Else FAIL.

ATT Subcommand Test

ATT Subcommand Test

Verify functionality and support of ATT MSC subcommand used in Multi-View support.

- Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence.

Tester to send SET_INT{FEAT_REQ}.

FAIL if WRITE_BURST from Sink does not arrive within TGRT_BURST{max}.

FAIL if SET_INT{FEAT_COMPLETE} does not arrive within TSINK_FEAT_COMPL{max}.

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT. It is necessary for the tester to record in a variable the support status for Multi-View indicated by the DUT in the VDI field of its WRITE_BURSTs of HEV video modes. If any VDI field indicates support for Multi-View, then the ATT command shall be accurately supported. If no VDI field indicates support for Multi-View, then the ATT command shall not be sent to the DUT by a connected Source. (Refer to MHL 3 Specification, Sections 5.11, and 7.9.1.)

  - If DUT's VDI_L[5:0] are not all '0', then DUT is indicating some supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to YES.

  - If DUT's VDI_L[5:0] are all '0', then DUT is indicating no supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to NO.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and MEAS_VIDEO_MULTIVIEW is NO; then PASS (SKIP), else continue.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is YES, and MEAS_VIDEO_MULTIVIEW is YES, then continue, else FAIL.

Invoke DUT to send ATT command.

If DUT ATT command is successful,  test equipment shall respond  with ATTK.

If  TATT_WAIT{max} elapses and no further commands are sent then FAIL, else continue.

If transaction completes without error then continue, else FAIL.

Tester to send ATT command.

FAIL if DUT does not respond with ATTK before TATT_WAIT{max} elapses.

If transaction completes without error then PASS, else FAIL.

AVLINK Packetization Tests

The AVLINK packetization tests are specific to MHL3 and are defined to verify low level AVLINK link compliance.  These tests are not defined for legacy MHL compatibility mode.

AVLINK Speed Test

Verify functionality and link speeds supported by sink DUT.

- Repeat the following steps for each speed advertised in the CDF in the variables CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600 as YES. At least one must be supported. If any other speed is not supported as indicated in the CDF, that iteration will receive a PASS(SKIP).

- The Tester advertises each supported CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600  in its TMDS_SPEEDS register in numerical order starting with TMDS_150.

- The Sink DUT is connected to the MHL 3 Tester.

- Verify that Sink DUT TMDS_SPEEDS register matches the CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600 values.

- If the TMDS_SPEEDS register is not correct, then FAIL.

- The Tester writes the Sink LINK_RATE register to indicate the speed being tested.

- Check that the Sink DUT writes to the Tester Source AVLINK_MODE_STATUS register indicating TMDS_NORMAL to verify that Sink DUT has connected at the correct speed.

- If the DUT does not indicate TMDS_NORMAL within 10 seconds, then end the test with FAIL.

- If DUT passes all iterations of the test, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; Short Packets

Verify functionality of the sink DUT AVLINK when it receives many small AV TMDS packets.

- The Tester Source connects to the Sink DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized into as many 1-byte-payload packets as possible, while respecting the packet jitter and buffer model rules relating to NACC_SYMBOL_DIFF{max}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; Long Packets

Verify functionality of the sink DUT AVLINK when it receives maximum sized AV TMDS packets.

- The Tester Source connects to the Sink DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized into as many 252-byte-payload packets as possible, while respecting the packet jitter and buffer model rules relating to NACC_SYMBOL_DIFF{max}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; Randomizer Reset Alignment

Verify functionality of the sink DUT AVLINK when it receives RR sequences in or around AV packets.

- The Tester Source connects to the Sink DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized so that all locations for groups of 4 RR’s in Table 4-9 are inserted at the indicated location once every TRR{min}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

Table 4-9. RR Insertion Alignment

| Feature | RR Group Location |
| --- | --- |
| 1 | Between SOP and Header 0 |
| 2 | Between Header 0 and Header 1 |
| 3 | Between Header 1 and Header 2 |
| 4 | Between Header 2 and Data 0 |
| 5 | Between Data 0 and Data 1 |
| 6 | Between the last Data and EOP |
| 7 | Immediately before SOP |
| 8 | Immediately after EOP |
| 9 | Between a pair of GAP symbols |

AVLINK Packetization Basic Protocol; Slow Randomizer Resets

Verify functionality of the sink DUT AVLINK when it receives RR sequences with maximum spacing.

- The Tester Source connects to the Sink DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized so that a group of 4 RR’s are inserted once every TRR{max}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; Buffer Model Timing

Verify functionality of the Sink DUT when AVLINK Data is delayed.

- The Tester Source connects to the Sink DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized to control data jitter.

    - During each Vertical Retrace and Horizontal Retrace interval, each AV packet is sent starting when its last data item becomes available.

    - The first 252 bytes of data from the beginning of each Video Data period are sent in an AV packet starting when its last data item becomes available.

    - The data from the middle of each Video Data period are sent as AV packets.  GAP symbols are added so that each AV packet is started at the time its last data item becomes available PLUS an additional delay of 256 Video Content Symbol times, consistent with the packet jitter and buffer model rules relating to NACC_SYMBOL_DIFF{max}.

    - The final 253 to 504 bytes of data from the end of each Video Data period are sent as 2 AV packets starting when their last data items become available.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Extended Content Protocol

Verify functionality when Extended Content packets are received by the sink DUT.

- If CDF_EXTENDED_CONTENT_SUPPORT is NO, then end test with PASS (SKIP).

- The Tester Source connects to the Sink DUT using the TMDS Link speed needed to force Extended Content Packetization.

- Command Tester Source to send AV data with the format specified in CDF_EXTENDED_CONTENT_SUPPORT.

  - The first 4 symbols sent must be RR’s.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Extended Content Protocol; Buffer Model Timing

Verify functionality of the Sink DUT when Extended Content AVLINK Data is delayed.

- If CDF_EXTENDED_CONTENT_SUPPORT is NO, then end test with PASS (SKIP).

- The Tester Source connects to the Sink DUT using the TMDS Link speed needed to force Extended Content Packetization.

- Command Tester Source to send AV data with the format specified in CDF_EXTENDED_CONTENT_SUPPORT.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized to control data jitter.

    - During each Vertical Retrace and Horizontal Retrace interval, each AV packet is sent starting when its last data item becomes available.

    - Starting from line 0, even numbered Scan Lines are sent as Extended Content packets starting when enough data is available to let them complete.

    - Starting from line 1, odd numbered Scan Lines are sent as Extended Content packets starting when enough data is available to let them complete PLUS an additional delay of 256 Video Content Symbol times.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

3D, Multi-View and Dual-3D

Table 4-10. 3D and Multi-View Testing Configurations

| Configuration | VDI_L Settings |  |  | Description (3D/Multiview mode support) |
| --- | --- | --- | --- | --- |
|  | VDI_L[6] | VDI_L[5:3] | VDI_L[2:0] |  |
| 1 | 0 | 0 | 0 | No Multi-View or 3D support |
| 2 | 0 | 0 | Non-zero | 3D |
| 3 | 0 | Non-zero | 0 | 4-View |
| 4 | 0 | Non-zero | Non-zero | 4-View, 2-View, 3D |
| 5 | 1 | 0 | 0 | None (invalid setting) |
| 6 | 1 | 0 | Non-zero | 2-View, 3D |
| 7 | 1 | Non-zero | 0 | 4-View, Dual3D |
| 8 | 1 | Non-zero | Non-zero | 4-View, 2-View, Dual3D, 3D |
| - Note1: VDI_L[7] = 1 to indicate separate audio for multiview. |  |  |  |  |
| - Note2: VDI_L[2:0] is for LR,TB,FS_SUPP respectively. |  |  |  |  |
| - Note3: VDI_L[5:3] is for TBLR,FSLR, FSTB respectively. |  |  |  |  |

3D Support

Verify functionality and support of 3D and Multi-View devices.

- Connect DUT and perform Discovery

Tester to send SET_INT(FEAT_REQ)

FAIL if WRITE_BURST from Sink does not arrive within TGRT_BURST{max}.

FAIL if SET_INT(FEAT_COMPLETE) does not arrive within TSINK_FEAT_COMPL{max}.

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT. It is necessary for the tester to record in a variable the support status for Multi-View indicated by the DUT in the VDI field of its WRITE_BURSTs of HEV video modes. If any VDI field indicates support for Multi-View, then the ATT command shall be accurately supported. If no VDI field indicates support for Multi-View, then the ATT command shall not be sent to the DUT by a connected Source. (Refer to MHL 3 Specification, Sections 5.11, and 7.9.1.)

  - If DUT's VDI_L[5:0] are not all '0', then DUT is indicating some supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to YES.

  - If DUT's VDI_L[5:0] are all '0', then DUT is indicating no supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to NO.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and MEAS_VIDEO_MULTIVIEW is NO; then PASS (SKIP), else continue.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is YES, and MEAS_VIDEO_MULTIVIEW is YES, then continue, else FAIL.

If the DUT’s VDI_L[2:0]  are NOT all ‘0’ and each bit matches its corresponding CDF field as below then continue to test, else FAIL.

  - VDI_L[0]=’1’ if CDF_VIDEO_FS_SUPP field is not empty, else = ‘0’.

  - VDI_L[1]=’1’ if CDF_VIDEO_TB_SUPP field is not empty, else = ‘0’.

  - VDI_L[2]=’1’ if CDF_VIDEO_LR_SUPP field is not empty, else = ‘0’.

In the sequential order of the descriptors, Tester to send out video modes as indicated in MHL spec Table 5-10 (MHL Sink 3D Video Mode Support Requirements), plus any additional modes indicated in the CDF with correct AVI, VSIF (MHL_VID_FMT, MHL_3D_FMT_TYPE…). Operator to verify that video on each downstream sink device is displayed properly.

Repeat test from step 9 for each resolution listed.

If DUT passes all iterations of the test, then PASS, else FAIL.

Multi-View Support

Verify Multi-View functionality and support of Sink DUT.

- Connect DUT and perform Discovery.

Tester to send SET_INT(FEAT_REQ).

FAIL if WRITE_BURST from Sink does not arrive within TGRT_BURST(max).

FAIL if SET_INT(FEAT_COMPLETE) does not arrive within TSINK_FEAT_COMPL(max).

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT. It is necessary for the tester to record in a variable the support status for Multi-View indicated by the DUT in the VDI field of its WRITE_BURSTs of HEV video modes. If any VDI field indicates support for Multi-View, then the ATT command shall be accurately supported. If no VDI field indicates support for Multi-View, then the ATT command shall not be sent to the DUT by a connected Source. (Refer to MHL 3 Specification, Sections 5.11, and 7.9.1.)

  - If DUT's VDI_L[5:0] are not all '0', then DUT is indicating some supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to YES.

  - If DUT's VDI_L[5:0] are all '0', then DUT is indicating no supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to NO.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and MEAS_VIDEO_MULTIVIEW is NO; then PASS (SKIP), else continue.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is YES, and MEAS_VIDEO_MULTIVIEW is YES, then continue, else FAIL.

If the DUT’s VDI_L[5:0] are NOT all ‘0’ and each bit matches its corresponding CDF field as below then continue to test, else FAIL.

  - VDI_L[0]=’1’ if CDF_VIDEO_FS_SUPP field is not empty, else = ‘0’.

  - VDI_L[1]=’1’ if CDF_VIDEO_TB_SUPP field is not empty, else = ‘0’.

  - VDI_L[2]=’1’ if CDF_VIDEO_LR_SUPP field is not empty, else = ‘0’.

  - VDI_L[3]=’1’ if CDF_VIDEO_FSTB_SUPP field is not empty, else = ‘0’.

  - VDI_L[4]=’1’ if CDF_VIDEO_FSLR_SUPP field is not empty, else = ‘0’.

  - VDI_L[5]=’1’ if CDF_VIDEO_TBLR_SUPP field is not empty, else = ‘0’.

  - VDI_L[6]=’1’ if CDF_VIDEO_DUAL3D field is not empty, else = ‘0’.

  - VDI_L[7]=’1’ if CDF_VIDEO_SEP_AUD field is YES, else = ‘0’.

In the sequential order of the descriptors, Tester to send out as below, and operator to verify that video, audio on each downstream sink device are being produced properly.

  - Video mode, starting with a 2View mode if DUT supports,  with correct AVI, VSIF (MHL_VID_FMT, MHL_3D_FMT_TYPE…).

  - And with separate audio for each views If VDI_L[7] = ‘1’ (else, one audio stream for the whole multi-view).

Repeat test from step 9 for each resolution listed in the order below.

  - All 2View modes.

  - All 4View modes

  - All Dual3D modes.

If DUT passes all iterations of the test, then PASS, else FAIL.

AV SYNC

Sink AV Sync Test

Verify functionality of Sink DUT's AV Sync function.

- If CDF _AV_SYNC_SUPPORTED is YES  then continue to test, else PASS(SKIP).

Connect DUT and perform Discovery.

Configure the MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and PCM_2CH_44kHz audio with 1kHz sine wave test tone. Source VSIF indicates no audio delay.

Wait 10 seconds for DUT to send Write Burst request for Audio Delay Correction to the Source (Test Equipment).  After 10 second timeout, if no Audio delay correction request is made, continue to step 9, else skip to step 11.

Source device (test Equipment)  sends VSIF with AV_DELAY_SYNC field set to maximum value (0xFFFFF) which equals 1.04 seconds.

Wait 10 seconds for DUT to send Write Burst request for Audio Delay Correction to the Source (Test Equipment).  After 10 second timeout, if no Audio delay correction request is made a second time then FAIL test.

Analyze Audio Delay Data Structure for valid data, if burst data is invalid then FAIL, else continue.

Source device (test Equipment)  responds with requested delay.

If DUT properly reqested Audio delay from source, and all steps passed, then PASS.

EDID

EDID Read Test

Verify that the Sink DUT's EDID is accessible and accurate.

- Connect MHL EDID Reader to DUT.

Wait until SET_HPD command is received.

Read Block0. If Extension Flag(byte 0x7E) is 0, then FAIL, else read all extended blocks.

If check_sum is correct, then continue to test, else FAIL.

Read Block0 and extended blocks again and compare.

If they do not match, then FAIL.

Examine EDID:

  - If all Video Modes found in the EDID are listed in the CDF, then continue to test, else FAIL.

  - If all Pixel Encodings found in the EDID are listed in the CDF, then continue to test, else FAIL.

  - If all audio modes found in the EDID are listed in the CDF, then PASS, else FAIL.

# Dongle Test

Dongles may have a male (Type-M) or female (Type-F) connector on the downstream, non-MHL side but a male type connector (plug) is attached on the upstream side for both types of Dongle device. Thus the same type of TPA board will be used for both Type-M and Type F Dongle devices.

## Electrical Tests

TMDS Electrical Tests

All tests in the CTS 2, Section 5.1.1, shall be run against the Dongle DUT.

## System Tests

This test group defines the MHL3 legacy compliance testing in MHL 1 / MHL 2 mode.

The equipment setups shown in Section 2.3.2 are used throughout Section 5.2.

All MHL3 devices have an MHL 1 / MHL 2 compatibility mode test requirement.  Refer to MHL CTS 2 test procedures for the specific test steps for legacy tests.  New tests for MHL 3 are defined in Section 5.8.  The general Required Methodology for all MHL3 Sink DUT tests based on normative references to CTS 2.1 are tested in oCBUS mode.  Test results for each legacy mode test shall be recorded in the TRF by their test ID as listed in table 5-1 below.

Table 5-1. Dongle AVLINK Test Sequence

| Test ID | MHL 2 oCBUS Compatibility mode |
| --- | --- |
| 5.2.1.1 | X |
| 5.2.1.2 | X |
| 5.2.1.3 | X |
| 5.2.1.4 | X |
| 5.2.2.1 | X |
| 5.2.2.2 | X |
| 5.2.2.3 | X |
| 5.2.2.4 | X |
| 5.2.2.5 | X |
| 5.2.3.1 | X |
| 5.2.3.2 | X |
| 5.2.4.1 | X |
| 5.2.5.1 | X |
| 5.2.5.2 | X |
| 5.2.5.3 | X |
| 5.2.6.1 | X |
| 5.2.6.2 | X |
| 5.2.7.1 | X |
| 5.2.8.1 | X |
| 5.2.8.2 | X |
| 5.2.9.1 | X |
| 5.2.9.2 | X |

## CBUS Tests

Tests in Section 5.3 of the released MHL 2 Compliance Test Specification are executed using the varieties of CBUS as noted in Table 5-2. As each test is executed in Section 5.3, the test number beginning with "5.3", together with the CBUS mode as invoked from Table 5-2, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 5-2, according to which columns of Table 5-2 are marked with an 'X'.

Table 5-2. eCBUS oCBUS Suite CBUS Tests

| TEST | MHL 2 oCBUS Compatibility mode |
| --- | --- |
| 5.3.3.2 | X |
| 5.3.3.3 | X |
| 5.3.4.1 | X |
| 5.3.4.2 | X |
| 5.3.4.3 | X |
| 5.3.5.1 | X |
| 5.3.6.1 | X |
| 5.3.6.2 | X |
| 5.3.6.3 | X |
| 5.3.6.4 | X |
| 5.3.7.1 | X |
| 5.3.7.2 | X |
| 5.3.8.1 | X |
| 5.3.8.2 | X |
| 5.3.9.1 | X |
| 5.3.10.1 | X |
| 5.3.10.2 | X |
| 5.3.11.1 | X |
| 5.3.11.2 | X |
| 5.3.11.3 | X |
| 5.3.11.4 | X |
| 5.3.12.1 | X |
| 5.3.12.2 | X |
| 5.3.13.1 | X |
| 5.3.14.1 | X |
| 5.3.14.2 | X |
| 5.3.14.3 | X |
| 5.3.15.1 | X |
| 5.3.15.2 | X |
| 5.3.16.1 | X |
| 5.3.17.1 | X |
| 5.3.17.2 | X |
| 5.3.17.3 | X |
| 5.3.18.1 | X |
| 5.3.19.1 | X |
| 5.3.20.1 | X |
| 5.3.21.1 | X |
| 5.3.22.1 | X |
| 5.3.23.1 | X |
| 5.3.23.2 | X |
| 5.3.23.3 | X |
| 5.3.24.1 | X |
| 5.3.25.1 | X |
| 5.3.26.1 | X |

## eCBUS oCBUS / MHL 3 Legacy Protocol Tests

Tests in Section 6.3 of the released MHL 2 Compliance Test Specification are executed using the varieties of CBUS as noted in Table 5-3. As each test is executed in Section 6.3, the test number beginning with "6.3", together with the CBUS mode as invoked from Table 5-3, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 5-3, according to which columns of Table 5-3 are marked with an 'X'.

Table 5-3. eCBUS oCBUS / MHL 3 Suite Normal Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.3.1.1 | X | X | If DUT Supports |
| 6.3.2.1 | X | X | If DUT Supports |
| 6.3.3.1 | X | X | If DUT Supports |
| 6.3.3.2 | X | X | If DUT Supports |
| 6.3.3.3 | X | X | If DUT Supports |
| 6.3.3.4 | X | X | If DUT Supports |
| 6.3.3.5 | X | X | If DUT Supports |
| 6.3.3.6 | X | X | If DUT Supports |
| 6.3.3.7 | X | X | If DUT Supports |
| 6.3.5.1 | X | X | If DUT Supports |
| 6.3.5.2 | X | X | If DUT Supports |
| 6.3.5.3 | X | X | If DUT Supports |
| 6.3.5.4 | X | X | If DUT Supports |
| 6.3.5.5 | X | X | If DUT Supports |
| 6.3.5.6 | X | X | If DUT Supports |
| 6.3.5.7 | X | X | If DUT Supports |
| 6.3.5.8 | X | X | If DUT Supports |
| 6.3.8.1 | X | X | If DUT Supports |
| 6.3.8.2 | X | X | If DUT Supports |
| 6.3.10.1 | X | X | If DUT Supports |
| 6.3.10.2 | X | X | If DUT Supports |
| 6.3.10.3 | X | X | If DUT Supports |
| 6.3.10.4 | X | X | If DUT Supports |
| 6.3.10.5 | X | X | If DUT Supports |
| 6.3.10.6 | X | X | If DUT Supports |
| 6.3.10.7 | X | X | If DUT Supports |
| 6.3.10.8 | X | X | If DUT Supports |
| 6.3.11.20 | X | X | If DUT Supports |
| 6.3.11.21 | X | X | If DUT Supports |
| 6.3.13.1 |  |  |  |
| 6.3.13.2 |  |  |  |
| 6.3.14.1 |  |  |  |
| 6.3.14.2 |  |  |  |
| 6.3.15.1 | X | X | If DUT Supports |
| 6.3.15.2 | X | X | If DUT Supports |
| 6.3.17.2 |  |  |  |
| 6.3.17.3 |  |  |  |
| 6.3.18.1 |  |  |  |
| 6.3.18.2 |  |  |  |
| 6.3.18.3 |  |  |  |
| 6.3.18.4 |  |  |  |
| 6.3.20.2 | X | X | If DUT Supports |
| 6.3.20.3 | X | X | If DUT Supports |
| 6.3.21.1 | X | X | If DUT Supports |
| 6.3.21.2 | X | X | If DUT Supports |
| 6.3.21.3 | X | X | If DUT Supports |
| 6.3.21.4 | X | X | If DUT Supports |
| 6.3.21.5 | X | X | If DUT Supports |

Notes on Table 4-3:

- The Required Methodology text in MHL 2.x CTS calls out oCBUS parameter values which must be replaced with MHL 3 parameter values during the test of an MHL 3 DUT.

Table 5-4. eCBUS oCBUS / MHL 3 Suite Robustness Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.3.4.1 |  |  |  |
| 6.3.6.1 | X | X | If DUT Supports |
| 6.3.6.2 | X | X | If DUT Supports |
| 6.3.6.3 | X | X | If DUT Supports |
| 6.3.6.4 | X | X | If DUT Supports |
| 6.3.6.5 | X | X | If DUT Supports |
| 6.3.7.1 | X | X | If DUT Supports |
| 6.3.9.1 | X | X | If DUT Supports |
| 6.3.11.1 | X | X | If DUT Supports |
| 6.3.11.2 | X | X | If DUT Supports |
| 6.3.11.3 | X | X | If DUT Supports |
| 6.3.11.4 | X | X | If DUT Supports |
| 6.3.11.5 | X | X | If DUT Supports |
| 6.3.11.6 | X | X | If DUT Supports |
| 6.3.11.7 | X | X | If DUT Supports |
| 6.3.11.8 | X | X | If DUT Supports |
| 6.3.11.9 | X | X | If DUT Supports |
| 6.3.11.10 | X | X | If DUT Supports |
| 6.3.11.11 | X | X | If DUT Supports |
| 6.3.11.12 | X | X | If DUT Supports |
| 6.3.11.13 | X | X | If DUT Supports |
| 6.3.11.14 | X | X | If DUT Supports |
| 6.3.11.15 | X | X | If DUT Supports |
| 6.3.11.16 | X | X | If DUT Supports |
| 6.3.11.17 | X | X | If DUT Supports |
| 6.3.11.19 | X | X | If DUT Supports |
| 6.3.11.22 | X | X | If DUT Supports |
| 6.3.11.23 | X | X | If DUT Supports |
| 6.3.11.24 | X | X | If DUT Supports |
| 6.3.12.1 | X | X | If DUT Supports |
| 6.3.12.2 | X | X | If DUT Supports |
| 6.3.12.3 | X | X | If DUT Supports |
| 6.3.12.4 | X | X | If DUT Supports |
| 6.3.12.5 | X | X | If DUT Supports |
| 6.3.12.6 | X | X | If DUT Supports |
| 6.3.12.7 | X | X | If DUT Supports |
| 6.3.12.8 | X | X | If DUT Supports |
| 6.3.12.9 | X | X | If DUT Supports |
| 6.3.16.1 | X | X | If DUT Supports |
| 6.3.16.2 | X | X | If DUT Supports |
| 6.3.19.1 |  |  |  |
| 6.3.19.2 |  |  |  |
| 6.3.19.3 |  |  |  |
| 6.3.19.4 |  |  |  |
| 6.3.22.1 | X | X | If DUT Supports |
| 6.3.22.2 | X | X | If DUT Supports |
| 6.3.22.3 | X | X | If DUT Supports |

## eCBUS MHL 3 Active Mode Dongle Tests

Tests in Section 6.4 of this Compliance Test Specification are executed using the varieties of CBUS as noted in Table 5-5. As each test is executed in Section 6.4, the test number beginning with "6.4", together with the CBUS mode as invoked from Table 5-5, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 5-5, according to which columns of Table 5-5 are marked with an 'X'.

Table 5-5. eCBUS MHL 3 Active Mode Dongle Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.4.3.1 | X |  |  |
| 6.4.3.1 | X |  |  |
| 6.4.3.2 | X |  |  |
| 6.4.3.3 | X |  |  |
| 6.4.3.4 | X |  |  |
| 6.4.3.7 | X |  |  |
| 6.4.3.8 | X |  |  |
| 6.4.4.2 | X |  |  |
| 6.4.6.1 | X |  |  |
| 6.4.6.2 | X |  |  |
| 6.4.7.2 |  | X |  |
| 6.4.8.2 |  |  | If DUT Supports |
| 6.4.9.1 |  | X | If DUT Supports |
| 6.4.10.1 |  | X | If DUT Supports |
| 6.4.10.2 |  | X | If DUT Supports |
| 6.4.10.3 |  | X | If DUT Supports |
| 6.4.10.4 |  | X | If DUT Supports |
| 6.4.10.5 |  | X | If DUT Supports |
| 6.4.11.1 |  | X | If DUT Supports |
| 6.4.11.2 |  | X | If DUT Supports |
| 6.4.11.3 |  | X | If DUT Supports |
| 6.4.12.1 |  | X | If DUT Supports |
| 6.4.12.2 |  | X | If DUT Supports |
| 6.4.12.3 |  | X | If DUT Supports |
| 6.4.13.1 |  | X | If DUT Supports |
| 6.4.13.2 |  | X | If DUT Supports |
| 6.4.13.3 |  | X | If DUT Supports |
| 6.4.14.1 |  | X | If DUT Supports |
| 6.4.15.1 |  | X | If DUT Supports |
| 6.4.16.2 |  | X | If DUT Supports |
| 6.4.16.3 |  | X | If DUT Supports |
| 6.4.16.4 |  | X | If DUT Supports |
| 6.4.16.5 |  | X | If DUT Supports |
| 6.4.17.2 |  | X | If DUT Supports |
| 6.4.18.4 |  | X | If DUT Supports |

## eCBUS MHL 3 Robustness Mode Dongle Tests

Tests in Section 6.5 of this Compliance Test Specification are executed using the varieties of CBUS as noted in Table 5-6. As each test is executed in Section 6.5, the test number beginning with "6.5", together with the CBUS mode as invoked from Table 5-6, shall be recorded in the TRF. There will therefore be multiple records of each test in Table 5-6, according to which columns of Table 5-6 are marked with an 'X'.

Table 5-6. eCBUS MHL 3 Robustness Mode Dongle Tests

| TEST | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| 6.5.1.1 |  | X | If DUT Supports |
| 6.5.1.1 |  | X | If DUT Supports |
| 6.5.1.2 |  | X | If DUT Supports |
| 6.5.2.1 |  | X | If DUT Supports |
| 6.5.2.2 |  | X | If DUT Supports |
| 6.5.2.3 |  | X | If DUT Supports |
| 6.5.3.2 |  | X | If DUT Supports |
| 6.5.3.3 |  | X | If DUT Supports |
| 6.5.3.4 |  | X | If DUT Supports |
| 6.5.4.1 |  | X | If DUT Supports |
| 6.5.5.1 |  | X | If DUT Supports |
| 6.5.5.2 |  | X | If DUT Supports |
| 6.5.5.3 |  | X | If DUT Supports |

## MHL 3 Electrical Tests

TMDS and eCBUS Electrical Test Setups

Figure 5-1. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Dongle Test

Figure 5-2. Differential TMDS, Single-Ended MHL CLK and eCBUS-S FWD Signals Calibration for Dongle Jitter Tolerance Test

Figure 5-3. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Dongle Test

Figure 5-4. Differential TMDS, Differential MHL CLK and eCBUS-D FWD Signals Calibration for Dongle Jitter Tolerance Test

Figure 5-5. Test configuration at TP3 for Dongle TMDS input and eCBUS-S FWD input

Figure 5-6. Test configuration at TP3 for Dongle TMDS input and eCBUS-D FWD input

Figure 5-7. Test configuration at TP3 for Dongle eCBUS-S BWD output

Figure 5-8. Test configuration at TP3 for Dongle eCBUS-D BWD output

Figure 5-9. Test configuration at TP2 for Dongle TMDS and eCBUS-S impedance

Figure 5-10. Test configuration at TP2 for Dongle TMDS and eCBUS-D impedance

TMDS and eCBUS Electrical Test Procedures

Single-Ended High Level Input Voltage Tolerance of Differential TMDS Data: VSE_HIGH_DF_TMDS_DATA

This test confirms that the Dongle device supports the single-ended high level voltage of the differential TMDS data input signal allowed by the specification.

Test Signal Calibration

- Make connections shown in Figure 5-1 for DUT with eCBUS-S and Figure 5-3 For DUT with eCBUS-D and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS

      - eCBUS-S: 1000ps

      - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS

      - eCBUS-S single-ended swing: 310mV

      - eCBUS-D differential swing: 550mV

  - TMDS single-ended high level:

    - Max single-ended high Level: VTERM + 20mV

    - Min single-ended high level: VTERM – 1200mV

  - eCBUS DC DC level:

    - eCBUS-S: 155mV

    - eCBUS-D: VTERM – 228mV

Tolerance Test

- Make connections shown in Figure 5-5 for DUT with eCBUS-S and Figure 5-6 for DUT with eCBUS-D. Use Max single-ended high level in A.1.vi.1 of Test Signal Calibration described above for TMDS signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count.

Change the TMDS signal output single-ended high lelve to the Min single-ended high level setting in A.1.vi.1 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Single-Ended Low Level Input Voltage Tolerance of Differential TMDS Data: VSE_LOW_DF_TMDS_DATA

This test confirms that the Dongle device supports the single-ended low level voltage of the differential TMDS data input signal allowed by the specification.

Test Signal Calibration

- Make connections shown in Figure 5-1 for DUT with eCBUS-S and Figure 5-3 For DUT with eCBUS-D and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

Rise/fall times:

  - TMDS: 60ps

  - eCBUS

    - eCBUS-S: 1000ps

    - eCBUS-D: 250ps

Swing voltage:

  - TMDS differential swing: 675mV

  - eCBUS

    - eCBUS-S single-ended swing: 310mV

    - eCBUS-D differential swing: 550mV

TMDS single-ended low level:

  - Max single-ended low Level: VTERM – 380mV

  - Min single-ended low level: VTERM – 1500mV

eCBUS DC DC level:

  - eCBUS-S: 155mV

  - eCBUS-D: VTERM – 228mV

Tolerance Test

- Make connections shown in Figure 5-5 for DUT with eCBUS-S and Figure 5-6 for DUT with eCBUS-D. Use Max single-ended low Level in A.1.vi.1 of Test Signal Calibration described above for TMDS signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count.

Change the TMDS signal output single-ended low lelve to the Min single-ended low Level setting in A.1.vi.1 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Single-Ended High Level Input Voltage Tolerance of Differential MHL CLK and eCBUS-D FWD Data: VSE_HIGH_DF_MHL_CLK, VSE_HIGH_DF_eCBUS_FWD

This test confirms that the Dongle device supports the single-ended high level voltage of the differential MHL CLK and eCBUS-D FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 5-3 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-D differential swing: 550mV

  - TMDS DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D single-ended high level:

      - Max single-ended high Level: VTERM + 20mV

      - Min single-ended high level: VTERM – 200mV

Tolerance Test

- Make connections shown in Figure 5-6. Use Max single-ended high level setting in A.1.vii.1 of Test Signal Calibration described above for eCBUS-D FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-D FWD signal output single-ended high level to the Min single-ended high level setting in A.1.vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Single-Ended Low Level Input Voltage Tolerance of Differential MHL CLK and eCBUS-D FWD Data: VSE_LOW_DF_MHL_CLK, VSE_LOW_DF_eCBUS_FWD

This test confirms that the Dongle device supports the single-ended low level voltage of the differential MHL CLK and eCBUS-D FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 5-3 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-D differential swing: 550mV

  - TMDS DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D single-ended low level:

      - Max single-ended low Level: VTERM – 180mV

      - Min single-ended low level: VTERM – 550mV

Tolerance Test

- Make connections shown in Figure 5-6. Use Max single-ended low level setting in A.1.vii.1 of Test Signal Calibration described above for eCBUS-D FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-D FWD signal output single-ended low level to the Min single-ended high level setting in A.1.vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Differential input swing voltage Tolerance of Differential TMDS Data: VIDF_SWING_DF_TMDS_DATA

This test confirms that the Dongle device supports the differential swing voltage of the differential TMDS data input signal allowed by the specification.

Test Signal Calibration

- Make connections shown in Figure 5-1 For DUT with eCBUS-S and Figure 5-3 For DUT with eCBUS-D and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS

      - eCBUS-S: 1000ps

      - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing:

      - Max swing: 1000mV

      - Min swing: 350mV

    - eCBUS

      - eCBUS-S single-ended swing: 310mV

      - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS

      - eCBUS-S: 155mV

      - eCBUS-D: VTERM – 228mV

Tolerance Test

- Make connections shown in Figure 5-5 for DUT with eCBUS-S and Figure 5-6 for DUT with eCBUS-D. Use Max swing setting in A.1.v.1.a of Test Signal Calibration described above for TMDS signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count.

Change the TMDS signal output swing to the Min swing setting in A.1.v.1.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Differential Input Swing Voltage Tolerance of Differential MHL CLK and eCBUS-D FWD Data: VIDF_SWING_DF_MHL_CLK, VIDF_SWING_DF_eCBUS_FWD

This test confirms that the Dongle device supports the differential swing voltage of the differential MHL CLK and eCBUS-D FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 5-3 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-D differential swing

      - Max swing: 700mV

      - Min swing: 400mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D: VTERM – 228mV

Tolerance Test

- Make connections shown in Figure 5-6. Use Max swing setting in A.1.v.2.a of Test Signal Calibration described above for eCBUS-D FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-D FWD signal output swing to the Min swing setting in A.1.v.2.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Single-Ended High Level Input Voltage Tolerance of Single-Ended MHL CLK and eCBUS-S FWD Data: VSE_HIGH_SE_MHL_CLK, VSE_HIGH_SE_eCBUS_FWD

This test confirms that the Dongle device supports the single-ended high level voltage of the single-ended MHL CLK and eCBUS-S FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-S.

Test Signal Calibration

- Make connections shown in Figure 5-1 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-S: 1000ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-S single-ended swing: 310mV

  - TMDS DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-S single-ended high level:

      - Max single-ended high Level: 400mV

      - Min single-ended high level: 220mV

Tolerance Test

- Make connections shown in Figure 5-5. Use Max single-ended high level setting in A.1.vii.1 of Test Signal Calibration described above for eCBUS-S FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-S FWD signal output single-ended high lelve to the Min single-ended high level setting in A.1.vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Single-Ended Low Level Input Voltage Tolerance of Single-Ended MHL CLK and eCBUS-S FWD Data: VSE_LOW_SE_MHL_CLK, VSE_LOW_SE_eCBUS_FWD

This test confirms that the Dongle device supports the single-ended low level voltage of the single-ended MHL CLK and eCBUS-S FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-S.

Test Signal Calibration

- Make connections shown in Figure 5-1 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-S: 1000ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-S single-ended swing: 310mV

  - TMDS DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-S single-ended low level:

      - Max single-ended low Level: 30mV

      - Min single-ended low level: -30mV

Tolerance Test

- Make connections shown in Figure 5-5. Use Max single-ended low level setting in A.1.vii.1 of Test Signal Calibration described above for eCBUS-S FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-S FWD signal output single-ended low lelve to the Min single-ended low level setting in A.1.vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Single-Ended Input Swing Voltage Tolerance of Single-Ended MHL CLK and eCBUS-S FWD Data: VISE_SWING_SE_MHL_CLK, VISE_SWING_SE_eCBUS_FWD

This test confirms that the Dongle device supports the single-ended swing voltage of the single-ended MHL CLK and eCBUS-S FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-S.

Test Signal Calibration

- Make connections shown in Figure 5-1 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-S: 1000ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-S single-ended swing

      - Max swing: 370mV

      - Min swing: 250mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-S: 155mV

Tolerance Test

- Make connections shown in Figure 5-5. Use Max swing setting in A.1.v.2.a of Test Signal Calibration described above for eCBUS-S FWD signal output.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the eCBUS-S FWD signal output swing to the Min swing setting in A.1.v.2.b of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Differential Intra-Pair Skew Tolerance of Differential TMDS Data: TSKEW_TP3_DF_TMDS_DATA

This test confirms that the Dongle device tolerates the differential intra-pair skew of the differential TMDS data input signal allowed by the specification.

Test Signal Calibration

- Make connections shown in Figure 5-1 For DUT with eCBUS-S and Figure 5-3 For DUT with eCBUS-D and calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS

      - eCBUS-S: 1000ps

      - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS

      - eCBUS-S single-ended swing: 310mV

      - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS

      - eCBUS-S: 155mV

      - eCBUS-D: VTERM – 228mV

  - TMDS intra-pair skew

    - Positive 25ps: positive output leads negative output by 25ps.

    - Negative 25ps: negative output leads positive output by 25ps.

Tolerance Test

- Make connections shown in Figure 5-5 for DUT with eCBUS-S and Figure 5-6 for DUT with eCBUS-D. Use Positive 25ps skew setting in A.1.vii.1 of Test Signal Calibration described above for TMDS intra-pair skew.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count.

Change the TMDS signal intra-pair skew to the Negative 25ps setting in A.1.vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Differential Intra-Pair Skew Tolerance of Differential MHL CLK and eCBUS-D FWD Data: TSKEW_TP3_DF_MHL_CLK, TSKEW_TP3_DF_eCBUS_FWD

This test confirms that the Dongle device tolerates the differential intra-pair skew of the differential MHL CLK and eCBUS-D FWD data input signals allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 5-3 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 675mV

    - eCBUS-D differential swing: 550mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D: VTERM – 228mV

  - Differential MHL CLK and eCBUS-D FWD intra-pair skew

    - Positive 50ps: positive output leads negative output by 50ps.

    - Negative 50ps: negative output leads positive output by 50ps.

Tolerance Test

- Make connections shown in Figure 5-6. Use Positive 50ps skew setting in A.1.vii.1 of Test Signal Calibration described above for MHL CLK and eCBUS-D FWD intra-pair skew.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the eCBUS RX BIST Error Count.

Change the MHL CLK and eCBUS-D signal intra-pair skew to the Negative 50ps setting in A.1.vii.2 of Test Signal Calibration described above.

Repeat steps 2 to 9.

If the Error Count is zero in steps 9 and 11, then PASS; else FAIL.

Jitter Tolerance of Differential MHL CLK, eCBUS-D FWD Data and Differential TMDS Data

This test confirms that the Dongle device tolerates the differential MHL CLK jitter and eCBUS-D FWD data eye diagram impairments and differential TMDS data eye diagram impairments allowed by the specification. This test is applied only to the DUT with eCBUS-D.

Test Signal Calibration

- Make connections shown in Figure 5-3 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-D: 250ps

  - Swing voltage:

    - TMDS differential swing: 350mV

    - eCBUS-D differential swing: 400mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-D: VTERM – 228mV

  - TMDS intra-pair skew, MHL CLK/eCBUS-D intra-pair skew

    - Positive skew: positive output leads negative output by 25ps for TMDS and by 50ps for MHLCLK/eCBUS-D.

    - Negative skew: negative output leads positive output by 25ps for TMDS and by 50ps for MHL CLK/eCBUS-D.

Make connections shown in Figure 5-4 and continue calibration of the test signal for the following condition.

  - Differential MHL CLK and eCBUS-D FWD data jitter

    - 7MHz jitter

      - Add 67ps pk-pk jitter of 7MHz jitter frequency to the MHL CLK and eCBUS-D FWD data.

      - Confirm that eCBUS-D FWD data eye diagram requirement is met. If not, increase the eCBUS FWD data swing until the eye diagram passes.

    - 10MHz jitter

      - Add 67ps pk-pk jitter of 10MHz jitter frequency to the MHL CLK and eCBUS-D FWD data.

      - Confirm that eCBUS-D FWD data eye diagram requirement is met. If not, increase the eCBUS FWD data swing until the eye diagram passes.

  - Differential TMDS data eye diagram

    - 1MHz jitter

      - With 7MHz jitter above in A.2.i.1, add 1MHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 7MHz MHL CLK jitter and TMDS data swing voltage.

    - 500kHz jitter

      - With 10MHz jitter above in A.2.i.2, add 500kHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 10MHz MHL CLK jitter and TMDS data swing voltage.

Tolerance Test

- Make connections shown in Figure 5-6.

Use Positive skew setting in A.1.vii.1, 7MHz jitter in A.2.i.1 and 1MHz jitter in A.2.ii.1 of Test Signal Calibration described above.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test and eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count.

Change the calibration setting to Positive skew setting in A.1.vii.1, 10MHz jitter in A.2.i.2 and 500kHz jitter in A.2.ii.2 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in A.1.vii.2, 7MHz jitter in A.2.i.1 and 1MHz jitter in A.2.ii.1 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in A.1.vii.2, 10MHz jitter in A.2.i.2 and 500kHz jitter in A.2.ii.2 of Test Signal Calibration described above

Repeat steps 2 to 10.

Repeat steps 1 to 16 for all other supported TMDS data rates.

If the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count are zero in all cases, then PASS; else FAIL.

Jitter Tolerance of Single-Ended MHL CLK, eCBUS-S FWD Data and Differential TMDS Data

This test confirms that the Dongle device tolerates the single-ended MHL CLK jitter and eCBUS-S FWD data eye diagram impairments and differential TMDS data eye diagram impairments allowed by the specification. This test is applied only to the DUT with eCBUS-S.

Test Signal Calibration

- Make connections shown in Figure 5-1 And calibrate the test signal for the following condition.

  - PATTERN: AVLINK BIST PRBS with randomizing, eCBUS BIST PRBS

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p

  - Rise/fall times:

    - TMDS: 60ps

    - eCBUS-S: 1000ps

  - Swing voltage:

    - TMDS differential swing: 350mV

    - eCBUS-S single-ended swing: 250mV

  - DC level:

    - TMDS: VTERM – 338mV

    - eCBUS-S: 155mV

  - TMDS intra-pair skew

    - Positive skew: positive output leads negative output by 25ps for TMDS.

    - Negative skew: negative output leads positive output by 25ps for TMDS.

Make connections shown in Figure 5-2 and continue calibration of the test signal for the following condition.

  - Single-Ended MHL CLK and eCBUS-S FWD data jitter

    - 7MHz jitter

      - Add 150ps pk-pk jitter of 7MHz jitter frequency to the MHL CLK and eCBUS-S FWD data.

      - Confirm that eCBUS-S FWD data eye diagram requirement is met. If not, increase the eCBUS-S FWD data swing until the eye diagram passes.

    - 10MHz jitter

      - Add 150ps pk-pk jitter of 10MHz jitter frequency to the MHL CLK and eCBUS-S FWD data.

      - Confirm that eCBUS-S FWD data eye diagram requirement is met. If not, increase the eCBUS-S FWD data swing until the eye diagram passes.

  - Differential TMDS data eye diagram

    - 1MHz jitter

      - With 7MHz jitter above in 2.i.1, add 1MHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 7MHz MHL CLK jitter and TMDS data swing voltage.

    - 500kHz jitter

      - With 10MHz jitter above in 2.i.2, add 500kHz jitter to TMDS data until TMDS eye diagram at TP2 is marginally met.

      - If necessary to meet the eye diagram requirement, adjust 10MHz MHL CLK jitter and TMDS data swing voltage.

Tolerance Test

- Make connections shown in Figure 5-5.

Use Positive skew setting in A.1.vii.1, 7MHz jitter in A.2.i.1 and 1MHz jitter in A.2.ii.1 of Test Signal Calibration described above.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following AVLINK Rx BIST mode using the CBUS Source.

  - AVLINK PATTERN: PRBS with randomizing

  - AVLINK DATA RATE: supported highest data rate among 6Gbps, 3Gbps, and 1.5Gbps

  - AVLINK VIDEO MODE: 480p60

  - AVLINK DURATION: 3616 frames

Set the DUT in the following eCBUS RX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 70 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator and start AVLINK RX BIST BER test and eCBUS RX BIST BER test.

After BER test is done, switch the RF relay path in the RELT board to change the DUT eCBUS connection from the MHL3 Signal Generator to the CBUS Source.

Check the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count.

Change the calibration setting to Positive skew setting in A.1.vii.1, 10MHz jitter in A.2.i.2 and 500kHz jitter in 2.ii.2 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in A.1.vii.2, 7MHz jitter in A.2.i.1 and 1MHz jitter in A.2.ii.1 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Change the calibration setting to Negative skew setting in A.1.vii.2, 10MHz jitter in A.2.i.2 and 500kHz jitter in A.2.ii.2 of Test Signal Calibration described above.

Repeat steps 2 to 10.

Repeat steps 1 to 16 for all other supported TMDS data rates.

If the AVLINK RX BIST Error Count and eCBUS RX BIST Error Count are zero in all cases, then PASS; else FAIL.

Output DC Voltage of eCBUS-D BWD Data: VODC_DF_eCBUS_BWD

This test confirms that the DC voltage level of eCBUS-D BWD data output signal is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 5-8.

Set the MHL3 Signal Generator outputs as calibrated in Section 5.7.2.6 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD waveforms of positive and negative signals, measure the mean values of the voltage histogram for middle portion of eCBUS-D data high period.

From the eCBUS-D BWD waveforms of positive and negative signals, measure the mean values of the voltage histogram for middle portion of eCBUS-D data low period.

VODC_DF_eCBUS_BWD is the average value of the two mean values obtained in steps 8 and 9 for the positive and negative signals.

If VTERM – 375 mV <= VODC_DF_eCBUS_BWD  <= VTERM – 80 mV for both positive and negative signals, then PASS; else FAIL.

Differential Output Swing Voltage of eCBUS-D BWD Data: VDF_SWING_DF_eCBUS_BWD

This test confirms that the differential swing voltage of eCBUS-D BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 5-8.

Set the MHL3 Signal Generator outputs as calibrated in Section 5.7.2.6 with eCBUS-D FWD swing 550mV on eCBUS-D_SG ports. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: Fixed 10 pattern, 5 High/5 Low

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD Data waveform, measure the differential swing voltage (VDF_SWING_DF_eCBUS_BWD).

  - Obtain differential waveform using the scope’s math function.

  - Measure VDF_SWING_DF_eCBUS_BWD:

    - Obtain the mean values of the voltage histograms for the middle three TBIT_DF_eCBUS_BWD  periods of the 5 High and 5 Low bits of eCBUS-D BWD Data.

    - Obtain the differential swing voltage (VDF_SWING_DF_eCBUS_BWD) by subtracting the two mean values  in the previous step.

If 150mV <= VDF_SWING_DF_eCBUS_BWD <= 700mV, then PASS; else FAIL.

Output DC Voltage of eCBUS-S BWD Data: VODC_SE_eCBUS_BWD

This test confirms that the DC voltage level of eCBUS-S BWD data output signal is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 5-7.

Set the MHL3 Signal Generator outputs as calibrated in Section 5.7.2.9 with eCBUS-S FWD swing 310mV on eCBUS-S_SG port. eCBUS-S_EC port duplicates the eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD waveform, measure the mean value of the voltage histogram for middle portion of eCBUS-S data high period.

From the eCBUS-S BWD waveform, measure the mean value of the voltage histogram for middle portion of eCBUS-S data low period.

VODC_SE_eCBUS_BWD is the average value of the two mean values obtained in steps 8 and 9.

If 95 mV <= VODC_SE_eCBUS_BWD <= 215 mV, then PASS; else FAIL.

Single-Ended Output Swing Voltage of eCBUS-S BWD Data: VSE_SWING_SE_eCBUS_BWD

This test confirms that the single-ended output swing voltage of eCBUS-S BWD Data output is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 5-7.

Set the MHL3 Signal Generator outputs as calibrated in Section 5.7.2.9 with eCBUS-S FWD swing 310mV on eCBUS-S_SG port. eCBUS-S_EC port duplicates the eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD Data waveform, measure the single-ended swing voltage (VSE_SWING_SE_eCBUS_BWD) by subtracting the voltage histogram mean value of the data low period from the voltage histogram mean value of the data low period.

If 170 mV <= VSE_SWING_SE_eCBUS_BWD <= 370 mV, then PASS; else FAIL.

eCBUS-D BWD Data Eye Diagram at TP3

This test confirms that the eCBUS-D BWD Data Eye Diagram at TP3 is within the specified limits. This test is applied only to the DUT with eCBUS-D.

- Make connections shown in Figure 5-8.

Set the MHL3 Signal Generator outputs as calibrated in Section 5.7.2.12. Use Positive skew, 7MHz jitter for MHL CLK and eCBUS-D FWD Data, and 1MHz jitter for TMDS Data. eCBUS-D_EC ports duplicate eCBUS-D FWD signal of eCBUS-D_SG ports to cancel the eCBUS-D FWD signal from the bi-directional eCBUS-D signal and extract only eCBUS-D BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-D BWD Data waveform, measure the eye diagram.

  - Capture eCBUS-D BWD Data waveform of 500 microseconds long.

  - Apply the MHL 2.1 Reference Cable Equalizer to the eCBUS-D FWD Data waveform.

  - Measure the eye diagram using the MHL clock from the MHL3 Signal Generator. Process the clock as described in Section 3.7.2.19.

Change the calibration setting to Positive skew setting, 10MHz jitter for MHL CLK and eCBUS-D FWD Data, and 500kHz jitter for TMDS Data.

Repeat steps 3 to 8.

Change the calibration setting to Negative skew setting, 7MHz jitter for MHL CLK and eCBUS-D FWD Data, and 1MHz jitter for TMDS Data.

Repeat steps 3 to 8.

Change the calibration setting to Negative skew setting, 10MHz jitter for MHL CLK and eCBUS-D FWD Data, and 500kHz jitter for TMDS Data.

Repeat steps 3 to 8.

If the every eye diagram in steps 8, 10, 12 and 14 passes eye mask test shown in Figure 16-21 in MHL 3.0 specification, then PASS; else FAIL.

eCBUS-S BWD Data Eye Diagram at TP3

This test confirms that the eCBUS-S BWD Data Eye Diagram at TP3 is within the specified limits. This test is applied only to the DUT with eCBUS-S.

- Make connections shown in Figure 5-7.

Set the MHL3 Signal Generator outputs as calibrated in Section 5.7.2.13. Use Positive skew, 7MHz jitter for MHL CLK and eCBUS-S FWD Data, and 1MHz jitter for TMDS Data. eCBUS-S_EC port duplicates eCBUS-S FWD signal of eCBUS-S_SG port to cancel the eCBUS-S FWD signal from the bi-directional eCBUS-S signal and extract only eCBUS-S BWD signal from the DUT.

Perform CBUS discovery between the DUT and the CBUS Source.

Set the DUT in the following eCBUS TX BIST mode using the CBUS Source.

  - eCBUS PATTERN: PRBS pattern

  - eCBUS DURATION: 32.0 seconds

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS connection from the CBUS Source to MHL3 Signal Generator.

Finish eCBUS training between the DUT and the MHL3 Signal Generator.

From the eCBUS-S BWD Data waveform, measure the eye diagram.

  - Capture eCBUS-S BWD Data waveform of 500 microseconds long.

  - Apply the Low Pass Filter defined MHL 3.0 Specification Section 16.3.1.4 after the Worst Case Cable Emulator.

  - Measure the eye diagram using the MHL clock from the MHL3 Signal Generator. Process the clock as described in Section 3.7.2.26.

Change the calibration setting to Positive skew setting, 10MHz jitter for MHL CLK and eCBUS-D FWD Data, and 500kHz jitter for TMDS Data.

Repeat steps 3 to 8.

Change the calibration setting to Negative skew setting, 7MHz jitter for MHL CLK and eCBUS-D FWD Data, and 1MHz jitter for TMDS Data.

Repeat steps 3 to 8.

Change the calibration setting to Negative skew setting, 10MHz jitter for MHL CLK and eCBUS-D FWD Data, and 500kHz jitter for TMDS Data.

Repeat steps 3 to 8.

If the every eye diagram in steps 8, 10, 12 and 14 passes eye mask test shown in Figure 16-22 in MHL 3.0 specification, then PASS; else FAIL.

Differential Impedance of TMDS: ZD_THRU_TMDS_DGL, ZDT_ TMDS_DGL

This test confirms that the differential impedance of the dongle device is within the range allowed by the specification.

Figure 5-11. Set reference point for differential mode

Figure 5-12. Dongle – Differential Impedance Test Symbology

Before testing, compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables).

- Connect the M3-S TPA SRC-R for eCBUS-S or M3-D TPA SRC-R for eCBUS-D to the TDR oscilloscope through a DC block.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%).

Record the time at the peak of change point in Figure 5-11. This is the start point of the Dongle DUT input connector (TIN).

Calculate TTERM = TIN + CDF_D_DF_TERM_DISTANCE. TTERM is the dongle termination point for differential signals. CDF_D_DF_TERM_DISTANCE is obtained from CDF of the Dongle DUT.

Calculate TCABLESTART and TCABLEEND. TCABLESTART and TCABLEEND are the start and end points of the cable portion in Dongle for differential signals.

  - If (TTERM <= TIN + 6ns), TCABLESTART = TCABLEEND = TIN + (TTERM – TIN) / 2.

  - If (TTERM > TIN + 6ns), TCABLESTART = TIN + CDF_D_DF_CABLE_START_DISTANCE and TCABLEEND = TIN + CDF_D_DF_CABLE_END_DISTANCE . CDF_D_DF_CABLE_START_DISTANCE and CDF_D_DF_CABLE_END_DISTANCE are obtained from CDF of the Dongle DUT.

Connect the equipment as shown in Figure 5-9 for eCBUS-S or in Figure 5-10 for eCBUS-D.  Provide Vbus if DUT is unpowered dongle. Do not provide Vbus if DUT is powered dongle.

Turn on the Dongle DUT and enable MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Source board.

Set the DUT BIST Impedance Mode to AV LINK RX using the CBUS Source.

Trigger the BIST.

Record the differential impedance increment ZDINCREMENT_A from TCABLESTART to TCABLEEND.

Calculate ZDINCREMENT_B.

  - If (ZDINCREMENT_A < 0), ZDINCREMENT_B = 0.

  - If (ZDINCREMENT_A >= 0), ZDINCREMENT_B = ZDINCREMENT_A.

Measure the differential impedance ZDTHROUGH1 from TIN to TCABLESTART and ZDTHROUGH2_WITHCABLE from TCABLEEND to TTERM.

Calculate ZDTHROUGH2 = ZDTHROUGH2_WITHCABLE – ZDINCREMENT_B.

Measure the differential impedance ZDT_WITHCABLE from TTERM to TTERM + 1ns.

Calculate ZDT = ZDT_WITHCABLE – ZDINCREMENT_B.

Evaluate ZDTHROUGH1, ZDTHROUGH2, and ZDT.

  - If (85 ohms <= [ZDTHROUGH1, ZDTHROUGH2] <= 115 ohms) AND (90 ohms <= ZDT <= 110 ohms), it is PASS

  - If (65 ohms <= [ZDTHROUGH1, ZDTHROUGH2] < 85 ohms) OR (115 ohms < [ZDTHROUGH1, ZDTHROUGH2] <= 125 ohms), the impedance is in the excursion range. If an excursion occurs no more than one time for  ZDTHROUGH1 and no more than one time for ZDTHROUGH2 AND the duration of each excursion is less than 350ps AND (90 ohms <= ZDT <= 110 ohms), it is PASS.

All other cases are FAIL.

Differential Impedance of eCBUS-D: ZD_THRU_eCBUS_DGL, ZDT_eCBUS_DGL

This test confirms that the differential impedance of the dongle device is within the range allowed by the specification.

Figure 5-13. Set reference point for differential mode

Figure 5-14. Dongle – Differential Impedance Test Symbology

Before testing, compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables).

- Connect the M3-D TPA SRC-R to the TDR oscilloscope through a DC block.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%).

Record the time at the peak of change point in Figure 5-13. This is the start point of the Dongle DUT input connector (TIN).

Calculate TTERM = TIN + CDF_D_DF_TERM_DISTANCE. TTERM is the dongle termination point for differential signals. CDF_D_DF_TERM_DISTANCE is obtained from CDF of the Dongle DUT.

Calculate TCABLESTART and TCABLEEND. TCABLESTART and TCABLEEND are the start and end points of the cable portion in Dongle for differential signals.

  - If (TTERM <= TIN + 6ns), TCABLESTART = TCABLEEND = TIN + (TTERM – TIN) / 2.

  - If (TTERM > TIN + 6ns), TCABLESTART = TIN + CDF_D_DF_CABLE_START_DISTANCE and TCABLEEND = TIN + CDF_D_DF_CABLE_END_DISTANCE. CDF_D_DF_CABLE_START_DISTANCE and CDF_D_DF_CABLE_END_DISTANCE are obtained from CDF of the Dongle DUT.

Connect the equipment as shown in Figure 5-10.  Provide Vbus if DUT is unpowered dongle. Do not provide Vbus if DUT is powered dongle.

Turn on the Dongle DUT and enable MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Source board.

Record the differential impedance increment ZDINCREMENT_A from TCABLESTART to TCABLEEND.

Calculate ZDINCREMENT_B.

  - If (ZDINCREMENT_A < 0), ZDINCREMENT_B = 0.

  - If (ZDINCREMENT_A >= 0), ZDINCREMENT_B = ZDINCREMENT_A.

Set the DUT BIST Impedance_Mode to eCBUS-D TX HIGH and eCBUS-D Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the differential impedance ZDTHROUGH1_HIGH from TIN to TCABLESTART and ZDTHROUGH2_WITHCABLE_HIGH from TCABLEEND to TTERM.

Calculate ZDTHROUGH2_HIGH = ZDTHROUGH2_WITHCABLE_HIGH – ZDINCREMENT_B.

Measure the differential impedance ZDT_WITHCABLE_HIGH from TTERM to TTERM + 1ns.

Calculate ZDT_HIGH = ZDT_WITHCABLE_HIGH – ZDINCREMENT_B.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the TDR Scope or VNA-base TDR to CBUS Source.

Set the DUT BIST Impedance_Mode to eCBUS-D TX LOW and eCBUS-D Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-D connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the differential impedance ZDTHROUGH1_LOW from TIN to TCABLESTART and ZDTHROUGH2_WITHCABLE_LOW from TCABLEEND to TTERM.

Calculate ZDTHROUGH2_LOW = ZDTHROUGH2_WITHCABLE_LOW – ZDINCREMENT_B.

Measure the differential impedance ZDT_WITHCABLE_LOW from TTERM to TTERM + 1ns.

Calculate ZDT_LOW = ZDT_WITHCABLE_LOW – ZDINCREMENT_B.

Calculate ZDTHROUGH and ZDT.

  - ZDTHROUGH= (ZDTHROUGH_HIGH+ ZDTHROUGH_LOW)/2

  - ZDT= (ZDT_HIGH+ ZDT_LOW)/2

Evaluate ZDTHROUGH1, ZDTHROUGH2, and ZDT.

  - If (85 ohms <= [ZDTHROUGH1, ZDTHROUGH2] <= 115 ohms) AND (90 ohms <= ZDT <= 110 ohms), it is PASS

  - If (65 ohms <= [ZDTHROUGH1, ZDTHROUGH2] < 85 ohms) OR (115 ohms < [ZDTHROUGH1, ZDTHROUGH2] <= 125 ohms), the impedance is in the excursion range. If an excursion occurs no more than one time for  ZDTHROUGH1 and no more than one time for ZDTHROUGH2 AND the duration of each excursion is less than 350ps AND (90 ohms <= ZDT <= 110 ohms), it is PASS.

  - All other cases are FAIL.

Single-Ended Impedance of eCBUS-S: ZS_THRU_eCBUS_DGL, ZST_eCBUS_DGL

This test confirms that the single-ended impedance of the dongle device is within the range allowed by the specification.

Figure 5-15. Set reference point for single-ended signal

Figure 5-16. Dongle – Single-ended Impedance Test Symbology

- Connect the M3-S TPA SRC-R to the TDR oscilloscope through a DC block.

Set the effective rise time of the single-ended TDR pulse to 500ps (20 – 80%).

Record the time at the peak of change point in Figure 5-15. This is the start point of the Dongle DUT input connector (TIN).

Calculate TTERM = TIN +CDF_D_SE_TERM_DISTANCE. TTERM is the dongle termination point for single-ended signals. CDF_D_SE_TERM_DISTANCE is obtained from CDF of the Dongle DUT.

Calculate TCABLESTART and TCABLEEND. TCABLESTART and TCABLEEND are the start and end points of the cable portion in Dongle for single-ended signals.

  - If (TTERM <= TIN + 10ns), TCABLESTART = TCABLEEND = TIN + (TTERM – TIN) / 2.

  - If (TTERM > TIN + 10ns), TCABLESTART = TIN + CDF_D_SE_CABLE_START_DISTANCE and TCABLEEND = TIN + CDF_D_SE_CABLE_END_DISTANCE. CDF_D_SE_CABLE_START_DISTANCE and CDF_D_SE_CABLE_END_DISTANCE are obtained from CDF of the Dongle DUT.

Connect the equipment as shown in Figure 5-9. Provide Vbus if DUT is unpowered dongle. Do not provide Vbus if DUT is powered dongle.

Turn on the Dongle DUT and enable MHL mode through Device Discovery and CBUS Information Exchange with the CBUS Source board.

Record the single-ended impedance increment ZSINCREMENT_A from TCABLESTART to TCABLEEND.

Calculate ZSINCREMENT_B.

  - If (ZSINCREMENT_A < 0), ZSINCREMENT_B = 0.

  - If (ZSINCREMENT_A >= 0), ZSINCREMENT_B = ZSINCREMENT_A.

Set the DUT BIST Impedance_Mode to eCBUS-D TX LOW and eCBUS-D Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the single-ended impedance ZSTHROUGH1_HIGH from TIN to TCABLESTART and ZSTHROUGH2_WITHCABLE_HIGH from TCABLEEND to TTERM.

Calculate ZSTHROUGH2_HIGH = ZSTHROUGH2_WITHCABLE_HIGH – ZSINCREMENT_B.

Measure the single-ended impedance ZST_WITHCABLE_HIGH from TTERM to TTERM + 3 ns.

Calculate ZST_HIGH = ZST_WITHCABLE_HIGH – ZSINCREMENT_B.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from TDR Scope or VNA-base TDR to the CBUS Source.

Set the DUT BIST Impedance_Mode to eCBUS-S TX LOW and eCBUS-S Rx using the CBUS Source.

Trigger the BIST.

Switch the RF relay path in the RELT board to change the DUT eCBUS-S connection from the CBUS Source to TDR Scope or VNA-base TDR.

Measure the single-ended impedance ZSTHROUGH1_LOW from TIN to TCABLESTART and ZSTHROUGH2_WITHCABLE_LOW from TCABLEEND to TTERM.

Calculate ZSTHROUGH2_LOW = ZSTHROUGH2_WITHCABLE_LOW – ZSINCREMENT_B.

Measure the single-ended impedance ZST_WITHCABLE_LOW from TTERM to TTERM + 3 ns.

Calculate ZST_LOW = ZST_WITHCABLE_LOW – ZSINCREMENT_B.

Calculate ZSTHROUGH and ZST.

  - ZSTHROUGH= (ZSTHROUGH_HIGH+ ZSTHROUGH_LOW)/2

  - ZST= (ZST_HIGH+ ZST_LOW)/2

Evaluate ZSTHROUGH1, ZSTHROUGH2, and ZST.

  - If (35 ohms <= [ZSTHROUGH1, ZSTHROUGH2 ]<= 57.5 ohms) AND (42.5 ohms <= ZST <= 57.5 ohms), it is PASS

  - If (30 ohms <= [ZSTHROUGH1, ZSTHROUGH2] < 35 ohms) OR (57.5 ohms < [ZSTHROUGH1, ZSTHROUGH2] <= 62.5 ohms), the impedance is in the excursion range. If an excursion occurs no more than one time for  ZSTHROUGH1 and no more than one time for ZSTHROUGH2 AND the duration of each excursion is less than 1000ps AND (42.5 ohms <= ZST <= 57.5 ohms), it is PASS.

  - All other cases are FAIL.

## MHL 3 System Tests

This section defines tests unique to MHL 3 devices connected in eCBUS-S or eCBUS-D mode, using AVLink or CBUS to transfer data in protocols not supported by MHL 1 / MHL 2 devices.

All MHL 3 devices have a legacy MHL1/ MHL2 compatibility mode test requirement as defined in Section 5.2.  Refer to MHL CTS 2.1 test procedures for the specific test steps.  New tests for MHL 3 are defined in Section 5.8.  The general Required Methodology for all Dongle DUT tests with normative references to CTS 2.1 are tested in oCBUS mode, in MHL 3 mode;  eCBUS-S and, if supported by the DUT, eCBUS-D. Test results for each mode sequence will be recorded TESTxxxx- S for eCBUS-S mode, and TESTxxxx-D for eCBUS-D mode (if supported).  If eCBUS-D is not supported, then the result for this test sequence will be PASS (SKIP).  The general Required Methodology are defined below.

General Required Methodology

- Connect DUT to test MHL 3 Dongle test equipment using MHL 3 cable in eCBUS-S mode.  Execute tests listed in Section 5.8.

Execute TESTxxxx and record results as PASS or FAIL for TESTxxxx-S.

Connect DUT to test MHL 3 Source test equipment using MHL 3 cable in eCBUS-D mode if supported.  Repeat execution of tests in Section 5.8.

If eCBUS-D mode is not supported, result is PASS (SKIP).  If eCBUS-D mode is supported,  execute TESTxxxx and record results as PASS or FAIL for TESTxxxx-D.

Connect DUT to test MHL 2 Source test equipment using MHL 2 cable in oCBUS mode.  Execute tests in section 5.2 of MHL1/ MHL 2 CTS.

Execute TESTxxxx and record results as PASS or FAIL for TESTxxxx.

When required, the tester must follow proper WRITE_BURST protocol including but not limit to having correct BURST_IDs, WRT_REQ, GRT_WRT handshaking, and their timings in both MHL3 and MHL1 /MHL2 tests.

If  all test sequences pass, overall test result is PASS, else FAIL.

Table 5-7. Dongle AVLINK Test Sequence

| Test ID | MHL3 Mode eCBUS-S | MHL3 Mode eCBUS-D |
| --- | --- | --- |
| 5.8.1.1 | X | If DUT Supports |
| 5.8.2.1 | X | If DUT Supports |
| 5.8.2.2 | X | If DUT Supports |
| 5.8.2.3 | X | If DUT Supports |
| 5.8.3.1 | X | If DUT Supports |
| 5.8.3.2 | X | If DUT Supports |
| 5.8.3.3 | X | If DUT Supports |
| 5.8.3.4 | X | If DUT Supports |
| 5.8.3.5 | X | If DUT Supports |
| 5.8.3.6 | X | If DUT Supports |
| 5.8.3.7 | X | If DUT Supports |
| 5.8.4.1 | X | If DUT Supports |
| 5.8.5.1 | X | If DUT Supports |
| 5.8.5.2 | X | If DUT Supports |
| 5.8.6.1 | X | If DUT Supports |
| 5.8.7.1 | X | If DUT Supports |
| 5.8.7.2 | X | If DUT Supports |
| 5.8.7.3 | X | If DUT Supports |
| 5.8.7.4 | X | If DUT Supports |
| 5.8.7.5 | X | If DUT Supports |
| 5.8.7.6 | X | If DUT Supports |
| 5.8.7.7 | X | If DUT Supports |
| 5.8.7.8 | X | If DUT Supports |
| 5.8.8.1 | X | If DUT Supports |
| 5.8.8.2 | X | If DUT Supports |
| 5.8.9.1 | X | If DUT Supports |
| 5.8.10.1 | X | If DUT Supports |

RX BIST

AVLink RX BIST Test

Confirm that the Dongle DUT supports required RX BIST and check functionality.

- Connect DUT to MHL AVLINK Generator and initiate discovery.

Source Tester to issue BIST_SETUP Command with the following values:

  - AV Link_DATA_RATE: Use the highest AV Link data rate supported by Dongle DUT.

  - AV Link Pattern : Fixed8.

  - AV Link_VIDEO MODE:  using the highest video mode supported by Dongle DUT that is indicated in the CDF.

  - AV Link Duration = 0x01  (Minimum # of Frames/ Time).

  - AV Link Fixed H  8’b10000010  (for bitwire inverse, for Fixed10 pattern).

  - AV Link Fixed L:

    - 8’b11110101 (for Fixed10 pattern)

    - 8'b00000000 (for Fixed8 pattern)

  - AV Link Randomizer

    - 0x01 (for Fixed8 pattern)

    - 0x00 (otherwise)

  - All other parameters are programed to 0x00 for this command (offset: 0x03 to 0x07, and 0x0F).

Fail if BIST_READY command is not received by Source Tester before TBIST_SETUP[MAX).

Source Tester to examine the received BIST_READY command.

If AV LINK ERROR bit is equal to 1 then Pass[SKIP]  (Tester Error).

Fail if AV LINK Ready bit is not 1.

Source Tester to send BIST_TRIGGER command with only AV LINK RX BIST enabled (other bits = 0).

Source Tester to send 33 Frames  or 1.1 second of Video data (depending on BIST pattern).

Source Tester to issue BIST_REQUEST_STAT command.

Source Testet to examine BIST_RETURN_STAT data.  Fail if AV_LINK_STAT_H and AV_LINK_STAT_L are not equal to 0x00.

Continue step 2 with Fixed10 pattern, and with PRBS pattern if CDF_AVLINK_BIST_PRBS_SUPP field is YES.

If BIST patterns Pass, then PASS, else FAIL.

Video Tests

Additional  Video Formats in 24-bits-per-pixel Mode

Verify that Dongle DUT supports the video formats with no distortion.

Refer to the following table and perform tests using video modes specified in CDF. Use ramp pattern.

| VIC | Formats | Field Rate (Hz) | Picture Aspect Ratio | Source | Bits/Pixel | Data Rate (Gbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 31 | 1080p50 | 50 | 16:9 | CEA-861F | 24 |  |
| 16 | 1080p60 | 60 | 16:9 | CEA-861F | 24 |  |
| M5 | 1080p48w | 48 | 21:9 |  | 24 |  |
| 89 | 1080p50w | 50 | 64:27 |  | 24 |  |
| 86 | 1080p24w | 24 | 64:27 |  | 24 |  |
| 87 | 1080p25w | 25 | 64:27 |  | 24 |  |

- If any of the following video formats are supported CDF_VIDEO_1080p_50,  CDF_VIDEO_1080p_60, CDF_VIDEO_1080p_48w, CDF_VIDEO_1080p_50w, CDF_VIDEO_1080p_24w, CDF_VIDEO_1080p_25w,continue with test, else PASS(SKIP).

Configure MHL Signal Generator to send out test patterns in one of the video formats in the CDF.

If DUT displays the patterns with no distortion, then continue to test, else FAIL.

Repeat the test from step 2 for all video formats indicated in the CDF.

If none of the video format fails, then PASS.

Additional  Video Formats in 16-bits-per-pixel Mode

Verify that Dongle DUT supports the video formats with no distortion.

Refer to the following table and perform tests using video modes specified in CDF. Use ramp pattern.

| VIC | Formats | Field Rate (Hz) | Picture Aspect Ratio | Source | Bits/Pixel | Data Rate (Gbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 64 | 1080p100 | 100 | 16:9 | CEA-861F | 16 | 5.94 |
| 63 | 1080p120 | 120 | 16:9 | CEA-861F | 16 | 5.94 |
| 95 | 2160p30 | 30 | 16:9 |  | 16 | 5.94 |
| 94 | 2160p25 | 25 | 16:9 |  | 16 | 5.94 |
| 93 | 2160p24 | 24 | 16:9 |  | 16 | 5.94 |
| 98 | 2160p24 | 24 | 256:135 |  | 16 | 5.94 |
| 99 | 2160p25 | 25 | 256:135 |  | 16 | 5.94 |
| 100 | 2160p30 | 30 | 256:135 |  | 16 | 5.94 |
| M4 | 2160p24w | 24 | 21:9 |  | 16 | 5.94 |
| M12 | 2160p25w | 25 | 21:9 |  | 16 | 5.94 |
| M6 | 1080p96w | 96 | 21:9 |  | 16 | 5.94 |

- If any of the following formats are supported,  CDF_VIDEO_16BIT_1080p_100, CDF_VIDEO_16BIT_1080p_120,  CDF_VIDEO_16BIT_2160p_30, CDF_VIDEO_16BIT_2160p_25 CDF_VIDEO_16BIT_2160p_24, CDF_VIDEO_16BIT_2160p_25w,  CDF_VIDEO_16BIT_2160p_24w,  CDF_VIDEO_16BIT_1080p_96w continue with test, else PASS(SKIP).

Configure MHL Signal Generator to send out test patterns in one of the video formats in the CDF.

If DUT displays the patterns with no distortion, then continue to test, else FAIL.

Repeat the test from step 2for all video formats indicated in the CDF.

If none of the video format fails, then PASS.

High End  Video Formats

Verify that Dongle DUT supports the video formats with no distortion.

- If CDF_PRODUCT_HEV_CAPABLE is NO then PASS(SKIP).

If CDF_VIDEO_HEV is NO then FAIL.  Else, continue to test.

Tester to read Dongle DUT’s EDID.

Tester to send SET_INT(FEAT_REQ).

If WRITE_BURST from Dongle is not received within TGRT_BURST{max} then FAIL.

If SET_INT(FEAT_COMPLETE) from Dongle is not received within TSINK_FEAT_COMPL{max} then FAIL.

Tester to compare Dongle DUT’s EDID and Dongle DUT WRITE_BURST data for HEV support modes.

FAIL if both EDID and WRITE_BURST data have identical HEV information (e.g. EDID and WRITE_BURST both have VIC value of 95 for 2160p30).

Compare list of supported modes (either in EDID or in the WRITE_BURST data) to CDF. If there is a mismatch, then FAIL. Or if the list does not have all the modes as indicated in table 4-8 then FAIL.

In sequential order of  HEVD configure test equipment to send out the corresponding video mode that are in Table 4-8, High-End Format Support Requirement. If DUT displays the video pattern with no distortion, then continue to test, else FAIL.

Repeat the test from step 10 for all video formats indicated.

If all video formats in in the list pass, then PASS, else FAIL.

Audio Test

IEC 60958 / IEC61937 for Audio Support

Verify that Dongle converts audio properly.

- If one of the CDF_AUDIO_2CH_32kHz,  CDF_AUDIO_2CH_44kHz,  CDF_AUDIO_2CH_48kHz fields in the CDF is set to NO, then FAIL, else continue.

Configure the MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and one of audio format such as PCM_2CH_192kHz, as indicated in the CDF.

Configure the MHL Signal Generator to send out  1 kHz sine wave.

If DUT (or A/V Display with DUT attached) regenerates the 1 kHz sound with no distortion, then continue the test, else FAIL.

Repeat the test from step 2 for other sampling frequency that is indicated in the CDF as in step 1.

If any of the Audio formats fails the test, then FAIL, else PASS.

Audio Clock Regeneration for Audio Support

Verify that Dongle properly regenerate audio when different Audio Clock Regeneration parameters are used.

- If one of the CDF_AUDIO_2CH_32kHz,  CDF_AUDIO_2CH_44kHz,  CDF_AUDIO_2CH_48kHz fields in the CDF is set to NO, then FAIL, else continue.

Configure the MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and one of audio format such as PCM_2CH_192kHz, as indicated in the CDF.

Configure MHL Signal Generator to send out the test patterns with 1kHz sine wave using minimum N parameter.

  - If A/V Display with DUT attached reproduces 1kHz sine wave with no distortion, then continue to test, else FAIL.

Configure MHL Signal Generator to send out test patterns with 1kHz sine wave using maximum N parameter.

  - If A/V Display with DUT attached reproduces 1kHz sine wave with no distortion, then continue to test, else FAIL.

  - The corresponding audio format selection is under the "Audio Format" pull-down menu.

Repeat the test from step 2 for other sampling frequency that is indicated in the CDF as in step 1.

If any of the Audio modes fails the test, then FAIL, else PASS.

IEC 60958 / IEC61937 while in 16-bits-per-pixel Mode

Verify that Dongle, while in 16-bits-per-pixel mode, converts audio properly.

- If CDF_VIDEO_16BIT field in the CDF indicates no support for 16-bits-per-pixel mode then end test with PASS (SKIP).

If any of CDF_AUDIO_2CH_32kHz, CDF_AUDIO_2CH_44kHz, CDF_AUDIO_2CH_48kHz, CDF_AUDIO_2CH_88kHz, CDF_AUDIO_2CH_96kHz, CDF_AUDIO_2CH_176kHz , CDF_AUDIO_2CH_192kHz field in the CDF is YES, then continue to test, else end test with PASS (SKIP).

Configure MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and one of PCM_2CH_32kHz, PCM_2CH_44.1kHz,PCM_2CH_48kHz, PCM_2CH_88.1kHz, PCM_2CH_96kHz, PCM_2CH_176kHz, PCM_2CH_192kHz modes and any as indicated in the CDF.

Configure the MHL Signal Generator to send out  1 kHz sine wave.

If DUT (or A/V Display with DUT attached)  regenerates the 1 kHz sound with no distortion, then continue the test, else FAIL.

Repeat the test from step 3 for all remaining audio modes indicated.

If any of the Audio formats fails the test, then FAIL, else PASS.

Audio Clock Regeneration while in 16-bits-per-pixel Mode

Verify that Dongle, while in 16-bits-per-pixel mode, properly regenerate audio when different Audio Clock Regeneration parameters are used.

- If CDF_VIDEO_16BIT field in the CDF indicates no support for 16-bits-per-pixel mode then end test with PASS (SKIP).

If any of CDF_AUDIO_2CH_32kHz, CDF_AUDIO_2CH_44kHz, CDF_AUDIO_2CH_48kHz, CDF_AUDIO_2CH_88kHz, CDF_AUDIO_2CH_96kHz, CDF_AUDIO_2CH_176kHz , CDF_AUDIO_2CH_192kHz field in the CDF is YES, then continue to test, else end test with PASS (SKIP).

Configure MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and one of PCM_2CH_32kHz, PCM_2CH_44.1kHz,PCM_2CH_48kHz, PCM_2CH_88.1kHz, PCM_2CH_96kHz, PCM_2CH_176kHz, PCM_2CH_192kHz modes and any as indicated in the CDF.

Configure MHL Signal Generator to send out the test patterns with 1kHz sine wave using minimum N parameter.

  - If A/V Display with DUT attached reproduces 1kHz sine wave with no distortion, then continue to test, else FAIL.

Configure MHL Signal Generator to send out test patterns with 1kHz sine wave using maximum N parameter.

  - If A/V Display with DUT attached reproduces 1kHz sine wave with no distortion, then continue to test, else FAIL.

Repeat the test from step 2 for all remaining audio modes indicated.

If any of the Audio modes fails the test, then FAIL, else PASS.

High-Bitrate Audio

Verify that Dongle supports the High-Bitrate Audio formats and reproduces audio properly.

- If CDF_AUDIO_HBR field in the CDF is NO then end test with PASS (SKIP).

Configure the MHL Signal Generator output a Video format, such as 720p60,  that Dongle can support that carries HBR signal such Dolby TrueHD.

Perform listening test (with DUT or A/V Display attached DUT)

If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

Repeat the test from step 2 for other HBR format if it is indicated in the CDF.

If any of the Audio formats fails the test, then FAIL, else PASS.

High-Bitrate in 16-bits-per-pixel Mode

Verify that Dongle supports the High-Bitrate Audio formats and reproduces audio properly while in 16-bits-per-pixel Mode

- If CDF_VIDEO_16BIT in the CDF is NO then end test with PASS (SKIP).

If CDF_AUDIO_2CH_32kHz fields in the CDF is NO then end test with PASS (SKIP).

Configure the MHL Signal Generator output a Video format, such as 1080p60, that Dongle can support that carries HBR signal such Dolby TrueHD.

Perform listening test (with DUT or A/V Display attached DUT)

If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

Repeat the test from step 2 for other HBR format if it is indicated in the CDF.

If any of the Audio formats fails the test, then FAIL, else PASS.

Audio Data Tunneling Receiving Support

Verify that Dongle DUT is capable of receiving Audio via  Audio data tunneling.

- If the CDF_ADT_RECEIVE_SUPPORT is set to YES, then continue else end test with PASS (SKIP).

- Tester Source sends SET_INT(FEAT_REQ) to Dongle DUT.

- Tester Source allows Dongle DUT to execute WRITE_BURST command by responding to REQ_WRT interrupt with GRT_WRT interrupt.

- If WRITE_BURST command from Dongle DUT is not arriving within TGRT_BURST(max) then FAIL.

- Examine the WRITE_BURST data.  If basic audio (32khz, 44.1khz and 48khz) is not supported then FAIL.

- Tester to reject (by responding with 0x01,0x02,0x03) any attempt to do TDM Allocation by DUT.

- Tester uses section 13.10 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation that has Feature_ID=0x02 and has enough bandwidth to carry ADT data as indicated in the CDF.

- FAIL if DUT does not respond by indicating 0x00 (Accepted).

- Configure Source Tester to basic ADT Audio ( 32khz, 44.1khz and 48khz) formats, plus the corresponding audio InfoFrame packet, audio regeneration packet.  Note that Audio InfoFrame and Audio Clock Regeneration packets transmission shall be sent at least once each TAIF0.

- If CDF_ADT_PLAYING_SUPPORT is YES, perform listening test based on CDF_ADT_RECEIVE_PROC, else skip to step 12.

- If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

- If any of Audio formats fails, then FAIL, else PASS.

Audio Data Tunneling Transmitting Support

Verify that Dongle is capable of Transmitting Audio via  Audio data tunneling.

- If CDF_ADT_SEND_SUPPORT is set to YES, then continue else end test with PASS (SKIP).

- Tester to wait for SET_INT(FEAT_REQ) from Dongle DUT.  Fail if this command is not sent within TSRC_FEAT_REQ{max}.

- Tester, using WRITE_BURST, to inform DUT of the support of basic audio (32khz, 44.1khz and 48khz) and any audio format in the CDF.

- Tester to reject (by responding with 0x01,0x02,0x03) any attempt to do TDM Allocation by DUT.

- Tester uses Section 13.10 “Assignment of Virtual Channels” method to direct the DUT to use a new TDM Allocation that has Feature_ID=0x02 and has enough bandwidth to carry Audio data as indicated in the CDF.

- FAIL if DUT does not respond by indicating 0x00 (Accepted).

- Configure Tester to enable its TMDS (such as sending any video format).

- Configure DUT to send out all Basic ADT Audio (32khz, 44.1khz and 48khz) formats, based on CDF_ADT_SEND_PROC.

- If the Audio InfoFrame and VSIF are not coming from the DUT at least once every 2 fields, then FAIL.

- Examine the Audio InfoFrame and VSIF data (including but not limit to Channel Status, Channel Count, AV_DELAY_SYNC…). If they do not match the Audio mode being sent, then FAIL.

- If Audio Regeneration packet is also sent by the DUT, verify the N and CTS value and their relationship with the link clock of the current audio format according to Section 14.2.1 (Audio Data Tunneling and Audio Clock Regeneration).  FAIL if the relationship and the restriction are incorrect.

- If CDF_ADT_PLAYING_SUPPORT is YES, perform listening test, else skip to step 14.

- If no sound, extraneous sound (e.g. clacking sound), or unnecessary mute (e.g. short term mute, etc) then FAIL.

- Configure Tester to disable TMDS output, then repeat the test from Step 8.

- If all ADT formats specified in the CDF pass, then PASS, else FAIL.

HDCP2.2 Test

HDCP2.2

Verify that Dongle complies with HDCP2.2 Specification.

- If the CDF_HDCP_SUPPORT field in CDF is YES, then continue to test, else end test with PASS (SKIP).

Follow the HDCP2.2-ON-MHL CTS procedure. If the CDF_HDCP_REPEATER field in the CDF is YES, then include all HDCP Repeater tests.

If all tests in the HDCP2.2-ON-MHL CTS procedure pass, then PASS; else FAIL.

*Note: HDCP test tool must support clock stretch.

RBP Sub-Command Tests

The test implementation shall include UTF character sequences for all lengths of characters (1 to 4 8-bit bytes or octets), with both invalid and valid values in each of those lengths.

RBP Sub-Commands Receiving Test

Verify that DUT responds to valid RBP sub-commands.

- If CDF indicates CDF_RBP_RECV_SUPPORT is YES then continue test, else PASS (SKIP).

Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence

Determine supported button codes from CDF and test each for reception of proper code and that RBPK is received after each command from the DUT

Repeat step 3 for each code identified in the CDF.

Tester asks operator to confirm the operation was performed in response to the code received by the DUT.

From the CDF determine if any unsupported codes exist and send that code to force and RBPE opcode 0x01 (ineffective code)

If DUT passes all iterations of the test, then PASS. Else FAIL.

RBP Sub-Commands Transmitting Test

Verify that DUT sends valid RBP sub-commands.

- If CDF indicates CDF_RBP_SEND_SUPPORT is YES then continue test, else PASS (SKIP).

Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence

Determine supported button codes from CDF and test each for transmission and correct reception of the code by the test equipment and reception of  RBPK is received after each command from the DUT.

Repeat step 3 for each code identified in the CDF

From the CDF list of supported codes, the tester picks one to not support so that an unsupported code condition exists and tester responds with RBPE opcode 0x01 (ineffective code).  Ask test operator to confirm the DUT exhibits the proper error handling such as displying an error message.

If DUT passes all iterations of the test, then PASS. Else FAIL.

ATT Subcommand Test

The ATT MSC subcommand is new for MHL 3 and is used in conjunction with Multi-View support.

ATT Subcommand Test

Verify functionality and support of ATT MSC subcommand used in Multi-View support.

- Connect DUT to MHL3  Protocol/ System Analyzer and perform discovery sequence.

Tester to send SET_INT{FEAT_REQ}.

FAIL if WRITE_BURST from Dongle does not arrive within TGRT_BURST{max}.

FAIL if SET_INT{FEAT_COMPLETE} does not arrive within TSINK_FEAT_COMPL{max}.

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT.

If the DUT’s VDI_L[5:0]  are all ‘0’, and CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and CDF_VIDEO_MULTIVIEW field is NO; then PASS (SKIP), else continue.

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT. It is necessary for the tester to record in a variable the support status for Multi-View indicated by the DUT in the VDI field of its WRITE_BURSTs of HEV video modes. If any VDI field indicates support for Multi-View, then the ATT command shall be accurately supported. If no VDI field indicates support for Multi-View, then the ATT command shall not be sent to the DUT by a connected Source. (Refer to MHL 3 Specification, Sections 5.11, and 7.9.1.)

  - If DUT's VDI_L[5:0] are not all '0', then DUT is indicating some supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to YES.

  - If DUT's VDI_L[5:0] are all '0', then DUT is indicating no supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to NO.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and MEAS_VIDEO_MULTIVIEW is NO; then PASS (SKIP), else continue.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is YES, and MEAS_VIDEO_MULTIVIEW is YES, then continue, else FAIL.

Invoke DUT to send ATT command.

If DUT ATT command is successful,  test equipment shall respond  with ATTK.

If  TATT_WAIT{max} elapses and no further commands are sent then FAIL, else continue.

If transaction completes without error then continue, else FAIL.

Tester to send ATT command.

FAIL if DUT does not respond with ATTK before TATT_WAIT{max} elapses.

If transaction completes without error then PASS, else FAIL.

AVLINK Packetization Tests

The AVLINK packetization tests are specific to MHL3 and are defined to verify low level AVLINK link compliance.  These tests are not defined for legacy MHL compatibility mode.

AVLINK Speed Test

Verify functionality and link speeds supported by Dongle DUT.

- Repeat the following steps for each speed advertised in the CDF in the variables CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600 as YES. At least one must be supported. If any other speed is not supported as indicated in the CDF, that iteration will receive a PASS(SKIP).

- The Tester advertises each supported CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600  in its TMDS_SPEEDS register in numerical order starting with TMDS_150.

- The Dongle DUT is connected to the MHL 3 Tester.

- Verify that Dongle DUT TMDS_SPEEDS register matches the CDF_LINK_SPEED_150,  CDF_LINK_SPEED_300,  CDF_LINK_SPEED_600  values.

- If the TMDS_SPEEDS register is not correct, then FAIL.

- The Tester writes the Dongle LINK_RATE register to indicate the speed being tested.

- Check that the Dongle DUT writes to the Tester Source AVLINK_MODE_STATUS register indicating TMDS_NORMAL to verify that Dongle DUT has connected at the correct speed.

- If the DUT does not indicate TMDS_NORMAL within 10 seconds, then end the test with FAIL.

- If DUT passes all iterations of the test, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; Short Packets

Verify functionality of the Dongle DUT AVLINK when it receives many small AV TMDS packets.

- The Tester Source connects to the Dongle DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized into as many 1-byte-payload packets as possible, while respecting the packet jitter and buffer model rules relating to NACC_SYMBOL_DIFF{max}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; long packets

Verify functionality of the Dongle DUT AVLINK when it receives maximum sized AV TMDS packets.

- The Tester Source connects to the Dongle DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized into as many 252-byte-payload packets as possible, while respecting the packet jitter and buffer model rules relating to NACC_SYMBOL_DIFF{max}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; Randomizer Reset Alignment

Verify functionality of the Dongle DUT AVLINK when it receives RR sequences in or around AV packets.

- The Tester Source connects to the Dongle DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized so that all locations for groups of 4 RR’s in Table 5-8 are inserted at the indicated location once every TRR{min}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

Table 5-8. RR Insertion Alignment

| Feature | RR Group Location |
| --- | --- |
| 1 | Between SOP and Header 0 |
| 2 | Between Header 0 and Header 1 |
| 3 | Between Header 1 and Header 2 |
| 4 | Between Header 2 and Data 0 |
| 5 | Between Data 0 and Data 1 |
| 6 | Between the last Data and EOP |
| 7 | Immediately before SOP |
| 8 | Immediately after EOP |
| 9 | Between a pair of GAP symbols |

AVLINK Packetization Basic Protocol; Slow Randomizer Resets

Verify functionality of the Dongle DUT AVLINK when it receives RR sequences with maximum spacing.

- The Tester Source connects to the Dongle DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized so that a group of 4 RR’s are inserted once every TRR{max}.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Basic Protocol; Buffer Model Timing

Verify functionality of the Dongle DUT when AVLINK Data is delayed.

- The Tester Source connects to the Dongle DUT using TMDS_150.

- Command Tester Source to send 720x480p 59.94/60Hz or 720x576p 50Hz video.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized to control data jitter.

    - During each Vertical Retrace and Horizontal Retrace interval, each AV packet is sent starting when its last data item becomes available.

    - The first 252 bytes of data from the beginning of each Video Data period are sent in an AV packet starting when its last data item becomes available.

    - The data from the middle of each Video Data period are sent as AV packets.  GAP symbols are added so that each AV packet is started at the time its last data item becomes available PLUS an additional delay of 256 Video Content Symbol times, consistent with the packet jitter and buffer model rules relating to NACC_SYMBOL_DIFF{max}.

    - The final 253 to 504 bytes of data from the end of each Video Data period are sent as 2 AV packets starting when their last data items become available.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Extended Content Protocol

Verify functionality when Extended Content packets are received by the Dongle DUT.

- If CDF_EXTENDED_CONTENT_SUPPORT is NO, then end test with PASS (SKIP).

- The Tester Source connects to the Dongle DUT using the TMDS Link speed needed to force Extended Content Packetization.

- Command Tester Source to send AV data with the format specified in CDF_EXTENDED_CONTENT_SUPPORT.

  - The first 4 symbols sent must be RR’s.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

- If no failures are detected, then PASS. Else FAIL.

AVLINK Packetization Extended Content Protocol; Buffer Model Timing

Verify functionality of the Dongle DUT when Extended Content AVLINK Data is delayed.

- If CDF_EXTENDED_CONTENT_SUPPORT is NO, then end test with PASS (SKIP).

- The Tester Source connects to the Dongle DUT using the TMDS Link speed needed to force Extended Content Packetization.

- Command Tester Source to send AV data with the format specified in CDF_EXTENDED_CONTENT_SUPPORT.

  - The first 4 symbols sent must be RR’s.

  - Data is packetized to control data jitter.

    - During each Vertical Retrace and Horizontal Retrace interval, each AV packet is sent starting when its last data item becomes available.

    - Starting from line 0, even numbered Scan Lines are sent as Extended Content packets starting when enough data is available to let them complete.

    - Starting from line 1, odd numbered Scan Lines are sent as Extended Content packets starting when enough data is available to let them complete PLUS an additional delay of 256 Video Content Symbol times.

- Examine the displayed AV data for 5 seconds.  If Video or Audio errors are detected by the operator, then FAIL.

If no failures are detected, then PASS. Else FAIL.

3D, Multi-View and Dual-3D

Table 5-9. 3D and Multi-View Testing Configurations

| Configuration | VDI_L Settings |  |  | Description (3D/Multiview mode support) |
| --- | --- | --- | --- | --- |
|  | VDI_L[6] | VDI_L[5:3] | VDI_L[2:0] |  |
| 1 | 0 | 0 | 0 | No Multi-View or 3D support |
| 2 | 0 | 0 | Non-zero | 3D |
| 3 | 0 | Non-zero | 0 | 4-View |
| 4 | 0 | Non-zero | Non-zero | 4-View, 2-View, 3D |
| 5 | 1 | 0 | 0 | None (invalid setting) |
| 6 | 1 | 0 | Non-zero | 2-View, 3D |
| 7 | 1 | Non-zero | 0 | 4-View, Dual3D |
| 8 | 1 | Non-zero | Non-zero | 4-View, 2-View, Dual3D, 3D |
| - Note1: VDI_L[7] = 1 to indicate separate audio for multiview. |  |  |  |  |
| - Note2: VDI_L[2:0] is for LR,TB,FS_SUPP respectively. |  |  |  |  |
| - Note3: VDI_L[5:3] is for TBLR,FSLR, FSTB respectively. |  |  |  |  |

3D Support

Verify 3D functionality and support of Dongle DUT.

- Connect DUT and perform Discovery

Tester to send SET_INT(FEAT_REQ)

FAIL if WRITE_BURST from Dongle does not arrive within TGRT_BURST{max}.

FAIL if SET_INT(FEAT_COMPLETE) does not arrive within TSINK_FEAT_COMPL{max}.

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT. It is necessary for the tester to record in a variable the support status for Multi-View indicated by the DUT in the VDI field of its WRITE_BURSTs of HEV video modes. If any VDI field indicates support for Multi-View, then the ATT command shall be accurately supported. If no VDI field indicates support for Multi-View, then the ATT command shall not be sent to the DUT by a connected Source. (Refer to MHL 3 Specification, Sections 5.11, and 7.9.1.)

  - If DUT's VDI_L[5:0] are not all '0', then DUT is indicating some supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to YES.

  - If DUT's VDI_L[5:0] are all '0', then DUT is indicating no supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to NO.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and MEAS_VIDEO_MULTIVIEW is NO; then PASS (SKIP), else continue.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is YES, and MEAS_VIDEO_MULTIVIEW is YES, then continue, else FAIL.

If the DUT’s VDI_L[2:0]  are NOT all ‘0’ and each bit matches its corresponding CDF field as below then continue to test, else FAIL.

  - VDI_L[0]=’1’ if CDF_VIDEO_FS_SUPP field is not empty, else = ‘0’.

  - VDI_L[1]=’1’ if CDF_VIDEO_TB_SUPP field is not empty, else = ‘0’.

  - VDI_L[2]=’1’ if CDF_VIDEO_LR_SUPP field is not empty, else = ‘0’.

In the sequential order of the descriptors, Tester to send out video modes as indicated in MHL spec Table 5-10 (MHL Sink 3D Video Mode Support Requirements), plus any additional modes indicated in the CDF with correct AVI, VSIF (MHL_VID_FMT, MHL_3D_FMT_TYPE…). Operator to verify that video on each downstream sink device is displayed properly.

Repeat test from step 9 for each resolution listed.

If DUT passes all iterations of the test, then PASS, else FAIL.

Multi-View Support

Verify Multi-View/Dual-3D functionality and support of Dongle DUT.

- Connect DUT and perform Discovery.

Tester to send SET_INT(FEAT_REQ).

FAIL if WRITE_BURST from Dongle does not arrive within TGRT_BURST(max).

FAIL if SET_INT(FEAT_COMPLETE) does not arrive within TSINK_FEAT_COMPL(max).

Tester to examine the WRITE_BURST data (especially the VDI’s) to determine capabilities of the DUT. It is necessary for the tester to record in a variable the support status for Multi-View indicated by the DUT in the VDI field of its WRITE_BURSTs of HEV video modes. If any VDI field indicates support for Multi-View, then the ATT command shall be accurately supported. If no VDI field indicates support for Multi-View, then the ATT command shall not be sent to the DUT by a connected Source. (Refer to MHL 3 Specification, Sections 5.11, and 7.9.1.)

  - If DUT's VDI_L[5:0] are not all '0', then DUT is indicating some supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to YES.

  - If DUT's VDI_L[5:0] are all '0', then DUT is indicating no supported Multi-View:

    - Set MEAS_VIDEO_MULTIVIEW to NO.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is NO, and MEAS_VIDEO_MULTIVIEW is NO; then PASS (SKIP), else continue.

If CDF_PRODUCT_MULTIVIEW_CAPABLE field is YES, and MEAS_VIDEO_MULTIVIEW is YES, then continue, else FAIL.

If the DUT’s VDI_L[5:0] are NOT all ‘0’ and each bit matches its corresponding CDF field as below then continue to test, else FAIL.

  - VDI_L[0]=’1’ if CDF_VIDEO_FS_SUPP field is not empty, else = ‘0’.

  - VDI_L[1]=’1’ if CDF_VIDEO_TB_SUPP field is not empty, else = ‘0’.

  - VDI_L[2]=’1’ if CDF_VIDEO_LR_SUPP field is not empty, else = ‘0’.

  - VDI_L[3]=’1’ if CDF_VIDEO_FSTB_SUPP field is not empty, else = ‘0’.

  - VDI_L[4]=’1’ if CDF_VIDEO_FSLR_SUPP field is not empty, else = ‘0’.

  - VDI_L[5]=’1’ if CDF_VIDEO_TBLR_SUPP field is not empty, else = ‘0’.

  - VDI_L[6]=’1’ if CDF_VIDEO_DUAL3D field is not empty, else = ‘0’.

  - VDI_L[7]=’1’ if CDF_VIDEO_SEP_AUD field is YES, else = ‘0’.

In the sequential order of the descriptors, Tester to send out as below, and operator to verify that video, audio on each downstream sink device are being produced properly.

  - Video mode, starting with a 2View mode if DUT supports,  with correct AVI, VSIF (MHL_VID_FMT, MHL_3D_FMT_TYPE…).

  - And with separate audio for each views If VDI_L[7] = ‘1’ (else, one audio stream for the whole multi-view).

Repeat test from step 9 for each resolution listed in the order below.

  - All 2View modes.

  - All 4View modes

  - All Dual3D modes.

If DUT passes all iterations of the test, then PASS, else FAIL.

AV SYNC

AV SYNC TEST

Verify functionality Dongle AV Sync function.

- If CDF_AV_SYNC_SUPPORTED is YES  then continue to test, else PASS(SKIP).

Connect DUT and perform Discovery.

Configure the MHL Signal Generator to output 720x480p 59.94/60Hz or 720x576p 50Hz and PCM_2CH_44kHz audio with 1kHz sine wave test tone. Source VSIF indicates no audio delay.

Wait 10 seconds for DUT to send Write Burst request for Audio Delay Correction to the Source (Test Equipment).  After 10 second timeout, if no Audio delay correction request is made, continue to step 9, else skip to step 11.

Source device (test Equipment)  sends VSIF with AV_DELAY_SYNC field set to maximum value (0xFFFFF)which equals 1.04 seconds.

Wait 10 seconds for DUT to send Write Burst request for Audio Delay Correction to the Source (Test Equipment).  After 10 second timeout, if no Audio delay correction request is made a second time then FAIL test.

Analyze Audio Delay Data Structure for valid data, if burst data is invalid then FAIL, else continue.

Source device (test Equipment)  responds with requested delay.If DUT properly reqested Audio delay from source, and all steps passed, then PASS.

EDID

EDID Read Test

Verify that the Dongle EDID is accessible and accurate.

- Connect MHL EDID Reader to DUT.

Wait until SET_HPD command is received.

Read Block0. If Extension Flag(byte 0x7E) is 0, then FAIL, else read all extended blocks.

If check_sum is correct, then continue to test, else FAIL.

Read Block0 and extended blocks again and compare.

If they do not match, then FAIL.

Examine EDID:

  - If all Video Modes  found in the EDID are listed in the CDF , then continue to test, else FAIL.

  - If all Pixel Encodings found in the EDID are listed in the CDF, then continue to test, else FAIL.

  - If all audio modes found in the EDID are listed in the CDF , then PASS, else FAIL.

# Tests Common to Sources, Sinks and Dongles

The following additional tests are required for Sources, Sinks and Dongles unless noted otherwise in this section.

## Electrical Tests

There are no electrical tests common to sources, sinks and dongles in this release.

## System Tests

There are no system tests common to Sources, Sinks and Dongles in this release.

## CBUS Tests

Refer to CTS 2 Section 6.3, and the test listings in Sections 3.4, 4.4, and 5.4.

## eCBUS MHL 3 Active Mode Tests

Test in this section are applied to Sources, Sinks and Dongles as identified in Sections 3.5, 4.5, 5.5, respectively. Since the test methodology does not change significantly when testing to each of these device categories, the tests are collected here.

Each test starts from a state in which it is electrically disconnected from the DUT.

The Test Equipment may choose to provide VBUS to a Source DUT when it is not running any test.

If the Test Equipment provides VBUS to the DUT while it is not testing, then the VBUS drive will be removed at the start of each test.

Notes on Default Test Setup

- The tests are done using typical electrical and timing parameters for oCBUS, and eCBUS.

- The Tester and the DUT are connected with an MHL3 eCBUS-S Cable.

- The Tester uses 8-bit eCBUS-S mode for those tests which use eCBUS-S.

- The Tester sets its MHL_VERSION register to announce that it is an MHL 3 device.

- The Tester acts and reacts like a normal CBUS or eCBUS device.

- The DUT and Tester understand the commands READ_XDEVCAP and WRITE_XSTAT.

- The Tester sends required commands within time limits specified in the standard.

Certain tests can change one or more of these default settings.

Common Routines

The routines in this section are invoked from other tests later in Section 6.4. Each of these routines establishes a connected state between the tester and the DUT. The routine returns the pass/fail status from its own steps. These are incorporated into the overall pass/fail status of each test which invokes these routines.

Since these routines are invoked at a higher level by individual tests, the tests in Section 6.4.2 are not reported in the Test Result File.

Each test in Section 6.4.2 is defined with a syntax using a routine name and parameter list. Each routine returns a result which indicates the success or failure of the routine. That result affects the overall test result of the invoking test.

Tester Source Proposes to Transition to a New CBUS/eCBUS Mode.

Routine Tester_Source_Proposes_eCBUS_Mode(CBUS_MODE_UP_OR_DOWN) returns RETURN_CODE

- Tester Source sends an MSC_MSG RAP message with Action Code CBUS_MODE_UP_OR_DOWN.

If DUT replies with an MSC_MSG RAPK[NO_ERROR], return PASS.

If DUT replies with an MSC_MSG RAPK[RESPONDER_BUSY], Tester Source waits for Sink DUT to send an MSC_MSG RAP with Action Code CBUS_MODE_UP_OR_DOWN.

If Sink DUT does not reply with MSC_MSG RAPK[NO_ERROR], and also does not reply with an MSC_MSG RAPK[RESPONDER_BUSY] followed by an MSC_MSG RAP with Action Code CBUS_MODE_UP_OR_DOWN within 10 seconds, return FAIL.

Tester Source replies with an MSC_MSG RAPK[NO_ERROR].

Return PASS.

Sink DUT and Tester Source Make eCBUS Connection.

A Tester Source does a sequence of steps in order to establish an oCBUS, eCBUS-S or an eCBUS-D connection.

Routine Tester_Source_makes_eCBUS_Connection(CBUS_MODE) returns RETURN_CODE

- Set RETURN_CODE to PASS.

If Tester Source has not yet established an oCBUS connection, do oCBUS Connection steps.

Tester Source waits for a 1 Kohm pulldown Discovery resistor on CBUS to signal the Sink DUT is ready to attempt a connection using oCBUS.

Tester Source makes oCBUS Wake and Discovery pulses, and watches for Sink DUT to accept the connection.

Return FAIL if Tester Source doesn’t open an oCBUS connection within 30 seconds of starting the procedure.

Tester Source does WRITE_STAT to MHL_VERSION_STAT indicating that it supports MHL 3.

Tester Source initializes its Device Capability Registers and Extended Device Capability Registers, taking into account CBUS_MODE which is passed in.

Tester Source signals DCAP_RDY + XDEVCAP_SUPP followed by DCAP_CHG.

Tester Source declines any attempts by the DUT to change TDM Slot allocations.

Tester Source waits until Sink DUT signals DCAP_RDY + XDEVCAP_SUPP followed by DCAP_CHG.

Return FAIL if Sink DUT does not signal DCAP_RDY + XDEVCAP_SUPP followed by DCAP_CHG within T_SINK_DCAP_RDY from when Discovery completes.

If CBUS_MODE indicates that an eCBUS connection is required, do eCBUS Connection steps.

Tester Source waits until Sink DUT does WRITE_STAT to MHL_VERSION_STAT and signals XDEVCAP_SUPP (if not already done).

Return FAIL if Sink DUT does not do WRITE_STAT to MHL_VERSION_STAT and signal XDEVCAP_SUPP within T_SINK_DCAP_RDY from when Discovery completes.

Tester Source reads Sink DUT Device Capability Registers and Extended Device Capability Registers.

Return FAIL if Tester Source and Sink DUT do not both support the eCBUS mode named in CBUS_MODE.

Tester Source tries to move to eCBUS mode using Tester_Source_Proposes_eCBUS_Mode (CBUS_MODE_UP).

If RETURN_CODE is not PASS, then set test result to RETURN_CODE and end the test; else continue.

Tester Source initiates eCBUS-S or eCBUS-D mode, as indicated in CBUS_MODE.

If the DUT fails to establish the intended eCBUS connection within TeCBUS_SETUP_ATTEMPT{max}, then set RETURN_CODE to FAIL.

Return RETURN_CODE to invoking test.

Source DUT and Tester Sink Make eCBUS Connection.

A Tester Sink does a sequence of steps in order to establish an oCBUS, eCBUS-S or an eCBUS-D connection.

Routine Tester_Sink_makes_eCBUS_Connection(CBUS_MODE) returns RETURN_CODE

- Set RETURN_CODE to PASS.

If Tester Sink has not yet established an oCBUS connection, do oCBUS connection steps.

Tester Sink displays a 1 Kohm pulldown Discovery resistor on CBUS to signal the Source DUT to attempt a connection using oCBUS.

Tester Sink reacts to oCBUS Wake and Discovery pulses from the Source DUT to accept the connection.

Return FAIL if Tester Sink doesn’t open an oCBUS connection within 30 seconds of starting the procedure.

Tester Sink does WRITE_STAT to MHL_VERSION_STAT indicating that it supports MHL 3

Tester Sink initializes its Device Capability Registers and Extended Device Capability Registers, taking into account CBUS_MODE which is passed in.

Tester Sink signals DCAP_RDY + XDEVCAP_SUPP followed by DCAP_CHG.

Tester Sink declines any attempts by the DUT to change TDM Slot allocations.

Tester Sink waits until Source DUT signals DCAP_RDY + XDEVCAP_SUPP.

Return FAIL if Source DUT does not signal DCAP_RDY + XDEVCAP_SUPP within 10 seconds.

If CBUS_MODE indicates that an eCBUS connection is required, do eCBUS Connection steps.

Tester Sink waits until Source DUT does WRITE_STAT to MHL_VERSION_STAT and signals XDEVCAP_SUPP (if not already done).

Return FAIL if Source DUT does not do WRITE_STAT to MHL_VERSION_STAT and signal XDEVCAP_SUPP within 10 seconds.

Tester Sink reads Source DUT Device Capability Registers and Extended Device Capability Registers.

Return FAIL if Tester Sink and Source DUT do not both support the eCBUS mode named in CBUS_MODE.

Tester Sink waits until Source DUT sends an MSC_MSG RAP with Action Code CBUS_MODE_UP.

Return FAIL if Source DUT does not signal CBUS_MODE_UP within 10 seconds.

Tester Sink sends an MSC_MSG RAPK{NO_ERROR}.

Tester Sink initiates eCBUS-S or eCBUS-D mode, as indicated in CBUS_MODE.

If the DUT fails to establish the intended eCBUS connection within TeCBUS_SETUP_ATTEMPT{max}, then set RETURN_CODE to FAIL.

Return RETURN_CODE to invoking test.

Connect Tester to DUT

Routine Tester_makes_eCBUS_Connection(CBUS_MODE) returns RETURN_CODE

- For Source DUT, invoke Tester_Sink_makes_eCBUS_Connection(CBUS_MODE).

For Sink DUT, invoke Tester_Source_makes_eCBUS_Connection(CBUS_MODE).

For Dongle DUT, invoke Tester_Source_makes_eCBUS_Connection(CBUS_MODE).

Return RETURN_CODE to invoking test.

oCBUS MSC commands

DUT Replies to CBUS READ_XDEVCAP Command

Verify that DUT replies to the MHL3 CBUS READ_XDEVCAP command by returning correct Extended Device Capability Register data.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

Use MHL3 CDF which contains expected Extended Device Capability Register values.

- Tester and DUT establish an oCBUS connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester issues READ_XDEVCAP to read DUT Extended Device Capability Registers.

If register contents are not the expected values, CDF_XCR_ECBUS_SPEEDS, CDF_XCR_TMDS_SPEEDS, CDF_XCR_ECBUS_DEV_ROLES, CDF_XCR_LOG_DEV_MAPX, then FAIL; else PASS.

DUT Replies to the MHL3 CBUS WRITE_XSTAT Command by Returning an ACK

Verify that DUT replies to the MHL3 CBUS WRITE_XSTAT command by returning an ACK.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

- Tester and DUT establish an oCBUS  connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester issues WRITE_XSTAT with address 0x90 and data 0x00 to change DUT Extended Device Status Registers.

FAIL if DUT does not respond to the WRITE_XSTAT with an ACK.

Observe: DUT Does Not Send New MHL3 CBUS Commands to MHL 2 Tester

Verify that DUT does not send new MHL3 CBUS command(s) to a Tester which acts like an MHL 2 device.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

- Tester sets its MHL_VERSION register to advertise that it is an MHL 2 device.

Tester set to respond to READ_XDEVCAP and WRITE_XSTAT with an ABORT.

Tester and DUT establish an oCBUS  connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Observe the CBUS to see if DUT sends READ_XDEVCAP or WRITE_XSTAT command.

FAIL if DUT sends MHL3 commands while it is connected to an MHL 2 device.

Test ends after 10 seconds.  If no FAILs reported, test ends with PASS.

Observe: DUT Sends New MHL3 CBUS Command(s) to MHL 3 Tester

Verify that DUT sends new MHL3 CBUS command(s) with correct arguments.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

- Tester and DUT establish an oCBUS  connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Observe the CBUS to make sure DUT sends READ_XDEVCAP and WRITE_XSTAT command sequences correctly while in CBUS mode.

Test ends after 10 seconds, or when DUT attempts to transition to eBUS mode.  If no FAILs reported, test ends with PASS.

Observe: Source DUT Device Implements Heartbeat.

Verify that Source DUT issues required heartbeat timings.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink |  |  |  |
| Dongle |  |  |  |

- Tester and DUT establish an oCBUS connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink does not do regular CBUS activity, requiring Source DUT to need to issue Heart Beat messages.

FAIL if Source DUT does not issue GET_STATE commands often enough to meet the T_MHL12_HEART_BEAT{max} requirement.

Test ends after 10 sedonss. If no FAILs reported, then test ends with PASS; else test ends with FAIL.

Observe: Source DUT Reads Extended Device Capability Registers after it gets DCAP_RDY or DCAP_CHG

Verify that Source DUT reads Extended Device Capability Registers after it gets DCAP_RDY or DCAP_CHG.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink |  |  |  |
| Dongle |  |  |  |

- Tester and DUT establish an oCBUS  connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink issues a WRITE_STAT indicating DCAP_RDY + XDEVCAP_SUPP

FAIL if Source DUT does not re-read the Extended Device Capability Registers within T_SRC_READ_XDCAP{max} of the WRITE_STAT.

Tester Sink issues a SET_INT indicating DCAP_CHG.

FAIL if Source DUT does not re-read the Extended Device Capability Registers within T_SRC_READ_XDCAP{max} of the SET_INT.

Observe: Sink DUT Reads Extended Device Capability Registers after it gets DCAP_RDY or DCAP_CHG

Verify that Sink DUT reads Extended Device Capability Registers after it gets DCAP_RDY or DCAP_CHG.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

- Tester and DUT establish an oCBUS  connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source issues a WRITE_STAT indicating DCAP_RDY + XDEVCAP_SUPP .

FAIL if Sink DUT does not complete re-reading the Extended Device Capability Registers within T_SINK_READ_XDCAP{max} of the WRITE_STAT.

Tester issues a SET_INT indicating DCAP_CHG.

FAIL if Sink DUT does not re-read the Extended Device Capability Registers within T_SINK_READ_XDCAP{max} of the SET_INT.

DUT Handshakes Correctly when Tester Negotiates for New eCBUS TDM Slot Allocations

Verify that DUT handshakes correctly when Tester negotiates for new eCBUS TDM Slot allocations.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

- Tester and DUT establish an oCBUS  connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester sets registers, sends DCAP_RDY + XDEVCAP_SUPP and DCAP_CHG, and reads DUT registers as required.

If Tester Source receives an MSC_MSG RAP message with Action Code CBUS_MODE_UP, it responds by sending an MSC_MSG RAPK{RESPONDER_BUSY}.

Tester uses Section 13 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation where eMSC gets 1 less TDM slot than the default and T-CBUS gets 1 more TDM slot than the default.

FAIL if DUT does not respond by either indicating 0x00 (Accepted) or one of 0x01, 0x02, or 0x03 {Rejected}.

oCBUS CBUS_MODE_UP fails: Source, Sink, Dongle

Source DUT Notices Failure of oCBUS to eCBUS Transition

Verify that DUT notices when a transition from oCBUS to either eCBUS-S or eCBUS-D fails, and then disconnects and re-attempts discovery.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink |  |  |  |
| Dongle |  |  |  |

- If CDF_eCBUS_D_SUPPORT is YES, then Tester Sink advertises both eCBUS-S and eCBUS-D support in its Extended Device Capability register ECBUS_SPEEDS.

- Otherwise, Tester Sink advertises eCBUS-S support.

- Tester Sink and DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink stays in oCBUS mode. It does not establish an eCBUS-S or eCBUS-D connection.

Tester Sink waits until Source DUT sends an MSC_MSG RAP{CBUS_MODE_UP}.

FAIL if Source DUT does not send CBUS_MODE_UP within 10 seconds.

Tester Sink HIGH-Z’s the CBUS.

Tester Sink waits 2 seconds.

Tester Sink and DUT establish an oCBUS connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else return PASS.

Sink DUT Notices Failure of oCBUS to eCBUS Transition

Verify that Sink DUT notices when a transition from oCBUS to either eCBUS-S or eCBUS-D fails, and then disconnects and re-attempts discovery.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

- If CDF_eCBUS_D_SUPPORT is YES then Tester Source advertises both eCBUS-S and eCBUS-D support in its Extended Device Capability register ECBUS_SPEEDS.

- Otherwise, Tester Source advertises eCBUS-S support.

- Tester and DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source stays in oCBUS mode. It does not establish an eCBUS-S or eCBUS-D connection.

Tester_Source_Proposes_eCBUS_Mode(CBUS_MODE_UP).

If RETURN_CODE is PASS {SKIP} or FAIL, then set result to FAIL and end the test.

Tester Source waits 2 seconds.

Tester and DUT establish an oCBUS connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else return PASS.

oCBUS CBUS_MODE_UP: Source

Source DUT Writes MHL_VERSION_STAT and Signals DCAP_RDY + XDEVCAP_SUPP

Verify that Source DUT writes MHL_VERSION_STAT and signals DCAP_RDY + XDEVCAP_SUPP then DCAP_CHG in time after discovery completes.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink |  |  |  |
| Dongle |  |  |  |

- Tester and DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

FAIL if Source DUT does not do WRITE_STAT to MHL_VERSION_STAT within 10 seconds from when discovery completes.

FAIL if Source DUT does not signal DCAP_RDY + XDEVCAP_SUPP and then DCAP_CHG within  10 seconds from when discovery completes.

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Source DUT Receives MSC_MSG RAP{CBUS_MODE_UP}

Verify that Source DUT receives an MSC_MSG RAP message CBUS_MODE_UP, it responds with an MSC ACK, before trying to connect using an eCBUS mode.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink |  |  |  |
| Dongle |  |  |  |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eCBUS_D_SUPPORT is YES then Tester Sink advertises both eCBUS-S and eCBUS-D support in its Extended Device Capability register ECBUS_SPEEDS.

Otherwise, Tester Sink advertises eCBUS-S support.

Tester Sink and Source DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink reads Source DUT Device Capability Registers and Extended Device Capability Registers.

Tester Sink waits until Source DUT sends an MSC_MSG RAP{CBUS_MODE_UP}.

FAIL if Source DUT does not signal CBUS_MODE_UP within 10 seconds.

Tester Sink responds by sending MSC_MSG RAPK{RESPONDER_BUSY}.

FAIL if  Source DUT does not respond with a link level MSC ACK within T_RAP_WAIT{max}.

Tester Sink sends an MSC_MSG RAP with Action Code CBUS_MODE_UP.

FAIL if Source DUT does not respond with a link-level MSC ACK.

FAIL if Source DUT does not send an MSC_MSG RAPK{NO_ERROR} within T_RAP_WAIT{max}..

FAIL if Source DUT sends eCBUS Signals before it receives the link level MSC ACK to the RAPK response it sends.

Source DUT Attempts eCBUS-S Connection

Verify that Source DUT tries to make an eCBUS-S connection based on its and the other side's Extended Device Capability registers when only eCBUS-S is possible.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink |  |  |  |
| Dongle |  |  |  |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eCBUS_S_SUPPORT is NO, then FAIL and end the test.

Tester Sink advertises eCBUS-S support in its Extended Device Capability register ECBUS_SPEEDS.

Tester Sink and Source DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink reads Source DUT Device Capability Registers and Extended Device Capability Registers.

Tester Sink waits until Source DUT sends an MSC_MSG RAP{CBUS_MODE_UP}.

FAIL if Source DUT does not signal CBUS_MODE_UP within 10 seconds.

Tester Sink sends an MSC_MSG RAPK{NO_ERROR}.

FAIL if Source DUT does not respond with a link-level MSC ACK.

FAIL if Source DUT sends eCBUS-S or eCBUS-D Signals before it sends back its link level MSC ACK to the RAPK response it receives.

FAIL if Source DUT does not attempt to establish an eCBUS-S connection.

Source DUT First Tries eCBUS-D and then eCBUS-S

Verify that Source DUT first tries eCBUS-D, and when that fails it then tries eCBUS-S, when both are possible.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source | X |  |  |
| Sink |  |  |  |
| Dongle |  |  |  |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eCBUS_S_SUPPORT is NO, or CDF_eCBUS_D_SUPPORT is NO, then PASS (SKIP) and end the test.

- Tester Sink advertises both eCBUS-S and eCBUS-D support in its Extended Device Capability register ECBUS_SPEEDS.

Tester Sink is set to only allow eCBUS-S discovery to complete. eCBUS-D is not allowed to complete.

Tester and DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink waits until Source DUT signals DCAP_RDY + XDEVCAP_SUPP.

Tester Sink waits until Source DUT sends an MSC_MSG RAP with Action Code CBUS_MODE_UP.

FAIL if Source DUT does not signal CBUS_MODE_UP within 10 seconds.

Tester Sink sends an MSC_MSG RAPK[NO_ERROR].

FAIL if Source DUT does not respond with a link-level MSC ACK.

FAIL if Source DUT does not first attempt to establish an eCBUS-D connection.

FAIL if Source DUT does not eventually switch, and attempt to establish an eCBUS-S connection.

oCBUS CBUS_MODE_UP: Sink or Dongle

Sink DUT Writes MHL_VERSION_STAT and Signals DCAP_RDY + XDEVCAP_SUPP

Verify that Sink DUT writes MHL_VERSION_STAT and signals DCAP_RDY + XDEVCAP_SUPP then DCAP_CHG in time after discovery completes.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

- Tester and DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

FAIL if Sink DUT does not signal DCAP_RDY + XDEVCAP_SUPP and then DCAP_CHG within T_SINK_DCAP_RDY from when discovery completes.

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Sink DUT Receives MSC_MSG RAP{CBUS_MODE_UP}

Verify that Sink DUT receives an MSC_MSG RAP message CBUS_MODE_UP, then responds with an ACK, before trying to connect using an eCBUS mode.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink | X |  |  |
| Dongle | X |  |  |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester Source advertises both eCBUS-S and eCBUS-D support in its Extended Device Capability register ECBUS_SPEEDS.

- Tester and DUT establish an oCBUS connection:: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source reads Sink DUT Device Capability Registers and Extended Device Capability Registers.

Tester_Source_Proposes_eCBUS_Mode(CBUS_MODE_UP).

If RETURN_CODE is not PASS, then set result to RETURN_CODE and end the test; else continue.

FAIL if Sink DUT sends eCBUS Signals before CBUS_MODE_UP dialog completes.

eCBUS-S Start-Up

eCBUS-S Source DUT Follows the Source Start-Up State Machine

Verify that eCBUS-S Source DUT follows the Source start-up state machine.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X |  |
| Sink |  |  |  |
| Dongle |  |  |  |

Observe eCBUS-S bus for eCBUS activity.

- If CDF_eCBUS_S_SUPPORT is NO, then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS-S connection: Tester_makes_eCBUS_Connection(eCBUS-S).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink or other equipment observes the activity on the eCBUS-S.

FAIL if Source DUT does not follow the eCBUS-S Start-Up state machine.

FAIL if Source DUT does not continue by executing the Link Level eCBUS TDM Slot Start-Up state machine.

eCBUS-S Sink DUT Follows the Sink Start-Up State Machine

Verify that eCBUS-S Sink DUT follows the Sink Start-Up State Machine.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  | X |  |
| Dongle |  | X |  |

Observe eCBUS-D bus for eCBUS activity.

- If CDF_eCBUS_S_SUPPORT is NO, then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS-S connection: Tester_makes_eCBUS_Connection(eCBUS-S).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source or other equipment observes the activity on the eCBUS-S.

FAIL if Sink DUT does not follow the eCBUS-S Start-Up state machine.

FAIL if Sink DUT does not continue by executing the Link Level eCBUS TDM Slot Start-Up state machine.

eCBUS-D Start-Up

The Tester and the DUT are connected with an MHL3 eCBUS-D Cable.

eCBUS-D Source DUT Follows the Source Start-Up State Machine

Verify that eCBUS-D Source DUT follows the Source Start-Up State Machine.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  | X |
| Sink |  |  |  |
| Dongle |  |  |  |

Observe eCBUS-D bus for eCBUS activity.

- If CDF_eCBUS_D_SUPPORT is NO, then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS-D connection: Tester_makes_eCBUS_Connection(eCBUS-D).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink or other equipment observes the activity on the eCBUS-D.

FAIL if Source DUT does not follow the eCBUS-D Start-Up state machine.

FAIL if Source DUT does not continue by executing the Link Level eCBUS TDM Slot Start-Up state machine.

eCBUS-D Sink DUT Follows the Sink Start-Up State Machine

Verify that eCBUS-D Sink DUT follows the Sink Start-Up State Machine.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  |  | X |
| Dongle |  |  | X |

Observe eCBUD-D bus for eCBUS activity.

- If CDF_eCBUS_D_SUPPORT is NO, then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS-D connection: Tester_makes_eCBUS_Connection(eCBUS-D).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source or other equipment observes the activity on the eCBUS-D.

FAIL if Sink DUT does not follow the eCBUS-D Start-Up state machine.

FAIL if Sink DUT does not continue by executing the Link Level eCBUS TDM Slot Start-Up state machine.

eCBUS-S, eCBUS-D VBUS: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

DUT Maintains its VBUS Drive when it Switches from CBUS to eCBUS-S or eCBUS-D

Verify that DUT maintains its VBUS drive when it switches from CBUS to eCBUS-S or eCBUS-D.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe VBUS activity.

- If CDF_VBUS_DRIVER is NO then PASS (SKIP) and end the test.

Tester is programmed to not output VBUS, and to put a load on VBUS to make it observable.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

FAIL if DUT stops driving VBUS before, during, or after the transition from CBUS to eCBUS-S or eCBUS-D.

eCBUS-S, eCBUS-D CBUS1: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Observe: DUT Sends CBUS1_IDLE Symbols (with Value COM2) in TDM Slot 0 when it has No Other CBUS1 Activity

Verify that  DUT sends CBUS1_IDLE symbols (witih value COM2) in TDM Slot 0 when it has no other CBUS1 information to send.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Observe the eCBUS to make sure that the DUT sends CBUS1 IDLE symbols when it is not sending CBUS1 information.

FAIL if DUT sends anything besides valid 3-slot CBUS1 commands, 1-Slot CBUS1 responses, or CBUS1 IDLE symbols (COM1),

Test ends after 10 seconds. If no FAILs reported, test ends with PASS.

Observe: DUT Sends CBUS1 Packets using 3 Consecutive TDM Slot 0 Times

Verify that  DUT transfers MHL 1 / MHL 2  MHL 3 CBUS command packets as CBUS1 packets using 3 consecutive TDM Slot 0 times, starting with a Start Code followed by 2 Payload bytes. The last byte has a correct CRC.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Observe the eCBUS to make sure that the DUT sends 3-Slot CBUS1 commands or 1-Slot CBUS1 Responses when it is not sending CBUS1 information.

FAIL if DUT sends anything besides valid 3-Slot CBUS1 commands, 1-Slot CBUS1 responses, or CBUS1 IDLE symbols (COM1).

Test ends after 10 seconds. If no FAILs reported, test ends with PASS.

Observe: DUT does Not Try to Start a CBUS1 Command while one from the Tester is in Progress

Verify that Sink DUT does not try to start a CBUS1 command while one from the Tester Source is in progress.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source issues READ_DEVCAP CBUS1 commands quickly for 10 seconds to read DUT Device Capability Registers, and receives DUT Register values in response.

Observe the eCBUS during the Register read period.

FAIL if DUT tries to start a CBUS1 command by sending CBUS1_START after it has received a CBUS1_START and before it has sent either a CBUS1_ACK or a CBUS1_NACK.

DUT Responds to a CBUS1 Message with Either a CBUS1 ACK or a CBUS1 NACK

Verify that DUT responds immediately to a CBUS1 message with either a CBUS1_ACK or a CBUS1_NACK.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester issues READ_DEVCAP CBUS1 commands to read DUT Device Capability Registers, and receives DUT Register values in response. (Each byte of a READ_DEVCAP command by the Tester is sent using 3 consecutive Tester TDM Slot 0 times.)

Observe the eCBUS during the Register read period.

FAIL if DUT does not respond with a CBUS1_ACK or CBUS1_NACK the first DUT CBUS1 TDM Slot 0 after the DUT receives each Tester 3-Slot CBUS1 message from the Tester.

DUT Replies to READ_XDEVCAP with Extended Device Capability Register

Verify that DUT replies to new MHL3 CBUS command READ_XDEVCAP by returning correct Extended Device Capability Register data.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester issues READ_XDEVCAP to read DUT Extended Device Capability Registers.

FAIL if register contents are not the expected values indicated in the various CDF fields.

eCBUS-S, eCBUS-D eMSC Training (if Enabled): Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Observe: DUT Executes the eMSC Link Training Procedure when the eCBUS Link is Initialized

Verify that DUT executes the eMSC Link Training procedure when the eCBUS Link is initialized.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Observe eMSC activity.

FAIL if DUT does not do eMSC Link Training after eCBUS Synchronization completes so the DUT can find Tester “eMSC Link Maintenance Slot 1” TDM Slot 1 data.

FAIL if DUT does not exit eMSC Link Training when it has sent at least receives at least N_EMSC_INIT_SYNC SYNC symbols in DUT eMSC TDM Slots and has received at least N_EMSC_INIT_SYNC SYNC symbols in Tester eMSC TDM Slots.

FAIL if DUT does not send a MARK1 as the first “eMSC Link Maintenance Slot 1” TDM Slot 1 symbol it sends after leaving eMSC Link Training.

Observe: DUT Periodically Sends a MARK1 or MARK2 Symbol in the “eMSC Link Maintenance Slot 1”

Verify that DUT periodically sends a MARK1 or MARK2 symbol in the “eMSC Link Maintenance Slot 1” TDM Slot as required by the eMSC Link Maintenance procedure.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Observe eMSC activity.

FAIL if DUT does not send a MARK1 or MARK2 symbol in the “eMSC Link Maintenance Slot 1” TDM Slots.

DUT Executes the eMSC Link Training Procedure

Verify that DUT executes the eMSC Link Training procedure when it receives a sequence of eMSC SYNC symbols.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester sends a sequence of at least N_EMSC_INIT_SYNC SYNC symbols in Tester eMSC TDM Slots.

FAIL if DUT does not do eMSC Link Training by sending at least N_EMSC_INIT_SYNC SYNC symbols in DUT eMSC TDM Slots.

FAIL if DUT does not exit eMSC Link Training when it has sent at least receives at least N_EMSC_INIT_SYNC SYNC symbols in DUT eMSC TDM Slots and has received at least N_EMSC_INIT_SYNC SYNC symbols in Tester eMSC TDM Slots.

FAIL if DUT does not send a MARK1 as the first “eMSC Link Maintenance Slot 1” symbol it sends after leaving eMSC Link Training.

eCBUS-S, eCBUS-D eMSC TDM Slot Reallocation (if Enabled): Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

DUT has Correct Initial eCBUS TDM Slot Allocation Values after oCBUS Discovery

Verify that DUT has correct initial eCBUS TDM Slot allocation values after oCBUS discovery when allocation not changed.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS  connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester uses the default eMSC TDM Slot assignment.

Tester executes an eMSC BLOCK transaction containing 255 "0" bytes as payload.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the eMSC BLOCK transaction, retry it up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the eMSC BLOCK transaction with an eMSC ACK or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

DUT Uses the eCBUS TDM Slot Allocation from Preceding CBUS Period.

Verify that DUT uses the eCBUS TDM Slot Allocation which was agreed upon during the immediately preceding CBUS period.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an oCBUS  connection. Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then end test reporting RETURN_CODE; else continue.

Tester uses section 15.10 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation where eMSC gets 1 less TDM slot that default and T-CBUS gets 1 more TDM slot than default.

If the DUT responds with 0x01, 0x02, or 0x03 (Rejected), then PASS (SKIP).

If the DUT does not respond with 0x00 (Accepted) then FAIL.

Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then end test reporting RETURN_CODE; else continue.

Tester uses the updated eMSC TDM Slot assignment.

Tester executes an eMSC BLOCK transaction containing 255 “0” bytes as payload.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the eMSC BLOCK transaction, retry it up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the eMSC BLOCK transaction with an eMSC ACK or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

DUT Handshakes Correctly when Tester Negotiates for a New eCBUS TDM Slot Allocation

Verify that DUT handshakes correctly when Tester negotiates for a new eCBUS TDM Slot allocation during eCBUS mode, then starts using the new allocation.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester uses section 15.10 “Assignment of Virtual Channels” method to direct  the DUT to use a new TDM Allocation where eMSC gets 1 less TDM slot that default and T-CBUS gets 1 more TDM slot than default. If the DUT responds with 0x01, 0x02, or 0x03 (Rejected), then PASS (SKIP).

If the DUT response with 0x01, 0x02, or 0x03 (Rejected), then PASS (SKIP).

If the DUT does not respond with 0x00 (Accepted) then FAIL

Tester uses the updated eMSC TDM Slot assignment.

Tester executes an eMSC BLOCK transaction containing 255 "0" bytes as payload.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the eMSC BLOCK transaction, retry it up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the eMSC BLOCK transaction with an eMSC ACK or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

eCBUS-S, eCBUS-D eMSC Sending (if Enabled): Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Observe: DUT Sends eMSC NULL Symbols when it has Nothing Else to Send

Verify that  DUT sends eMSC NULL symbols when it has nothing else to send.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUSconnection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Observe the eCBUS to make sure that the DUT sends eMSC NULL symbols when it is not sending eMSC information. Apply this check to all eMSC Slots except the periodic “eMSC Link Maintenance Slot 1” TDM Slots.

FAIL if DUT sends anything besides eMSC NULL symbols when it is not sending CBUS1 information.

Test ends after 10 seconds. If no FAILs reported, test ends with PASS.

Observe: DUT Sends an eMSC REQUEST Command, then Waits for a GRANT

Verify that DUT sends an eMSC REQUEST command, then waits until it receives a GRANT or MARK2 before sending data. Check that when it receives an eMSC GRANT, it sends a DATASTART followed by data followed by a valid Data CRC.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUSconnection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Decline any attempts by the DUT to change TDM Slot allocations.

Observe the eCBUS to find times when the DUT sends the eMSC Two-Byte command REQUEST. The Tester will respond to this with an eMSC GRANT.

FAIL if DUT sends the Two-Byte REQUEST command with an invalid Command-byte CRC value.

FAIL if DUT then sends anything besides eMSC NULL, one of the valid eMSC Responses ACK, NACK, or ERROR, or an eMSC Link Maintenance symbol, until it receives an eMSC Response or timeout in response to the Two-Byte REQUEST command.

Observe that once the DUT sends a REQUEST and receives an eMSC GRANT, it sends an eMSC DATASTART symbol followed by the specified amount of data followed by a 2-byte Data CRC sequence. The Tester will respond to this with an eMSC GRANT.

FAIL if DUT sends anything besides DATASTART after it receives the eMSC ACK.

FAIL if DUT then sends anything besides the specified number of data bytes followed by a valid 2-byte Data CRC sequence.

FAIL if DUT then sends anything besides eMSC NULL, one of the valid eMSC Responses ACK, NACK, or ERROR, or an eMSC Link Maintenance symbol, until it receives an eMSC Response or timeout in response to the Two-Byte command.

Test ends after 10 seconds. If no FAILs reported, test ends with PASS.

Observe: DUT Sends an eMSC REQUEST Command, then Waits for a MARK2

Verify that DUT sends an eMSC REQUEST command, then waits until it receives a MARK2 before sending data. Check that when it receives a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot, it sends a DATASTART followed by data followed by a valid Data CRC.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Decline any attempts by the DUT to change TDM Slot allocations.

Observe the eCBUS to find times when the DUT sends the eMSC Two-Byte command REQUEST. The Tester will respond to this with a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

FAIL if DUT sends the Two-Byte REQUEST command with an invalid Command-byte CRC value.

FAIL if DUT then sends anything besides eMSC NULL, one of the valid eMSC Responses ACK, NACK, or ERROR, or an eMSC Link Maintenance symbol, until it receives an eMSC Response or timeout in response to the Two-Byte REQUEST command.

Observe that once the DUT sends a REQUEST and receives a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot, it sends an eMSC DATASTART symbol followed by the specified amount of data followed by a Two-byte Data CRC sequence. The Tester will respond to this with a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

FAIL if DUT sends anything besides DATASTART after it receives the MARK2.

FAIL if DUT then sends anything besides the specified number of data bytes followed by a valid Two-byte Data CRC sequence.

FAIL if DUT then sends anything besides eMSC NULL, one of the valid eMSC Responses ACK, NACK, or ERROR, or an eMSC Link Maintenance symbol, until it receives an eMSC Response or timeout in response to the Two-Byte command.

Test ends after 10 seconds. If no FAILs reported, test ends with PASS.

eCBUS-S, eCBUS-D eMSC Receiving (if Enabled): Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

DUT Receives a Valid eMSC Block Transfer REQUEST

Verify that DUT which receives a valid eMSC Block Transfer REQUEST sequence eventually responds by sending either an eMSC GRANT or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot. When it gets the following block of data, it eventually responds by sending either an eMSC GRANT or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUSconnection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester executes 1 eMSC REQUEST command with length 256.

If the Tester receives a timeout in response to the REQUEST, retry it up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to each REQUEST with an eMSC GRANT. Or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

Tester sends an eMSC DATASTART symbol, followed by 256 bytes of 0’s for data and a valid Data CRC.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the DATASTART, retry the whole command starting with REQUEST up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to each DATASTART symbol sequence with an eMSC ACK. Or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

FAIL if DUT responds with an eMSC ACK, eMSC NACK, eMSC ERROR or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot later than T_EMSC_RESP_TMOUT{max} from receiving the last symbol of the DATASTART sequence.

eCBUS-S 12-bit TDM Slot Mode: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S Cable.

DUT Sends CBUS1 Traffic Correctly when Using 12-bit TDM Slot Mode to Send 8 TDM Slot Data Bits

Slot Mode to send 8 TDM Slot Data bits.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_12_BIT_TDM_SLOT_MODE_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an oCBUS connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester requests that DUT enter Low Latency Tunneling mode.

Tester sets itself to use the 12-bit TDM Slot Mode assignment.

Tester and DUT agree to use 12-bit TDM Slot assignment mode.

Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS-S).

If RETURN_CODE is PASS (SKIP) or FAIL, then end test reporting RETURN_CODE; else continue.

Tester issues READ_DEVCAP to read DUT’s Device Capability registers.

FAIL if register contents are not the expected values indicated in the various CDF fields.

FAIL if TDM slots do not correctly include 4 bits of Low Latency Channel information.

eCBUS-S, eCBUS-D CBUS_MODE_ DOWN: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Source DUT Reads Extended Device Capability Registers after it Gets DCAP_RDY or DCAP_CHG

Verify that Source DUT reads Extended Device Capability Registers after it gets DCAP_RDY or DCAP_CHG.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  |  |  |
| Dongle |  |  |  |

- Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink issues a WRITE_STAT indicating DCAP_RDY + XDEVCAP_SUPP.

FAIL if Source DUT does not re-read the Extended Device Capability Registers within T_SRC_READ_XDCAP{max} of the WRITE_STAT.

Tester issues a SET_INT indicating DCAP_CHG.

FAIL if Source DUT does not re-read the Extended Device Capability Registers within T_SRC_READ_XDCAP{max} of the SET_INT.

Sink DUT Reads Extended Device Capability Registers after it Gets DCAP_RDY or DCAP_CHG

Verify that Sink DUT reads Extended Device Capability Registers after it gets DCAP_RDY or DCAP_CHG.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source issues a WRITE_STAT indicating DCAP_RDY + XDEVCAP_SUPP.

FAIL if Sink DUT does not complete re-reading the Extended Device Capability Registers within T_SINK_READ_XDCAP{max} of the WRITE_STAT.

Tester issues a SET_INT indicating DCAP_CHG.

FAIL if Sink DUT does not re-read the Extended Device Capability Registers within T_SINK_READ_XDCAP{max} of the SET_INT.

DUT Switches eCBUS-S or eCBUS-D to oCBUS in Response to CBUS_MODE_DOWN

Verify DUT switches eCBUS-S or eCBUS-D to oCBUS in response to CBUS_MODE_DOWN.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester sends an MSC_MSG RAP with Action Code CBUS_MODE_DOWN.

Tester waits until DUT sends an MSC_MSG RAPK{NO_ERROR}.

FAIL if DUT does not return MSC_MSG RAPK within T_RAP_WAIT{max}..

Tester attempts to reconnect in oCBUS mode.

FAIL if Tester and DUT do not re-establish an oCBUS connection.

Tester issues READ_DEVCAP to read DUT’s Device Capability registers.

FAIL if register contents are not the expected values indicated in the various CDF fields.

DUT Maintains its VBUS Drive (if any) when it Switches from eCBUS to oCBUS

Verify that DUT maintains its VBUS drive (if any) when it switches from eCBUS to oCBUS.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe VBUS activity.

- If CDF_VBUS_DRIVER is NO then PASS (SKIP) and end the test.

Tester is programmed to not output VBUS, and to put a load on VBUS to make it observable.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester sends an MSC_MSG RAP{CBUS_MODE_DOWN}.

Tester waits until DUT sends an MSC_MSG RAPK{NO_ERROR}.

FAIL if DUT does not return MSC_MSG RAPK within T_RAP_WAIT{max}..

Tester attempts to reconnect in oCBUS mode.

FAIL if Tester and DUT do not re-establish an oCBUS connection.

FAIL if DUT stops driving VBUS before, during, or after the transition from eCBUS_S or eCBUS-D to oCBUS.

DUT Maintains its eCBUS Slot Allocation Switching from eCBUS to oCBUS

Verify that DUT maintains its eCBUS TDM Slot Allocation when it switches from eCBUS to oCBUS.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection:
Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester uses section 15.10 "Assignment of Virtual Channels" method to direct  the DUT to use a new TDM Allocation where eMSC gets 1 more TDM slot that default and T-CBUS gets 1 less TDM slot than default.

If the DUT responds with 0x01, 0x02, or 0x03 (Rejected), then PASS (SKIP).

If the DUT does not respond with 0x00 (Accepted) then FAIL.

Tester sends an MSC_MSG RAP with Action Code CBUS_MODE_DOWN.

Tester waits until DUT sends an MSC_MSG RAPK{NO_ERROR}.

FAIL if DUT does not return MSC_MSG RAPK within T_RAP_WAIT{max}..

Tester attempts to reconnect in oCBUS mode.

FAIL if Tester and DUT do not re-establish an oCBUS connection.

Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester uses the updated eMSC TDM Slot assignment.

Tester executes an eMSC BLOCK transaction containing 255 "0" bytes as payload.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the eCBUS BLOCK transaction, retry it up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to each eCBUS BLOCK transaction with an eMSC ACK or a MARK2 in an "eMSC Link Maintenance Slot 1" TDM Slot.

eCBUS-S, eCBUS-D CBUS_MODE_ DOWN Fails: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Source DUT Notices when a Transition from eCBUS to oCBUS Fails, and then DUT Disconnects

Verify that Source DUT notices when a transition from eCBUS to oCBUS fails, and then the DUT disconnects and re-attempts discovery.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  |  |  |
| Dongle |  |  |  |

- Tester and DUT establish an eCBUS connection: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Sink is programmed to not automatically return to oCBUS mode after mode transition from eCBUS.

Tester Sink sends an MSC_MSG RAP with Action Code CBUS_MODE_DOWN.

Tester Sink waits until Source DUT sends an MSC_MSG RAPK{NO_ERROR}.

FAIL if Source DUT does not signal NO_ERROR within T_RAP_WAIT{max}..

Tester Sink HIGH-Z’s the CBUS.

Tester Sink waits 2 seconds.

Tester and DUT establish an oCBUS connection: Tester_makes_eCBUS_Connection(CBUS_MODE).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else return PASS.

Sink DUT Notices when a Transition from eCBUS to oCBUS Fails, then the DUT Disconnects

Verify that Sink DUT notices when a transition from eCBUS to oCBUS fails, and then the DUT disconnects and re-attempts discovery.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester Source is programmed to not automatically return to oCBUS mode after mode transition.

Tester Source sends an MSC_MSG RAP with Action Code CBUS_MODE_DOWN.

Tester Source waits until Source DUT sends an MSC_MSG RAPK{NO_ERROR}.

FAIL if Sink DUT does not signal NO_ERROR within T_RAP_WAIT{max}..

Tester Source HIGH-Z’s the CBUS.

Tester Source waits 2 seconds.

Tester attempts to reconnect in oCBUS mode.

FAIL if Tester and DUT do not re-establish an oCBUS connection.

eCBUS-S, eCBUS-D Power Tests: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Source DUT Postdiscovery VBUS Input Current Draw

Verify that a MHL3 Source DUT does not  violate Postdiscovery current limits when connected to a MHL3 Powered Sink.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  |  |  |
| Dongle |  |  |  |

Execute the following test procedure once for each row in Table 6-1, substituting for MHL version, device, PLIM bits, IVBUS delivery.

- Connect Source DUT to Tester Sink port using 20cm MHL Test Cable.

Set Tester port to disconnected state.

Set Tester port to drive current limited IVBUS_Prediscovery (100mA) onto VBUS.

Set Tester  port to use typical timings and to exhibit typical resistances and capacitances.

Set Tester port to display MHL version, device, and PLIM bits where applicable.

Set Tester port to drive current limited IVBUS_Delivery onto VBUS.

Set Tester port POW bit = 1.

Tester Sink and Source DUT establish an eCBUS connection:
Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

Allow time for  Source DUT to increase its current load on the VBUS.

FAIL if DUT draws more than IVBUS_Delivery.

FAIL if DUT draws more than 1.5A and CDF_SRC_IS_DIRECT_ATTACH is NO.

Check to see if DUT is simultaneously driving VBUS.

FAIL if DUT is simultaneously driving VBUS.

If all preceding steps pass, then PASS; else FAIL.

Table 6-1. Tester Sink Emulation

|  | MHL Version | Device | PLIM | IVBUS delivery | Purpose |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.x | Sink | (0,0,0) | 500mA | 500mA draw test |
| 2 | 3.x | Sink | {0,0,1) | 900mA | 900mA draw test |
| 3 | 3.x | Sink | {0,1,0) | 1.5A | 1.5A draw test |
| 4 | 3.x | Dongle | {0,1,1) | 100mA | 100mA draw test |
| 5 | 3.x | Sink | {1,0,0) | 2.0A | 2.0A draw test |

Sink DUT VBUS Output

Verify that a MHL3 Sink supplies power over the VBUS as required.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  | X | X |
| Dongle |  |  |  |

- If the CDF value CDF_SINK_DC indicates that the Sink is a DC powered Sink and if the CDF value CDF_SINK_POWERED also indicates that Sink DUT provides no VBUS output, then end the test with PASS (SKIP); else continue.

If the Sink DUT has an external power source accessory, connect the DUT to active power now.

Disconnect Sink DUT from Tester Source port.

Set Tester Source port to disconnected state.

Set Tester Source port to use typical timings and to exhibit typical resistances and capacitances.

Tester Source does not drive VBUS.

Set Tester Source to apply an IVBUS_PREDISCOVERY{max} load (100mA) to the VBUS.

Connect Sink DUT to Tester Source port using 20cm MHL Test Cable.

Tester Source and Sink DUT establish an eCBUS connection:
Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

Set Sink DUT into Active State based on the CDF procedure CDF_PROC_SET_ACTIVE.

Tester Source measures the VBUS driven by the Sink and loaded by the Tester Source.

If VBUS Voltage is not within the range VVBUS_TP1_SINK_DRV{min} to VVBUS_TP1_SINK_DRV{max}, then FAIL.

Tester Source signals DCAP_RDY followed by DCAP_CHG.

Tester Source waits until Sink DUT signals DCAP_RDY.

Tester Source reads Sink DUT Device Capability Registers and Extended Device Capability Registers.

Tester Source reads Sinks DEV_CAT register.

FAIL if DEV_CAT register not read as SINK, and immediately terminate the test.

FAIL if DEV_CAT POW bit is not ‘1’.

If {PLIM2, PLIM1, PLIM0} are {1,0,0}, set Tester Source to apply a load of 2.0A to the VBUS.

Else if {PLIM2, PLIM1, PLIM0} are {0,1,0}, set Tester Source to apply a load of 1.50A to the VBUS.

Else if {PLIM2, PLIM1, PLIM0} are {0,0,1}, set Tester Source to apply a load of 900mA to the VBUS.

Else if the CDF value CDF_SINK_DC indicates that Sink DUT is DC powered and if the Sink DUT has no external power source attached, then continue; else end test with a FAIL.

  - If {PLIM2, PLIM1, PLIM0} are {0,0,0}, set Tester Source to apply a load of 500mA to the VBUS.

  - If {PLIM2, PLIM1, PLIM0} are {0,1,1}, set Tester Source to apply a load of 100mA to the VBUS.

If VBUS Voltage is within the range VVBUS_TP1_SINK_DRV{min} to VVBUS_TP1_SINK_DRV{max}, then continue; else end test with FAIL.

If all preceding steps pass, then PASS; else FAIL.

Sink DUT VBUS Type A Output Limit

Verify that a MHL3 Sink does not advertise >1500mA capability if using a HDMI Type A connector.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  | X | X |
| Dongle |  |  |  |

- If the Sink DUT does not use a HDMI Type A connector, then end the test with a PASS (SKIP); else continue.

If the CDF value CDF_SINK_DC indicates that the Sink is a DC powered Sink and if the CDF value CDF_SINK_POWERED also indicates that Sink DUT provides no VBUS output, then end the test with PASS (SKIP); else continue.

If the Sink DUT has an external power source accessory, connect the DUT to active power now.

Disconnect Sink DUT from Tester Source port.

Set Tester Source port to disconnected state.

Set Tester Source port to use typical timings and to exhibit typical resistances and capacitances.

Tester Source does not drive VBUS.

Set Tester Source to apply an IVBUS_PREDISCOVERY{max} load (100mA) to the VBUS.

Connect Sink DUT to Tester Source port using 20cm MHL Test Cable.

Tester Source and Sink DUT establish an eCBUS connection:
Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

Set Sink DUT into Active State based on the CDF procedure CDF_PROC_SET_ACTIVE.

Tester Source measures the VBUS driven by the Sink and loaded by the Tester Source.

Tester Source signals DCAP_RDY followed by DCAP_CHG.

Tester Source waits until Sink DUT signals DCAP_RDY.

Tester Source reads Sink DUT Device Capability Registers and Extended Device Capability Registers.

Tester Source reads Sinks DEV_CAT register.

FAIL If {PLIM2, PLIM1, PLIM0} is {1, 0, 0}.

If all preceding steps pass, then PASS; else FAIL.

Dongle DUT VBUS Output

Verify that a powered Dongle supplies power over the VBUS as required.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  |  |  |
| Dongle |  | X | X |

- If the CDF value CDF_D_POWERED indicates that DUT is a Powered Dongle, then continue, else end test with PASS (SKIP).

If necessary, connect Dongle DUT to Tester Source port.

Connect the Dongle DUT to the downstream device, and connect power to the Dongle DUT.

Set Tester Source port to disconnected state.

Set Tester Source port to use typical timings and to exhibit typical resistances and capacitances.

Tester does not drive VBUS.

Set Tester Source to apply IVBUS_PREDISCOVERY{max} load (100 mA) to the VBUS.

Tester Source measures the VBUS driven by the Dongle DUT and loaded by the Tester Source.

FAIL if VBUS Voltage is not within the range VVBUS_TP1_DNGL_DRV {min} to VVBUS_TP1_DNGL_DRV{max}.

Connect Dongle DUT to Tester Source port.

Tester Source and Dongle DUT establish an eCBUS connection:
Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

Tester Source reads Dongle DUT Device Capability Registers and Extended Device Capability Registers.

Tester Source reads Dongle DEV_CAT register.

FAIL if DEV_CAT register not read as DONGLE, and immediately terminate the test.

FAIL if DEV_CAT POW bit is not ‘1’.

If {PLIM2, PLIM1, PLIM0} are {0,1,1}, Set Tester Source to apply IVBUS_DONGLE_TO_SOURCE load (100mA) to the VBUS.

Else if {PLIM2, PLIM1, PLIM0} are {0,0,0}, Set Tester Source to apply IVBUS_DONGLE_TO_SOURCE load (500mA) to the VBUS.

Else if {PLIM2, PLIM1, PLIM0} are {0,0,1}, Set Tester Source to apply IVBUS_DONGLE_TO_SOURCE load (900mA) to the VBUS.

Else if {PLIM2, PLIM1, PLIM0} are {0,1,0}, set Tester Source to apply IVBUS_DONGLE_TO_SOURCE load (1500mA) to the VBUS.

Else if {PLIM2, PLIM1, PLIM0} are {1,0,0}, set Tester Source to apply IVBUS_DONGLE_TO_SOURCE load (2000mA) to the VBUS.

Else end test with FAIL. Report invalid {PLIM2, PLIM1, PLIM0} settings.

FAIL if VBUS Voltage is not within the range VVBUS_TP1_DNGL_DRV {min} to VVBUS_TP1_DNGL_DRV{max}.

If all preceding steps pass, then PASS; else FAIL.

eCBUS-S, eCBUS-D HSIC Tunneling: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Test USB HSIC Tunneling.

Reserved

eCBUS-S USB 1 (Full/Low Speed) Tunneling: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S Cable.

Test USB 1 (Full/Low Speed) Tunneling.

Reserved

eCBUS-S, eCBUS-D eCBUS BIST Tests: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS BIST Receiver Test.

Verify that DUT receives and checks eCBUS BIST PRBS patterns.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection:
Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

- If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

- Tester uses BIST_SETUP procedure to prepare DUT to receive eCBUS PRBS BIST pattern.

- Observe Tester output pattern on eCBUS-S or eCBUS-D as appropriate.

- Use BIST_TRIGGER to cause DUT to run eCBUS BIST.

- Wait until BIST procedure finishes.

- Use BIST_REQUEST_STAT to request error count from DUT.

- Capture data from DUT BIST_RETURN_STAT response.

- FAIL if DUT error count is not 0

- FAIL if Tester output BIST pattern is not a valid eCBUS BIST PRBS pattern.

eCBUS BIST Sender Test.

Verify that DUT sends correct eCBUS BIST PRBS patterns.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection:
Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester uses BIST_SETUP procedure to prepare DUT to send eCBUS PRBS BIST pattern.

Observe DUT output pattern on eCBUS-S or eCBUS-D as appropriate.

Use BIST_TRIGGER to cause DUT to run eCBUS BIST.

Wait until BIST procedure finishes.

FAIL if Tester error count is not 0

FAIL if DUT output BIST pattern is not a valid eCBUS BIST PRBS pattern.

## eCBUS MHL 3 Robustness Tests

Tests are done using the Default Test Setup described in Section 6.4, unless overridden in the test description.

Several of these tests depend on the Tester being able to carefully record the activity on the eCBUS.

Some of these tests need the Tester to do unusual interface behavior, or make unusual message sequences.

eCBUS-S, eCBUS-D TDM Slot Numbering: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

eCBUS-S DUT uses 25 TDM Slots, and eCBUS-D DUT uses 200 TDM Slots

Verify that eCBUS-S and eCBUS-D connections use the correct overall number of TDM slots.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester or other equipment observes the activity on the eCBUS.

Observe correct CBUS1 behavior based on CBUS1 activity observed in DUT TDM Slot 0.

If eMSC is enabled, Observe correct eMSC behavior based on eMSC activity observed in the “eMSC Link Maintenance Slot 1” TDM Slots.

FAIL if either CBUS1 or eMSC behavior does not indicate that the correct number of TDM Slots are being used.

Alternatively, infer that the DUT is using the correct number of TDM Slots. This inference can be made if the Tester is known to use the correct number of TDM Slots, and the Tester and DUT can exchange CBUS1 and eMSC messages.

eCBUS-S, eCBUS-D Sink DUT has Correct TDM Slot 0 Alignment

Verify that Sink DUT places its TDM Slot 0 the correct phase offset from the Source TDM Slot 0.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  |  |  |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester or other equipment observes the activity on the eCBUS.

Observe correct CBUS1 behavior based on CBUS1 activity observed in DUT TDM Slot 0.

If eMSC is enabled, Observe correct eMSC behavior based on eMSC activity observed in the DUT Sink’s “eMSC Link Maintenance Slot 1” TDM Slots.

FAIL if either CBUS1 or eMSC behavior indicates that the DUT has aligned its Sink TDM Slot 0 to an illegal slot offset from the Tester Source TDM Slot 0.

eCBUS-S, eCBUS-D TDM Slot Synchronization Recovery: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

DUT Recognizes a Long Period when there is Illegal CBUS1 Activity in TDM Slot 0

Verify that  DUT recognizes a long period when there is illegal activity in TDM Slot 0. DUT tries to Resynchronize by sending the Link Level eCBUS TDM Slot Synchronization Request Pattern of COM1-IDLEs. DUT finishes Resynchronization effort when it receives a Link Level eCBUS Slot Synchronization Pattern of COM2-IDLEs.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester is set to continuously send a data value of 0x01 in Tester TDM Slot 0 (this is neither CBUS1_IDLE (COM2) nor CBUS1_START).

FAIL if DUT does not notice this TDM Slot 0 illegal behavior, and start sending the Link Level eCBUS Slot Synchronization Request Pattern of “COM1, (F_TDM_TOTAL – 1) * IDLE”.

Tester is set to respond to the DUT Link Level eCBUS Slot Synchronization Request Pattern by sending the Link Level eCBUS Slot Synchronization  Response Pattern of “COM2, (F_TDM_TOTAL – 1) * IDLE”.

Tester watches for the DUT to stop sending the Link Level eCBUS Slot Synchronization Request Pattern.

Tester stops sending the Link Level eCBUS Slot Synchronization Response Pattern when it sees the Link Level eCBUS Slot Synchronization Request pattern terminate.  It reverts to normal eCBUS behavior.

FAIL if DUT does not stop sending the Link Level eCBUS Slot Synchronization Request Pattern in response to seeing the Link Level eCBUS Slot Synchronization Response Pattern.

FAIL of Tester cannot do a CBUS1 READ_DEVCAP command to the DUT, and get a valid response, after the link is Resynchronized.

DUT Attempts Link Level TDM Slot Synchronization and Never Receives COM2-IDLEs

Verify that  DUT which tries to do Link Level TDM Slot Synchronization and which never receives COM2-IDLEs proceeds to disconnect the eCBUS.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester is set to continuously send a data value of 0x01 in Tester TDM Slot 0 (this is neither CBUS1_IDLE (COM2) nor CBUS1_START).

FAIL if DUT does not notice this TDM Slot 0 illegal behavior, and start sending the Link Level eCBUS Slot Synchronization Request Pattern of “COM1, (F_TDM_TOTAL – 1) * IDLE”.

Tester does not respond to the eCBUS Resynchronization Pattern for more than T_CBUS1_TIMEOUT{max}.

FAIL if DUT does not stop sending the eCBUS Resynchronization Request Pattern.

FAIL if eCBUS-S Source DUT does not follow the eCBUS-S Synchronization state machine.

FAIL if eCBUS-S Sink DUT does not follow the eCBUS-S Synchronization state machine.

FAIL if eCBUS-D Source DUT does not follow the eCBUS-D Synchronization state machine.

FAIL if eCBUS-D Sink DUT does not follow the eCBUS-D Synchronization state machine.

DUT Recognizes the Link Level TDM Slot Synchronization Request Pattern of COM1-IDLES

Verify that DUT recognizes the Link Level TDM Slot Synchronization Request pattern of COM1-IDLES and responds by sending back the Link Level TDM Slot Synchronization Response Pattern of COM2-IDLES.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester is set to force an eCBUS Resynchronization. It sends the Link Level eCBUS Slot Synchronization Request Pattern of “COM1, (F_TDM_TOTAL – 1) * IDLE”.

Once it sees the Link Level eCBUS Slot Synchronization Response pattern, the Tester stops sending the Link Level eCBUS Slot Synchronization Request Pattern, and reverts to normal eCBUS behavior.

FAIL if DUT does not notice the Link Level eCBUS Slot Synchronization Request Pattern, and start sending the Link Level eCBUS Slot Synchronization Response Pattern of “COM2, (F_TDM_TOTAL – 1) * IDLE”.

FAIL if Tester and DUT do not finish the Resynchronization procedure.

FAIL of Tester cannot do a CBUS1 READ_DEVCAP command to the DUT, and get a valid response, aster the link is Resynchronized.

eCBUS-S, eCBUS-D CBUS1 Tests: Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Reserved

DUT Responds with a CBUS1 NACK when it Receives a CBUS1 Message with a Bad CRC

Verify that DUT responds with a CBUS1_NACK when it receives a CBUS1 message with a bad CRC.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

The Tester starts sending the 3-symbol CBUS1 command READ_DEVCAP.  The first message containing the Command Byte is sent once with an inverted Checksum.

FAIL if DUT does not respond to the 3-symbol Tester READ_DEVCAP CBUS1 message by returning a CBUS1_NACK response.

Tester repeats the sending of the CBUS1 READ_DEVCAP with a correct checksum.

FAIL if DUT does not return valid information.

DUT Retransmits a CBUS1 Packet the Correct Number of Times

Verify that DUT retransmits a CBUS1 packet the correct number of times when it receives a sequence of CBUS1 NACKs.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester is set to watch the CBUS1 TDM Slot 0 time for the 3-symbol CBUS1 messages from DUT.

Tester responds to the messages from the DUT with a CBUS1 NACK  It does this (N_CBUS1_RETRY_SINK – 1) times or (N_CBUS1_RETRY_SOURCE – 1) as appropriate.

FAIL if DUT does not retransmit the CBUS message as required, resulting in final command completion.

DUT which Receives Several Consecutive CBUS1 NACKs Pauses its Retransmission

Verify that DUT which receives several CBUS1 NACKs in a row pauses its retransmission so Livelock cannot occur.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

Observe eCBUS-S and/or eCBUS-D bus for eCBUS activity.

- Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester is set to watch the CBUS1 TDM Slot 0 time for the 3-symbol CBUS1 messages from DUT.

Tester responds to the messages from the DUT with a CBUS1 NACK  It does this (N_CBUS1_RETRY_SINK – 1) times or (N_CBUS1_RETRY_SOURCE – 1) as appropriate.

FAIL if Source DUT does not add a delay between individual Retransmission attempts as required for a Source to avoid CBUS1 Collision-based Arbitration to complete.

FAIL if Sink DUT does not add a delay between individual Retransmission attempts as required fpr a Sink to avoid CBUS1 Collision-based Arbitration to complete.

eCBUS-S, eCBUS-D eMSC (if enabled): Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

DUT Recognizes a Problem with the MARK1 or MARK2

Verify that DUT recognizes a problem with the MARK1 or MARK2 in “eMSC Link Maintenance Slot 1” TDM Slot, and that it re-executes the eMSC Link Training procedure.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester is set to continuously send a data value of 0x01 in the “eMSC Link Maintenance Slot 1” TDM Slot (this is neither MARK1 or MARK2).

FAIL if DUT does not notice this “eMSC Link Maintenance Slot 1” TDM Slot 1 illegal behavior, and start sending the eMSC Link Training Pattern consisting of SYNC characters in all DUT eMSC TDM Slots.

Tester responds to the reception of the eMSC Link Training Pattern by sending SYNC in all Tester eMSC TDM Slots.

FAIL if DUT stops sending the eMSC Training Pattern before it has both sent and received N_EMSC_INIT_SYNC copies of the SYNC symbol.

FAIL if DUT does not continue sending SYNC symbols until the next DUT TDM Slot 1.

FAIL if DUT does not send a MARK1 Symbol in the DUT TDM Slot 1 immediately after its last SYNC.  This identifies the DUT “eMSC Link Maintenance Slot 1” TDM Slot.

Tester stops sending SYNC symbols in the Tester eMSC TDM slots.  It sends SYNC until it finally sends MARK1 in its Tester TDM Slot 1.  This identifies the Tester “eMSC Link Maintenance Slot 1” TDM Slot.

Tester executes an eMSC BLOCK transaction containing 255 “0” bytes as payload.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the eMSC BLOCK transaction, retry it up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the eMSC transaction with an eMSC ACK or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

eCBUS-S, eCBUS-D eMSC (if enabled): Source, Sink, Dongle

The Tester and the DUT are connected with an MHL3 eCBUS-S or eCBUS-D Cable.

If the CDF_eCBUS_S_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-S cable, with the tester indicating that it only supports eCBUS-S..

If the CDF_eCBUS_D_SUPPORT is YES, then a set of tests is run using an MHL3 eCBUS-D cable.

eCBUS-S tests can be grouped together, and eCBUS-D tests can be grouped together, to minimize cable swapping.

Observe: DUT Retries a Block Transfer REQUEST Command on eMSC Timeout

Verify that DUT retries a Block Transfer REQUEST Command sequence enough times if it receives an eMSC timeout.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester is set to watch for an eMSC REQUEST command. It waits until all symbols of the command are received, then simply waits to force an eMSC Timeout. It times out the eMSC command (T_EMSC_RETRY{min} – 1) times, then responds with an eMSC GRANT.

FAIL if DUT does not wait at least T_EMSC_ REQ_TIMEOUT {min} before re-transmitting.

FAIL if DUT does not re-transmit the eMSC Command T_EMSC_RETRY{min} times.

DUT which Receives an eMSC Block Transfer REQUEST with Bad Opcode Responds with Timeout

Verify that DUT which receives an eMSC Block Transfer REQUEST which contains a bad Opcode CRC responds by doing a Timeout.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester executes a REQUEST command with 256 bytes of data and an inverted Command CRC.

FAIL if DUT does not timeout the response to the REQUEST command.

Tester executes a REQUEST command with 256 bytes of data and a valid Command CRC.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the REQUEST, retry the REQUEST up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the REQUEST with an eMSC GRANT or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

FAIL if DUT responds with an eMSC GRANT, eMSC NACK, eMSC ERROR or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot later than T_EMSC_RESP_TMOUT{max} from receiving the last symbol of the REQUEST sequence.

Tester sends DATASTART followed by 256 bytes of data followed by a valid Data CRC.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the REQUEST, retry the whole command from the REQUEST and including the inverted Command CRC up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the REQUEST with an eMSC ACK. Or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

FAIL if DUT responds with an eMSC ACK, eMSC NACK, eMSC ERROR or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot later than T_EMSC_RESP_TMOUT{max} from receiving the last symbol of the DATASTART sequence.

DUT which Receives an eMSC Block Transfer DATASTART with Bad Data CRC Responds with eMSC Error

Verify that DUT which receives an eMSC Block Transfer DATASTART sequence which contains a bad Data CRC responds by sending an eMSC ERROR.

|  | oCBUS | eCBUS-S | eCBUS-D |
| --- | --- | --- | --- |
| Source |  | X | X |
| Sink |  | X | X |
| Dongle |  | X | X |

- If CDF_eMSC_SUPPORT is NO then PASS (SKIP) and end the test.

Tester and DUT establish an eCBUS connection:: Tester_makes_eCBUS_Connection(eCBUS_MODE_TO_TEST).

If RETURN_CODE is PASS (SKIP) or FAIL, then set test result to RETURN_CODE and end the test; else continue.

Tester executes a REQUEST command with 256 bytes of data and a valid Command CRC.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the REQUEST, retry the REQUEST up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the REQUEST with an eMSC GRANT or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

FAIL if DUT responds with an eMSC GRANT, eMSC NACK, eMSC ERROR or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot later than T_EMSC_RESP_TMOUT{max} from receiving the last symbol of the REQUEST sequence.

Tester executes a DATASTART command with 256 bytes of data and an inverted Data CRC.

If the Tester receives an eMSC NACK, or eMSC Timeout in response to the DATASTART, retry the whole command from the REQUEST and including the inverted DATA CRC up to N_EMSC_RETRY{min} times.

FAIL if DUT which did not NACK does not reply to the DATASTART with an eMSC ERROR response.

Tester executes a REQUEST command with 256 bytes of data and a valid Command CRC.

If the Tester receives an eMSC NACK, eMSC ERROR, or eMSC Timeout in response to the REQUEST, retry the REQUEST up to N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the REQUEST with an eMSC GRANT or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

FAIL if DUT responds with an eMSC GRANT, eMSC NACK, eMSC ERROR or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot later than T_EMSC_RESP_TMOUT{max} from receiving the last symbol of the REQUEST sequence.

Tester executes a DATASTART command with 256 bytes of data and a valid Data CRC.

If the Tester receives an eMSC NACK, or eMSC Timeout in response to the DATASTART, retry the whole command from the REQUEST and including a valid DATA CRC until an overall total of N_EMSC_RETRY{min} times.

FAIL if DUT does not eventually respond to the DATASTART with an eMSC ACK. Or a MARK2 in an “eMSC Link Maintenance Slot 1” TDM Slot.

# Cable Test

## Mechanical tests

Cable Assembly Mechanical Tests

Connector Maximum Envelope

Verify that DUT’s connector shell and cable fit inside minimum allowable receptacle envelope.

- Measure all overmold dimensions.

Verify that all dimensions fall within maximum permitted value in each connector specification.

Measure the maximum extension from the shell in the rigid portion of the connector.

Verify that all dimensions fall within maximum permitted value in each connector specification.

If all measurements fall within limits, then PASS, else FAIL.

CBUS Shielding

Verify that DUT’s CBUS is shielded. Adopter shall provide assembly drawing for cable with CDF.

- Verify by review of Adopter's Cable assembly drawings that the CBUS is shielded.

If assembly drawings do not show shielding around CBUS, then FAIL; else PASS.

## Electrical Tests

Cable Assembly Electrical Tests for eCBUS-S cable

All tests in the CTS 2.1, Section 7.2.1, shall be run against the Cable DUT. In MHL 2.1 CTS, MHL+ and MHL- represent TMDS+ and TMDS-, and CBUS represents eCBUS.

Single-Ended eCBUS Characteristic Impedance

This test confirms that the MHL cable Single-Ended eCBUS Characteristic Impedance is within the range allowed in the specification.

Figure 7-1. Single-Ended Characteristic Impedance Test Setup

Figure 7-2. Set reference point for Single-Ended

If cable length is greater than 600mm, then:

- Connect the M3-S TPA SRC-R to the TDR oscilloscope.

Set the effective rise time of the Single-Ended TDR pulse to 200ps (20 – 80%).

Record the time at the peak of the change point in Figure 7-2. This is the connection point (TCONN).

Connect the M3-S TPA SRC-R to the source end of the Cable DUT and measure the Single-Ended impedance from TCONN to TCONN + 2 ns.

The Single-Ended impedance from TCONN to TCONN + 1 ns is ZO_TRANSITION and the Single-Ended impedance from TCONN + 1 ns to TCONN + 2 ns is ZO_CABLE.

  - If (42.5 ohms <= ZO_TRANSITION <= 57.5 ohms) AND (45 ohms <= ZO_CABLE <= 55 ohms), it is PASS

  - If (32.5 ohms <= ZO_TRANSITION < 42.5 ohms) OR (57.5 ohms < ZO_TRANSITION <= 62.5 ohms), the impedance is in the excursion range. If the excursion occurs at a single time AND the duration of the single excursion is less than 250ps AND (45 ohms <= ZO_CABLE <= 55 ohms), it is PASS.

  - All other cases are FAIL.

Repeat Step 5 for the sink end of the Cable DUT.

If cable length is less than or equal to 600 mm:

- Connect the M3-S TPA SRC-R to the TDR oscilloscope.

Set the effective rise time of the Single-Ended TDR pulse to 200ps (20 – 80%).

Record the time at the peak of the change point in Figure 7-2. This is the connection point (TCONN200).

Connect the M3-S TPA SRC-R to the source end of the Cable DUT and measure the Single-Ended impedance from TCONN200 to TCONN200 + 1 ns.

The Single-Ended impedance from TCONN600 to TCONN200 + 1 ns is ZO_TRANSITION

Set the effective rise time of the Single-Ended TDR pulse to 50ps (20 – 80%).

Record the time at the peak of the change point in Figure 7-2. This is the connection point (TCONN50).

Connect the M3-S TPA SRC-R to the source end of the Cable DUT and measure the Single-Ended impedance from TCONN50 + 1 ns to TCONN50 + 1.5 ns.

The Single-Ended impedance from TCONN50 + 1 ns to TCONN50 + 1.5 ns is ZO_CABLE_SHORT and the arithmetic mean of ZO_CABLE_SHORT is ZO_CABLE_AVE.

  - If (42.5 ohms <= ZO_TRANSITION <= 57.5 ohms) AND (42.5 ohms <=ZO_CABLE_SHORT <= 57.5 ohms) AND (45 ohms < ZO_CABLE_AVE<=55  ohms), it is PASS

  - If (32.5 ohms <= ZO_TRANSITION < 42.5 ohms) OR (57.5 ohms < ZO_TRANSITION <= 62.5 ohms), the impedance is in the excursion range. If the excursion occurs at a single time AND the duration of the single excursion is less than 250ps AND (42.5 ohms <=ZO_CABLE_SHORT <= 57.5 ohms) AND (45 ohms < ZO_CABLE_AVE<=55 ohms), it is PASS

  - All other cases are FAIL.

Repeat Step 5-9 for the sink end of the Cable DUT.

Single-Ended eCBUS Insertion Loss

This test confirms that MHL cable Single-Ended eCBUS Insertion Loss meets the required specification.

Figure 7-3. Single-Ended eCBUS Insertion Loss Test Setup

- Connect the co-ax cables as shown in Figure 7-3.

Calibrate the co-ax cables using the calibration kit.

Connect the M3-S TPA SRC-R board, M3-S TPA SINK-R and Cable DUT as shown in Figure 7-3.

Measure the 2-port S parameters for eCBUS line.

Calculate S21 for eCBUS Insertion Loss:

  - If S21 is better than the limit in the table below, it is PASS.

| Frequency (MHz) | Attenuation (dB) |
| --- | --- |
| 0 | 0.5 |
| 1 | 0.5 |
| 10 | 0.8 |
| 100 | 2.8 |
| 300 | 5.0 |
| 1000 | 10.0 |

  - Otherwise it is FAIL.

Near-End Crosstalk

This test confirms that the MHL cable Near-End Crosstalk meets the required specification.

Figure 7-4. Near-End Crosstalk Test Setup

If the cable supports Single-Ended eCBUS, then:

- Connect the co-ax cables as shown in Figure 7-4.

Calibrate the co-ax cables using the calibration kit.

Connect the M3-S TPA SRC-R board and Cable DUT as shown in Figure 7-4.

Measure the 3-port S parameters for TMDS+, TMDS- and eCBUS lines.

Calculate Near-End Crosstalk between differential TMDS and eCBUS

  - If Near-End Crosstalk is better than the limit in the table below, it is PASS.

| Frequency Range | Near-End Crosstalk (dB) |
| --- | --- |
| 0-30MHz | -30 |
| >30 to 300MHz | -20 |
| >300MHz to 825MHz | -15 |
| >825MHz to 5.1GHz | -11.5 |

  - Otherwise FAIL.

Repeat steps 4-5 for the sink end of the Cable DUT.

EMI

This test confirms that MHL cable EMI meets the required specification.

- Set MHL3 EMI source device, MHL3 EMI sink device, and cable DUT in accordance to the methods described in CISPR 22: 2008, Section 10.

Perform full scan and check if the cable meet Class B radiated emissions requirements specified in CISPR 22: 2008, Section 6, Table 6 and 8.

Submit test result to ATC.

Cable Assembly Electrical Tests for eCBUS-D cable

MHL3 cable shall pass Cable Assembly Electrical Test in MHL2.1 CTS. In 2.1 MHL CTS, MHL+ and MHL- represent TMDS+ and TMDS-, and CBUS represents oCBUS in MHL3. eCBUS-D MHL3 cable shall passes all the tests of 7.2.2.1, 7.2.2.2, 7.2.2.3, , 7.2.2.4, and 7.2.2.4 additionaly.

Wire Assignment

This test confirms that all specified connections are present in cable.

- Refer to one connector as “Connector 1” and the other as “Connector 2”.

Check if TMDS channel (D+ and D-), CBUS (oCBUS, eCBUS+, and eCBUS-), VBUS, and GND pins are assigned in Connector 1 and Connector 2.

Check connection for seven pins (D+, D-, oCBUS, eCBUS+, eCBUS-, VBUS, and GND) between Connector 1 and Connector 2.

If no connection is specified between Connector 1 and Connector 2, then FAIL.

If connection is specified between Connector 1 and Connector 2 but no valid connection, then FAIL.

Differential Intra-Pair Skew

This test confirms that the MHL cable Differential eCBUS Intra-Pair Skew does not exceed the maximum value allowed in the specification.

Figure 7-5. Differential Intra-Pair Skew Test Setup

- Set the TDR oscilloscope to output differential.

Connect the equipment as shown in Figure 7-5.

Set vertical scale 100 mV/div and horizontal scale 400 ps/div.

Measure the differential skew at the 50% crossing times of the two differential edges on the TDT head side.*

  - If the differential skew is less than or equal to 43ps, then PASS.

  - Otherwise FAIL.

*Need to compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables).

Differential Characteristic Impedance

This test confirms that the MHL cable Differential eCBUS Characteristic Impedance is within the range allowed in the specification.

Figure 7-6. Illustration of Cable Length for the Impedance Test

Figure 7-7. Differential Characteristic Impedance Test Setup

Figure 7-8. Set reference point for differential mode

Before testing, compensate the differential intra-pair skew caused by the test fixtures (TPA boards and co-ax cables) and measure cable length as shown in Figure 7-6.

If cable length is greater than 300mm, then:

- Connect the M3-D TPA SRC-R to the TDR oscilloscope.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%). Record the time at the peak of the change point in Figure 7-8. This is the connection point (TCONN).

Connect the M3-D TPA SRC-R to the source end of the Cable DUT and measure the differential impedance from TCONN to TCONN + 2 ns.

The differential impedance from TCONN to TCONN + 1 ns is ZOD_TRANSITION and the differential impedance from TCONN + 1 ns to TCONN + 2 ns is ZOD_CABLE.

  - If (85 ohms <= ZOD_TRANSITION <= 115 ohms) AND (90 ohms <= ZOD_CABLE <= 110 ohms), it is PASS

  - If (65 ohms <= ZOD_TRANSITION < 85 ohms) OR (115 ohms < ZOD_TRANSITION <= 125 ohms), the impedance is in the excursion range. If the excursion occurs at a single time AND the duration of the single excursion is less than 250ps AND (90 ohms <= ZOD_CABLE <= 110 ohms), it is PASS.

  - All other cases are FAIL.

Repeat Step 4 for the sink end of the Cable DUT.

If cable length is less than or equal to 300mm, then:

- Connect the M3-D TPA SRC-R to the TDR oscilloscope.

Set the effective rise time of the differential TDR pulse to 200ps (20 – 80%). Record the time at the peak of the change point in Figure 7-8. This is the connection point (TCONN200).

Connect the M3-D TPA SRC-R to the source end of the Cable DUT and measure the differential impedance from TCONN200 to TCONN200 + 1 ns.

The Differential impedance from TCONN200 to TCONN200 + 1 ns is ZOD_TRANSITION

Set the effective rise time of the differential TDR pulse to 50ps (20 – 80%). Record the time at the peak of the change point in Figure 7-8. This is the connection point (TCONN50).

Connect the M3-D TPA SRC-R to the source end of the Cable DUT and measure the differential impedance from TCONN50 +1 ns to TCONN50 + 1.5 ns.

The Differential impedance from TCONN50 + 1 ns to TCONN50 + 1.5 ns is ZOD_CABLE_SHORT and the arithmetic mean of ZOD_CABLE_SHORT is ZOD_CABLE_AVE.

  - If(85 ohms <=ZOD_TRANSITION <=115 ohms) AND (85 ohms <= ZOD_CABLE_SHORT <=115 ohms) AND (90 ohms <=ZOD_CABLE_AVE <= 110 ohms), it is PASS.

  - If (65 ohms <= ZOD_TRANSITION < 85 ohms) OR (115 ohms < ZOD_TRANSITION <= 125 ohms), the impedance is in the excursion range. If the excursion occurs at a single time AND the duration of the single excursion is less than 250ps AND (85 ohms <= ZOD_CABLE_SHORT <=115 ohms) AND (90 ohms <=ZOD_CABLE_AVE <= 110 ohms), it is PASS.

  - All other cases are FAIL.

Repeat Step 4-7 for the sink end of the Cable DUT.

Near-End Crosstalk

This test confirms that MHL cable Near-End Crosstalk meets the required specification.

Figure 7-9. Near-End Crosstalk Test Setup

If the cable supports Single-Ended eCBUS, then:

- Connect the co-ax cables as shown in Figure 7-9.

Calibrate the co-ax cables using the calibration kit.

Connect the M3-D TPA SRC-R board and Cable DUT as shown in Figure 7-9.

Measure the 2-port S parameters for TMDS+ and eCBUS+ lines.

Calculate S21 for Near-End Crosstalk

  - If S21 is better than the limit in the table below, it is PASS.

| Frequency Range | Near-End Crosstalk (dB) |
| --- | --- |
| 0-30MHz | -30 |
| >30 to 300MHz | -20 |
| >300MHz to 825MHz | -15 |
| >825MHz to 5.1GHz | -11.5 |

  - Otherwise FAIL.

Repeat steps 4-5 for (TMDS+, eCBUS-), (TMDS-, eCBUS+), (TMDS-, eCBUS-) pairs.

Repeat steps 4-6 for the sink end of the Cable DUT.

EMI

This test confirms that MHL cable EMI meets the required specification.

- Set MHL3 EMI source device, MHL3 EMI sink device, and cable DUT in accordance to the methods described in CISPR 22: 2008, Section 10.

Perform full scan and check if the cable meet Class B radiated emissions requirements specified in CISPR 22: 2008, Section 6, Table 6 and 8.

Submit test result to ATC.

END OF TEST SUITE

# Appendices

## Capability Declaration Form (CDF)

Each Adopter submits their system for compliance testing accompanied by a CDF. The CDF specifies the capabilities of the system, such as the list of supported video and audio modes.

The CDF uses fields defined in this CTS. Fields are listed in Table 8-1 with definitions. In all cases, definitions for these fields or features as written in the MHL Specification take precedence over the simplified definitions listed here.

Table 8-1. CDF Field Definitions

| CDF Field Name |  | CDF Field Definition |  |
| --- | --- | --- | --- |
| CDF_AUDIO_2CH_32kHz | Device supports 2-channel audio at 32kHz sample rate. |  |  |
| CDF_AUDIO_2CH_44kHz | Device supports 2-channel audio at 44.1kHz sample rate. |  |  |
| CDF_AUDIO_2CH_48kHz | Device supports 2-channel audio at 48kHz sample rate. |  |  |
| CDF_AUDIO_2CH_88kHz | Device supports 2-channel audio at 88.1kHz sample rate. |  |  |
| CDF_AUDIO_2CH_96kHz | Device supports 2-channel audio at 96kHz sample rate. |  |  |
| CDF_AUDIO_2CH_176kHz | Device supports 2-channel audio at 176kHz sample rate. |  |  |
| CDF_AUDIO_2CH_192kHz | Device supports 2-channel audio at 192kHz sample rate. |  |  |
| CDF_AUDIO_SAMPLE_SIZE |  | Define the sample size (in bits) of audio data. |  |
| CDF_AUDIO_ADT_SUPPORT |  | Device supports Audio Data Tunneling |  |
| CDF_AVI_SUPPORT |  | Device supports AVI InfoFrames. |  |
| CDF_CR_ADOPTER_ID_H | High-order byte of 16-bit MHL ADOPTER ID register. |  |  |
| CDF_CR_ADOPTER_ID_L |  | Low-order byte of 16-bit MHL ADOPTER ID register. |  |
| CDF_CR_AUD_2CH |  | Device supports 2-channel audio. |  |
| CDF_CR_AUD_8CH |  | Device supports 8-channel audio. |  |
| CDF_CR_AUD_HBR |  | Device indicates HBR Audio support in Cap. Reg. |  |
| CDF_AUDIO_HBR |  | Device supports HBR Audio. |  |
| CDF_AUDIO_MAX_FS_MULTI_CH |  | Maximum audio sample rate for multi-channel. |  |
| CDF_AUDIO_PCM_CHANNELS |  | Maximum number of audio channels in L-PCM. |  |
| CDF_CR_BANDWIDTH | Define maximum TMDS bandwidth (refer to Specification). |  |  |
| CDF_CR_DEVICE_ID_H |  | High-order byte of 16-bit MHL DEVICE ID register. |  |
| CDF_CR_DEVICE_ID_L |  | Low-order byte of 16-bit MHL DEVICE ID register. |  |
| CDF_CR_INT_SIZE |  | Number of Interrupt Registers. |  |
| CDF_CR_LD_AUDIO |  | Logical Device Map: device supports audio handling. |  |
| CDF_CR_LD_DISPLAY |  | Logical Device Map: device supports display output. |  |
| CDF_CR_LD_GUI |  | Logical Device Map: device support graphical user interface. |  |
| CDF_CR_LD_MEDIA | Logical Device Map: device supports media handling. |  |  |
| CDF_CR_LD_RECORD |  | Logical Device Map: device supports recording. |  |
| CDF_CR_LD_SPEAKER |  | Logical Device Map: device supports audio output. |  |
| CDF_CR_LD_TUNER |  | Logical Device Map: device includes tuner control. |  |
| CDF_CR_LD_VIDEO |  | Logical Device Map: device supports video handling. |  |
| CDF_CR_MHL_VER_MAJOR | Major MHL Specification revision supported by device. |  |  |
| CDF_CR_MHL_VER_MINOR | Minor MHL Specification revision supported by device. |  |  |
| CDF_CR_MHL_VERSION | Combination of MHL_VER_MAJOR and MHL_VER_MINOR. |  |  |
| CDF_CR_POW | Define the supported power mode (refer to Specification). |  |  |
| CDF_CR_RAP_SUPPORT |  | Device supports RAP sub-commands. |  |
| CDF_CR_RCP_SUPPORT |  | Device supports RCP sub-commands. |  |
| CDF_CR_SCRATCHPAD_SIZE |  | Number of Scratchpad Registers for WRITE_BURST. |  |
| CDF_CR_SP_SUPPORT |  | Device supports WRITE_BURST (Scratchpad). |  |
| CDF_CR_STAT_SIZE |  | Number of Status Registers. |  |
| CDF_CR_SUPP_ISLANDS |  | Device supports data islands. |  |
| CDF_CR_SUPP_PPIXEL |  | Device supports PackedPixel mode. |  |
| CDF_CR_SUPP_RGB444 |  | Device supports RGB 4:4:4 color space. |  |
| CDF_CR_SUPP_VGA |  | Device supports VGA (640x480) video mode. |  |
| CDF_CR_SUPP_16BPP |  | Device supports 16-bits-per-pixel mode. |  |
| CDF_CR_SUPP_YCBCR422 |  | Device supports YcbCr 4:2:2 color space. |  |
| CDF_CR_SUPP_YCBCR444 |  | Device supports YcbCr 4:4:4 color space. |  |
| CDF_CR_UCP_RECV_SUPPORT |  | Device supports receiving UCP sub-commands. |  |
| CDF_CR_UCP_SEND_SUPPORT |  | Device supports sending UCP sub-commands. |  |
| CDF_CR_SUPP_VT |  | Device support video types. |  |
| CDF_CR_VT_CINEMA | Device supports Cinema video type (refer to Specification). |  |  |
| CDF_CR_VT_GAME | Device supports Game video type (refer to Specification). |  |  |
| CDF_CR_VT_GRAPHICS | Device supports Graphics video type (refer to Specification). |  |  |
| CDF_CR_VT_PHOTO | Device supports Photo video type (refer to Specification). |  |  |
| CDF_CR_DEV_TYPE | Device type as indicated in Capability Register. |  |  |
| CDF_CR_PLIM | Device's power output limit in Capability Register. |  |  |
| CDF_XCR_ECBUS_SPEEDS | Device Extended CR eCBUS Bus Speed support. |  |  |
| CDF_XCR_TMDS_SPEEDS | Device Extended CR TMDS Speed support |  |  |
| CDF_XCR_ECBUS_DEV_ROLES | Device Extended CR Device Roles |  |  |
| CDF_XCR_LOG_DEV_MAPX | Device Extended CR Logical Device Map (Extended) |  |  |
| CDF_D_POWERED | Dongle device outputs VBUS power. |  |  |
| CDF_VBUS_DRIVER |  | Device maintains VBUS output when switching from oCBUS to eCBUS. |  |
| CDF_D_ACCEPTS_POWER_FROM_SOURCE |  | Dongle device accepts VBUS power input from Source. |  |
| CDF_D_CM_CABLE_END_DISTANCE |  | Define common-mode length of Dongle’s captive cable. |  |
| CDF_D_CM_CABLE_START_DISTANCE |  | Define common-mode length of Dongle’s captive cable. |  |
| CDF_D_CM_TERM_DISTANCE | Define common-mode length of Dongle’s termination. |  |  |
| CDF_D_DF_CABLE_END_DISTANCE |  | Define differential length of Dongle’s captive cable. |  |
| CDF_D_DF_CABLE_START_DISTANCE |  | Define differential length of Dongle’s captive cable. |  |
| CDF_D_DF_TERM_DISTANCE |  | Define differential length of Dongle’s termination. |  |
| CDF_D_SE_CABLE_END_DISTANCE |  | Define single-ended length of Dongle’s captive cable. |  |
| CDF_D_SE_CABLE_START_DISTANCE |  | Define single-ended length of Dongle’s captive cable. |  |
| CDF_D_SE_TERM_DISTANCE |  | Define single-ended length of Dongle’s termination. |  |
| CDF_D_MAX_CBUS_CAP |  | Define maximum CBUS capacitance of Dongle. |  |
| CDF_D_MAX_POWER_DOWN | Define maximum wait time when powering down Dongle. |  |  |
| CDF_D_MAX_POWER_UP | Define maximum wait time when powering up Dongle. |  |  |
| CDF_D_MAX_STANDBY_TO_ACTIVE |  | Define maximum wait time from Standby to Active. |  |
| CDF_D_POWERED |  | Define if Dongle is powered or unpowered. |  |
| CDF_DDC_SEGMENT_READ_SUPPORT |  | Device supports DDC Segment Read commands. |  |
| CDF_HDCP_SUPPORT |  | Device supports HDCP. |  |
| CDF_HDCP_REPEATER | Device is an HDCP Repeater, internally decrypting and re-encrypting the content stream, as defined in the HDCP Specification. |  |  |
| CDF_LOG_DEV_MAP_CHANGE | Device allows Logical Device Map to change in real-time. |  |  |
| CDF_PROC_SET_ACTIVE |  | Define procedure to set Device into Active mode. |  |
| CDF_PROC_SET_STANDBY |  | Define procedure to set Device into Standby mode. |  |
| CDF_PROC_SOURCE_POWER_STANDBY |  | Define if Source Device can output VBUS in Standby. |  |
| CDF_PRODUCT_HEV_CAPABLE |  | Device supports High End Video modes. |  |
| CDF_VIDEO_HEV |  | Device is capable of generating High End Video. |  |
| CDF_VIDEO_16_BIT |  | Device supports 16-bits-per-pixel video mode. |  |
| CDF_PRODUCT_3D_CAPABLE |  | Define is capable on other ports to support 3D video. |  |
| CDF_RAP_SUPPORT |  | Device supports RAP sub-commands. |  |
| CDF_CR_RBP_SUPPORT |  | Device supports RBP. |  |
| CDF_CR_XVYCC_SUPPORT |  | Device supports xvYCC pixel formatting. |  |
| CDF_CR_ECBUS_S_075 |  | Device indicates 75MHz eCBUS-S in Cap. Reg. |  |
| CDF_CR_ECBUS_S_8BIT |  | Device indicates 8-bit on eCBUS-S in Cap. Reg. |  |
| CDF_CR_ECBUS_S_12BIT |  | Device indicates 12-bit on eCBUS-S in Cap. Reg. |  |
| CDF_CR_ECBUS_D_150 |  | Device indicates 150MHz eCBUS-D in Cap. Reg. |  |
| CDF_CR_ECBUS_D_8BIT |  | Device indicates 8-bit on eCBUS-D in Cap. Reg. |  |
| CDF_CR_TMDS_150 |  | Device indicates 1.50GHz link speed in Cap. Reg. |  |
| CDF_CR_TMDS_300 |  | Device indicates 3.00GHz link speed in Cap. Reg. |  |
| CDF_CR_TMDS_600 |  | Device indicates 6.00GHz link speed in Cap. Reg. |  |
| CDF_CR_DEV_HOST |  | Device indicated USB Host support in Cap. Reg. |  |
| CDF_CR_DEV_DEVICE |  | Device indicates USB Device support in Cap. Reg. |  |
| CDF_CR_DEV_CHARGER |  | Device indicates USB Charger support in Cap. Reg. |  |
| CDF_CR_HID_HOST |  | Device indicates HID Host support in Cap. Reg. |  |
| CDF_CR_HID_DEVICE |  | Device indicates HID Device support in Cap. Reg. |  |
| CDF_CR_LD_PHONE |  | Device indicates Log Dev of Phone in Cap. Reg. |  |
| CDF_RBP_RECV_SUPPORT |  | Device supports receiving RBP sub-commands. |  |
| CDF_RBP_SEND_SUPPORT |  | Device supports sending RBP sub-commands. |  |
| CDF_RCP_RECEIVE |  | Device supports receiving RCP sub-commands. |  |
| CDF_RCP_SEND |  | Device supports sending RCP sub-commands. |  |
| CDF_SINK_CABLE_DETECT_TO_R_DISCOVER |  | Define max wait time from Cable Detect to ZCBUS check. |  |
| CDF_SINK_CM_TERM_DISTANCE |  | Define common-mode length of Sink termination. |  |
| CDF_SINK_DF_TERM_DISTANCE |  | Define differential length of Sink termination. |  |
| CDF_SINK_SE_TERM_DISTANCE |  | Define single-ended length of Sink's termination. |  |
| CDF_DirSink_DF_CABLE_END_DISTANCE |  | Define differential length of Direct Attach Sink's input. |  |
| CDF_DirSink_DF_CABLE_START_DISTANCE |  | Define differential length of Direct Attach Sink's input. |  |
| CDF_DirSink_SE_CABLE_END_DISTANCE |  | Define single-ended length of Direct Attach Sink's input. |  |
| CDF_DirSink_SE_CABLE_START_DISTANCE |  | Define single-ended length of Direct Attach Sink's input. |  |
| CDF_SINK_MAX_STANDBY_TO_ACTIVE |  | Define max wait time from Standby to Active. |  |
| CDF_SINK_WAKE_FROM_STANDBY |  | Define max wait time from power-off to Standby. |  |
| CDF_SINK_DC |  | Device is a DC-powered Sink. |  |
| CDF_SINK_POWERED |  | Device can output VBUS. |  |
| CDF_SOURCE_YQ_FULL |  | Device support full-range colors with YQ field. |  |
| CDF_SRC_DF_TERM_DISTANCE |  | Define differential length of Source's termination. |  |
| CDF_SRC_SE_TERM_DISTANCE |  | Define single-ended length of Source's termination. |  |
| CDF_SRC_POWERED |  | Device is powered. |  |
| CDF_SRC_PROC_DF_SIGNAL_ONLY |  | Define procedure to output differential signal only. |  |
| CDF_SRC_PROC_STANDBY_DISCOVERY |  | Define procedure to change from Standby to do discovery. |  |
| CDF_SRC_IS_DIRECT_ATTACH |  | Device is a Direct Attach Source. |  |
| CDF_UCP_RECV_APPLICATION |  | Define application which supports UCP receiving. |  |
| CDF_UCP_RECV_SUPPORT |  | Device supports receiving UCP sub-commands. |  |
| CDF_UCP_SEND_APPLICATION |  | Define application which supports UCP sending. |  |
| CDF_UCP_SEND_SUPPORT |  | Device supports sending UCP sub-commands. |  |
| CDF_VIDEO_1080i_50 |  | Device supports 1080i/50Hz video mode. |  |
| CDF_VIDEO_1080i_60 |  | Device supports 1080i/60Hz video mode. |  |
| CDF_VIDEO_1280x720P_50_3D_Frame | Device supports 720p 3D / 50Hz Frame Sequential mode. |  |  |
| CDF_VIDEO_1280x720P_50_3D_Top_Bottom |  | Device supports 720p 3D / 50Hz Top-Bottom mode. |  |
| CDF_VIDEO_1280x720P_60_3D_Frame | Device supports 720p 3D / 60Hz Frame Sequential mode. |  |  |
| CDF_VIDEO_1280x720P_60_3D_Top_Bottom |  | Device supports 720p 3D / 60Hz Top-Bottom mode. |  |
| CDF_VIDEO_1920x1080i_50_3D_Left_Right |  | Device supports 1080i 3D / 50Hz Left-Right mode. |  |
| CDF_VIDEO_1920x1080i_60_3D_Left_Right |  | Device supports 1080i 3D / 60Hz Left-Right mode. |  |
| CDF_VIDEO_1920x1080p_24_3D_Frame | Device supports 1080p 3D / 24Hz Frame Sequential mode. |  |  |
| CDF_VIDEO_1920x1080p_24_Top_Bottom |  | Device supports 1080p 3D / 24Hz Top-Bottom mode. |  |
| CDF_VIDEO_3D |  | Device supports 3D video. |  |
| CDF_VIDEO_480i_60_2X |  | Device supports 480i / 60Hz (2x replication) video. |  |
| CDF_VIDEO_480i_60_4X |  | Device supports 480i / 60Hz (4x replication) video. |  |
| CDF_VIDEO_480p_60 |  | Device supports 480p / 60Hz video. |  |
| CDF_VIDEO_480p_60_2X |  | Device supports 480p / 60Hz (2x replication) video. |  |
| CDF_VIDEO_576i_50_2X |  | Device supports 576i / 50Hz (2x replication) video. |  |
| CDF_VIDEO_576i_50_4X |  | Device supports 576i / 50Hz (4x replication) video. |  |
| CDF_VIDEO_576p_50 |  | Device supports 576p / 50Hz video. |  |
| CDF_VIDEO_576p_50_2X |  | Device supports 576p / 50Hz (2x replication) video. |  |
| CDF_VIDEO_576p_50_4X |  | Device supports 576p / 50Hz (4x replication) video. |  |
| CDF_VIDEO_720p_50 |  | Device supports 720p / 50Hz video. |  |
| CDF_VIDEO_720p_60 |  | Device supports 720p / 60Hz video. |  |
| CDF_VIDEO_PACKEDPIXEL |  | Device supports PackedPixel mode. |  |
| CDF_VIDEO_RGB |  | Device supports RGB color space. |  |
| CDF_VIDEO_VGA |  | Device supports VGA video mode. |  |
| CDF_VIDEO_YCBCR_422 |  | Device supports YCbCr 4:2:2 color space. |  |
| CDF_VIDEO_YCBCR_444 |  | Device supports YCbCr 4:4:4 color space. |  |
| CDF_VIDEO_YCC_FULL |  | Device supports full-range YCbCr. |  |
| CDF_VIDEO_MULTIVIEW |  | Device supports Multi view function. |  |
| CDF_PRODUCT_MULTIVIEW_CAPABLE |  | Device supports Multi view function. |  |
| CDF_VIDEO_FS_SUPP |  | Device supports Frame Sequential timings. |  |
| CDF_VIDEO_FSLR_SUPP |  | Device supports FS combined with LR video. |  |
| CDF_VIDEO_FSTB_SUPP |  | Device supports FS combined with TB video. |  |
| CDF_VIDEO_LR_SUPP |  | Device supports Left-Right video timings. |  |
| CDF_VIDEO_TB_SUPP |  | Device supports Top-Bottom video timings. |  |
| CDF_VIDEO_TBLR_SUPP |  | Device supports TB combined with LR video. |  |
| CDF_VIDEO_SEP_AUD |  | Device supports multi-stream audio for Multi-View. |  |
| CDF_VIDEO_DUAL3D |  | Device supports DUAL-3D function. |  |
| CDF_AV_SYNC_SUPPORTED |  | Device supports AV Sync (audio delay function). |  |
| CDF_ADT_RECEIVE_SUPPORT |  | Device supports receiving audio through ADT. |  |
| CDF_ADT_SEND_SUPPORT |  | Device supports sending audio through ADT. |  |
| CDF_ADT_PLAYING_SUPPORT |  | Device supports receiving and rendering ADT audio. |  |
| CDF_ADT_SEND_PROC |  | Procedure to start ADT sending from DUT. |  |
| CDF_ADT_RECEIVE_PROC |  | Procedure to start ADT receiving into DUT. |  |
| CDF_LINK_SPEED_150 |  | Device supports 1.50GHz link speed. |  |
| CDF_LINK_SPEED_300 |  | Device supports 3.00GHz link speed. |  |
| CDF_LINK_SPEED_600 |  | Device supports 6.00GHz link speed. |  |
| CDF_EXTENDED_CONTENT_SUPPORT |  | Device supports Extended Content Types in video. |  |
| CDF_PROC_SRC_EXTENDED_CONTENT_SELECT |  |  | Procedure to select different Extended Content types to Source DUT output. |
| CDF_eMSC_SUPPORT |  | Device supports eMSC. |  |
| CDF_eCBUS_D_SUPPORT |  | Device supports eCBUS-D. |  |
| CDF_eCBUS_S_SUPPORT |  | Device supports eCBUS-S. |  |
| CDF_12_BIT_TDM_SLOT_MODE_SUPPORT |  | Device supports 12-bit eCBUS timings. |  |
| CDF_8_BIT_TDM_SLOT_MODE_SUPPORT |  | Device supports 8-bit eCBUS timings. |  |
| CDF_AVLINK_BIST_PRBS_SUPP |  | Device supports AVLINK BIST PRBS. |  |

## Test Results Form (TRF)

The Test Results Form is created by an authorized test center (ATC) while performing the tests from this CTS on the device under test (DUT).

END_OF_DOCUMENT

Macro Listings

Visual Basic for Applications Macros

Macro Listing

'-------------------------------------------------------------------------------------------- ' output_Test_Definitions_to_CSV '-------------------------------------------------------------------------------------------- Sub output_Test_Definitions_to_CSV() Dim docStream1 As FileSystemObject, outStream1 As TextStream Dim docStream2 As FileSystemObject, outStream2 As TextStream Dim strFilePath As String, strFileName As String Dim strCSVFileFullPath As String, strCSVFileName As String Dim strOutFileFullPath As String, strOutFileName As String Dim doc As Document Dim rng As Range Dim bm As Bookmark Dim tbl As Table Dim par As Paragraph, parFind As Paragraph Dim i As Integer, j As Integer, k As Integer, m As Integer Dim str As String, strIndex As String, strPhrase As String Dim strTestHeading As String, strTestObjective As String Dim strTestGroup As String Dim strPrevTestObjective As String, strPrevTestHeading As String Dim strPrevTestGroup As String, strPrevTestIndex As String Dim strCDFList As String, strPrevCDFList As String Dim bAppendText As Boolean, bFoundTest As Boolean Dim bOutputTest As Boolean, bOutputHeadings As Boolean strFilePath = ActiveDocument.Path strFileName = ActiveDocument.Name strCSVFileName = Left(strFileName, InStrRev(strFileName, ".")) & "csv" strCSVFileFullPath = strFilePath & "\" & strCSVFileName strOutFileName = Left(strFileName, InStrRev(strFileName, ".")) & "txt" strOutFileFullPath = strFilePath & "\" & strOutFileName 'Debug.Print strOutFileFullPath & vbCrLf & strOutFileName Set docStream1 = New FileSystemObject Set outStream1 = docStream1.CreateTextFile(strCSVFileFullPath, Overwrite:=True) Set docStream2 = New FileSystemObject Set outStream2 = docStream2.CreateTextFile(strOutFileFullPath, Overwrite:=True) Set doc = ActiveDocument m = 0 bFoundTest = False bOutputTest = False bOutputHeadings = True strTestHeading = "" strTestGroup = "" strTestObjective = "" strTestIndex = "" strCDFList = "" For Each par In doc.Paragraphs strPhrase = Left(par.Range.Text, Len(par.Range.Text) – 1) strPrevTestGroup = strTestGroup strPrevTestIndex = strTestIndex strPrevTestHeading = strTestHeading strPrevTestObjective = strTestObjective strPrevCDFList = strCDFList bAppendText = False str = Format(par.OutlineLevel, "000") & ": " & Left(par.Style, 25) Select Case par.Style Case "Heading 1" If bFoundTest Then bOutputTest = True bFoundTest = False strTestGroup = "" strTestIndex = "" strTestHeading = "" strTestObjective = "" strCDFList = "" Case "Heading 2" If bFoundTest Then bOutputTest = True bFoundTest = False bAppendText = True strTestGroup = "" strTestIndex = "" strTestHeading = "" strTestObjective = "" strCDFList = "" Case "Test Group Heading" If bFoundTest Then bOutputTest = True bFoundTest = False bAppendText = True strTestGroup = strPhrase strTestIndex = "" strTestHeading = "" strTestObjective = "" strCDFList = "" k = 0 Case "Test Heading" If bFoundTest Then bOutputTest = True bFoundTest = True bAppendText = True bResetFields = False strTestIndex = par.Range.ListFormat.ListString strTestHeading = strPhrase strTestObjective = "" strCDFList = "" Case "Test Objective" bFoundTest = True bAppendText = True strCDFList = "" strTestObjective = strPhrase Case Else End Select Set rng = par.Range With rng With .Find .ClearFormatting .Text = "<(CDF_[A-Za-z_0-9]{1,100})" .Forward = True .Wrap = wdFindStop .MatchWholeWord = False .MatchWildcards = True .MatchSoundsLike = False .MatchAllWordForms = False .Execute End With Do While .Find.Found And k < 25 And rng.InRange(par.Range) Set parFind = .Paragraphs(1) strIndex = parFind.Range.ListFormat.ListString strTitle = Left(parFind.Range.Text, Len(parFind.Range.Text) – 1) pg = parFind.Range.Information(wdActiveEndAdjustedPageNumber) If strCDFList <> "" Then strCDFList = strCDFList & "," strCDFList = strCDFList & rng.Text .Find.Execute k = k + 1 Loop End With If bOutputTest Then If bOutputHeadings Then outStream1.WriteLine "Test Index" & vbTab & _ "Test Group" & vbTab & _ "Test Heading" & vbTab & "Test Objective" & vbTab & _ "CDF Usage" bOutputHeadings = False End If outStream1.WriteLine strPrevTestIndex & vbTab & _ strPrevTestGroup & vbTab & _ strPrevTestHeading & vbTab & _ strPrevTestObjective & vbTab & _ strPrevCDFList bOutputTest = False End If If bAppendText Then str = str & " [" & strPhrase & "]" outStream2.WriteLine str & "{" & strCDFList & "}" End If m = m + 1 Next par outStream1.Close outStream2.Close StatusBar = "Completed Processing" End Sub

'-------------------------------------------------------------------------------------------- ' getTestIndex(par) ' Search from the paragraph backwards (toward beginning of the document) to find ' a paragraph which uses the style "Test Heading". Return that paragraph's full number. ' If the target is not found within 10 paragraphs, then return empty string. '-------------------------------------------------------------------------------------------- Function getTestIndex(par As Paragraph) As String Dim k As Integer k = 0 Do While par.Range.Style <> "Test Heading" And k < 10 Set par = par.Previous Loop If par.Range.Style = "Test Heading" Then getTestIndex = par.Range.ListFormat.ListString Else getTestIndex = "" End If End Function Function isCategoryTested(strDevCat As String, tbl As Table) As Integer Dim i As Integer, j As Integer isCategoryTested = 0 For i = 2 To 4 Step 1 If InStr(1, tbl.Rows(i).Cells(1).Range.Text, strDevCat, vbBinaryCompare) > 0 Then For j = 2 To 4 Step 1 If InStr(1, tbl.Rows(i).Cells(j).Range.Text, "X", vbBinaryCompare) > 0 Then isCategoryTested = i Exit For End If Next j End If Next i End Function '-------------------------------------------------------------------------------------------- ' fillTable_with_Test_Crossreferences '-------------------------------------------------------------------------------------------- Sub fillTable_with_Test_Crossreferences() Dim doc As Document Dim par As Paragraph Dim rng As Range, rngTarget As Range Dim str As String Dim strIndex As String Dim i As Integer, j As Integer, k As Integer, m As Integer Dim strSectionBookmark(6) As String Dim strSuiteBookmark(6) As String Dim strDevCategory(6) As String Dim tbl As Table Dim tblTarget As Table Dim myCell As Cell Dim myRow As Row, rowTarget As Row Dim bFoundTestHeading As Boolean Dim bAddTest As Boolean strSectionBookmark(0) = "SUITE_Source_CBUS_4" strSectionBookmark(1) = "SUITE_Source_CBUS_5" strSectionBookmark(2) = "SUITE_Sink_CBUS_4" strSectionBookmark(3) = "SUITE_Sink_CBUS_5" strSectionBookmark(4) = "SUITE_Dongle_CBUS_4" strSectionBookmark(5) = "SUITE_Dongle_CBUS_5" strSuiteBookmark(0) = "SUITE_CBUS_6_4" strSuiteBookmark(1) = "SUITE_CBUS_6_5" strSuiteBookmark(2) = "SUITE_CBUS_6_4" strSuiteBookmark(3) = "SUITE_CBUS_6_5" strSuiteBookmark(4) = "SUITE_CBUS_6_4" strSuiteBookmark(5) = "SUITE_CBUS_6_5" strDevCategory(0) = "Source" strDevCategory(1) = "Source" strDevCategory(2) = "Sink" strDevCategory(3) = "Sink" strDevCategory(4) = "Dongle" strDevCategory(5) = "Dongle" Set doc = ActiveDocument '------------------------------------------------------------------------------- ' Loop through 3.4, 3.5, 4.4, 4.5, 5.4, 5.5, adding rows to the summary table ' in each section according to which tests in 6.4 and 6.5 are executed. '------------------------------------------------------------------------------- For i = Lbound(strSectionBookmark) To Ubound(strSectionBookmark) – 1 Step 1 Debug.Print "Checking Section '" & strSectionBookmark(i) & "'" '------------------------------------------------------------------------------- ' Point to the summary table in section 3.4 or 3.5 or 4.4 or 4.5 or 5.4 or 5.5 '------------------------------------------------------------------------------- Set rngTarget = doc.Bookmarks(strSectionBookmark(i)).Range Set tblTarget = rngTarget.Tables(1) If tblTarget.Rows.Count >= 3 Then For j = tblTarget.Rows.Count To 3 Step -1 ' Remove rows previously added tblTarget.Rows(j).Delete ' so that the table has only Next j ' one empty row after headings. End If '------------------------------------------------------------------------------- ' Loop through the suite of tests, checking the first table in each test. '------------------------------------------------------------------------------- Set rng = doc.Bookmarks(strSuiteBookmark(i)).Range For Each tbl In rng.Tables If tbl.Cell(2, 1).Range.Style = "Category List" Then ' Found mapping table Set par = tbl.Range.Paragraphs(1) strIndex = getTestIndex(par) If Len(strIndex) > 0 Then ' Test number found, now check table… Debug.Print vbTab & strIndex j = isCategoryTested(strDevCategory(i), tbl) ' If Category is tested... If j > 0 Then tblTarget.Rows.Add Set rowTarget = tblTarget.Rows.Last rowTarget.Cells(1).Range.Text = strIndex ' Test Number in Col 1 For k = 2 To 4 Step 1 str = tbl.Rows(j).Cells(k).Range.Text str = Left(str, Len(st–) - 2) ' Drop CRLF rowTarget.Cells(k).Range.Text = str Next k End If End If End If Next tbl Next i Debug.Print "Finished updating summary tables." End Sub

Sub outputTestCoverageTable() Dim doc As Document Dim myStyle As Style Dim par As Paragraph Dim rng As Range, rngBody As Range Dim str As String, strStyleName As String, strRangeText As String, strTestTitle As String Dim i As Integer, iCount As Integer, j As Integer Dim bStyleOkay As Boolean Dim strTestGroup(2) As String Dim wdAppl As Word.Application Dim wdDocu As Word.Document Dim strFilePath As String, strDocFileFullName Dim tbl(2) As Table Dim myRow As Row Set wdAppl = CreateObject("Word.Application") strFilePath = ActiveDocument.Path Debug.Print "Path: " & strFilePath strDocFileFullName = strFilePath & "\" & "Output.docx" Set wdDocu = wdAppl.Documents.Open(strDocFileFullName) Set tbl(1) = wdDocu.Range.Tables(1) Set tbl(2) = wdDocu.Range.Tables(2) Debug.Print "Check for proper paragraph styles and sequence in each test definition..." strTestGroup(1) = "Included" strTestGroup(2) = "Postponed" Set rngBody = ActiveDocument.Bookmarks("BODY").Range For j = 1 To 2 Step 1 For i = 3 To tbl(j).Rows.Count tbl(j).Rows(i).Delete Next i i = 0 iCount = 0 For Each par In rngBody.Paragraphs With par.Range strRangeText = Left(.Text, Len(.Text) - 1) 'Strip CR If .Style Is Nothing Then Else Select Case .Style Case "Normal", "Table Grid", "Test Objective", "Test Group Heading": Case "Test Heading": strTestTitle = Left(.Text, Len(.Text) - 1) Case "Hidden Test Details": If InStr(1, strRangeText, strTestGroup(j)) Then iCount = iCount + 1 Debug.Print "HTD[" & Format(iCount, "0000") & "]-" & _ strTestGroup(j) & ": {" & strRangeText & "}" tbl(j).Rows.Add Set myRow = tbl(j).Rows(tbl(j).Rows.Count) myRow.Cells(1).Range.Text = Format(iCount) myRow.Cells(2).Range.Text = Left(strRangeText, InStr(1, strRangeText, " ") - 1) myRow.Cells(3).Range.Text = strTestTitle End If Case "RequiredMethods": Case Else: End Select End If End With Next par 'wdDocu.Range.InsertParagraph Next j wdDocu.Save wdDocu.Close End Sub

DOCUMENT REVISION HISTORY

Information in the following change history is intended for use by the author and developers of this document, and not for general distribution to MHL adopters.

Do not use any header formats in this section, so that no entries from these pages appear in the document’s Table of Contents.

Entries with multiple pages indicate multiple edited points in the document. When a specific page number appears more than once in an entry, it indicates multiple edits on that page.

If a bookmark cross-reference appears with the words Error! Bookmark not defined., or a heading cross-reference appears with the words Error! Reference source not found., it means a link is unresolved in Word.  Change the link to a proper target, or re-code it as plain text. Note that page numbers which are not actual cross-reference links to targets may be out of step with revisions to the document, and may not point to the exact page of the change.

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 rc01 | 2013-06-11 | WA |  | Base document is the released CTS 2.1. |
|  |  |  |  | Insert "{Legacy, bCBUS, eCBUS-S, eCBUS-D}" or variant for subset of these CBUS modes, into the "Test Usage" style paragraph for each test in sections 3.3, 4.3, 5.3, 6.3. |
|  | 2013-06-18 | WA | 2 | Add 3.0 entry to Doc Rev Hist with 2013-09. |
|  |  |  | 25 | Add 3.0 entry to Revision History. Remove all previous entries. |
|  |  |  | 3 | Remove Abbreviated Table of Contents. |
|  |  |  |  | Clarify at start of 3.3, 4.3, 5.3, 6.3 the usage of multiple CBUS modes. |
|  |  |  | 2 | Remove previous versions from Doc Rev Hist. |
|  | 2013-06-25 | WA | 35 | Add Section 3.4: eCBUS Normal Tests. |
|  |  |  | 38 | Add Section 3.5: Source Robustness Tests. |
|  |  |  | 40 | Add Section 3.6: Source eCBUS Normal Mode Tests. |
|  |  |  |  | Add Section 3.7: Source eCBUS Robustness Tests. |
|  |  |  |  | Copy eCBUS tests suite from Source into Sink section (4.4-4.7). |
|  | Above changes from file "MHL3_CBUS_Test_Proposal_021.docx", received from Silicon Image (LB), 2013-06-11. |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 0.80 rc01 | 2013-06-25 | WA |  | Add Source Electrical test suite to 3.1.2-3.1.3. |
|  |  |  | 93 | Add Sink Electrical test suite to 4.1.2-4.1.3. |
|  |  |  | 145 | Add Dongle Electrical test suite to 5.1.2. |
|  |  |  |  | Add Cable Electrical test suite to 7.2.2. |
|  | Above changes from file "CTS-3_0_Main-Electrical-Rev0_80.docx", received from SIMG (BSung), 2013-06-22. |  |  |  |
|  |  |  | throughout | Change all other acronym variations to bCBUS. |
|  |  |  |  | Add new Source Audio tests, including multi-channel and HBRA. |
|  |  |  |  | Add new Sink Audio tests including HBRA. |
|  |  |  |  | Add new Dongle Audio tests including HBRA. |
|  | Above changes from file "CTS-30_Main-EarlyDraft_AVLINK_Tests_VJF-2013-05-21_DN.docx", received from SIMG (DNg), 2013-06-13. |  |  |  |
|  | 2013-06-28 | WA |  | Move TMDS and eCBUS Electrical configuration section to level 4 within Test Procedures section 3.1.2. |
| The above version has been delivered to Silicon Image as Word file on 2013-06-28. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 0.80 rc02 | 2013-07-02 | WA |  | Replace 'BCBUS' with 'bCBUS' throughout. |
|  |  |  |  | Add three new common subroutines to attach DUT to tester. Edit into the first few steps of each test in 3.3, 4.3, 5.3. |
|  |  |  |  | Reduce spacing between paragraphs of like style, and between tables. |
|  |  |  |  | Remove "Objective" subheading in each test; remove section references to the MHL 3.0 Spec (since those section numbers are not final anyway). Adjust numbering to begin at 1 for each test. Adjust indentation of ordered lists to be consistent. |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 0.80 rc02 | 2013-07-08 | WA | 35, 88, 140 | Refer to MHL 2 CTS section 6.3 for CBUS tests, without copying them into this CTS. Replace cross-ref links to section 6.3 in this document with hard-coded test numbers within CTS 2.1. Delete Section 6.3 of this document, since only normative references are needed to CTS 2.1. |
|  |  |  | 189 | Move remaining new tests from 6.3 to beginning of 6.4. |
|  |  |  |  | Remove sections 3,4, 3.5, 4.4, 4.5, 5.4, 5.5; and use instead the common tests for MHL 3 eCBUS Normal Mode Tests (6.4) and MHL 3 eCBUS Robustness Tests (6.5). |
|  |  |  |  | The above 3 major reorganizations remove ~200 pages. Need to be sure that any test invoked in CTS 2.x Section 6.3 can accurately be done in the various CBUS modes and for the device categories. Need to be sure that any test invoked in Sections 6.4-6.5 can accurately be done in various CBUS modes and for the device categories. |
| The above version has been delivered to Silicon Image as Word file on 2013-07-08. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 0.80 rc03 | 2013-07-09 | WA |  | Delete sections 3,3, 4.3, 5.3, and replace with a table in each, with references to each pertinent test in CTS 2 sections 3.3, 4.3, 5.3. |
|  |  |  | 190, 191, 200, 221 | Delete 6.4.5.3, 6.4.5.4, 6.4.6.1, 6.4.6.2, 6.4.13.5, 6.5.3.5; all are run by norm ref to CTS 2 Section 6.3 in sections 3.4, 4.4, 5.4. Per SIMG/LB, 7/09.{BGZ#28755} |
|  |  |  | 38, 40, 89, 92, 143, 144 | Insert lookup tables in 3.5, 3.6, 4.5, 4.6, 5.5, 5.6 to refer to suites of tests in this CTS sections 6.5 and 6.6, applied to CBUS modes. This sets the total page count at ~300 pages. |
|  | 2013-07-14 | WA | 14 | Add intro to System test equipment. Per SIMG/VF, 7/12. |
| The above version has been delivered to Silicon Image as Word file on 2013-07-09. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 0.80 rc03 | 2013-07-14 | WA | 14 | Add intro to System test equipment. Per SIMG/VF, 7/12. |
|  |  |  | 21 | Add prompt for future required patterns to 2.4. |
|  |  |  | 33 | Insert intro to Source System Testing. |
|  |  |  |  | Insert AVLink TX BIST Test. |
|  |  |  |  | Insert AVLink RX BIST Test. |
|  |  |  | 33 | Remove content from most of 3.2, referring to CTS 2; with some new tests (incomplete Required Methodologies). |
|  |  |  | 86 | Remove content from most of 4.2, referring to CTS 2; with some new tests (incomplete Required Methodologies). |
|  |  |  | 138 | Remove content from most of 5.2, referring to CTS 2; with some new tests (incomplete Required Methodologies). |
|  | Above changes from file "CTS-3_0_Main-rc02-2013-07-02-1300-w_bgz _VF_AVLINK_Ver2_with legacy test removed.docx", received from SIMG/VF, 2013-07-12. |  |  |  |
|  |  |  |  | Accept all changes, repair style definitions in Word (bug), etc. |
|  |  |  | 15 | Add 2.3 "Organization of this Document". |
|  |  |  | 35 | Change 3.4 title. |
|  |  |  |  | Insert Table 3-1 with parameter name changes for Sect 3.1. |
|  |  |  |  | Replace test list for 3.1 with new table (now 3.2). |
|  |  |  | 36 | Update test list in table for 3.4. |
|  | Above changes from file "CTS-3_0_Main-rc03-2013-07-09-2300-w_bgz_LB2.docx", received from SIMG/LB, 2013-07-12. |  |  |  |
| The above version has been delivered to 5CTT as Word and PDF file on 2013-07-14. |  |  |  |  |

| Rev | Date | Author | Page(s) |  | Change |
| --- | --- | --- | --- | --- | --- |
| 3.0 0.80 rc04 | 2013-07-15 | WA | 41, 93, 145 |  | Add separate sub-section to include new MHL 3 Electrical tests. |
|  |  |  | 70, 125, 173 |  | Add separate sub-section to include new MHL 3 System tests. |
|  |  |  |  |  | Change caption style to include Section number and '-' separator, for both Table and Figure captions. This prevents later figure or table numbers from changing, outside the scope of 1 Section. |
|  |  |  | 26 | Update "Organization of…" to include new sub-sections. Combine content from earlier "Organization" sub-section (page 15). |  |
|  |  |  |  |  | Fix paragraph style assignments throughout for consistency. |
|  |  |  | 88 |  | Change 4.4 title. |
|  |  |  | 87 |  | Insert new Table 4-1 into 4.3. |
|  |  |  | 88 |  | Insert new Table 4-2 into 4.4. |
|  |  |  | 89 |  | Add second table to 4.4 for Robustness suite. |
|  |  |  | 139 |  | Insert new Table 5-1 into 5.3. |
|  |  |  | 140 |  | Change title of 5.4 |
|  |  |  | 140 |  | Add new table 5-2 to section 5.4. |
|  |  |  | 142 |  | Add second table to 5.4 for Robustness tests |
|  | Above changes from file "CTS-3_0_Main-rc03-2013-07-09-2300-w_bgz_LB2.docx", received from SIMG/LB, 2013-07-12. |  |  |  |  |
|  |  |  |  |  | Fix Required Meth to all use "Required Methodology Heading" style. |
|  |  |  |  |  | Delete 'xl1' through 'xl124' paragraph styles (not used). |
|  | 2013-07-16 | WA | 39 |  | Update test coverage in Table 3-5 in Section 3.5. |
|  |  |  | 40 |  | Update test coverage in Table 3-6 in Section 3.6. |
|  |  |  | 91 |  | Update test coverage in Table 4-4 in Section 4.5. |
|  |  |  | 92 |  | Update test coverage in Table 4-5 in Section 4.6. |
|  |  |  | 143 |  | Update test coverage in Table 5-4 in Section 5.5. |
|  |  |  | 145 |  | Update test coverage in Table 5-5 in Section 5.6. |
|  | Above changes from file "CTS-3_0_Main-rc03-2013-07-09-2300-w_bgz_LB4.docx", received from SIMG/LB, 2013-07-16. |  |  |  |  |
| The above version has been delivered to Silicon Image as Word file on 2013-07-15. |  |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |  |
| --- | --- | --- | --- | --- | --- |
| 3.0 0.80 rc04 | 2013-07-16 | WA | 187 | Fix heading level on 6.4.2. |  |
|  |  |  |  | Insert missing H3 heading. |  |
|  |  |  | 199, 200, 218 |  | Change title of test. |
|  |  |  | 197, 197, 197, 198 |  | Change or add test step. |
|  |  |  | 193 | Split 6.4.9.1 into two tests: one for source, one for sink. |  |
|  |  |  | 34 | Remove individual tests from 3–2 - moved to 3.8. |  |
|  |  |  | 86 | Remove individual tests from 3–2 - moved to 3.8. |  |
|  |  |  | 139 | Remove individual tests from 5–2 - moved to 5.8. |  |
|  |  |  |  | Delete "Test Usage" paragraph throughout Section 6. Use tables. |  |
|  |  |  |  | ~275 pages, with shortening of 3.2, 4.2, 5.2, etc. |  |
|  |  |  |  | Replace "Legacy CBUS" with CBUS_1M throughout, using Bookmark cross-reference. |  |
|  | 2013-07-17 | WA |  | Remove 6.4.10.3 from Table 3-4. Per LB, 7/16. |  |
|  |  |  |  | Remove 6.5.3.1 from Table 3-6. Per LB, 7/16. |  |
|  |  |  | 187 | Tie 6.4 to tables in 3.5, 4.5, 5.5 in introductory paragraph. |  |
|  |  |  | 187 | Clarify and enhance Default Test Setup in 6.4.1. |  |
|  |  |  |  | Re-word heading 6.4.4. |  |
|  |  |  |  | Insert objective statements in many tests in Section 6.4, 6.5. |  |
|  |  |  | 192 | Add Req'd Method to 6.4.7.6. |  |
|  |  |  |  | Clarify "all available" Dev Cap reg list in 6.4.9.1. |  |
|  | Above changes from file " CTS-3_0_Main-rc03-2013-07-09-2300-w_bgz_LB6.docx", received from SIMG/LB, 2013-07-16. |  |  |  |  |

| Rev | Date | Author | Page(s) |  | Change |
| --- | --- | --- | --- | --- | --- |
| 3.0 0.80 rc04 | 2013-07-17 | WA | 187 |  | Eliminate duplicated "common routines" in 6.4. |
|  |  |  | 187 |  | Describe common routines in "subroutine" terms, in 6.4. |
|  |  |  | 188 |  | Re-write 6.4.2.1 common routine for Tester Source to Sink DUT. |
|  |  |  | 189 |  | Re-write 6.4.2.2 common routine for Tester Sink to Source DUT. |
|  |  |  |  |  | Add high-level common routine for Connect Tester to DUT, to 6.4.2. |
|  |  |  |  |  | Replace simple "Tester and DUT establish a CBUS connection" step, with 3-step invokation of common routine and handling of RETURN_CODE – throughout 6.4 and 6.5. |
|  |  |  | 33 | Remove mention of video & audio capture times in 3.2 int–o - see CTS 2. |  |
|  |  |  |  |  | Substitute CBUS_1M for "CBUS" in table headings, etc., where needed. |
|  |  |  | throughout |  | Setup new paragraph style "Test Objective", with distinctive font. Use for objective statement at each test, removing former level-5 headings. |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 0.80 rc04 | 2013-07-17 | WA | 17 | Clarify use of CBUS for DDC in HDCP Test Equipment. Per SIMG. 7/18. |
|  |  |  | 23 | Expand audio tone req'mt to all supported Fs rates to 192. |
|  |  |  |  | Replace lookup table for legacy System tests in 3.2. |
|  |  |  |  | Replace lookup table for new System tests in 3.8. |
|  |  |  | 74 | Add 3 new tests to 3.8.4 Audio for 60958. |
|  |  |  |  | Replace 'PackedPixel' with '16-bit' in Section 3.8.4. |
|  |  |  | 79 | Add HDCP Test in 3.8.5. |
|  |  |  |  | Replace Sink AVLink test sequence table in 4.2. |
|  |  |  |  | Insert Sink AVLink Test Sequence of new tests in 4.8. |
|  |  |  | 127 | Insert 3 new tests into 4.8.2 Video Tests. |
|  |  |  | 128 | Insert 2 new audio tests in 4.8.3. |
|  |  |  |  | Replace dongle AVLink test sequence table in 5.2. |
|  |  |  |  | Insert Dongle AVLink test sequence table for new tests in 5.8. |
|  |  |  | 175 | Add 3 new video tests to 5.8.2. |
|  |  |  | 176 | Add 2 new audio tests in 5.8.3. |
|  | Above changes from file " CTS-3_0_Main-rc04-2013-07-16-1200-w_bgz_DN_VF_AVLINK.docx", received 2013-07-18 from Silicon Image. |  |  |  |
|  |  |  | 233 | Delete Section 8 Informati–e - re: sequencing test suites. |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-07-18. |  |  |  |  |
| The above version has been delivered to Contributors as watermarked PDF file on 2013-07-18. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 rc05 | 2013-08-20 | WA |  | Accept all changes, remove all comments from previous draft. |
|  |  |  |  | Add column for test titles, fix xref links, in Table 3-1 Electrical. |
|  |  |  |  | Change style through 3.7 from "Heading 4" to "Test Heading". |
|  |  |  | 78 | Add 3.8.4.11 ADT Support. |
|  |  |  |  | Add 4.8.3.7-4.8.3.8 ADT Support. |
|  |  |  |  | Add 4.8.8.2-4.8.8.3 ADT Support. |
|  |  |  | 137 | Delete previous ADT test group headings. |
|  | 2013-08-24 | WA | 71 | Replace Method in 3.8.2.1 AVLink TX BIST. |
|  |  |  | 72 | Add 3.8.3.1 Video Formats. |
|  |  |  | 72 | Add 3.8.3.2 Pixel Encoding. |
|  |  |  | 73 | Add 3.8.3.3 High-End Video Formats |
|  |  |  |  | Delete 3.8.7 USB/HSIC tests. |
|  |  |  | 81 | Add 3.8.8.1 AVLink Speed Test. |
|  |  |  | 81 | Add 3.8.8.2 Packetization Basic Protocol |
|  |  |  | 81 | Add 3.8.8.3 Packetization Basic Protocol Timing. |
|  |  |  | 82 | Add 3.8.8.4 Packetization Extended Content Protocol. |
|  |  |  | 82 | Add 3.8.8.5 Packetization Extended Content Protocol Timing. |
|  |  |  | 82 | Rewrite 3.8.9.1 Multi-View Test. |
|  |  |  | 84 | Add 3.8.10.1 EDID Read Test. |
|  |  |  | 127 | Rewrite 4.8.1.1 AVLink BIST Test. |
|  |  |  | 127 | Edit test step in 4.8.2.1. |
|  |  |  | 128 | Add 4.8.2.3 High-End Video Formats. |
|  |  |  | 132 | Delete 4.8.6 USB/HSIC tests. |
|  |  |  | 132 | Delete AVLink Packetization Tests. |
|  |  |  |  | Rewrite 4.8.7.1 Downstream Multi-View test. |
|  |  |  | 137 | Add 4.8.8.1 AV Sync test. |
|  |  |  | 137 | Add 4.8.9.1 EDID Read test. |
|  |  |  | 179 | Delete Dongle RBP, ATT, USB/HSIC tests. |
|  |  |  | 174 | Rewrite AVLink RX BIST test. |
|  |  |  | 176 | Rewrite 5.8.2.3 High-End Video Formats test. |
|  |  |  |  | Delete dongle AVLink Packetization tests. |
|  |  |  |  | Rewrite 5.8.5.1 Multi-View test. |
|  |  |  | 185 | Replace 5.8.6.1 ADT Support. Rewrite 5.8.7.1 AV Sync test. |
|  |  |  | 186 | Add 5.8.8.1 EDID Read Test. |
|  |  |  |  | Add heading but no content for 5.8.8.2, 5.8.8.3. |
|  | Above changes from file "CTS-3_0_Main-rc04-2013-07-16-1200-w_bgz_VF_AVLINK_08_23_2013_EDITS.docx", received 2013-08-23 from Silicon Image. |  |  |  |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-08-25. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |  |
| --- | --- | --- | --- | --- | --- |
| 3.0 1.0 rc06 | 2013-08-26 | WA |  | Edit 96 and 192kHz into 3.8.3.1 Audio Test.{BGZ#29931} |  |
|  |  |  | 233 | Edit CDF_ field for 88.1, 96, 176 and 192kHz Audio into appendix. {BGZ#29931} |  |
|  |  |  |  |  | Add 88.1 thru 192kHz to 5.8.3.4.{BGZ#29931} |
|  |  |  |  |  | Insert CDF_ hyperlinks for audio modes into test step. {BGZ#29931} |
|  |  |  |  | Add "Category List" table after 'Objective' in each test in 6.4 and 6.5. {BGZ#29981} |  |
|  |  |  | 190, 91, 143 |  | Remove 6.4.3.1 Source tries CBUS_1M if bCBUS Fails. {BGZ#29985} |
|  |  |  | 190, 143, 91, 39 |  | Delete 6.4.3.2 Sink tries CBUS_1M if bCBUS Fails. {BGZ#29986} |
|  |  |  | 190 |  | Delete 6.4.3.3 Sink replies to READ_XDEVCAP with MHL2 Cable. {BGZ#29987} (Lookup tables not updated – these will be cleaned or created automatically later…) |
|  |  |  | throughout |  | Replace CBUS_1M' with 'OCBUS', later to be replaced with simply 'CBUS'. {BGZ#29988} |
|  |  |  | 49 occurrences |  | Replaced 2 steps of IF…THEN…ELSE with 1 step. {BGZ#29990} |
|  |  |  |  | Add IF … THEN … ELSE statement in 6.4.4.1-6.4.16.1. {BGZ#29993} |  |
|  |  |  | 190 | Remove mention of MHL 3 Cable in 6.4.3.4. {BGZ#29997} |  |
|  |  |  | 190 | Replace test method, fix title, obj, in 6.4.3.6. {BGZ#29998} |  |
|  |  |  | 191 | Delete 6.4.4.1 as obsolete. {BGZ#29999} |  |
|  |  |  | 191 | Delete 6.4.4.2 as obsolete. {BGZ#30001} |  |
|  |  |  | 191 | Delete 6.4.4.3 as obsolete. {BGZ#30002} |  |
|  |  |  | 191 | Delete 6.4.4.4 as obsolete. {BGZ#30003} |  |
|  |  |  | 191 | Delete Test Group 6.4.4, moves later groups up. {BGZ#30004} |  |
|  |  |  |  |  | Clarify eCBUS-D vs. eCBUS-S in steps, in 6.4.5.1, 6.4.5.2, 6.4.6.2. {BGZ#30007} |
|  |  |  | 30 total occurrences |  | Reverse sense of eCBUS-S and eCBUS-D in cable matching tester indication. {BGZ#3008} |
|  |  |  | 203, 204 |  | Replace 3 steps in 6.4.13.1, 6.4.13.2, 6.4.13.3. {BGZ#30011} |
|  |  |  | 204 | Delete step in 6.4.13.3. {BGZ#30011} |  |
|  |  |  | 203 | Remove mention of bCBUS in 6.4.13.2. {BGZ#30011} |  |
|  |  |  |  | Add detail for next connection step in 6.4.13.2. {BGZ#30015} |  |
|  |  |  | 207 | Add detail for next connection step in 6.4.16.1. {BGZ#30015} |  |
|  |  |  |  | Add detail for next connection step in 6.4.17.4. {BGZ#30015} |  |

| Rev | Date | Author | Page(s) | Change |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0 1.0 rc06 | 2013-08-27 | WA | 205 | Delete 6.4.14.2 on Speculative Commands. Obsolete. {BGZ#30012} |  |  |
|  |  |  | 207 | Delete 6.4.15.1 Speculative... Obsolete. {BGZ#30013} |  |  |
|  |  |  | 191, 192, 207, 193, 196, 193 |  | Rewrite test step in 6.4.4.5, 6.4.4.6, 6.4.16.1, 6.4.5.2, 6.4.7.1, 6.4.5.1. {BGZ#30014} |  |
|  |  |  | 209 |  | Insert missing step in 6.4.17.2. {BGZ#30016} |  |
|  |  |  | 210 | Rewrite and retitle 6.4.17.4 eCBUS TDM Slot Alloc... {BGZ#30017} |  |  |
|  |  |  | ~38 edits | Fix Test_makes_eCBUS_Connection in many tests. {BGZ#30019} |  |  |
|  |  |  | 203 | Replace test method in 6.4.13.2. {BGZ#30020} |  |  |
|  |  |  | 201 | Remove MHL 3 Cable from 6.4.11.5. {BGZ#30021} |  |  |
|  |  |  | 221 | Replace 3 steps in 6.5.4.1. {BGZ#30022} |  |  |
|  |  |  | 222 | Delete 6.5.5.1, 6.5.5.2, 6.5.5.3 Obsolete. {BGZ#30023} |  |  |
|  |  |  | 222 | Delete 6.5.5.5, 6.5.5.6 Obsolete. {BGZ#30024} |  |  |
|  |  |  | 34 | Delete Table 3-3 not needed. {BGZ#29979} |  |  |
|  | 2013-09-06 | WA |  | Delete 3 columns from Table 3-3 "eCBUS MHL 1/ MHL 2 Suite CBUS Tests" (was 3-4). Per SIMG/LB, 8/27. {BGZ#29979} |  |  |
|  |  |  | 210 | Remove space character (2 places) in symbol in 6.4.16.4.{BGZ#29983} |  |  |
|  |  |  | 25 | Edit History sentence to remove bCBUS. |  |  |
|  |  |  | 36, 37, 39, 40, 87, 88, 89, 91, 92, 139, 140, 142, 143, 145 |  |  | Delete bCBUS column in Tables. |
|  |  |  | all | Change bCBUS to use DOCPROPERTY 'Legacy' set to oCBUS. |  |  |
|  |  |  | 187 | Add explanation at beginning of 6.4. |  |  |
|  |  |  | 188 | Add new 6.4.2.1 common routine. |  |  |
|  |  |  |  | Delete old 6.4.2.2 common routine, leave stub. |  |  |
|  |  |  | 188 | Add timeout step to 6.4.2.3 common routine. |  |  |
|  |  |  | 188 | Add steps to 6.4.2.3 common routine. |  |  |
|  |  |  | 188 | Delete steps, add timing parm to 6.4.2.3. |  |  |
|  |  |  | 189 | Change steps, add timeout parm in 6.4.2.4. |  |  |
|  |  |  | 189 | Add common routine 6.4.2.5 Connect Tester to DUT. |  |  |
|  |  |  | 190, 190 | Enter parms for CBUS mode in 6.4.3.1, 6.4.3.2. |  |  |

| Rev | Date | Author | Page(s) | Change |  |
| --- | --- | --- | --- | --- | --- |
| 3.0 1.0 rc06 | 2013-09-06 | WA | 190 | Change title of 6.4.3.3. |  |
|  |  |  | 190, 191, 191, 192 |  | Add parms to CBUS test in 6.4.3.3, 6.4.3.4, 6.4.3.5, 6.4.3.6, 6.4.3.7, etc. |
|  |  |  | 190 |  | Update steps in 6.4.3.3. |
|  |  |  | 191, 191, 191, 192 |  | Clarify test step in 6.4.3.4, 6.4.3.5, 6.4.3.6, 6.4.3.7. |
|  |  |  | 193, 194, 195 |  | Rewrite 6.4.4.1, 6.4.4.2, 6.4.5.2, 6.4.5.3. |
|  |  |  |  | Modify 6.4.5.1 and add test parm for CBUS test. |  |
|  |  |  |  | Continue in like fashion through section 6.4… |  |
|  |  |  | 206 | Add new test: 6.4.13.3. |  |
|  |  |  |  | Continue through section 6.4, making similar changes. |  |
|  | Above changes from file "CTS-3_0_Main-rc06-2013-08-27-1000_LB3 .docx", received from SIMG/LB, 8/30. |  |  |  |  |
|  |  |  | 17, 17 | Replace Figures 2-10, 2-11. |  |
|  |  |  | 33 | Edit eCBUS-S column, add tests, delete 1 test, in Table 3-2. |  |
|  |  |  | 71 | Replace Table 3-8 in section 3.8.2 (add caption). |  |
|  |  |  | 70 | Remove heading 3.8.1 General Test Setups (affects later sections). |  |
|  |  |  | 72 | Delete 2 steps in 3.8.1.1 (new numbering). |  |
|  |  |  | 76 | Delete 'preface table' from 3.8.3.5, and others. |  |
|  |  |  | 76, 76, 77, 77, 78, 78 |  | Edit step in 3.8.3.5, and others. |
|  |  |  | 86 | Insert caption for table 4-1 in 4.2. |  |
|  |  |  | 86 | Insert, remove tests from Table 4-1. |  |
|  |  |  | 126 | Insert caption for Table 4-7 System Tests in 4.8. |  |
|  |  |  | 126 | Replace table with new Table 4-7. |  |
|  |  |  | 129, 129, 130 | Change PackedPixel to 16-bits-per-pixel in 4.8.3.3, 4.8.3.4, 4.8.3.6. |  |
|  |  |  | 130 | Fix CDF field name for HBR test in 4.8.3.5. |  |
|  |  |  |  | Change register field name in 4.8.3.7. |  |
|  |  |  | 130 | Delete test 4.8.3.8. |  |
|  |  |  | 137 | Delete tests 4.8.7.2, 4.8.7.3. |  |
|  | Above changes from file "CTS-3_0_Main-rc06-2013-08-27-1000_AVLINK_VF_DN_EDITS_08_30_2013.docx", from SIMG/VF/DNg, 2013-08-30. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 rc06 | 2013-09-06 | WA | 138 | Add caption for Table 5-1 Dongle AVLink Test Sequence. |
|  |  |  | 138 | Insert tests, delete tests, in Table 5-1. |
|  |  |  | 173 | Insert caption for Table 5-7 Dongle AVLink System Tests. |
|  |  |  | 173 | Replace Table 5-7 test list. |
|  |  |  |  | Fix step in 5.8.3.3, 5.8.3.4. |
|  |  |  | 178 | Fix CDF field name in 5.8.3.5. |
|  |  |  | 178 | Replace PackedPixel in 5.8.3.6. |
|  |  |  | 178 | Insert new test 5.8.3.7 ADT Support. |
|  |  |  | 185 | Delete 5.8.6.1 ADT Test. |
|  |  |  | 187 | Delete 5.8.8.2, 5.8.8.3 |
|  | Above changes from file "CTS-3_0_Main-rc06-2013-08-27-1000_AVLINK_VF_DN_EDITS_08_30_2013.docx", from SIMG/VF/DNg, 2013-08-30. |  |  |  |
| 3.0 1.0 fc01 | 2013-09-15 | WA |  | Begin from the file "CTS-3_0_Main-rc06-2013-09-06-1100_Fork1.docx", which had no marked change. Received 2013-09-12, 17:14 PDT. |
|  |  |  | 14 | Insert paragraph in 2,3.2. Per SIMG/VF, 9/12. |
|  |  |  | 14 | Delete description of test table, in 2.3.2. Per SIMG/VF, 9/12. |
|  |  |  | 14, 14, 15 | Change equipment name to "MHL3" in 2.3.2.1, 2.3.2.2, 2.3.3.1. |
|  |  |  | 15, 15 | Change drawings in Fig 2-2, 2-3. |
|  |  |  | 15 | Delete section on MHL Test Cable (20cm). |
|  |  |  | 17 | Delete sections on MHL D.A. extension cable. |
|  |  |  | 33 | Refine introduction text in 3.2.1. |
|  |  |  | 33 | Remove unneeded columns, rows in Table 3-2. |
|  |  |  | 39 | Table 3-6 to be filled in with macro later. |
|  |  |  | 40 | Table 3-7 to be filled in with macro later. |
|  |  |  | 70 | Rewrite intro to Section 3.8. |
|  |  |  | 71 | Delete tests prior to 3.8.1.1 from Table 3-8. |
|  |  |  | 72 | Update step #5 in 3.8.2.1. |
|  |  |  | 73 | Rename 3.8.2.3. |
|  |  |  | 73 | Clean up test steps in 3.8.2.3. |
|  |  |  | 82 | Retitle 3.8.8, add intro. |
|  |  |  | 83 | Retitle and rewrite 3.8.8.1. |
|  |  |  | 83 | Add test 3.8.8.2. |
|  |  |  | 84 | Add intro to 3.8.9. |
|  |  |  | 86 | Rewrite intro to 4.2. |
|  |  |  | 86 | Shorten and simplify Table 4-1. |
|  |  |  | 87 | Remove 2 columns from Table 4-2. |
|  | Above changes from file "", received from SIMG 2013-09-13, 21:55. |  |  |  |

| Rev | Date | Author | Page(s) | Change |  |
| --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc01 | 2013-09-15 | WA | 91, 92 | Tables 4-5, 4-6 to be filled in with macro. |  |
|  |  |  | 126 | Delete rows prior to 4.8.1.1 from Table 4-7. |  |
|  |  |  | 128 | Refine steps in test 4.8.2.3. |  |
|  |  |  | 132 | Retitle 4.8.8. |  |
|  |  |  | 135 | Refine steps in test 4.8.8.1. |  |
|  |  |  | 136 | Add test 4.8.8.2. |  |
|  |  |  | 138 | Rewrite intro to 5.2. |  |
|  |  |  | 138 | Delete 2 columns from Table 5-1. |  |
|  |  |  |  | Delete rows after 5.2.9.2 in Table 5-1. |  |
|  |  |  | 143, 145 | Rows in Tables 5-5, 5-6 to be filled in with macro. |  |
|  |  |  | 173 | Delete rows prior to 5.8.1.1 from Table 5-7. |  |
|  |  |  | 176 | Refine steps in test 5.8.2.3. |  |
|  |  |  |  | Add Req'd Method to test 5.8.3.7. |  |
|  |  |  | 184 | Rewrite test 5.8.8.1, add test 5.8.8.2. |  |
|  |  |  | 196 | Retitle 6.4.7. |  |
|  |  |  | 196, 197 | Reword title and objective in tests 6.4.7.1, 6.4.7.2. |  |
|  |  |  | 197 | Reword title in 6.4.8. |  |
|  |  |  | 197, 198 | Reword title and objective in tests 6.4.8.1, 6.4.8.2. |  |
|  |  |  | 199, 200, 200, 201, 201, 202, 202 |  | Remove extraneous string in steps in 6.4.10.2, 6.4.10.3, 6.4.10.4, 6.4.10.5, 6.4.11.1, 6.4.11.2, 6.4.11.3. |
|  |  |  | 207 | Remove extra word in title of test 6.4.14.1. |  |
|  |  |  | 217 | Retitle 6.5.2. |  |
|  |  |  | 218, 219 | Retitle test 6.5.2.2, 6.5.2.3. |  |
|  |  |  | 222 | Replace word in objective of test 6.5.5.2. |  |
|  | Above changes from file "CTS-3_0_Main-rc06-2013-09-06-1100_Fork2.docx", received from SIMG 2013-09-13, 21:55. |  |  |  |  |
|  |  |  | thruout | Remove all {REF …} and {HYPERLINK …} fields from CDF_ references, leaving only the CDF_ field name as text. |  |

| Rev | Date | Author | Page(s) | Change |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc01 | 2013-09-17 | WA | 72 | Add Test Objective to 3.8.2.2. |  |  |  |
|  |  |  | 80, 80, 180, 180 |  | Fix typo in CDF field name in 3.8.5.1, 3.8.5.2, 5.8.5.1, 5.8.5.2. |  |  |
|  |  |  | 83 | Fix title of 3.8.8.2. |  |  |  |
|  |  |  | 128 | Reword step in 4.8.2.2. |  |  |  |
|  |  |  |  | Use correct CDF fields in step in 4.8.3.2. |  |  |  |
|  |  |  | 135, 136 | Retitle 4.8.8.1, 4.8.8.2. |  |  |  |
|  |  |  |  | Add last step in general method in 5.2. |  |  |  |
|  |  |  | 184 | Retitle 5.8.8. |  |  |  |
|  |  |  | 184, 185 | Retitle 5.8.8.1, 5.8.8.2. |  |  |  |
|  | Above changes from file "CTS-3_0_Main-fc01-2013-09-15-1000_VF_EDIT.docx", received from SIMG/VF, 2013-09-16, 17:27. |  |  |  |  |  |  |
|  |  |  | 82 | Edit intro in 3.8.8. |  |  |  |
|  |  |  | 83 | Fix timing parameter name. |  |  |  |
|  |  |  | 83 | Insert missing step in 3.8.8.1. |  |  |  |
|  |  |  | 83 | Change step # reference in 3.8.8.2. |  |  |  |
|  |  |  | 127 | Replace test 4.8.1.1. |  |  |  |
|  |  |  | 128, 135 | Replace steps in 4.8.2.3, 4.8.8.1. |  |  |  |
|  |  |  | 136 | Fix timing parameter format (2 places) in 4.8.8.2. |  |  |  |
|  |  |  | 136 | Indent 2 steps to be 10a and 10b, in 4.8.8.2. |  |  |  |
|  |  |  | 174 | Replace steps in 5.8.1.1, 5.8.2.3. |  |  |  |
|  |  |  | 184 | Fix timing parameter format (2 places) in 5.8.8.1. |  |  |  |
|  |  |  | thruout | Replace "NO" with NO, and "YES" with YES. |  |  |  |
|  | Above changes from file "CTS-3_0_Main-fc01-2013-09-15-1000_DN9–6 - B.docx", received from SIMG/VF, 2013-09-16, 20:11. |  |  |  |  |  |  |
|  |  |  |  |  |  | Fix CDF field name in 4.8.2.1, 4.8.2.2, 5.8.2.1, 5.8.2.2. |  |
|  |  |  | 131 | Delete leading space in title of 4.8.4. |  |  |  |
|  |  |  | 193, 194, 201, 202, 202, 203, 203, 204, 204, 205, 206, 207, 207, 209, 221, 222, 222, 223 |  |  |  | Reword step in 6.4.4.1, 6.4.4.2, 6.4.5.2, 6.4.11.1, 6.4.11.2, 6.4.11.3, 6.4.12.1, 6.4.12.2, 6.4.12.3, 6.4.13.1, 6.4.13.2, 6.4.13.3, 6.4.14.1, 6.4.15.1, 6.4.16.4, 6.5.4.1, 6.5.5.1, 6.5.5.2, 6.5.5.3 to show CDF field check. |
|  |  |  | 198, 198, 201, 202, 204, 204, 206, 208, 210 |  |  |  | Rewrite intro sentences in 6.4.9, 6.4.10, 6.4.11, 6.4.12, 6.4.13, 6.4.14, 6.4.15, 6.4.16, 6.4.17, 6.4.19 to refer to CDF field names. |
|  |  |  | 198 | Edit step to show CDF field value test in 6.4.9.1. |  |  |  |
|  |  |  | 211 | Add intro to 6.4.18. |  |  |  |
|  |  |  | 207 | Delete eCBUS-D iterations from 6.4.15.1. |  |  |  |
|  |  |  | 222, 222, 222, 222, 222 |  |  |  | Rewrite intro sentences in 6.5.1, 6.5.2, 6.5.3, 6.5.4, 6.5.5 to refer to CDF field names. |
|  | Above changes from file "CTS-3_0_Main-fc01-2013-09-15-1000_LB.docx", received from SIMG/VF, 2013-09-16, 18:55. |  |  |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc01 | 2013-09-17 | WA | 33 | Insert 10 figures for Electrical Test setups. |
|  |  |  | 33 | Move figures and completely replace tests 3.1.1.1-3.1.1.25. |
|  |  |  | 224 | Replace all Cable Electrical Tests. |
|  | Above changes from file "CTS-3_0_Main-rc06-2013-08-27-0800 – Bsung.docx", received SIMg/BSu, 2013-09-16 12:55. |  |  |  |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-09-17 16:00 PDT. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |  |
| --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-09-18 | WA |  | Change style for test group paragraphs to "Test Group Heading". |  |
|  |  |  |  | Finish macro to output .csv file of CDF usage per test, with info. |  |
|  |  |  |  | Insert macro listing in back-matter (not part of CTS itself). |  |
|  |  |  |  | Reformat test steps to use RequiredMethod style consistently. |  |
|  | 2013-09-19 | WA | 77 | Fix CDF_ field name to replace dash with underscore. |  |
|  |  |  | 38, 40, 91, 92, 143, 144 |  | Populated tables in 3.5, 3.6, 4.5, 4.6, 5.5, 5.6 with CBUS test numbers. |
|  | 2013-09-20 | WA | 187-216 | Reword intro in 6.4-6.5 to use CDF field names instead of English phrases. |  |
|  |  |  | 195, 195, 196, 197, 197, 198 |  | Rewrite step in test to use CDF field, not English phrase. |
|  |  |  | 198, 209 | Change CDF field name in test step. |  |
|  |  |  | 212, 213, 214, 214 |  | Revise test steps in 6.4.18.1, 6.4.18.2, 6.4.18.3, 6.4.18.4. |
|  |  |  | 212 | Fix broken cross-reference. |  |
|  | Above changes from file " CTS-3_0_Main-fc01-2013-09-17-1500_LB1.docx", received SIMG/LB, 2013-09-20. |  |  |  |  |
|  |  |  | 34 | Replace cross-ref to 6.3 in this CTS with simple '6.3' to CTS 2. |  |
|  |  |  | 34 | Fix broken cross-references. |  |

| Rev | Date | Author | Page(s) | Change |  |
| --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-09-21 | WA | 33 | Rewrite General Test Setups, 3.2.1. |  |
|  |  |  | 70, 125, 173 |  | Clarify labeling of TRF results, in 3.8, 4.8, 5.8. |
|  |  |  | 80, 131, 180 |  | Add step to 3.8.5.1, 4.8.5.1, 5.8.5.1.{BGZ#30569} |
|  |  |  | 80, 132, 132, 180, 180 |  | Fix steps #1,5 in 3.8.5.2, 4.8.5.2, 5.8.5.2.{BGZ#30567} |
|  |  |  | 81 | Clarify step in 3.8.7.1. |  |
|  |  |  | 86, 138 | Rewrite intro to 4.2, 5.2. |  |
|  |  |  | 127, 127, 175, 176 |  | List explicit CDF fields in step #1 of 4.8.2.1, 4.8.2.2, 5.8.2.1, 5.8.2.2. |
|  |  |  | 186 | Clarify steps in 5.8.10.1.{BGZ#30369} |  |
|  | Above changes from file "CTS-3_0_Main-fc01-2013-09-17-1500_VF_edits.docx", received from Silicon Image / VF, 2013-09-20. |  |  |  |  |
|  |  |  | 73 | Add {max} to timing parm in 3.8.2.3. |  |
|  |  |  | 78, 78 | Clarify title of 3.8.3.11, rewrite method. |  |
|  |  |  | 79 | Add test 3.8.3.12. "Audio Data Tunneling Transmitting Support ". |  |
|  |  |  | 130 | Retitle and rewrite method in 4.8.3.7. |  |
|  |  |  | 130 | Add test 4.8.3.8 "Audio Data Tunneling Transmitting Support". |  |
|  |  |  |  | Clarify steps in 4.8.8.2. |  |
|  |  |  | 178, 192 | Fix typo. |  |
|  |  |  | 178 | Rewrite 5.8.3.7. |  |
|  |  |  | 179 | Add new test 5.8.3.8. |  |
|  |  |  | 185 | Rewrite steps in 5.8.8.2. |  |
|  | Above changes from file "CTS-3_0_Main-fc01-2013-09-17-1500_DN.docx", received from Silicon Image / DN, 2013-09-20. |  |  |  |  |
|  | 2013-09-23 | WA | 132 | Add Sink AVLink packetization tests in 4.8.7. |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-09-27 | WA | 81 | Modify 2 steps in 3.8.7.1. |
|  |  |  | 81, 82 | Specify exact video mode to use in 3.8.7.2, 3.8.7.3. |
|  |  |  | thruout | Fix typo: 'connect' to 'connects'. |
|  |  |  | 82, 82 | Change handling of video mode selection in 3.8.7.4, 3.8.7.5. |
|  |  |  |  | Fix typo: 'comand' to 'command' (3 places). |
|  |  |  | 131 | Fix CDF_ field name used in 4.8.5.1. |
|  |  |  |  | Rewrite 4.8.7.1 again. |
|  |  |  |  | Rewrite group 4.8.7 AVLINK tests. |
|  |  |  |  | Refer group 5.8.7. to 4.8.7. |
|  |  |  | 188 | Clarify step in 6.4.2.3. |
|  |  |  | 190 | Add CDF_ fields to 6.4.3.1. |
|  |  |  | 193, 193, 194 | Add, delete steps in 6.4.4.1, 6.4.4.2, 6.4.5.2. |
|  |  |  | 195, 195, 196 | Rearrange steps in 6.4.5.3, 6.4.5.4, 6.4.5.5, 6.4.6.2. |
|  |  |  | 197 | Remove word in 6.4.8 title. |
|  |  |  | 207 | Reword title in 6.4.15. |
|  |  |  | 207 | Delete most of intro paragraph in 6.4.15. |
|  |  |  | 207 | Retitle and reword intro in 6.4.15.1. |
|  |  |  | 207 | Change CDF field in step in 6.4.15.1. |
|  |  |  | 207 | Clarify steps in 6.4.15.1 |
|  |  |  | 211 | Add DEV_CATs to title of 6.4.18. |
|  |  |  | 215, 215 | Add 'HSIC' to title of 6.4.19, 6.4.19.1. |
|  |  |  | 215 | Add group 6.4.20 for USB1 Tunneling (placeholder only). |
|  |  |  | 215 | Add group 6.4.21 BIST, 6.4.21.1, 6.4.21.2. |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-09-21-1200_LB4.docx", received from Silicon Image / LB, 2013-09-26. |  |  |  |
|  |  |  | 74 | Retitle 3.8.3.1 to include Basic Audio. |
|  |  |  | 74 | Replace step #1 in 3.8.3.1. |
|  |  |  | 74 | Modify loop step in 3.8.3.1. |
|  |  |  | 74 | Add 3 "Basic Audio" Fs rows to table in 3.8.3.1. |
|  |  |  | 74 | Modify objective and steps in 3.8.3.2 to cover all Fs rates. |
|  |  |  | 75 | Modify objective and steps in 3.8.3.3 to cover all Fs rates. |
|  |  |  | 79 | Check Audio InfoFrame, in 2 steps in 3.8.3.12. |
|  |  |  | 83 | Add table to 3.8.8. intro. |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-09-21-1200_DN.docx", received from Silicon Image / DN, 2013-09-25. |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-09-27 | WA | 83, 83 | Rewrite 3.8.8.1, 3.8.8.2. |
|  |  |  | 128, 129 | Modify title, objective, steps for Fs rates in 4.8.3.1, 4.8.3.2. |
|  |  |  | 131 | Fix typo in 4.8.3.8. |
|  |  |  | 131 | Check Audio InfoFrame, in 2 steps in 4.8.3.8. |
|  |  |  | 135 | Add table to 4.8.8. intro. |
|  |  |  | 135, 136 | Rewrite 4.8.8.1, 4.8.8.2. |
|  |  |  | 176, 177 | Modify 5.8.3.1, 5.8.3.2 for Fs rates. |
|  |  |  | 179 | Fix 2 steps in 5.8.3.8 to use Audio InfoFrame. |
|  |  |  | 184 | Add table to 5.8.8. intro. |
|  |  |  | 184, 185 | Rewrite 5.8.8.1, 5.8.8.2. |
|  |  |  |  | Fix typo: change 'Sink' to 'Dongle' (2 places) in 5.8.9.1. |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-09-21-1200_DN.docx", received from Silicon Image / DN, 2013-09-25. |  |  |  |
|  |  |  | 71 | Add 3.8.8.2 to suite in Table 3-8. |
|  |  |  | 80 | Rewrite intro, test 3.8.6.1 for ATT subcommand. |
|  |  |  | 81 | Rewrite 3.8.7.1. |
|  |  |  | 126 | Add tests to Table 4-7: 4.8.7.4-4.8.7.8. |
|  |  |  | 126 | Add tests to Table 4-7: 4.8.8.2. |
|  |  |  | 132 | Rewrite 4.8.6.1 for ATT. |
|  |  |  | 132 | Rewrite 4.8.7 group 4.8.7.1-4.8.7.8. |
|  |  |  | 174 | Add tests to Table 5-7: 5.8.7.4-5.8.7.8, 5.8.8.2. |
|  |  |  | 174 | Fix typo: change 'Sink' to 'Dongle' (2 places) in 5.8.1.1. |
|  |  |  |  | Fix typo: change 'Sink' to 'Dongle' (3 places) in 5.8.2.3. |
|  |  |  | 178, 178 | Fix typo: change 'Sink' to 'Dongle' (1 place) in 5.8.3.5, 5.8.3.6. |
|  |  |  | 178 | Fix typo: change 'Sink' to 'Dongle' (3 places) in 5.8.3.7. |
|  |  |  | 179 | Fix typo: change 'Sink' to 'Dongle' in 5.8.3.8. |
|  |  |  | 180 | Rewrite 5.8.6.1 ATT subcommand test. |
|  |  |  | 181 | Rewrite group 5.8.7: 5.8.7.1-5.8.7.8 |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-09-21-1200_VF_EDIT_9_26.docx", received from Silicon Image / VF, 2013-09-27. |  |  |  |

| Rev | Date | Author | Page(s) | Change |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-09-27 | WA | thruout | Replace eCBUSD with eCBUS-D. |  |  |
|  |  |  |  | Replace CBUS! With oCBUS (2 places). |  |  |
|  |  |  |  | Clarify that generator is MHL 1 / 2, in 2.3.1.6. |  |  |
|  |  |  | 14 | Clarify equipment is AVLink Analyzer in 2.3.2.1. |  |  |
|  |  |  | 14 | Clarify equipment is AVLink Generator in 2.3.2.2. |  |  |
|  |  |  | 15 | Clarify CBUS capabilitites in 2.3.3.1. |  |  |
|  |  |  | 16 | Clarify that CBUS Source Board is MHL3, in 2.3.3.3. |  |  |
|  |  |  | 16 | Clarify that CBUS Sink Board is MHL3, in 2.3.3.4. |  |  |
|  |  |  | 17 | Remove CBUS RxSense Board. |  |  |
|  |  |  | 17 | Remove MSC Test Tool. |  |  |
|  |  |  | 21 | Add TPA board in 2.3.5.3. |  |  |
|  |  |  | 24 | Remove Capacitance Measurement sections (2.5.2). |  |  |
|  |  |  | 26 | In 2.8 intro, replace "good" and "bad" machine with "normal" and "robustness" (2 places). |  |  |
|  |  |  | 33, 34 | Clarify caption of Tables 3-2, 3-3. |  |  |
|  |  |  | 36, 37, 39, 40 |  |  | Change X to "If DUT Supports" for eCBUS-D col in Tables 3-4, 3-5, 3-6, 3-7. |
|  |  |  | 86, 87, 33, 138, 139 |  |  | Clarify col heading in Tables 4-1, 4-2, 3-2, 4-7, 5-1, 5-2, 5-7. |
|  |  |  | 88, 89, 91, 92 |  |  | Change X to "If DUT Supports" for eCBUS-D col in Tables 4-3, 4-4, 4-5, 4-6. |
|  |  |  | 140, 143, 143, 145 | Change X to "If DUT Supports" for eCBUS-D col in Tables 5-3, 5-4, 5-5, 5-6. |  |  |
|  |  |  |  | Replace 'CBUS' with 'oCBUS' in all "Category List" headings. |  |  |
|  |  |  | 203 | Fix typo in 6.4.12.2. |  |  |
|  |  |  | 211, 211 | Clarify objective in 6.4.17.1, 6.4.17.2. |  |  |
|  | Above changes from markups on printed file, received from Silicon Image /ES, 2013-09-26. |  |  |  |  |  |
|  | 2013-09-30 | WA |  | Remove 6.3.16.1 and 6.3.16.2 from Tables 3-5 (not a Source test), 3.4, 4.3, 5.3 (Robustness tests). Per SIMG/LB, 9/27.{BGZ#30782} |  |  |
|  | 2013-10-02 | WA |  | Remove use of 'HBRA' acronym. Change CDF field to "…HBR". |  |  |
|  |  |  | 81, 133, 181 |  | Explicitly list CDF_LINK_SPEED fields in 3.8.7.1, 4.8.7.1, 5.8.7.1. |  |
|  |  |  | 193 | Remove 2 extra steps from 6.4.4.1. {BGZ#30810} |  |  |
|  |  |  | 193 | Fix typo in 6.4.4.1.{BGZ#30810} |  |  |
|  |  |  | 193 | Remove 'possibly' from objective in 6.4.4.1. |  |  |
|  |  |  | 203 | Fix typo in objective of 6.4.12.1. |  |  |
|  |  |  | 219 | Remove "Bad Machine" from title of group 6.5.3. |  |  |
|  | Above changes for SIMG/ES, 10/02.{BGZ#30815} |  |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-10-03 | WA | 14 | Clarify description of AVLink Prot Analyz. |  |  |
|  |  |  | 15 | Clarify description of AVLink Sig Gen. |  |  |
|  |  |  | 80 | Rewrite method in 3.8.6.1. |  |  |
|  |  |  | 84 | Add Source AV SYNC Test 3.8.9, 3.8.9.1. |  |  |
|  |  |  | 130 | Add missing space. |  |  |
|  |  |  | 132 | Rewrite 4.8.6.1. |  |  |
|  |  |  | 135, 184 | Retitle 4.8.8, 5.8.8. |  |  |
|  |  |  | 137 | Rewrite method in 4.8.9.1. |  |  |
|  |  |  | 180 | Rewrite method in 5.8.6.1. |  |  |
|  |  |  | 185 | Rewrite method in 5.8.9.1. |  |  |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-09-27-1300_VF_review_9_27.docx", received from SIMG/VF, 2013-10-02. |  |  |  |  |  |
|  |  |  | 73 | Modify method in 3.8.2.3. |  |  |
|  |  |  | 128 | Rewrite method in 4.8.2.3. |  |  |
|  |  |  | 176 | Rewrite method in 5.8.2.3. |  |  |
|  |  |  | 183 | Fix paragraph style. |  |  |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-09-27-1300_DN.docx", received from SIMG/DN, 2013-10-01. |  |  |  |  |  |
|  |  |  | 203, 203, 204, 205, 206, 207, 210, 222, 223, 223 |  |  | Fix typo. |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-10-02-1700_LB.docx", received from SIMG/LB, 2013-10-02. |  |  |  |  |  |
|  |  |  | 209, 208 | Add citation to CDF_CR_* field name. |  |  |
|  |  |  | 201 | Add citation to CDF_XCR_* field name. |  |  |
|  |  |  | 26 | Add section 2.9, with specific styles for headings, to list CDF fields not used in individual tests. |  |  |
|  |  |  |  | Replace conjunction of "do not" with explicit "do not" (4 places). |  |  |
|  |  |  |  | Replace conjunction of "is not" with explicit "is not" (2 places). |  |  |
|  |  |  | 193 | Remove 'possibly' from objective in 6.4.4.2.{BGZ#30815} |  |  |
|  |  |  | 212 | Change CDF field ref to use NO instead of 0. |  |  |
|  |  |  | 175, 127, 127, 176 |  | Change 'P' to 'p' in CDF field names to be consistent with CTS 2 syntax. |  |
|  |  |  | 209, 208 | Remove CDF_XCR_* reference – not needed for CDF. |  |  |
|  |  |  | 201 | Remove CDF_CR_* reference – not needed for CDF. |  |  |

| Rev | Date | Author | Page(s) |  | Change |
| --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-10-07 | WA | 11 |  | Add 16G sampling scope for MHL 3, in 2.3.1.2. |
|  |  |  | 11 |  | Add 13G sampling probe for MHL 3, in 2.3.1.4. |
|  |  |  | 11 |  | Add Jitter Analyzer for MHL 3, in 2.3.1.8. |
|  |  |  | 11 |  | Add 6Gbps Signal Gen for MHL 3, in 2.3.1.10. |
|  |  |  |  |  | Fix spelling typo (4 places). |
|  |  |  | 12 |  | Add Clock Jitter Gen for MHL 3, in 2.3.1.12. |
|  |  |  | 12 |  | Add Cable Emulator for MHL 3, in 2.3.1.14. |
|  |  |  | 12 |  | Add TTC for MHL 3, in 2.3.1.16. |
|  |  |  | 12 |  | Add Bias Tee for MHL 3, in 2.3.1.20. |
|  |  |  | 13 |  | Add RELT Board for MHL 3, in 2.3.1.23, with Fig. |
|  |  |  | 14 |  | Add Dummy Term Board for MHL 3, in 2.3.1.24. |
|  |  |  | 18-19 |  | Add TPA for MHL 3, in 2.3.5.2, with Fig and Table. |
|  |  |  | 33 |  | Delete intro and test list table in 3.1. |
|  |  |  | 33 |  | Remove 10 figures for Electrical Tests in 3.1.1. |
|  |  |  |  |  | Add VOFF Test in 3.1.1.1, with Fig. |
|  |  |  | 33 |  | Replace 3.1.1.1-end with 3.1.1.1-3.1.1.20. |
|  |  |  | 41 |  | Delete previous 3.7.1 Figs and stubs. |
|  |  |  | 41-45 |  | Insert new setup figs, in 3.7.1. |
|  |  |  | 45-70 |  | Insert new test methods in Test Group 3.7.2. |
|  |  |  | 93 |  | Delete test setups in 4.7.1. |
|  |  |  | 93-100 |  | Insert new test setup figures in 4.7.1. |
|  |  |  | 100 |  | Delete previous methods and stubs in 4.7.2. |
|  |  |  | 100-125 |  | Replace Sink electrical tests in 4.7.2. |
|  |  |  |  |  | Delete test setups in 5.7.1. |
|  |  |  | 145 |  | Delete previous methods and stubs in 5.7.2. |
|  |  |  | 149-173 |  | Insert new Dongle electrical tests in 5.7.2. |
|  |  |  | 224-233 |  | Replace all Cable electrical tests in 7.2. |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-09-21-1200_BSung_Sep30.docx", received from Silicon Image / BS, 2013-10-06. |  |  |  |  |
|  |  |  | 193 |  | Fix step #7 in test 6.4.4.2. {BGZ#30836} |
|  |  |  | 83 |  | Add FAIL to step #8 in 3.8.8.1. {BGZ#30847} |
|  |  |  | 74, 75 |  | Fix title in 3.8.3.2, 3.8.3.3. |
|  |  |  | 2 |  | Modify ReleaseDate to October 2013. |
|  |  |  | 18 |  | Redraw TPA in Fig 2-11 with tie points in Visio. |
|  |  |  |  | Replace a lot of broken figure cross-references temporarily with "in Figure TBD". |  |
|  |  |  | 8 |  | Delete 2 rows from Background description, since "References" and "Equipment" no longer in each test; and H5 and H6 are not used. |
|  |  |  | 8 |  | Remove def'n of TEST_ and MEAS_ fields -- not used. |

| Rev | Date | Author | Page(s) |  | Change |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-10-08 | WA | 72, 127, 174 |  |  | Fix typo, clarify setup in 3.8.1.1, 4.8.1.1, 5.8.1.1. {BGZ#30861} |  |  |
|  |  |  | 195 | Delete 6.4.5.4 which tests for "eCBUS-D Only", not allowed. {BGZ#30863} |  |  |  |  |
|  |  |  | 195 |  | Change step #1 in 6.4.5.3 to FAIL if no eCBUS-S. {BGZ#30864} |  |  |  |
|  |  |  | 41 |  | Remove unnneeded summary table from 3.7. |  |  |  |
|  |  |  |  |  | Fix figure cross-references throughout Sections 3, 4, 5. |  |  |  |
|  |  |  | 195 |  | Remove unneeded step #3 in 6.4.5.3.{BGZ#30864} |  |  |  |
|  |  |  | 72, 127, 174 |  |  | Delete phrase in step in 3.8.1.1, 4.8.1.1, 5.8.1.1.{BGZ#30861} |  |  |
|  | 2013-10-08 | WA | 33 |  | Replace fully-defined legacy tests in 3.1.1 with pointer to CTS 2.1. |  |  |  |
|  |  |  | 85 |  | Replace fully-defined legacy tests in 4.1.1 with pointer to CTS 2.1. |  |  |  |
|  |  |  | 138 |  | Replace fully-defined legacy tests in 4.1.1 with pointer to CTS 2.1. |  |  |  |
|  |  |  | 11 |  | Remove MHL 2 test equipment definitions from section 2.3.1. |  |  |  |
|  | Above changes from Silicon Image / BS, 2013-10-08. {BGZ#30643}{BGZ#28896} |  |  |  |  |  |  |  |
|  |  |  |  |  | Replace "the RF relay path" with "the RF relay path in the RELT board", to make it clear where the RELT board is used. {BGZ#30874} |  |  |  |
|  | 2013-10-09 | WA | 18 |  | Delete section on TPA boards for MHL 2. |  |  |  |
|  |  |  | 18 |  | Change caption for Fig 2-11. |  |  |  |
|  |  |  | 119, 121, 124, 226, 226, 226, 227, 227, 228, 228, 229, 231, 231, 231, 232, 232, 232 |  |  |  |  | Fix cross-reference. |
|  |  |  | 119 |  | Add 2 steps to 4.7.2.25. |  |  |  |
|  |  |  | 119 |  | Fix step number references. |  |  |  |
|  |  |  | 121, 121, 122, 122 |  |  |  | Change steps #8-9, 13-14, 22-23, 30-31 in 4.7.2.26. |  |
|  |  |  | 124, 124, 124, 125 |  |  |  | Change steps #8-9, 13-14, 22-23, 30-31 in 4.7.2.27. |  |
|  |  |  | 168 |  | Add steps #8-9 in 5.7.2.20. |  |  |  |
|  |  |  | 170, 170 |  | Change steps #10-11, 18-19 in 5.7.2.21. |  |  |  |
|  |  |  | 172, 172 |  | Change steps #10-11, 18-19 in 5.7.2.22. |  |  |  |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-10-08-1700 - BSung.docx", received from Silicon Image / BS, 2013-10-09.{BGZ#28896} |  |  |  |  |  |  |  |
|  |  |  | 188 |  | Delete unused 6.4.2.2.{BGZ#30872} |  |  |  |
|  |  |  |  |  | Replace (MAX) with {max} as parameter limit (12 places). |  |  |  |
|  |  |  |  |  | Fix typos listed in {BGZ#30876}. |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc02 | 2013-10-09 | WA | 133 | Replace caption on Table 4-9 (wrong style) and nearby xref. |
|  |  |  | 182 | Replace caption on Table 5-8 (wrong style) and nearby xref. |
|  |  |  |  | Replace "Normal Mode" with "Active Mode" in 3.5, 4.5, 5.5, 6.4. |
|  |  |  | 9 | Remove unneeded def'n of 'Normal Mode' in section 2. |
|  |  |  |  | Replace "Normal Mode" with "24-bits-per-pixel mode" in 3.8.2.1, 4.8.2.1 and 5.8.2.1. |
|  |  |  |  | Replace "16-bit Mode" with "16-bits-per-pixel mode". |
|  |  |  |  | Replace "24-bit Mode" with "24-bits-per-pixel mode". |
|  |  |  | 9 | Fix typo in definition of 'FAIL' condition in 2.2.1. |
|  |  |  | 234 | Add CDF_CR_SUPP_16BPP to Appendix. |
|  |  |  | 11 | Clarify bandwidth req'mt in 2.3.1.1. |
|  |  |  | 11 | Clarify bandwidth req'mt in 2.3.1.6. |
|  |  |  | 12 | Rewrite req'mts for Combiner for MHL 3 in 2.3.1.10. |
|  |  |  | thruout | Change 'KHz' to 'kHz'. |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-10-09 17:00 PDT. |  |  |  |  |

| Rev | Date | Author | Page(s) |  | Change |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.0 1.0 fc03 | 2013-10-10 | WA |  |  | Add paragraph in style "Hidden Test Details" to each test. |  |  |
|  |  |  |  |  | Increase indentation on Test Heading and Test Group Heading styles to 1.0", especially for later numbered Tests. |  |  |
|  |  |  |  |  | Remove bold when citing CDF_ fields by name. |  |  |
|  |  |  |  |  | Mark each Test Heading with paragraph 'Included' or 'Postponed' for December 2013 CTS. |  |  |
|  |  |  |  |  | Fix capitalization throughout Section 3,4,5 test titles. |  |  |
|  | 2013-10-11 | WA | 71, 126, 173 |  |  |  | Delete unneeded column in Tables 3-7, 4-7, 5-7. |
|  |  |  | 36, 37, 39, 40 |  |  |  | Delete unneeded column in Tables 3-3, 3-4, 3-5, 3-6. |
|  |  |  | 88, 89, 91, 92 |  |  |  | Delete unneeded column in Tables 4-3, 4-4, 4-5, 4-6. |
|  |  |  | 140, 142, 143, 145 |  |  |  | Delete unneeded column in Tables 5-3, 5-4, 5-5, 5-6. |
|  | 2013-10-15 | WA | 11 |  | Rewrite 2.3.1.6 signal generator req'mts. |  |  |
|  |  |  | 14 |  | Add 2.3.1.15 EMI Source Device. |  |  |
|  |  |  | 14 |  | Add 2.3.1.16 EMI Sink Device. |  |  |
|  |  |  |  |  | Change 'VSKEW' to 'TSKEW' throughout. |  |  |
|  | Above changes from file "CTS-3_0_Main-fc02-2013-10-09-1200_BSung.docx", received Silicon Image / BS, 2013-10-15. |  |  |  |  |  |  |
|  |  |  |  |  | Fix nested, ordered list numbering throughout Sections 3,4,5,7. |  |  |
|  |  |  | 138 |  | Delete 5.2.3.3, 5.2.3.4 from Table 5-1. |  |  |
|  |  |  | 86 |  | Delete 4.2.3.3, 4.2.3.4 from Table 4-1. |  |  |
|  |  |  | 138 |  | Change typo 'Source' to 'Dongle' in 5.1.1. |  |  |
|  |  |  |  |  | Add CDF_ field check as step #1 in 4.8.6.1. |  |  |
|  |  |  | 132 |  | Remove CDF_ field check from step #7 in 4.8.6.1. |  |  |
|  |  |  | 85 |  | Change typo 'Source' to 'Sink' in 5.1.1. |  |  |
|  |  |  | 40 |  | Remove unneeded test 6.4.5.5 from Table 3-5. |  |  |
|  |  |  | 128 |  | Change 'similar' to 'identical' in step #8 of 4.8.2.3. |  |  |
|  | Above changes from review by Sony, received 2013-10-15. |  |  |  |  |  |  |
|  |  |  | 80, 80, 131, 132, 180, 180 |  |  | Change CDF field related to RBP Support (6 places).{BGZ#30887} |  |
|  |  |  | 236 |  | Add CDF_RBP_RECV_SUPPORT and CDF_RBP_SEND_SUPPORT to Appendix. |  |  |
|  |  |  | 233-239 |  | Add missing CDF fields to Appendix Table 8-1. |  |  |
|  |  |  | 224 | Add 7.1.1.2 CBUS Shielding test - examine drawing. Per 5CTT, 10/15. |  |  |  |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-10-15 22:30 PDT. |  |  |  |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc04 | 2013-10-16 | WA |  | Remove "Hidden Test Details" paragraphs (throughout). |
|  |  |  | 14 | Clarify that Dummy Termination Board can be the DUT itself. Per SIMG/BS, 10/16.{BGZ#28896} |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-10-16. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc05 | 2013-10-16 | WA | 176 | Change 'similar' to 'identical' in step #8 of 5.8.2.3. |
|  |  |  | 127, 175 | Fix typo in title of 4.8.2, 5.8.2. |
|  |  |  |  | Change 4.8.6.1 and 5.8.6.1 back to fc02 format. |
|  | 2013-10-17 | WA | 8 | Clarify and distinguish CDF_ variable names from MEAS_ variable names, in 2.1.2. |
|  |  |  | 132 | Modify VDI detection and testing of ATT command in 4.8.6.1. |
|  |  |  | 180 | Modify VDI detection and testing of ATT command in 5.8.6.1. |
|  |  |  | 136 | Modify VDI detection and testing of 3D modes in 4.8.8.2. |
|  |  |  | 185 | Modify VDI detection and testing of 3D modes in 5.8.8.2. |
|  |  |  | 135 | Modify VDI detection and testing of 3D modes in 4.8.8.1. |
|  |  |  | 184 | Modify VDI detection and testing of 3D modes in 5.8.8.1. |
|  |  |  | 73 | Clarify step #4 in 3.8.2.3. |
|  |  |  | 73, 73 | Fix table cross-ref in steps #5 and #6 in 3.8.2.3. |
|  |  |  | 72 | Change 861E to 861F in step #4 of 3.8.2.1. |
|  |  |  | 72 | Make AVI check dependent on CDF_AVI_SUPPORT, in 3.8.2.1. |
|  |  |  | 73 | Make AVI check dependent on CDF_AVI_SUPPORT, in 3.8.2.3. |
|  |  |  | 80 | Modify VDI detection and testing of ATT command in 3.8.6.1. |
|  | Above changes per markups received in file "Comment for MHL_CTS_3 0_fc04_Sony_20131017_1500.xls" from Sony, 2013-10-17. |  |  |  |
|  |  |  |  | Fix subscripting of font in V, T, I symbols (throughout). |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-10-17. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc06 | 2013-10-17 | WA | 80 | Modify VDI detection and testing of ATT command in 3.8.6.1. |
|  | Above change per markups received in file "Comment for MHL_CTS_3 0_fc05_Sony_20131018_1130.xls" from Sony, 2013-10-18. |  |  |  |
| The above version has been delivered to 5CTT as Word and PDF files on 2013-10-17, 22:30PDT. |  |  |  |  |
| The above version has been approved as CTS 3.0 "Main" by 5CTT on 2013-10-17. |  |  |  |  |
| The above version has been delivered to 5CMT on 2013-10-17. |  |  |  |  |

| Rev | Date | Author | Page(s) | Change |
| --- | --- | --- | --- | --- |
| 3.0 1.0 fc07 | 2013-10-18 | WA | 36 | Remove 'pink' shading from Table 3-3, formerly used to tag December tests. |
|  | 2013-10-23 | WA | 224 | Clarify 7.2.1 CTS 2.1 req'ment. Per Sony, 10/22. |
|  |  |  | 74 | Fix CDF_ field check for supported audio modes in DUT in 3.8.3.1. Per Sony, 10/22. |
|  |  |  | 15 | Fix broken cross-ref formatting. |
| The above version has been delivered to 5CTT, 5CMT as Word and PDF files on 2013-10-23. |  |  |  |  |
| The above version is approved by vote of 5CMT as CTS 3.0 "Main" on 2013-10-22. |  |  |  |  |

__END_OF_CHANGE_HISTORY__