[image: image1.wmf]

Agilent U4305A Protocol Exerciser for PCI Express Gen3
SW Version 8.5
April 05 , 2012
	· Software Version
	8.05.106

	· Internal Build
	8.5.44.06

	· FPGA Versions
	4.91 (Exerciser image) , 5.105 (PTC image)

	· Firmware version
	0.8

	· Platform Version
	1.09.0005 (Release)

	· Package GUID
	22C82A09-2921-4d79-90F2-1E970F7C5200

	· AgtUSBManager
	Version 3.7

	· Update GUI
	Version 3.0

	· Branch
	ati_pcie_integration_exerciser_gen3_ptc_fixes


Overview

PCIe 3.0 exerciser can help you validate your device whether it is a server or an add-in card. The emulated personality is easily switched through software for either a root complex or an end point.  It has support for PCIe, MRIOV, SRIOV protocols. 

www.agilent.com/find/pci-express
Current OSes supported

Windows XP 32/64, Windows 7 32/64

Important usage notes

Refer to Help for details on how to install and use this package.

1. If you have USB Device(s) (U4305A) attached to your system but only one or none of them receives a valid module number (module numbers on the U4305A devices are 10001, 10002 etc.),  this might be due to one of the following reasons:

a. The USB Device(s) has been plugged in for the first time after rebooting the host controller PC and another USB Device or module was already locked by a session. Please close all existing sessions and update the module numbers. Now run Start->Programs->Agilent SPT->Tools->Diagnostics and hit the button 'Update Module Numbers'. 

b. If you are working on a system with Windows XP SP2 this might be a problem with the loopback addresses. This has been fixed by Microsoft in the patch KB893066. You can download the patch at the Microsoft site http://www.microsoft.com/downloads/

2. If your USB controller supports USB2.0 make sure that you use the delivered (or corresponding) USB2.0 cable only.

3. Release 7.3(E2960B Gen2 Exerciser) & Release 8.x (U4305A Gen3 Exerciser) cannot exist on same machine.

4. If you don't see modules after updating Firmware then restart AgtN2XUSBManagerService & AgtN2XResourceManager.

5. Windows 7 support

6. Run the installer exe in Administrator Mode 

7. User Account control Settings should be "Never notify"

8. The location of the OpenE script has been changed from 'C:\AgilentPciExpress' to <Exerciser 

9. Install Folder>\8.x Release\Samples\PCIe Port.

PTC related

1. PTC works at x1 link-width. Higher lanes might be required to be physically masked for DUTs with higher link-widths.

Software Version History
SW 8.5.105(Patch)
· PTC Bug Fixes

· SAD00448433: ECRC third sub-test is repeatedly failing for all DUTs 
· EQ, FLT, LTR Test cases removed LinkUp/LinkDown code which was not required to fix some of the rare issues, unable to reproduce easily. 
· BIOS changes for improvement to avoid some timing issues.
SW 8.5.104(Patch)
· PTC Bug Fixes

· SAD00448434: DLL related test-cases randomly 

· SAD00448435: Equalization Coefficient test-case: Last sub-test - ambiguity in PTC specs 

SW 8.5.103(Patch)
· PTC Bug Fixes

· SAD00448444: various issues/suggestion about the speed reporting 

· SAD00448435: Equalization Coefficient test-case: Last sub-test - ambiguity in PTC specs 

· SAD00448438: Loopback from Configuration is failing for all speeds for many devices 

· Separate category for Gen2 PTC test cases & new Gen3PTC test cases
· Test cases will not try Linkup on higher Linkwidths

SW 8.5.102(Patch)
· PTC Bug Fixes

· SAD00447832: PTC not available in offline mode

· SAD00448441: Test: DLL_05_02_17 produced a yellow caution result

· SAD00448432: LTR test-cases: test-case fails with the message `LTR message not received when LTR is enabled'

· SAD00448434: DLL related test-cases randomly

· SAD00448437: Equalization test-cases - preserving the last test results and test-cases logs typo mistake

· SAD00448439: ASPM L1 and ECRC test-case - final result log is incorrect
SW 8.5.18(Release)
· PTC (Protocol Test Compliance) functionality added

· Exerciser Bug Fixes

· SAD00423891: Sometimes unable to add Exerciser port. One port is not adding and giving error pop up `Unknown capability'.

· SAD00446792: AgtPCIEExerciser SiStatusGet  API not working as expected

· SAD00444201: Exerciser stuck in Link Init/Link Active on restart of server

· SAD00444202: MRIOV - VL flow control Issue

· SAD00446382: If the Exerciser driver is not installed on system under test (Wind2008 server), while going to D3 hot, Exerciser doesnot complete Configcycle

· SAD00447345: Exerciser sometime updating the NPdata credit incorrectly 

SW 8.4

· Additional LTSSM test cases.

· Separate data memory compare for each function.

· Support for pattern matchers.

· Support for different transceiver (Tx) settings at Gen 1, Gen 2, Gen 3 speeds.

SW 8.3

· Power management support - L0s, L1

· ECRC error detection and injection.

· Single Root I/O Virtualization software license for PCIe 5Gbps

· Five functions for use with MRIOV, SRIOV and PCIe Gen 3

· PCIe port access using memory cycles 

· Windows 7 (for Enterprise & Professional 64 bit OS).

SW 8.2

· MRIOV Gen2 Exerciser.

· PCI Express Gen3 Exerciser.

SW 8.1 

· PCI Express Gen3 LTSSM Tester.

· Windows XP, 7 support (for Enterprise & Professional 32bit OS only).
Readme for Agilent U4305A Protocol Exerciser for PCI Express Gen3


