Infiniium 5483x Bios Settings for Windows XP and Windows 98

How to modify Mesa BIOS Screens for Windows 98

1) Enter CMOS Setup Utility (Press DEL key during Agilent Splash Screen).

2) Go to Power Management Setup Menu.

3) Disable ACPI Function (See Figure 1) by pressing Page Up.

4) Press ESC to return to Main Menu.

5) Go to Advanced Chipset Functions.

6) Confirm “On-Chip Video Window Size” is set to Disabled (See Figure 2).

7) If  “On-Chip Video Window Size” is set to 64 MB, Disable by pressing Page Up.

8) Press ESC to return to Main Menu.

9) Go to “Save and Exit Setup”.

10) Press Y to accept changes.

How to modify Mesa BIOS Screens for Windows XP

1) Enter CMOS Setup Utility (Press DEL key during Agilent Splash Screen).

2) Go to Power Management Setup Menu.

3) Enable ACPI Function (See Figure 1) by pressing Page Up.

4) Press ESC to return to Main Menu.

5) Go to Advanced Chipset Functions.

6) Confirm “On-Chip Video Window Size” is set to 64MB (See Figure 2).

7) If  “On-Chip Video Window Size” is set to Disabled, enable by pressing Page Up.  The enabled value is 64MB.

8) Press ESC to return to Main Menu.

9) Go to “Save and Exit Setup”.

10) Press Y to accept changes.

[image: image1.jpg]i

:
i

Menu Level »

n
il

Allows you to enable

T
i

1
CM Thermal-Throttling

Resume by Alarm
x Bate(of Month) Alarm ]
x Tine(hh:mn:ss) Alarm 8: B: B
» Reload Global Timer EvewtsiPress Enter]

ESC:Exit Fi:Gemeral Help




 Figure 1
[image: image2.jpg]LTI




Figure 2

