# Infiniium 5483x Bios Settings for Windows XP and Windows 98

### How to modify Mesa BIOS Screens for Windows 98

- Enter CMOS Setup Utility (Press DEL key during Agilent Splash Screen).

- Go to Power Management Setup Menu.

- Disable ACPI Function (See Figure 1) by pressing Page Up.

- Press ESC to return to Main Menu.

- Go to Advanced Chipset Functions.

- Confirm “On-Chip Video Window Size” is set to Disabled (See Figure 2).

- If  “On-Chip Video Window Size” is set to 64 MB, Disable by pressing Page Up.

- Press ESC to return to Main Menu.

- Go to “Save and Exit Setup”.

- Press Y to accept changes.

### How to modify Mesa BIOS Screens for Windows XP

- Enter CMOS Setup Utility (Press DEL key during Agilent Splash Screen).

- Go to Power Management Setup Menu.

- Enable ACPI Function (See Figure 1) by pressing Page Up.

- Press ESC to return to Main Menu.

- Go to Advanced Chipset Functions.

- Confirm “On-Chip Video Window Size” is set to 64MB (See Figure 2).

- If  “On-Chip Video Window Size” is set to Disabled, enable by pressing Page Up.  The enabled value is 64MB.

- Press ESC to return to Main Menu.

- Go to “Save and Exit Setup”.

- Press Y to accept changes.

Figure 1

Figure 2