N2X IPTV QoE Test Solution

Lab Exercise #1: IPTV QoE measurements via IPTV QoE PA and Using the Test Manager for Automation

Test Objective

In a realistic triple play user environment, subscribers behave in a dynamic fashion. A household receiving triple play services from a single provider may be simultaneously changing IPTV channels, browsing the Internet, and having multiple VoIP telephone conversations. When scaled across thousands of subscribers, this dynamic behavior can be very demanding on the control plane of IPTV network elements, and potentially jeopardize the quality of experience IPTV viewers receive.

The objective of this test is to test the DUT’s’ ability to support thousands of dynamic triple play subscribers, each receiving a combination of video, voice and data traffic, without compromising the video quality experienced by any of the subscribers. The test uses industry-standard RFC 4445 Media Delivery Index metrics to validate IPTV media quality on a per-subscriber basis. Video capture and replay is used to visually examine the impact of packet delay and loss (indicated by poor Delay Factor and Media Loss Rate values) on media quality.

Target Customer:

This test is relevant to engineers building and testing access network equipment, such as a B-RAS, Access Node (DSLAM), Edge Router, BNG, or MSPP, which will be deployed in an IPTV infrastructure. Target NEM groups include: ‘IPTV dev test’, ‘system test’ and ‘POC labs’. Target CSP groups include ‘service validation’ and ‘pre-deployment test’.

Key Features to Highlight

- Dynamic IPTV subscriber emulation

- Triple play (video, voice and data) traffic generation

- Highly scalable per-subscriber video quality metrics

Competitive Note
N2X can provide per subscriber statistics for thousands of subscribers/port, with multiple services per subscriber. Ixia supports only 256 streams/port (256/port) which means IXIA can’t measure QoS per subscriber and per service. Spirent uses V-factor for video quality analysis, which can only scale to a few streams per port.

Competitive Note
N2X can make measurements on MPEG-2 and H.264. Spirent’s V-factor does not support MPEG-4/H.264, which is very important for HDTV.

Competitive Note
N2X can scale across multiple ports in a single system with multiple access protocols running concurrently on each port. IneoQuest is a point solution for video and multiple modules cannot be controlled as part of the same test session.

Equipment Required

N2X Equipment

- Two N5551A/B “Tahu” Tri-rate Ethernet ports

N2X Software and Slides

All of the N2X software and materials to perform this demo are listed below, and can be downloaded from the IPTV QoE Test Solution – AE Resources Web Page:
<<URL to be provided>>

- E7881A/B Packets and Protocols License

  - N2X System Release 6.7 or later

- N5570A IPTV Quality of Experience Productivity Application

- E7877A Video Generation and Analysis License

- E7828A IGMP License (for release 6.9 and later only)

Device Under Test

This particular test assumes that the DUT is acting as an IPv4 multicast (IGMP) replicator. The device must have Ethernet interfaces (Ethernet, Fast Ethernet or GbE), either electrical or optical. Typical DUTs include B-RAS/BNGs, EARs and BSA/BSRs.

Test Set-up

Network Diagram: Testing against a real DUT

Video Source

Subscribers

100 VLANs,

simulating 100 subscribers

IGMP joins from 100 subscribers

Real video feeds to subscribers

Bi-directional data & VoIP traffic

N2X Port N2XVidPort

N2X Port N2XSubPort

DUTSubPort

DUTVidPort

DUT Configuration:

Configure 100 VLANs on DUTSubPort according to the addresses assigned to your group. Each VLAN represents a single subscriber. Create a single VLAN on DUTVidPort according to your assigned addresses.

Lab Instructions

Port Setup

- Click Ports from the main window top tool bar. In Port Selection dialog, select one test port, change its type to “Video Server”. Select another test port. Change its type to “Video Subscriber”. Click Ok. (See the Appendix for the Port assignment for your group)

Note: “Video Server” ports can send regular traffic as well, but will be limited due to the video load on them.

Note: Selecting a wrong personality for the test port will result in inability to get MDI statistics and capture video traffic.

Figure 1: Port and Port Type Selection

- Click Physical Layer from the main window side tool bar.

- Configure the ports selected

- In the Physical Configuration dialog, change the Media Type to SPF if using optical connection, or RJ45 if using copper wire.

Note: For SPF, Step 5 and 6 must be performed. For other media type, skip step 5 and 6.

- Click Turn All Laser Off

- Click Turn ALL Laser On

- Close physical layer configuration

Start IPTV Productivity Application

- From the Packets and Protocols window, click Applications on the tool bar. The Applications panel is displayed.

- On the bottom of the Application panel, click Productivity tab.

- IPTV > IPTV Quality of Experience (v 2.04). Double-click to bring up the application.

Task 1: Select a Test Scenario

- Click Configure from the top tool bar. The Configure Parameters dialog box is displayed.

- Click Configure Test tab.

- Click Test Scenario tab.

- Select Test Case: Sustained Performance from the drop down list

- Specify a join delay threshold. Join delays exceeding this threshold will be reported in the test report.

- Specify Test duration : 0:01:00

Task 2: Configure Video Source

- Click Video Source tab.

- In the Video Sources area, select the default port row, click Remove.

- Click Add Port. New Source Port dialog box is displayed.

- Configure fields, and click OK.

  - Port: <your video source port number>

  - Port Type: Video Server

  - VLAN Range: 1-1023

  - Tester IP Addr : <your Tester IP address for the Video Source port>

  - SUT IP Addr: <your SUT IP address for the Video Source port>

Figure 2 Defining a Video Source Port

Note: The port type property cannot be changed once the port is added. The only way to change this property is to remove and re-add the port.

Note: Correctly defining a port personality is important. Selecting a Video Server port personality enables video clips selection. More on video clips selection later.

- From the Video Sources area, click Add Sub-Interfaces. Add Video Server Source Interface dialog box is displayed.

Figure 3 Adding a Video Server Sub-Interface

- Configure the fields, and click OK.

  - Total VLANs in Pool: 1

  - Starting VLAN: 1

  - Tester IP Addr : <your Tester IP address for the Video Source port>

  - SUT IP Addr: <your SUT IP address for the Video Source port>

- From the Channels (Multicast Groups) area, select all the default channels, and click Remove.

- Click Add. Add Channels dialog box is displayed.

Figure 4 Defining Multicast Group Addresses

- Configure fields for Channel Definition:

  - Num Channels: 8

  - Group: <your starting address for the multicast group pool> For example: Group 1 – 225.1.0.10

  - Source Test Port

  - Sub-Interface

  - Source IP Address

Note: A maximum of 512 channels is supported per test session.

- Click Channel Content tab.

Competitive Note:
N2X can make measurements on SDTV and HDTV. Spirent’s V-factor does not support MPEG-4, which is mandatory for HDTV.

Figure 5 Defining Channel Content for a Multicast Group

- Select Video Clip.

Note: The selection for Video Clip is only enabled when the Source Test Port you defined in the Channel Definition tab is a Video Server personality.

- Click  to add a video clip.

- Define MDI Thresholds for the multicast group.

Note: MDI threshold defines acceptable video performance level. Video exceeding the thresholds will be reported in the test report.

- (Optional) Click Channel Priority to configure VLAN priority and QoS values.

- Click OK.

Figure 6 Video Source Configuration

- (Optional) In the Channels (Multicast Groups) area, click Multicast Address / IGMP Config. Multicast Address Editor dialog box is displayed.

Figure 7 IGMP Configuration

- (Optional) Here you can configure the first 23 bits of multicast address for all the multicast groups or configure IGMP version for all the subscribers.

Task 3: Configure Video Subscribers

- Click Subscribers tab.

- Select the default subscriber test port row, click Remove. (All the interfaces will also be removed)

- Click Add Port. New Subscriber dialog box is created.

N2X can emulate a subscriber per VLAN, and provide per subscriber statistics for thousands of subscribers/port, with multiple services per subscriber. Ixia has a small number of streams per port (256/port) which means IXIA can’t measure QoS per subscriber and per service. Spirent uses V-factor for video quality analysis, which can only scale to a few streams per port.

Figure 8 Defining a Subscriber Port

- Configure fields, and click OK.

  - Port: <your video subscriber port number>

  - Port Type: Ethernet

  - VLAN Range: 1-1023

  - Tester IP Addr : <your Tester IP address for the Video Subscriber port>

  - SUT IP Addr: <your SUT IP address for the Video Subscriber port>

  - VLAN Config: 1:1 Video, Voice, Data

Note: 1:1 Video, Voice Data—to send subscriber video, voice, and data traffic through their individual subscriber VLANs (CVLANs). These VLAN IDs are configured on the Add/Edit Subscriber Pool dialog box, Starting VLAN field.

You will be creating two subscriber pools with 50 subscribers in each pool.

- Select the subscriber port row, click Add Subscriber Pool. Add Ethernet Subscriber Pool dialog box is displayed.

- Configure fields, and click OK.

  - Total Subscribers in Pool: 50

  - Name: pool_1

  - Starting VLAN: 1

  - Residential Gateway: Router with NAT

  - IP Address Assignment: Manual

  - #STB: 1

  - IP Phone enabled

  - PC enabled

  - Tester IP Address: <your first Tester IP address for the Video Subscriber port> For example: Group 1 – 101.1.1.2

  - SUT IP Address: <your first SUT IP address for the Video Subscriber port> For example: Group 1 – 101.1.1.1

Note: IP Phone simulates a VoIP phone sending bidirectional voice traffic. PC simulates a PC accessing Internet services. You will learn how to configure the traffic sent by these devices in task 4.

Figure 9 Configuring the First Subscriber Pool

- Repeat step 38 and 39 to create a second subscriber pool with the following changes:

  - Name: pool_2

  - Starting VLAN: 51

  - Tester IP Address: <your 51st Tester IP address for the Video Subscriber port> For example: Group 1 – 101.1.51.2

  - SUT IP Address: <your 51st SUT IP address for the Video Subscriber port> For example: Group 1 – 101.1.51.1

Figure 10 Configuring a Second Subscriber Pool

- In the Watching Channels area, select the first subscriber pool, and click Select Channels. The Channel Selector dialog box is displayed.

Figure 11 Defining Subscribed Multicast Groups for a Subscriber Pool

Note: For the Sustained Performance test, each subscriber within a pool has 1 TV changing among 4 channels using a defined schedule. Use up and down arrows to define the order of channel changing. Use right and left arrows to add/remove channels to/from the list of Selected Channels.

- Add the first four defined channels from the list of All Available Channels to the list of Selected Channels.

- For the second subscriber pool, repeat step 41 and 42 to add the other 4 channels (channel-5 to channel-8) to the list of Selected Channels.

- (Optional) In the Watching Channels area, click Define Times to define the duration of channel watching for each channel.

Figure 12 Subscribers Configuration

Task 4: Configure Background Traffic

Background traffic simulates a real-world scenario in which video traffic is sent alongside Internet traffic like voice and data. The HSIA-like and voice-like traffic are sent from each server to each client in both directions. In other words, the PA sets up a partial mesh and bi-directional traffic between the sources (server ports) and destinations (subscriber pools). Upstream and downstream traffic defines the content being sent between the sources and destinations.

- Observe that the video source port you configured in Task 2 is automatically added to the list of Server ports.

- Observe that the subscriber pools you configured in Task 3 are automatically added to the list of Clients.

Figure 13 Background Traffic

- Observe that there are four pre-defined traffic streams:

Data Dn Stream—Internet-like traffic being downloaded from servers to the subscribers.

Data Up Stream—Internet control packets being uploaded from subscribers to servers.

Voice Dn Stream—VoIP traffic being sent from servers to the subscribers.

Voice Up Stream—VoIP traffic being sent from subscribers to servers.

- (Optional) Select a Data stream and click  to modify the properties like packet length or QoS values.

Figure 14 Properties of a Data Stream

- (Optional) Select a Voice stream and click  to modify the properties like Codec or QoS values.

Figure 15 Properties of a Voice Stream

Task 5: Configure Test Control

- Click Test Control tab.

- Traffic: Phased Subscriber Pool Join/Leave – defines the delay of IGMP join messages sent between the subscriber pools.

Note: With the introduction to IGMP join/leave rates control in release 6.9, the users can further fine-tune the delay of IGMP join/leave messages sent between each subscriber. As a result, the phased join/leave latency will be obsolete 6.9 onwards.

- In Run Test area, select Run Mode: Setup only.

Note: In the Setup only mode, the PA will only setup the test configurations in N2X. You can then use the underlying N2X to further fine-tune the test, for example, to define more traffic to simulate a wider range of triple play services. In this lab exercise, you will learn how to set up a capture filter to capture a video stream.

Note: If you do not need to fine-tune anything in N2X, select Setup and run mode. This will configure the test and start the routing emulations, traffic generation, and statistics measurements.

- Test Topology: select Test against SUT.

Note:  If you use two test ports connected in loopback, with one port transmitting to a second port and the second port transmitting to the first port, select Test Topology: Test in back to back connection.  In this mode the tester does not require SUT communication to, for example, resolve MAC addresses and an emulated DHCP server is setup on the server port if needed.

Figure 16:  Test control

- Close the configuration window.

- You have now completed the test configuration, close the Configure Parameters window.

- Click  to run the test.

Check N2X Setup

- Observe that the Link Layer is setup. N2X Packets and Protocols window: Setup tool bar: Link Layer > Ethernet tab > LAN/VLAN Addresses.

Figure 17 Link Layer Setup

- Observe that voice and data traffic are setup for both directions: video server from/to video subscribers. N2X Packets and Protocols window : Setup tool bar : Traffic

Figure 18 Background Traffic Setup

- Observe that the IGMP emulations are set up. N2X Packets and Protocols window: Setup tool bar: Emulation.

Figure 19 IGMP Emulations Setup

- Observe that multicast group pools are set up. N2X Packets and Protocols window: menu bar: Actions > View Multicast Group Pools.

Figure 20 Multicast Group Pools Setup

- Observe that multicast traffic is setup. N2X Packets and Protocols window: Setup bar: Video.

Figure 21 Video Clips Setup

- Select a video stream. Click  to play the video clip.

Setup Capture Filter and Trigger

Recall that we specified the PA to run in Setup Only mode. The following part of the lab exercise provides an example of fine-tuning the N2X configurations before using the PA to generate traffic and collect measurements. You will learn how to setup a capture filter to capture a video stream based on the multicast address.

- N2X Packets and Protocols window: Setup bar: Capture.

- In the Setup – Capture area, click Cyclic.

- Enable the Video Subscriber port for capturing.

- Select the Video Subscriber port row, click  to configure capture properties. <Port> Capture Properties dialog box is displayed.

- Set buffer size to 60%.

- Click Edit Frame Matchers. The Frame Matcher Library Editor dialog box is displayed.

Figure 22 Frame Matcher Library Editor

- Select the capture port (your video subscriber port) from the On port drop-down list, click Add.

- Enter name: Channel_1_Captured.

- Enable the Pattern check box.

- Click Edit Patterns. The Pattern Library Editor dialog box is displayed.

Figure 23 Defining a Pattern on a Multicast Address

- Click Add to add a pattern.

- Enter Pattern name: Channel_1_Multicast_Address.

- In the Field details area, select destination _address.

- In the Value area, select IP, enter <your multicast address for channel 1>. For example, group 1 – 225.1.0.10.

- Click Exact.

- Click Done.

- Click Close.

- Back in the Frame Matcher Library Editor dialog: Pattern, select Channel_1_Multicast_Address from the drop-down list.

- Click Apply.

- Click Close.

- Back to N2X Packets and Protocols main window: Setup – Capture. Enable the frame matcher to store frames upon filtering.

Figure 24: Setup Capture Filter

- Click  to start capturing.

Run Test

- Go back to the IPTV QoE PA main window.

- Click Configure.

- Click Test Control tab.

- Select Run Mode: Run only.

- Close the Configure Parameters window.

- Click  to generate traffic and collect measurements.

- Observe the Graphs for real-time results: Ave. Join Latency vs. Time, MDI DF vs. Time, MDI MLR vs. Time.

Figure 25: Real-time Graphs for Join Latency and MDI Measurements

Replay Captured Multicast Traffic

- Stop Capture.

- Select the Video Subscriber port row. Right click > Save Capture to File. The Save Capture <Port> dialog box is displayed.

- Specify a capture file name, click OK.

- Select the Video Subscriber port row. Right click > Open Capture File. Select the saved .cap.

- Right click on the .cap file > View Video.

Figure 26 Save, restore and replay capture

Test Report

Figure 27: Report of Latency Measurements

- When the test is done, Click Report tab.

- Observe measurements for per VLAN and Channel stats.

- Observe the Threshold Exceedance Event entries.

Figure 28: Report of MDI Measurements Exceeding the Threshold Values

## IPTV Test Manger

Last updated: May8, 2007 by Ken Ho

## Overview

The IPTV PA Test Manager allows a user to prepare test scripts to execute a test with multiple test cases.

After test execution, the user is able to retrieve the test results for further processing using API methods provided by the IPTV PA Manager. (The data files, in CSV formats, generated during a test can be imported directly to another application such as Microsoft Excel. However, Agilent does not guarantee that the format of the data files will not change.)

## Features and Limitations:

The Test Manager is composed of two independent parts:
    (1) automated test execution, and
    (2) test data post-processing.

The features and limitations of the Test Manager are:

- The Test Manager uses only IPTV Configuration File to specify test cases. The Test Manager does not provide an API to enable scripts to modify the configuration. (The current set of IPTV PA API functions to 'Update' the configuration will be available to the user. However, its use is discouraged because modifying a configuration without saving it and then running a test case will result in mismatched configuration file and results file.)

- The Test Manager will only record the final results generated by IPTV (join/leave delays, MDI results). It will not capture nor store intermediate results (such as IP address correctness or timestamp values).

- The user's responsibilities are:

-   prepare configuration files as test cases
-   prepare test case specification file to define scope of testing
-   define tcl procedures to evaluate results (post processing)
-   Execute a test session (defined in Test Case Specification File)
-   Post-process data files

- Each test data file will contain metadata of the test. These include test case, test date, run sequence, etc. The set of test data files may be visualized as a database of test result tables. The post-processing API will provide method to manipulate this "database" (such as extracting rows and columns, combining tables, etc.) Additionally, the API will provide rudimentary methods to perform simple data filtering. The API can be extended by the user to implement custom data filtering and calculation methods.

## Tutorial

### Step 1: Preparation of IPTV Configuration Files.

The user is required to prepare a set of IPTV Configuration Files to execute a set of test scenarios. It is assumed that the user is familiar with this task and no further explanation is provided here.

### Step 2. Manually generate a Test Specification File.

The Test Specification file describes to the Test Manager what test cases to test. This file is structured as a tcl data structure (lists and arrays).

The following is a sample Test Specification File.

#

# IPTV PA Test Manager

#

# Sample Test Case Specification File

#

#

# Notes:

#   A test specification ("test spec") contains a number of test cases.

#   The automated testing of IPTV runs all test cases specified by a test spec.

#

# The following variables are supplied by the user. ("Free form" means no specific

# data formating is required.)

#   specName:       Descriptive name of test specification. (Free form. Required.)

#   specNumber:     Specification number of ID.  (Free form. Required.)

#   revision:       Revision number of the test specification.  (Free form. Required.)

#   author:         Author of the test specification file.  (Free form. Required.)

#   date:           Date of spec file generation.  (Free form. Required.)

#   reportDir:      Path of data file location in unix notation

#                        Format: drive:/dir/dir/... (e.g. c:/abc/xyz)

#                        This parameter is optional. If not provided, the reportDir

#                        in each specified configuration file is used.

#                        The generated data file names defaults to:

#                             --.csv

#

#   testCaseName:   Name of test case. (Free Form. Required.)

#   purpose:        (Free Form. Required: may contain empty string.)

#   requirmentReference:    Reference to test requirment documentation.

#                           (Free Form. Required: may contain empty string.)

#   description:            Desciption of test case.

#                           (Free Form. Required: may contain empty string.)

#   configFile:      Full path name of config file (in unix format). (Required.)

#   firstSequenceNo: Each test run is numbered by a sequence number. This parameter

#                    specifies the first sequence number. If not specified, the

#                    first sequence number defaults to 1. (Optional.)

#   runModes:        Defines run modes of each test run. The format is:

#                       {  ...}

#                     may be one of:

#                       SETUP_RUN

#                       RUN_ONLY

#

#

namespace eval ::TestSpec {

array unset testSpecInfo

array set testSpecInfo \

[list \

specName                "MyTestSuite"                                           \

specNumber              "1234.44"                                               \

revision                "6.8.1.3"                                               \

author                  "H H Beans"                                             \

date                    "2007-01-16"                                            \

reportDir               "C:/users/ken/projects/oslo/test-manager/result"        \

]

set testCaseList \

[list \

[list \

testCaseName        "MyTestCase"                                           \

purpose             "blah blah blah"                                       \

reqmtRef            "Montreal Product Def 105"                             \

description         "blah blah blah"                                       \

configFile          "C:/users/ken/projects/oslo/test-manager/config/demo-9-jd.cfg.tcl"   \

firstSequenceNo     5                                                      \

runModes            {SETUP_RUN RUN_ONLY RUN_ONLY RUN_ONLY}                 \

] \

[list \

testCaseName        "MyTestCase"                                           \

purpose             "blah blah blah"                                       \

reqmtRef            "Montreal Product Def 106"                             \

description         "blah blah blah"                                       \

configFile          "C:/users/ken/projects/oslo/test-manager/config/demo-9-sp.cfg.tcl"   \

firstSequenceNo     5                                                      \

runModes            {SETUP_RUN RUN_ONLY RUN_ONLY RUN_ONLY}                 \

] \

]

}

In this example, the Test Manager will run the test based on two configuration files:
    demo-9-jd.cfg.tcl
    demo-9-sp.cfg.tcl

For each config file, the test will be repeated four times (SETUP_RUN once, and then RUN_ONLY three times).

### STEP 3. Execute the test.

Execute the test specification:

> tclsh83 iptv.test-manager.tcl RUN_TEST myTestSpecFile.txt

### STEP 4. Post process the data files.

Post-test data processing of test results requires executing the Test Manager's Post Processing API methods through a tcl console or script. These API methods allows the user to combine, extract, query, and filter the data in data files.

> tclsh83

> source iptv.post-process-result.tcl

> namespace import ::AgtIptvTestManager::*

> help

The post-processing API consists of the following functions:

    ::AgtIptvTestManager::PrintList
    ::AgtIptvTestManager::PrintArray
    ::AgtIptvTestManager::ListDirs
    ::AgtIptvTestManager::ListFiles
    ::AgtIptvTestManager::GetReportFileProperties
    ::AgtIptvTestManager::LoadReportFile
    ::AgtIptvTestManager::CombineResultSets
    ::AgtIptvTestManager::CombineResultSetsAllColumns
    ::AgtIptvTestManager::CombineResultSetsCommonColumns
    ::AgtIptvTestManager::AddColumn
    ::AgtIptvTestManager::ListColumns
    ::AgtIptvTestManager::GetNumberOfRows
    ::AgtIptvTestManager::GetRows
    ::AgtIptvTestManager::GetColumns
    ::AgtIptvTestManager::SaveResultSet
    ::AgtIptvTestManager::LoadResultSet
    ::AgtIptvTestManager::SortResultSet
    ::AgtIptvTestManager::AggregateResultSet
    ::AgtIptvTestManager::FilterResultSet
    ::AgtIptvTestManager::PrintResultSet

(For detail description of the functions, type the function name followed by '?'. E.g.: LoadReportFile ?)

In addition, the following filtering operators are available. (The '?' does not work with these operators):

    ::AgtIptvTestManager::GE
    ::AgtIptvTestManager::GT
    ::AgtIptvTestManager::EQ
    ::AgtIptvTestManager::NE
    ::AgtIptvTestManager::LE
    ::AgtIptvTestManager::LT

The following aggregate functions are also available. (The '?' does not work with these operators):

    ::AgtIptvTestManager::AGG_AVG
    ::AgtIptvTestManager::AGG_SUM
    ::AgtIptvTestManager::AGG_MAX
    ::AgtIptvTestManager::AGG_MIN

For information on developing your own customized operators, type:
     help op

The usual sequence to post process the data files are as follows:

set myResultSet [LoadReportFile <file-path-name>]

PrintResultSet myResultSet

set filteredRs [FilterResultSet myResultSet ...]

SaveResultSet filteredRs

Detailed Description of API functions.

::AgtIptvTestManager::PrintList
    Description:
        Print a list and the corresponding index.
    Synopsis:
        ::AgtIptvTestManager::PrintList <tcl-list-name>
    Parameters:
        tcl-list-name:  The name of the tcl list to be printed.
    Returns:
         Nothing.

::AgtIptvTestManager::PrintArray
    Description:
        Print an a tcl array.
    Synopsis:
        ::AgtIptvTestManager::PrintArray <tcl-array-name>
    Parameters:
        tcl-array-name:  The name of the tcl array to be printed.
    Returns:
         Nothing.

::AgtIptvTestManager::ListDirs
    Description:
        Get a list of subdirectories in the current working directory.
    Synopsis:
        ::AgtIptvTestManager::ListDirs [<pattern>]
    Parameters:
        pattern:            (Optional.) File name filter pattern (e.g. '2007.03.14-*'). Defaults to all files if parameter is not provided.
    Returns:
         A list of directory names in the current directory.

::AgtIptvTestManager::ListFiles
    Description:
        Get a list of files in the current working directory.
    Synopsis:
        ::AgtIptvTestManager::ListDirs [<pattern>]
    Parameters:
        pattern:            (Optional.) File name filter pattern (e.g. '2007.03.14-*'). Defaults to all files if parameter is not provided.
    Returns:
         A list of file names in the current directory.

::AgtIptvTestManager::GetReportFileProperties
    Description:
        Get properties from a report file.
        For example: array set props [GetReportFileProperties] "myreport.csv"
    Synopsis:
        ::AgtIptvTestManager::GetReportFileProperties <report-file-name>
    Parameters:
        report-file-name: Name of report file.
    Returns:
         A tcl array containing property names and property values.

::AgtIptvTestManager::LoadReportFile
    Description:
        Read report file and return it as a tcl data structure.
        Example: set rs [LoadReportFile myReportFile.csv]
    Synopsis:
        ::AgtIptvTestManager::LoadReportFile <report-file-name>
    Parameters:
        report-file-name: Name of report file.
    Returns:
         A tcl structure of the loaded result set.

::AgtIptvTestManager::CombineResultSets
    Description:
        Given a set of result sets, combine them together (by appending all the rows) into one   new result set. The new result set is returned.
        NOTE: All result sets must have the same number of columns and column names must match 1-to-1. Column orders are not important and are not preserverd.
        E.g.         set rs1  [LoadReportFile ...]
             set rs2  [LoadReportFile ...]
             set rs   [CombineResultSets rs1 rs2 ]
             # NOT: set rs [CombineResultSets $rs1 $rs2 ]
    Synopsis:
        ::AgtIptvTestManager::CombineResultSets <result-set-name> [<result-set-name> ...]
    Parameters:
        result-set-name: Name of result set.
    Returns:
         A new result set containing the combined result sets.

::AgtIptvTestManager::CombineResultSetsAllColumns
    Description:
        Combine the input result sets and return the new combined result set. All columns are  included.
            E.g.:
                rs1:
                    A   B
                    1   2
                    3   4
                rs2:
                    A   C
                    5   6
                    7   8
                set rs [combinedResultSet rs1 rs2]
                    A   B   C
                    1   2   ""
                    3   4   ""
                    5   ""  6
                    7   ""  8

    Synopsis:
        ::AgtIptvTestManager::CombineResultSetsAllColumns <result-set-name> [<result-set-name> ...]
    Parameters:
        result-set-name: Name of result set.
    Returns:
         A new result set containing the combined result sets.

::AgtIptvTestManager::CombineResultSetsCommonColumns
    Description:
        Combine the input result sets and only common columns are included.
            E.g.:
                rs1:
                    A   B   C
                    1   2   10
                    3   4   11
                rs2:
                    A   C   D
                    5   6   20
                    7   8   21
                set [combinedResultSet rs1 rs2]
                    A   C
                    1   10
                    3   11
                    5   6
                    7   8

    Synopsis:
        ::AgtIptvTestManager::CombineResultSetsCommonColumns <result-set-name> [<result-set-name> ...]
    Parameters:
        result-set-name: Name of result set.
    Returns:
         A new result set containing the combined result sets.

::AgtIptvTestManager::AddColumn
    Description:
        Add a new column whose values are function of other given columns.
        E.g.: proc ::AgtIptvTestManager::RANGE { minVal maxVal } {
                  return [expr maxVal - minVal]
              }
              set rs1 [AddColumn rs RANGE range minCol maxCol]
              rs:
                 minCol  maxCol
                 1       10
                 5       10

              rs1:
                 minCol  maxCol range
                 1       10     9
                 5       10     5
    Synopsis:
        ::AgtIptvTestManager::AddColumn <result-set-name> <function> <new-col-name> <col-name> [<col-name> ...]
    Parameters:
        result-set-name:	Name of result set.
        function:		Name of an evaluation function that accepts a number of column names and returns a computed value.
        new-col-name:	The new column name
        col-name:	Name of a column to be passed to the filter-function.
    Returns:
         A new result set containing the old result set plus the newly added column.

::AgtIptvTestManager::ListColumns
    Description:
        List the column names of a result set.
    Synopsis:
        ::AgtIptvTestManager::ListColumns <result-set-name>
    Parameters:
        result-set-name: Name of result set.
    Returns:
         A tcl list containing column names.

::AgtIptvTestManager::GetNumberOfRows
    Description:
        Get the number of rows in a result set.
    Synopsis:
        ::AgtIptvTestManager::GetNumberOfRows <result-set-name>
    Parameters:
        result-set-name: Name of result set.
    Returns:
         An integer value indicating the number of data rows (excluding the header row).

::AgtIptvTestManager::GetRows
    Description:
        Get some or all rows from a result set. The rows returned starts at first-row (zero-based) and ends at last-row (zero-based), inclusive.
    Synopsis:
        ::AgtIptvTestManager::GetNumberOfRows <result-set-name> <first-row> <last-row>
    Parameters:
        result-set-name: Name of result set.
        first-row:       First row number (zero based).
                         If first-row < 0, then first-row is assumed to be 0.
        last-row:        Last row number (zero based).
                         If last-row > number of rows - 1, then last is assumed to be number of rows - 1.
                         "last" can be have the value of "end".
    Returns:
         A new result set containing a subset of rows of the original result set.

::AgtIptvTestManager::GetColumns
    Description:
        Get some of all columns from a result set.

           E.g.:
                  rs1:
                      A   B   C
                      1   2   10
                      3   4   11
                  set rs [GetColumns rs1 {"A" "C"} ]
                  rs:
                      A   C
                      1   10
                      3   11

    Synopsis:
        ::AgtIptvTestManager::GetColumns <result-set-name> <col-name> [<col-name> ...]
    Parameters:
        result-set-name: Name of result set.
        col-name:        Name of a column.
    Returns:
         A new result set containing a subset of columns of the original result set.

::AgtIptvTestManager::SaveResultSet
    Description:
        Save a result set to a comma delimited file. The saved result set file is different from a report file generated by the IPTV PA. The saved result set file contains no metadata except for column names.)
    Synopsis:
        ::AgtIptvTestManager::SaveResultSet <result-set-name> <filename>
    Parameters:
        result-set-name: Name of result set.
        filename:        Name of the file to save to.
    Returns:
         1 if saved successfully. 0 otherwise.

::AgtIptvTestManager::LoadResultSet
    Description:
        Load a saved result set (not an IPTV PA generated report file).
    Synopsis:
        ::AgtIptvTestManager::LoadResultSet <filename>
    Parameters:
        file-name:       Name of a previously saved result set file.
    Returns:
         The newly loaded result set.

::AgtIptvTestManager::SortResultSet
    Description:
        Sort the rows of a result set in the given column order. Values in result sets that are "n/a" will be treated by the tcl sort engine as a string and sorted accordingly.
    Synopsis:
        ::AgtIptvTestManager::SortResultSet <result-set-name> [<tcl-lsort-option> <tcl-lsort-option> ...] <col-name> [<col-name> ...]
    Parameters:
        result-set-name:  Name of result set.
        tcl-lsort-option:  (Optional) Any parameter that begins with a '-' character
                          is considered to be a tcl lsort option and will be passed to
                          the tcl lsort function.
        col-name:         Name of a column.
    Returns:
         A new result set containing the sorted result set.

::AgtIptvTestManager::AggregateResultSet
    Description:
        Calculate the aggregate value of a column of a result set.
        For example: set avg [AggregateResultSet rs AGG_AVG delay]
           where delay is a column name and agv holds a double value containing the average of all the values of the delay column.
    Synopsis:
        ::AgtIptvTestManager::AggregateResultSet <result-set-name> <aggregate-function> <col-name>
    Parameters:
        result-set-name:     Name of result set.
        aggregate-function:  Name of an aggregate function that accepts a
                          column name and returns a value (the type is dependent.
                          on the aggregate function.
                          Some standard aggregate functions are:
                          AGG_SUM, AGG_AVG, AGG_MIN, AGG_MAX
                          You may implement your own aggregate function accepts a tcl list with
                          any number of values and returns a single value.
        col-name:         Name of a column to be passed to the aggregate-function.
    Returns:
         A numerical result of the aggregation operation.

::AgtIptvTestManager::FilterResultSet
    Description:
        Filter a result set.
        A row will be selected by the 'filter' (and added to the new result set)
        if the filter-expression evaluates to true on the row. A filter-expression
        specifies how to compare various column values. For example:
          (([LT delayA 6.0] && [GT delayA 3.0]) || [EQ delayB 1.0])
        where delayA and delayB are column names.

        If your column name contains '(' or ')', you need to escape them
        with backslashes '\'. For example, WatchTime\(ms\).

        For example:
           set rs2 [FilterResultSet rs {(([LT delayA 6.0] && [GT delayA 3.0]) || [EQ delayB 1.0])} ]
        Note: the filter expression must be enclosed by curly brackets '{' and '}'.
    Synopsis:
        ::AgtIptvTestManager::FilterResultSet <result-set-name> <filter-expression>
    Parameters:
        result-set-name:    Name of result set.
        filter-expressiion: A logical expression that operates on column names
                            and constant values. The following operators are
                            supported:
                              ! || &&
                            The following filter-functions are also supported:
                              LT LE EQ NE GT GE
                            A filter-function subexpression have the following format:
                               [ filter-function colName constant ], or
                               [ filter-function colName colName  ]
        filter-function:  Name of an evaluation function that accepts two
                          column names or constant values and returns a boolean value.
                          Some standard filter-functions are:
                          GT, GE, EQ, NE, LT, LE.
                          You may implement your own customized tcl procedure that accepts
                          two values and returns 0 or 1.
        col-name:         Name of a column to be passed to the filter-function.
    Returns:
         A new result set containing filtered rows.

::AgtIptvTestManager::PrintResultSet
    Description:
        Prints the result set in a nice format. The "printed" output
        is actually returned.
    Synopsis:
        ::AgtIptvTestManager::PrintResultSet <result-set-name> [<start-row> <end-row>]
    Parameters:
        result-set-name:  Name of result set.
        start-row:        (Optional) Start printing from this row. (1 is the first row.)
        end-row:          (Optional) Stop printing at this row. (Integer or 'end'.)
    Returns:
         A string (with newline characters) containing the formatted output.

New Features Added in Release 6.9

- IPTV Test Manager

- IGMP Join/Leave Rate Control

- IGMPoPPP

Enhancement in Release 6.9

- Native Ethernet video server (Before 6.9, you must add a sub-interface to a video source port.)

- Multiple subscriber pools on a native Ethernet with the same network prefix. (Before 6.9, only one subscriber pool is supported for the native Ethernet configuration)

- When VLAN configuration is “1:1 Video, Voice, Data”, switch is supported as the RG type. (Before 6.9, only router with NAT as RG is supported.)

- When VLAN configuration is “N:1 Video, 1:1 Voice, Data”, router with NAT is supported as the RG type. (Before 6.9, only switch as RG is supported)

- When VLAN configuration is “N:1 Video, 1:1 Voice, Data” with IGMP messages sent on multicast VLAN, manual IP address assignment is supported. (Before 6.9, only use of DHCP IP address assignment is allowed.)

- MPEG-2 TS decodes using Build-In decoder (“Honda”)

Appendix

CONTROLLERS

  - Group 1 – 5 Controller1  DELL 1425
Controller Address: 141.184.123.48
User Logins: N2xUser1, N2xUser2, N2xUser3, N2xUser4, N2xUser5
Password: Agilent

  - Group 6 – 10 Controller2 DELL 1425
Controller Address: 141.184.123.47
User Logins: N2xUser1, N2xUser2, N2xUser3, N2xUser4, N2xUser5
Password: Agilent

|  | Group1 | Group2 | Group3 | Group4 | Group5 |  | Group6 | Group7 | Group8 | Group9 | Group10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Video Source Port | 101/1 | 101/3 | 102/2 | 103/1 | 104/1 |  | 104/1 | 201/1 | 101/2 | 102/1 | 102/3 |
| Connector | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |  | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |
| Tester IP Addr | 100.1.1.2 | 102.1.1.2 | 104.1.1.2 | 106.1.1.2 | 201.1.1.2 |  | 203.1.1.2 | 205.1.1.2 | 120.1.1.2 | 122.1.1.2 | 124.1.1.2 |
| SUT IP Addr | 100.1.1.1 | 102.1.1.1 | 104.1.1.1 | 106.1.1.1 | 201.1.1.1 |  | 203.1.1.1 | 205.1.1.1 | 120.1.1.1 | 122.1.1.1 | 124.1.1.1 |
| Router Interface | FE 1/0 | FE 1/2 | FE 1/4 | FE 1/6 | FE 2/1 |  | FE 2/3 | FE 2/5 | FE 12/0 | FE 12/2 | FE 12/4 |
| Multicast Addr Range |  |  |  |  |  |  |  |  |  |  |  |
| From | 225.1.0.10 | 225.1.0.20 | 225.1.0.30 | 225.1.0.40 | 225.1.0.50 |  | 225.1.0.60 | 225.1.0.70 | 225.1.0.80 | 225.1.0.90 | 225.1.0.100 |
| To | 225.1.0.19 | 225.1.0.29 | 225.1.0.39 | 225.1.0.49 | 225.1.0.59 |  | 225.1.0.69 | 225.1.0.79 | 225.1.0.89 | 225.1.0.99 | 225.1.0.109 |
| Subscriber Port | 101/2 | 102/1 | 102/3 | 103/2 | 104/2 |  | 104/2 | 201/2 | 101/3 | 102/2 | 103/1 |
| Connector | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |  | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |
| Tester IP Addr |  |  |  |  |  |  |  |  |  |  |  |
| From | 101.1.1.2 | 103.1.1.2 | 105.1.1.2 | 107.1.1.2 | 202.1.1.2 |  | 204.1.1.2 | 206.1.1.2 | 121.1.1.2 | 123.1.1.2 | 125.1.1.2 |
| To | 101.1.100.2 | 103.1.100.2 | 105.1.100.2 | 107.1.100.2 | 202.1.100.2 |  | 204.1.100.2 | 206.1.100.2 | 121.1.100.2 | 123.1.100.2 | 125.1.100.2 |
| SUT IP Addr |  |  |  |  |  |  |  |  |  |  |  |
| From | 101.1.1.1 | 103.1.1.1 | 105.1.1.1 | 107.1.1.1 | 202.1.1.1 |  | 204.1.1.1 | 206.1.1.1 | 121.1.1.1 | 123.1.1.1 | 125.1.1.1 |
| To | 101.1.100.1 | 103.1.100.1 | 105.1.100.1 | 107.1.100.1 | 202.1.100.1 |  | 204.1.100.1 | 206.1.100.1 | 121.1.100.1 | 123.1.100.1 | 125.1.100.1 |
| Router Interface | FE 1/1 | FE 1/3 | FE 1/5 | FE 1/7 | FE 2/2 |  | FE 2/4 | FE 2/6 | FE 12/1 | FE 12/3 | FE 12/5 |
|  | Controller 1 : 141.184.123.48 |  |  |  |  |  | Controller 2: 141.184.123.47 |  |  |  |  |