- N2X IPTV QoE Test Solution

Lab Exercise #2: IPTV/MSTV QoE Measurements Using N2X, in Conjunction with IGMPoPPP and “IGMPoDHCP”

- Test Objective

Building upon the foundations of lab 1, this exercise shows how to use N2X without the IPTV QoE Productivity Application to make MDI quality of experience measurements on simulated subscribers. This approach is suitable when dynamic subscriber behavior (e.g., channel changing) and channel zapping measurements are not required. The benefits are that it scales to higher levels than the PA-based approach and it does not require the PA license. It is also somewhat simpler in that it only involves a single software product: the main N2X application.

The purpose of this test is to assess quality of experience for video subscribers connecting to the network using PPPoE or DHCP. The performance of these access protocols is not the focus of the test. Instead, MDI measurements are used to determine if video quality of experience is adequate when the traffic is delivered over PPP or when DHCP-assigned addresses are used. This lab also shows how N2X can be used to make MDI measurements on traffic delivered from a third party source, which could be a video server or a real Microsoft TV architecture.

- Target Customer:

This test is relevant to engineers building and testing access network equipment, such as a B-RAS, Access Node (DSLAM or PON OLT), Edge Router, Layer 2 switch, BNG, or MSPP, which will be deployed in an IPTV infrastructure. Target NEM groups include: ‘IPTV dev test’, ‘system test’ and ‘POC labs’. Target CSP groups include ‘service validation’ and ‘pre-deployment test’.

- Key Features to Highlight

- Highly scalable per-subscriber video quality metrics

- IGMPoPPP/DHCP support

- Microsoft TV support (MDI and protocol decodes)

- 10 GbE video subscriber/server (rel 6.9) support

Competitive Note
IneoQuest can’t support IGMPoPPP or IGMPoDHCP, which means they can’t compete at CSPs and NEMs that require this type of access architecture.

Competitive Note
No competitor supports Microsoft TV completely. Our solution, though limited, is first-to-market and delivers some of the diagnostic value our customers desperately need.

Competitive Note
IXIA relies on a CPU-based solution (IxLoad) for their IPTV testing, which is not as reliable and deterministic as N2X’s hardware-based measurement engine.  It is also limited to 200Mb/s generation and analysis of real video traffic, which is only 2% of a 10G pipe!

- Equipment Required

- N2X Equipment

- Two N5551A/B “Tahu” Tri-rate Ethernet ports

- N2X Software and Slides

All of the N2X software and materials to perform this demo are listed below, and can be downloaded from the IPTV QoE Test Solution – AE Resources Web Page:
<<URL to be provided>>

- E7881A/B Packets and Protocols License

  - N2X System Release 6.7 or later

- E7888A Access Protocols Software License (for PPP)

- E7887A DHCP Protocol Software License

- E7877A Video Generation and Analysis License

- E7828A IGMP License (for release 6.9 and later only)

- Device Under Test

This particular test assumes that the DUT is acting as an IPv4 multicast (IGMP) replicator. The device must have Ethernet interfaces (Ethernet, Fast Ethernet or GbE), either electrical or optical. Typical DUTs include B-RAS/BNGs, EARs and BSA/BSRs.

- Test Set-up

- Network Diagram: Testing against a real DUT

Video Source

Subscribers

1 VLAN,

with 100 subscribers

IGMP joins from 100 subscribers

Real video feeds to subscribers

Bi-directional data & VoIP traffic

N2X Port N2XVidPort

N2X Port N2XSubPort

DUTSubPort

DUTVidPort

- DUT Configuration:

Configure a single VLAN on DUTSubPort according to the addresses assigned to your group. Each VLAN represents a single subscriber. Create a single VLAN on DUTVidPort according to your assigned addresses.

- Lab Instructions

At the end of this lab, you will learn how to:

- setup link-layer communication

- configure IGMP host emulation

- configure IGMPoDHCP emulation

- configure IGMPoPPP emulation

- create multicast address group pools

- setup multicast traffic

- setup bi-directional background traffic

- monitor MDI statistics by setting up field statistics

- General guidelines for simulating video subscribers:

Choose a

Video Subscriber personality for a test port.

Simulate multiple IGMP hosts.

Define a multicast group pool.

Set up the IGMP hosts to join the multicast groups.

Start IGMP emulation.

Optional: Leave and Join different multicast groups.

Selecting a wrong personality for the test port will result in inability to get MDI statistics and capture video traffic.

This lab introduces variations of IGMP hosts: IGMPoDHCP and IGMPoPPP.

Setting up subscribers to join the multicast groups represents the broadcast TV and Pay-Per-View channels.

The test ports begin sending the IGMP messages to simulate subscribers switching to multicast channels.

This optional step simulates channel-changing subscribers by leaving and joining different multicast groups.

- Selecting Ports

- Click Ports from the main window top tool bar. In Port Selection dialog, select one test port, change its type to “Video Server”. Select another test port. Change its type to “Video Subscriber”. Click Ok. (See the Appendix for the Port assignment for your group)

Note: These ports can send regular traffic as well, but will be limited due to the video load on them.

- Figure 1: Port and Port Type Selection

- Click Physical Layer from the main window side tool bar.

- Configure the ports selected

- In the Physical Configuration dialog, change the Media Type to SPF if using optical connection, or RJ45 if using copper wire.

Note: For SPF, Step 5 and 6 must be performed. For other media type, skip step 5 and 6.

- Click Turn All Laser Off

- Click Turn ALL Laser On

- Close physical layer configuration

- Resolving Link Layer

Note: To enable a test port to establish a communication link with its connected SUT interface, you must configure its link framing and addressing. There are two ways to do so.  For non-GPF protocols like BGP-4, RSVP, IGMP*, and PPPoX*, the link layer configuration and address discovery is through the Link dialog box. You must manually initiate ARP/NDP to resolve addresses. For GPF protocols like DHCP (v4 and v6), MLD, LACP, and MPLS OAM, the link layer configuration is through the <Emulated Devices> Properties dialog box. See N2X Help for details on GPF protocols.

*IGMP and PPPoE becomes part of GPF protocols in 6.9 releases. The link layer configuration is done through the <Emulated Devices> Properties dialog. Address discovery prior to 6.9 needs to be done in the Link dialog box.

For practice purpose, the following procedures describe the link layer setup though the Link dialog box.

- From the Packets and Protocols window: Setup bar, click Link Layer. The Link dialog box is displayed.

- Click the Ethernet tab.

- Click LAN/VLAN Addresses. The LAN/VLAN IPv4 Addresses dialog box is displayed.

- In the display area, select the LAN or VLAN address pool listed under a tester row. Click Edit.

- Figure 2 Link Layer Configuration

- Enable VLAN ID 1 with value being 1.

- Configure SUT IP Address. (See the Appendix for the Tester and SUT IP address for your group) Click OK.

- Repeat step 11-13 for another test port.

- On the LAN/VLAN IPv4 Addresses dialog box, click Send All ARP Requests.

Note: Green indicators suggest successful link layer resolution.

- Setting up Multicast Traffic

- From the Packets and Protocols window: Setup bar, click Video.

- In the Setup – Video area, remove the default Video Source.   >

- From the Setup – Video button bar, click New Video Source. A Video Source row is added.

Note: If you have configured the link layer correctly, the IP address for the video source should be your assigned Tester IP Address for the Video Source Port.

- Select the Video Source row and click on New Channel Pool from the Setup – Video   button bar. A channel pool row is added. Double-click on the channel pool row to bring up the <Channel Pool Name> Properties dialog box.

- Under the Channels tab, enter the Name, Channels, and Video Clip Name.

- Name: e.g. IPTV1

- Channels: 10

- Video Clip: Click Browse, and select Animals_VOD_NTSC_H264_2.1Mbps_06MB.ts.

- Click the Fields tab.

- Figure 3 Configuring Video Source Properties

- Check that VLAN tag is enabled with VLAN ID being 1.

- In the Layer 3 > IPv4 Destination Address field, select Multicast to simulate a broadcast TV or Pay-Per-View channel sent to multiple TVs at the same time. Type the first multicast address assigned to your group. For example, group 1 will enter 225.1.0.10.

- To generate 10 unique addresses for all channels in the pool, click Field Modifier > Increment. Change the Prefix length (bits) from 24 to 32.

- Defining a Multicast Group Pool

A multicast group pool allows you to simulate many multicast groups so that you can send traffic to many different multicast groups at once. In this exercise, it is also used for the subscribers (IGMP hosts and their variations) to identify which multicast groups to join.

- From the Packets and Protocols window: Setup bar, click Emulation.

- From the Packets and Protocols window: Menu bar, click Actions > View Multicast Group Pools.

- Select an IPv4 row and click Add to create a new multicast group pool.

- Enter the Group Name, First Address, Num Addresses, and Modifier values.

- Group Name: E.g. Group1_Multicast_Pool

- First Address: Each group is assigned 10 multicast addresses. Enter the first address from your group’s assigned multicast address range. For example, 225.1.0.10 for Group 1

- Num Addresses: 10

- Modifier: 1/32

- Figure 4 Multicast Group Pools Setup

- Creating IGMP Sessions

The IGMP Emulation Software simulates multicast subscribers in the lab, subjecting routers to real-world requests to join and leave multicast groups.

- A) IGMP Host Emulation Setup

- From the Packets and Protocols window: Setup bar, click Emulation.

- In the Setup – Emulation area, select the Video Subscriber port row, and click New from the Setup – Emulation button bar.

- Create a group of 100 IGMP host emulations. Click OK. The IGMP Host Properties dialog is displayed.

- Figure 5  Creating IGMP Hosts

- Click <Click to Add Item> under Ethernet > VLANs. Enter VLAN ID = 1.

Note: You can configure stacked VLANs by clicking <Click to Add Item> again.

- Configure the Tester IPv4 fields:

- From: the first video subscriber tester IP address assigned to your group.                                         For example, group 1: 101.1.1.2

- To: the first video subscriber tester IP addressed incremented by 100 with a prefix of 32.                                            For example, group 1: 101.1.1.101

- Prefix length: 32 bits

- Configure the SUT IPv4 fields.

- Enter the first SUT IP address assigned to your group. For example, group 1: 101.1.1.1.

Note: In releases 6.7 onwards, GPF protocols provide automatic SUT address discovery: anytime N2X encounters a SUT IP address and VLAN combination that does not have an associated MAC address, N2X sends an ARP or NDP message to discover the SUT MAC address.

- Figure 6  IGMP Host Properties – VLAN and IP Addresses Configuration

- Click IGMP Host tab.

- IGMP version fields : IGMPv2

- From the button bar, click Group Membership. The Group Membership Profiles dialog box is displayed.

Note:  This dialog box is used to define the multicast groups to be joined and simulate subscribers joining and leaving multicast groups. For IGMPv3 sessions, you can also use it to define the sources that are (not) allowed to send traffic to multicast groups.

- Under the Group Membership Profiles tab, click Add. Add Multicast Group Pools dialog box is displayed.

- Enable the multicast group pool that you created from step 28. Click OK.

- Figure 7 Creating Multicast Group Membership for an IGMP Host

- B) IGMPoDHCP Emulation Setup

- Repeat step 29 and 30 to create emulations.

- Create a group of 100 IGMP Host with DHCP Client emulations. Click OK. The IGMP Host with DHCP Client Properties dialog is displayed.

- Click <Click to Add Item> under Ethernet > VLANs. Enter VLAN ID = 1.

- (Optional) Click DHCP Client tab. Configure the properties of DHCP client.

- Repeat step 36 to 40 to configure IGMP host fields and multicast group membership.

- C) IGMPoPPP Emulation Setup

- Repeat step 29 and 30 to create emulations.

- Create a group of 100 IGMP Host over PPP Client emulations. Click OK. The IGMP Host with PPP Client Properties dialog is displayed.

- Click <Click to Add Item> under Ethernet > VLANs. Enter VLAN ID = 1.

- Click PPPoE tab.

- Configure IPCP fields.

- From: the first video subscriber tester IP address assigned to your group.                                         For example, group 1: 101.1.1.2

- To: the first video subscriber tester IP addressed incremented by 100 with a prefix of 32.                                            For example, group 1: 101.1.1.101

- Prefix length: 32 bits

- Repeat step 36 to 40 to configure IGMP host fields and multicast group membership.

- Figure 8 Configuring PPPoE Properties

- Creating Background Traffic

You can create some traffic stream groups to simulate a real-world scenario in which video traffic is sent alongside Internet traffic like voice and data. The following procedures describe setup for bi-directional traffic.

- From the Packets and Protocol windows: Setup bar, click Traffic.

- In the Setup – Traffic area, select the Video Server port row.

- Click on New > IPv4 Mesh to create traffic mesh. The TrafficMesh Properties dialog box is displayed.

- Figure 9 Creating an IPv4 Traffic Mesh

Note: A traffic mesh controls the distribution and orientation of the traffic across multiple stream groups. It also defines how traffic is routed from sources, through the SUT, to destinations.

- Distribution: Partial mesh. (This sends traffic from each source to all destinations.)

- Orientation: Bidirectional. (This sends traffic sources to destinations, and also from destinations to sources.)

- Figure 10 Defining the Distribution and Orientation for a Traffic Mesh

- Click Sources and Destination tab.

- Select Video Server Port > Add Source.

- Select Video Subscriber > Add Destination.

- Figure 11 Defining Sources and Destinations for a Traffic Mesh

- Observe that a stream group is added to your Video Server port.

- Figure 12  A Traffic Stream Group Added to the Video Server Port

- Observe that a stream group is added to your Video Subscriber port.

- Figure 13 A Traffic Stream Group Added to the Video Subscriber Port

Note: If you have resolved the Link Layer correctly, the L3 Source and Destination IPv4 addresses are default to the first Tester IP address for both the Video Server and the Video Subscriber ports.

- Double-click on the traffic stream group for the Video Server port. The TrafficMesh Properties dialog is displayed.

- Check the VLAN configuration is correct. (VLAN ID is 1 in this lab exercise)

- Check the Source and Destination addresses are correct. (The source address should be the first Tester IP address for your Video Server port. The destination address should be the first Tester IP address for your Video Subscriber port.)

- (Optional) Configure the PDU builder to create data-like or voice-like contents.

- Repeat step 62 to 65 for the Video Subscriber port.

- Setting up Field Statistics

Note: In this lab, we would want to monitor MDI statistics based on the channels (multicast addresses).

- From N2X Packets and Protocols main window: Results - Realtime, click Setup from the button bar. Setup Measurements dialog box is displayed.

- Under the Measurements tab: Predefined selection combo box > MDI.

- Figure 14 Selecting MDI Statistics

- Click Ports tab.

- Select the Video Subscriber port row. Click  .  <Port> Measurement Properties dialog box is displayed.

- Collect measurements based on Field values.

- From the Field dropdown list, select ipv4_destination.

- Enter 225.1.0.0 as the Base value. Click OK.

- Figure 15 Setting up Field Statistics

- Click Fields tab.

- Highlight multicast addresses that you are interested in monitoring. Right click > Add to Measurements Table.

- Figure 16 Adding Entries to the Measurements Table

- Click Fields tab.

- Highlight multicast addresses that you are interested in monitoring. Right click > Add to Measurements Table.

- Enabling Emulations and Running the Test

- A) To Enable IGMP Host Emulation

- Enable IGMP Host Emulation.

- In the Setup – Emulation area, click Actions-IGMP Host from the button bar. Click Join All.

Note: You must join the multicast groups in order to receive the multicast traffic. Join and Leave can be done while the traffic is running.

- Figure 17 Enabling IGMP Host Emulation

- Click  to start traffic.

- Observe measurements in the Results – Realtime area. All the MDI statistics are collected.

Note: *very important* The MDI statistics MUST be in the cumulative mode. Make sure you enable  from the button bar.

- Figure 18 MDI Measurements

- Stop traffic.

- Go back to the Setup – Emulation area, click Actions-IGMP Host from the button bar. Click Leave All.

- Disable IGMP Host Emulation.

- To Enable IGMP Host with DHCP Client Emulation

- Figure 19 Starting DHCP Services

- Enable IGMP Host with DHCP Client Emulation.

- In the Setup – Emulation area, click Actions > DHCP Client > Start.

Note: The state of the emulations will change from Init to Bounded. A bounded state indicates successful IP addresses allocation.

- In the Setup – Emulation area, click Actions > IGMP Host > Join All.

- Repeat step 80 to 82 to start traffic and observe measurements.

- Go back to the Setup – Emulation area, click Actions > IGMP Host > Leave All.

- Click Actions > DHCP Client > Release.

- Disable IGMP Host with DHCP Client Emulation.

- To Enable IGMP Host with PPPoE Client Emulation

- Figure 20 Starting PPPoE Services

- Enable IGMP Host with PPPoE Client Emulation.

- In the Setup – Emulation area, click Actions > PPPoE Client > Open Connections.

Note: The state of the emulations will change from Discovery to Established. A Established state indicates successful IP addresses allocation.

- In the Setup – Emulation area, click Actions > IGMP Host > Join All.

- Repeat step 80 to 82 to start traffic and observe measurements.

- Go back to the Setup – Emulation area, click Actions > IGMP Host > Leave All.

- Click Actions > PPPoE Client > Close Connections.

- Disable IGMP Host with PPPoE Client Emulation.

- Showing the MSTV Traffic Protocol Decodes

Microsoft TV uses a proprietary protocol encapsulation to transmit video traffic over RTP.  It uses a form of RFC2250 with Microsoft-specific RTP extensions.  N2X can decode this traffic by capturing the data and then using the Flexible Decoder to examine the packets.  A XML file is required in order to decode the traffic, and it is important that this XML file is only given to customers who are valid MSTV licensees (i.e. they are customers or partners of Microsoft’s for MSTV).  We sell this XML decode file as N5572A.

This section of the lab shows how to use the N5572A to decode this MSTV traffic.

Figure 21: the basic idea of RFC 2250; take a transport stream delivered in UDP and wrap it in RTP, using the RTP extension data to ameliorate problems associated with IP video delivery.  The transport packets labeled as lime green in the RTP stream are colored to indicate that some packets will be introduced to the stream as part of the stream packing.  ECM packets are newly-created by the Acquisition Server, while the PSI stream is reconstructed from the original.

99.  Capture some MSTV traffic.  Since we do not have an MSTV server in our lab or any source of real MSTV traffic, we will provide a sample capture file for this demo.

- Click Capture. In the Capture window, select the Video Subscriber port.

- Open a capture file.  Select mstv-a-server.cap.  This is a file we captured from a real MSTV network at Alcatel.

  - Click “Decode”.

- In the Packet Decode window, select the Flexible Decoder mode.

- Examine the UDP header in the captured data, and determine which UDP Destination Port number contains the MSTV traffic.  Make a note of this number.

- Go to the directory C:\Program Files\Agilent\N2X\RouterTester900\UserData\Protocols.

- Edit the file AgtPdu_RTPMSTV.xml.  NOTE: for this lab, since multiple groups are sharing controllers, we will not be able to actually change any values in the XML file.

- In the first line of XML code: <enum value=”7523” name=”N2X: RTP for MSTV”…, change this value to the one noted in step 102.

NOTE: for this lab, since multiple groups are sharing controllers, we will not be able to actually change any values in the XML file.

- Save the file.

- Exit and restart N2X Packets and Protocols (this reloads the XML file).

- Repeat steps 100-102.

- Examine packet # 7.  Note the RTP MSTV header, and the Extension Bit which signifies the presence of this MSTV extension.

- Examine the 20-byte MSTV IPTV Header Extension.

- Notice the NTP (Network Time Protocol) timestamps.  These are used in MSTV for timing and synchronization.  Now that you know this information, we will have to kill you ;)  Bill Gates is listening.

- Examine packet #32.  Notice the field “PUSI contained in datagram”.  PUSI means “Payload Unit Start Indicator”.  This indicates that this packet contains the first packet of a full frame of video.  If the Random Acccess Point (RAP) bit were set, it would indicate that this packet contained an I-frame, or the start of a video sequence!

Advanced info for keeners! ##### PUSI bit When DVR systems are generating index files, efficiency can be gained from indicating how long the I-frame for each random-access point is so that reads of data from the disk system can be optimized. The Acquisition Server/repacketizer indicates in the RTP header whether the given RTP packet contains an MPEG transport packet with the PUSI bit set for the pid which is carrying the master stream. This allows client-side DVR to be performed without examining the transport stream at all. ##### Random Access Point Random access points are signaled using a single bit in the main IPTV extension. This is used by two components downstream from the Acquisition Server. First, the DServer uses this indicator to determine which points are valid ICC start locations without needing to parse the stream and examine the ECMs. Second, the client DVR subsystem uses this indicator to be able to detect random-access points for its index file creation without needing to parse the stream. When this bit is set, the MPEG transport packets starting from the first one contained within the RTP packet are assumed to be acceptable both as an instant channel change start location and as a frame to be displayed during trick modes. While the Acquisition Server is processing the incoming stream, if it detects that the next MPEG transport packet in the stream indicates the beginning of a random-access point, it will close out the previous RTP packet and begin a new packet, as well as creating the required PAT, PMT, and ECM packets to enable immediate use of the packets upon reception.

- Now look under MP2T Header in the same packet.  You should see 7 MPEG-2 packets contained in this one RTP/UDP packet.  Most IPTV networks, including MSTV, pack 7 MPEG-2 TS packets (188 bytes each) into a single UDP or RTP packet.

- Smile and take a deep breath.

- Close the Packet Decode Window.  We are so done with that.

- Select the mstv-a-server.cap file again.  Click on “View Video”.

- Watch the video clip!  We can demultiplex and view captured MSTV video traffic too.  Of course, the encryption has to be turned off.  Bill would never give us the keys to that.

- Pat yourself and your lab partner(s) on the back, go get a coffee or take a break.

- Appendix

- CONTROLLERS

  - Group 1 – 5 Controller1  DELL 1425
Controller Address: 141.184.123.48
User Logins: N2xUser1, N2xUser2, N2xUser3, N2xUser4, N2xUser5
Password: Agilent

  - Group 6 – 10 Controller2 DELL 1425
Controller Address: 141.184.123.47
User Logins: N2xUser1, N2xUser2, N2xUser3, N2xUser4, N2xUser5
Password: Agilent

|  | Group1 | Group2 | Group3 | Group4 | Group5 |  | Group6 | Group7 | Group8 | Group9 | Group10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Video Source Port | 101/1 | 101/3 | 102/2 | 103/1 | 104/1 |  | 104/1 | 201/1 | 101/2 | 102/1 | 102/3 |
| Connector | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |  | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |
| Tester IP Addr | 100.1.1.2 | 102.1.1.2 | 104.1.1.2 | 106.1.1.2 | 201.1.1.2 |  | 203.1.1.2 | 205.1.1.2 | 120.1.1.2 | 122.1.1.2 | 124.1.1.2 |
| SUT IP Addr | 100.1.1.1 | 102.1.1.1 | 104.1.1.1 | 106.1.1.1 | 201.1.1.1 |  | 203.1.1.1 | 205.1.1.1 | 120.1.1.1 | 122.1.1.1 | 124.1.1.1 |
| Router Interface | FE 1/0 | FE 1/2 | FE 1/4 | FE 1/6 | FE 2/1 |  | FE 2/3 | FE 2/5 | FE 12/0 | FE 12/2 | FE 12/4 |
| Multicast Addr Range |  |  |  |  |  |  |  |  |  |  |  |
| From | 225.1.0.10 | 225.1.0.20 | 225.1.0.30 | 225.1.0.40 | 225.1.0.50 |  | 225.1.0.60 | 225.1.0.70 | 225.1.0.80 | 225.1.0.90 | 225.1.0.100 |
| To | 225.1.0.19 | 225.1.0.29 | 225.1.0.39 | 225.1.0.49 | 225.1.0.59 |  | 225.1.0.69 | 225.1.0.79 | 225.1.0.89 | 225.1.0.99 | 225.1.0.109 |
| Subscriber Port | 101/2 | 102/1 | 102/3 | 103/2 | 104/2 |  | 104/2 | 201/2 | 101/3 | 102/2 | 103/1 |
| Connector | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |  | RJ45 | RJ45 | RJ45 | RJ45 | RJ45 |
| Tester IP Addr |  |  |  |  |  |  |  |  |  |  |  |
| From | 101.1.1.2 | 103.1.1.2 | 105.1.1.2 | 107.1.1.2 | 202.1.1.2 |  | 204.1.1.2 | 206.1.1.2 | 121.1.1.2 | 123.1.1.2 | 125.1.1.2 |
| To | 101.1.100.2 | 103.1.100.2 | 105.1.100.2 | 107.1.100.2 | 202.1.100.2 |  | 204.1.100.2 | 206.1.100.2 | 121.1.100.2 | 123.1.100.2 | 125.1.100.2 |
| SUT IP Addr |  |  |  |  |  |  |  |  |  |  |  |
| From | 101.1.1.1 | 103.1.1.1 | 105.1.1.1 | 107.1.1.1 | 202.1.1.1 |  | 204.1.1.1 | 206.1.1.1 | 121.1.1.1 | 123.1.1.1 | 125.1.1.1 |
| To | 101.1.100.1 | 103.1.100.1 | 105.1.100.1 | 107.1.100.1 | 202.1.100.1 |  | 204.1.100.1 | 206.1.100.1 | 121.1.100.1 | 123.1.100.1 | 125.1.100.1 |
| Router Interface | FE 1/1 | FE 1/3 | FE 1/5 | FE 1/7 | FE 2/2 |  | FE 2/4 | FE 2/6 | FE 12/1 | FE 12/3 | FE 12/5 |
|  | Controller 1 : 141.184.123.48 |  |  |  |  |  | Controller 2: 141.184.123.47 |  |  |  |  |

This format is documented in a Microsoft Confidential document titled “Microsoft IPTV carriage of MPEG transport: RFC 2250, IPTV extensions, Applications for Ad Splicing”, Geoffrey Smith, March 24, 2006.