N2X Integrated L2-7 Solution/Stateful TCP Lab Exercises

Lab Exercise #3:  Triple-Play/Application Policy Test

Todd Law/Ryan Crawford

Version 2

May 16, 2007

Test Objective

Description:  This lab is to show stateful TCP in a basic triple-play usage scenario.  The lab extends concepts introduced in the previous two labs. In the lab, the AE will configure Triple play traffic to be received by a subscriber port. The subscriber egress queue will prioritize application traffic so that revenue-generating applications, such as IPTV and VOIP, will have priority over low-priority applications like P2P (peer-to-peer) and HTTP.

The AE will over-subscribe two ingress ports using stateful TCP traffic and simulated IPTV and VoIP traffic to stress the device under test’s ability to correctly enforce application policy.

Target Customer:

The target DUT is almost any kind of forwarding device (routers, switches, BRASs/BNGs).  The customer is anyone testing such devices at a NEM or SP.

Key Features to Highlight

- Integration of stateful and stateless traffic

- Existing triple-play functionality on N2X

- Ease of use in adjusting bandwidth load with slider bar

Equipment Required

N2X Equipment

Equipment:  Three Tahu (N5551A/B) ports.  Three ports of DUT

Other Equipment

DUT:  3 x 10/100 ports of DUT (Cisco ISR 2800)

N2X Software and Slides

Example:

All of the N2X software and materials to perform this demo are listed below, and can be downloaded from [URL to be provided].  Those items are:

[R&D to place all of these documents on a single web-page and provide links to each item]

N2X 6.9.1.20

Software Licenses for N5575A & N5576A

Lab Exercises (this document)

Pre-study material

Lab Instructions

Test Set-up

Network Diagram

Connect the N2X to the DUT as shown below.

regional

concentrator

Internet

Data

N2X Test Port B

regional

concentrator

Internet

Data

100bE

N2X Test Port C

Router

100bE

100bE

Web Clients

N2X Test Port A

Web Servers

Device Under Test

The DUT is a Juniper ERX-1440.  You will use three 10/100 ports for this test.

Port Setup

- Connect to the controller via Windows Terminal Services:  Start/All Programs/Accessories/Communications/Remote Desktop Connection; use the IP address given to you by your instructor.

- Clear any previous N2X sessions:  Start/All Programs/Accessories/Communications/Command Prompt; n2xcleanup

- Launch the N2X application:  Start/All Programs/Agilent N2X/N2X Packets 6.9 Internal Release/RouterTester900

- Select three test ports:

Figure 1:  Selecting Ports

- For all selected ports, choose the personality:  Stateful TCP & Application Data; Click “OK”

Figure 2:  Choosing the Stateful TCP personality

Setting up link layer

- Setup the link-layer according to your group’s assigned IP Addresses. (note: wiill be provided in the lab)

Setting up Stateful Traffic

- Click on the “Traffic” icon, then click on the first port as shown below

Figure 3:  Setting up Stateful Traffic

- Create a connection group by double-clicking under Stateful Connections.

- Double-click on the connection group to configure its parameters (see Figure 5).

Figure 4:  Connection Group

- Define two connection groups with the following sets of parameters (HINT:  Use Copy/Paste to accelerate this step or the “CreateTCPTraffic” Quicktest):

  - Connection Group 1

    - Re-Name: HTTP Reference Port2-Port1

    - Application Type:  HTTP v1.1 GET data transfer

    - Size of Bulk Data Transfer:  100000 bytes

    - Client Test Port:  A

    - Server Test Port:  B

    - Number of clients:  253

    - Number of servers:  1

    - Client distribution:  One Server for all Clients

    - Client Max Load:  10%

    - Server Max Load: 10%

  - Connection Group 2

    - Re-Name: Bronze – BitTorrent Port2-Port1

    - Application Type:  BitTorrent Data transfer

    - Size of Bulk Data Transfer:  100000 bytes

    - Client Test Port:  A

    - Server Test Port:  B

    - Number of clients:  253

    - Number of servers:  253

    - Client distribution:  Separate Server for each Client

    - Client Max Load:  10%

    - Server Max Load:  10%

- Update the MAC, IP, addresses for both the Client and Server addresses for each Connection Group, based on the Link Layer configuration. (note: not necessary if the “CreateTCPTraffic” Quicktest is used.)

Integration of stateful and stateless traffic is a key differentiator for N2X’s stateful TCP solution.

- Use the N2X stateless traffic model to set up traffic streams as follows:

  - Stream Group 1 (simulated Video Broadcast)

    - Re-Name: Silver – Video – Port2-Port1

    - Source Port:  C

    - Destination Port: A

    - Load:  10%

    - Encapsulation:  RTP/UDP/IP

    - Priority Type:  DiffServ/PHB:  Assured Forwarding/ Class1, Low Drop Precedence (10)

  - Stream Group 2 (simulated VoIP traffic downstream)

    - Re-Name: Gold  – Voice – Port2-Port1

    - Source Port :C

    - Destination Port: A

    - Load:  10%

    - Encapsulation:  RTP/UDP/IP

    - Priority Type: DiffServ/PHB:  Expedited Forwarding/46

Note: You did not insert any Video or Voice payloads into the data. Why? So we can insert the test payload in the TX traffic.

Setting up Measurements

- Click on Setup in the statistics area to open the statistics setup dialogue. Choose the following Measurements and them to the Table and Graphs.

- Select the ports tab and un-select the following port stats:

- Select the following Stream Stats:

- Select the following Connection Groups Stats: (note: Only Select the Client side Stats)

Starting Traffic

- In the main dialogue, click on the Traffic button to start traffic.

- Use the Slider bar to set the Tx Load for Port B (the Stateful Traffic Port) to 50%

- Use the Slider bar to set the Tx Load for Port C (the Stateless Traffic Port) to 50%

- Observe the statistics….Notice that all traffic should be received completely.

- Use the Slider bar to set the Tx Load for Port B to 80%

- Observe that the Server Tx Load for Port B will try to reach 80%; however, it doesn’t, Why?

- Take a look at the statistics for simulated VoIP and IPTV traffic to see that packets are getting through and with no loss.

- Use the Slider bar to set the Tx Load for Port C (the Stateless Traffic Port) to 80%. What do you see? How does TCP Traffic respond to the overload condition?

- Use the Slider bar to set the Tx Load for Port B to 20%. What do you see? How are the Video and Voice Traffic Latency, and Packet Loss affected over the test run?