N2X Integrated L2-7 Solution/Stateful TCP Lab Exercises

Lab Exercise #1: Lab 1:  Layer-4 Forwarding Performance

Todd Law

Version 1

April 13, 2007

Test Objective

Description:  This lab is the most basic demonstration of the stateful TCP functionality.  The test will be performed back-to-back (without DUT) to demonstrate N2X’s ability to generate and measure stateful traffic with re-transmissions.  The AE will be able to quickly establish 10s of thousands of TCP connections and rapidly load the GbE interface with stateful traffic.  A QuickTest special tool will be used to cause packet loss to demonstrate TCP re-transmissions and the effects of packet loss on goodput.

Target Customer:

The target DUT is almost any kind of forwarding device (routers, switches, BRASs/BNGs) and also some security devices such as IPSs/IDSs (Intrusion Prevention/Detection Systems), but NOT Firewalls.  The customer is anyone testing such devices at a NEM or SP.

Key Features to Highlight

- Ability to create 1Gbit/s of stateful TCP traffic

- Ease of use in setting up TCP connections

- Ease of use in adjusting bandwidth load with slider bar

Equipment Required

N2X Equipment

Equipment:  Two Tahu (N5551A/B) ports, chassis, controller

Other Equipment

Equipment:  Ethernet cable

N2X Software and Slides

Example:

All of the N2X software and materials to perform this demo are listed below, and can be downloaded from [URL to be provided].  Those items are:

[R&D to place all of these documents on a single web-page and provide links to each item]

N2X 6.7.1.17

Software Licenses for N5575A & N5576A

Packet Loss/Delay injection QuickTest

Lab Exercises (this document)

Pre-study material 1

Lab Instructions

Test Set-up

Network Diagram:  Testing back-to-back

Connect any two ports back-to-back using an Ethernet cable:

regional

concentrator

Internet

Data

TCP Servers

N2X Test Port 1

TCP Clients

N2X Test Port 2

BNG

or other TCP-capable router

GbE

GbE

Device Under Test

The DUT for this test is simply an Ethernet cable to connect back to back.

Port Setup

- Connect to the controller via Windows Terminal Services:  Start/All Programs/Accessories/Communications/Remote Desktop Connection; use the IP address given to you by your instructor.

- Clear any previous N2X sessions:  Start/All Programs/Accessories/Communications/Command Prompt; n2xcleanup

- Launch the N2X application:  Start/All Programs/Agilent N2X/N2X Packets 6.9 Internal Release/RouterTester900

- Select two test ports:

Figure 1:  Selecting Ports

- For all selected ports, choose the personality:  Stateful TCP & Application Data; Click “OK”

Figure 2:  Choosing the Stateful TCP Personality

Setting up Stateful Traffic

- Click on the “Traffic” icon, then click on the first port as shown below

No competitor has integrated stateful and stateless traffic like N2X.

Figure 3:  Setting up Stateful Traffic

- Create a connection group by double-clicking under Stateful Connections.

- Double-click on the connection group to configure its parameters.

Figure 4:  Connection Group

- Set the general parameters of the connection group as follows:

Figure 5:  Modifying a Connection Group's General Parameters

- Set the Client Address Parameters as follows

Figure 6:  Modifying a Connection Group’s Client Addresses

- Set the Server Address Parameters as follows

Figure 7:  Modifying a Connection Group's Server Addresses

- Set the Advanced Parameters as follows:

Figure 8:  Modifying a Connection Group's Advanced Parameters

- Click OK to apply and close the dialogue.

Setting up Measurements

- Click on Setup in the statistics area to open the statistics setup dialogue.

Figure 9:  Setting up Measurements

- In the All view of Setup Measurements, de-select all the statistics that are set by default.  These do not matter for this test.  Then, click on TCP.

Figure 10:  Turning off N2X Stream Statistics

- In the TCP view, select the following parameters to

Competitive N2X’s TCP statistics are more complete and precise (10ns accuracy) than any competitor.

Figure 11:  Selecting TCP-specific statistics

- Click on the Connection Groups tab

Figure 12:  Viewing statistics by connection group

- Without closing the Setup Measurements click on the Traffic button in the main dialogue window to start stateful traffic.

Figure 13:  Starting Traffic

- Examine the statistics in the Setup Measurements dialogue.

Figure 14:  Viewing live measurements

- Close the Setup Measurements dialogue box.

- In the main window, click on the bottom (server) port, then click on the connection group line.

Figure 15:  Selecting ports and connection group for bandwidth adjustment (slider bar)

Demonstrating the Slider Bar

- Grab the slider bar and move it right to 100% load.

Figure 16:  Adjusting the slider bar

No competitor has a slider bar for stateful traffic.  This is a key differentiator to demonstrate.

No competitor can so easily create 100% bandwidth for stateful traffic.  Showing this is critical for any demo.

Introducing packet loss and showing the effects of re-transmissions

- Open the Applications dialogue and start the QuickTest labeled TcpCapture.

Figure 17:  Starting the TcpCapture QuickTest

- Make sure that port 1 is the same as port A in the QuickTest, then use the QuickTest to inject 5% Tx packet loss.

Figure 18:  Injecting packet loss with the QuickTest

- Go back to the main application and re-open the connection group dialogue, and turn off fast-re-transmit.  Note the change in statistics.

No competitor supports re-transmit at scale. The point of this part of the demonstration is that, if you are not testing with re-transmissions and at high bandwidth, your tests are not realistic or meaningful.  The Rx statistics will be quite different depending on whether this feature is turned on or off!

Figure 19:  Turning off fast re-transmit

- Close all dialogue boxes then clean up the session by running: Programs/Accessories/Communications/Command Prompt; n2xcleanup