N2X Integrated L2-7 Solution/Stateful TCP Lab Exercises

Lab Exercise #2:  Stateful TCP QoS Test

Todd Law/Ryan Crawford

Version 2

May 16, 2007

Test Objective

Description:  This lab focuses on how the stateful TCP solution can be used to test subscriber rate policy.  A practical example is given to test QoS levels agreements for different subscriber types.  An egress port will be configured to allow two different subscriber types, one with high speed access, and another with low speed access.

Background:  Service Providers have many options for implementing QoS – this example uses a combination of two mechanisms IP TOS bits, to achieve a simple example of how policy might be applied.

Target Customer:

The target DUT is almost any kind of forwarding device (routers, switches, BRASs/BNGs).  The customer is anyone testing such devices at a NEM or SP.

Key Features to Highlight

- TCP mechanisms like Fast Retransmit and Retransmit Timeout

Equipment Required

N2X Equipment

Equipment:  Three Tahu (N5551A/B) ports.  Three ports of DUT

Other Equipment

DUT:  3 x 10/100 ports of DUT (ERX-1440)

N2X Software and Slides

Example:

All of the N2X software and materials to perform this demo are listed below, and can be downloaded from [URL to be provided].  Those items are:

[R&D to place all of these documents on a single web-page and provide links to each item]

N2X 6.7.1.20

Software License for N5575A

Lab Exercises (this document)

Pre-study material

Lab Instructions

Test Set-up

Network Diagram

Connect the N2X to the DUT as shown below.

regional

concentrator

Internet

Data

TCP Servers

N2X Test Port B

TCP Clients

N2X Test Port A

BNG

or other TCP-capable router

GbE

GbE

Device Under Test

The DUT is a Juniper ERX-1440.  You will use three 10/100 ports for this test.

Port Setup

- Connect to the controller via Windows Terminal Services:  Start/All Programs/Accessories/Communications/Remote Desktop Connection; use the IP address given to you by your instructor.

- Clear any previous N2X sessions:  Start/All Programs/Accessories/Communications/Command Prompt; n2xcleanup

- Launch the N2X application:  Start/All Programs/Agilent N2X/N2X Packets 6.9 Internal Release/RouterTester900

- Select two test ports:

Figure 1:  Selecting Ports

- For all selected ports, choose the personality:  Stateful TCP & Application Data; Click “OK”

Figure 2:  Choosing the Stateful TCP personality

Setting up Stateful Traffic

- Click on the “Traffic” icon, then click on the first port as shown below

Figure 3:  Setting up Stateful Traffic

- Create a connection group by double-clicking under Stateful Connections.

- Double-click on the connection group to configure its parameters (see Figure 5).

Figure 4:  Connection Group

- Define two connection groups with the following sets of parameters (HINT:  Use Copy/Paste to accelerate this step or the “CreateTCPTraffic” Quicktest):

  - Connection Group 1

    - Re-Name: TCP Reference

    - Application Type:  TCP Open/Close with data transfer

    - Size of Bulk Data Transfer:  100000 bytes

    - Client Test Port:  A

    - Server Test Port:  B

    - Number of clients:  10

    - Number of servers:  10

    - Client distribution:  Separate Server for each Client

    - Client Max Load:  10%

    - Server Max Load: 10%

    - Server Addresses:

      - IPv4 priority type:  TOS

      - Precedence: Flash Override = 4

  - Connection Group 2

    - Re-Name: TCP Behavior Group

    - Application Type:  TCP Open/Close with data transfer

    - Size of Bulk Data Transfer:  100000 bytes

    - Client Test Port:  A

    - Server Test Port:  B

    - Number of clients:  10

    - Number of servers:  10

    - Client distribution:  Separate Server for each Client

    - Client Max Load:  10%

    - Server Max Load:  10%

    - Server Addresses:

      - IPv4 priority type:  TOS

      - Precedence: Network Control = 7

- Update the MAC, IP, addresses for both the Client and Server addresses for each Connection Group, based on the Link Layer configuration. (note: not necessary if the “CreateTCPTraffic” Quicktest is used.)

Setting up Measurements

- Click on Setup in the statistics area to open the statistics setup dialogue. Choose the following Measurements and them to the Table and Graphs.

- Select the ports tab and un-select the following port stats:

- Select the following Connection Groups Stats:

Starting Traffic

- In the main dialogue, click on the Traffic button to start traffic.

- Using the slider bar, ramp up the server load for each connection group to 20%.

- Observe the statistics….Notice that all traffic should be received completely. The router allows 20% load through without enforcing a bandwidth limit.

- Using the slider bar, ramp up the server load for “TCP Behavior Group” to 30%

- Observe that the “TCP Behavior Group” Server has TCP Timeouts, TCP Fast Retransmit Occurrences, and TCP Payload Octet Retransmitted

- Open the “TCP Behavior Group” Connection Group and disable Fast Retransmit.

- What do you see? How does Fast Retransmit affect TCP Goodput Octets?

- Open the “TCP Behavior Group” Connection Group and set Retransmit timeout to 100 ms.

- What do you see? How does Retransmit Timeout affect TCP Goodput Octets?