# MIMO 3x3 Demo Configuration Guide

This is a rough guide to setting up a 3x3 MIMO demo.  Refer to the 2x2 demo and config guides for more detail.

The configuration includes 3 VSGs in 1 chassis, and 3 VSAs in a second chassis.

VSGs in slots 2,6,15

Ref slot 10

Chassis 2: 3 VSAs

VSAs in slots 3,7,12

Ref slot 10

There are two ways to operate the two chassis:

- Using two separate controllers.  In this case, you will run the M9381A MIMO Demo Tool on one controller that is connected to the VSG chassis. On the second controller, you will run 89600, and you will need to manually route the triggers on the backplane for the VSA chassis.

- Using a single controller to control both chassis.  Follow the instructions from the web.  There is a chtml file that can be downloaded that describes several ways to connect multiple chassis.  Any of these methods can be used.  The best performance will be with a high performance desktop controller with a STAR configuration to the two chassis.

http://www.home.agilent.com/agilent/editorial.jspx?cc=US&lc=eng&ckey=2359963&nid=-11143.0.00&id=2359963

To set up VSGs

Run self test, save IVI config entries (no reference)

Before run self test, must route triggers on backplane of the VSG chassis:

Note: these will be different when running all VSGs simultaneously.  Demo program will take care of

VSA:  Run self tests on all 3 VSAs

To test 89600, route triggers on the VSA chassis:

Or use this CE script:

<<3ChanVSATrigRouting.iseq>>

/ No description yet.

private void Untitled() {

Agilent.AgM9018.Interop.IAgM9018 chassis = new Agilent.AgM9018.Interop.AgM9018Class();

chassis.Initialize("PXI15::0::0::INSTR", false, false, "");

chassis.TriggerBus.Connect(1, Agilent.AgM9018.Interop.AgM9018TrigBusEnum.AgM9018TrigBus1To2To3);

chassis.TriggerBus.Connect(2, Agilent.AgM9018.Interop.AgM9018TrigBusEnum.AgM9018TrigBus2To1);

chassis.TriggerBus.Connect(3, Agilent.AgM9018.Interop.AgM9018TrigBusEnum.AgM9018TrigBus3To2To1);

chassis.Close();

}

3 chan 80 MHz split from 1 M9381A

3 chan 80 MHz split from 1 MXG-B

Note the MIMO M9381A Demo Tool works for a 3x3 configuration.  Set Number of Channels to 3, and ensure the VISA resource strings match the hardware configuration.