[image: image1.png]A EE



Agilent OpenLAB CDS for LC Neophyte
Lab 15: Batch Review

LAB EXERCISE

LAB TITLE

Batch Review

Revision: 1.0
LAB OBJECTIVES

The purpose of this lab is to familiarize you with batch review features.

Batch Review

If you generate a large number of samples on a daily basis and would like a convenient way to view the results and check the quality of integration then Batch Review may help. Batch Review allows you to step through calibration standards, control samples, and unknown samples adjusting the integration parameters and reviewing the results. Batch files are the result of sequences.
1) In the Data Analysis view, select Batch, Load Batch. Navigate to the result set that you created during the sequence lab. Load the file with the extension .b.
2) The Setup Batch dialog box will load. Select the sequence method that contains your calibration table.
3) Select all the runs for processing. OK the panel.
4) Examine the integration for the first data file. If you would like to view a new data file every 10 seconds, press Start.

5) If you would like to change the integration events for a particular file, then Pause the batch review.

6) [image: image4.png]5



Use the Marked changes to current run for save with batch file tool to mark those changes you would like to keep. If you do not want to keep an integration change, select the discard tool. To make the changes permanent, you would also save the changes as part of the batch file.

7) Use the batch options to add a column to the Batch Table. Do the same for the Report Table. 
8) Select one of your calibrated compounds from the List of Compounds on the right hand side. Selected columns are listed in the Batch Table. Use the filter buttons
[image: image6.png].. Agilent Technologies
Innovating the HP Way





to display the columns that are specific for calibration runs.

9) Calibrate the batch by clicking on [image: image2.png]


.

10)  Analyze the batch by clicking on [image: image3.png][Startay]



. (You might want to reduce the processing pause in the batch options)
11)  Print or preview the report. Are all the selected columns displayed?

12)  Save the batch under a new name and exit the batch. 

13)  Check the difference between loading the original batch and the one that you just saved. 

� EMBED PBrush  ���









Page 2
2/9/2012

[image: image5.png]5



_938198151

