# Glossary

A  B  C  D  E  F  G  H  I  J  K  L  M  N  O  P  Q  R  S  T  U  V  W  X  Y  Z

AMR – Adaptive Multi-Rate

An audio data compression scheme, which is optimized for speech coding. The 3GPP adopted AMR as the standard speech codec.

B Frames

"B" frames are bidirectional prediction frames, and contain the part of the .jpg that changes between frames. These frames receive predictive information from the "P" frames.

DNA – Distributed Network Analyzer – DNA, DNA MX, and DNA PRO

An Agilent network test hardware platform for use with the Network Analyzer software and/or with the Signaling Analyzer software. The Triple Play Analyzer can capture data with the DNA and DNA PRO.

EIT – Event Information Table

A file that contains information about the MPEG2-TS transport stream to parse for the set-top box. Data stored in the EIT includes the transport stream ID, network ID, event and start times, and descriptor tags to name a few.

ES – Elementary Stream

An elementary stream contains audio or video data. Video encoders and audio encoders output an elementary stream, and the format of the elementary stream depends on the codec or data that is carried in the stream.

Entropy

The amount of true information in a signal or event, as compared to the redundant, nonessential information.

FEC – Forwarding Equivalence Class

A term used in Multi-Protocol Label Switching (MPLS) to describe a set of packets that have similar or identical characteristics, and which may be forwarded in the same way.

Frame

A unit of information in a Layer 2 protocol data unit in ATM. A frame consists of either a header or a trailer, which contains a frame alignment pattern, other fields for routing and operational data, and a payload that contains the data transported by the frame.

Gateway

A network node that is designed to interface with other networks thatuse a different set of protocols.

GOP – Group of Pictures

A layer in MPEG coding, between the sequence layer and the picture layer, which contains I, B, and P frames.

ICC – Instant Channel Change

IPTV technology that employs an ICC server, which sends bursts of unicast video traffic to the set-top box. ICC servers store seconds of video in a circular buffer to support instant channel change.

I Frame

"Intra" frames that contain the primary .jpg image for the frame.

IGMP – Internet Group Management Protocol

A network layer communications protocol used for online video. IGMP (Internet Group Management Protocol) is used to establish and manage the Internet Protocol multicast group memberships, and is an integral part of the IP multicast specification.

IPTV – Internet Protocol Television

An Internet delivery service architecture, which provides digital television content to computers using the Internet Protocol over a network.

IPv4 – Internet Protocol version 4

The Internet Protocol's fourth version, and the dominant internet protocol in the network layer of the OSI model. IPv4 is used for service request responses from the transport layer and service requests to the data link layer.

Jitter

Small or random variations in the time or phase of a signal, which can result in errors or synchronization loss. Jitter is typically a variation of one or more signal characteristics, such as in the interval between pulses, the amplitude of cycles, or the frequency or phase of cycles.

Join Latency

The time it takes for data to begin flowing after an application issues a command to join a multicast group.

Leave Latency

The time it takes for data to stop flowing after an application issues a command to leave a multicast group (applies to IGMPv2 and IGMPv3 only).

LIM – Line Interface Module

A removable physical interface module used with the Agilent DNA, DNA MX, and DNA PRO. Various LIM types exist, ranging from LAN Ethernet interfaces to WAN T1/E1, T3/E3, V-Series, and optical interfaces.

Lost Packets

The event when one or more data packets fail to reach their destination over a network, resulting in performance issues or jitter. Packet loss can be caused by various factors, including signal degradation, corrupted packets, or problems in network hardware.

MDI – Media Delivery Index – Delay Factor (MDI-DF) and Media Loss Rate (MDI-MLR)

A measurement for IPTV networks. MDI is represented mathematically using two components: the Delay Factor (DF), and the Media Loss Rate (MLR). The mathematical representation of MDI is DF:MLR, where DF refers to the variances of the IP packet inter-arrival time, and MLR is the number of transport stream packets lost over a specific time period.

MGCP  – Media Gateway Control Protocol

A protocol used internally within a system, which can appear externally as a single VoIP gateway. The system includes a calling participant, a media gateway, and a signaling gateway. This client-server protocol is used by telephony providers for subscriber control.

MOS – Mean Opinion Score (MOS)

A numerical indication of the perceived quality of media after it has been compressed, transmitted, and received. MOS is expressed as a single number in the range of 1 to 5, where 1 is lowest perceived quality, and 5 is the highest.

MPEG-1 – Moving Pictures Experts Group Standard 1

A group of audio and video coding and compression standards defined by the Moving Picture Experts Group (MPEG). The original MPEG-1 video standard for acceptable video quality was 1.5 Mbits/second with a 352 x 240 resolution.

MPEG-2 – Moving Pictures Experts Group Standard 2

A group of coding and compression standards for audio and video defined by the Moving Picture Experts Group (MPEG), and published as the ISO/IEC 13818 international standard. MPEG-2 is used to encode audio and video for broadcast signals, including direct broadcast satellite and Cable TV.

MPEG2-TS – Moving Pictures Experts Group Standard 2 – Transport Stream

The transport stream component of MPEG-2.

MPEG-4 – Moving Pictures Experts Group Standard 4

A group of audio and video coding standards used to compress audio and video digital data defined by the ISO/IEC Moving Picture Experts Group (MPEG). The MPEG-4 standard is used for streaming media over the Web, CD distribution, videophone conversations, and broadcast TV. MPEG-4 incorporates many MPEG-1 and MPEG-2 features, and provides the concept of defining profiles and levels so that designers can define specific sets of capabilities for multiple applications.

MPEG PES – MPEG Packetized Elementary Stream

See PES.

MPLS – MultiProtocol Label Switching

An IETF initiative that integrates Layer 2 information for network links (bandwidth, latency, utilization) into Layer 3 (IP) to simplify and improve IP-packet exchange. MPLS gives network operators the flexibility to divert and route traffic around link failures, congestion, and bottlenecks. When packets enter an MPLS-based network, Label Edge Routers (LERs) assign a label identifier to them. These labels contain information based on the routing table entry (destination, bandwidth, and delay), and refer to the IP header field (which contains the source IP address), the Layer 4 socket number information, and service.

MSTV – Microsoft TV

Technology used to bring TV services to consumers worldwide by delivering broadcast-quality video and integrated services over broadband.

Multicast

The delivery of information to multiple destinations simultaneously.

Multi-User Server/Client

Software that supports a client/server model. A server is the host machine that provides services to clients, and the clients are requesters of services from the server. This model provides a convenient way to interconnect clients that are distributed across the same network segment.

MySQL

MySQL is a database management system, which handles structured collections of data. MySQL adds, accesses, and processes the data stored in a databases, which is used by the Triple Play Analyzer.

NIC – Network Interface Card

Network adapter interface hardware inserted into the computer for interfacing with a network. Each NIC has a unique MAC address.

Orphan Audio

An audio stream that is not in an RTP session with another participant.

Packet

A packet is the basic unit of data in a transport stream, consisting of a sync byte, followed by three flags and a 13-bit PID. A 4-bit continuity counter follows, and additional optional transport fields may follow. The remainder of the packet is payload.

PAT – Program Association Table.

The Program Association Table lists PIDs for all PMTs in the stream.

PCMA

PCMA is an audio payload type specified in CCITT/ITU-T recommendation G.711.

PCMU

The encoding used for the Internet media type audio/basic, and is specified in CCITT/ITU-T recommendation G.711.

PCR Jitter – Program Clock Reference Jitter

A random variation in the time of the PCR clock reference. The PCR is made up of a 33-bit PCR base number and a 9-bit high-resolution extension, for a total of 42 bits. Two PCRs are used in the MPEG transport stream – the local PCR and the packet header PCR. The local PCR is compared with the packet header PCR to produce the PCR phase error.

PES – Packetized Elementary Stream

Defined by the MPEG communication protocol as an elementary stream that is packetized by adding a packet header to every "x" number of bytes. An elementary stream is the most basic component of IPTV, and can contain video, audio, control data, or other data.

P Frames

"P" frames are considered to be major predictive frames, and contain differences in the video as compared to the original "I" frame. "P" frames are both forward and backward predicting, and provide information to the "B" frames regarding changes in the video.

PID – Program ID

Each table or elementary stream in a transport stream is identified by a 13-bit PID.

PMT – Program Map Tables

PMTs contain information about programs, and describe which PIDs contain data that is relevant to the program. PMTs provide meta data about the program streams, such as whether the program contains an MPEG-2 video stream.

QoE – Quality of Experience

QoE describes the performance of the service from the consumer's point of view. QoE tests verify the network, encoding, and signaling.

QoS – Quality of Service

A telecommunication network's probability to meet a certain traffic contract.

R Factor

The transmission rating factor R for voice performance metrics can be in the range from 0 to 100, where R = 0 represents an extremely bad quality, and R = 100 represents a very high quality.

RTP – Real-time Transport Protocol

A standardized packet format used to deliver audio and video over the Internet, built on top of the User Datagram Protocol (UDP). RTP carries data that contains real-time characteristics, such as interactive audio and video. RTP is used in both multicast and unicast applications. It is used with RTSP and RTCP in streaming media systems.

RTSM – Run Time Store to Media

A high-speed data transfer capability of the Agilent Network Analyzer, which is used to capture and save network data frames to files.

RTSP – Real Time Streaming Protocol (RTSP)

A protocol used in streaming media systems, and allows a client to remotely control a streaming media server by issuing commands such as play and pause, and allowing time-based access to files on a server. RTSP servers can use RTP as the transport protocol for the actual audio and video data.

SIP –Session Initiation Protocol (SIP)

A protocol used to initiate, modify, and terminate an interactive user session that includes multimedia video and voice elements. SIP is a 3GPP signaling protocol, and is a leading signaling protocol for VoIP. SIP is used to set up and tear down voice or video calls.

SQL – Structured Query Language.

A popular computer language used to create, retrieve, update, and delete data from relational database management systems.

TCP – Transmission Control Protocol (TCP)

The connection-oriented transport protocol in the Internet suite. TCP is connection-oriented transport protocol, and uses three phases: connection establishment, data transfer, and connection release. TCP is a layer 4 protocol, also known as the Transport Layer.

TCP/IP

The TCP/IP protocol suite includes the Transmission Control Protocol (TCP) and the Internet Protocol (IP). TCP/IP is a set of communications protocols that implement the protocol stack on which the Internet and commercial networks operate.

Throughput

The percentage of a bit stream or the effective bit rate that is used by protocol header bits, data bits, FCS bits, flags, and zero bit insertion bits.

UDP – User Datagram Protocol

A connection-less protocol in TCP/IP that corresponds to the Transport Layer in the ISO/OSI model. UDP converts data messages into packets to be sent over the Internet Protocol.

Unicast

The sending of information packets to a single destination in a computer network. Unicast servers provide a data stream to a single user.

VMOS – Video MOS

A numerical indication of the perceived quality of video media after it has been compressed, transmitted, and received. Video MOS is expressed as a single number in the range of 1 to 5, where 1 is lowest perceived quality, and 5 is the highest.

VoD – Video on Demand

An interactive television service, which allows users to select and view video content over a computer network. VoD systems can either stream the content or download it.

VoIP – Voice over Internet Protocol

A technology that allows voice calls over a broadband Internet connection.

Zap

A term referred to the time it takes between switching channel over the IGMP protocol. The Video Control view displays the trend and distribution of channel "zap" times.