About this data

Alprazolam d0 and d5 with two fragments ea.

1 ul inj., 3 inj. Ea level, 0.5 – 5000 pg on column

0, 0.5, 1, 5, 10, 20, 50, 100, 200, 500, 1000, 2000, 5000 pg on column

ISTD at 50 pg

MeOH w/ 0.1% Formic Acid pH3

Zorbax C18 2.1x50 mm, 3.5 um particle, ~ 96 bar

CE 30 EV 50 ms Dwell

RT 1.25 min., run time 2 min.

Tuned for analytical work FWHH 0.65 amu on Q1 and Q3 (there could be more signal to get when opening up).