Known fixes/patches for E6474A, version 9.3:
1) GSM Patch specifically for E6458C-100 GAP receivers to correctly make GSM Channel Analyzer measurements at the same measurement speed as all other E645xx GSM receivers.

a. GsmBch 8-Aug-06.exe is a self-extracting zipped file that will replace all affected BCH measurement DLL files in the folder C:\Program Files\Agilent Technologies\E6474a\Measurement.
2) Kyocera 2325 CDMA cable-alarm issue when used with 
a. Kyocera 2325 Patch.doc is document used to install this patch listed below.
b. DCCDMA.zip

3) The new SamsungWcdma.dll fixes UMTS/HSDPA Compression Mode measurements as mobiles is passing calls from 3G network to 2G network and fixes RLC Uplink statistics from reporting values over 100%.  As of October 20, the newest SamsungWcdma.dll and CommMngr.dll fix the missing UMTS messages which under certain network conditions the Qualcomm UE’s would appear to stall the sending of trace logs for up to 6 seconds, this gap would appear in the recorded data file SD5 and could also cause buffer overrun both on the UE and in the drive test application.
A. Close E6474A application before renaming or copying any files.

B. Unzip the file “Agilent_DLL_file_10-20-2006.zip” from the folder “UMTS UE DLL Update” and follow directions below which replaces the files listed below.

C. In the folder “C:\Program Files\Agilent Technologies\E6474a”, 

a. rename “SamsungWcdma.dll”  to “SamsungWcdma.dll_093”

b. rename “CommMngr.dll”  to “CommMngr.dll_093”
D. The unzipped file SamsungWcdma.dll needs to be copied to the folder “C:\Program Files\Agilent Technologies\E6474a”

E. The unzipped file CommMngr.dll needs to be copied to the folder “C:\Program Files\Agilent Technologies\E6474a”

F. Now you are ready to re-launch E6474A for making measurements.
4) iDEN Control Channel issue described as a problem with a Color Code value being clipped when it became a server (the issue is only when Channel and CC are displayed in decimal).
a. iDEN Patch readme.doc provides step-by-step to replace and register DLLs in the file below.
b. OCX-OCF.zip

5) CDMA SMS (Sierra Wireless 555 Aircard) WAMS patch fixes the problem to allow WAMS to correctly send CDMA2000 SMS commands to the Sierra Wireless 555 Aircard..
a. SierraSMS.exe needs to be replaced in the folder C:\Program Files\Agilent Technologies\E6474a\sequencer.

All of these patches are available through the Agilent Anonymous ftp server under the folder “E6474A_9.3_Patches”

Server:  ftp.agilent.com
IP Address: 192.25.91.205

Path to account: /pub/wireless/transfers/

To Login: ftp://ftp.agilent.com, with user name Anonymous and Password of your.email@company.com
