!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

A.M.P.E.R.E! IS....           /\/\      /\/\

//\/\\  /\//\/\\    ||

= Hosehead =           ||    //    \\//\/  =======||=======

=======||=======    \/            ||

= Shadow Lord =        ||                  =======||=======

=======||=======__  /\      //    ||

= Brain Injured =      ||    \\/__\//\\  /\//     ||

/\/\      ||     \/  \/  \\//\/      ||      /\/\

//\/\\/\  /||              \/         ||\  /\//\/\\

______//____\/\\//||  _        _   _  _   _  ||\\//\/    \\

/_o_ooo_oooo_\  \/ || /_\ |\/| |_> |_ |_| |_  || \/   _____\\______

\__________/         | |.|  |.| . |_.| \.|_.        /_o_oo__ooooo_\

\___________/

PRESENTING:  HARDWARE TOYS FOR BIG BOYS!... PHONE HACKS!

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

YOU ALL LOVE 'EM... WE ALL LOVE 'EM... FIRST WE HAD PERF#*%SOU!?% - THE

HACK... NEXT WE HAD AMA@^iI-THE HACK... AGAIN, WE GRACE YOUR SCREEN WITH...

SOME MORE OF OUR OWN HARDWARE HACKS! FOR ALL YOU HACKS! HACK, HACK, HACK!

--- ---

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

BROUGHT TO YOU BY A.M.P.E.R.E.!

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

= USE THEM! = HAVE THEM! = BUILD THEM! = WE TAKE NO RESPONSIBILITY FOR 'EM! =

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

Ok you cretins... this is as simple as it gets!!!!!

Some cool hacks that dont get any easier than this! If you cant handle these,

why dont you stay awake in school for once......

HACK #1...  TELEPHONE / CASSETTE RECORDER INTERFACE

This  little  beaut  will  start  a cassette recorder up when your phone is

lifted  off  hook.   You need a recorder that has one of those remote start

options  that  are  triggered  by  a  switch on the microphone or a similar

method.

Hook  the  unit  up  as  indicated.....  The line you hook up to is the red

conductor  in  your phone system.  One note to remember, if you want any of

these  items  to work on ALL phones, put it in the line that feeds ALL your

phones.   If  you  want it to trigger on ONE phone, put it in the line that

feeds ONLY THE DESIRED phone.

You  can  use  any  P  channel  FET  for  Q1  in  this project, but Q2 is a

darlington  transistor, so if you look for a replacement, be sure to get an

appropriate  unit.   Parts  are  not  critical, most any ' junk box ' parts

within  reason will work fine.  The listed parts are GUARANTEED to work, as

I  have  FULLY  WORKING  prototypes  of all these circuits using the listed

parts.

Hook this one across both lines coming in on your phone ( red and green ).

HACK #2...  LINE IN USE INDICATOR

If you take the ' Recorder ON * ' line and put a LED in series with it, add

a  9  volt  battery,  you  have  the  line  in use indicator.  A bit on the

elaborate side for the purpose, it does work well.

HACK #3...  FM WIRELESS TELEPHONE TRANSMITTER

You  gotta  love this one!....  Make your telephone into a conference phone

so  all  in  the room can hear your discussion over a FM radio.  This is as

simple  as  it  can  be made and still perform well.  Keep the antenna wire

away  from  the  rest  of  the phone wiring and circuitry, or you will lose

considerable  range.  Coil L2 is made of 22 gauge wire, 5 turns over a 5/32

diameter  form.  Recommended antenna length is 12-18", unless you want your

neighbors  to hear you also.  This little sucker really gets out well, dont

let it's simplicity fool ya!  It is good for at least 100', and more with a

good antenna.  If you use a larger value capacitor for C3, you can omit C2,

as  C2  is just fine tuning of the value of C3, which sets the frequency of

broadcast.   With  fine  tuning  of C2, and spreading the turns of L2, this

unit will effectively tune from 70 to 110 mhz.  Tuning it between 88 to 108

mhz will allow you to use your FM radio for a reciever.

Hook this one in SERIES with the red wire on the phone! It takes the power

it needs from the phone line. It can be detected by your telco and any

telephone line analyser if there is a reason to look for problems...

Dont do anything I wouldn't do with this little bugger..... Heheheh!

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

Created for your enjoyment by your friends at A.M.P.E.R.E!

A.M.P.E.R.E! is......

*****  HOSEHEAD  ***  SHADOW LORD  ***  BRAIN INJURED  *****

Hack with the BEST!.... FUCK the rest!!

IF ALL ELSE FAILS:

Your  problem....   they  worked here!  Try prayer once and waste some more

time!   Hope this all helps you with some inspirations for better living on

your  Amiga!   If not, well, go get a real life, and not just a Sears life!

Sorry, 'Brand Central' doesn't have them either...

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

Some Immortal Thoughts from us:........

' The best things in life may be free, but the rest makes up for it. '

' Of all the things I have lost, I miss my mind the most. '

' Life is too short to drink cheap booze! '

' I used to jog, but the ice kept jumping out of my glass. '

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!

A.M.P.E.R.E! = Shadow Lord - HoseHead - Brain Damaged =

Coming to a well-chillin' BBS near you......

!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!*!