C64S - Commodore 64 Software Emulator

Registered version 2.60, February 1996

(C) 1991-1996 Miha Peternel

C64S USER'S MANUAL

This document is (C) 1996 by Miha Peternel. No part of this

document may be reprinted without written permission of the

author.

____________________

1. INTRODUCTION

Welcome to a software emulator of the good old fat C64. It

will allow you to run most of the original C64 software on

your PC.

The C64S emulator and accompanying utilities are copyrighted

(C) 1995 by Miha Peternel and may not be copied, sold, hired

or distributed in any way without written permission of the

author. C64 and 1541 ROM code is copyrighted (C) by Commodore

Business Machines.

Minimum requirements:

- 386 or better

- 640K RAM (500K low ram free)

- VGA display (register compatible)

- DOS 3.0 or newer

Optional:

- a soundcard:

Gravis UltraSound (best for sound output)

Sound Blaster (all versions or DSP compatible soundcards)

Pro Audio Spectrum (8 or 16 bit PCM)

Covox or compatible DAC (digital to analog converter)

- PC joystick (1 or 2)

- tape/floppy interface

You need at least a cached 386DX40 with a fast VGA for real

time emulation. A soundcard is recommended, though the sound

can also be played through PC speaker (at rather low

quality).

The following files are essential for proper emulator

operation:

- C64S.EXE          C64 Software Emulator (386+)

- SYSTEM.EXE        C64S system emulation (386+)

- CONFIG.EXE        C64S hardware configuration utility (286+)

- ROMCODE.C64       C64 and 1541 ROM code

- SIDWAVES.C64      SID waveforms

The following utilities make the life easier:

- COM1541.EXE       Transfers 1541 disk contents to .D64 files (286+)

- TAPEIO.EXE        Turbo Tape loader for PC (286+)

- MAKETAPE.EXE      Creates .T64 files and imports .PRG files (286+)

- FIXTAPE.EXE       Fixes bad CONV64 created .T64 files (286+)

Documentation:

- C64S.DOC          A guide to the emulator you are reading

- LPT64.DOC         LPT64 connector description

- LPT1541.DOC       1541 connection scheme

- LPTTAPE.DOC       Tape recorder connection scheme

- T64.DOC           .T64 file structure

- D64.DOC           .D64 file structure

______________________

2. CONFIGURATION

C64S comes with an automated configuration program, which

lets you easily configure emulator options to best suit your

hardware. To start the configuration program change to C64S

directory and type on the command line:

CONFIG

Or in case of any troubles:

CONFIG -s

The configuration program will autodetect the presence of

soundcards, analogue joysticks and parallel (LPT) ports.

Some users may have many add-on cards installed in the

computer in which case the autodetection procedures may fail

or even crash the computer. If you notice any such trouble

try running the configuration program with -s switch as

suggested above. This switch will tell the program to skip

autodetection procedure.

The configuration program will then try to read current

saved settings. If no settings are saved, it will use

default settings. Also, you can always use -d switch to use

default settings instead of the saved settings.

In the configuration screen, you may use Up and Down arrows

to move around, Left, Right and Enter keys to change the

highlighted setting.

Configuration overview:

- Sound Output

C64S can autodetect Gravis UltraSound, Sound Blaster (or

true DSP compatible) and Pro Audio Spectrum soundcards. If

the sound output is set to Autodetect, the priority of

selecting sound output is: GUS, PAS, SB, PC speaker. If the

configuration program fails to detect your soundcard's port,

try setting it manually in the configuration screen. Note

that C64S can autodetect and use PAS soundcard only if

MVSOUND.SYS device driver is loaded (this is done by the PAS

installation program). The sound synchronization option

determines at what level the emulator synchronizes emulated

sound output with real sound output. Setting sound

synchronization to High enables C64S to reproduce digital

speech and other digital effects flawlessly, whereas Low

setting saves some CPU time.

- Analogue joysticks

If this option is set to Autodetect, C64S will autodetect

the number of analogue PC joysticks connected. Setting this

option to 1 or 2 joysticks will force C64S to read the

selected number of  joysticks. C64S provides two methods of

scanning joysticks. Compensating mode takes less CPU time

but might fail with some joysticks or newer "speed

compensating" game ports. Compatible mode will work with all

joysticks. You are advised to try Compensating mode first.

If it fails, change to Compatible mode.

- Video mode

C64S works in two VGA modes. Default Extended mode is VGA

368*240 pixels, 16 colours. Compatible mode is provided in

case you are running the emulator on a laptop with LCD

display or if your VGA system cannot display Extended mode.

Compatible mode is standard VGA 320*200 pixels, 16 colours.

- External devices

C64S supports real-time communication with external CBM

devices connected to PC's LPT port. Setting Use external

devices must be set to Autodetect or to Yes to use any

external device. If you don't use any external devices set

Use external devices to No to speed up emulator loading. The

same goes for External 1541 drive and External CBM printer

settings. C64S can autodetect printer as device 4 or 5, and

1541 or compatible disk drives as devices 8-11. The lowest

device number (8-11) not assigned to external 1541 drive is

assigned to emulated 1541 drive. The lowest device number (4-

5) not assigned to external CBM printer is assigned to PC

printer.

- PC printer port

This port setting is used for direct PC printer

communication. Default setting is LPT1, but you may need to

change this setting if you have more than one LPT port. If

you try to assign a non-existing LPT port, the selection

will be marked as not available (N/A).

- Tape port & CBM IEEE port

These port settings are used by TAPEIO and COM1541

utilities. CBM IEEE port is also used for direct 1541 and

CBM printer communication. If you have only one available

LPT port it will be autodetected. If you have many you are

recommended to use LPT2 or LPT3 since LPT1 is usually a

printer port. In case of a non-existing LPT port assignment

the selection will be marked as not available (N/A).

IMPORTANT: If you run C64S in multitasking environment

(Windows, OS/2), you are strongly advised to disable sound

output and set joystick scanning mode to Compatible. This

will prevent general slow down and other difficulties in

such environments, but still won't make possible to use

external CBM devices. If you have a GUS, you may have sound

enabled, but it may conflict with soundcard drivers used by

operating system. Exit environment and run C64S from plain

DOS to enjoy full emulation performance.

When you have finished with configuration, press End and

Return to save settings and exit. You may always press Esc

to abort configuration without saving.

___________________

3. FIRST STEPS

This step by step introduction is here to help beginners use

the emulator for the first time:

1. If you are running the emulator for the first time, and

you have not run the CONFIG utility yet, please refer to the

previous chapter for the information about hardware

configuration.

2. Run the emulator by changing to the emulator's directory

(if necessary) and typing on the DOS command line:

C64S

The well-known blue C64 screen appears.

3. Press F9 to get to the emulator desktop. Press ALT T to

reach tape file screen or ALT D to reach disk file screen.

Select an image file using arrow keys and press ENTER. The

selected image file is now "open" and you will enter the

directory screen with the list of files stored in the image

file.

4. Select a program with cursor keys and press L to load it.

Pressing ENTER instead of L will return you to C64 screen.

In this case you need to press SHIFT+TAB (which equals

SHIFT+RUN/STOP) to load a tape program and run it

automatically. To load and run a disk program type

LOAD"NAME",8,1 and RUN.

5. Press F10 to enter joystick option screen. Configure the

joystick port emulation as you wish. You can even change the

keys to emulate joysticks. IMPORTANT: If you are using

analogue PC joystick and you are running the emulator for

the first time, press `R' and follow the steps to

recalibrate the joystick.

6. After you have finished configuring joystick settings,

press ESC to get back to C64 screen and enjoy the nostalgia.

:)

7. You can always press F10 to enter option screen, where

you can adjust numerous emulation settings to improve the

behaviour of the program you are running.

8. When bored press CTRL-ALT-DEL or CTRL-ALT-INS to reset

the emulator and proceed from step 3.

9. CTRL-BREAK gets you back to DOS prompt.

Please read the following chapters for more information on

emulator behaviour.

________________

4. IN ACTION

The emulator emulates all the components of C64 needed to

make the programs run correctly, display genuine graphics,

synthesize sound, and emulate I/O operations.

The keyboard layout is original C64 with some exceptions due

to a different PC layout.

Some special emulator features make the work with the

emulator easier. The tapes and disks to be used with the

emulator are stored in a special format (referred to as

image files) on the hard disk. You can select the tape

position or a disk using emulator desktop. Emulation

settings can be changed either with the CONFIG program

(hardware settings) or in the option screen (other

settings).

The following keys have special meaning:

- F9 enters emulator desktop

- F10 enters joystick option screen

- F11 pressed prevents screen updates

Great for uninterrupted sound reproduction !!!

- ESC is an exit/cancel key

- CTRL+BREAK exits the emulator

- CTRL+ALT+DEL or CTRL+ALT+BackSpace emulate reset

- CTRL+ALT+INS causes brutal reset (for reset-protected

programs, actually it rewrites the CBM80 sign and performs a

normal reset)

- PRINT SCREEN captures screen to a Windows bitmap file

(filename C64S0xxx.BMP)

Table of replaced keys with their equivalents:

- C=           Alt

- Ins/Del      BackSpace

- Clr/Home     Home

-            Insert

-             Delete

- Run/Stop     Tab

- Restore      F12

Default joystick emulation keys:

- Primary set (1st emulated joystick):

Default primary set consists of arrow keys and right Alt

key.

- Secondary set (2nd emulated joystick):

W    up

S    down

U    left

I    right

O    fire

_________________________

5. EMULATOR DESKTOP

Press F9 in the emulator to enter the emulator desktop. The

desktop consist of multiple screens that allow you to

manipulate the tape and disk images used in the emulation,

you can change several emulation parameters and you can even

monitor the emulator CPU and memory (both C64 and 1541).

The top screen line represents the main menu. Press Alt key

to enter main menu. Move around with arrow keys and select a

topic by pressing enter. Or, you can select a main menu

topic directly by pressing Alt plus the highlighted letter.

In all the the desktop screens the bottom screen line lists

the actions available. Press the higlighted letter (without

Alt!) to select the action. Enter will always select the

left-most action.

The following is the list of desktop screens and functions

available:

3.1. TAPE

The tape file screen lets you select the tape image file to

be used in the emulation. Move around with arrow keys, press

Enter to select the tape or change the directory. When you

select a tape image file, you will enter the tape directory

screen. Move around and press Enter to set the tape pointer

position. You will get back to the emulation screen. When

you enter the LOAD command or (Shift+Tab to LOAD and RUN),

the emulator will read a file from the position set in the

tape directory menu. When saving a file, the emulator will

always add a file to the end of the tape image, regardless

of the position set (it will not overwrite anything). You

can load and run a program easily by pressing L in the tape

directory screen.

The actions available in the tape file screen are:

- Open

Opens the highlighted tape image file and enters tape

directory screen (equivalent is Enter).

- Files

If a tape image file is open, it jumps to the tape

directory screen (equivalent is right arrow key).

- Description

Lets you edit the highlighted tape's description.

- New

Enter the file name, tape desctription and maximum

directory size and the emulator will create an empty tape

image file.

- CD (Change Drive)

Press a drive letter (A-Z) to select a drive where the

emulator will search for .T64 files.

The actions available in the tape directory screen:

- Set

Sets tape pointer position and returns to the emulation

screen (equivalent is Enter).

- Load

Reboots C64, loads highlighted file and runs it.

- Name

Lets you edit the higlighted file's name.

- Delete

Deletes the highlited file.

- Import

Select the file from the file list and the emulator will

import it to the selected tape image file. The files to be

imported must be in the two formats:

- standard 1541 .PRG format:

2 bytes - load address, other bytes - file data.

- German .Pnn format (nn is in range 00-99)

8 bytes - "C64File" description, zero terminated

17 bytes - file name, zero terminated

1 byte - record size (defaults to 0 for .PRG files)

If the file selected is not in one of the listed two

formats, you may get unexpected results.

- Export

Saves the highlighted file to a .PRG or .P00 file. All

other extensions default to .PRG format (you can not export

to a .T64 file).

- Freeze

Saves full emulator state in a file. When you load such a

file, it will start automatically from the point when it was

saved. This function is not yet standardized. That means

that the files you save with this function may not be

loadable in the forthcoming versions of C64S.

- Tape

Jumps to the tape file screen (equivalent is left arrow

key).

3.2. DISK

The disk file screen lets you select the disk image file to

be used in the emulation. Move around with arrow keys, press

Enter to select the disk or change the directory. When you

select a disk image file, you will enter the disk directory

screen. Press Enter to set the disk and get back to the

emulation screen.  You can proceed with C64 commands to use

disk image file. You can load and run a program easily by

pressing L in the disk directory screen. Be careful,

changing disk image file when an emulated program uses it

can damage contents of the image file.

Actions in the disk file screen:

- Select

Selects the disk image to be used in the emulator

(equivalent to pressing Enter).

- Files

If a disk image file is open, it jumps to the disk

directory screen (equivalent is right arrow key).

- New

Enter file name, disk title and disk ID and the emulator

will create an empy disk image file. To use it, select it in

the disk file screen.

- CD (Change Drive)

Press a drive letter (A-Z) to select a drive where the

emulator will search for .D64 files.

The actions available in the disk directory screen:

- Set

Sets disk pointer position and returns to the emulation

screen (equivalent is Enter).

- Load

Reboots C64, loads highlighted file to original address

and runs it.

- Import

Select the file from the file list and the emulator will

import it to the selected disk image file. The file to be

imported must be in .PRG or .Pnn format. If the file

selected is not in one of these two formats, you may get

unexpected results.

- Export

Saves the highlighted file to a .PRG or .P00 file. All

other extensions default to .PRG format (you can not export

to a .T64 file). Currently only PRG files are exported

properly.

- Disk

Jumps to the disk file screen (equivalent is left arrow

key).

3.3. OPTIONS

There are two option screens available: the general

options screen and the joystick options screen.

The option screen actions are:

- Change

Changes the higlighted option. It either selects the

highlighted radio button, changes the check box status or

asks the user to edit the higlighted field. If the radio

button list title is highlighted, Change selects the next

radio button.

- Joystick

Enters joystick option screen.

- General

Enters general option screen.

- Swap

Swaps joystick #1 settings with joystick #2 settings. This

is to help you reassign joystick emulation settings.

- Recalibrate

Runs joystick recalibration. Move joysticks as the

emulator asks you to recalibrate correctly. If the emulator

displays "Cannot recalibrate" message run CONFIG and check

if analogue joysticks are enabled (the option must be set to

Autodetect). If joystick scanning mode is set to

Compensating, try setting it to Compatible. Save

configuration, restart the emulator and run joystick

recalibration again.

Options overview:

- Screen refresh

C64 updates on-screen picture 50 frames per second in PAL

or 60 frames per second in NTSC version. That means the

emulator must generate A LOT of video data and transfer it

to the VGA card. As this is the most time consuming part of

the emulaton, the emulator allows you to control the screen

refresh rate thus making possible to achieve original

emulation speed also on slower PC computers. However, if the

screen refresh rate is to low, it might affect the animation

smoothnes, or, in some cases, it will disturb the functions

that depend on accurate screen refresh rates, per example

some games use hardware sprite collision detection, which

occurs only when the screen is updated. Keep this in mind if

you notice such problems (bullets flying through objects or

similar).

Screen refresh settings:

- Synchronized

Automatically synchronizes the screen refresh rate with

the speed of your computer. It will autoselect the screen

refresh rate between original refresh rate and the custom

frame rate. This way you can limit the lowest frame rate. It

is advised that you input the custom frame rate between 1/3

and 1/10 and set the screen refresh to synchronized.

- Original

Selects original screen refresh rate (1/1).

- Custom

Selects custom screen refresh rate. 1/X means 1 out of X

frames is displayed. 1/1 means each frame is displayed, 1/10

means only every 10th frame is displayed thus saving 90%

time for faster CPU emulation.

- CPU speed

C64's 6510 CPU runs at a fixed clock speed of around 1

MHz. The emulator lets you control the CPU speed, which is

useful in some applications. However, to achieve higher CPU

speeds you need a fast PC (486/66 will run at 250-450%

speed). The percentage displayed in the parantheses in the

general option screen is the current estimated speed.

CPU speed settings:

- Original

Tries to near original (100%) speed.

- Maximum

Forces maximum achievable speed.

- Custom

Use this setting to force a desired speed.

IMPORTANT: To achieve higher CPU speeds, you must set the

screen refresh to synchronized or to custom at a frame rate

in range of 1/10.

- VIC options

The emulator lets you control sprite emulation in order to

gain faster emulation or to cheat in the games that rely on

the VIC chip when detecting object collisions. Set all

options to back to "on" for original emulation.

- Machine type

There were two major types of C64 machines manufactured

for two different TV standards: PAL (used in Europe) and

NTSC (used in USA). Though most of the programs will run ok

independent of this setting, the emulator lets you switch

between the machine types to gain better compatibility. Try

changing this setting when you notice on-screen garbage or

music playing either too fast or too slow.

- Joystick options

You can control the emulation of two C64 joystick ports.

You can use keyboard emulation or analogue joystick

emulation. To use analogue joysticks, they must be enabled

in the hardware configuration (set to Autodetect or the

number of joysticks connected) and successfully

recalibrated.

The emulator allows you to define two joystick key sets.

Primary key set is used, when only one jostick is emulated.

Both key sets are used, when both joysticks are emulated by

keyboard. In this case primary key set emulates joystick

port #2, and secondary key set emulates joystick port #1.

IMPORTANT: When the keyboard joystick emulation is

enabled, the keys used for joystick emulation don't respond

in the original way anymore. Per example: if key 'A' is used

to emulate joystick direction, you won't be able to type

character 'A'. When such conflicts occur, either press F10

and disable keyboard joystick emulation, or change the keys

used for joystick emulation so they will not conflict with

the keys that need to operate originally. Some keyboards

also behave strange when multiple keys are pressed at the

same time. This is the most annoying when emulating

joystick, per example you might notice that you can't move

up-left and shot at the same time. If you notice such or

similar behaviour, try changing the joystick emulation keys.

3.4. MONITOR

Built-in monitor lets you watch the programs when running.

In the monitor screen you see:

- 6510 registers (top right)

- 6510 code (top left)

- memory dump (bottom)

Move around with arrow keys, move faster with Page Up,

Page Down, Home and End keys.

Data dumped at I/O addresses $D000-$DFFF is the latest

data written or read from the I/O address, it is not always

what CPU will read next.

The actions available are:

- Mode

Switches focus between code and dump.

- Bank

Selects memory bank. 0-7 selects a C64 bank (equal to low

3 bits of memory location $01), 8 selects emulated 1541

memory.

- Goto

Prompts for a new address and jumps to it.

- Find

Finds data in the memory dump bank. Data must be hex,

numbers longer than 2 digits are treated as word values and

converted to LO/HI when searching.

- Where

Moves code display to the instruction where emulation was

interrupted.

- Trace (F7)

Executes one instruction and moves code display.

- Step (F8)

Steps over the instruction skipping code display of

subroutine branches. In case of a subroutine call, the

subroutine is executed in debug mode. You can interrupt

subroutine execution by pressing Esc.

- Here (F4)

Sets a breakpoint at the position of code display and runs

C64 emulation in debug mode until CPU reaches the

instruction at the breakpoint or Esc key is pressed.

IMPORTANT: Running C64 emulation in debug mode will cause 30-

50% slow down. Exit debug mode by pressing Esc, once you

have finished monitoring the programs.

3.5. HELP

Online help provides basic information the user needs when

running C64S. It is meant more like online reference and not

a copy of the manual.

Help usage:

- Use arrow keys to move around topics.

- Press Enter or R to jump to the highlighted topic.

- Press I to get help index.

- Use Page Up and Page down keys to list help screens.

___________

6. TIPS

Q: What are the most important changes from version 1.1B to

2.00 ?

A: There are many:

- improved sound on GUS, PAS, SB and speaker (all in native

mode)

- disk directory is accessible in the same way as tape

directory

- COM1541 uses fastloader to import disks 10 times faster

- TAPEIO has been improved to work with any type of LPT port

Q: I selected a disk image file in the disk file screen. How

can I use it ?

A: Using 1541 drive operates in the original way. The CBM

DOS commands (except format) work, too. Type LOAD"$",8,1 to

load directory, LIST to see it, LOAD"prg name",8 to load a

program (you may use wildcards) and RUN to start it. To load

a file to its original address, you need to type LOAD"prg

name",8,1. Run a machine code program writing SYS xxxx,

where xxxx is the program entry address (decimal).

Q: My display does not show the whole C64 screen !

A: Run CONFIG and set display mode to Compatible. You must

do this if you use a laptop computer with an LCD display.

Q: When I press certain keys the emulator seems to dump

random characters, arrows and noumbers !

A: You need to enter joystick option screen (press F10) and

disable joystick emulation. C64 scans keyboard and joysticks

at the same I/O addresses and this is the resulting side

effect.

Q: Keyboard joystick emulation does not work O.K. !

A: Try changing joystick emulation keys (press F10). Note

that some keyboards behave strange when multiple keys are

pressed at the same time. If you notice that particular keys

work O.K. when pressed one by one, but malfunction when

pressed together, you must change the joystick emulation

keys (find a combination that will work O.K. with your

keyboard).

Q: I have difficulties using analogue joystick. What can I

do ?

A: You must recalibrate it in the joystick option screen

(press F10 and `R'). If the emulator displays the "Cannot

recalibrate" message, exit the emulator, run CONFIG and

check that analogue joystick option is set to Autodetect or

to the correct number of connected joysticks. If joystick

scanning mode is set to Compensating, try setting it to

Compatible. When you change joystick scanning mode, you also

need to recalibrate joysticks in the emulator! Also, if you

are using a single analogue joystick, try assigning analogue

joystick emulation to one port only.

Q: The emulator doesn't autodetect my PAS soundcard !

A: Check that MVSOUND.SYS device driver is loaded in your

CONFIG.SYS file. If you installed PAS software properly the

following line should be placed in CONFIG.SYS:

device=c:\spectrum\mvsound.sys [parameters]

Run PAS installation software to install the driver or read

your PAS User's Guide to find out how to modify CONFIG.SYS

and configure MVSOUND.SYS.

Q: Can I use PC printer with emulated C64 programs ?

A: Yes, but only for pure text output. Printing graphics or

changing text attributes will not work, because CBM printers

and PC printers use different sets of control codes. To use

PC printer, you must run CONFIG and select the LPT port

where your printer is connected (default is LPT1 which will

work OK, if you only have one LPT port).

Q: Can I use 1541 disk drive or CBM printer with emulated

C64 programs ?

A: Yes. Any CBM devices connected to LPT port can be used

with emulated C64 programs. Run CONFIG and check that use of

external devices is enabled or set to autodetect. If you get

into problems, please refer to the configuration section of

the manual.

Q: May I run C64S from Windows, Windows NT or OS/2 ?

A: Yes, but C64S runs much slower in such environments. This

slow down especially affects sound output. If you notice any

troubles you are advised to run CONFIG, disable sound output

(set to No sound) and set joystick scanning mode to

compatible.  Run C64S from plain DOS to gain best

performance.

Q: DISK IO banner keeps blinking in bottom left corner !

A: DISK IO banner is blinking when a user program is running

in emulated 1541 RAM, usually it's fast loader or protection

code. If the banner keeps blinking forever it probably means

the program got stuck. Press Esc to terminate DISK IO mode.

Q: How can I achieve the highest possible emulation speed ?

A: Enter general options screen. Set the CPU speed to

maximum. Set screen refresh to custom and enter custom frame

rate 1/99. If the application does not use sprites set show

sprites to off. Press ESC and watch your application go mad.

To gain even more speed run CONFIG and disable sound

emulation and analogue joysticks. Also, if possible, disable

memory managers like QEMM, EMM and 386MAX. If you are using

Windows or OS/2, try running C64S from plain DOS.

Q: I prefer 320*200 graphics resolution used in the demo

version !

A: Run CONFIG and set display mode to Compatible.

Q: What is the best soundcard to be used for the sound

emulation ?

A: Gravis UltraSound. It takes the least CPU time and

produces the best output. The new GUS driver uses advanced

wavetable synthesis to generate crystal clear 44 kHz 16 bit

sound output.

Q: What to do to listen to C64 tunes on a slower PC ?

A: Run CONFIG and check that sound synchronizing is enabled.

Then run C64S and enter general options screen. First, set

CPU speed to original. Set screen refresh to synchronized

and enter custom frame rate 1/10. Now press ESC. If the

sound seems to be disturbed, hold down F11. If the

reproduction seems to be too slow run CONFIG, disable sound

synchronizing and try again. If this does not help your

computer is unfortunately too slow.

_____________________

7. COMPATIBILITY

The following is the compatibility list.

+ means implemented and compatible (++ is new from v0.9, +++

is new from v1.0)

- means not implemented or not compatible

* means planned in the future

CPU

++ full 6510 instruction set

+++ $01 memory banking and I/O

+++ $00 CPU I/O direction control

+ flags (not 100% compatible in decimal operations)

++ cycle counts

* boundary crossing I/O tricks

VIC

+ all graphic modes

++ border

++ open border (upper and lower)

+ sprites

++ interrupts: raster, sprite collisions (no light pen IRQ)

+ multiplied sprites

+ hardware scroll

+ shrunk screen

++ cycle exact screen changes

++ DMA

++ screen snapshots

* forced DMA

* full open border (left and right)

* more $D011 and $D016 tricks

SID

+ supports GUS, SB, DAC and PC speaker

+ all 3 channels

+++ channel 3 muting

++ 8 bit sampled original waveforms

+ ADSR envelope control

+ master volume control

+ pulse width modulation

++ $D418 amplitude output

+++ 8000 Hz synthesis on PC speaker (6 bit), SB (8 bit) and

PAS (8/16 bit)

+++ 44100 Hz synthesis on GUS (16 bit wavetable)

- no channel to channel effects

* filtering (at least for digital sound effects)

* SB AWE 32 native mode driver

CIA 1

+ keyboard emulation

+ joystick emulation

+ timer A

++ timer B

+ IRQ control

+ IRQ

++ time of day (C64 synchronized)

+++ time of day alarm

* rebindable keyboard

* serial port

CIA 2

+ VIC bank select

++ serial bus communication

++ timer A

++ timer B

+++ NMI control

+++ NMI

++ time of day (C64 synchronized)

+++ time of day alarm

1541

+ ROM level emulation (16K RAM & I/O + 16K ROM)

+ supports CBM DOS 2.6 commands except format

+ fast (ROM trapped) C64 to 1541 connection

+ image file operation

++ 1541 disk transfer utility

++ emulates direct serial bus communication

+++ supports 35 and 40 track .D64 image files

+++ suppors sector error information

+++ real 1541 connection (LPT port)

* improved (GCR coded) image file format

TAPE

+ ROM trapped LOAD and SAVE routines

+ tape image files with directory

+ ultra fast loading

++ TapeIO Turbo Tape loader for PC tape connector

++ MakeTape utility

* tape digitizing

PRINTER

+++ supports PC printers (for pure text output only)

+++ supports CBM printers (connected to LPT port)

EMU-DETECTION

+++ $DFFF read flip flop ($AA,$55)

+++ $DFFE manufacturer code (`S'=C64S)

+++ $DFFC version number ($0112=2.00)

+++ $DFA0 message (tiltle, version, $0D, copyright, $00)

_____________________

8. FILE TRANSFERS

The emulator uses two types of image files to store the

information in the format C64 programs can handle. There are

two types of image files:

- .T64 files represent tape image files. Though the C64 tape

drives were slow and unreliable this type of storage on PC

systems has certain advantages over other methods. The most

important is that a tape image file contains a tape

directory with the information about all the files stored in

the image file. This way the user can easily see the

contents and select the file he or she wishes to access.

Instead of slow C64 loader fast PC routines take care of

loading and saving thus minimizing the time necessary for

such operations. The last important feature is that multiple

C64 files can be stored in one tape image file. This helps

the user organize big collections of C64 software. The

emulator provides the tools for manipulating .T64 files: the

tools built in the emulator desktop, MAKETAPE utility for

creating tape image files and importing files, FIXTAPE

utility to fix CONV64 created files and TAPEIO utility,

which is a Turbo Tape loader for PC.

- .D64 files represent disk image files. These files are

actually a faithful reproduction of the 1541 formatted

floppy disk contents. They contain the data of all disk

sectors in the order they appear on a real 1541 disk. The

emulator emulates 1541 floppy drive so that emulated C64

programs can read and write emulated disks just like real

disks. Of course, you can store hundreds of floppy disks to

image files and use them simply by selecting them in the

disk file selection screen. The emulator also includes a

COM1541 utility that will let you transfer data from 1541

disks to image files.

8.1. Using COM1541 utility

COM1541 will let you transfer data from 1541 (or compatible)

floppy drive to disk image files that can be used with the

emulator. If you keep a big collection of 1541 disks, you

can now easily transfer them to your PC system.

To run COM1541 successfully you need to do the following:

If  you are running COM1541 for the first time, run

CONFIG and set the 1541 port to your LPT port's I/O address.

If you do not know the address you will need to experiment

to find the right one.

Connect PC's LPT port and 1541's serial port with the

cable (please refer to files LPT64.DOC and LPT1541.DOC for

details about the connection).

Then you can run COM1541 by typing on the command line:

COM1541

If you are not running COM1541 from the emulator's home

directory, you need to use a switch to specify LPT port I/O

address. Use 1 for 278, 2 for 378 and 3 for 3BC. Example:

COM1541 2

COM1541 will try to connect using LPT port at I/O address

378.

Upon successful program execution the 1541 drive led should

blink and the drive motor start running for a while. A short

menu will display on the screen:

[D] Display disk directory

[I] Import full disk (fast mode, all sectors, all info)

[A] Import allocated sectors only (normal mode)

[F] Import full disk (normal mode, all sectors)

[X] Exit

Insert a disk to 1541 drive and press `D' to see the

directory. If  the directory is displayed O.K., everything

is ready for disk transfer. It is up to you to select a

method. Usually selecting `I' will transfer the disks the

fastest and correctly. This method uses a custom fast loader

which works 10 times faster than normal loading mode and

also transfers sector error information which is used by

protected programs. Methods `A' and `F' are provided in case

fast mode fails to work. Method `A' transfers only sectors

marked as allocated in BAM to conserve on transfering time.

Method `F' transfers all sectors like method `I', but

doesn't transfer sector error information.

After selecting a transfer method, COM1541 will prompt for a

file name. If a file with the same name already exists it

will be rewritten! You do not need to add an extension,

because COM1541 will add .D64 by default. If no file name is

given, the default file name is IMPORT.D64. If this is the

case, rename the file after transferring to avoid possible

later difficulties.

When COM1541 finishes transferring the disk, exit with `X'.

The disk image file is now ready to use with the emulator.

Run C64S and check it.

8.2. Using TAPEIO utility

TAPEIO utility is actually a TURBO TAPE compatible loader

for PC. The files loaded without errors are stored to a .T64

file so they can be used immediately with the emulator. To

use it you need to connect a C64 tape recorder to PC's LPT

port (please refer to files LPT64.DOC and LPTTAPE.DOC for

details about connection). A VGA compatible display is

necessary for graphic display of incoming tape data.

The syntax is:

TAPEIO [tapename[.T64]] [port]

If you are running TAPEIO from the emulator's directory it

will read the tape port selected with the CONFIG utility

(default setting is 378). If you want to select a different

port, you must specify it on the command line. Use 1 for

278, 2 for 378 and 3 for 3BC. Run TAPEIO with no parameters

to see the list of LPT port noumbers. TAPEIO will not load

anything if the port is not set correctly.

If a .T64 file with the given name does not exist, the

utility creates an empty tape image file.

When you run TAPEIO, you are first presented with a record

joustage screen. The border flashes and there is a red

(dark) vertical line in the middle of the screen. Now press

PLAY on the tape recorder. What you should see on the screen

is the data coming from the tape recorder displayed as

yellow graphics scrolling up. If the communication is O.K.,

the yellow graphics should form two jagged stripes. Now you

can use Left and Right arrow keys to move the red line in-

between the stripes. Use Up and Down arrow keys to change

the display scale if necessary. This procedure is required

to accommodate the loader to the speed of your computer. You

can use a screw driver to adjust the tape head position in

order to improve signal quality.

When the red line is located as described above press Enter

to start the loader. The loader will start searching for a

file header. When a header is found, all the data is dumped

onto the screen. When a file is being loaded, the border

flashes in blue colours and the increasing percentage is

displayed on the screen. This percentage tells the amount of

the file already loaded. If the file is loaded with no

errors it is added to the tape image file given on the

command line. TAPEIO will continue loading files until the

tape image directory is full or it is interrupted by

pressing ESC.

Press ESC at any time to exit TAPEIO and return to DOS.

8.3. Using MAKETAPE utility

If you transfer C64 files using the shareware X1541 utility

or if you download them from a FTP site or a BBS, you will

probably get the files in the raw C64 format (with usual

extension .PRG). The files look like: 2 bytes - start

address, other bytes - code. MAKETAPE allows you to create

empty tape image files and import raw C64 files. In addition

to that MAKETAPE also imports .P00 files.

The syntax is:

MAKETAPE [tapename] [file1] [file2] ...

MAKETAPE first checks if tapename.T64 already exists. In

this case it opens the file, otherwise it creates an empty

.T64 file with the given name. If the tapename is succeeded

by filenames file1, file2..., MAKETAPE adds each file listed

to the end of the tape file.

Example:

You got a file called JOKE.PRG, which you know is a C64

file. You would like to create a new tape image file named

"JOKES.T64" and import your file, so you can use it in the

emulator. The solution is as simple as that:

MAKETAPE jokes joke.prg

Run the emulator, enter tape file screen and you will see a

new entry "JOKES" listed among other tape image files. You

can edit the description. Open the tape and "JOKE.PRG" will

be listed in the tape directory.

8.4. Using FIXTAPE utility

The first demo version of C64S was released without the

MAKETAPE utility. As a result a quick hack utility CONV64

appeared. In best efforts to translate C64 software into the

.T64 format the respective author simply used the

Cybernoid's header not knowing that a lot of important

information is stored in the .T64 file. Among other things

the program end address was not adjusted according to the

imported file's size. As a side effect many crunched

programs refused to work, longer programs did not fully load

and BASIC programs complained with an OUT OF MEMORY message.

The emulator now checks if it loads from a CONV64 file and

tries to fix the mess. LOAD ERROR message is generated to

inform the user something is wrong. If you notice such

behaviour run FIXTAPE to fix the tape image file. You are

encouraged to make a copy of the file before trying to fix

it for safety reasons.

The syntax of FIXTAPE is:

FIXTAPE [tapename1] [tapename2] ...

FIXTAPE will check all the listed files (tapename.T64). It

will display Fixed upon successful correction or O.K. if the

file needs no fixing. Run this utility with one file tape

images only !

_______________

9. CONTACTS

North American representatives:

C64S 2.00 is now available. Contact:

North America orders $69.95us.

Other $69.95us + $5.00us SH.

For Credit Card (VISA or MasterCard) orders

Phone, Fax, Mail or Email to:

Phone:  206.828.9001

Fax:    206.828.9011

Email:  lab@seattle.wa.com

CompuServe:     71202,560

FTP:    as.seattlelab.com (204.29.31.1)

WWW:    www.seattlelab.com (204.29.31.1)

For Check or Money Order orders send to:

Seattle Lab

214 First St.

Kirkland, WA 98033

U.S.A.

European representatives:

Vertrieb, Support und Registrierung fr Deutschland,

sterreich und Schweiz:

phs EDV Beratung

Michael Penzkofer

Davenstedter Str. 10

D-30449 Hannover

Tel.: 49-511-444 193

Fax : 49-511-458 1524

BBS : 49-511-455 011  (300..14400 Bd)

Prodaja in podpora v Sloveniji:

Prodajo in podporo v Sloveniji izvaja Miha Peternel osebno.

Cena emulatorja je 2500 SIT, placa se po povzetju. V paket

ni vkljucen vmesnik za prikljucitev Commodorjevih zunanjih

enot, vendar dodam navodilo za izdelavo le-tega. Za

informacije in narocila uporabite enega od spodaj navedenih

naslovov.

Author:

In case you would like to contact me you can choose among

the addresses listed below. I prefer E-mail, because it's

the cheapest, the fastest and I will always get the messages

if addressed to both E-mail addresses. I am looking forward

to hearing ANY ideas on how to improve the emulator

(software and documentation). Also, feel free to ask any

questions.

E-mail: (preferred)

miha@hermes.si

miha.peternel@abm.gn.apc.org

E-mail messages to both addresses if possible.

Phone:

+386-64-311-959 (fax/modem on demand)

8-11 PM, Central European Time

Snail mail:

Miha Peternel

Pot v Bitnje 66

SI-64000 Kranj

Slovenia

Special greets go to: Marko Mkel, Uros Platise, Udo van

den Heuvel, Jeff Gilmour, Gert Ensing, Ulrik/Surprise!

Productions and Gore/Future Crew.

______________

CONTENTS

1. Introduction

Basic C64S information.

2. Configuration

Hardware configuration guide.

3. First Steps

Step by step introduction to C64S system.

4. In Action

The differences between real and emulated C64.

5. Emulator Desktop

Detailed description of desktop and built-in tools.

6. Tips

Trouble shooting and tailoring emulation.

7. Compatibility

Detailed compatibility list.

8. File Transfers

A guide on file transfers with C64S utilities.

9. Contacts

Getting in touch with representatives and the author.