so, here we go with beta release 3 of the winice dumper.

i added support for dumping from 16 bit ring3, 32 bit ring0  and V86 code,

that is you can dump (or at least try to ;-) from all execution modes you

can come across in win95.

the other feature which i've added in this release is the 'emulation' of

the original PAGEIN command which solves another problem as well: if some

pages were not present in the area you were trying to dump, you would end

up in a crash. not anymore... hopefully. the previously mentioned

'emulation' means that if you dump a 1 byte 'long' area, then you basically

have the original PAGEIN functionality (but you still have to specify a

VALID path/filename). in fact, you've got even more since you can now use

it in 16 bit ring3 and V86 code as well (the original PAGEIN command

supported 32 bit ring3 and ring0 'only').

anyway, there're some features to test, some come to my mind are:

1. some general notes: always make sure that interrupts are enabled,

otherwise the dumper wont work. if for some reason they are disabled

you can try to change IF manually from winice before issuing the dumper

command, however i dont know whether it will help or not (of course

the dumper routine will be executed, however consider that the program

you're debugging did have a reason to disable interrupts...). anyway,

experiment ;-)

2. i had some problems with the expression evaluator when in 16 bit mode i

specified CS:IP as the start address. i definitely did dump something,

however it was not what i wanted. in other words the start address

was calculated incorrectly (the dumper itself always uses a flat 32

bit start address). the only way i found to get around this is:

PAGEIN flat(cs:ip) 12345 c:\dump.bim

(i.e. use the flat() operator). the same problem can occur with any

strange address specifications, if you had problems with incorrectly

calculated addresses, try out flat().

3. when you try to dump from 16 bit mode (try MSHEARTS.EXE ;-), think twice

before you specify the size of the area you want to dump. 'cos segments

are 16 bit (64k) and you can easily specify a length, which in linear

address space will point outside of a valid segment (i.e. commited memory).

when you are in V86 mode, probably it's a bad idea to try to dump the

video memory... so don't get insaitable ;-)

4. testing in ring0: besides making sure that interrupts enabled, somehow

you should ensure that you're not trying to re-enter the VMM or IFSMgr.

there's no straightforward way to do it if you just blindly pressed

ctrl-d and landed in winice somewhere in the middle of the kernel or

whatever, but if you're debugging a VXD you know doesnt use IFSmgr then

you're probably safe. again, experiment and let me know when it worked and

when it didnt. btw, VMM re-entrancy seems to be of little importance, 'cos

the VMM calls itself many times, so if you see it happening in the code

you're debugging, then it's probably safe to use the dumper as well.

5. the only commands which i 'killed' is 'ver ice' (the original PAGEIN is

'emulated'). if you encounter problems with something else, then it's

probably not my fault ;-). anyway, do let me know about it.