Chapter 4

Topical Cross-reference for Processor Instructions	50

Processor

Interpreting Processor Instructions	53

Flags	53

Syntax	54

Examples	54

Clock Speeds	54

Timings on the 8088 and 8086 Processors	55

Timings on the 80286-80486 Processors	56

Interpreting Encodings	56

Interpreting 80386–80486 Encoding Extensions	59

16-bit Encoding	60

32-bit Encoding	60

Address-Size Prefix	60

Operand-Size Prefix	60

Encoding Differences for 32-Bit Operations	60

Scaled Index Base Byte	61

Instructions	64

Topical Cross-reference for Processor Instructions

#### Arithmetic

| ADC | ADD | DEC |
| --- | --- | --- |
| DIV | IDIV | IMUL |
| INC | MUL | NEG |
| SBB | SUB | XADD# |

#### BCD Conversion

| AAA | AAD | AAM |
| --- | --- | --- |
| AAS | DAA | DAS |

#### Bit Operations

| AND | BSF§ | BSR§ |
| --- | --- | --- |
| BT§ | BTC§ | BTR§ |
| BTS§ | NOT | OR |
| RCL | RCR | ROL |
| ROR | SAR | SHL/SAL |
| SHLD§ | SHR | SHRD§ |
| XOR |  |  |

#### Compare

| BT§ | BTC§ | BTR§ |
| --- | --- | --- |
| BTS§ | CMP | CMPS |
| CMPXCHG# | TEST |  |

#### Conditional Set

| SETA/SETNBE§ | SETAE/SETNB§ | SETB/SETNAE§ |
| --- | --- | --- |
| SETBE/SETNA§ | SETC§ | SETE/SETZ§ |
| SETG/SETNLE§ | SETGE/SETNL§ | SETL/SETNGE§ |
| SETLE/SETNG§ | SETNC§ | SETNE/SETNZ§ |
| SETNO§ | SETNP/SETPO§ | SETNS§ |
| SETO§ | SETP/SETPE§ | SETS§ |

* 80186–80486 only.	† 80286–80486 only.

§ 80386–80486 only.	# 80486 only.

Conditional Transfer

| BOUND* | INTO | JA/JNBE |
| --- | --- | --- |
| JAE/JNB | JB/JNAE | JBE/JNA |
| JC | JCXZ/JECXZ | JE/JZ |
| JG/JNLE | JGE/JNL | JL/JNGE |
| JLE/JNG | JNC | JNE/JNZ |
| JNO | JNP/JPO | JNS |
| JO | JP/JPE | JS |

#### Data Transfer

| BSWAP# | CMPXCHG# | LDS/LES |
| --- | --- | --- |
| LEA | LFS/LGS/LSS§ | LODS |
| MOV | MOVS | MOVSX§ |
| MOVZX§ | STOS | XADD# |
| XCHG | XLAT/XLATB |  |

#### Flag

| CLC | CLD | CLI |
| --- | --- | --- |
| CMC | LAHF | POPF |
| PUSHF | SAHF | STC |
| STD | STI |  |

#### Input/Output

| IN | INS* |
| --- | --- |
| OUT | OUTS* |

#### Loop

| JCXZ/JECXZ | LOOP |
| --- | --- |
| LOOPE/LOOPZ | LOOPNE/LOOPNZ |

* 80186–80486 only.	† 80286–80486 only.

§ 80386–80486 only.	# 80486 only.

Process Control

| ARPL† | CLTS† | LAR† |
| --- | --- | --- |
| LGDT/LIDT/LLDT† | LMSW† | LSL† |
| LTR† | SGDT/SIDT/SLDT† | SMSW† |
| STR† | VERR† | VERW† |
| MOV special§ | INVD# | INVLPG# |
| WBINVD# |  |  |

#### Processor Control

| HLT | LOCK |
| --- | --- |
| NOP | WAIT |

#### Stack

| PUSH | PUSHF | PUSHA* |
| --- | --- | --- |
| PUSHAD* | POP | POPF |
| POPA* | POPAD* | ENTER* |
| LEAVE* |  |  |

#### String

| MOVS | LODS | STOS |
| --- | --- | --- |
| SCAS | CMPS | INS* |
| OUTS* | REP | REPE/REPZ |
| REPNE/REPNZ |  |  |

#### Type Conversion

| CBW | CWD |
| --- | --- |
| CWDE§ | CDQ§ |
| BSWAP# |  |

#### Unconditional Transfer

| CALL | INT | IRET |
| --- | --- | --- |
| RET | RETN/RETF | JMP |

* 80186–80486 only.	† 80286–80486 only.

§ 80386–80486 only.	# 80486 only.

tc "Processor"§tc "Interpreting Processor Instructions"§Interpreting Processor Instructions

The following sections explain the format of instructions for the 8086, 8088, 80286, 80386, and 80486 processors. Those instructions begin on page 64.

## Flags

Only the flags common to all processors are shown. If none of the flags is affected by the instruction, the flag line says No change. If flags can be affected, a two-line entry is shown. The first line shows flag abbreviations as follows:

| Abbreviation | Flag |
| --- | --- |
| O | Overflow |
| D | Direction |
| I | Interrupt |
| T | Trap |
| S | Sign |
| Z | Zero |
| A | Auxiliary carry |
| P | Parity |
| C | Carry |

The second line has codes indicating how the flag can be affected:

| Code | Effect |
| --- | --- |
| 1 | Sets the flag |
| 0 | Clears the flag |
| ? | May change the flag, but the value is not predictable |
| blank | No effect on the flag |
| ± | Modifies according to the rules associated with the flag |

Syntax

Each encoding variation may have different syntaxes corresponding to different addressing modes. The following abbreviations are used:

reg    A general-purpose register of any size.

segreg    One of the segment registers: DS, ES, SS, or CS (also FS or GS on the 80386–80486).

accum    An accumulator register of any size: AL or AX (also EAX on the 80386–80486).

mem    A direct or indirect memory operand of any size.

label    A labeled memory location in the code segment.

src,dest    A source or destination memory operand used in a string operation.

immed    A constant operand.

In some cases abbreviations have numeric suffixes to specify that the operand must be a particular size. For example, reg16 means that only a 16-bit (word) register is accepted.

#### Examples

One or more examples are shown for each syntax. Their position is not related to the clock speeds in the right column.

## Clock Speeds

Column 3 shows the clock speeds for each processor. Sometimes an instruction may have more than one clock speed. Multiple speeds are separated by commas. If several speeds are part of an expression, they are enclosed in parentheses. The following abbreviations are used to specify variations:

EA    Effective address. This applies only to the 8088 and 8086 processors, as described in the next section.

b,w,d    Byte, word, or doubleword operands.

pm    Protected mode.

n    Iterations. Repeated instructions may have a base number of clocks plus a number of clocks for each iteration. For example, 8+4n means 8 clocks plus 4 clocks for each iteration.

noj    No jump. For conditional jump instructions, noj indicates the speed if the condition is false and the jump is not taken.

m    Next instruction components. Some control transfer instructions take different times depending on the length of the next instruction executed. On the 8088 and 8086, m is never a factor. On the 80286, m is the number of bytes in the instruction. On the 80386–80486, m is the number of components. Each byte of encoding is a component, and the displacement and data are separate components.

W88,88    8088 exceptions. See “Timings on the 8088 and 8086 Processors,” following.

Clocks can be converted to nanoseconds by dividing 1 microsecond by the number of megahertz (MHz) at which the processor is running. For example, on a processor running at 8 MHz, 1 clock takes 125 nanoseconds (1000 MHz per nanosecond / 8 MHz).

The clock counts are for best-case timings. Actual timings vary depending on wait states, alignment of the instruction, the status of the prefetch queue, and other factors.

### Timings on the 8088 and 8086 Processors

Because of its 8-bit data bus, the 8088 always requires two fetches to get a 16-bit operand. Therefore, instructions that work on 16-bit memory operands take longer on the 8088 than on the 8086. Separate 8088 timings are shown in parentheses following the main timing. For example, 9 (W88=13) means that the 8086 with any operands or the 8088 with byte operands take 9 clocks, but the 8088 with word operands takes 13 clocks. Similarly, 16 (88=24) means that the 8086 takes 16 clocks, but the 8088 takes 24 clocks.

On the 8088 and 8086, the effective address (EA) value must be added for instructions that operate on memory operands. A displacement is any direct memory or constant operand, or any combination of the two. The following shows the number of clocks to add for the effective address:

| Components | EA Clocks | Examples |
| --- | --- | --- |
| Displacement | 6 | mov ax,stuff mov ax,stuff+2 |
| Base or index | 5 | mov ax,[bx] mov ax,[di] |
| Displacement plus base or index | 9 | mov ax,[bp+8] mov ax,stuff[di] |
| Base plus index (BP+DI, BX+SI) | 7 | mov ax,[bx+si] mov ax,[bp+di] |

| Components | EA Clocks | Examples |
| --- | --- | --- |
| Base plus index (BP+SI, BX+DI) | 8 | mov ax,[bx+di] mov ax,[bp+si] |
| Base plus index plus displacement (BP+DI+disp, BX+SI+disp) | 11 | mov ax,stuff[bx+si] mov ax,[bp+di+8] |
| Base plus index plus displacement (BP+SI+disp, BX+DI+disp) | 12 | mov ax,stuff[bx+di] mov ax,[bp+si+20] |
| Segment override | EA+2 | mov ax,es:stuff mov ax,ds:[bp+10] |

### Timings on the 80286–80486 Processors

On the 80286–80486 processors, the effective address calculation is handled by hardware and is therefore not a factor in clock calculations except in one case. If a memory operand includes all three possible elements — a displacement, a base register, and an index register — then add one clock. On the 80486, the extra clock is not always used. Examples are shown in the following.

| mov ax,[bx+di] | ;No extra |
| --- | --- |
| mov ax,array[bx+di] | ;One extra |
| mov ax,[bx+di+6] | ;One extra |

80186 and 80188 timings are different from 8088, 8086, and 80286 timings. They are not shown in this manual. Timings are also not shown for protected-mode transfers through gates or for the virtual 8086 mode available on the 80386–80486 processors.

Note

# Interpreting Encodings

Encodings are shown for each variation of the instruction. This section describes encoding for all processors except the 80386–80486. The encodings take the form of boxes filled with 0s and 1s for bits that are constant for the instruction variation, and abbreviations (in italics) for the following variable bits or bitfields:

d    Direction bit. If set, do memory to register; the reg field is the destination. If clear, do register to memory or register to register; the reg field is the source.

a    Accumulator direction bit. If set, move accumulator register to memory. If clear, move memory to accumulator register.

w    Word/byte bit. If set, use 16-bit or 32-bit operands. If clear, use 8-bit operands.

s    Sign bit. If set, sign-extend 8-bit immediate data to 16 bits.

mod    Mode. This 2-bit field gives the register/memory mode with displacement. The possible values are shown below:

| mod | Meaning |
| --- | --- |
| 00 | This value can have two meanings: If r/m is 110, a direct memory operand is used. If r/m is not 110, the displacement is 0 and an indirect memory operand is used. The operand must be based, indexed, or based indexed. |
| 01 | An indirect memory operand is used with an 8-bit displacement. |
| 10 | An indirect memory operand is used with a 16-bit displacement. |
| 11 | A two-register instruction is used; the reg field specifies the destination and the r/m field specifies the source. |

reg    Register. This 3-bit field specifies one of the general-purpose registers:

| reg | 16/32-bit if w=1 | 8-bit if w=0 |
| --- | --- | --- |
| 000 | AX/EAX | AL |
| 001 | CX/ECX | CL |
| 010 | DX/EDX | DL |
| 011 | BX/EBX | BL |
| 100 | SP/ESP | AH |
| 101 | BP/EBP | CH |
| 110 | SI/ESI | DH |
| 111 | DI/EDI | BH |

The reg field is sometimes used to specify encoding information rather than a register.

sreg    Segment register. This field specifies one of the segment registers:

| sreg | Register |
| --- | --- |
| 000 | ES |
| 001 | CS |
| 010 | SS |
| 011 | DS |
| 100 | FS |
| 101 | GS |

r/m    Register/memory. This 3-bit field specifies a register or memory r/m operand.

If the mod field is 11, r/m specifies the source register using the reg field codes. Otherwise, the field has one of the following values:

| r/m | Operand Address |
| --- | --- |
| 000 | DS:[BX+SI+disp] |
| 001 | DS:[BX+DI+disp] |
| 010 | SS:[BP+SI+disp] |
| 011 | SS:[BP+DI+disp] |
| 100 | DS:[SI+disp] |
| 101 | DS:[DI+disp] |
| 110 | SS:[BP+disp]* |
| 111 | DS:[BX+disp] |

* If mod is 00 and r/m is 110, then the operand is treated as a direct memory operand. This means that the operand [BP] is encoded as [BP+0] rather than having a short-form like other register indirect operands. Encoding [BX] takes one byte, but encoding [BP] takes two.

disp    Displacement. These bytes give the offset for memory operands. The possible lengths (in bytes) are shown in parentheses.

data    Data. These bytes give the actual value for constant values. The possible lengths (in bytes) are shown in parentheses.

If a memory operand has a segment override, the entire instruction has one of the following bytes as a prefix:

| Prefix | Segment |
| --- | --- |
| 00101110 (2Eh) | CS |
| 00111110 (3Eh) | DS |
| 00100110 (26h) | ES |
| 00110110 (36h) | SS |
| 01100100 (64h) | FS |
| 01100101 (65h) | GS |

#### Example

As an example, assume you want to calculate the encoding for the following statement (where warray is a 16-bit variable):

add     warray[bx+di], -3

First look up the encoding for the immediate-to-memory syntax of the ADD instruction:

100000sw  mod,000,r/m  disp (0, 1, or 2)  data (0, 1, or 2)

Since the destination is a word operand, the w bit is set. The 8-bit immediate data must be sign-extended to 16 bits to fit into the operand, so the s bit is also set. The first byte of the instruction is therefore 10000011 (83h).

Since the memory operand can be anywhere in the segment, it must have a 16-bit offset (displacement). Therefore the mod field is 10. The reg field is 000, as shown in the encoding. The r/m coding for [bx+di+disp] is 001. The second byte is 10000001 (81h).

The next two bytes are the offset of warray. The low byte of the offset is stored first and the high byte second. For this example, assume that warray is located at offset 10EFh.

The last byte of the instruction is used to store the 8-bit immediate value –3 (FDh). This value is encoded as 8 bits (but sign-extended to 16 bits by the processor).

The encoding is shown here in hexadecimal:

83 81 EF 10 FD

You can confirm this by assembling the instruction and looking at the resulting assembly listing.

# Interpreting 80386–80486 Encoding Extensions

This book shows 80386–80486 encodings for instructions that are available only on the 80386–80486 processors. For other instructions, encodings are shown only for the 16-bit subset available on all processors. This section tells how to convert the 80286 encodings shown in the book to 80386–80486 encodings that use extensions such as 32-bit registers and memory operands.

The extended 80386–80486 encodings differ in that they can have additional prefix bytes, a Scaled Index Base (SIB) byte, and 32-bit displacement and immediate bytes. Use of these elements is closely tied to the segment word size. The use type of the code segment determines whether the instructions are processed in 32-bit mode (USE32) or 16-bit mode (USE16). Current versions of MS-DOS® and Microsoft® Windows™ use 16-bit mode only. Windows NT uses 32-bit mode.

The bytes that can appear in an instruction encoding are:

16-Bit Encoding

| Opcode | mod-reg-r/m | disp | immed |
| --- | --- | --- | --- |
| (1-2) | (0-1) | (0-2) | (0-2) |

## 32-Bit Encoding

| Address-Size (67h) | Operand-Size (66h) | Opcode | mod-reg-r/m | Scaled Index Base | disp | immed |
| --- | --- | --- | --- | --- | --- | --- |
| (0-1) | (0-1) | (1-2) | (0-1) | (0-1) | (0-4) | (0-4) |

Additional bytes may be added for a segment prefix, a repeat prefix, or the LOCK prefix.

### Address-Size Prefix

The address-size prefix determines the segment word size of the operation. It can override the default size for calculating the displacement of memory addresses. The address prefix byte is 67h. The assembler automatically inserts this byte where appropriate.

In 32-bit mode (USE32 or FLAT code segment), displacements are calculated as 32-bit addresses. The effective address-size prefix must be used for any instructions that must calculate addresses as 16-bit displacements. In 16-bit mode, the defaults are reversed. The prefix must be used to specify calculation of 32-bit displacements.

### Operand-Size Prefix

The operand-size prefix determines the size of operands. It can override the default size of registers or memory operands. The operand-size prefix byte is 66h. The assembler automatically inserts this byte where appropriate.

In 32-bit mode, the default sizes for operands are 8 bits and 32 bits (depending on the w bit). For most instructions, the operand-size prefix must be used for any instructions that use 16-bit operands. In 16-bit mode, the default sizes are 8 bits and 16 bits. The prefix must be used for any instructions that use 32-bit operands. Some instructions use 16-bit operands, regardless of mode.

### Encoding Differences for 32-Bit Operations

When 32-bit operations are performed, the meaning of certain bits or fields is different from their meaning in 16-bit operations. The changes may affect default operations in 32-bit mode, or 16-bit mode operations in which the address-size prefix or the operand-size prefix is used. The following fields may have a different meaning for 32-bit operations from their meaning as described in the “Interpreting Encodings” section:

w    Word/byte bit. If set, use 32-bit operands. If clear, use 8-bit operands.

s    Sign bit. If set, sign-extend 8-bit and 16-bit immediate data to 32 bits.

mod    Mode. This field indicates the register/memory mode. The value 11 still indicates a register-to-register operation with r/m containing the code for a 32-bit source register. However, other codes have different meanings as shown in the tables in the next section.

reg    Register. The codes for 16-bit registers are extended to 32-bit registers. For example, if the reg field is 000, EAX is used instead of AX. Use of 8-bit registers is unchanged.

sreg    Segment register. The 80386 has the following additional segment registers:

| sreg | Register |
| --- | --- |
| 100 | FS |
| 101 | GS |

r/m    Register/memory. If the r/m field is used for the source register, 32-bit registers are used as for the reg field. If the field is used for memory operands, the meaning is completely different from the meaning used for 16-bit operations, as shown in the tables in the next section.

disp    Displacement. This field is 4 bytes for 32-bit addresses.

data    Data. Immediate data can be up to 4 bytes.

### Scaled Index Base Byte

Many 80386–80486 extended memory operands are too complex to be represented by a single mod-reg-r/m byte. For these operands, a value of 100 in the r/m field signals the presence of a second encoding byte called the Scaled Index Base (SIB) byte. The SIB byte is made up of the following fields:

ss  index  base

ss    Scaling Field. This two-bit field specifies one of the following scaling factors:

| ss | Scale |
| --- | --- |
| 00 | 1 |
| 01 | 2 |
| 10 | 4 |
| 11 | 8 |

index    Index Register. This three-bit field specifies one of the following index registers:

| index | Register |
| --- | --- |
| 000 | EAX |
| 001 | ECX |
| 010 | EDX |
| 011 | EBX |
| 100 | no index |
| 101 | EBP |
| 110 | ESI |
| 111 | EDI |

ESP cannot be an index register. If the index field is 100, the ss field 
must be 00.

Note

base    Base Register. This 3-bit field combines with the mod field to specify the base register and the displacement. Note that the base field only specifies the base when the r/m field is 100. Otherwise, the r/m field specifies the base.

The possible combinations of the mod, r/m, scale, index, and base fields are as follows:

§

If a memory operand has a segment override, the entire instruction has one of the prefixes discussed in the preceding section, “Interpreting Encodings,” or one of the following prefixes for the segment registers available only on the 80386–80486:

| Prefix | Segment |
| --- | --- |
| 01100100 (64h) | FS |
| 01100101 (65h) | GS |

#### Example

Assume you want to calculate the encoding for the following statement (where warray is a 16-bit variable). Assume that the instruction is used in 16-bit mode.

add     warray[eax+ecx*2], -3

First look up the encoding for the immediate-to-memory syntax of the ADD instruction:

100000sw  mod,000,r/m  disp (0, 1, or 2)	  data (1 or 2)

This encoding must be expanded to account for 80386–80486 extensions. Note that the instruction operates on 16-bit data in a 16-bit mode program. Therefore, the operand-size prefix is not needed. However, the instruction does use 32-bit registers to calculate a 32-bit effective address. Thus the first byte of the encoding must be the effective address-size prefix, 01100111 (67h).

The opcode byte is the same (83h) as for the 80286 example described in the “Interpreting Encodings” section.

The mod-reg-r/m byte must specify a based indexed operand with a scaling factor of two. This operand cannot be specified with a single byte, so the encoding must also use the SIB byte. The value 100 in the r/m field specifies an SIB byte. The reg field is 000, as shown in the encoding. The mod field is 10 for operands that have base and scaled index registers and a 32-bit displacement. The combined mod, reg, and r/m fields for the second byte are 10000100 (84h).

The SIB byte is next. The scaling factor is 2, so the ss field is 01. The index register is ECX, so the index field is 001. The base register is EAX, so the base field is 000. The SIB byte is 01001000 (48h).

The next 4 bytes are the offset of warray. The low bytes are stored first. For this example, assume that warray is located at offset 10EFh. This offset only requires 2 bytes, but 4 must be supplied because of the addressing mode. A 32-bit address can be safely used in 16-bit mode as long as the upper word is 0.

The last byte of the instruction is used to store the 8-bit immediate value –3 (FDh). The encoding is shown here in hexadecimal:

67 83 84 48 00 00 EF 10 FD

# Instructions

This section provides an alphabetical reference to the instructions for the 8086, 8088, 80286, 80386, and 80486 processors.

AAA    ASCII Adjust After Addition

Adjusts the result of an addition to a decimal digit (0–9). The previous addition instruction should place its 8-bit sum in AL. If the sum is greater than 9h, AH is incremented and the carry and auxiliary carry flags are set. Otherwise, the carry and auxiliary carry flags are cleared.

O	D	I	T	S	Z	A	P	C
?				?	?	±	?	±

Flags

00110111

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| AAA | aaa | 88/86 286 386 486 | 8 3 4 3 |

AAD    ASCII Adjust Before Division

Converts unpacked BCD digits in AH (most significant digit) and AL (least significant digit) to a binary number in AX. This instruction is often used to prepare an unpacked BCD number in AX for division by an unpacked BCD digit in an 8-bit register.

O	D	I	T	S	Z	A	P	C
?				±	±	?	±	?

Flags

11010101    00001010

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| AAD | aad | 88/86 286 386 486 | 60 14 19 14 |

AAM    ASCII Adjust After Multiply

Converts an 8-bit binary number less than 100 decimal in AL to an unpacked BCD number in AX. The most significant digit goes in AH and the least significant in AL. This instruction is often used to adjust the product after a MUL instruction that multiplies unpacked BCD digits in AH and AL. It is also used to adjust the quotient after a DIV instruction that divides a binary number less than 100 decimal in AX by an unpacked BCD number.

O	D	I	T	S	Z	A	P	C
?				±	±	?	±	?

Flags

11010100    00001010

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| AAM | aam | 88/86 286 386 486 | 83 16 17 15 |

AAS    ASCII Adjust After Subtraction

Adjusts the result of a subtraction to a decimal digit (0–9). The previous subtraction instruction should place its 8-bit result in AL. If the result is greater than 9h, AH is decremented and the carry and auxiliary carry flags are set. Otherwise, the carry and auxiliary carry flags are cleared.

O	D	I	T	S	Z	A	P	C
?				?	?	±	?	±

Flags

00111111

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| AAS | aas | 88/86 286 386 486 | 8 3 4 3 |

ADC    Add with Carry

Adds the source operand, the destination operand, and the value of the carry flag. The result is assigned to the destination operand. This instruction is used to add the more significant portions of numbers that must be added in multiple registers.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

000100dw    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| ADC reg,reg | adc dx,cx | 88/86 286 386 486 | 3 2 2 1 |
| ADC mem,reg | adc WORD PTR m32[2],dx | 88/86 286 386 486 | 16+EA (W88=24+EA) 7 7 3 |
| ADC reg,mem | adc dx,WORD PTR m32[2] | 88/86 286 386 486 | 9+EA (W88=13+EA) 7 6 2 |

100000sw    mod, 010,r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| ADC reg,immed | adc dx,12 | 88/86 286 386 486 | 4 3 2 1 |
| ADC mem,immed | adc WORD PTR m32[2],16 | 88/86 286 386 486 | 17+EA (W88=23+EA) 7 7 3 |

0001010w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ADC accum,immed | adc ax,5 |  | 88/86 286 386 486 |  | 4 3 2 1 |  |

ADD    Add

Adds the source and destination operands and puts the sum in the destination operand.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

000000dw    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ADD reg,reg | add ax,bx |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| ADD mem, reg | add total, cx add array[bx+di], dx |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 7 3 |  |
| ADD reg,mem | add cx,incr add dx,[bp+6] |  | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 7 6 2 |  |

100000sw    mod, 000,r/m    disp (p,1, or2)    data (1or2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ADD reg,immed | add bx,6 |  | 88/86 286 386 486 |  | 4 3 2 1 |  |
| ADD mem,immed | add amount,27 add pointers[bx][si],6 |  | 88/86 286 386 486 |  | 17+EA (W88=23+EA) 7 7 3 |  |

0000010w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ADD accum,immed | add ax,10 |  | 88/86 286 386 486 |  | 4 3 2 1 |  |

AND    Logical AND

Performs a bitwise AND operation on the source and destination operands and stores the result in the destination operand. For each bit position in the operands, if both bits are set, the corresponding bit of the result is set. Otherwise, the corresponding bit of the result is cleared.

O	D	I	T	S	Z	A	P	C
0				±	±	?	±	0

Flags

001000dw    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| AND reg,reg | and dx,bx |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| AND mem,reg | and bitmask,bx and [bp+2],dx |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 7 3 |  |
| AND reg,mem | and bx,masker and dx,marray[bx+di] |  | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 7 6 2 |  |

100000sw    mod, 100, r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| AND reg,immed | and dx,0F7h |  | 88/86 286 386 486 |  | 4 3 2 1 |  |
| AND mem,immed | and masker, 100lb |  | 88/86 286 386 486 |  | 17+EA(W88=24+EA) 7 7 3 |  |

0010010w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| AND accum,immed | and ax,0B6h |  | 88/86 286 386 486 |  | 4 3 2 1 |  |

ARPL    Adjust Requested Privilege Level

80286–80486 Protected Only    Verifies that the destination Requested Privilege Level (RPL) field (bits 0 and 1 of a selector value) is less than the source RPL field. If it is not, ARPL adjusts the destination RPL to match the source RPL. The destination operand should be a 16-bit memory or register operand containing the value of a selector. The source operand should be a 16-bit register containing the test value. The zero flag is set if the destination is adjusted; otherwise, the flag is cleared. ARPL is useful only in 80286–80486 protected mode. See Intel documentation for details on selectors and privilege levels.

O	D	I	T	S	Z	A	P	C
					±

Flags

01100011    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ARPL reg,reg | arpl ax,cx |  | 88/86 286 386 486 |  | — 10 20 9 |  |
| ARPL mem,reg | arpl selector,dx |  | 88/86 286 386 486 |  | — 11 21 9 |  |

BOUND    Check Array Bounds

80286–80486 Only    Verifies that a signed index value is within the bounds of an array. The destination operand can be any 16-bit register containing the index to be checked. The source operand must then be a 32-bit memory operand in which the low and high words contain the starting and ending values, respectively, of the array. (On the 80386–80486 processors, the destination operand can be a 32-bit register; in this case, the source operand must be a 64-bit operand made up of 32-bit bounds.) If the source operand is less than the first bound or greater than the last bound, an interrupt 5 is generated. The instruction pointer pushed by the interrupt (and returned by IRET) points to the BOUND instruction rather than to the next instruction.

No change

Flags

01100010    mod,reg, r/m    disp (2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| BOUND reg16,mem32 BOUND reg32,mem64* | bound di,base-4 | 88/86 286 386 486 | — noj=13† noj=10† noj=7 |

* 80386–80486 only.

† See INT for timings if interrupt 5 is called.

BSF/BSR    Bit Scan

80386–80486 Only    Scans an operand to find the first set bit. If a set bit is found, the zero flag is cleared and the destination operand is loaded with the bit index of the first set bit encountered. If no set bit is found, the zero flag is set. BSF (Bit Scan Forward) scans from bit 0 to the most significant bit. BSR (Bit Scan Reverse) scans from the most significant bit of an operand to bit 0.

O	D	I	T	S	Z	A	P	C
					±

Flags

00001111    10111100    mod, reg, r/m    disp (0, 1, 2, or 4)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| BSF reg16,reg16 BSF reg32,reg32 | bsf cx,bx | 88/86 286 386 486 | — — 10+3n* 6–42† |
| BSF reg16,mem16 BSF reg32,mem32 | bsf ecx,bitmask | 88/86 286 386 486 | — — 10+3n* 7–43§ |

00001111    10111101    mod, reg, r/m    disp (0, 1, 2, or 4)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| BSR reg16,reg16 BSR reg32,reg32 | bsr cx,dx |  | 88/86 286 386 486 |  | — — 10+3n* 103 – 3n# |  |
| BSR reg16,mem16 BSR reg32,mem32 | bsr eax,bitmask |  | 88/86 286 386 486 |  | — — 10+3n* 104 – 3n# |  |

* n = bit position from 0 to 31.
clocks = 6 if second operand equals 0.

† Clocks = 8 +
                4 for each byte scanned +
                3 for each nibble scanned +
                3 for each bit scanned in last nibble
                   or 6 if second operand equals 0.

§ Same as footnote above, but add 1 clock.

# n = bit position from 0 to 31.
clocks = 7 if second operand equals 0.

BSWAP    Byte Swap

80486 Only    Takes a single 32-bit register as operand and exchanges the first byte with the fourth, and the second byte with the third. This instruction does not alter any bit values within the bytes and is useful for quickly translating between 8086-family byte storage and storage schemes in which the high byte is stored first.

No change

Flags

00001111    11001 reg

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| BSWAP reg32 | bswap eax bswap ebx |  | 88/86 286 386 486 |  | — — — 1 |  |

BT/BTC/BTR/BTS    Bit Tests

80386–80486 Only    Copies the value of a specified bit into the carry flag, where it can be tested by a JC or JNC instruction. The destination operand specifies the value in which the bit is located; the source operand specifies the bit position. BT simply copies the bit to the flag. BTC copies the bit and complements (toggles) it in the destination. BTR copies the bit and resets (clears) it in the destination. BTS copies the bit and sets it in the destination.

O	D	I	T	S	Z	A	P	C
								±

Flags

00001111    10111010    mod, BBB*,r/m    disp (0, 1, 2, or 4)    data (1)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| BT reg16,immed8† | bt ax,4 | 88/86 286 386 486 | — — 3 3 |
| BTC reg16,immed8† BTR reg16,immed8† BTS reg16,immed8† | bts ax,4 btr bx,17 btc edi,4 | 88/86 286 386 486 | — — 6 6 |
| BT mem16,immed8† | btr DWORD PTR [si],27 btc color[di],4 | 88/86 286 386 486 | — — 6 3 |
| BTC mem16,immed8† BTR mem16,immed8† BTS mem16,immed8† | btc DWORD PTR [bx],27 btc maskit,4 btr color[di],4 | 88/86 286 386 486 | — — 8 8 |

00001111    10BBB011*    mod, reg, r/m    disp (0, 1, 2, or 4)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| BT reg16,reg16† | bt ax,bx | 88/86 286 386 486 | — — 3 3 |
| BTC reg16,reg16† BTR reg16,reg16† BTS reg16,reg16† | btc eax,ebx bts bx,ax btr cx,di | 88/86 286 386 486 | — — 6 6 |

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| BT mem16,reg16† | bt [bx],dx | 88/86 286 386 486 | — — 12 8 |
| BTC mem16,reg16† BTR mem16,reg16† BTS mem16,reg16† | bts flags[bx],cx btr rotate,cx btc [bp+8],si | 88/86 286 386 486 | — — 13 13 |

* BBB is 100 for BT, 111 for BTC, 110 for BTR, and 101 for BTS.

† Operands also can be 32 bits (reg32 and mem32).

CALL    Call Procedure

Calls a procedure. The instruction pushes the address of the next instruction onto the stack and jumps to the address specified by the operand. For NEAR calls, the offset (IP) is pushed and the new offset is loaded into IP.

For FAR calls, the segment (CS) is pushed and the new segment is loaded into CS. Then the offset (IP) is pushed and the new offset is loaded into IP. A subsequent RET instruction can pop the address so that execution continues with the instruction following the call.

No change

Flags

11101000    disp (2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CALL label | call upcase | 88/86 286 386 486 | 19 (88=23) 7+m 7+m 3 |

10011010    disp (4)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CALL label | call FAR PTR job call distant | 88/86 286 386 486 | 28 (88=36) 13+m,pm=26+m* 17+m,pm=34+m* 18,pm=20* |

11111111    mod,010,r/m

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CALL reg | call ax | 88/86 286 386 486 | 16 (88=20) 7+m 7+m 5 |
| CALL mem16 | call pointer | 88/86 | 21+EA (88=29+EA) |
| CALL mem32† | call [bx] | 286 386 486 | 11+m 10+m 5 |

11111111    mod,011,r/m

Encoding

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| CALL mem32 | call far_table[di] | 88/86 | 37+EA (88=53+EA) |  |
| CALL mem48† | call DWORD PTR [bx] | 286 386 486 | 16+m,pm=29+m* 22+m,pm=38+m* 17,pm=20* |  |

* Timings for calls through call and task gates are not shown, since they are used primarily in operating systems.

† 80386–80486 32-bit addressing mode only.

CBW    Convert Byte to Word

Converts a signed byte in AL to a signed word in AX by extending the sign bit of AL into all bits of AH.

No change

Flags

10011000*

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CBW | cbw | 88/86 286 386 486 | 2 2 3 3 |

* CBW and CWDE have the same encoding with two exceptions: in 32-bit mode, CBW is preceded by the operand-size byte (66h) but CWDE is not; in 16-bit mode, CWDE is preceded by the operand-size byte but CBW is not.

CDQ    Convert Double to Quad

80386–80486 Only    Converts the signed doubleword in EAX to a signed quadword in the EDX:EAX register pair by extending the sign bit of EAX into all bits of EDX.

No change

Flags

10011001*

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CDQ | cdq |  | 88/86 286 386 486 |  | — — 2 3 |  |

* CWD and CDQ have the same encoding with two exceptions: in 32-bit mode, CWD is preceded by the operand-size byte (66h) but CDQ is not; in 16-bit mode, CDQ is preceded by the operand-size byte but CWD is not.

CLC    Clear Carry Flag

Clears the carry flag.

O	D	I	T	S	Z	A	P	C
								0

Flags

11111000

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CLC | clc |  | 88/86 286 386 486 |  | 2 2 2 2 |  |

CLD    Clear Direction Flag

Clears the direction flag. All subsequent string instructions will process up (from low addresses to high addresses) by increasing the appropriate index registers.

O	D	I	T	S	Z	A	P	C
	0

Flags

11111100

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CLD | cld |  | 88/86 286 386 486 |  | 2 2 2 2 |  |

CLI    Clear Interrupt Flag

Clears the interrupt flag. When the interrupt flag is cleared, maskable interrupts are not recognized until the flag is set again with the STI instruction. In protected mode, CLI clears the flag only if the current task’s privilege level is less than or equal to the value of the IOPL flag. Otherwise, a general-protection fault occurs.

O	D	I	T	S	Z	A	P	C
		0

Flags

11111010

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CLI | cli |  | 88/86 286 386 486 |  | 2 3 3 5 |  |

CLTS    Clear Task-Switched Flag

80286–80486 Privileged Only    Clears the task-switched flag in the Machine Status Word (MSW) of the 80286, or the CR0 register of the 80386–80486. This instruction can be used only in system software executing at privilege level 0. See Intel documentation for details on the task-switched flag and other privileged-mode concepts.

No change

Flags

00001111    00000110

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CLTS | clts |  | 88/86 286 386 486 |  | — 2 5 7 |  |

CMC    Complement Carry Flag

Complements (toggles) the carry flag.

O	D	I	T	S	Z	A	P	C
								±

Flags

11110101

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CMC | cmc |  | 88/86 286 386 486 |  | 2 2 2 2 |  |

CMP    Compare Two Operands

Compares two operands as a test for a subsequent conditional-jump or set instruction. CMP does this by subtracting the source operand from the destination operand and setting the flags according to the result. CMP is the same as the SUB instruction, except that the result is not stored.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

001110dw    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CMP reg,reg | cmp di,bx cmp dl,cl | 88/86 286 386 486 | 3 2 2 1 |
| CMP mem,reg | cmp maximum,dx cmp array[si],bl | 88/86 286 386 486 | 9+EA (W88=13+EA) 7 5 2 |
| CMP reg,mem | cmp dx,minimum cmp bh,array[si] | 88/86 286 386 486 | 9+EA (W88=13+EA) 6 6 2 |

100000sw    mod, 111,r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CMP reg,immed | cmp bx,24 | 88/86 286 386 486 | 4 3 2 1 |
| CMP mem,immed | cmp WORD PTR [di],4 cmp tester,4000 | 88/86 286 386 486 | 10+EA (W88=14+EA) 6 5 2 |

0011110w    data (1 or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| CMP accum,immed | cmp ax,1000 | 88/86 286 386 486 |  | 4 3 2 1 |  |

CMPS/CMPSB/CMPSW/CMPSD    Compare String

Compares two strings. DS:SI must point to the source string and ES:DI must point to the destination string (even if operands are given). For each comparison, the destination element is subtracted from the source element and the flags are updated to reflect the result (although the result is not stored). DI and SI are adjusted according to the size of the operands and the status of the direction flag. They are increased if the direction flag has been cleared with CLD, or decreased if the direction flag has been set with STD.

If the CMPS form of the instruction is used, operands must be provided to indicate the size of the data elements to be processed. A segment override can be given for the source (but not for the destination). If CMPSB (bytes), CMPSW (words), or CMPSD (doublewords on the 80386–80486 only) is used, the instruction determines the size of the data elements to be processed.

CMPS and its variations are normally used with repeat prefixes. REPNE (or REPNZ) is used to find the first match between two strings. REPE (or REPZ) is used to find the first mismatch. Before the comparison, CX should contain the maximum number of elements to compare. After a REPNE CMPS, the zero flag is clear if no match was found. After a REPE CMPS, the zero flag is set if no mismatch was found.

When the instruction finishes, ES:DI and DS:SI point to the element that follows (if the direction flag is clear) or precedes (if the direction flag is set) the match or mismatch. If CX decrements to 0, ES:DI and DS:SI point to the element that follows or precedes the last comparison. The zero flag is set or clear according to the result of the last comparison, not according to the value of CX.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

1010011w

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CMPS [[segreg:]] src, [[ES:]] dest CMPSB [[[[segreg:[[ src, ]]ES:]] dest]] CMPSW [[[[segreg:[[ src, ]]ES:]] dest]] CMPSD [[[[segreg:[[ src, ]]ES:]] dest]] | cmps source,es:dest repne cmpsw repe cmpsb repne cmpsd | 88/86 286 386 486 | 22 (W88=30) 8 10 8 |

CMPXCHG    Compare and Exchange

80486 Only    Compares the destination operand to the accumulator (AL, AX, or EAX). If equal, the source operand is copied to the destination. Otherwise, the destination is copied to the accumulator. The instruction sets flags according to the result of the comparison.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

00001111    1011000b    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| CMPXCHG mem,reg | cmpxchg warr[bx],cx cmpxchg string,bl | 88/86 286 386 486 | — — — 7–10 |
| CMPXCHG reg,reg | cmpxchg dl,cl cmpxchg bx,dx | 88/86 286 386 486 | — — — 6 |

CWD    Convert Word to Double

Converts the signed word in AX to a signed doubleword in the DX:AX register pair by extending the sign bit of AX into all bits of DX.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

10011001*

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CWD | cwd |  | 88/86 286 386 486 |  | 5 2 2 3 |  |

* CWD and CDQ have the same encoding with two exceptions: in 32-bit mode, CWD is preceded by the operand-size byte (66h) but CDQ is not; in 16-bit mode, CDQ is preceded by the operand-size byte but CWD is not.

CWDE    Convert Word to Extended Double

80386–80486 Only    Converts a signed word in AX to a signed doubleword in EAX by extending the sign bit of AX into all bits of EAX.

No change

Flags

10011000*

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| CWDE | cwde |  | 88/86 286 386 486 |  | — — 3 3 |  |

* CBW and CWDE have the same encoding with two exceptions: in 32-bit mode, CBW is preceded by the operand-size byte (66h) but CWDE is not; in 16-bit mode, CWDE is preceded by the operand-size byte but CBW is not.

DAA    Decimal Adjust After Addition

Adjusts the result of an addition to a packed BCD number (less than 100 decimal). The previous addition instruction should place its 8-bit binary sum in AL. DAA converts this binary sum to packed BCD format with the least significant decimal digit in the lower four bits and the most significant digit in the upper four bits. If the sum is greater than 99h after adjustment, the carry and auxiliary carry flags are set. Otherwise, the carry and auxiliary carry flags are cleared.

O	D	I	T	S	Z	A	P	C
?				±	±	±	±	±

Flags

00100111

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| DAA | daa |  | 88/86 286 386 486 |  | 4 3 4 2 |  |

DAS    Decimal Adjust After Subtraction

Adjusts the result of a subtraction to a packed BCD number (less than 100 decimal). The previous subtraction instruction should place its 8-bit binary result in AL. DAS converts this binary sum to packed BCD format with the least significant decimal digit in the lower four bits and the most significant digit in the upper four bits. If the sum is greater than 99h after adjustment, the carry and auxiliary carry flags are set. Otherwise, the carry and auxiliary carry flags are cleared.

O	D	I	T	S	Z	A	P	C
?				±	±	±	±	±

Flags

00101111

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| DAS | das |  | 88/86 286 386 486 |  | 4 3 4 2 |  |

DEC    Decrement

Subtracts 1 from the destination operand. Because the operand is treated as an unsigned integer, the DEC instruction does not affect the carry flag. To detect any effects on the carry flag, use the SUB instruction.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±

Flags

1111111w    mod, 001,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| DEC reg8 | dec cl |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| DEC mem | dec counter |  | 88/86 286 386 486 |  | 15+EA (W88=23+EA) 7 6 3 |  |

01001 reg

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| DEC reg16 | dec ax |  | 88/86 |  | 3 |  |
| DEC reg32* |  |  | 286 386 486 |  | 2 2 1 |  |

* 80386–80486 only.

DIV    Unsigned Divide

Divides an implied destination operand by a specified source operand. Both operands are treated as unsigned numbers. If the source (divisor) is 16 bits wide, the implied destination (dividend) is the DX:AX register pair. The quotient goes into AX and the remainder into DX. If the source is 8 bits wide, the implied destination operand is AX. The quotient goes into AL and the remainder into AH. On the 80386–80486, if the source is EAX, the quotient goes into EAX and the remainder into EDX.

O	D	I	T	S	Z	A	P	C
?				?	?	?	?	?

Flags

1111011w    mod, 110,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| DIV reg | div cx div dl |  | 88/86 286 386 486 |  | b=80–90,w=144–162 b=14,w=22 b=14,w=22,d=38 b=16,w=24,d=40 |  |
| DIV mem | div [bx] div fsize |  | 88/86 286 386 486 |  | (b=86–96,w=150–168)+EA* b=17,w=25 b=17,w=25,d=41 b=16,w=24,d=40 |  |

* Word memory operands on the 8088 take (158–176)+EA clocks.

ENTER    Make Stack Frame

80286-80486 Only    Creates a stack frame for a procedure that receives parameters passed on the stack. When immed16 is 0, ENTER is equivalent to push bp, followed by mov bp,sp. The first operand of the ENTER instruction specifies the number of bytes to reserve for local variables. The second operand specifies the nesting level for the procedure. The nesting level should be 0 for languages that do not allow access to local variables of higher-level procedures (such as C, Basic, and FORTRAN). See the complementary instruction LEAVE for a method of exiting from a procedure.

No change

Flags

11001000    data (2)    data (1)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| ENTER immed16,0 | enter 4,0 | 88/86 286 386 486 | — 11 10 14 |
| ENTER immed16,1 | enter 0,1 | 88/86 286 386 486 | — 15 12 17 |
| ENTER immed16,immed8 | enter 6,4 | 88/86 286 386 486 | — 12+4(n – 1) 15+4(n – 1) 17+3n |

HLT    Halt

Stops CPU execution until an interrupt restarts execution at the instruction following HLT. In protected mode, this instruction works only in privileged mode.

No change

Flags

11110100

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| HLT | hlt | 88/86 286 386 486 | 2 2 5 4 |

IDIV    Signed Divide

Divides an implied destination operand by a specified source operand. Both operands are treated as signed numbers. If the source (divisor) is 16 bits wide, the implied destination (dividend) is the DX:AX register pair. The quotient goes into AX and the remainder into DX. If the source is 8 bits wide, the implied destination is AX. The quotient goes into AL and the remainder into AH. On the 80386–80486, if the source is EAX, the quotient goes into EAX and the remainder into EDX.

O	D	I	T	S	Z	A	P	C
?				?	?	?	?	?

Flags

1111011w    mod, 111,r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| IDIV reg | idiv bx idiv dl | 88/86 286 386 486 | b=101–112,w= 165–184 b=17,w=25 b=19,w=27,d=43 b=19,w=27,d=43 |
| IDIV mem | idiv itemp | 88/86 286 386 486 | (b=107–118,w=171– 190)+EA* b=20,w=28 b=22,w=30,d=46 b=20,w=28,d=44 |

* Word memory operands on the 8088 take (175–194)+EA clocks.

IMUL    Signed Multiply

Multiplies an implied destination operand by a specified source operand. Both operands are treated as signed numbers. If a single 16-bit operand is given, the implied destination is AX and the product goes into the DX:AX register pair. If a single 8-bit operand is given, the implied destination is AL and the product goes into AX. On the 80386–80486, if the operand is EAX, the product goes into the EDX:EAX register pair. The carry and overflow flags are set if the product is sign-extended into DX for 16-bit operands, into AH for 8-bit operands, or into EDX for 32-bit operands.

Two additional syntaxes are available on the 80186–80486 processors. In the two-operand form, a 16-bit register gives one of the factors and serves as the destination for the result; a source constant specifies the other factor. In the three-operand form, the first operand is a 16-bit register where the result will be stored, the second is a 16-bit register or memory operand containing one of the factors, and the third is a constant representing the other factor. With both variations, the overflow and carry flags are set if the result is too large to fit into the 16-bit destination register. Since the low 16 bits of the product are the same for both signed and unsigned multiplication, these syntaxes can be used for either signed or unsigned numbers. On the 80386–80486, the operands can be either 16 or 32 bits wide.

A fourth syntax is available on the 80386–80486. Both the source and destination operands can be given specifically. The source can be any 16- or 32-bit memory operand or general-purpose register. The destination can be any general-purpose register of the same size. The overflow and carry flags are set if the product does not fit in the destination.

O	D	I	T	S	Z	A	P	C
±				?	?	?	?	±

Flags

1111011w    mod, 101,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| IMUL reg | imul dx |  | 88/86 286 386 486 |  | b=80–98,w=128–154 b=13,w=21 b=9–14,w=9–22,d=9–38* b=13–18,w=13–26,d=13–42 |  |
| IMUL mem | imul factor |  | 88/86 286 386 486 |  | (b=86–104,w=134–160)+EA† b=16,w=24 b=12–17,w=12–25,d=12–41* b=13–18,w=13–26, d=13–42 |  |

* The 80386–80486 processors have an early-out multiplication algorithm. Therefore, multiplying an 
8-bit or 16-bit value in EAX takes the same time as multiplying the value in AL or AX.

† Word memory operands on the 8088 take (138–164)+EA clocks.

011010s1    mod, reg, r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| IMUL reg16,immed IMUL reg32,immed* | imul cx,25 | 88/86 286 386 486 | — 21 b=9–14,w=9–22,d=9–38† b=13–18,w=13–26,d=13–42 |  |
| IMUL reg16,reg16,immed IMUL reg32,reg32,immed* | imul dx,ax,18 | 88/86 286 386 486 | — 21 b=9–14,w=9–22,d=9–38† b=13–18,w=13–26,d=13–42 |  |

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| IMUL reg16,mem16,immed IMUL reg32,mem32,immed* | imul bx,[si],60 | 88/86 286 386 486 | — 24 b=12–17,w=12–25,d=12–41† b=13–18,w=13–26,d=13–42 |  |

00001111    10101111    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| IMUL reg16,reg16 IMUL reg32,reg32* | imul cx,ax | 88/86 286 386 486 | — — w=9–22,d=9–38 b=13–18,w=13–26,d=13–42 |  |
| IMUL reg16,mem16 IMUL reg32,mem32* | imul dx,[si] | 88/86 286 386 486 | — — w=12–25,d=12–41 b=13–18,w=13–26,d=13–42 |  |

* 80386–80486 only.

† The variations depend on the source constant size; destination size is not a factor.

IN    Input from Port

Transfers a byte or word (or doubleword on the 80386–80486) from a port to the accumulator register. The port address is specified by the source operand, which can be DX or an 8-bit constant. Constants can be used only for port numbers less than 255; use DX for higher port numbers. In protected mode, a general-protection fault occurs if IN is used when the current privilege level is greater than the value of the IOPL flag.

No change

Flags

1110010w    data (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| IN accum,immed | in ax,60h |  | 88/86 286 386 486 |  | 10 (W88=14) 5 12,pm=6,26* 14,pm=9,29*† |  |

1110110w

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| IN accum,DX | in ax,dx in al,dx |  | 88/86 286 386 486 |  | 8 (W88=12) 5 13,pm=7,27* 14,pm=8,28*† |  |

* First protected-mode timing: CPL £ IOPL. Second timing: CPL > IOPL.

† Takes 27 clocks in virtual 8086 mode.

INC    Increment

Adds 1 to the destination operand. Because the operand is treated as an unsigned integer, the INC instruction does not affect the carry flag. If a signed carry requires detection, use the ADD instruction.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±

Flags

1111111w    mod,000,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| INC reg8 | inc cl |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| INC mem | inc vpage |  | 88/86 286 386 486 |  | 15+EA (W88=23+EA) 7 6 3 |  |

01000 reg

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| INC reg16 INC reg32* | inc bx |  | 88/86 286 386 486 |  | 3 2 2 1 |  |

* 80386–80486 only.

INS/INSB/INSW/INSD    Input from Port to String

80286-80486 Only    Receives a string from a port. The string is considered the destination and must be pointed to by ES:DI (even if an operand is given). The input port is specified in DX. For each element received, DI is adjusted according to the size of the operand and the status of the direction flag. DI is increased if the direction flag has been cleared with CLD or decreased if the direction flag has been set with STD.

If the INS form of the instruction is used, a destination operand must be provided to indicate the size of the data elements to be processed, and DX must be specified as the source operand containing the port number. A segment override is not allowed. If INSB (bytes), INSW (words), or INSD (doublewords on the 80386–80486 only) is used, the instruction determines the size of the data elements to be received.

INS and its variations are normally used with the REP prefix. Before the repeated instruction is executed, CX should contain the number of elements to be received. In protected mode, a general-protection fault occurs if INS is used when the current privilege level is greater than the value of the IOPL flag.

No change

Flags

0110110w

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| INS [[ES:]] dest, DX | ins es:instr,dx | 88/86 | — |
| INSB [[[[ES:]] dest, DX]] | rep insb | 286 | 5 |
| INSW [[[[ES:]] dest, DX]] | rep insw | 386 | 15,pm=9,29* |
| INSD [[[[ES:]] dest, DX]] | rep insd | 486 | 17,pm=10,32* |

* First protected-mode timing: CPL £ IOPL. Second timing: CPL > IOPL.

INT    Interrupt

Generates a software interrupt. An 8-bit constant operand (0 to 255) specifies the interrupt procedure to be called. The call is made by indexing the interrupt number into the Interrupt Vector Table (IVT) starting at segment 0, offset 0. In real mode, the IVT contains 4-byte pointers to interrupt procedures. In privileged mode, the IVT contains 8-byte pointers.

When an interrupt is called in real mode, the flags, CS, and IP are pushed onto the stack (in that order), and the trap and interrupt flags are cleared. STI can be used to restore interrupts. See Intel documentation and the documentation for your operating system for details on using and defining interrupts in privileged mode. To return from an interrupt, use the IRET instruction.

O	D	I	T	S	Z	A	P	C
		0	0

Flags

11001101    data (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| INT immed8 | int 25h |  | 88/86 286 386 486 |  | 51 (88=71) 23+m,pm=(40,78)+m* 37,pm=59,99* 30,pm=44,71* |  |

11001100

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| INT 3 | int 3 |  | 88/86 286 386 486 |  | 52 (88=72) 23+m,pm=(40,78)+m* 33,pm=59,99* 26,pm=44,71* |  |

* The first protected-mode timing is for interrupts to the same privilege level. The second is for interrupts to a higher privilege level. Timings for interrupts through task gates are not shown.

INTO    Interrupt on Overflow

Generates Interrupt 4 if the overflow flag is set. The default MS-DOS behavior for Interrupt 4 is to return without taking any action. For INTO to have any effect, you must define an interrupt procedure for Interrupt 4.

O	D	I	T	S	Z	A	P	C
		±	±

Flags

11001110

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| INTO | into |  | 88/86 286 386 486 |  | 53 (88=73),noj=4 24+m,noj=3,pm=(40, 78)+m* 35,noj=3,pm=59,99* 28,noj=3,pm=46,73* |  |

* The first protected-mode timing is for interrupts to the same privilege level. The second is for interrupts to a higher privilege level. Timings for interrupts through task gates are not shown.

INVD    Invalidate Data Cache

80486 Only    Empties contents of the current data cache without writing changes to memory. Proper use of this instruction requires knowledge of how contents are placed in the cache. INVD is intended primarily for system programming. See Intel documentation for details.

No change

Flags

00001111    00001000

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| INVD | invd |  | 88/86 286 386 486 |  | — — — 4 |  |

INVLPG    Invalidate TLB Entry

80486 Only    Invalidates an entry in the Translation Lookaside Buffer (TLB), used by the demand-paging mechanism in virtual-memory operating systems. The instruction takes a single memory operand and calculates the effective address of the operand, including the segment address. If the resulting address is mapped by any entry in the TLB, this entry is removed. Proper use of INVLPG requires understanding the hardware-supported demand-paging mechanism. INVLPG is intended primarily for system programming. See Intel documentation for details.

No change

Flags

00001111    00000001    mod, reg, r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| INVLPG | invlpg pointer[bx] invlpg es:entry |  | 88/86 286 386 486 |  | — — — 12* |  |

* 11 clocks if address is not mapped by any TLB entry.

IRET/IRETD    Interrupt Return

Returns control from an interrupt procedure to the interrupted code. In real mode, the IRET instruction pops IP, CS, and the flags (in that order) and resumes execution. See Intel documentation for details on IRET operation in privileged mode. On the 80386–80486, the IRETD instruction should be used to pop a 32-bit instruction pointer when returning from an interrupt called from a 32-bit segment. The F suffix prevents epilogue code from being generated when ending a PROC block. Use it to terminate interrupt service procedures.

O	D	I	T	S	Z	A	P	C
±	±	±	±	±	±	±	±	±

Flags

11001111

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| IRET | iret |  | 88/86 |  | 32 (88=44) |  |
| IRETD* |  |  | 286 |  | 17+m,pm=(31,55)+m† |  |
| IRETF |  |  | 386 |  | 22,pm=38,82† |  |
| IRETDF* |  |  | 486 |  | 15,pm=20,36 |  |

* 80386–80486 only.

† The first protected-mode timing is for interrupts to the same privilege level within a task. The second is for interrupts to a higher privilege level within a task. Timings for interrupts through task gates are not shown.

Jcondition    Jump Conditionally

Transfers execution to the specified label if the flags condition is true. The condition is tested by checking the flags shown in the table on the following page. If condition is false, no jump is taken and program execution continues at the next instruction. On the 8086–80286 processors, the label given as the operand must be short (between –128 and +127 bytes from the instruction following the jump).* The 80386–80486 processors allow near jumps (–32,768 to +32,767 bytes). On the 80386–80486, the assembler generates the shortest jump possible, unless the jump size is explicitly specified.

When the 80386–80486 processors are in FLAT memory model, short jumps range from –128 to +127 bytes and near jumps range from –2 to +2 gigabytes. There are no far jumps.

No change

Flags

0111cond    disp (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| Jcondition label | jg bigger jo SHORT too_big jpe p_even |  | 88/86 286 386 486 |  | 16,noj=4 7+m,noj=3 7+m,noj=3 3,noj=1 |  |

00001111    1000cond    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| Jcondition label† | je next jnae lesser js negative |  | 88/86 286 386 486 |  | — — 7+m,noj=3 3,noj=1 |  |

* If a source file for an 8086–80286 program contains a conditional jump outside the range of –128 to +127 bytes, the assembler emits a level 3 warning and generates two instructions (including an unconditional jump) that are the equivalent of the desired instruction. This behavior can be enabled and disabled with the OPTION LJMP and OPTION NOLJMP directives.

† Near labels are only available on the 80386–80486. They are the default.

Jump Conditions

| Opcode* | Mnemonic | Flags Checked | Description |
| --- | --- | --- | --- |
| size 0010 | JB/JNAE | CF=1 | Jump if below/not above or equal (unsigned comparisons) |
| size 0011 | JAE/JNB | CF=0 | Jump if above or equal/not below (unsigned comparisons) |
| size 0110 | JBE/JNA | CF=1 or ZF=1 | Jump if below or equal/not above (unsigned comparisons) |
| size 0111 | JA/JNBE | CF=0 and ZF=0 | Jump if above/not below or equal (unsigned comparisons) |
| size 0100 | JE/JZ | ZF=1 | Jump if equal (zero) |
| size 0101 | JNE/JNZ | ZF=0 | Jump if not equal (not zero) |
| size 1100 | JL/JNGE | SF_OF | Jump if less/not greater or equal (signed comparisons) |
| size 1101 | JGE/JNL | SF=OF | Jump if greater or equal/not less (signed comparisons) |
| size 1110 | JLE/JNG | ZF=1 or SF_OF | Jump if less or equal/not greater (signed comparisons) |
| size 1111 | JG/JNLE | ZF=0 and SF=OF | Jump if greater/not less or equal (signed comparisons) |
| size 1000 | JS | SF=1 | Jump if sign |
| size 1001 | JNS | SF=0 | Jump if not sign |
| Opcode* | Mnemonic | Flags Checked | Description |
| size 0010 | JC | CF=1 | Jump if carry |
| size 0011 | JNC | CF=0 | Jump if not carry |
| size 0000 | JO | OF=1 | Jump if overflow |
| size 0001 | JNO | OF=0 | Jump if not overflow |
| size 1010 | JP/JPE | PF=1 | Jump if parity/parity even |
| size 1011 | JNP/JPO | PF=0 | Jump if no parity/parity odd |

* The size bits are 0111 for short jumps or 1000 for 80386–80486 near jumps.

JCXZ/JECXZ    Jump if CX is Zero

Transfers program execution to the specified label if CX is 0. On the 80386–80486, JECXZ can be used to jump if ECX is 0. If the count register is not 0, execution continues at the next instruction. The label given as the operand must be short (between –128 and +127 bytes from the instruction following the jump).

No change

Flags

11100011    disp (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| JCXZ label JECXZ label* | jcxz not found |  | 88/86 286 386 486 |  | 18,noj=6 8+m,noj=4 9+m,noj=5 8,noj=5 |  |

* 80386–80486 only.

JMP    Jump Unconditionally

Transfers program execution to the address specified by the destination operand. Jumps are near (between –32,768 and +32,767 bytes from the instruction following the jump), or short (between –128 and +127 bytes), or far (in a different code segment). Unless a distance is explicitly specified, the assembler selects the shortest possible jump. With near and short jumps, the operand specifies a new IP address. With far jumps, the operand specifies new IP and CS addresses.

When the 80386–80486 processors are in FLAT memory model, short jumps range from –128 to +127 bytes and near jumps range from –2 to +2 gigabytes.

No change

Flags

11101011    disp (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| JMP label | jmp SHORT exit |  | 88/86 286 386 486 |  | 15 7+m 7+m 3 |  |

11101001    disp (2*)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| JMP label | jmp close jmp NEAR PTR distant |  | 88/86 286 386 486 |  | 15 7+m 7+m 3 |  |

11101010    disp (4*)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| JMP label | jmp FAR PTR close jmp distant |  | 88/86 286 386 486 |  | 15 11+m,pm=23+m† 12+m,pm=27+m† 17,pm=19† |  |

11111111    mod,100,r/m    disp (0 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| JMP reg16 JMP mem32§ | jmp ax |  | 88/86 286 386 486 |  | 11 7+m 7+m 5 |  |
| JMP mem16 JMP mem32§ | jmp WORD PTR [bx] jmp table[di] jmp DWORD PTR [si] |  | 88/86 286 386 486 |  | 18+EA 11+m 10+m 5 |  |

11111111    mod,101,r/m    disp (4*)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| JMP mem32 JMP mem48§ | jmp fpointer[si] jmp DWORD PTR [bx] jmp FWORD PTR [di] |  | 88/86 286 386 486 |  | 24+EA 15+m,pm=26+m 12+m,pm=27+m 13,pm=18 |  |

* On the 80386–80486, the displacement can be 4 bytes for near jumps or 6 bytes for far jumps.

† Timings for jumps through call or task gates are not shown, since they are normally used only in operating systems.

§ 80386–80486 only. You can use DWORD PTR to specify near register-indirect jumps or FWORD PTR to specify far register-indirect jumps.

LAHF    Load Flags into AH Register

Transfers bits 0 to 7 of the flags register to AH. This includes the carry, parity, auxiliary carry, zero, and sign flags, but not the trap, interrupt, direction, or overflow flags.

No change

Flags

10011111

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LAHF | lahf |  | 88/86 286 386 486 |  | 4 2 2 3 |  |

LAR    Load Access Rights

80286-80486 Protected Only    Loads the access rights of a selector into a specified register. The source operand must be a register or memory operand containing a selector. The destination operand must be a register that will receive the access rights if the selector is valid and visible at the current privilege level. The zero flag is set if the access rights are transferred, or cleared if they are not. See Intel documentation for details on selectors, access rights, and other privileged-mode concepts.

O	D	I	T	S	Z	A	P	C
					±

Flags

00001111    00000010    mod, reg, r/m    disp (0, 1, 2, or 4)

Encoding

| Syntax | Examples | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- |
| LAR reg16,reg16 LAR reg32,reg32* | lar ax,bx | 88/86 286 386 486 | — 14 15 11 |  |
| LAR reg16,mem16 LAR reg32,mem32* | lar cx,selector | 88/86 286 386 486 | — 16 16 11 |  |

* 80386–80486 only.

LDS/LES/LFS/LGS/LSS    Load Far Pointer

Reads and stores the far pointer specified by the source memory operand. The instruction moves the pointer’s segment value into DS, ES, FS, GS, or SS (depending on the instruction). Then it moves the pointer’s offset value into the destination operand. The LDS and LES instructions are available on all processors. The LFS, LGS, and LSS instructions are available only on the 80386–80486.

No change

Flags

11000101    mod, reg, r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LDS reg,mem | lds si,fpointer |  | 88/86 286 386 486 |  | 16+EA (88=24+EA) 7,pm=21 7,pm=22 6,pm=12 |  |

11000100    mod, reg, r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LES reg,mem | les di,fpointer |  | 88/86 286 386 486 |  | 16+EA (88=24+EA) 7,pm=21 7,pm=22 6,pm=12 |  |

00001111    10110100    mod, reg, r/m    disp (2 or 4)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LFS reg,mem | lfs edi,fpointer |  | 88/86 286 386 486 |  | — — 7,pm=25 6,pm=12 |  |

00001111    10110101    mod, reg, r/m    disp (2 or 4)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LGS reg,mem | lgs bx,fpointer |  | 88/86 286 386 486 |  | — — 7,pm=25 6,pm=12 |  |

00001111    10110010    mod, reg, r/m    disp (2 or 4)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LSS reg,mem | lss bp,fpointer |  | 88/86 286 386 486 |  | — — 7,pm=22 6,pm=12 |  |

LEA    Load Effective Address

Calculates the effective address (offset) of the source memory operand and stores the result in the destination register. If the source operand is a direct memory address, the assembler encodes the instruction in the more efficient MOV reg,immediate form (equivalent to MOV reg, OFFSET mem).

No change

Flags

10001101    mod, reg, r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LEA reg16,mem LEA reg32,mem* | lea bx,npointer |  | 88/86 286 386 486 |  | 2+EA 3 2 1† |  |

* 80386–80486 only.

† 2 if index register used.

LEAVE    High Level Procedure Exit

Terminates the stack frame of a procedure. LEAVE reverses the action of a previous ENTER instruction by restoring SP and BP to the values they had before the procedure stack frame was initialized. LEAVE is equivalent to mov sp,bp, followed by pop bp.

No change

Flags

11001001

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LEAVE | leave |  | 88/86 286 386 486 |  | — 5 4 5 |  |

LES/LFS/LGS    Load Far Pointer to Extra Segment

See LDS.

LGDT/LIDT/LLDT    Load Descriptor Table

Loads a value from an operand into a descriptor table register. LGDT loads into the Global Descriptor Table, LIDT into the Interrupt Vector Table, and LLDT into the Local Descriptor Table. These instructions are available only in privileged mode. See Intel documentation for details on descriptor tables and other protected-mode concepts.

No change

Flags

00001111    00000001    mod, 010,r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LGDT mem48 | lgdt descriptor |  | 88/86 286 386 486 |  | — 11 11 11 |  |

00001111    00000001    mod, 011,r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LIDT mem48 | lidt descriptor |  | 88/86 286 386 486 |  | — 12 11 11 |  |

00001111    00000000    mod, 010,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LLDT reg16 | lldt ax |  | 88/86 286 386 486 |  | — 17 20 11 |  |
| LLDT mem16 | lldt selector |  | 88/86 286 386 486 |  | — 19 24 11 |  |

LMSW    Load Machine Status Word

80286-80486 Privileged Only    Loads a value from a memory operand into the Machine Status Word (MSW). This instruction is available only in privileged mode. See Intel documentation for details on the MSW and other protected-mode concepts.

No change

Flags

00001111    00000001    mod, 110,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LMSW reg16 | lmsw ax |  | 88/86 286 386 486 |  | — 3 10 13 |  |
| LMSW mem16 | lmsw machine |  | 88/86 286 386 486 |  | — 6 13 13 |  |

LOCK    Lock the Bus

Locks out other processors during execution of the next instruction. This instruction is a prefix. It must precede an instruction that accesses a memory location that another processor might attempt to access at the same time. See Intel documentation for details on multiprocessor environments.

No change

Flags

11110000

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LOCK instruction | lock xchg ax,sem |  | 88/86 286 386 486 |  | 2 0 0 1 |  |

LODS/LODSB/LODSW/LODSD    Load Accumulator 
from String

LODS/LODSB/LODSW/LODSD    Load Accumulator from String

Loads the accumulator register with an element from a string in memory. DS:SI must point to the source element, even if an operand is given. For each source element loaded, SI is adjusted according to the size of the operand and the status of the direction flag. SI is incremented if the direction flag has been cleared with CLD or decremented if the direction flag has been set with STD.

If the LODS form of the instruction is used, an operand must be provided to indicate the size of the data elements to be processed. A segment override can be given. If LODSB (bytes), LODSW (words), or LODSD (doublewords on the 80386–80486 only) is used, the instruction determines the size of the data elements to be processed and whether the element will be loaded to AL, AX, or EAX.

LODS and its variations are not used with repeat prefixes, since there is no reason to repeatedly load memory values to a register.

No change

Flags

1010110w

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| LODS [[segreg:]]src LODSB [[[[segreg:]]src]] LODSW[[[[segreg:]]src]] LODSD [[[[segreg:]]src]] | lods es:source lodsw | 88/86 286 386 486 |  | 12 (W88=16) 5 5 5 |  |

LOOP/LOOPW/LOOPD    Loop

Loops repeatedly to a specified label. LOOP decrements CX (without changing any flags) and, if the result is not 0, transfers execution to the address specified by the operand. On the 80386–80486, LOOP uses the 16-bit CX in 16-bit mode and the 32-bit ECX in 32-bit mode. The default can be overridden with LOOPW (CX) or LOOPD (ECX). If CX is 0 after being decremented, execution continues at the next instruction. The operand must specify a short label (between –128 and +127 bytes from the instruction following the LOOP instruction).

No change

Flags

11100010    disp (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LOOP label LOOPW label* LOOPD label* | loop wend |  | 88/86 286 386 486 |  | 17,noj=5 8+m,noj=4 11+m 7,noj=6 |  |

* 80386–80486 only.

LOOPcondition/LOOPconditionW/LOOPconditionD    Loop Conditionally

Loops repeatedly to a specified label if condition is met and if CX is not 0. On the 80386–80486, these instructions use the 16-bit CX in 16-bit mode and the 32-bit ECX in 32-bit mode. This default can be overridden with the W (CX) or D (ECX) forms of the instruction. The instruction decrements CX (without changing any flags) and tests whether the zero flag was set by a previous instruction (such as CMP). With LOOPE and LOOPZ (they are synonyms), execution is transferred to the label if the zero flag is set and CX is not 0. With LOOPNE and LOOPNZ (they are synonyms), execution is transferred to the label if the zero flag is cleared and CX is not 0. Execution continues at the next instruction if the condition is not met. Before entering the loop, CX should be set to the maximum number of repetitions desired.

No change

Flags

11100001    disp (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LOOPE label LOOPEW label* LOOPED label* LOOPZ label LOOPZW label* LOOPZD label* | loopz again |  | 88/86 286 386 486 |  | 18,noj=6 8+m,noj=4 11+m 9,noj=6 |  |

11100000    disp (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LOOPNE label LOOPNEW label* LOOPNED label* LOOPNZ label LOOPNZW label* LOOPNZD label* | loopnz for_next |  | 88/86 286 386 486 |  | 19,noj=5 8,noj=4 11+m 9,noj=6 |  |

* 80386–80486 only.

LSL    Load Segment Limit

80286-80486 Protected Only    Loads the segment limit of a selector into a specified register. The source operand must be a register or memory operand containing a selector. The destination operand must be a register that will receive the segment limit if the selector is valid and visible at the current privilege level. The zero flag is set if the segment limit is transferred, or cleared if it is not. See Intel documentation for details on selectors, segment limits, and other protected-mode concepts.

O	D	I	T	S	Z	A	P	C
					±

Flags

00001111    00000011    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| LSL reg16,reg16 LSL reg32,reg32* | lsl ax,bx | 88/86 286 386 486 |  | — 14 20,25† 10 |  |
| LSL reg16,mem16 LSL reg32,mem32* | lsl cx,seg_lim | 88/86 286 386 486 |  | — 16 21,26† 10 |  |

* 80386–80486 only.

† The first value is for byte granular; the second is for page granular.

LSS    Load Far Pointer to Stack Segment

See LDS.

LTR    Load Task Register

80286-80486 Protected Only    Loads a value from the specified operand to the current task register. LTR is available only in privileged mode. See Intel documentation for details on task registers and other protected-mode concepts.

No change

Flags

00001111    00000000    mod, 011,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| LTR reg16 | ltr ax |  | 88/86 286 386 486 |  | — 17 23 20 |  |
| LTR mem16 | ltr task |  | 88/86 286 386 486 |  | — 19 27 20 |  |

MOV    Move Data

Moves the value in the source operand to the destination operand. If the destination operand is SS, interrupts are disabled until the next instruction is executed (except on early versions of the 8088 and 8086).

No change

Flags

100010dw    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV reg,reg | mov dh,bh mov dx,cx mov bp,sp | 88/86 286 386 486 |  | 2 2 2 1 |  |
| MOV mem,reg | mov array[di],bx mov count,cx | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 3 2 1 |  |
| MOV reg,mem | mov bx,pointer mov dx,matrix[bx+di] | 88/86 286 386 486 |  | 8+EA (W88=12+EA) 5 4 1 |  |

1100011w    mod, 000,r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV mem,immed | mov [bx],15 mov color,7 | 88/86 286 386 486 |  | 10+EA (W88=14+EA) 3 2 1 |  |

1011w reg    data (1 or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV reg,immed | mov cx,256 mov dx,OFFSET string | 88/86 286 386 486 |  | 4 2 2 1 |  |

101000aw    disp (2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV mem,accum | mov total,ax | 88/86 286 386 486 |  | 10 (W88=14) 3 2 1 |  |
| MOV accum,mem | mov al,string | 88/86 286 386 486 |  | 10 (W88=14) 5 4 1 |  |

100011d0    mod,sreg, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV segreg,reg16 | mov ds,ax | 88/86 286 386 486 |  | 2 2,pm=17 2,pm=18 3,pm=9 |  |
| MOV segreg,mem16 | mov es,psp | 88/86 286 386 486 |  | 8+EA (88=12+EA) 5,pm=19 5,pm=19 3,pm=9 |  |
| MOV reg16,segreg | mov ax,ds | 88/86 286 386 486 |  | 2 2 2 3 |  |
| MOV mem16,segreg | mov stack_save,ss | 88/86 286 386 486 |  | 9+EA (88=13+EA) 3 2 3 |  |

MOV    Move to/from Special Registers

80386–80486 Only    Moves a value from a special register to or from a 32-bit general-purpose register. The special registers include the control registers CR0, CR2, and CR3; the debug registers DR0, DR1, DR2, DR3, DR6, and DR7; and the test registers TR6 and TR7. On the 80486, the test registers TR3, TR4, and TR5 are also available. See Intel documentation for details on special registers.

O	D	I	T	S	Z	A	P	C
?				?	?	?	?	?

Flags

00001111    001000d0    11, reg*, r/m

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV reg32, controlreg | mov eax,cr2 | 88/86 286 386 486 |  | — — 6 4 |  |
| MOV controlreg,reg32 | mov cr0,ebx | 88/86 286 386 486 |  | — — CR0=10,CR2=4,CR3=5 4,CR0=16 |  |

00001111    001000d1    11, reg*, r/m

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV reg32,debugreg | mov edx,dr3 | 88/86 286 386 486 |  | — — DR0–3=22,DR6–7=14 10 |  |
| MOV debugreg,reg32 | mov dr0,ecx | 88/86 286 386 486 |  | — — DR0–3=22,DR6–7=16 11 |  |

00001111    001001d0    11,reg*, r/m

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| MOV reg32,testreg | mov edx,tr6 | 88/86 286 386 486 |  | — — 12 4,TR3=3 |  |
| MOV testreg, reg32 | mov tr7,eax | 88/86 286 386 486 |  | — — 12 4,TR3=6 |  |

* The reg field contains the register number of the special register (for example, 000 for CR0, 011 for DR7, or 111 for TR7).

MOVS/MOVSB/MOVSW/MOVSD    Move String Data

Moves a string from one area of memory to another. DS:SI must point to the source string and ES:DI to the destination address, even if operands are given. For each element moved, DI and SI are adjusted according to the size of the operands and the status of the direction flag. They are increased if the direction flag has been cleared with CLD, or decreased if the direction flag has been set with STD.

If the MOVS form of the instruction is used, operands must be provided to indicate the size of the data elements to be processed. A segment override can be given for the source operand (but not for the destination). If MOVSB (bytes), MOVSW (words), or MOVSD (doublewords on the 80386–80486 only) is used, the instruction determines the size of the data elements to be processed.

MOVS and its variations are normally used with the REP prefix.

No change

Flags

1010010w

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| MOVS [[ES:]]dest,[[segreg:]]src MOVSB [[[[ES:]]dest,[[segreg:]]src]] MOVSW [[[[ES:]]dest,[[segreg:]]src]] MOVSD [[[[ES:]]dest,[[segreg:]]src]] | rep movsb movs dest,es:source | 88/86 286 386 486 | 18 (W88=26) 5 7 7 |

MOVSX    Move with Sign-Extend

80386–80486 Only    Moves and sign-extends the value of the source operand to the destination register. MOVSX is used to copy a signed 8-bit or 16-bit source operand to a larger 16-bit or 32-bit destination register.

No change

Flags

00001111    1011111w    mod, reg, r/m    disp (0, 1, 2, or 4)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| MOVSX reg,reg | movsx eax,bx movsx ecx,bl movsx bx,al | 88/86 286 386 486 | — — 3 3 |
| Syntax | Examples | CPU | Clock Cycles |
| MOVSX reg,mem | movsx cx,bsign movsx edx,wsign movsx eax,bsign | 88/86 286 386 486 | — — 6 3 |

MOVZX    Move with Zero-Extend

80386–80486 Only    Moves and zero-extends the value of the source operand to the destination register. MOVZX is used to copy an unsigned 8-bit or 16-bit source operand to a larger 16-bit or 32-bit destination register.

No change

Flags

00001111    1011011w    mod, reg, r/m    disp (0, 1, 2, or 4)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| MOVZX reg,reg | movzx eax,bx movzx ecx,bl movzx bx,al |  | 88/86 286 386 486 |  | — — 3 3 |  |
| MOVZX reg,mem | movzx cx,bunsign movzx edx,wunsign movzx eax,bunsign |  | 88/86 286 386 486 |  | — — 6 3 |  |

MUL    Unsigned Multiply

Multiplies an implied destination operand by a specified source operand. Both operands are treated as unsigned numbers. If a single 16-bit operand is given, the implied destination is AX and the product goes into the DX:AX register pair. If a single 8-bit operand is given, the implied destination is AL and the product goes into AX. On the 80386–80486, if the operand is EAX, the product goes into the EDX:EAX register pair. The carry and overflow flags are set if DX is not 0 for 16-bit operands or if AH is not 0 for 8-bit operands.

O	D	I	T	S	Z	A	P	C
±				?	?	?	?	±

Flags

1111011w    mod, 100, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| MUL reg | mul bx mul dl | 88/86 286 386 486 | b=70–77,w=118–133 b=13,w=21 b=9–14,w=9–22,d=9–38* b=13–18,w=13–26,d=13–42 |
| MUL mem | mul factor mul WORD PTR [bx] | 88/86 286 386 486 | (b=76–83,w=124–139)+EA† b=16,w=24 b=12–17,w=12–25,d=12–41* b=13–18,w=13–26,d=13–42 |

* The 80386–80486 processors have an early-out multiplication algorithm. Therefore, multiplying an 
8-bit or 16-bit value in EAX takes the same time as multiplying the value in AL or AX.

† Word memory operands on the 8088 take (128–143)+EA clocks.

NEG    Two’s Complement Negation

Replaces the operand with its two’s complement. NEG does this by subtracting the operand from 0. If the operand is 0, the carry flag is cleared. Otherwise, the carry flag is set. If the operand contains the maximum possible negative value (–128 for 8-bit operands or –32,768 for 16-bit operands), the value does not change, but the overflow and carry flags are set.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

1111011w    mod, 011, r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| NEG reg | neg ax |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| NEG mem | neg balance |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 6 3 |  |

NOP    No Operation

Performs no operation. NOP can be used for timing delays or alignment.

No change

Flags

10010000*

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| NOP | nop |  | 88/86 286 386 486 |  | 3 3 3 3 |  |

* The encoding is the same as XCHG AX,AX.

NOT    One’s Complement Negation

Toggles each bit of the operand by clearing set bits and setting cleared bits.

No change

Flags

1111011w    mod, 010, r/m    disp (0,1,or2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| NOT reg | not ax |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| NOT mem | not masker |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 6 3 |  |

OR    Inclusive OR

Performs a bitwise OR operation on the source and destination operands and stores the result to the destination operand. For each bit position in the operands, if either or both bits are set, the corresponding bit of the result is set. Otherwise, the corresponding bit of the result is cleared.

O	D	I	T	S	Z	A	P	C
0				±	±	?	±	0

Flags

000010dw    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| OR reg,reg | or ax,dx |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| OR mem,reg | or bits,dx or [bp+6],cx |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 7 3 |  |
| OR reg,mem | or bx,masker or dx,color[di] |  | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 7 6 2 |  |

100000sw    mod,001, r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| OR reg,immed | or dx,110110b |  | 88/86 286 386 486 |  | 4 3 2 1 |  |
| OR mem,immed | or flag_rec,8 |  | 88/86 286 386 486 |  | (b=17,w=25)+EA 7 7 3 |  |

0000110w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| OR accum,immed | or ax,40h |  | 88/86 286 386 486 |  | 4 3 2 1 |  |

OUT    Output to Port

Transfers a byte or word (or a doubleword on the 80386–80486) to a port from the accumulator register. The port address is specified by the destination operand, which can be DX or an 8-bit constant. In protected mode, a general-protection fault occurs if OUT is used when the current privilege level is greater than the value of the IOPL flag.

No change

Flags

1110011w    data (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| OUT immed8,accum | out 60h,al |  | 88/86 286 386 486 |  | 10 (88=14) 3 10,pm=4,24* 16,pm=11,31* |  |

1110111w

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| OUT DX,accum | out dx,ax out dx,al |  | 88/86 286 386 486 |  | 8 (88=12) 3 11,pm=5,25* 16,pm=10,30* |  |

* First protected-mode timing: CPL < IOPL. Second timing: CPL > IOPL.

OUTS/OUTSB/OUTSW/OUTSD    Output String to Port

80186–80486 Only    Sends a string to a port. The string is considered the source and must be pointed to by DS:SI (even if an operand is given). The output port is specified in DX. For each element sent, SI is adjusted according to the size of the operand and the status of the direction flag. SI is increased if the direction flag has been cleared with CLD, or decreased if the direction flag has been set with STD.

If the OUTS form of the instruction is used, an operand must be provided to indicate the size of data elements to be sent. A segment override can be given. If OUTSB (bytes), OUTSW (words), or OUTSD (doublewords on the 80386–80486 only) is used, the instruction determines the size of the data elements to be sent.

OUTS and its variations are normally used with the REP prefix. Before the instruction is executed, CX should contain the number of elements to send. In protected mode, a general-protection fault occurs if OUTS is used when the current privilege level is greater than the value of the IOPL flag.

No change

Flags

0110111w

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| OUTS DX, [[segreg:]] src OUTSB [[DX, [[segreg:]] src]] OUTSW [[DX, [[segreg:]] src]] OUTSD [[DX, [[segreg:]] src]] | rep outs dx,buffer outsb rep outsw | 88/86 286 386 486 | — 5 14,pm=8,28* 17,pm=10,32* |

* First protected-mode timing: CPL < IOPL. Second timing: CPL > IOPL.

POP    Pop

Pops the top of the stack into the destination operand. The value at SS:SP is copied to the destination operand and SP is increased by 2. The destination operand can be a memory location, a general-purpose 16-bit register, or any segment register except CS. Use RET to pop CS. On the 80386–80486, 32-bit values can be popped by giving a 32-bit operand. ESP is increased by 4 for 32-bit pops.

No change

Flags

01011 reg

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| POP reg16 POP reg32* | pop cx |  | 88/86 286 386 486 |  | 8 (88=12) 5 4 1 |  |

10001111    mod,000,r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| POP mem16 POP mem32* | pop param |  | 88/86 286 386 486 |  | 17+EA (88=25+EA) 5 5 6 |  |

000,sreg,111

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| POP segreg | pop es pop ds pop ss |  | 88/86 286 386 486 |  | 8 (88=12) 5,pm=20 7,pm=21 3,pm=9 |  |

00001111    10,sreg,001

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| POP segreg* | pop fs pop gs |  | 88/86 286 386 486 |  | — — 7,pm=21 3,pm=9 |  |

* 80386–80486 only.

POPA/POPAD    Pop All

80186-80486 Only    Pops the top 16 bytes on the stack into the eight general-purpose registers. The registers are popped in the following order: DI, SI, BP, SP, BX, DX, CX, AX. The value for the SP register is actually discarded rather than copied to SP. POPA always pops into 16-bit registers. On the 80386–80486, use POPAD to pop into 32-bit registers.

No change

Flags

01100001

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| POPA POPAD* | popa |  | 88/86 286 386 486 |  | — 19 24 9 |  |

* 80386–80486 only.

POPF/POPFD    Pop Flags

Pops the value on the top of the stack into the flags register. POPF always pops into the 16-bit flags register. On the 80386–80486, use POPFD to pop into the 32-bit flags register.

O	D	I	T	S	Z	A	P	C
±	±	±	±	±	±	±	±	±

Flags

10011101

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| POPF POPFD* | popf |  | 88/86 286 386 486 |  | 8 (88=12) 5 5 9,pm=6 |  |

* 80386–80486 only.

PUSH/PUSHW/PUSHD    Push

Pushes the source operand onto the stack. SP is decreased by 2 and the source value is copied to SS:SP. The operand can be a memory location, a general-purpose 16-bit register, or a segment register. On the 80186–80486 processors, the operand can also be a constant. On the 80386–80486, 32-bit values can be pushed by specifying a 32-bit operand. ESP is decreased by 4 for 32-bit pushes. On the 8088 and 8086, PUSH SP saves the value of SP after the push. On the 80186–80486 processors, PUSH SP saves the value of SP before the push. The PUSHW and PUSHD instructions push a word (2 bytes) and a doubleword (4 bytes), respectively.

No change

Flags

01010 reg

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| PUSH reg16 PUSH reg32* PUSHW reg16 PUSHD reg32* | push dx |  | 88/86 286 386 486 |  | 11 (88=15) 3 2 1 |  |

11111111    mod, 110,r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| PUSH mem16 PUSH mem32* | push [di] push fcount |  | 88/86 286 386 486 |  | 16+EA (88=24+EA) 5 5 4 |  |

00,sreg,110

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| PUSH segreg PUSHW segreg PUSHD segreg* | push es push ss push cs |  | 88/86 286 386 486 |  | 10 (88=14) 3 2 3 |  |

00001111    10,sreg,000

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| PUSH segreg PUSHW segreg PUSHD segreg* | push fs push gs |  | 88/86 286 386 486 |  | — — 2 3 |  |

011010s0    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| PUSH immed PUSHW immed PUSHD immed* | push 'a' push 15000 |  | 88/86 286 386 486 |  | — 3 2 1 |  |

* 80386–80486 only.

PUSHA/PUSHAD    Push All

80186–80486 Only    Pushes the eight general-purpose registers onto the stack. The registers are pushed in the following order: AX, CX, DX, BX, SP, BP, SI, DI. The value pushed for SP is the value before the instruction. PUSHA always pushes 16-bit registers. On the 80386–80486, use PUSHAD to push 32-bit registers.

No change

Flags

01100000

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| PUSHA PUSHAD* | pusha |  | 88/86 286 386 486 |  | — 17 18 11 |  |

* 80386–80486 only.

PUSHF/PUSHFD    Push Flags

Pushes the flags register onto the stack. PUSHF always pushes the 16-bit flags register. On the 80386–80486, use PUSHFD to push the 32-bit flags register.

No change

Flags

10011100

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| PUSHF PUSHFD* | pushf |  | 88/86 286 386 486 |  | 10(88=14) 3 4 4,pm=3 |  |

* 80386–80486 only.

RCL/RCR/ROL/ROR    Rotate

Rotates the bits in the destination operand the number of times specified in the source operand. RCL and ROL rotate the bits left; RCR and ROR rotate right.

ROL and ROR rotate the number of bits in the operand. For each rotation, the leftmost or rightmost bit is copied to the carry flag as well as rotated. RCL and RCR rotate through the carry flag. The carry flag becomes an extension of the operand so that a 9-bit rotation is done for 8-bit operands, or a 17-bit rotation for 16-bit operands.

On the 8088 and 8086, the source operand can be either CL or 1. On the 80186–80486, the source operand can be CL or an 8-bit constant. On the 80186–80486, rotate counts larger than 31 are masked off, but on the 8088 and 8086, larger rotate counts are performed despite the inefficiency involved. The overflow flag is modified only by single-bit variations of the instruction; for multiple-bit variations, the overflow flag is undefined.

O	D	I	T	S	Z	A	P	C
±								±

Flags

1101000w    mod, TTT*,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ROL reg,1 ROR reg,1 | ror ax,1 rol dl,1 |  | 88/86 286 386 486 |  | 2 2 3 3 |  |
| RCL reg,1 RCR reg,1 | rcl dx,1 rcr bl,1 |  | 88/86 286 386 486 |  | 2 2 9 3 |  |
| ROL mem,1 ROR mem,1 | ror bits,1 rol WORD PTR [bx],1 |  | 88/86 286 386 486 |  | 15+EA (W88=23+EA) 7 7 4 |  |
| RCL mem,1 RCR mem,1 | rcl WORD PTR [si],1 rcr WORD PTR m32[0],1 |  | 88/86 286 386 486 |  | 15+EA (W88=23+EA 7 10 4 |  |

1101001w    mod, TTT*,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ROL reg,CL ROR reg,CL | ror ax,cl rol dx,cl |  | 88/86 286 386 486 |  | 8+4n 5+n 3 3 |  |
| RCL reg,CL RCR reg,CL | rcl dx,cl rcr bl,cl |  | 88/86 286 386 486 |  | 8+4n 5+n 9 8–30 |  |
| ROL mem,CL ROR mem,CL | ror color,cl rol WORD PTR [bp+6],cl |  | 88/86 286 386 486 |  | 20+EA+4n (W88=28+EA+4n) 8+n 7 4 |  |
| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| RCL mem,CL RCR mem,CL | rcr WORD PTR [bx+di],cl rcl masker |  | 88/86 286 386 486 |  | 20+EA+4n (W88=28+EA+4n) 8+n 10 9–31 |  |

1100000w    mod,TTT*,r/m    disp (0, 1, or 2)    data (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| ROL reg,immed8 ROR reg,immed8 | rol ax,13 ror bl,3 286 |  | 88/86 286 386 486 |  | — 5+n 3 2 |  |
| RCL reg,immed8 RCR reg,immed8 | rcl bx,5 rcr si,9 |  | 88/86 286 386 486 |  | — 5+n 9 8–30 |  |
| ROL mem,immed8 ROR mem,immed8 | rol BYTE PTR [bx],10 ror bits,6 |  | 88/86 286 386 486 |  | — 8+n 7 4 |  |
| RCL mem,immed8 RCR mem,immed8 | rcl WORD PTR [bp+8], rcr masker,3 |  | 88/86 286 386 486 |  | — 8+n 10 9–31 |  |

* TTT represents one of the following bit codes: 000 for ROL, 001 for ROR, 010 for RCL, or 011 for RCR.

REP    Repeat String

Repeats a string instruction the number of times indicated by CX. First, CX is compared to 0; if it equals 0, execution proceeds to the next instruction. Otherwise, CX is decremented, the string instruction is performed, and the loop continues. REP is used with MOVS and STOS. REP also can be used with INS and OUTS on the 80186–80486 processors. On all processors except the 80386–80486, combining a repeat prefix with a segment override can cause errors if an interrupt occurs.

No change

Flags

11110011    1010010w

Encoding

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| REP MOVS dest,src REP MOVSB [[dest,src]] REP MOVSW [[dest,src]] REP MOVSD [[dest,src]]* | rep movs source,dest rep movsw | 88/86 286 386 486 | 9+17n (W88=9+25n) 5+4n 7+4n 12+3n# |  |

11110011    1010101w

Encoding

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| REP STOS dest REP STOSB [[dest]] REP STOSW [[dest]] REP STOSD [[dest]]* | rep stosb rep stos dest | 88/86 286 386 486 | 9+10n (W88=9+14n) 4+3n 5+5n 7+4n† |  |

11110011    1010101w

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| REP LODS dest REP LODSB [[dest]] REP LODSW [[dest]] REP LODSD [[dest]]* | rep lodsb rep lods dest | 88/86 286 386 486 | — — — 7+4n† |

11110011    0110110w

Encoding

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| REP INS dest,DX REP INSB [[dest,DX]] REP INSW [[dest,DX]] REP INSD [[dest,DX]]* | rep insb rep ins dest,dx | 88/86 286 386 486 | — 5+4n 13+6n,pm=(7,27)+6n§ 16+8n,pm=(10,30)+8n§ |  |

11110011    0110111w

Encoding

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| REP OUTS DX,src REP OUTSB [[src]] REP OUTSW [[src]] REP OUTSD [[src]]* | rep outs dx,source rep outsw | 88/86 286 386 486 | — 5+4n 12+5n,pm=(6,26)+5n§ 17+5n,pm=(11,31)+5n§ |  |

* 80386–80486 only.

# 5 if n = 0, 13 if n = 1.

† 5 if n = 0.

§ First protected-mode timing: CPL £ IOPL. Second timing: CPL > IOPL.

REPcondition    Repeat String Conditionally

Repeats a string instruction as long as condition is true and the maximum count has not been reached. REPE and REPZ (they are synonyms) repeat while the zero flag is set. REPNE and REPNZ (they are synonyms) repeat while the zero flag is cleared. The conditional-repeat prefixes should only be used with SCAS and CMPS, since these are the only string instructions that modify the zero flag. Before executing the instruction, CX should be set to the maximum allowable number of repetitions. First, CX is compared to 0; if it equals 0, execution proceeds to the next instruction. Otherwise, CX is decremented, the string instruction is performed, and the loop continues. On all processors except the 80386–80486, combining a repeat prefix with a segment override may cause errors if an interrupt occurs during a string operation.

O	D	I	T	S	Z	A	P	C
					±

Flags

11110011    1010011w

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| REPE CMPS src,dest REPE CMPSB [[src,dest]] REPE CMPSW [[src,dest]] REPE CMPSD [[src,dest]]* | repz cmpsb repe cmps src,dest | 88/86 286 386 486 |  | 9+22n (W88=9+30n) 5+9n 5+9n 7+7n# |  |

11110011    1010111w

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| REPE SCAS dest REPE SCASB [[dest]] REPE SCASW [[dest]] REPE SCASD [[dest]]* | repe scas dest repz scasw | 88/86 286 386 486 |  | 9+15n (W88=9+19n) 5+8n 5+8n 7+5n# |  |

11110010    1010011w

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| REPNE CMPS src,dest REPNE CMPSB [[src,dest]] REPNE CMPSW [[src,dest]] REPNE CMPSD [[src,dest]]* | repne cmpsw repnz cmps src,dest | 88/86 286 386 486 |  | 9+22n (W88=9+30n) 5+9n 5+9n 7+7n# |  |

11110010    1010111w

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| REPNE SCAS des REPNE SCASB [[dest]] REPNE SCASW [[dest]] REPNE SCASD [[dest]]* | repne scas dest repnz scasb | 88/86 286 386 486 |  | 9+15n (W88=9+19n) 5+8n 5+8n 7+5n* |  |

* 80386–80486 only.

# 5 if n=0.

RET/RETN/RETF    Return from Procedure

Returns from a procedure by transferring control to an address popped from the top of the stack. A constant operand can be given indicating the number of additional bytes to release. The constant is normally used to adjust the stack for arguments pushed before the procedure was called. The size of a return (near or far) is the size of the procedure in which the RET is defined with the PROC directive. RETN can be used to specify a near return; RETF can specify a far return. A near return pops a word into IP. A far return pops a word into IP and then pops a word into CS. After the return, the number of bytes given in the operand (if any) is added to SP.

No change

Flags

11000011

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| RET RETN | ret retn |  | 88/86 286 386 486 |  | 16 (88=20) 11+m 10+m 5 |  |

11000010    data (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| RET immed16 RETN immed16 | ret 2 retn 8 |  | 88/86 286 386 486 |  | 20 (88=24) 11+m 10+m 5 |  |

11001011

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| RET RETF | ret retf |  | 88/86 286 386 486 |  | 26 (88=34) 15+m,pm=25+m,55* 18+m,pm=32+m,62* 13,pm=18,33* |  |

11001010    data (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| RET immed16 RETF immed16 | ret 8 retf 32 |  | 88/86 286 386 486 |  | 25 (88=33) 15+m,pm=25+m,55* 18+m,pm=32+m,68* 14,pm=17,33* |  |

* The first protected-mode timing is for a return to the same privilege level; the second is for a return to a lesser privilege level.

ROL/ROR    Rotate

See RCL/RCR.

SAHF    Store AH into Flags

Transfers AH into bits 0 to 7 of the flags register. This includes the carry, parity, auxiliary carry, zero, and sign flags, but not the trap, interrupt, direction, or overflow flags.

O	D	I	T	S	Z	A	P	C
				±	±	±	±	±

Flags

10011110

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SAHF | sahf |  | 88/86 286 386 486 |  | 4 2 3 2 |  |

SAL/SAR    Shift

See SHL/SHR/SAL/SAR.

SBB    Subtract with Borrow

Adds the carry flag to the second operand, then subtracts that value from the first operand. The result is assigned to the first operand. SBB is used to subtract the least significant portions of numbers that must be processed in multiple registers.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

000110dw    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SBB reg,reg | sbb dx,cx |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| SBB mem,reg | sbb WORD PTR m32[2],dx |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 6 3 |  |
| SBB reg,mem | sbb dx,WORD PTR m32[2] |  | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 7 7 2 |  |

100000sw    mod,011, r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SBB reg,immed | sbb dx,45 |  | 88/86 286 386 486 |  | 4 3 2 1 |  |
| SBB mem,immed | sbb WORD PTR m32[2],40 |  | 88/86 286 386 486 |  | 17+EA (W88=25+EA) 7 7 3 |  |

0001110w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SBB accum,immed | sbb ax,320 88/86 |  | 4 86 386 486 |  | 3 2 1 |  |

SCAS/SCASB/SCASW/SCASD    Scan String Flags

Scans a string to find a value specified in the accumulator register. The string to be scanned is considered the destination. ES:DI must point to that string, even if an operand is specified. For each element, the destination element is subtracted from the accumulator value and the flags are updated to reflect the result (although the result is not stored). DI is adjusted according to the size of the operands and the status of the direction flag. DI is increased if the direction flag has been cleared with CLD, or decreased if the direction flag has been set with STD.

If the SCAS form of the instruction is used, an operand must be provided to indicate the size of the data elements to be processed. No segment override is allowed. If SCASB (bytes), SCASW (words), or SCASD (doublewords on the 80386–80486 only) is used, the instruction determines the size of the data elements to be processed and whether the element scanned for is in AL, AX, or EAX.

SCAS and its variations are normally used with repeat prefixes. REPNE (or REPNZ) is used to find the first element in a string that matches the value in the accumulator register. REPE (or REPZ) is used to find the first mismatch. Before the scan, CX should contain the maximum number of elements to scan. After a REPNE SCAS, the zero flag is clear if the string does not contain the accumulator value. After a REPE SCAS, the zero flag is set if the string contains nothing but the accumulator value.

When the instruction finishes, ES:DI points to the element that follows (if the direction flag is clear) or precedes (if the direction flag is set) the match or mismatch. If CX decrements to 0, ES:DI points to the element that follows or precedes the last comparison. The zero flag is set or clear according to the result of the last comparison, not according to the value of CX.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

1010111w

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| SCAS [[ES:]] dest SCASB [[[[ES:]] dest]] SCASW [[[[ES:]] dest]] SCASD [[[[ES:]] dest]]* | repne scasw repe scasb scas es:destin | 88/86 286 386 486 |  | 15 (W88=19) 7 7 6 |  |

* 80386–80486 only

SETcondition    Set Conditionally

80386–80486 Only    Sets the byte specified in the operand to 1 if condition is true or to 0 if condition is false. The condition is tested by checking the flags shown in the table on the following page. The instruction is used to set Boolean flags conditionally.

No change

Flags

00001111    1001cond    mod,000,r/m

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| SETcondition reg8 | setc dh setz al setae bl | 88/86 286 386 486 |  | — — 4 true=4, false=3 |  |
| SETcondition mem8 | seto BTYE PTR [ebx] setle flag sete Booleans[di] | 88/86 286 386 486 |  | — — 5 true=3, false=4 |  |

Set Conditions

| Opcode | Mnemonic | Flags Checked | Description |
| --- | --- | --- | --- |
| 10010010 | SETB/SETNAE | CF=1 | Set if below/not above or equal (unsigned comparisons) |
| 10010011 | SETAE/SETNB | CF=0 | Set if above or equal/not below (unsigned comparisons) |
| 10010110 | SETBE/SETNA | CF=1 or ZF=1 | Set if below or equal/not above (unsigned comparisons) |
| 10010111 | SETA/SETNBE | CF=0 and ZF=0 | Set if above/not below or equal (unsigned comparisons) |
| 10010100 | SETE/SETZ | ZF=1 | Set if equal/zero |
| 10010101 | SETNE/SETNZ | ZF=0 | Set if not equal/not zero |
| Opcode | Mnemonic | Flags Checked | Description |
| 10011100 | SETL/SETNGE | SF_OF | Set if less/not greater or equal (signed comparisons) |
| 10011101 | SETGE/SETNL | SF=OF | Set if greater or equal/not less (signed comparisons) |
| 10011110 | SETLE/SETNG | ZF=1 or SF_OF | Set if less or equal/not greater or equal (signed comparisons) |
| 10011111 | SETG/SETNLE | ZF=0 and SF=OF | Set if greater/not less or equal (signed comparisons) |
| 10011000 | SETS | SF=1 | Set if sign |
| 10011001 | SETNS | SF=0 | Set if not sign |
| 10010010 | SETC | F=1 | Set if carry |
| 10010011 | SETNC | CF=0 | Set if not carry |
| 10010000 | SETO | OF=1 | Set if overflow |
| 10010001 | SETNO | OF=0 | Set if not overflow |
| 10011010 | SETP/SETPE | PF=1 | Set if parity/parity even |
| 10011011 | SETNP/SETPO | PF=0 | Set if no parity/parity odd |

SGDT/SIDT/SLDT    Store Descriptor Table

80286-80486 Only    Stores a descriptor table register into a specified operand. SGDT stores the Global Descriptor Table; SIDT, the Interrupt Vector Table; and SLDT, the Local Descriptor Table. These instructions are generally useful only in privileged mode. See Intel documentation for details on descriptor tables and other protected-mode concepts.

No change

Flags

00001111    00000001    mod,000,r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SGDT mem48 | sgdt descriptor |  | 88/86 286 386 486 |  | — 11 9 10 |  |

00001111    00000001    mod,001,r/m    disp (2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SIDT mem48 | sidt descriptor |  | 88/86 286 386 486 |  | — 12 9 10 |  |

00001111    00000000    mod, 000,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SLDT reg16 | sldt ax |  | 88/86 286 386 486 |  | — 2 2 2 |  |
| SLDT mem16 | sldt selector |  | 88/86 286 386 486 |  | — 3 2 3 |  |

SHL/SHR/SAL/SAR    Shift

Shifts the bits in the destination operand the number of times specified by the source operand. SAL and SHL shift the bits left; SAR and SHR shift right.

With SHL, SAL, and SHR, the bit shifted off the end of the operand is copied into the carry flag, and the leftmost or rightmost bit opened by the shift is set to 0. With SAR, the bit shifted off the end of the operand is copied into the carry flag, and the leftmost bit opened by the shift retains its previous value (thus preserving the sign of the operand). SAL and SHL are synonyms.

On the 8088 and 8086, the source operand can be either CL or 1. On the 80186–80486 processors, the source operand can be CL or an 8-bit constant. On the 80186–80486 processors, shift counts larger than 31 are masked off, but on the 8088 and 8086, larger shift counts are performed despite the inefficiency. Only single-bit variations of the instruction modify the overflow flag; for multiple-bit variations, the overflow flag is undefined.

O	D	I	T	S	Z	A	P	C
±				±	±	?	±	±

Flags

1101000w    mod,TTT*,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SAR reg,1 | sar di,1 sar cl,1 |  | 88/86 286 386 486 |  | 2 2 3 3 |  |
| SAL reg,1 SHL reg,1 SHR reg,1 SAR mem,1 | shr dh,1 shl si,1 sal bx,1 sar count,1 |  | 88/86 286 386 486 |  | 2 2 3 3 |  |
|  |  |  | 88/86 286 386 486 |  | 15+EA (W88=23+EA) 7 7 4 |  |
| SAL mem,1 SHL mem,1 SHR mem,1 | sal WORD PTR m32[0],1 shl index,1 shr unsign[di],1 |  | 88/86 286 386 486 |  | 15+EA (W88=23+EA) 7 7 4 |  |

1101001w    mod,TTT*,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SAR reg,CL | sar bx,cl sar dx,cl |  | 88/86 286 386 486 |  | 8+4n 5+n 3 3 |  |
| SAL reg,CL SHL reg,CL SHR reg,CL | shr dx,cl shl di,cl sal ah,cl |  | 88/86 286 386 486 |  | 8+4n 5+n 3 3 |  |
| SAR mem,CL | sar sign,cl sar WORD PTR [bp+8],cl |  | 88/86 286 386 486 |  | 20+EA+4n (W88=28+EA+4n) 8+n 7 4 |  |
| SAL mem,CL SHL mem,CL SHR mem,CL | shr WORD PTR m32[2],cl sal BYTE PTR [di],cl shl index,cl |  | 88/86 286 386 486 |  | 20+EA+4n (W88=28+EA+4n) 8+n 7 4 |  |

1100000w    mod,TTT*,r/m    disp (0, 1, or 2)    data (1)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SAR reg,immed8 | sar bx,5 sar cl,5 |  | 88/86 286 386 486 |  | — 5+n 3 2 |  |
| SAL reg,immed8 SHL reg,immed8 SHR reg,immed8 | sal cx,6 shl di,2 shr bx,8 |  | 88/86 286 386 486 |  | — 5+n 3 2 |  |
| SAR mem,immed8 | sar sign_count,3 sar WORD PTR [bx],5 |  | 88/86 286 386 486 |  | — 8+n 7 4 |  |
| SAL reg,immed8 SHL reg,immed8 SHR reg,immed8 | shr mem16,11 shl unsign,4 sal array[bx+di],14 |  | 88/86 286 386 486 |  | — 8+n 7 4 |  |

* TTT represents one of the following bit codes: 100 for SHL or SAL, 101 for SHR, or 111 for SAR.

SHLD/SHRD    Double Precision Shift

80386–80486 Only    Shifts the bits of the second operand into the first operand. The number of bits shifted is specified by the third operand. SHLD shifts the first operand to the left by the number of positions specified in the count. The positions opened by the shift are filled by the most significant bits of the second operand. SHRD shifts the first operand to the right by the number of positions specified in the count. The positions opened by the shift are filled by the least significant bits of the second operand. The count operand can be either CL or an 8-bit constant. If a shift count larger than 31 is given, it is adjusted by using the remainder (modulo) of a division by 32.

O	D	I	T	S	Z	A	P	C
?				±	±	?	±	±

Flags

00001111    10100100    mod,reg,r/m    disp (0, 1, or 2)    data (1)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| SHLD reg16,reg16,immed8 SHLD reg32,reg32,immed8 | shld ax,dx,10 | 88/86 286 386 486 | — — 3 2 |
| SHLD mem16,reg16,immed8 SHLD mem32,reg32,immed8 | shld bits,cx,5 | 88/86 286 386 486 | — — 7 3 |

00001111    10101100    mod,reg,r/m    disp (0, 1, or 2)    data (1)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| SHRD reg16,reg16,immed8 SHRD reg32,reg32,immed8 | shrd cx,si,3 | 88/86 286 386 486 | — — 3 2 |
| SHRD mem16,reg16,immed8 SHRD mem32,reg32,immed8 | shrd [di],dx,13 | 88/86 286 386 486 | — — 7 3 |

00001111    10100101    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| SHLD reg16,reg16,CL SHLD reg32,reg32,CL | shld ax,dx,cl | 88/86 286 386 486 | — — 3 3 |
| SHLD mem16,reg16,CL SHLD mem32,reg32,CL | shld masker,ax,cl | 88/86 286 386 486 | — — 7 4 |

00001111    10101101    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| SHRD reg16,reg16,CL SHRD reg32,reg32,CL | shrd bx,dx,cl | 88/86 286 386 486 | — — 3 3 |
| SHRD mem16,reg16,CL SHRD mem32,reg32,CL | shrd [bx],dx,cl | 88/86 286 386 486 | — — 7 4 |

SMSW    Store Machine Status Word

80286-80486 Only    Stores the Machine Status Word (MSW) into a specified memory operand. SMSW is generally useful only in protected mode. See Intel documentation for details on the MSW and other protected-mode concepts.

No change

Flags

00001111    00000001    mod,100,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SMSW reg16 | smsw ax |  | 88/86 286 386 486 |  | — 2 2 2 |  |
| SMSW mem16 | smsw machine |  | 88/86 286 386 486 |  | — 3 3 3 |  |

STC    Set Carry Flag

Sets the carry flag.

O	D	I	T	S	Z	A	P	C
								1

Flags

11111001

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| STC | stc |  | 88/86 286 386 486 |  | 2 2 2 2 |  |

STD    Set Direction Flag

Sets the direction flag. All subsequent string instructions will process down (from high addresses to low addresses).

O	D	I	T	S	Z	A	P	C
	1

Flags

11111101

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| STD | std |  | 88/86 286 386 486 |  | 2 2 2 2 |  |

STI    Set Interrupt Flag

Sets the interrupt flag. When the interrupt flag is set, maskable interrupts are recognized. If interrupts were disabled by a previous CLI instruction, pending interrupts will not be executed immediately; they will be executed after the instruction following STI.

O	D	I	T	S	Z	A	P	C
		1

Flags

11111011

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| STI | sti |  | 88/86 286 386 486 |  | 2 2 3 5 |  |

STOS/STOSB/STOSW/STOSD    Store String Data

Stores the value of the accumulator in a string. The string is the destination and must be pointed to by ES:DI, even if an operand is given. For each source element loaded, DI is adjusted according to the size of the operand and the status of the direction flag. DI is incremented if the direction flag has been cleared with CLD or decremented if the direction flag has been set with STD.

If the STOS form of the instruction is used, an operand must be provided to indicate the size of the data elements to be processed. No segment override is allowed. If STOSB (bytes), STOSW (words), or STOSD (doublewords on the 80386–80486 only) is used, the instruction determines the size of the data elements to be processed and whether the element comes from AL, AX, or EAX.

STOS and its variations are often used with the REP prefix to fill a string with a repeated value. Before the repeated instruction is executed, CX should contain the number of elements to store.

No change

Flags

1010101w

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| STOS [[ES:]] dest STOSB [[[[ES:]] dest]] STOSW [[[[ES:]] dest]] STOSD [[[[ES:]] dest]]* | stos es:dstring rep stosw rep stosb | 88/86 286 386 486 |  | 11 (W88=15) 3 4 5 |  |

* 80386–80486 only

STR    Store Task Register

80286-80486 Only    Stores the current task register to the specified operand. This instruction is generally useful only in privileged mode. See Intel documentation for details on task registers and other protected-mode concepts.

No change

Flags

00001111    00000000    mod, 001, reg    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| STR reg16 | str cx |  | 88/86 286 386 486 |  | — 2 2 2 |  |
| STR mem16 | str taskreg |  | 88/86 286 386 486 |  | — 3 2 3 |  |

SUB    Subtract

Subtracts the source operand from the destination operand and stores the result in the destination operand.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

001010dw    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SUB reg,reg | sub ax,bx sub bh,dh |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| SUB mem,reg | sub tally,bx sub array[di],bl |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 6 3 |  |

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SUB reg,mem | sub cx,discard sub al,[bx] |  | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 7 7 2 |  |

100000sw    mod,101,r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SUB reg,immed | sub dx,45 sub bl,7 |  | 88/86 286 386 486 |  | 4 3 2 1 |  |
| SUB mem,immed | sub total,4000 sub BYTE PTR [bx+di],2 |  | 88/86 286 386 486 |  | 17+EA (W88=25+EA) 7 7 3 |  |

0010110w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| SUB accum,immed | sub ax,32000 |  | 88/86 286 386 486 |  | 4 3 2 1 |  |

TEST    Logical Compare

Tests specified bits of an operand and sets the flags for a subsequent conditional jump or set instruction. One of the operands contains the value to be tested. The other contains a bit mask indicating the bits to be tested. TEST works by doing a bitwise AND operation on the source and destination operands. The flags are modified according to the result, but the destination operand is not changed. This instruction is the same as the AND instruction, except the result is not stored.

O	D	I	T	S	Z	A	P	C
0				±	±	?	±	0

Flags

1000010w    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| TEST reg,reg | test dx,bx test bl,ch |  | 88/86 286 386 486 |  | 3 2 2 1 |  |
| TEST mem,reg TEST reg,mem* | test dx,flags test bl,bitarray[bx] |  | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 6 5 2 |  |

1111011w    mod,000,r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| TEST reg,immed | test cx,30h test cl,1011b |  | 88/86 286 386 486 |  | 5 3 2 1 |  |
| TEST mem,immed | test masker,1 test BYTE PTR [bx],03h |  | 88/86 286 386 486 |  | 11+EA 6 5 2 |  |

1010100w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| TEST accum,immed | test ax,90h |  | 88/86 286 386 486 |  | 4 3 2 1 |  |

* MASM transposes TEST reg,mem; that is, it is encoded as TEST mem,reg.

VERR/VERW    Verify Read or Write

80286-80486 Protected Only    Verifies that a specified segment selector is valid and can be read or written to at the current privilege level. VERR verifies that the selector is readable. VERW verifies that the selector can be written to. If the segment is verified, the zero flag is set. Otherwise, the zero flag is cleared.

O	D	I	T	S	Z	A	P	C
					±

Flags

00001111    00000000    mod, 100,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| VERR reg16 | verr ax |  | 88/86 286 386 486 |  | — 14 10 11 |  |
| VERR mem16 | verr selector |  | 88/86 286 386 486 |  | — 16 11 11 |  |

00001111    00000000    mod, 101,r/m    disp (0, 1, or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| VERW reg16 | verw cx |  | 88/86 286 386 486 |  | — 14 15 11 |  |
| VERW mem16 | verw selector |  | 88/86 286 386 486 |  | — 16 16 11 |  |

WAIT    Wait

Suspends processor execution until the processor receives a signal that a coprocessor has finished a simultaneous operation. It should be used to prevent a coprocessor instruction from modifying a memory location that is being modified simultaneously by a processor instruction. WAIT is the same as the coprocessor FWAIT instruction.

No change

Flags

10011011

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| WAIT | wait |  | 88/86 286 386 486 |  | 4 3 6 1–3 |  |

WBINVD    Write Back and Invalidate Data Cache

80486 Only    Empties the contents of the current data cache after writing changes to memory. Proper use of this instruction requires knowledge of how contents are placed in the cache. WBINVD is intended primarily for system programming. See Intel documentation for details.

No change

Flags

00001111    00001001

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| WBINVD | wbinvd |  | 88/86 286 386 486 |  | — — — 5 |  |

XADD    Exchange and Add

80486 Only    Adds the source and destination operands and stores the sum in the destination; simultaneously, the original value of the destination is moved to the source. The instruction sets flags according to the result of the addition.

O	D	I	T	S	Z	A	P	C
±				±	±	±	±	±

Flags

00001111    1100000b    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| XADD mem,reg | xadd warr[bx],ax xadd string,bl | 88/86 286 386 486 |  | — — — 4 |  |
| XADD reg,reg | xadd dl,al xadd bx,dx | 88/86 286 386 486 |  | — — — 3 |  |

XCHG    Exchange

Exchanges the values of the source and destination operands.

No change

Flags

1000011w    mod,reg,r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| XCHG reg,reg | xchg cx,dx xchg bl,dh xchg al,ah | 88/86 286 386 486 |  | 4 3 3 3 |  |
| XCHG reg,mem XCHG mem,reg | xchg [bx],ax xchg bx,pointer | 88/86 286 386 486 |  | 17+EA (W88=25+EA) 5 5 5 |  |

10010 reg

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| XCHG accum,reg16* XCHG reg16,accum* | xchg ax,cx xchg cx,ax | 88/86 286 386 486 |  | 3 3 3 3 |  |

* On the 80386–80486, the accumulator may also be exchanged with a 32-bit register.

XLAT/XLATB    Translate

Translates a value from one coding system to another by looking up the value to be translated in a table stored in memory. Before the instruction is executed, BX should point to a table in memory and AL should contain the unsigned position of the value to be translated from the table. After the instruction, AL contains the table value at the specified position. No operand is required, but one can be given to specify a segment override. DS is assumed unless a segment override is given. XLATB is a synonym for XLAT. Either version allows an operand, but neither requires one.

No change

Flags

11010111

Encoding

| Syntax | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| XLAT [[[[segreg:]] mem]] XLATB [[[[segreg:]] mem]] | xlat xlatb es:table | 88/86 286 386 486 |  | 11 5 5 4 |  |

XOR    Exclusive OR

Performs a bitwise exclusive OR operation on the source and destination operands and stores the result in the destination. For each bit position in the operands, if both bits are set or if both bits are cleared, the corresponding bit of the result is cleared. Otherwise, the corresponding bit of the result is set.

O	D	I	T	S	Z	A	P	C
0				±	±	?	±	0

Flags

001100dw    mod, reg, r/m    disp (0, 1, or 2)

Encoding

| Syntax | Examples | CPU |  | Clock Cycles |  |
| --- | --- | --- | --- | --- | --- |
| XOR reg,reg | xor cx,bx xor ah,al |  | 88/86 286 386 486 |  | 3 2 2 1 |
| XOR mem,reg | xor [bp+10],cx xor masked,bx |  | 88/86 286 386 486 |  | 16+EA (W88=24+EA) 7 6 3 |
| XOR reg,mem | xor cx,flags xor bl,bitarray[di] |  | 88/86 286 386 486 |  | 9+EA (W88=13+EA) 7 7 2 |

100000sw    mod,110,r/m    disp (0, 1, or 2)    data (1 or 2)

Encoding

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| XOR reg,immed | xor bx,10h xor bl,1 | 88/86 286 386 486 | 4 3 2 1 |
| XOR mem,immed | xor Boolean,1 xor switches[bx],101b | 88/86 286 386 486 | 17+EA (W88=25+EA) 7 7 3 |

0011010w    data (1 or 2)

Encoding

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| XOR accum,immed | xor ax,01010101b |  | 88/86 286 386 486 |  | 4 3 2 1 |  |