Chapter 1

Topical Cross-reference for Coprocessor Instructions	146

Coprocessor

Interpreting Coprocessor Instructions	148

Syntax	148

Examples	148

Clock Speeds	148

Instruction Size	148

Architecture	149

Topical Cross-reference for Coprocessor Instructions

#### Arithmetic

| FABS | FADD/FIADD | FADDP |
| --- | --- | --- |
| FCHS | FDIV/FIDIV | FDIVP |
| FDIVR/FIDIVR | FDIVRP | FMUL/FIMUL |
| FMULP | FPREM | FPREM1§ |
| FRNDINT | FSCALE | FSQRT |
| FSUB/FISUB | FSUBP | FSUBR/FISUBR |
| FSUBRP | FXTRACT |  |

#### Compare

| FCOM/FICOM | FCOMP/FICOMP | FCOMPP |
| --- | --- | --- |
| FSTSW/FNSTSW | FTST | FUCOM§ |
| FUCOMP§ | FUCOMPP§ | FXAM |

#### Load

| FLD/FILD/FBLD | FLDCW | FLDENV |
| --- | --- | --- |
| FRSTOR | FXCH |  |

#### Load Constant

| FLD1 | FLDL2E | FLDL2T |
| --- | --- | --- |
| FLDLG2 | FLDLN2 | FLDPI |
| FLDZ |  |  |

#### Processor Control

| FCLEX/FNCLEX | FDECSTP | FDISI/FNDISI* |
| --- | --- | --- |
| FENI/FNENI* | FFREE | FINCSTP |
| FINIT/FNINIT | FLDCW | FNOP |
| FRSTOR | FSAVE/FNSAVE | FSETPM_ |
| FSTCW/FNSTCW | FSTENV/FNSTENV | FSTSW/FNSTSW |
| FWAIT |  |  |

#### Store Data

| FSAVE/FNSAVE | FST/FIST | FSTCW/FNSTCW |
| --- | --- | --- |
| FSTENV/FNSTENV | FSTP/FISTP/FBSTP | FSTSW/FNSTSW |

#### Transcendental

| F2XM1 | FCOS§ | FPATAN |
| --- | --- | --- |
| FPREM | FPREM1§ | FPTAN |
| FSIN§ | FSINCOS§ | FYL2P1 |
| FYL2X |  |  |

* 8087 only			† 80287 only.			§ 80387–80486 only.

Interpreting Coprocessor Instructions

This section provides an alphabetical reference to instructions of the 8087, 80287, and 80387 coprocessors. The format is the same as the processor instructions except that encodings are not provided. Differences are noted in the following.

The 80486 has the coprocessor built in. This one chip executes all the instructions listed in the previous section and this section.

#### Syntax

Syntaxes in Column 1 use the following abbreviations for operand types:

| Syntax | Operand |
| --- | --- |
| reg | A coprocessor stack register |
| memreal | A direct or indirect memory operand storing a real number |
| memint | A direct or indirect memory operand storing a binary integer |
| membcd | A direct or indirect memory operand storing a BCD number |

#### Examples

The position of the examples in Column 2 is not related to the clock speeds in Column 3.

#### Clock Speeds

Column 3 shows the clock speeds for each processor. Sometimes an instruction may have more than one possible clock speed. The following abbreviations are used to specify variations:

| Abbreviation | Description |
| --- | --- |
| EA | Effective address. This applies only to the 8087. See the Processor Section, “Timings on the 8088 and 8086 Processors,” for an explanation of effective address timings. |
| s,l,t | Short real, long real, and 10-byte temporary real. |
| w,d,q | Word, doubleword, and quadword binary integer. |
| to, fr | To or from stack top. On the 80387 and 80486, the to clocks represent timings when ST is the destination. The fr clocks represent timings when ST is the source. |

#### Instruction Size

The instruction size is always 2 bytes for instructions that do not access memory. For instructions that do access memory, the size is 4 bytes on the 8087 and 80287. On the 80387 and 80486, the size for instructions that access memory is 4 bytes in 16-bit mode, or 6 bytes in 32-bit mode.

On the 8087, each instruction must be preceded by the WAIT (also called FWAIT) instruction, thereby increasing the instruction’s size by 1 byte. The assembler inserts WAIT automatically by default, or with the .8087 directive.

#### Architecture

The 8087, 80287, and 80387 coprocessors, along with the 80486, have several common elements of architecture. All have a register stack made up of eight 80-bit data registers. These can contain floating-point numbers in the temporary real format. The coprocessors also have 14 bytes of control registers. Figure 5.1 shows the format of registers.

§

Fig. 1.1    Coprocessor Registers

The most important control registers are the control word and the status word. Figure 5.2 shows the format of these registers.

§

Fig. 1.2    Control Word and Status Word

F2XM1    2X–1

Calculates Y = 2X – 1. X is taken from ST. The result, Y, is returned in ST. X must be in the range 0 £ X £ 0.5 on the 8087/287, or in the range –1.0 £ X £ +1.0 on the 80387–80486.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| F2XM1 | f2xm1 |  | 87 287 387 486 |  | 310–630 310–630 211–476 140–279 |  |

FABS    Absolute Value

Converts the element in ST to its absolute value.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FABS | fabs |  | 87 287 387 486 |  | 10–17 10–17 22 3 |  |

FADD/FADDP/FIADD    Add

Adds the source to the destination and returns the sum in the destination. If two register operands are specified, one must be ST. If a memory operand is specified, the sum replaces the value in ST. Memory operands can be 32- or 64-bit real numbers or 16- or 32-bit integers. If no operand is specified, ST is added to ST(1) and the stack is popped, returning the sum in ST. For FADDP, the source must be ST; the sum is returned in the destination and ST is popped.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FADD [[reg,reg]] | fadd st,st(2) fadd st(5),st fadd |  | 87 287 387 486 |  | 70–100 70–100 to=23–31, fr=26–34 8–20 |  |
| FADDP reg,ST | faddp st(6),st |  | 87 287 387 486 |  | 75–105 75–105 23–31 8–20 |  |
| FADD memreal | fadd QWORD PTR [bx] fadd shortreal |  | 87 287 387 486 |  | (s=90–120,s=95–125)+EA s=90–120,l=95–125 s=24–32,l=29–37 8–20 |  |
| FIADD memint | fiadd int16 fiadd warray[di] fiadd double |  | 87 287 387 486 |  | (w=102–137,d=108 –143)+EA w=102–137,d=108 –143 w=71–85,d=57–72 w=20–35,d=19–32 |  |

FBLD    Load BCD

See FLD.

FBSTP    Store BCD and Pop

See FST.

FCHS    Change Sign

Reverses the sign of the value in ST.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FCHS | fchs |  | 87 287 387 486 |  | 10–17 10–17 24–25 6 |  |

FCLEX/FNCLEX    Clear Exceptions

Clears all exception flags, the busy flag, and bit 7 in the status word. Bit 7 is the interrupt-request flag on the 8087, and the error-status flag on the 80287, 80387, and 80486. The instruction has wait and no-wait versions.

| Syntax |  | Examples |  | CPU |  | Clock Cycles* |
| --- | --- | --- | --- | --- | --- | --- |
| FCLEX FNCLEX | fclex |  | 87 287 387 486 |  | 2–8 2–8 11 7 |  |

* These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FCOM/FCOMP/FCOMPP/FICOM/FICOMP    Compare

Compares the specified source operand to ST and sets the condition codes of the status word according to the result. The instruction subtracts the source operand from ST without changing either operand. Memory operands can be 32- or 64-bit real numbers or 16- or 32-bit integers. If no operand is specified or if two pops are specified, ST is compared to ST(1) and the stack is popped. If one pop is specified with an operand, the operand is compared to ST. If one of the operands is a NAN, an invalid-operation exception occurs (see FUCOM for an alternative method of comparing on the 80387–80486).

| Syntax | Examples | CPU | Clock Cycles |  |
| --- | --- | --- | --- | --- |
| FCOM [[reg]] | fcom st(2) fcom | 87 287 387 486 | 40–50 40–50 24 4 |  |
| FCOMP [[reg]] | fcomp st(7) fcomp | 87 287 387 486 | 42–52 42–52 26 4 |  |
| FCOMPP | fcompp | 87 287 387 486 | 45–55 45–55 26 5 |  |
| FCOM memreal | fcom shortreals[di] fcom longreal | 87 287 387 486 | (s=60–70,l=65–75)+EA s=60–70,l=65–75 s=26,l=31 4 |  |
| FCOMP memreal | fcomp longreal fcomp shorts[di] | 87 287 387 486 | (s=63–73,l=67–77)+EA s=63–73,l=67–77 s=26,l=31 4 |  |
| FICOM memint | ficom double ficom warray[di] | 87 287 387 486 | (w=72–86,d=78–91)+EA w=72–86,d=78–91 w=71–75,d=56–63 w=16–20,d=15–17 |  |
| FICOMP memint | ficomp WORD PTR [bp+6] ficomp darray[di] | 87 287 387 486 | (w=74–88,d=80–93)+EA w=74–88,d=80–93 w=71–75,d=56–63 w=16–20,d=15–17 |  |

Condition Codes for FCOM

| C3 | C2 | C1 | C0 | Meaning |
| --- | --- | --- | --- | --- |
| 0 | 0 | ? | 0 | ST > source |
| 0 | 0 | ? | 1 | ST < source |
| 1 | 0 | ? | 0 | ST = source |
| 1 | 1 | ? | 1 | ST is not comparable to source |

FCOS    Cosine

80387–80486 Only    Replaces a value in radians in ST with its cosine. If |ST| < 263, the C2 bit of the status word is cleared and the cosine is calculated. Otherwise, C2 is set and no calculation is performed. ST can be reduced to the required range with FPREM or FPREM1.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FCOS | fcos |  | 87 287 387 486 |  | — — 123–772* 257–354† |  |

* For operands with an absolute value greater than p/4, up to 76 additional clocks may be required.

† For operands with an absolute value greater than p/4, add n clocks where n = operand/(p/4).

FDECSTP    Decrement Stack Pointer

Decrements the stack-top pointer in the status word. No tags or registers are changed, and no data is transferred. If the stack pointer is 0, FDECSTP changes 
it to 7.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FDECSTP | fdecstp |  | 87 287 387 486 |  | 6–12 6–12 22 3 |  |

FDISI/FNDISI    Disable Interrupts

8087 Only    Disables interrupts by setting the interrupt-enable mask in the control word. This instruction has wait and no-wait versions. Since the 80287, 80387, and 80486 do not have an interrupt-enable mask, the instruction is recognized but ignored on these coprocessors.

| Syntax |  | Examples |  | CPU |  | Clock Cycles* |
| --- | --- | --- | --- | --- | --- | --- |
| FDISI FNDISI | fdisi |  | 87 287 387 486 |  | 2–8 2 2 3 |  |

* These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FDIV/FDIVP/FIDIV    Divide

Divides the destination by the source and returns the quotient in the destination. 
If two register operands are specified, one must be ST. If a memory operand is specified, the quotient replaces the value in ST. Memory operands can be 32- or 
64-bit real numbers or 16- or 32-bit integers. If no operand is specified, ST(1) is divided by ST and the stack is popped, returning the result in ST. For FDIVP, 
the source must be ST; the quotient is returned in the destination register and ST 
is popped.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FDIV [[reg,reg]] | fdiv st,st(2) fdiv st(5),st |  | 87 287 387 486 |  | 193–203 193–203 to=88, fr=91 73 |  |
| FDIVP reg,ST | fdivp st(6),st |  | 87 287 387 486 |  | 197–207 197–207 91 73 |  |
| FDIV memreal | fdiv DWORD PTR [bx] fdiv shortreal[di] fdiv longreal |  | 87 287 387 486 |  | (s=215–225,l=220– 230)+EA s=215–225,l=220–230 s=89,l=94 73 |  |
| FIDIV memint | fidiv int16 fidiv warray[di] fidiv double |  | 87 287 387 486 |  | (w=224–238,d=230– 243)+EA w=224–238,d=230 –243 w=136–140,d=120 –127 w=85–89,d=84–86 |  |

FDIVR/FDIVRP/FIDIVR    Divide Reversed

Divides the source by the destination and returns the quotient in the destination. If two register operands are specified, one must be ST. If a memory operand is specified, the quotient replaces the value in ST. Memory operands can be 32- or 64-bit real numbers or 16- or 32-bit integers. If no operand is specified, ST is divided by ST(1) and the stack is popped, returning the result in ST. For FDIVRP, the source must be ST; the quotient is returned in the destination register and ST is popped.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDIVR [[reg,reg]] | fdivr st,st(2) fdivr st(5),st fdivr |  | 87 287 387 486 |  | 194–204 194–204 to=88, fr=91 73 |  |  |
| FDIVRP reg,ST | fdivrp st(6),st |  | 87 287 387 486 |  | 198–208 198–208 91 73 |  |  |
| FDIVR memreal | fdivr longreal fdivr shortreal[di] |  | 87 287 387 486 |  | (s=216–226,l=221 –231)+EA s=216–226,l=221–231 s=89,l=94 73 |  |  |
| FIDIVR memint | fidivr double fidivr warray[di] |  | 87 287 387 486 |  | (w=225–239,d=231 –245)+EA w=225–239,d=231 –245 w=135–141,d=121–128 w=85–89,d=84–86 |  |  |

FENI/FNENI    Enable Interrupts

8087 Only    Enables interrupts by clearing the interrupt-enable mask in the control word. This instruction has wait and no-wait versions. Since the 80287, 80387, and 80486 do not have interrupt-enable masks, the instruction is recognized but ignored on these coprocessors.

| Syntax |  | Examples |  | CPU |  | Clock Cycles* |
| --- | --- | --- | --- | --- | --- | --- |
| FENI FNENI | feni |  | 87 287 387 486 |  | 2–8 2 2 3 |  |

* These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FFREE    Free Register

Changes the specified register’s tag to empty without changing the contents of the register.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FFREE ST(i) | ffree st(3) |  | 87 287 387 486 |  | 9–16 9–16 18 3 |  |

FIADD/FISUB/FISUBR/
FIMUL/FIDIV/FIDIVR    Integer Arithmetic

See FADD, FSUB, FSUBR, FMUL, FDIV, and FDIVR.

FICOM/FICOMP    Compare Integer

See FCOM.

FILD    Load Integer

See FLD.

FINCSTP    Increment Stack Pointer

Increments the stack-top pointer in the status word. No tags or registers are changed, and no data is transferred. If the stack pointer is 7, FINCSTP changes it 
to 0.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FINCSTP | fincstp |  | 87 287 387 486 |  | 6–12 6–12 21 3 |  |

FINIT/FNINIT    Initialize Coprocessor

Initializes the coprocessor and resets all the registers and flags to their default values. The instruction has wait and no-wait versions. On the 80387–80486, the condition codes of the status word are cleared. On the 8087/287, they are unchanged.

| Syntax |  | Examples |  | CPU |  | Clock Cycles* |
| --- | --- | --- | --- | --- | --- | --- |
| FINIT FNINIT | finit |  | 87 287 387 486 |  | 2–8 2–8 33 17 |  |

* These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FIST/FISTP    Store Integer

See FST.

FLD/FILD/FBLD    Load

Pushes the specified operand onto the stack. All memory operands are automatically converted to temporary-real numbers before being loaded. Memory operands can be 32-, 64-, or 80-bit real numbers or 16-, 32-, or 64-bit integers.

| Syntax | Examples | CPU | Clock Cycles |
| --- | --- | --- | --- |
| FLD reg | fld st(3) | 87 287 387 486 | 17–22 17–22 14 4 |
| FLD memreal | fld longreal fld shortarray[bx+di] fld tempreal | 87 287 387 486 | (s=38–56,l=40–60,t= 53–65)+EA s=38–56,l=40–60,t= 53–65 s=20,1=25,t=44 s=3,l=3,t=6 |
| FILD memint | fild mem16 fild DWORD PTR [bx] fild quads[si] | 87 287 387 486 | (w=46–54,d=52–60,q=60–68)+EA w=46-54,d=52-60,q= 60-68 w=61–65,d=45–52,q=56–67 w=13–16,d=9–12,q= 10–18 |
| FBLD membcd | fbld packbcd | 87 287 387 486 | (290–310)+EA 290–310 266–275 70–103 |

FLD1/FLDZ/FLDPI/FLDL2E/
FLDL2T/FLDLG2/FLDLN2    Load Constant

FLD1/FLDZ/FLDPI/FLDL2E/FLDL2T/FLDLG2/FLDLN2    Load Constant

Pushes a constant onto the stack. The following constants can be loaded:

| Instruction | Constant |
| --- | --- |
| FLD1 | +1.0 |
| FLDZ | +0.0 |
| FLDPI | p |
| Instruction | Constant |
| FLDL2E | Log2(e) |
| FLDL2T | Log2(10) |
| FLDLG2 | Log10(2) |
| FLDLN2 | Loge(2) |

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FLD1 | fld1 |  | 87 287 387 486 |  | 15–21 15–21 24 4 |  |
| FLDZ | fldz |  | 87 287 387 486 |  | 11–17 11–17 20 4 |  |
| FLDPI | fldpi |  | 87 287 387 486 |  | 16–22 16–22 40 8 |  |
| FLDL2E | fldl2e |  | 87 287 387 486 |  | 15–21 15–21 40 8 |  |
| FLDL2T | fldl2t |  | 87 287 387 486 |  | 16–22 16–22 40 8 |  |
| FLDLG2 | fldlg2 |  | 87 287 387 486 |  | 18–24 18–24 41 8 |  |
| FLDLN2 | fldln2 |  | 87 287 387 486 |  | 17–23 17–23 41 8 |  |

FLDCW    Load Control Word

Loads the specified word into the coprocessor control word. The format of the control word is shown in the “Interpreting Coprocessor Instructions” section.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FLDCW mem16 | fldcw ctrlword |  | 87 287 387 486 |  | (7–14)+EA 7–14 19 4 |  |

FLDENV/FLDENVW/FLDENVD
Load Environment State

Loads the 14-byte coprocessor environment state from a specified memory location. The environment includes the control word, status word, tag word, instruction pointer, and operand pointer. On the 80387–80486 in 32-bit mode, the environment state is 28 bytes.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FLDENV mem | fldenv [bp+10] |  | 87 |  | (35–45)+EA |  |
| FLDENVW mem* |  |  | 287 |  | 35–45 |  |
| FLDENVD mem* |  |  | 387 486 |  | 71 44,pm=34 |  |

* 80387–80486 only.

FMUL/FMULP/FIMUL    Multiply

Multiplies the source by the destination and returns the product in the destination. If two register operands are specified, one must be ST. If a memory operand is specified, the product replaces the value in ST. Memory operands can be 32- or 64-bit real numbers or 16- or 32-bit integers. If no operand is specified, ST(1) is multiplied by ST and the stack is popped, returning the product in ST. For FMULP, the source must be ST; the product is returned in the destination register and ST is popped.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FMUL [[reg,reg]] | fmul st,st(2) fmul st(5),st fmul |  | 87 287 387 486 |  | 130–145 (90–105)* 130–145 (90–105)* to=46–54 (49), fr= 29–57 (52)† 16 |  |
| FMULP reg,ST | fmulp st(6),st |  | 87 287 387 486 |  | 134–148 (94–108)* 134–148 (94–108)* 29–57 (52)† 16 |  |
| FMUL memreal | fmul DWORD PTR [bx] fmul shortreal[di+3] fmul longreal |  | 87 287 387 486 |  | (s=110–125,l=154– 168)+EA§ s=110–125,l=154 –168§ s=27–35,l=32–57 s=11,l=14 |  |
| FIMUL memint | fimul int16 fimul warray[di] fimul double |  | 87 287 387 486 |  | (w=124–138,d=130 –144)+EA w=124–138,d=130 –144 w=76–87,d=61–82 w=23–27,d=22–24 |  |

* The clocks in parentheses show times for short values—those with 40 trailing zeros in their fraction because they were loaded from a short-real memory operand.

† The clocks in parentheses show typical speeds.

§ If the register operand is a short value—having 40 trailing zeros in its fraction because it was loaded from a short-real memory operand—then the timing is (112–126)+EA on the 8087 or 112–126 on the 80287.

FNinstruction    No-Wait Instructions

Instructions that have no-wait versions include FCLEX, FDISI, FENI, FINIT, FSAVE, FSTCW, FSTENV, and FSTSW. Wait versions of instructions check for unmasked numeric errors; no-wait versions do not. When the .8087 directive is used, the assembler puts a WAIT instruction before the wait versions and a NOP instruction before the no-wait versions.

FNOP    No Operation

Performs no operation. FNOP can be used for timing delays or alignment.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FNOP | fnop |  | 87 287 387 486 |  | 10–16 10–16 12 3 |  |

FPATAN    Partial Arctangent

Finds the partial tangent by calculating Z = ARCTAN(Y / X). X is taken from ST and Y from ST(1). On the 8087/287, Y and X must be in the range 0 £ Y < X < ¥. On the 80387–80486, there is no restriction on X and Y. X is popped from the stack and Z replaces Y in ST.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FPATAN | fpatan |  | 87 287 387 486 |  | 250–800 250–800 314–487 218–303 |  |

FPREM    Partial Remainder

Calculates the remainder of ST divided by ST(1), returning the result in ST. The remainder retains the same sign as the original dividend. The calculation uses the following formula:

remainder = ST – ST(1) * quotient

The quotient is the exact value obtained by chopping ST / ST(1) toward 0. The instruction is normally used in a loop that repeats until the reduction is complete, as indicated by the condition codes of the status word.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FPREM | fprem |  | 87 287 387 486 |  | 15–190 15–190 74–155 70–138 |  |

Condition Codes for FPREM and FPREM1

| C3 | C2 | C1 | C0 | Meaning |
| --- | --- | --- | --- | --- |
| ? | 1 | ? | ? | Incomplete reduction |
| 0 | 0 | 0 | 0 | quotient MOD 8 = 0 |
| 0 | 0 | 0 | 1 | quotient MOD 8 = 4 |
| 0 | 0 | 1 | 0 | quotient MOD 8 = 1 |
| 0 | 0 | 1 | 1 | quotient MOD 8 = 5 |
| 1 | 0 | 0 | 0 | quotient MOD 8 = 2 |
| 1 | 0 | 0 | 1 | quotient MOD 8 = 6 |
| 1 | 0 | 1 | 0 | quotient MOD 8 = 3 |
| 1 | 0 | 1 | 1 | quotient MOD 8 = 7 |

FPREM1    Partial Remainder (IEEE Compatible)

80387–80486 Only    Calculates the remainder of ST divided by ST(1), returning the result in ST. The remainder retains the same sign as the original dividend. The calculation uses the following formula:

remainder = ST – ST(1) * quotient

The quotient is the integer nearest to the exact value of ST / ST(1). When two integers are equally close to the given value, the even integer is used. The instruction is normally used in a loop that repeats until the reduction is complete, as indicated by the condition codes of the status word. See FPREM for the possible condition codes.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FPREM1 | fprem1 |  | 87 287 387 486 |  | — — 95–185 72–167 |  |

FPTAN    Partial Tangent

Finds the partial tangent by calculating Y / X = TAN(Z). Z is taken from ST. Z must be in the range 0 £ Z £ p / 4 on the 8087/287. On the 80387–80486, |Z| must be less than 263. The result is the ratio Y / X. Y replaces Z, and X is pushed into ST. Thus, Y is returned in ST(1) and X in ST.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FPTAN | fptan |  | 87 287 387 486 |  | 30–540 30–540 191–497* 200–273† |  |

* For operands with an absolute value greater than p/4, up to 76 additional clocks may be required.

† For operands with an absolute value greater than p/4, add n clocks where n = operand/(p/4).

FRNDINT    Round to Integer

Rounds ST from a real number to an integer. The rounding control (RC) field of the control word specifies the rounding method, as shown in the introduction to this section.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FRNDINT | frndint |  | 87 287 387 486 |  | 16–50 16–50 66–80 21–30 |  |

FRSTOR/FRSTORW/FRSTORD    Restore Saved State

Restores the 94-byte coprocessor state to the coprocessor from the specified memory location. In 32-bit mode on the 80387–80486, the environment state takes 108 bytes.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FRSTOR mem FRSTORW mem* FRSTORD mem* | frstor [bp–94] |  | 87 287 387 486 |  | (197–207)+EA † 308 131,pm=120 |  |

* 80387–80486 only.

† Clock counts are not meaningful in determining overall execution time of this instruction. Timing is determined by operand transfers.

FSAVE/FSAVEW/FSAVED/FNSAVE/
FNSAVEW/FNSAVED    Save Coprocessor State

Stores the 94-byte coprocessor state to the specified memory location. In 32-bit mode on the 80387–80486, the environment state takes 108 bytes. This instruction has wait and no-wait versions. After the save, the coprocessor is initialized as if FINIT had been executed.

| Syntax |  | Examples |  | CPU |  | Clock Cycles§ |
| --- | --- | --- | --- | --- | --- | --- |
| FSAVE mem FSAVEW mem* FSAVED mem* FNSAVE mem FNSAVEW mem* FNSAVED mem* | fsave [bp–94] fsave cobuffer |  | 87 287 387 486 |  | (197–207)+EA † 375–376 154,pm=143 |  |

* 80387–80486  only.

† Clock counts are not meaningful in determining overall execution time of this instruction. Timing is determined by operand transfers.

§ These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FSCALE    Scale

Scales by powers of 2 by calculating the function Y = Y * 2X. X is the scaling factor taken from ST(1), and Y is the value to be scaled from ST. The scaled result replaces the value in ST. The scaling factor remains in ST(1). If the scaling factor is not an integer, it will be truncated toward zero before the scaling.

On the 8087/287, if X is not in the range –215 £ X < 215 or if X is in the range 0 < X < 1, the result will be undefined. The 80387–80486 have no restrictions on the range of operands.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FSCALE | fscale |  | 87 287 387 486 |  | 32–38 32–38 67–86 30–32 |  |

FSETPM    Set Protected Mode

80287 Only    Sets the 80287 to protected mode. The instruction and operand pointers are in the protected-mode format after this instruction. On the 80387–80486, FSETPM is recognized but interpreted as FNOP, since the 80386/486 processors handle addressing identically in real and protected mode.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FSETPM | fsetpm |  | 87 287 387 486 |  | — 2–8 12 3 |  |

FSIN    Sine

80387–80486 Only    Replaces a value in radians in ST with its sine. If |ST| < 263, the C2 bit of the status word is cleared and the sine is calculated. Otherwise, C2 is set and no calculation is performed. ST can be reduced to the required range with FPREM or FPREM1.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FSIN | fsin |  | 87 287 387 486 |  | — — 122–771* 257–354† |  |

* For operands with an absolute value greater than p/4, up to 76 additional clocks may be required.

† For operands with an absolute value greater than p/4, add n clocks where n = operand/(p/4).

FSINCOS    Sine and Cosine

80387–80486 Only    Computes the sine and cosine of a radian value in ST. The sine replaces the value in ST, and then the cosine is pushed onto the stack. If |ST| < 263, the C2 bit of the status word is cleared and the sine and cosine are calculated. Otherwise, C2 is set and no calculation is performed. ST can be reduced to the required range with FPREM or FPREM1.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FSINCOS | fsincos |  | 87 287 387 486 |  | — — 194–809* 292–365† |  |

* For operands with an absolute value greater than p/4, up to 76 additional clocks may be required.

† For operands with an absolute value greater than p/4, add n clocks where n = operand/(p/4).

FSQRT    Square Root

Replaces the value of ST with its square root. (The square root of –0 is –0.)

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FSQRT | fsqrt |  | 87 287 387 486 |  | 180–186 180–186 122–129 83–87 |  |

FST/FSTP/FIST/FISTP/FBSTP    Store

Stores the value in ST to the specified memory location or register. Temporary-real values in registers are converted to the appropriate integer, BCD, or floating-point format as they are stored. With FSTP, FISTP, and FBSTP, the ST register value is popped off the stack. Memory operands can be 32-, 64-, or 80-bit real numbers for FSTP or 16-, 32-, or 64-bit integers for FISTP.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FST reg | fst st(6) fst st |  | 87 287 387 486 |  | 15–22 15–22 11 3 |  |
| FSTP reg | fstp st fstp st(3) |  | 87 287 387 486 |  | 17–24 17–24 12 3 |  |
| FST memreal | fst shortreal fst longs[bx] |  | 87 287 387 486 |  | (s=84–90,l=96–104)+EA s=84–90,l=96–104 s=44,l=45 s=7,l=8 |  |
| FSTP memreal | fstp longreal fstp tempreals[bx] |  | 87 287 387 486 |  | (s=86–92,l=98–106, t=52–58)+EA s=86–92,l=98–106, t=52–58 s=44,l=45,t=53 s=7,l=8,t=6 |  |

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FIST memint | fist int16 fist doubles[8] |  | 87 287 387 486 |  | (w=80–90,d=82–92)+EA w=80–90,d=82–92 w=82-95,d=79-93 w=29–34,d=28–34 |  |
| FISTP memint | fistp longint fistp doubles[bx] |  | 87 287 387 486 |  | (w=82–92,d=84–94, q=94–105)+EA w=82–92,d=84–94, q=94–105 w=82–95,d=79–93, q=80–97 29–34 |  |
| FBSTP membcd | fbstp bcds[bx] |  | 87 287 387 486 |  | (520–540)+EA 520–540 512–534 172–176 |  |

FSTCW/FNSTCW    Store Control Word

Stores the control word to a specified 16-bit memory operand. This instruction has wait and no-wait versions.

| Syntax |  | Examples |  | CPU |  | Clock Cycles* |
| --- | --- | --- | --- | --- | --- | --- |
| FSTCW mem16 FNSTCW mem16 | fstcw ctrlword |  | 87 287 387 486 |  | 12–18 12–18 15 3 |  |

* These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FSTENV/FSTENVW/FSTENVD/FNSTENV/FNSTENVW/
FNSTENVD    Store Environment State

Stores the 14-byte coprocessor environment state to a specified memory location. The environment state includes the control word, status word, tag word, instruction pointer, and operand pointer. On the 80387–80486 in 32-bit mode, the environment state is 28 bytes.

| Syntax | Examples |  | CPU |  | Clock Cycles† |
| --- | --- | --- | --- | --- | --- |
| FSTENV mem FSTENVW mem* FSTENVD mem* FNSTENV mem FNSTENVW mem* FNSTENVD mem* | fstenv [bp–14] | 87 287 387 486 |  | (40–50)+EA 40–50 103–104 67,pm=56 |  |

* 80387–80486 only.

† These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FSTSW/FNSTSW    Store Status Word

Stores the status word to a specified 16-bit memory operand. On the 80287, 80387, and 80486, the status word can also be stored to the processor’s AX register. This instruction has wait and no-wait versions.

| Syntax |  | Examples |  | CPU |  | Clock Cycles* |
| --- | --- | --- | --- | --- | --- | --- |
| FSTSW mem16 FNSTSW mem16 | fstsw statword |  | 87 287 387 486 |  | 12–18 12–18 15 3 |  |
| FSTSW AX FNSTSW AX | fstsw ax |  | 87 287 387 486 |  | — 10–16 13 3 |  |

* These timings reflect the no-wait version of the instruction. The wait version may take additional clock cycles.

FSUB/FSUBP/FISUB    Subtract

Subtracts the source operand from the destination operand and returns the difference in the destination operand. If two register operands are specified, one must be ST. If a memory operand is specified, the result replaces the value in ST. Memory operands can be 32- or 64-bit real numbers or 16- or 32-bit integers. If no operand is specified, ST is subtracted from ST(1) and the stack is popped, returning the difference in ST. For FSUBP, the source must be ST; the difference (destination minus source) is returned in the destination register and ST is popped.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FSUB [[reg,reg]] | fsub st,st(2) fsub st(5),st fsub |  | 87 287 387 486 |  | 70–100 70–100 to=29–37, fr=26–34 8–20 |  |
| FSUBP reg,ST | fsubp st(6),st |  | 87 287 387 486 |  | 75–105 75–105 26–34 8–20 |  |
| FSUB memreal | fsub longreal fsub shortreals[di] |  | 87 287 387 486 |  | (s=90–120,s=95–125)+EA s=90–120,l=95–125 s=24–32,l=28–36 8–20 |  |
| FISUB memint | fisub double fisub warray[di] |  | 87 287 387 486 |  | (w=102–137,d=108143)+EA w=102–137,d=108– 143 w=71–83,d=57–82 w=20–35,d=19–32 |  |

FSUBR/FSUBRP/FISUBR    Subtract Reversed

Subtracts the destination operand from the source operand and returns the result in the destination operand. If two register operands are specified, one must be ST. If a memory operand is specified, the result replaces the value in ST. Memory operands can be 32- or 64-bit real numbers or 16- or 32-bit integers. If no operand is specified, ST(1) is subtracted from ST and the stack is popped, returning the difference in ST. For FSUBRP, the source must be ST; the difference (source minus destination) is returned in the destination register and ST is popped.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FSUBR [[reg,reg]] | fsubr st,st(2) fsubr st(5),st fsubr |  | 87 287 387 486 |  | 70–100 70–100 to=29–37, fr=26–34 8–20 |  |
| FSUBRP reg,ST | fsubrp st(6),st |  | 87 287 387 486 |  | 75–105 75–105 26–34 8–20 |  |
| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| FSUBR memreal | fsubr QWORD PTR [bx] fsubr shortreal[di] fsubr longreal |  | 87 287 387 486 |  | (s=90–120,s=95–125)+EA s=90–120,l=95–125 s=25–33,l=29–37 8–20 |  |
| FISUBR memint | fisubr int16 fisubr warray[di] fisubr double |  | 87 287 387 486 |  | (w=103–139,d=109– 144)+EA w=103–139,d=109– 144 w=72–84,d=58–83 w=20–55,d=19–32 |  |

FTST    Test for Zero

Compares ST with +0.0 and sets the condition of the status word according to the result.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FTST | ftst |  | 87 287 387 486 |  | 38–48 38–48 28 4 |  |

Condition Codes for FTST

| C3 | C2 | C1 | C0 | Meaning |
| --- | --- | --- | --- | --- |
| 0 | 0 | ? | 0 | ST is positive |
| 0 | 0 | ? | 1 | ST is negative |
| 1 | 0 | ? | 0 | ST is 0 |
| 1 | 1 | ? | 1 | ST is not comparable (NAN or projective infinity) |

FUCOM/FUCOMP/FUCOMPP    Unordered Compare

80387–80486 Only    Compares the specified source to ST and sets the condition codes of the status word according to the result. The instruction subtracts the source operand from ST without changing either operand. Memory operands are not allowed. If no operand is specified or if two pops are specified, ST is compared to ST(1). If one pop is specified with an operand, the given register is compared to ST.

Unlike FCOM, FUCOM does not cause an invalid-operation exception if one of the operands is NAN. Instead, the condition codes are set to unordered.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FUCOM [[reg]] | fucom st(2) fucom |  | 87 287 387 486 |  | — — 24 4 |  |
| FUCOMP [[reg]] | fucomp st(7) fucomp |  | 87 287 387 486 |  | — — 26 4 |  |
| FUCOMPP | fucompp |  | 87 287 387 486 |  | — — 26 5 |  |

Condition Codes for FUCOM

| C3 | C2 | C1 | C0 | Meaning |
| --- | --- | --- | --- | --- |
| 0 | 0 | ? | 0 | ST > source |
| 0 | 0 | ? | 1 | ST < source |
| 1 | 0 | ? | 0 | ST = source |
| 1 | 1 | ? | 1 | Unordered |

FWAIT    Wait

Suspends execution of the processor until the coprocessor is finished executing. This is an alternate mnemonic for the processor WAIT instruction.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FWAIT | fwait |  | 87 287 387 486 |  | 4 3 6 1–3 |  |

FXAM    Examine

Reports the contents of ST in the condition flags of the status word.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FXAM | fxam |  | 87 287 387 486 |  | 12–23 12–23 30–38 8 |  |

Condition Codes for FXAM

| C3 | C2 | C1 | C0 | Meaning |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | + Unnormal* |
| 0 | 0 | 0 | 1 | + NAN |
| 0 | 0 | 1 | 0 | – Unnormal* |
| 0 | 0 | 1 | 1 | – NAN |
| 0 | 1 | 0 | 0 | + Normal |
| 0 | 1 | 0 | 1 | + Infinity |
| 0 | 1 | 1 | 0 | – Normal |
| 0 | 1 | 1 | 1 | – Infinity |
| 1 | 0 | 0 | 0 | + 0 |
| 1 | 0 | 0 | 1 | Empty |
| 1 | 0 | 1 | 0 | – 0 |
| 1 | 0 | 1 | 1 | Empty |
| 1 | 1 | 0 | 0 | + Denormal |
| 1 | 1 | 0 | 1 | Empty* |
| 1 | 1 | 1 | 0 | – Denormal |
| 1 | 1 | 1 | 1 | Empty* |

* Not used on the 80387–80486. Unnormals are not supported by the 80387–80486. Also, the 80387–80486 use two codes instead of four to identify empty registers.

FXCH    Exchange Registers

Exchanges the specified (destination) register and ST. If no operand is specified, ST and ST(1) are exchanged.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FXCH [[reg]] | fxch st(3) fxch |  | 87 287 387 486 |  | 10–15 10–15 18 4 |  |

FXTRACT    Extract Exponent and Significand

Extracts the exponent and significand (mantissa) fields of ST. The exponent replaces the value in ST, and then the significand is pushed onto the stack.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FXTRACT | fxtract |  | 87 287 387 486 |  | 27–55 27–55 70–76 16–20 |  |

FYL2X    Y log2(X)

Calculates Z = Y log2(X). X is taken from ST and Y from ST(1). The stack is popped, and the result, Z, replaces Y in ST. X must be in the range 0 < X < ¥ and Y in the range –¥ < Y < ¥.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FYL2X | fyl2x |  | 87 287 387 486 |  | 900–1100 900–1100 120–538 196–329 |  |

FYL2XP1    Y log2(X+1)

Calculates Z = Y log2(X + 1). X is taken from ST and Y from ST(1). The stack is popped once, and the result, Z, replaces Y in ST. X must be in the range 0 < |X| < (1 – (Ö2 / 2)). Y must be in the range –¥ < Y < ¥.

| Syntax |  | Examples |  | CPU |  | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| FYL2XP1 | fyl2xp1 |  | 87 287 387 486 |  | 700–1000 700–1000 257–547 171–326 |  |

µ §

µ §