MY I2C PROJECT

This project is a standard hardware to software implementation, I was told to learn how to program I2C devices, so this became my task. So I went out and found all the information I could on how to program an I2C devices. Just doing my home work…(keeping my job.)

My first task was a TEMP MONITOR and EEPROM  (I2C) interface testing and programming. And with that it just so happened, that some one in the other division had written some software for about the same project. He had done most of my homework. And I want to thank that person. He did a great job in writing the code.

The next problem, what is I2C? Well it turns out that it is a (2) wire communication bus, (no big deal). The next problem was what do I need to drive the bus. Well that turned out to be the parallel port, a least all the books said that. The new problem was what lines do I use to drive this bus and is the logic hi or logic low etc. Well if you look a PHILIPS SEMI they have one set of lines wired one way, if you look a FAICHILD SEMI their driver shows the lines another way etc. (no real standard). Fairchild AP-59 shows I2C driver circuit. The big problem was the software programmers wrote their software to use any and all lines to drive the logic with any and all logic hi or logic low.  And I found out over short runs that you don't need a driver IC you can do it directly out of the parallel port with some pull-up resistors to the eeprom in some cases.

For those of you who are interested the parallel port pins are as follows.

PIN ------------------------------------------------------------Note: RBN was a second I2C   
       device on the same parallel port. Yes you can run more than one I2C on the same 
       parallel port. So long as the software drives don't use the same SCL & SDA lines.

- -STROBE-------------------   RNB  	5.00 VDC software SCL output

- D0----------------------------   RNB 	5.00 VDC

- D1----------------------------   RNB 	5.00 VDC

- D2----------------------------   RNB 	5.00 VDC

- D3----------------------------   RNB 	5.00 VDC

- D4----------------------------   RNB -    SCL /  SDA signals data output to key

- D5----------------------------   RNB -    SCL /  SDA signals data output to key

- D6----------------------------   RNB 	5.00 VDC

- D7-------------FAIRCHILD -SDA   DEMOBOARD TMC2070P7C

- -ACK1-----------------------   RNB  	0.0 VDC

- BUSY  -----------------------  RNB - BUSY   signals /  Fairchild Busy / data input

- PE-----------------------------  RNB  	5.00 VDC

- SLCT-------------------------  RNB       5.00 VDC

- -AUTOFEED---------------- RNB  	0.0 VDC software SDA output

- -ERROR---------------------- RNB  	0.0 VDC software data input

- -INIT-------------------------- RNB  	0.0 VDC

- SLCT IN---- FAIRCHILD  -SCL DEMOBOARD TMC2070P7C

  - GND

For parallel port information see Http://www.lvr.com/parport.htm

The next problem is how do you address a device, lets pick the EEPROM, (HA, HA) well all I2C eeproms have a binary address of (1010) known as the device type identifier. The next part of the binary word is (A2, A1, A0) this is the device address, the next bit (R/W) is the read or write bit.  After that follows your data.  How much data will standard I2C  address, about 16K of memory.  They now have I2C extended data and that can address 32K memory eeproms. Now, for more information on this you need to see FAIRCHILDS AP-NOTES ON I2C. Go to Http://www.fairchildsemi.com/  and lookup eeproms.

Well on to the software. Bear with me on this, there is a lot of information about this and most of this will not apply to what you want to do. But for those of you who want to go into this deep, you will need this VB code . Check out the following sites for I2C VB code.

Http://www.iae.nl/users/pybe/weather/ppdriver.htm

Http://www.lvr.com/parport.htm

Http://ftp.armory.com/pub/user/rstevew/lpt/io.txt

Http://support.microsoft.com/support/kb/articles/q154/0/78.asp

For the standard Qbasic code.

Http://d1.ifmo.ru/library/i2c/i2cibmpc.htm

This code was well written and may be of use to your project. You may have to make changes in the future.

Starting the code. First you have to know the logic levels the hardware is going to see.

Are you going to use driver gates (the 74LS05) these gates are inverter, they reverse the logic bits. You are going to use inverters with a parallel port? I choose dual-inverters it makes the code easy. You don't have to do any bit reversing. The code we wrote was for any EEPROM and an LM80 ic's  (see the source code). Most people don't show this, but it is part of the learning process.  You may need at some time to set the parallel port address in the source code  (you know 378, 379 etc.) LPT port numbers. Also you will need to set the toggle lines for (SCL) and (SDA)…  The executable with this zip file is set to LPT1… and pins 1 (SCL clock out) & 14 (SDA data out), 15 (data return info) I'll save everyone some work.. Now to test our program, we hooked up a scope and measured the I2C lines, well there should only be a clock line running and the data line running. Well it just so happened that we found 4 sets of signals? What the (F*#!k) is this. Well at this point our interest was peaked… and guess what, they were using the binary (1010 ) code address… hum…. What could this be, and why was it running, well being Test-Engineers we just had to get the bottom of this…. Was this going to screw up our software / hardware…???  Of course our code had to be altered to see what this was… We had to probe and see what we were dealing with, it was our job you know. The project had to be finished with no glitches….the boss does not like errors.

So it's not my fault that some hard-key company decided to use I2C for there EEPROM key.  And almost screwed up my project. By the way, here is a schematic of an I2C controlling an EEPROM key (not theirs).

Now with read and write source code, of course you can copy one EEPROM to another EEPROM we had to have this for our project. What does this project cost, Oh about $25.00 and some time, but it is well worth the learning experience. And most of it has been done for you. You will also have to make an interface cable too copy the source key and then program your key. Why you ask, because our program does not use the same (SDA) & (SCL) output lines. You can't see the (SDA) & (SCL) lines from the backside of the key (printer side); they disable the throughput when they access the key.  It makes it easier to see with a scope.  Can't have some one think that we are copying their key… and we can't show you their schematic, we only know the signals that we saw. So below is a schematic of an I2C to EEPROM interface (Ours), I suppose with very little modification it would work other places too. By the way we never tried it, the circuit was just too... simple!

Just a note: Not all hard-keys use the same (SDA & SCL lines) they can very from key 
                     to key you have to look with a scope at each key and find them…. Then you have a 50/50 chance of getting it right its hard to tell the SDA from the SCL without a digital analyzer, or a dual trace storage scope. (don’t have one, borrow one for the day.) Or do some trial and error searching. You will also need a 25 pin adapter cable open wire to look at the pins signals (you need a pass-through cable who's pins you can probe, ribbon cables are good for this you can count the pins from the red stripe).

We suggest you use an I2C demo board to get your software running if you make any changes in our source code. Before you go probing around and screw up YOUR EEPROM - KEY, we suggest that you read and write to the device you build first.

WRITE PROTECT:

The Write Protect pin stops you from writing into part of the memory. If it is tied to VCC then the EEPROM memory is protected from you putting data into it, If you tie the pin low then you can program the whole EEPROM. After you program the key you may want to write protect the EEPROM. Also save the software & file that copied the original key. You may need to make another one.

ADDRESSES:

The address pins are to give each EEPROM is own physical address, you can't address two EEPROMS memories if they don't have different phone numbers… You will have to find the address of the EEPROM you are trying to copy by trial and error (a whole total of 7 address possibilities[A2,A1,A0]). Remember that your address on your key will have to be the same as the key you copy.  Other wise the software will not see it. You should also check all the address lines some keys may have more than one EEPROM in them. That's right, you may have to add a second one in your circuit, the second one is the same as the first just with different addressing. The SDA & SCL line are the same, so is the BUSY line.  Only the device address lines will change. So you connect the devices in parallel except for the address lines.

THE SCHEMATIC:

First this schematic is for a non-passthrough key. That means you can't put a printer or another key behind it. Also I would use IC sockets on the key, you may have to replace a device for sum reason. Remember that this schematic is for OUR-KEY not the one you will be trying to copy. You will have to make some small changes. If you don't know how, find someone who does. The HEX inverter may need to be altered it depends on what the software wants to see. Positive going plus's or Negative. You may have to add a HEX inverter or take one away.  So you are going to have do some playing around to get it right.

ONE FINAL NOTE:

Some I2C companies are developing custom IC's using this bus, and the chip have their own address specified by the customer (type identifier) you may have to go through  several codes. As of now all I2C devices can be read (all memory) but most have write protect. Remember that I2C chips each chip type has its own code  (type identifier) so if you find the code you know what is in the key. With a few smarts you can change the source code we supplied into a key buster. Just run all (type id)+(address id) and store the info. You now have what is inside the key.

So You want to be a key builder!    Don't !!!

You would just be wasting your time. There is nothing in hardware that runs fast enough to keep up with the software.  That because the ports off the main CPU are all low speed…. The other problem is that windows 98 & 2000 have I2C addressing built right into them. The hackers would have your code cracked in days. Why did it work before…. Well up until now not too many people used I2C and there were limited drivers. That’s right you had to write your own I2C (Dll or Vxd) to link into your code, and tell it what eeprom memory register to get and transfer that back to your software.

Don't get me wrong a couple of 32 bit prime numbers are hard to crack. It does take some CPU time. But hardware wise you are naked, any hardware engineer would see through what you did in a moment. And there is not much you can do to protect yourself. A digital analyzer is just like a telephone tap. It can see all that special code and it can capture it too. So you would still be screwed. It took us about 2 minutes to figure out what was in the key.

My suggestion is keep your software at a reasonable price and offer an upgrade service on a yearly fee, and change your software at least twice a year, you will get more money that way.

And now that I have the chance: You software companies that charge in the thousands of dollars and hundreds of thousands of dollars range, deserve what you get from the hackers, all your software is over priced, and in most cases you give poor customer support. ( You Know Who You Are  ), I have talked to a lot of you and heard that cry of, we are just keeping up with the other companies price…… 
That’s a bunch of (C#@P)

For some final notes and added help:

What is a scope?  Well its call an oscilloscope.

What is a dual trace storage scope? That is a scope that has two traces the can store the signal of the digital pulses.

What is a digital analyzer? That is a black box that can capture and store digital pulses from and digital device. CPU, MEMORY, PARALLEL PORT, etc. They usually have at least 16 lines of input data and some as many as 32 - 64.

You can also have a software digital analyzer built right into your parallel port. How?

Here is how. http://et.nmsu.edu/~etti/winter97/computers/logic/logic.html

And the zip source code. http://et.nmsu.edu/~etti/winter97/computers/logic/logic.zip

Now this logic analyzer runs off your parallel port and contains 8 bits of info, and with a little modification you can look at all the pins and if you have an EPP port you can capture signals up to 350 kHz or 350,000  8 bit words a second.  This is good enough to capture any hard-key. That’s because the EPP port maximum speed is that.

Any I2C device (hardware) can have only one of 16 different ID's, not to many to choose from. The I2C source software supplied with this document uses LPT1 pins 1 SCL, 14 SDA data-out, 15 SDA data-input, if you use a direct 2 wire bus to a device you need to tie pins 14 & 15 together to address that device.

Now you may need some more help with the parallel port. Try this URL.
http://www.doc.ic.ac.uk/~ih/doc/par/   ………….or  this URL.
http://www.doc.ic.ac.uk/~ih/doc/

Now this should answer any questions about how to address a port.

Remember that all hard keys are simple the only thing they can do is put a few parts in them. And for those of you who think an ASIC is how they did it, I have some news for you, an ASIC costs about $30,000- $100,000 each time you engineer one. That's the chip cost. Not including the engineering time and the cost of production of a custom CHIP. These guys use off the shelf parts ……!