I2C WAVE FORMS

Device TYPE or ID 0101 hex inverted EEPROM

ADDRESS of the device on the I2C bus.

SDA

SCL

Data Inf.

This is the device ID expanded it does not have nice clean pulses like people expect. But the address is 0101 that is because they are using a hex inverter to drive the EEPROM so the logic has to be reversed. This is read as 1010 and that is an EEPROM

0

1

0

1

Now the reason we don't show the rest of the address is because each key can be different. It depends on what key you have. You will have to do trial and error. Testing to find what address the device uses. We have not shown the feed back line, here again the info feed back may or may not need to be inverted. The software determines what hardware device is need for driving and receiving data from the EEPROM.