Protel-Easytrax is a powerful software package for producing printed

circuit board (PCB) artwork.  It is an ideal system for those who require an

easy introduction to PCB design or for those who do not have complicated

boards to design.  It runs on the IBM-PC/XT/AT and PS/2 series and close

compatibles.

This chapter details the hardware and software you need to run the

program and provides some advice on installing the software.

HARDWARE CHECK LIST

You will need the following:

- An IBM microcomputer or compatible

- 2 floppy disk drives or one floppy drive and one hard drive

- Graphics card (to match graphics drivers supplies)

- Colour monitor

- One parallel printer port

- Microsoft mouse or compatible (recommended)

- Draft printer

- MS-DOS or PC-DOS (Version 2.0 or greater)

For the exact hardware requirements, you should check Appendix A at the back

of this manual.

The maximum size of a PCB file which can be loaded by Protel Easytrax in

the standard 640K RAM is approximately 235K bytes.  Files bigger than this

will not load and therefore require the use of expanded memory systems (EMS).

If you decide to purchase extra memory, please note that you require an

Expanded Memory Board rather than an Extended Memory Board.  The board should

conform to the widely used LIM standard.  LIM 3.2 to 4.0 is currently

supported.  We have used the "Hyperam AT" board by Hypertech when designing

the program.

If you have purchased a computer which has 1 Mbyte of memory on the

motherboard, it is likely that the extra memory acts as extended memory

rather than expanded memory.  You may have been supplied with some software

which emulates expanded memory. We cannot guarantee that this software works

correctly with Protel Easytrax.

Most boards can be configured to be either Expanded or Extended - you

should make sure your board is set to be Expanded.  However, you should avoid

running Protel Easytrax with software which emulates Expanded memory as it

will run much slower than normal.

SOFTWARE REQUIREMENTS

In addition to the Protel Easytrax software, you will need a copy of

either the MS-DOS or PC-DOS operating system.  The version number should be

2.0 or greater.

This manual assumes that you are familiar with some DOS operations such

as creating directories, changing directories, specifying path names,

formatting disks, deleting files, and copying files.

The software provides automatic backups of your files, but it is strongly

recommended that a regular backup of PCB files is made onto a different disk.

You can make backups by using the DOS Copy command.  For example, to copy

all your work files (which have a PCB extension) from the \FILES directory

on the hard disk to a floppy disk in drive A:, you might type an instruction

such as:

COPY *.PCB A:

provided you are currently in the \FILES directory.

INSTALLING THE SOFTWARE ON A HARD DISK

You should immediately make a backup copy of your disk.

If you are using the programs to a hard disk, make a subdirectory

called \Easytrax and one called \Files.

*NOTE* It is strongly recommended that you store your work files in a

different subdirectory to the Protel Easytrax systems files.

You have created the \Files subdirectory for storing work you might

create while using the program.  This also makes it easier for you to make

backups of your work.

The following instructions provide one means of setting up your hard

disk:

Move to the top directory on disk C: by typing a command such as:

CD\

Type:  MD Easytrax and press "ENTER"

Type:  MD Files and press "ENTER"

Type:  CD Easytrax and press "ENTER"

The prompt should now indicate that you are in the Easytrax subdirectory

(e.g. C:\Easytrax>)

Put the Program Disk in drive A:

Type:  COPY A:*.* and press "ENTER"

All the programs and files should now be copied on your hard disk.

If you are decompressing the BBS EasyFree.EXE File

a) run EasyFree - the file will de-compress

b) copy the de-compressed files to a HD 5.25" or a LD 3.5" (except EasyFree.exe)

c) run the Install Program From the Floppy and Follow the on Screen Prompts.

If you want the computer to place you in the \Files subdirectory

automatically, then you will need to build a batch command file ET.BAT.

The command example you need to use is:

C: (or what ever hard drive is used)

cd\easytrax\files

c:\easytrax\easyedit

If you do not know how to edit a batch file, then seek help-

it is not a good idea to create this file if you are not confident about the

DOS environment.

You should also alter the Path command in the AUTOEXEC.BAT file so that

it points to the directory in which you have stored the Protel Easytrax

programs and files.

The next step is to select a graphics driver program which suits the

graphics card you have in your microcomputer.

When the Protel Easytrax program is loaded it looks for a file called

GRAPH.DRV.  This file contains the information required to drive the graphics

display.  If no GRAPH.DRV file is found, the software assumes that you are

using a CGA card.  If you are using an EGA, VGA, Hercules, Paradise, Genoa,

or Vega Deluxe card then you must create the appropriate GRAPH.DRV file.

The instruction you should type at the DOS prompt is shown in the table

below.  Type the appropriate instruction and then press "ENTER" or click

"LEFT MOUSE".

CARD TYPE                                       INSTRUCTION

EGA                                             EGA

VGA Standard                                    VGA640

Vega Deluxe                                     VEGA

Hercules                                        HERC

Paradise 800                                    PARA800

Genoa EGA 640                                   GENOA640

Genoa EGA 800                                   GENOA800

Genoa EGA 912                                   GENOA912

If you later wish to return to using a CGA card type:

CGA then press "ENTER"

and the GRAPH.DRV file will be deleted.

Note that further graphics drivers may be made available in the future.

STARTING UP PROTEL EASYTRAX

Using a hard drive system, you can run the programs from any directory,

but you may want to move into the directory in which you want to store your

PCB files just to be on the safe side.

If it is the first time you have run the program, select the appropriate

graphics driver as described on the previous page.

You are now ready to run the program.  Simply type:

EASYEDIT and press "ENTER"

and the program will start.

You should now attempt some of the tutorials which will help you become

familiar with the features of Protel Easytrax.

TUTORIAL SECTION

GETTING STARTED

This tutorial shows you how to start the Protel Easytrax program, load

the demonstration board and then quit from the program.  It also introduces

you to some of the conventions used in the manual and some of the keys you

need to be familiar with.

It is suggested that you complete the first four tutorials before

attempting to create your own design.

In this tutorial you will learn the following skills:

using the keyboard;

identifying keys and using key combinations;

starting the Protel Easytrax program;

loading the DEMO file;

using the pop-up menus;

expanding and contracting the display;

identifying items on the board;

quitting from Protel Easytrax.

USING THE KEYBOARD

Throughout this manual a number of conventions have been used to help

you decide what to type.  One convention is that when a particular key is

referred to, it will be enclosed in chevrons.  Thus if you are asked to type

the key labelled A it will be enclosed in chevrons thus, <<A>>.  If you are

asked to hold down the key marked SHIFT it will be referred to as <<SHIFT>>,

and so on.

The main section of the keyboard comprises a set of letter keys and

number keys as well as some special keys.  The one you need to become familiar

with immediately is the key labelled <<ENTER>> or sometimes <<RETURN>> or

sometimes simply an arrow.  This key is often used to signal that you have

finished entering information.

Some keys are used in conjunction with one other key.  These are the

<<CTRL>>, <<SHIFT>>, and <<ALT>> keys.  Often you will be asked to type

something like <<CTRL>> <<A>>.  This means hold the <<CTRL>> key down with

one finger and press the <<A>> key with another.  Another important key is

<<BACKSPACE>> which deletes the character to the left of the cursor.

Keyboards vary considerably in their layout, but most have a set of

function keys which are labelled F1, F2, etc.  Sometimes these are on the

left hand side of the keyboard and sometimes they form a line along the top.

The use of function keys is optional with Protel Easytrax but they can speed

up operation considerably.

On the right hand side of the keyboard there is a numeric keypad, which

may also include a set of cursor keys.  (Look for the number keys which also

have arrows on them).  Some keyboards also have a set of cursor keys (the

keys with arrows on them) and some special keys between the main part of the

keyboard and the numeric keypad.  The arrow keys will be referred to as

<<LEFT>>, <<RIGHT>>, <<UP>>, and <<DOWN>>.  Other keys which will be used

often are those marked <<HOME>>, <<END>>, <<PGUP>>, and <<PGDN>>.  The <<DEL>>

key is used to delete the character which is currently highlighted.

If you have an "AT"-style keyboard with the extra set of cursor keys

between the main keyboard and the numeric keypad, then it is possible that

the <<SHIFT>> key will not work in conjunction with these cursor keys.

If you are using a mouse then the left button will be referred to as

<<LEFT MOUSE>> and is similar in operation to the <<ENTER>> key.  The right

button will be referred to as <<RIGHT MOUSE>> and is similar in operation to

the <<ESC>> key.  This usually takes you back one step or cancels an operation.

LOADING THE DEMONSTRATION BOARD

Using your hard drive, change to the directory on the hard disk from

which you want to run the Protel Easytrax programs.

Type:  EASYEDIT and then press <<ENTER>>.

The banner screen appears.  Press any key to move to the next screen.

A window opens in the top left-hand corner of the screen.  This is a standard

input window which you see many times while using Protel Easytrax.

LOAD PCB FILE NAME

*.PCB

Type DEMO and then either press the <<ENTER>> key or click the <<LEFT

MOUSE>> button.  (NOTE:  If, at this stage you did not want to load a file you

could press <<ESC>> or click the <<RIGHT MOUSE>> button).  The message:

LOADING FILE. PLEASE WAIT.

appears, then a diagram showing a simple Z80 microcomputer layout is displayed.

Notice the status line at the bottom of the screen which gives important

information about the board and the position of the cursor.  If you press the

cursor keys a few times, the numbers which indicate the position of the

cursor change.

The X and Y values are given relative to the position of the "origin"

which always has an X value of 0 and a Y value of 0.  It is written as X:0

Y:0.  If you have not changed the position of the origin, it is in the bottom

left hand corner of the board.

To reach a position given by X:500 Y:1000 when the cursor is at the

origin, you move the cursor 500 mils to the right then 1000 mils upwards.

Notice that all measurements are given in either mils or inches.  A mil

is one thousandth of an inch.

X:500 Y:1000 L:Top Layer     P:Round62 T:12 S:60 G:25

Next to the cursor position on the status line is the currently selected

layer.  In Protel Easytrax, tracks can be placed on a number of layers.  These

include the Top Layer, the Bottom Layer, and two Mid Layers.  Components are

always placed on the Top Layer and silk screening is usually reserved for the

Overlay.  You can switch these layers using the keys described at the end of

this tutorial.

Next to the currently selected layer, four other variables are displayed.

They are the current pad type (P:), the current track width (T:), the current

string height (S:), and the current value of the snap grid (G:).

The line below the status line is used to display important messages.

You should constantly check this line to see if there is a prompt displayed.

In this manual, the bottom line is referred to as the prompt line.

USING THE MENUS

Press the <<ENTER>> key or click the <<LEFT MOUSE>> button and the

MAIN MENU window (shown below) appears.

MAIN

Block

Current

Edit

File

Grid

Highlight

Information

Jump

Library

Move

Place

Repeat

Setup

Un-Delete

Zoom

One of the options in the menu is highlighted.  The highlight is called

the Selection Bar.  It can be moved with the mouse or the <<UP>> and <<DOWN>>

cursor keys.  You can "activate" or "select" a highlighted option by clicking

the <<LEFT MOUSE>> button or by pressing the <<ENTER>> key.

The demo circuit is too small to work with easily, so it needs to be

expanded.  Choose ZOOM from the MAIN MENU by highlighting it then clicking

the <<LEFT MOUSE>> button or pressing the <<ENTER>> key to select the option.

The following window appears:

ZOOM

Redraw

Pan

Expand

Contract

All

Select

Window

Select EXPAND.  The screen is redrawn and the diagram shown in more detail.

Select ZOOM EXPAND once again.  Notice the convention used here.  If you are

asked to select an option from a menu, the menu option is listed in italics

with the first character in bold type.  Thus a request to select ZOOM EXPAND

means choose the ZOOM option, then choose the EXPAND option.

IDENTIFYING ITEMS ON THE BOARD

Now move the mouse until the cursor is at X:1075 Y:3525.  If you are not

using a mouse, use the cursor keys.  If you hold the <<SHIFT>> key down while

pressing a cursor key on the numeric keypad, the cursor moves faster.  Try it.

As the cursor moves towards the position specified, notice that the

screen may be redrawn once or twice.  Remember that the cursor position is

displayed on the status line.

At X:1075 Y:3525 the cursor is at the junction of two tracks.  These are

the conductive lines which are usually placed on the top and bottom board

layers.  Thin tracks (e.g. 12 mils) are used for normal signals and thick

tracks (e.g. 30 mils) can be used for the power supply.

The horizontal track to the left has been laid on the bottom layer.  The

vertical track below the cursor is on the top layer.  They are connected by a

"via" which is a plated hole passing through the board which connects the

tracks.

Just to the left of the cursor is the outline of a component.  It is a

14 pin chip.  Notice at the top right of the chip there is a component

designator U11 and a description showing that it is a 1488 chip.

The component designator or a reference designator usually consists of a

letter indicating the type of component (e.g. U for an integrated circuit chip

C for a capacitor, R for a resistor, and so on), followed by a number.

One convention suggests that you should number each component in a

particular class starting from the top left hand corner of the board.  Thus

the capacitor closest to the top left hand corner should be C1, the next C2,

and so on.

Notice that the component has 13 circular "pads" and one square "pad".

The square pad is used to signify pin 1 on the chip.  The pads are the place

where the connection is made between the pins of the component and the tracks.

QUITTING FROM THE PROGRAM

It is important to know how to get out of the program at this stage.

Call up the MAIN MENU again by pressing <<ENTER>> or clicking <<LEFT MOUSE>>.

Select the File option and then select QUIT.

FILE

Clear

Dos

Files

Load

Path

Quit

Save

A window opens which asks you to confirm that you want to quit to DOS.

CONFIRM QUIT TO DOS

No

Yes

Choose the YES option if you do want to quit.

By now you should be familiar with the menu structure of Protel Easytrax

and how to use the mouse and the cursor keys.  You are given the opportunity

to reinforce these skills in the next tutorial.

SOME SPECIAL KEYS

There are some other keys which have a special function in the Protel

Easytrax program.  These are explained below:

<<END>>          Redraws or "refreshes" the screen

<<HOME>>         Centers the screen about the current cursor position

<<PGUP>>         This key has the same effect as choosing ZOOM EXPAND

<<PGDN>>         This key has the same effect as choosing ZOOM CONTRACT

<<*>>            This key is on the right of the keypad.  Sometimes it

also has <<PRTSC>> on the same key.  It is used to swap

between the currently selected signal layers (usually

the top layer and the bottom layer).  The effect can

be observed on the status line.

<<->>            This key is also on the keypad.  It is used to cycle

through all the currently selected layers.

<<+>>            This key is also on the keypad.  It is used to cycle

through the currently selected layers in the opposite

direction to the <<->> key.

<<SPACE>>        Stops a redraw of the screen when <<PGUP>>, <<PGDN>>,

or <<END>> have been used.

SPEEDING UP MENU SELECTIONS

Before finishing this tutorial you should be aware of a feature of

Protel Easytrax which helps speed up menu selections once you become familiar

with the menu structure.

Each menu option, including any of the MAIN MENU options can be chosen

by simply pressing the first key of the option to be selected.  Thus to choose

ZOOM EXPAND you only need to type Z then E.

TUTORIAL: USING MENUS

In the first tutorial you learned how to start the program and select

options from the menus.  In this tutorial you are given the opportunity to

practise these skills further.

In this tutorial you will learn the following skills:

getting information about the board;

locating components, tracks, vias, and pads;

changing setup options

MORE PRACTISE WITH MENUS

Select the MAIN MENU, then select Information.  (Remember, you could

also just press the letter I).  The following window appears:

INFORMATION

Board Dimensions

Components

Highlighted Pins

Library Components

Pwr/Gnd Pins

Status

Choose the Status option from the Information window.  Notice that information

is provided about the number of components, pads, holes, arcs, fills, strings,

tracks, and vias.  Each of these terms are described in more detail later.

You will also notice that information is provided about the name of the

current work file, the amount of free memory in the microcomputer, the current

path to the directory where the work files are stored, and the name of the

currently selected library.  You can also check the amount of disk space that

is available.

Click either mouse button or press the <<ENTER>> or <<ESC>> key to exit

from the status window.

STATUS

Free Memory                28559  Bytes

Free EMS Memory            0      Bytes

Arcs                       15

Components                 36

Fills                      0

Holes                      520

Pads                       435

Strings                    5

Tracks                     1554

Vias                       85

Work File Name             DEMO.PCB

Library                    EASYSTD.LIB

Current Path               C:\EASYTRAX

Disk Size                  21309440 Bytes

Disk Free                  180224   Bytes

Now choose Information Board Dimensions.  This window displays the dimensions

of the board.  Exit from this window and then choose Information Components

to see a list of the library components which have been placed on the board.

Remember, click the <<RIGHT MOUSE>> button or press the <<ESC>> key to exit

from each of these options.

You can find more information about a particular component by using the

Edit menu.  Choose Edit and the following menu appears:

EDIT

Component

Pad

Track

String

Via

Now choose Component.  The message on the prompt line (just below the status

line) prompts you to select a component.  Move the cursor until it is positioned

on the Z80 chip marked U5 (approximately X:1200 Y:1200).

Now click <<LEFT MOUSE>> or press <<ENTER>> and a window opens showing

the component designator, the library name (pattern) and a comment.

EDIT COMPONENT

Designator :  U5

Pattern    :  DIP40

Comment    :  Z80A CPU

Click <<RIGHT MOUSE>> or press <<ESC>> to remove the window.

Notice that the:   SELECT COMPONENT

prompt still appears on the prompt line.  Click the <<RIGHT MOUSE>> button or

press <<ESC>> again to exit from the EDIT option.

JUMPING TO COMPONENTS AND PINS

Now choose the Jump option from the MAIN MENU.  You can choose to jump

to a place on the board by specifying a component, a location, the origin, a

pad, or a string.

JUMP

Component

Location

Origin

Pad

String

This option will be extremely useful in the future when you are working with

a large board.

The Jump option is also useful if you wish to identify the pin numbers

for a component.  Choose Jump Component and you are asked to select a

component.  Notice that there is a question mark (?) on the left hand side

of the input window.  This indicates that there is an information window

available which will provide a list of components held in the memory of the

computer.

COMPONENT DESIGNATOR

?

Press <<ENTER>> or click <<LEFT MOUSE>> and the window with a range of choices

appears.  This is a list of the component designators.  Move the selection

bar to highlight J2 (a DB9 connector), then press <<ENTER>> or click <<LEFT

MOUSE>>.

The cursor moves to that component on the board and is situated at pin 1.

Now choose Jump Pad.  You will be asked to select a component.  Since the

cursor is already over the required component, press <<ENTER>> or click

<<LEFT MOUSE>>.  A window opens at the top of the screen prompting you to

enter a pin number.

Enter a pin number such a 5 then press <<ENTER>> or click <<LEFT MOUSE>>.

Notice that the cursor moves to that pin.  The window stays open allowing you

to try other pin numbers.  Click <<RIGHT MOUSE>> or press <<ESC>> to exit from

this window.

Now choose the Setup option from the MAIN MENU.  This menu allows you to

change layer colours, menu colours, the default pad type, and many other

factors which affect the layout.

SETUP

Component Text

Layer Colours

Menu Colours

Keys

Options

Redraw

Strings

Toggle Layers

Try changing the menu background colour by choosing Menu Colours then

clicking <<LEFT MOUSE>> or pressing <<ENTER>> and observing the changes in

colour.  Press <<ESC>> or click <<RIGHT MOUSE>> when you have finished

selecting colours.

By now you should be familiar with the menu structure of Protel Easytrax

and some of the features of board layout.  No attempt has been made to place

components or tracks or to change layers.  This will be done in subsequent

tutorials.

TUTORIAL: PLACING COMPONENTS

Protel Easytrax makes it easy for you to place components on the board

by providing you with a large number of pre-defined components in a file

called the Component Library.  In a later tutorial you learn how to create

your own components and your own component libraries.

In this tutorial you are shown how to select components from the library

and place them on the board.  In the next tutorial you are shown how to

connect pins on one component to pins on another component by laying tracks.

You will learn the following skills:

clearing the work space;

specifying a path;

selecting a library;

choosing a component from the library;

placing a component;

assigning component designators and comments;

moving a component;

saving a board.

GETTING STARTED

The aim of this tutorial is to select three components from the component

library and place them on the board.

You must first make sure that the board is clear.  Either restart the

Protel Easytrax program or choose File Clear Yes.  If you start the program

again, then press <<ESC>> or click the <<RIGHT MOUSE>> button in response to

the prompt "LOAD PCB FILE NAME".

SETTING THE PATH

If you are using a dual floppy drive system, it is likely that you want

to save your files on disk B:.  In this case choose File Path.  You should

then enter the path name:

B:\

This also means that any files you save are stored on this drive unless

you specify otherwise.

SELECTING THE LIBRARY

If you want to choose components from one of the libraries you must first

ensure that the library you want to choose the components from is the

currently selected library.

If the library has not been loaded, choose Library File.  A window opens

asking you to enter the name of the library file.  If you are using a floppy

drive system type:

EASYSTD.LIB

If you are using a hard drive system, type in the full path name of the

library file.

Now press <<ENTER>> or click <<LEFT MOUSE>> and the library becomes the

currently selected library allowing you to choose components from it.

PLACING COMPONENTS

Now press <<PGUP>> three times to expand the display.  Check the status

line to make sure that the currently selected layer is the Top Layer.  If it

isn't, then use the <<*>> key to select that layer.

Now choose Place Component.  An input window with the heading "NAME IN

LIBRARY" opens asking for the name of a pre-defined component.  Notice that

there is a question mark on the input line.

Remember, when you see a question mark in an input window such as this

it means that there is an information window available which lists the options

you can choose.  This means that if you now press <<ENTER>> or click <<LEFT

MOUSE>> the components in the library are listed.

In this case, however, you know the name of the component so you can type

it in.  Enter the name DIP14 and press <<ENTER>> or click <<LEFT MOUSE>>.

A window now opens and you are prompted to supply a component designator.

The question mark in the window means that if you press <<ENTER>> or click

<<LEFT MOUSE>>, the program automatically assigns a designator starting with

A1, then A2, and so on.

A component designator is usually a letter such a U, J, C, or R followed

by a one or two digit number.  Connectors are often given designators starting

with J, ICs are often given the designator U and capacitors the designator C.

Since the DIP14 is an IC, enter U1 then press <<ENTER>> or click <<LEFT

MOUSE>>.

You are now prompted to enter a short comment which appears on the over-

lay.  Type in 74LS04 and an outline of the component appears.

You now see the message:

MOVING COMPONENT

on the prompt line at the bottom of the screen.  Check the status line.  If

the cursor is not at X:1000 Y:1000, use the mouse or the cursor keys to move

the component to that position.  Press <<ENTER>> or click <<LEFT MOUSE>> to

fix the component position.

The steps you have carried out to place this component are:

Choose Place Component

Type DIP14

Press <<ENTER>> or click <<LEFT MOUSE>>

Type U1

Press <<ENTER>> or click <<LEFT MOUSE>>

Type 74LS04

Press <<ENTER>> or click <<LEFT MOUSE>>

Move the cursor to X:1000 Y:1000

Press <<ENTER>> or click <<LEFT MOUSE>>

The next step is to place a capacitor above the DIP14 chip.  At this

stage you are still in Place Component and the input window has the component

name DIP14 in it, since that was the last component chosen.

Type in the component name RAD.2 and the previous name disappears.

Enter the designator C1 and the comment 0.1uF.  Press <<ENTER>> or click

<<LEFT MOUSE>> after each entry.  The outline of the component now appears.

Use the cursor keys or the mouse to move it to X:1000 Y:1350.  Once again

press <<ENTER>> or click <<LEFT MOUSE>> to fix the component in place.

The steps you have carried out to place this component are:

Type RAD0.2

Press <<ENTER>> or click <<LEFT MOUSE>>

Type C1

Press <<ENTER>> or click <<LEFT MOUSE>>

Type 0.1uF

Press <<ENTER>> or click <<LEFT MOUSE>>

Move the cursor to X:1000 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Now choose the component labelled AXIAL0.4.  Assign the designator R1

and the comment 330R.  Use the mouse or the cursor keys to move the resistor

to X:150 Y:1350.  Press <<ENTER>> or click <<LEFT MOUSE>> to fix the resistor

in place.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from the Place Component

option.

The steps you have carried out to place this component are:

Type AXIAL0.4

Press <<ENTER>> or click <<LEFT MOUSE>>

Type R1

Press <<ENTER>> or click <<LEFT MOUSE>>

Type 330R

Press <<ENTER>> or click <<LEFT MOUSE>>

Move the cursor to X:150 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Press <<ESC>> or click <<RIGHT MOUSE>>

If you want to save your work, choose File Save and enter a filename.

Press <<ENTER>> or click <<LEFT MOUSE>> and the file is saved to the disk.

Remember that it is saved in the directory specified in the PATH variable

unless you specify otherwise.

*NOTE*  When you quit from Protel Easytrax, all the latest setup

parameters you have used are saved in a file called EASYEDIT.DFT.  This file

is loaded automatically whenever you use the Protel Easytrax program.  Thus,

if you change the menu colour, the type of cursor, the backup interval or

the name of the currently selected library, Protel Easytrax remembers this

and loads those parameters next time you use the program.  If you want to

restore the default parameters, simply delete the .DFT file.

TUTORIAL:  LAYING TRACKS

Now that you have placed three components on the board, the next task is

to connect some of the pins with tracks.  In this tutorial, pins are connected

with 12 mil tracks and 30 mil tracks.  The wider tracks are capable of

carrying higher currents.

In this tutorial you will learn the following skills:

identifying pins on components;

placing tracks;

understanding track separation;

changing the track width;

switching layers;

identifying vias;

getting information about the board.

The discussion which follows assumes that you are designing a two-sided

board.  If you are only going to be working with one-sided boards, don't

worry.  The principals involved are exactly the same.  You just have to plan

the placement of tracks more carefully on a one-sided board.

The tutorial also relies on the fact that you are going to place tracks

manually.  Protel Easytrax does not have a facility to allow you to place

tracks automatically - a process known as routing.

IDENTIFYING PINS

You are now going to connect some pins with tracks.  In order to explain

which pins are to be connected, the following notation is needed.

If pin 7 on the DIP14 component U1 is referred to, it is called U1-7.

If pin 2 on the resistor is referred to, it is called R1-2.

In this tutorial, the following pin connections are made:

CONNECTION                                     TRACK WIDTH

U1-14 to C1-2                                  30 mils

C1-2 to R1-1                                   30 mils

U1-7 to C1-1                                   30 mils

U1-6 to R1-2                                   12 mils

Now you need to know the pin numbers on each of the components.  Choose

Jump Pad.  The message:

SELECT COMPONENT

appears on the prompt line.  Use the cursor keys or the mouse to move any-

where over the component U1.  Press <<ENTER>> or click <<LEFT MOUSE>>.  A

window opens asking you to specify a pin number.  Type in the value 1 and

press <<ENTER>> or click <<LEFT MOUSE>>.  The cursor jumps to the square pin

in the top left hand corner of the component.

The input window is still open, so input the value 7, then 8, then 14.

You should now be able to identify each of the pins on this component.

Press <<ESC>> or the <<RIGHT MOUSE>> button to exit from this option.

Now use the option again to identify the pin numbers on the other two

components.

LAYING TRACKS

As a general rule, it is a good idea to place vertical tracks on the top

layer and horizontal tracks on the bottom layer. For convenience in this

tutorial, the rule is broken occasionally.

*HINT*  Tracks should be laid so that they are not too close because

cross-talk or solder bridging may occur.  A good rule to adopt is to make

normal tracks 12 mils wide and lay them out using a 50 mil grid if they run

parallel to each other.  This provides a separation between tracks of 38 mils

which is quite adequate for normal loads.  If a track passes between 62 mil

pads then a 25 mil grid provides a 13 mil separation.

Since this board is being laid out on a 25 mil grid, you should check to

make sure that the snap grid is set to that value.  If the grid is set to a

value such as one, you have to press the cursor keys too many times to move

the cursor.  If it is set to a value greater that 25 mils you cannot move to

each of the coordinates that you need to.  Choose Grid Snap, enter the value

25 then press <<ENTER>> or click <<LEFT MOUSE>>.

First connect pin 14 on the DIP14 chip to pin 2 on the capacitor.  Since

this is a 30 mil track and the default width is 12 mils, choose Current Track

Width and select 30 mils then press <<ENTER>> or click <<LEFT MOUSE>>.

Since it is a vertical track, check the status line to make sure that

the currently selected layer is the top layer.  If it is not, press the <<*>>

key on the keypad until it is selected.  Alternatively, choose the layer by

using the menus and selecting Current Layer Top Layer.

Choose Place Track.  The message:

SELECT TRACT START POINT

indicates that you should move the cursor to the point where you want the

track to start.  Move to X:1300 Y:1000 and the cursor should be on pin 14 of

the DIP14 chip.  Now press <<ENTER>> or click <<LEFT MOUSE>> and the prompt

changes to:

PLACE TRACK

Now, as you move the cursor a this track is visible.  Move to X:1300 Y:1350,

then press <<ENTER>> or click <<LEFT MOUSE>>.  A 30 mil track is drawn.  Now

move the cursor to X:1200 Y:1350 so that it is sitting on pin 2 of C1 and

press <<ENTER>> or click <<LEFT MOUSE>>.  This completes the connection, so

press <<ESC>> or click the <<RIGHT MOUSE>> button to restore the:

SELECT TRACK START POINT

prompt.

The steps you have taken to route the first connection are:

Select the top layer

Choose Place Track

Move the cursor to X:1300 Y:1000

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:1300 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:1200 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Press <<ESC>> or click <<RIGHT MOUSE>>

The current track width is still 30 mils, so now make the next 30 mil

connection from U1-7 to C1-1.  Move to X:1000 Y:400 so that the cursor is

over pin 7 on U1.  Press <<ENTER>> or click <<LEFT MOUSE>>.  Move to X:800

Y:400 and press <<ENTER>> or click <<LEFT MOUSE>>.  Repeat this at X:800

Y:1350 and X:1000 Y:1350.

Press <<ESC>> or click <<RIGHT MOUSE>> button to exit to the first

prompt again.

The steps you have taken to route the second connection are:

Move the cursor to X:1000 Y:400

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:800 Y:400

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:800 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:1000 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Press <<ESC>> or click <<RIGHT MOUSE>>

The next 30 mil connection is C1-2 to R1-1.  This is essentially

horizontal so it is placed on the bottom layer.  Use the <<*>> key to change

to the bottom layer.  Now move to pin 1 on R1 (X:150 Y:1350) and lay tracks

to connect to pin 2 on C1.  Move to X:150 Y:1500, X:1200 Y:1500, X:1200 Y:1350.

Press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from the Place

Track option.

The steps you have taken to route the third connection are:

Select the bottom layer

Move the cursor to X:150 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:150 Y:1500

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:1200 Y:1500

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:1200 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Press <<ESC>> or click <<RIGHT MOUSE>>

The final connection is between pin 6 on U1 and pin 2 on R1.  This is a

12 mil track so choose Current Track Width again and select a width of 12

mils.  Press <<ENTER>> or click <<LEFT MOUSE>> to exit from the Current

option.

Notice that there is already a connection running from U1-7 to C1-1

which is on the top layer.  You therefore need to lay the 12 mil track on the

bottom layer until it is to the left of the 30 mil track.

Make sure that the bottom layer is the currently selected layer.  If it

is not, press the <<*>> key until it is selected.  Now choose Place Track.

You are prompted to:

SELECT TRACK START POINT

Move the cursor to X:1000 Y:500 and press <<ENTER>> or click <<LEFT MOUSE>>.

Now move the cursor to X:550 Y:500 and press <<ENTER>> or click <<LEFT

MOUSE>>.  Now you want to lay a vertical section of track, so press <<*>>

to change to the top layer.  Move the cursor to X:550 Y:1350 then press

<<ENTER>> or click <<LEFT MOUSE>>.  Notice that when the track is laid, a via

is automatically placed at the junction of the tracks where they change from

the bottom layer to the top layer.

Now, press <<ESC>> or click the <<RIGHT MOUSE>> button twice to exit from

this option.

The steps you have taken to route the final connection are:

Choose Current Track Width

Select 12 mils

Press <<ENTER>> or click <<LEFT MOUSE>>

Select the bottom layer

Choose Place Track

Move the cursor to X:1000 Y:500

Press <<ENTER>> or click <<LEFT MOUSE>>

Move to X:550 Y:500

Press <<ENTER>> or click <<LEFT MOUSE>>

Select the top layer

Move to X:550 Y:1350

Press <<ENTER>> or click <<LEFT MOUSE>>

Press <<ESC>> or click <<RIGHT MOUSE>> twice

Check your layout, then choose File Save to save the current state of

the board.

You can now check the dimensions of the board by choosing the

Information Board Dimensions option.  A window opens showing the coordinates

of the bottom left hand corner of the board, as well as its size.  The

measurements are given in inches.  If you have followed the instructions in

this tutorial exactly, then the dimensions are the same as those shown below.

BOARD DIMENSIONS

Minimum X:     0.12     Inches

Minimum Y:     0.35     Inches

X Size   :     1.21     Inches

Y Size   :     1.17     Inches

Press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from this

option.  Now choose Information Components and you see the designators of the

three components that you placed on the board listed in the window.  Press

<<ESC>> or click the <<RIGHT MOUSE>> button to exit from this option.

If you now choose Information Status, the complete listing of information

about the board is displayed.

STATUS

Free Memory           :               71069   Bytes

Free Ems Memory       :               0       Bytes

Arcs                  :               1

Components            :               3

Fills                 :               0

Holes                 :               19

Pads                  :               18

Strings               :               0

Tracks                :               25

Vias                  :               1

Work File Name        :               TRACKS.PCB

Library               :               EASYSTD.LIB

Current Path          :               C:\EASYTRAX

Disk Size             :               21309440 Bytes

Disk Free             :               100352   Bytes

Some of the information is easy to follow.  There are three components

and 18 pads.  This is obvious.  Notice, however, that the number of tracks is

greater than the number you placed.  The reason is that each component outline

is made up of a number of tracks, as you will see in the next tutorial.

Notice also that there is one arc.  This is the notch at the top of the DIP14

chip.

The number of holes is 19.  This is because there are 18 holes for pads

and one hole for the via.

Press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from this

option.

TUTORIAL:  CREATING NEW COMPONENTS

In this tutorial you will learn how to create your own component pattern

and how to store the pattern in the library for later use.  You may not need

to do this for a while because a comprehensive library of component patterns

is supplied with the Protel Easytrax package.  In addition, many components

can be created by simply altering existing library patterns using the Library

Explode option.

Nevertheless, you will also learn other skills which will help you

become more familiar with the procedure for laying out a board.

The skills you will learn will be:

how to use the snap grid;

relating cursor movement to the snap grid;

laying tracks of a given length;

changing the currently selected pad type;

placing pads;

using the Repeat option;

placing arcs;

defining a block;

creating a new library;

adding a component to the library.

GETTING STARTED

The component that you are going to create in this tutorial is a simple

DIP16 pattern (which is already in the standard library - but the exercise

is useful).

Before you make any changes, either start the Protel Easytrax program

or choose File Clear Yes.

The pattern you are going to create is laid out on a 25 mil grid.  The

track segments are 800 mils, 75 mils, or 50 mils.  The radius of the arc

which makes up the "notch" in the chip is 25 mils, and the pads are placed

100 mils apart (measured from center to center).

Choose Grid Snap Grid and make sure the size of the grid is 25 mils.

This means that the cursor will be constrained to move in increments of 25

mils.

You need two types of pad for this pattern.  Pin 1 is a square pad of

size 62 mils.  The rest are circular with a diameter of 62 mils.

The next step is to make sure that the circular pad called ROUND62 is

currently selected pad and that the track width is set to 12 mils.

Choose Current Pad Type and select ROUND62.  Also check that the track

width is 12 mils.

You are now ready to start creating the component.

PLACING SOME TRACKS.

The tracks for this component will be placed on the overlay, so either

choose Current Layer Overlay to change to that layer, or press either <<+>>

or <<->> on the keypad until the Overlay is the current layer.

You are now ready to lay the tracks which will make up the inner part of

the component.

Move to X:1000 Y:1000 and press <<PGUP>> four times.  Choose Place Track.

You see the message:

SELECT TRACK START POINT

on the prompt line.  Make sure the cursor is on X:1000 Y:1000 then press

<<ENTER>> or click <<LEFT MOUSE>>

Move the cursor across to X:1200 Y:1000 than press <<ENTER>> or click

<<LEFT MOUSE>>.  A short track is drawn.  Now move the cursor up to X:1200

Y:1800 and press <<ENTER>> or click <<LEFT MOUSE>>.  A track which forms the

right side of the outline appears.  Now move across to X:1125 Y:1800 and

repeat the process.

Now you have to leave a gap for the notch, so press <<ESC>> or click

<<RIGHT MOUSE>> once.  You see the prompt:

SELECT TRACK START POINT

again.  Move the cursor to X:1075 Y:1800.  Press <<ENTER>> or click <<LEFT

MOUSE>> then lay a track to X:1000 Y:1800.  Now complete the outline by

laying a track down to X:1000 Y:1000.

Press <<ESC>> or click <<RIGHT MOUSE>> twice to finish laying tracks.

PLACING PADS AND USING REPEAT

You are now ready to place the pads so choose Place Pad.  The message

"PLACE PAD" appears on the prompt line at the bottom of the screen.  Move the

cursor to X:1250 Y:1050 and press <<ENTER>> or click <<LEFT MOUSE>>.  A

ROUND62 pad appears on the board.  The shape of the pad is determined by the

shape chosen for the currently selected pad in Current Pad Type.  You are

then prompted to place another pad.  Even though there are more to place,

press <<ESC>> or click <<RIGHT MOUSE>> to exit the Place Pad routine.

Because you have several pads to place, you can save time by using the

Repeat command.

Choose Repeat from the MAIN MENU and the following window opens.

REPEAT PLACEMENT

Execute Repeat

Count Default      :     7

X-Offset (Mils)    :     0

Y-Offset (Mils)    :     100

Each time an item is placed on the board, the information about the

latest placement is stored so that the action can be repeated.  The window

shown above contains the information which the program needs to repeat the

action.

You need to place seven more circular pads.  Since the pads are to be

placed in a vertical line 100 mils apart, the X offset is set to 0 and the Y

offset is set to 100 mils.  If they are not, change them.

Now choose Execute Repeat and change the Count Default to 7.  You are

prompted to:

CONFIRM MULTIPLE PLACE REPEAT

Choose Yes.  Notice that seven pads have been laid.  Your previous Place Pad

instruction has been repeated seven times and each one is 100 mils apart.

Press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from the Repeat

option.

Now choose Place Pad.  Move to X:950 Y:1050 and press <<ENTER>> or click

<<LEFT MOUSE>>.  Another pad appears.  Press <<ESC>> or click the <<RIGHT

MOUSE>> button to exit from this option.

Choose Repeat Execute Repeat again.  Change the count to 6, choose Yes

to confirm the placement and another six pads are placed.  Press <<ESC>> or

click the <<RIGHT MOUSE>> button to exit from this option.

Now you are ready to place pin 1 which a rectangular type, so choose

Current Pad Type and change the type to SQUARE62.

Choose Place Pad.  In response to the:

PLACE PAD

prompt, move the cursor to X:950 Y:1750 than press <<ENTER>> or click <<LEFT

MOUSE>> to place the rectangular pad.

Press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from this

option.  Now you can see the outline of most of the pattern and the two rows

of pads.

PLACING AN ARC

Now you need to create the notch at the top of the chip.  Choose Place

Arc.  Move to X:1100 Y:1800, which is the center of the arc, then press

<<ENTER>> or click <<LEFT MOUSE>>.  A circular shape appears.  Change the

radius to 25 mils by using the <<RIGHT>> arrow key to increase the radius

and the <<LEFT>> arrow key to decrease the radius.  Press <<ENTER>> or click

<<LEFT MOUSE>> to confirm the size.  The message:

SELECT ARC SEGMENTS

appears on the prompt line.

You only want the bottom half of the circle to appear.  To do this press

the <<UP>> arrow key and then the <<LEFT>> arrow key and the top part

disappears.  Press <<ENTER>> or click <<LEFT MOUSE>>.  The notch now appears.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

You are almost ready to add the component to the library.  The only

remaining task is to assign the pin numbers or designators.  The rectangular

pin will be pin 1, the one under that will be pin 2, and so on.  The pin in

the top right hand corner will become pin 16.

Choose Edit Pad and then select the rectangular pad by moving to it and

then pressing <<ENTER>> or clicking <<LEFT MOUSE>>.  Now choose Designator

and assign the value 1.  Press <<ESC>> or click <<RIGHT MOUSE>>.  Now select

the next pad and assign its designator.  Repeat this until you have assigned

the last one, press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from

the Edit Pad option.

ADDING THE COMPONENT TO THE LIBRARY

The next step is to define the new component as a block by choosing the

Block Define option.  When you choose the option the message "SELECT FIRST

CORNER" appears on the prompt line at the bottom of the screen.  Move the

cursor to X:850 Y:1900 and press <<ENTER>> or click <<LEFT MOUSE>>.

The component should now be highlighted indicating that it is a defined

block.  The prompt "SELECT REFERENCE POINT" is now displayed on the prompt

line.  The reference point is that part of the component that will be placed

at the cursor position during a later Place Component operation.  Make the

rectangular pad (i.e. pin 1) the reference point.  Move the cursor to the

center of the rectangular pad and press <<ENTER>> or click <<LEFT MOUSE>>.

Since you probably do not want to add this component to your standard

library, create a new library to store the component in.  Choose Library New

Library and enter the name NEW for the library.  Press <<ENTER>> or click

<<LEFT MOUSE>>.  The message:

CREATING NEW LIBRARY.PLEASE WAIT.

is displayed.  You have now created a new library, but it has not been made

the currently selected library.  This means that your component is not added

to the new library unless you nominate it as the currently selected library.

Choose Library File and nominate NEW as the library file name.  Press

<<ENTER>> or click <<LEFT MOUSE>>.  You can now check that it is the

currently selected library by choosing Information Status.  Press <<ESC>> or

click the <<RIGHT MOUSE>> button to exit from this option.

Choose Library Add.  You are asked to enter a name for the new component.

Enter a name and then press <<ENTER>> or click <<LEFT MOUSE>>.  Choose YES.

The component is added to the library.

If you now choose Library List you will see the component listed.  Press

<<ESC>> or click the <<RIGHT MOUSE>> button to exit from this option.

Your component can now be placed on the board just like any other library

component by choosing the Place Component option and selecting the component

from the list provided.  See Chapter 23 for more information on placing

components.

TIDYING UP

Before you complete this tutorial, you should delete the highlighted

block from the board.

This highlighted block cannot be used as a component - it is simply the

block you have used to create a library component.  To delete it from the

board, choose Block Inside Delete.  You are asked if you want delete ALL

LAYERS or the CURRENT LAYER ONLY.  Since pads belong to every layer you must

choose the first option.  When you press <<ENTER>> or click <<LEFT MOUSE>>,

the block is deleted.

TUTORIAL:  USING THE LINE EDITOR

The Protel Easytrax package contains a simple, yet effective, line

editor which is available in all input windows.

You will have already used an input window when you loaded the DEMO file

in the first tutorial.

LOAD PCB FILE NAME

*.PCB

You can use the line editor to change text and to correct errors which

may arise when typing information into an input window.

This section contains a short tutorial to help you become familiar with

the line editor and a summary of the functions available.

The skills you will learn will be:

using an input window;

deleting text at the cursor;

inserting text at the cursor;

moving around the window;

deleting an entire word or line;

exiting from an input window.

THE INPUT WINDOW EXPLAINED

The first point to note is that when an input window opens it may already

have some information on the edit line.  This may either be a "default"

specification such as *.PCB or DIP40 or a question mark.

If a specification such as *.PCB or a question mark appears, then

clicking <<LEFT MOUSE>> or pressing <<ENTER>> will cause a selection window

to open.  The window will contain some filenames.  One of these can be

selected by moving the selection bar to that name and then clicking <<LEFT

MOUSE>> or pressing <<ENTER>>.  This procedure overcomes the need for you to

type information.

Whenever the window opens, the cursor appears on the left side of the

edit line.  If you want to type a filename or the name of a component rather

than selecting one from a list then simply type the name and any text that

was already in the window will disappear.

You may sometimes make a typing error when entering a filename or a

component designator.  Alternatively, you may simply want to make a minor

change to the default value already in the window.  This is where the line

editor can help.

Now choose File from the MAIN MENU and then choose Load.  The input

window opens.

Notice that there is a default specification of *.PCB on the left hand

side and that the flashing cursor appears under the asterisk.

EDITING A LINE

Type the words:  PROTEL TASMANIA AUSTRALIA

LOAD PCB FILE NAME

PROTEL TASMANIA AUSTRALIA

Notice that the cursor is positioned after the A at the end of the line.

Now type <<CTRL>> <<A>> twice and note that the cursor has moved back to the

start of the work TASMANIA.  Now type the word HOBART followed by a space.

The new text is inserted to the left of the cursor position.

LOAD PCB FILE NAME

PROTEL HOBART TASMANIA AUSTRALIA

Press the <<DEL>> key twice.  The characters under the cursor are

deleted.

LOAD PCB FILE NAME

PROTEL HOBART TASMANIA AUSTRALIA

Now type <<CTRL>> <<T>> and the rest of the word TASMANIA is deleted.

Press the <<HOME>> key and then the <<END>> key and the cursor moves to

the beginning of the line then to the end of the line.  Press <<HOME>> again,

then use <<CTRL>> <<F>> to move the cursor right one word.  Notice that the

cursor is positioned under the H in the word HOBART.

Since you do not want to use the text you have just typed in, press

<<CTRL>> <<Y>> to delete the entire line.

By now you should be able to alter text in any window without typing the

whole line again.  To help you with the line editor, a full summary of the

features is given in this section.

LINE EDITOR FUNCTIONS SUMMARY

<<HOME>>                   Moving to the beginning of the line.

<<END>>                    Move to the end of the line.

<<INS>>                    Toggle insert mode.  Pressing <<INS>> once

switches to overstrike mode.  Pressing it

again switches back to insert mode.

<<ESC>>                    Quit without changing the line.

<<ENTER>>                  Accept the line.

<<DEL>>                    Delete the character at the cursor.

<<BACKSPACE>>              Delete the character to the left of the

cursor.

<<LEFT>>                   Move the cursor to the left one character.

An alternative is to use <<CTRL>> <<S>>.

<<RIGHT>>                  Move the cursor right one character.  An

alternative is to use <<CTRL>> <<D>>.

<<CTRL>> <<A>>             Move the cursor left one word.

<<CTRL>> <<F>>             Move the cursor right one word.

<<CTRL>> <<S>>             Move the cursor left one character.

<<CTRL>> <<D>>             Move the cursor right one character.

<<CTRL>> <<G>>             Delete the character at the cursor position.

<<CTRL>> <<H>>             Delete the character to the left of the

cursor.

<<CTRL>> <<R>>             Restore the line to its state on entry.

<<CTRL>> <<T>>             Delete the word to the right of the cursor.

<<CTRL>> <<Y>>             Delete the entire line.

<<CTRL>> <<X>>             Delete all characters from the position of

cursor to the end of the line.

TUTORIAL:  ASSIGNING KEY MACROS

A powerful, and very useful, feature of Protel Easytrax is the ability

to define your own macros.  If you have not heard the term macro before then

don't worry.  It is simply a term that is used to describe a method of

assigning commonly used keystrokes to a single name or key.

The use of macros helps to speed up operations that you need to carry

out often.

In this tutorial you will learn the following skills:

clearing previously assigned macros;

defining a new macro;

using the <<SCROLL LOCK>> key;

assigning a macro name;

assigning a macro to a function key;

implementing a macro;

using the record facility to define a macro;

terminating the record facility;

saving a macro;

creating nested macros.

DEFINING A MACRO

An example will help.  When laying out a board you quite often have to

delete a track.  In the first part of this tutorial you create a macro which

deletes the track the cursor is currently pointing to.

Before starting this tutorial, you should make sure that there are no

macros already defined.  Choose Setup Keys Clear to clear any macros from

the memory.

It is also a good idea to work with a blank board.  Choose File Clear

Yes to clear the current board from the memory.  Now press the <<PGUP>> key

three times to expand the screen.

Lay four tracks starting at X:1000 Y:1000.  Choose Place track then move

the cursor to that point and press <<ENTER>> or click <<LEFT MOUSE>>.  Now

move to X:1000 Y:2000 and press <<ENTER>> or click <<LEFT MOUSE>>.  A track

is placed.  Now, move to X:2000 Y:2000 and press <<ENTER>> or click <<LEFT

MOUSE>>.  Another track is placed.  Repeat the procedure at X:2000 Y:1000 and

X:1000 Y:1000.  You should see four tracks placed in a square.

To delete the first track, carry out the following steps:

Move the cursor to X:1000 Y:1200

Choose DELETE by pressing <<D>>

Choose TRACK by pressing <<T>>

Press <<ENTER>>

Press <<ESC>> to exit from the option

Notice that the first track is deleted.  As this is a sequence of

commands you might need to repeat often, it is useful to define them as a

macro.

To assign this sequence of keystrokes to a macro, choose Setup Keys.

The KEYS menu is displayed.

KEYS

Highlight

Clear

Delete

Edit

List

Load

Merge

New

Record

Rename

Save

Choose NEW to define a new macro.  A window opens prompting you to input

a name for the macro.  The name can be up to ten characters long.  Choose a

name that reflects the function to be carried out by the macro.

At this stage type in:

DELTRACK

Now you are asked which key to assign the macro to.  The program will

supply the name of the next available macro key such as <<F1>>.  This stands

for the function key labelled F1.

If you do not want to assign the macro to <<F1>> press one of the other

function keys.  You can also use the function keys in conjunction with the

<<CTRL>>, <<SHIFT>>, and <<ALT>> keys or any other key with the <<CTRL>> or

<<ALT>> keys.

At this stage, simply press <<ENTER>> or click <<LEFT MOUSE>> to select

<<F1>>.  Provided the key is available, a window opens to allow you to type

in the required keystrokes.  Type:

D T

to represent the commands Delete and Track.  You now need to insert the

<<ENTER>> key into the macro definition.  Unfortunately, if you do press this

key it terminates the macro.  To overcome this you can change to "literal"

mode by pressing the <<SCROLL LOCK>> key.  Now if you press the <<ENTER>> key

the command is inserted into the macro.  You can also press the <<ESC>> key

at this stage.

Press the <<SCROLL LOCK>> key again to turn the light off.  Now when you

press <<ENTER>> or click <<LEFT MOUSE>>, the macro is terminated and you are

able to enter the name of the next macro you want to define.  Press <<ESC>>

twice or click the <<RIGHT MOUSE>> button twice to finish defining macros.

Now move the cursor to X:1200 Y:2000 and press the <<F1>> key.  Notice

that the track is deleted.  Move the cursor to the other two tracks and

delete them in the same way.

The commands you entered to define the macro just described are:

Choose Setup by pressing the <<S>> key

Choose Keys by pressing the <<K>> key

Choose New by pressing the <<N>> key

Enter the name DELTRACK then press <<ENTER>>

Press <<ENTER>> to select <<F1>> as the key

Type D T

Press the <<SCROLL LOCK>> key

Press <<ENTER>>

Press <<ESC>>

Press the <<SCROLL LOCK>> key

Press <<ENTER>>

Press <<ESC>> twice

USING THE RECORD FACILITY

Another macro which can be quite useful is one which places a pad and

then moves to the next pad position.  To make sure that you are at the correct

Zoom level for this exercise, choose Zoom Select and then choose the value

5.

Now check to make sure that the Snap Grid is set to 25 mils.  Choose

Grid Snap Grid.  If the value is 25 simply press <<ENTER>> or click <<LEFT

MOUSE>>.  If it is not, enter the value 25 then press <<ENTER>> or click

<<LEFT MOUSE>>.

To define the new macro, choose Setup Keys Record.  This option lets you

carry out a set of actions and automatically stores your keystrokes in the

macro.

Enter the name of the macro as PADS then press <<ENTER>> or click <<LEFT

MOUSE>>.  You are then asked to specify the key to which the macro will be

assigned.  Hold down the <<CTRL>> key and press the function key marked <<F2>>.

Now you will hear a beep and the program is ready to "record" all your

keystrokes.

Choose Place Pad.  You are prompted to place a pad.  Press <<ENTER>> or

click <<LEFT MOUSE>> to place the pad and now press <<RIGHT>> four times.

The cursor will move to the right by 100 mils.  Press <<ESC>> or click the

<<RIGHT MOUSE>> button to exit from the Place Pad option.

Now, hold down the <<CTRL>> key and press the <<END>> key.  This turns

off the "recorder" and signifies that you have finished the macro.  The

computer beeps twice to make sure that you are aware that this has happened.

Now press <<CTRL>> <<F2>>.  Notice that a pad has been placed and the

cursor has moved to a new position ready to place another pad.  Press <<CTRL>>

<<F2>> again and notice that another pad has been placed.

USING NESTED MACROS

A powerful feature of macros is that one can call another.  That is

sometimes referred to as "nesting" macros.

An example for you to try is creating a pin array consisting of four rows

of pads each of which has five pads.  You should make sure that the pad laying

macro which you assigned to <<CTRL>> <<F2>> is still there.

Choose Setup Keys New and input the name Line.  Press <<ENTER>> or click

<<LEFT MOUSE>>.  You are prompted to supply the name of the function key.

Overwrite the default value by typing <<CTRL>> <<F3>>.

In the macro window type <<CTRL>> <<F2>> five times.  This will produce

a line of five pads.  Then press <<DOWN>> four times and <<SHIFT>> <<LEFT>>

twice to return the cursor to a point below the first pad.  Terminate this

macro definition by pressing <<ENTER>> or clicking <<LEFT MOUSE>>.

Now define another macro and call it Array.  Assign this definition to

<<CTRL>> <<F4>>.  Now press <<CTRL>> <<F3>> four times.  Terminate the

definition by pressing <<ENTER>> or by clicking <<LEFT MOUSE>>.

Now press <<ESC>> twice to exit from this option.  Pick a blank area on

the board and press <<CTRL>> <<F4>>.  The macro will execute automatically and

lay down the array of pads.

SAVING MACROS

If you want to save your macros for later use, you can choose Setup Keys

Save.  A window opens prompting you to enter the name of the:

SAVE KEY MACRO FILE

Enter a name then press <<ENTER>> or click <<LEFT MOUSE>>.  The file is

given the extension .KEY and all macros currently in the memory are stored in

the file.

If you want the file to be loaded automatically whenever Protel Easytrax

begins, then name the file EASYEDIT.KEY.

TUTORIAL:  PLOTTING A BOARD

This tutorial shows you how to do a simple check plot of the sample

.PCB file.  Protel Easytrax has a flexible set of output routines which allow

output to be sent to draft printers, pen and ink plotters, photoplotters, and

disk files.

In this tutorial you will learn the following skills:

loading the Easyplot program;

interpreting the menu levels;

interpreting the status screen;

loading a .PCB file

setting the serial interface parameters;

selecting the plot type;

loading a plotter driver;

setting the pen speed;

setting the pen size;

producing a draft plot.

In this tutorial the plotting is done on a simple, single-pen DMPL

plotter.  If you do not have a similar plotter you will need to adjust the

tutorial to suit your situation.

STARTING THE PROGRAM

Using a hard disk drive, you should make sure that you are in a directory

from which the EASYPLOT program is accessible.

To start the EASYPLOT program simply type:

EASYPLOT then press <<ENTER>>

at the DOS prompt.

The MAIN PLOT MENU is displayed.

MAIN PLOT MENU

File

Information

Options

Setup

Plot

Print

Gerber Plot

The MAIN PLOT MENU is displayed on the left side of the screen and a

status window is displayed at the bottom of the screen.

LOADING THE DEMO FILE

Choose File Load and enter the file name DEMO in the input window.  An

information window opens showing the number of pads, tracks, components, etc.

that are being loaded.  A second pass shows the percentage of the board which

has been analyzed.

When the file has been loaded, the status window is updated to show the

name of the .PCB file, the dimensions of the board and the amount of free

memory remaining.  The path name of the directory is also displayed.

Path:                         C:\FILES

Layout File:                  \FILES\DEMO

Layout Size:                  5900x5000

Free Memory:                  114654

You should check the layout size to make sure that the board will fit on

the paper at the scale you choose.  Thus, if a board has a layout size of

6000 x 5000, it will cover an area 6 inches by 5 inches.  If you plot it at a

scale of 2, it requires a sheet of paper which is at least 12 inches by 10

inches.

The File menu is still displayed.  Press <<ESC>> or click <<RIGHT MOUSE>>

to return to the MAIN PLOT MENU.

OVERVIEW OF PLOTTING A BOARD

Now you can use a plotter to produce a check plot.  First, make sure that

you have set up the plotter in accordance with the manufacturer's

specifications.  For the purposes of this tutorial, we use a DMPL single-pen

plotter with a serial interface running at 2400 baud.

If you need more information about connecting the plotter to your

computer, see the detailed explanation given in Appendix B.

The steps you need to take to set up the correct conditions for the

successful check plot are:

Set the serial port to the correct baud rate (Setup Serial Ports)

Make sure the other setup parameters are correct

Select Check Plot as the plot type (Options Type of Plot)

Set up the check plot parameters (Setup Check Plot)

Load the plotter driver (Setup Plotter Type)

Set the pen speed (Setup Pens)

Set the pen sizes (Setup Pens)

Let's assume that you have a DMPL plotter connected to a serial port of

your computer.  (If you haven't, then choose from the list of plotters

provided in Setup Plotter Type - there must be a plotter driver for your

plotter before you can use it with Protel Easytrax).

SETTING THE SERIAL PORT PARAMETERS

Choose Setup Serial Ports and choose either "Serial Port 1" or "Serial

Port 2" depending on which port you have connected the plotter to.  A

window opens showing each of the serial port parameters you can change.

SERIAL PORT 1 SETUP

Baud                  :               2400

Data Bits             :                  8

Handshake             :           Hardwire

Parity                :               None

Stop Bits             :                  1

If the baud rate is not set to the correct value (in this case 2400)

choose Baud and select the value 2400 by highlighting it and then pressing

<<ENTER>> or clicking <<LEFT MOUSE>>.  You are returned to the Setup Serial

Ports menu.

Check that each of the other parameters are set correctly for your

plotter.  If you are using a DMPL plotter then the parameters shown in the

previous diagram should be correct.  Press <<ESC>> or click the <<RIGHT

MOUSE>> button three times to exit from this option.

CHOOSING THE PLOT TYPE

In this tutorial you are going to produce a check plot, rather than a

plot of a particular layer or a specialized plot such as a solder mask.

Return to the MAIN PLOT MENU and choose Options Type of Plot.  A window

opens showing each of the plot types you can choose.  Use the arrow keys to

highlight Check Plot then press <<ENTER>> or click <<LEFT MOUSE>>.

Return to the MAIN PLOT MENU again, and choose Setup Check Plot.  Make

sure that the Top Layer, Bottom Layer, Overlay, and Multi Layer Pads options

are set to "on".  This ensures that all the information on each of those

layers is plotted.  If you set any of these to "off", the items on that layer

are not plotted.

Press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from this

option.

LOADING THE PLOTTER DRIVER

Choose Setup Plotter.  The entry next to Type shows the currently

selected plotter driver.  If it is not set to "DMPL", choose the option and

the program scans the directory and lists all the currently available plotter

drivers.  Select the DMPL plotter driver from the list and then press

<<ENTER>> or click <<LEFT MOUSE>>.  You are returned to the Setup Plotter

menu.

Choose Device and another window opens listing the options you can

choose for the port to connect the printer to.  Choose "Serial Port 1" by

moving the selection bar to that option then pressing <<ENTER>> or clicking

<<LEFT MOUSE>>.  Check the other options to make sure they are the same as

those shown in the menu following.

Make sure that the Scale is set to 1 so that the board is plotted at

normal size.  If you choose "Draft" for Quality, the tracks are plotted in

outline only.  If you choose "Final" they are drawn as solid lines.

If you set the Software Arc option to "On", the software creates arcs by

drawing a number of short line segments.  The length of these line segments

is set with the Arc Quality option.  Enter the value 5 then press <<ENTER>>

or click <<LEFT MOUSE>>.

Press <<ESC>> or click <<RIGHT MOUSE>> twice to return to the MAIN PLOT

MENU.

A possible set of options is shown below:

PLOTTER SETUP

Type                          :               DMPL

Device                        :      Serial Port 1

Scale                         :              1.000

X Offset                      :              0.000

Y Offset                      :              0.000

X Correction                  :              1.000

Y Correction                  :              1.000

Orientation                   :             Normal

Quality                       :              Draft

Software Arc                  :                 On

Arc Quality                   :                  5

Options                       :

Pens                          :

Note that some plotters have their origin in the center of the page,

therefore you might need to input negative values for the offsets on some

occasions.

SETTING THE PEN PARAMETERS

You now need to set the speed at which the pens are to be driven and the

size of pen that you have in the plotter.  Choose Setup Pens Check Plot Pens.

You now need to set the pen speed to a value of 2 inches per second.  This

figure seems to work the best with a DMPL plotter.

*HINT  You can set the pen speed to a higher value, but the ink in the

pen won't run quickly enough to form a clear line on the paper.  If you set

the value too low, then you may get ink "blotches" on the paper - it also

takes a long time to plot.  Experiment with different speeds until you are

happy with the result.

To input the pen speed, highlight the Pen Speed option and press

<<ENTER>> or click <<LEFT MOUSE>>.  Enter the value 2 in the input window,

then press <<ENTER>> or click <<LEFT MOUSE>>.  You are returned to the Check

Plot Pens menu.

You now need to set the size of the pen you are using.  This is vital so

that the printer driver can draw the tracks, arcs, vias, and pads accurately.

It is obvious that if you have a 26 mil diameter pen then it is impossible to

draw a 12 mil track at normal scale!

If you do not set the pen size, it defaults to 26 mils.  To change the

size, choose Pen Sizes from the Check Plot Pens menu.  Enter the value of the

diameter of the pen in mils and then press <<ENTER>> or click <<LEFT MOUSE>>.

You are returned to the Check Plot Pens menu.  Press <<ESC>> or click the

<<RIGHT MOUSE>> button four times to return to the MAIN PLOT MENU.

PLOTTING THE BOARD

You are now ready to proceed with the plot.  From the MAIN PLOT MENU

choose Plot.  You are asked to confirm that you want to proceed with the plot.

Before you choose Yes, check the large information window on the right-hand

side of the screen which displays all the plot parameters.  If any of these

are not correct you need to return to the MAIN PLOT MENU and change them.

When you are happy that everything is correct, choose Yes.  You are then

asked to insert pen 1 in the plotter.  Make sure it is correctly in place then

press <<ENTER>> or click <<LEFT MOUSE>>.

The plot should now begin to appear on the paper in the plotter.  When

it is finished, you are returned to the MAIN PLOT MENU.

You have successfully completed you first plot!

TUTORIAL:  FURTHER PLOTTING

In this tutorial you learn how to manipulate the EASYPLOT program to

plot particular layers, to change the scale of the plot and to offset a plot

on the plotting sheet so that you can produce more than one plot per sheet.

The skills you learn in this tutorial are:

selecting a particular plot layer;

changing the plot scale;

calculating offsets;

changing the offset value;

plotting more than one layer per sheet;

plotting on a laser printer.

Some of these skills are very useful because they enable you to save

costs on plotter paper.

It is assumed in this tutorial that you have acquired the skills outlined

in the previous tutorial and that you can load a file and choose the printer

and plotter driver.

DOUBLE SIZE PLOTS

Load the Demo file again.  For the purposes of this tutorial you are

going to plot the top layer of this board at twice the normal size.  It is

also assumed that you are going to use an A0 size HPGL plotter which takes

sheets of paper which are 44 inches in the X direction and 32 inches in the

Y direction.

To select this plotter, choose Setup Type and select HPGL as the plotter

driver.  Press <<ESC>> or click the <<RIGHT MOUSE>> button to exit from this

option.

To select the top layer choose Options Type of Plot.  A window opens

listing all the layers that you can choose to plot.

SELECT PLOT TYPE

Check Plot                    Top Layer

1 Mid Layer                   2 Mid Layer

3 Mid Layer                   4 Mid Layer

Bottom Layer                  Overlay

Ground Plane                  Power Plane

Top Solder Mask               Bottom Solder Mask

Pad Master                    Drill Guide

Move the highlight to "Top Layer" then press <<ENTER>> or click <<LEFT

MOUSE>>.  You are returned to the Options menu which now indicates that the

Top Layer is the layer currently selected for plotting.  Press <<ESC>> or

click the <<RIGHT MOUSE>> button to exit from this option.

Now choose Setup Scale.  An input window opens allowing you to type in

the scale which is to be 2.  Press <<ENTER>> or click <<LEFT MOUSE>> to exit.

The Demo board at normal scale is approximately 6 inches by 5 inches,

therefore at a scale of 2, the area taken up by the plot is 12 inches by 10

inches.

Now return to the MAIN PLOT MENU and plot the board as described in the

previous tutorial.

CALCULATING OFFSETS

It is quite likely that there is enough room left on the sheet of paper

in the plotter for another plot.  If this is the case then you can use the X

and Y offsets to change the position of the plot on the paper.

Normally a plot starts at the home position determined by the plotter.

Let's say that you are to plot the Demo board at a scale of 1.  The board

takes up an area which is 6 inches in the X direction by 5 inches in the Y

direction.  Normally, the origin of the board would be positioned at the

plotter origin (which for the HPGL plotter is in the center of the sheet).

This means that the X values range from -22 inches to +22 inches and the Y

values range from -16 inches to +16 inches.

Choose Setup X Offset and enter the value -14.  Press <<ENTER>> or click

<<LEFT MOUSE>>.  Now choose Setup Y Offset and enter the value -11.  The

output will now be positioned in one corner of the sheet.

Now, if you want to plot another layer on the same sheet, you can change

the X Offset to -8 and the second plot will be positioned next to the first.

PLOTTING ON A LASER PRINTER

The Easyplot program supports output to the HP Laserjet II series of

laser printers.  Output can be printed at 100, 150, or 300 dots per inch.

To use a laser printed, set up the print as described in the previous

tutorial, then choose Setup Printer Type and select one of the printer drivers

for the HP Laserjet.  For the purposes of this tutorial, choose the printer

driver which gives a resolution of 100 dots per inch.

Now choose Options Type of Plot and select "Check Plot".  Next, choose

Setup Printer Device and opt to send the output to a File.  Input the file-

name Demo then press <<ENTER>> or click <<LEFT MOUSE>>.

Note that for the purposes of this tutorial, we have opted to send the

output to a file.  It is also possible, of course, to send the output directly

to the printer by choosing the port that the laser printer is connected to.

The reason some people send the output to a file is that if they have a

number of layers they want to print, they create files and then use a DOS

batch file to print them all overnight.

Now return to the MAIN PLOT MENU and choose Print.  You need to move

the selection bar to this option and press <<ENTER>> or click <<LEFT MOUSE>>.

Simply pressing P selects the Plot option, not the Print option.

The print now proceeds and the various print passes are shown in the top

left-hand corner of the screen.  A message informs you of when the program is

printing to the disk file.  When the print is complete you hear a beep and are

asked to press any key to continue.  When you press a key, you are returned to

the MAIN PLOT MENU.

The file that has been created is given the filename DEMO.RCK.  The

extension shows that it is a Print file of a ChecK Plot.

You can now print the file by returning to DOS and typing a command such

as:

COPY DEMO.RCK COM1:

Be aware, however, that the file may take some minutes to print as the

print file generated is very large.

TUTORIAL:  GERBER PLOTS

In this tutorial you learn how to use the EASYPLOT program to create a

file which can be used with a Gerber-compatible photoplotter to produce high-

quality final artwork.

The skills you learn in this tutorial are:

selecting a particular plot layer;

selecting Setup parameters;

selecting apertures;

selecting the photoplotting method;

generating a match file;

generating a Gerber plot file.

Photoplotters produce highly accurate artwork - in fact they are as

accurate as the resolution of the PROTEL-EASYTRAX program.  The biggest

problem is that the equipment can be very expensive so most board designers

need to send their files to a photoplotting bureau.  For this reason, the

Easyplot program produces output files rather than driving a photoplotter

directly.

It is a good idea to talk to the bureau you are going to use before you

produce the plot files.

SETTING UP A GERBER PLOT

A detailed description of the way a Gerber-compatible photoplotter works

can be found towards the back of this manual in the chapter titled "Using

the Easyplot Program".  You may need to read some of the details before

commencing this tutorial.

You need to set up all the same options for a Gerber plot as you do for a

normal plot but there are some extra things to do as well.

Firstly, you should choose File Load and load the Demo PCB file.  This

process is exactly the same as for a normal plot or print.

In this tutorial you are going to produce a plot of the top layer and the

pad master.  For the first plot choose Options Type of Plot and select "Top

Layer".

NOTE that the "Check Plot" option does not work with a Gerber plot

because it is a time consuming and expensive way to check whether your plot

is correct.  You should use a printer or pen plotter to do your check plot and

only produce the Gerber plot when you are sure that it is absolutely correct.

Now choose Setup Gerber and define each of the setup options you require.

The options from which you can choose are shown in the menu displayed below:

GERBER SETUP

Output File                   :               \Files\Demo

X Offset                      :                     0.000

Y Offset                      :                     0.000

Aperture Table                :                   RCS.APT

Match Tolerance               :                        0%

G54                           :                       Off

Arc Quality                   :                         5

Options

The X and Y Offsets can be used in exactly the same way as they are for

a normal plot.  By changing these offsets you can choose to start plotting at

a different spot on the film.  This allows more than one plot to be performed

on one film, thus saving on the cost of the film.

Next you should choose the Aperture Table which matches that required by

the photoplotter that you are going to use.  While most photoplotters use the

Gerber standard for selecting apertures and for moving to a particular spot

on the film, they vary considerably in the apertures which are available.  To

overcome this problem there are a number of standard Aperture Tables supplied

with PROTEL-EASYTRAX.  Select the aperture table you require by choosing

Setup Gerber Aperture Table.  A list of the available aperture tables is

displayed.  Highlight the one you require then press <<ENTER>> or click

<<LEFT MOUSE>>.  The name is now displayed in the Gerber Setup menu.

Next you should set the Match Tolerance you require.  An example might

help here.  If you have a large number of circular 62 mil pads on your board,

these can be plotted quickly by selecting a 62 mil aperture and then "flashing"

a light onto the film. (NOTE: MOST PLOTTERS NO LONGER USE "WHEELS" MOST ARE LASER

DRIVEN NOW - SETTING ANY TOLERENCE OTHER THAN ZERO (0) USUALLY Produces SHORTED

PADS AND TRACES ON THE FILM or it WILL WIDDEN THE PAD/TRACE BEYOND the 8thou

MINIMUM CLEARENCES REQUIRED BY MOST PCB MANUFACTURES)

But what happens if the photoplotter does not have a 62 mil aperture?

Some have a 60 mil round aperture and for most plotting tasks you are likely

to require, this is close enough.  The Match Tolerance is the figure you set

to say what variation in area you can tolerate.  If the program is able to

match an aperture with a pad which is within the tolerance you set, then it

does so automatically.

The area of a 62 mil pad is 3.1415 x 31 x 31 = 3019 square mils.  The

area "flashed" by a 60 mil aperture is 3.1415 x 30 x 30 = 2827 square mils.

The percentage variation is calculated as (3019 - 2827) x 100/3019 = 6.35.

Therefore, if you set the Match Tolerance to 8%, say, the 60 mil aperture is

selected automatically.

Don't set the tolerance too high (e.g. 30%) otherwise there may be

problems fitting tracks between pads in some cases.

Some older Gerber plotters need to be sent the code G54 before an

aperture change.  Thus, if the aperture D10 is to be selected the software must

send a G54 code than a D10 code.  If the plotter requires this code to be

sent, set the G54 option "On".  If it does not need this code, set it to "Off".

Some plotters have the ability to draw an arc if the parameters are

supplied.  This is not the case with some Gerber photoplotters.  In order to

draw an arc, the plot program must ask the photoplotter to draw a number of

short line segments.  The greater the number of line segments used to draw

the arc, the longer it takes.  The Arc Quality option allows you to set the

length of the line segment which is used to draw the arc.  A value of 5 mils

is a reasonable compromise for most jobs.

The steps you have carried out to set up the Gerber plot are:

Choose File Load and load the Demo file

Choose Options Type of Plot and then select Top Layer

Press <<ESC>> or click <<RIGHT MOUSE>>

Choose Setup Gerber

Choose Aperture Table and select the required table

Set the Match Tolerance to 0% (any other number will generate "SUPRISES")

Set G54 to "Off"

Set Arc Quality to 5

Press <<ESC>> twice to return to the MAIN PLOT MENU

CREATING THE GERBER FILE

Now return to the MAIN PLOT MENU and select Gerber Plot.  The plotting

screen appears with the information window on the right-hand side of the

screen.

You are asked to confirm that you want to proceed with the Gerber plot.

This gives you the opportunity to inspect the parameters you have chosen for

the plot.  If any of them are incorrect, select No and change them.  If you

are ready to proceed select Yes.

The program now attempts to match every item on the board with an

aperture in the Aperture Table you selected.  If it cannot produce a match

within the tolerance you specified, it asks you to select from a table of the

closest apertures or enables you to step through the apertures to manually

select one.

You see a display similar to the one below:

MATCHING DRAFT CODES

ASSIGNING:                    Circular (28 x 28)

------------------------------------------------

Match Method                  Code      Shape Size

Flash Match                   :D19   Circular (40 x 40)

Stroke Match                  :D15   Circular (25 x 25)

Fill Match                    :D13   Circular (20 x 20)

A window then opens allowing you to select which method you want the

photoplotter to use to make the shape.

SELECT PLOT METHOD

Flash Method

Stroke Method

PAINT Method <-NOT RECOMMENDED!! For PADS that don't Match

Manual Assignment

Choose the "flash" method for pads, the "stroke" method to draw tracks

and the PAINT method to create pads of a different size to the available

apertures(NOT RECOMMENDED).  In general, if you choose the flash method,

select a size which is slightly smaller than the pad size - if you choose a

larger aperture, there may be too little space between the pad and nearby tracks.

Make sure that you do not choose the stroke method to create a pad.  If

you attempt to, the message:

CANNOT USE LINE APERTURE FOR FLASH

is displayed.

It is a good idea to choose as few Fill options as possible because these

usually take longer to complete.

When all apertures have been matched, the message:

MATCH FILE GENERATED

appears indicating that a file has been created which matches each shape to

be generated with an aperture which is available on the chosen photoplotter.

The file is given the extension .MAT.

NOTE: IF YOU WERE REQUIRED TO SELECT APERTURES THAT DID NOT MATCH THE SOURCE

SIZE,  PRESS <<ESC>> TO ABORT THE PLOT.  EXIT TO DOS, LOCATE THE ?.MAT

FILE..it will look like the sample below.

The match file shows the shapes which the program has tried to assign

apertures to and the aperture and exposure method which has been chosen - either

by the program or by you.

ASSIGNING     :          Circular          (28 x 28) <-Origional Source

STROKE        : D15      Circular          (25 x 25) <-Closest Match

ASSIGNING     :          Circular          (40 x 40) <-Origional Source

FLASH         : D19      Circular          (40 x 40) <-EXACT MATCH

ASSIGNING     :          Circular          (60 x 60)

FLASH         : D28      Circular          (62 x 62)

Locate ALL NON-EXACT MATCHES and make a Record of them.  Edit your APERTURE FILE

and ADD IN the UN MATCHED SIZES. Then Re-Run the EASYPLOT program.  This will

ensure that your Plots Match your Drafting.

NOTE: FILLS DO NOT REQUIRE EXACT MATCHES - They are made up by using combinations

of Aperture Sizes.  When ADDING ADDITIONAL SIZES to the APERTURE TABLE move

to the BOTTOM of the LIST and USE THE NEXT LOGICAL "D" number

ie. D42  Circular  40   40  0  on the next line start with D43 etc.

After returning to the Plot Program re start the Gerber Output Process again

and watch for any non-exact matches..when you are satisfied with the MATCHES

Press any key and the final Gerber output file is then generated.  The

Gerber file in this case is assigned the name DEMO.GTL. The extension

indicates that it is a Gerber output file and that it is a plot of the Top

Layer.

PLOTTING A PAD MASTER

When the first plot is complete you are returned to the MAIN PLOT MENU.

Now, as an example of a different plot, choose Options Type of Plot and

select Pad Master.  Press <<ESC>> to return to the MAIN PLOT MENU.

When you now choose to do the Gerber plot, you are asked whether you

want to reassign the draft codes.  If you are happy with the matches which

are stored in the .MAT file then choose No and the plot proceeds.

This time the Gerber output file is given the extension .GPM to indicate

that it is the Pad Master layer.

MANIPULATING BLOCKS

When you are laying out a printed circuit board, it quite often happens

that you need to manipulate a block of components.  For example, you may want

to copy a block of components, move some components, transfer a block from

one printed circuit board to another or save a block separately from the main

PCB file.

The options in the BLOCK menu allow you to manipulate blocks quickly and

efficiently.  They are a powerful feature of the PROTEL-EASYTRAX package and

can greatly speed up board layout.  You can define a block, copy a block, move

a block, hide a block, load or save a block, and delete a block.

BLOCK

Define

Hide

Copy

Inside Delete

Outside Delete

Read

Write

DEFINING A BLOCK

Before you can carry out block operations such as moving, copying, and

deleting, you must first define the block.

Choose the Define option from the Block Menu.  The following prompt

appears on the status line:

SELECT FIRST CORNER

Move the cursor to one corner of the block you want to highlight and

click the <<LEFT MOUSE>> button or press <<ENTER>>.  The prompt changes to:

SELECT SECOND CORNER

Move to the diagonally opposite corner and click the <<LEFT MOUSE>>

button or press <<ENTER>>.

The block is highlighted and the prompt:

SELECT REFERENCE POINT

appears.

The reference point is any point you decide to choose to assist you in

moving and placing the block.  When you move or copy a block, it is the

reference point which moves with the cursor.  Some people make one of the

corners the reference point, but it is also possible to make the corner of a

particular component in the block a reference point.  If you are marking a

block in order to create a library component, it is a good idea to select the

pad which will become pin 1 as the reference point.  The reference point can

also lie outside the block.

You can only define one block at a time, but if you want to define a new

block and there is one already defined, don't worry!  Simply go ahead and

define the new block and the previous block is released.

"HIDING" A BLOCK

Use Block Hide when you want to take the highlight off a particular

block but do not want to define a new block.

The HIDE option simply removes the highlight from a block.  You will

notice that the highlight disappears from the selected block once you choose

HIDE, indicating that the block is no longer defined.

MOVING A BLOCK

Once you have defined a block with BLOCK DEFINE, you can move it to

another place on the board.

Choose BLOCK MOVE and the prompt:

PLACE BLOCK

appears.  You can now specify the new position for the block.  Use the mouse

or the cursor keys to position the outline of the block in the required place.

Remember that the cursor position dictates where the reference point will be

placed.

Sometimes you may want to place the block at a different orientation to

the original.  There is a summary of the keys which can be used to rotate,

flip, and move the block during a MOVE operation.

When you place the block, a menu appears asking if you want the current

layer only or all layers.

If you simply want to drag the tracks on the current layer then choose

Current Layer Only.  With this option, no pads or vias are dragged since they

belong to every layer.  If you wish to affect all tracks, then choose All

Layers.  Then any pads or vias within the block, and tracks connected to them

on other layers are moved.

Click the <<LEFT MOUSE>> button or press <<ENTER>> to position the block.

The block is then moved.  A delay may occur while the tracks are sorted

and dragged to the new position.  If the tracks have been distorted, you may

have to re-route them later.

An important point to note is that is a component only lies partly within

the defined block it will not be moved.  A component must be completely

enclosed by the highlighting.  If it is not, you will need to redefine the

block.  However, if the associated text such as the component designator and

comment do not fall within the block outline, you do not need to redefine the

block.  The text is moved with the component regardless of whether it falls

within the block or not.

If a pad center or arc center falls within the block it is moved.  A

filled area must be completely enclosed.  A track which is completely within

the block is moved with the block.  If only one end of the track lies within

that block, then the track is stretched to maintain the connection.

Before a MOVE operation is performed, the program checks the limits of

the existing board.  If the block is to be moved to a place within that area,

the following window appears:

CONFIRM NEW BLOCK WILL OVERWRITE PCB

No

Yes

Choose Yes to overwrite the board or else choose No and move the block

outline to a different place on the board.

The following keys and buttons help you place the block in any

orientation.

<<SPACE>>          Rotates the block 90 degrees.  Pressing it twice

rotates the block 180 degrees and so on.

<<X>>              Pressing <<X>> flips the block in the X direction.

<<Y>>              Pressing <<Y>> flips the block in the Y direction.

<<LEFT>>           Pressing the left cursor key moves the block left.

<<RIGHT>>          Pressing the right cursor key moves the block right.

<<UP>>             Pressing the up cursor key moves the block upwards.

<<DOWN>>           Pressing the down cursor key moves the block down.

Mouse              Moving the mouse in any direction also moves the

block in that direction.

<<ENTER>>          Pressing the <<ENTER>> key fixes the block in its

current position.  Pressing <<LEFT MOUSE>> has the

same effect.

Note that these commands also work when you are placing or moving

components and strings.

COPYING A BLOCK

The COPY option is similar to the MOVE option with one important

difference.  When a block is copied, a new copy of each of the components in

the block is made and the original components remain.  The component

designators are incremented to the next free value to ensure that the new

designator is unique.  The new block is highlighted and becomes the defined

block.

Thus, if the component with the designator U2 is copies, the new

component is given a new designator depending on the number of "U" designators

already on the board.  If there were already the designators U1 to U6, the

copied component would be labelled U7.

Remember that when copying a block it is possible that you will overwrite

an existing part of the board.

DELETING A BLOCK

Before you can delete a block you must first define it.  Once it is

defined, you can delete everything in the defined area or everything outside

the defined area.  This includes all tracks, arcs, pads, vias, strings, fills,

and components.

Choose BLOCK INSIDE DELETE to delete everything inside the defined block.

Choose BLOCK OUTSIDE DELETE to delete everything outside the defined block.

In each case you are asked to confirm that you want to delete the area.

Choose Yes to confirm, No if you want to change your mind.

You are then asked if you want to delete the Current Layer Only or All

Layers.  Choose the option you require then press <<ENTER>> or click <<LEFT

MOUSE>>.

Care must be taken with block deletion commands.  It is recommended that

you save a copy of the file before attempting to delete a block.

One point to bear in mind when deleting outside a block is that a track

which passes completely through the defined block is also deleted.

If you try to delete without having defined a block, the message:

BLOCK NOT MARKED

is displayed.  You should then choose BLOCK DEFINE to define the block you

want to delete.

READING A BLOCK

Since there are common blocks of components that you may want to use on

many boards, it is possible to store a number of component configurations in

separate .PCB files and then read them into the current board when you require

them.  You can identify a file stored on the disk, then merge it into the

board you are working on in the computer's memory.

To read a block, choose BLOCK READ and specify the name of the .PCB

file.  The prompt "PLACE BLOCK" appears.  You are asked to confirm the fact

that you want to load a .PCB file.  Choose Yes to load the file.

Move the block outline until it is in the position you require.  Remember

to use <<SPACE>>, <<X>> and <<Y>> to rotate or flip the block, then click

<<LEFT MOUSE>> or press <<ENTER>>.

The block which is loaded remains as a defined block until you choose

BLOCK HIDE or else define another block.

SAVING PART OF A LAYOUT

If you identify part of a current layout as a block you might want to

use on another occasion, it is a good idea to save it on a disk as a separate

file.

To do this, define the section of the layout as a block using the BLOCK

DEFINE option.  Then, choose BLOCK WRITE.  You are prompted to supply a

"BLOCK WRITE FILE NAME".

Enter a filename.  If the filename you enter does not exist, the block

is written out to that file.  If it already exists, you are asked to confirm

that you want to overwrite the file.  If you do want to overwrite the file

choose Yes, if not, choose No then repeat the process with a different

filename.

You can then load this block into another .PCB file at a later time

using the BLOCK READ option.

VIEWING AND CHANGING THE CURRENT SETTINGS

If you select Current from the MAIN MENU you can view the current

settings.  If you need to change any of the settings then select them from

the Current menu and enter the new value.

CURRENT

Cursor Mode           :               Absolute

Floating Origin       :               0,0

Layer                 :               Bottom Layer

Pad Type              :               Square62

Pad Orientation       :               Normal

Track Width           :               12

String Size           :               60

Via Size              :               50

SETTING THE CURSOR MODE

The cursor mode can either be set to absolute or relative.  If the

cursor mode is absolute, the coordinates measure the X and Y distances from

the bottom left hand corner of the board.  If the cursor mode is relative,

then the cursor position is measured relative to some other position on the

board.  The origin of the relative cursor is set using Current Floating

Origin.

It is sometimes useful to set a relative cursor position if a number of

components need to be placed relative to the position of an important

component such as, say, a microprocessor.

SETTING A FLOATING ORIGIN

If you want to set the origin (i.e. X:0 Y:0) in a position other than the

bottom left hand corner of the board, move the cursor to the point which you

want to become the "floating origin".  Choose Current Floating Origin.  The

origin changes to the current cursor position.

When a floating cursor is set and you move the cursor to the left of the

floating origin you will notice the X value (as shown on the status line)

becomes negative.  Similarly, if you move the cursor below the floating

origin, then the Y value becomes negative.

You will not notice any change on the status line unless you set

Current Mode to Relative.

THE CURRENTLY SELECTED LAYER

SELECT LAYER

Top Layer

1 Mid Layer

2 Mid Layer

3 Mid Layer

4 Mid Layer

Bottom Layer

Overlay

Ground Plane

Power Plane

Board Layer

Current Layer indicates the layer which is currently selected.  Choosing

the option opens a window which displays all the layers and allows one to

become the selected layer.

The selected layer must be turned on before it can be activated.  Layers

can be turned on and off using Setup Toggle Layers.

If you select a layer which is not turned on to be the current layer,

the message:

CURRENT LAYER MUST BE TURNED ON

is displayed.

THE CURRENT PAD TYPE

To change the currently selected pad type, choose Current Pad Type.  An

information window opens which shows all the available pad types.  Move the

selection bar to the required type and click <<LEFT MOUSE>> or press the

<<ENTER>> key.

The default pad type is a circular pad with a diameter of 62 mils.  The

default diameter of 62 mils means that if two pads are placed 100 mils apart

then a 12 mil track can be laid between then with a clearance of 13 mils.

PAD ORIENTATION

Pads can be placed normally or they can be rotated through 90 degrees.

Choose NORMAL or ROTATED from the PAD ORIENTATION menu to select between

these options.

Move the selection bar to the option you want and then press <<ENTER>>

or click <<LEFT MOUSE>>.

This option is very useful since it increases the number of effective

pad types which are available.

THE DEFAULT TRACK WIDTH

This option allows you to change the default track width.  If you do not

change it, the default will be set to 12 mils.  If you want to change the

width, select the option from the menu.  A window opens allowing you to choose

the new value for the track width from the list provided.  The values range

from 10 mils to 100 mils.

Move the selection bar to the option you want and then press <<ENTER>>

or click <<LEFT MOUSE>>.

CHECKING THE TEXT HEIGHT

The Current String Size option allows you to choose the height of the

text which appears on the board.  The height can be changed at any time, but

this option should be set to the size which you use most often.

When you select Current String Size, a window appears with a list of

values from which you can make a selection.  The values vary from 48 mils to

560 mils.

THE DEFAULT VIA SIZE

The default via size is 50 mils.  This can be changed by selecting

Current Via Size and then choosing a new value from the list provided.  Move

the selection bar to the option you want and then press <<ENTER>> or click

<<LEFT MOUSE>>.

Vias can vary in size from 40 mils to 150 mils.  Be careful about making

vias too large as this will restrict your choices for places to lay tracks.

EXITING FROM THE CURRENT MENU

When you have finished making selections from the CURRENT menu, press

<<ESC>> or click the <<RIGHT MOUSE>> button to exit.

DELETING ITEMS ON THE BOARD

The DELETE option allows you to delete an arc, component, fill, highlight,

pad, string, track, or via.

DELETE

Arc

Component

Highlight

Pad

String

Track

Via

After you delete an item from the board it is often a good idea to

refresh the screen by pressing the <<END>> key.  This redraws the screen and

eradicates any untidy effects that a deletion may have caused.

When deleting tracks and arcs you need to be able to position the cursor

exactly over the item to be deleted.  Remember that the cursor is constrained

by the snap grid to move in increments of 25 mils.  It may be necessary to

reduce the size of the snap grid to, say, 15 mils to ensure that the cursor

can be positioned correctly.

There is a setup option which affects the way in which you delete items

from the board.  If you choose Setup Options Question Delete you can set the

option to "ON" or "OFF".  If it is set to "ON", you are asked to confirm

each deletion you make.  If it is set to "OFF", you are not asked to confirm

the deletion.

Some people set this option to "OFF", then if they realize they need to

restore a deleted item, they use the Un-Delete option.

DELETING AN ARC

An arc can only be deleted if the currently selected layer (as indicated

by the status line and the CURRENT menu) is the layer containing the arc.

As arcs are normally found on the overlay, this layer should be selected

before you attempt to delete one.

Choose DELETE ARC and you are prompted to select an arc.  Move the

cursor to anywhere on the arc to be deleted and click the <<LEFT MOUSE>> or

press <<ENTER>>.  You are asked to confirm that you want to delete the

highlighted arc, unless you have set the Question Delete option to "OFF".

If you choose YES, the arc is deleted.

If you hear a beep or if the arc is not deleted, you may need to do one

of two things.  First, check that you are on the correct layer.  Second, make

sure that the cursor is correctly positioned over the arc.  If you find it

difficult to position the cursor, you might have to change the size of the

snap grid or you might need to expand the view of the arc by pressing the

<<PGUP>> key.

Note that the cursor must be positioned on the arc itself for it to be

deleted.

DELETING A COMPONENT

You can delete a component regardless of the currently selected layer.

There is no need to change to the component layer prior to deleting a

component.

To delete a component, choose DELETE COMPONENT.  You are prompted to

select a component.  Use the mouse or the arrow keys to move to the component

you want to delete.

Now click <<LEFT MOUSE>> or press <<ENTER>> to select it.  A window opens

asking you to confirm that you really do want to delete the component.  Choose

YES and the component will be removed from the board.

If you click <<LEFT MOUSE>> or press <<ENTER>> when the cursor is not

over a component, an input window opens asking you to input the component

designator.  Enter the component designator of the component you want to

delete.  Be careful, because you can delete a component that is not currently

displayed on the screen!

DELETING A FILL

Any fill area you want to delete must be on the currently selected layer.

To delete a fill, choose the DELETE FILL option.  Move the cursor to the

required fill then press <<ENTER>> or click <<LEFT MOUSE>>.  Confirm that it

is the one you want to delete.

DELETING A HIGHLIGHT

If you want to delete a set of highlighted tracks, you must first choose

the HIGHLIGHT option from the MAIN MENU.  Highlight the required tracks as

described under the "Highlighting Nets" section of this manual.  It is then

a good idea to choose ZOOM ALL to fit the board on the screen so that you can

make sure that is has not highlighted any tracks that you do not intend to

be deleted.

Choose DELETE HIGHLIGHT and the highlighted section is deleted.  Remember

to press <<END>> to refresh the screen.

Note that deleting a highlight does not delete any components.

DELETING A PAD

A pad can be deleted from any layer since it belongs to every layer.

Choose the DELETE PAD option and the prompt "SELECT PAD" appears on the

status line.  Move the cursor to the pad to be deleted and then press

<<ENTER>> or click <<LEFT MOUSE>>.  If the pad cannot be deleted, you will

hear a beep.  If it can be deleted, a window appears asking you to confirm

that you want to delete the pad - and the pad appears in inverse.  Choose

YES and the pad is deleted.

You are then asked to select another pad.  When you have finished

deleting pads, press the <<ESC>> key or click <<RIGHT MOUSE>>.

Do not try to delete a pad which is part of a library component.  You

need to delete the entire component rather than just one pad.

DELETING A STRING

To delete a string, you must be on the same layer as the string to be

deleted.  If you are not, change layers by using the keys outlined in the

first tutorial.

Choose the DELETE STRING option and the prompt "SELECT STRING" is

displayed.  Move to the string you want to delete then press <<ENTER>> or

click <<LEFT MOUSE>>.

The string to be deleted appears in inverse so that you can confirm that

it is the one you want to delete.  Choose YES to confirm the deletion.

DELETING A TRACK

Note that you must be on the same layer as the track you want to delete.

To delete a track simply choose DELETE TRACK, then move the cursor to

the track required and either click the <<LEFT MOUSE>> or press <<ENTER>>.

The track you have chosen appears in inverse and you are asked to confirm

that you want to delete it.  Choose YES to delete it or NO to leave the track

there.  Press <<ESC>> or click <<RIGHT MOUSE>> when you have finished deleting

tracks.

If you cannot get the cursor to sit on the track you want to delete,

change the snap grid size in the GRID option of the MAIN MENU to a smaller

figure (e.g. from 25 down to 15 mils).

DELETING A VIA

A via can be deleted from any layer.

Choose DELETE VIA then move the cursor to the via to be deleted.  The

prompt "SELECT VIA" appears on the status line.  If your view of the via is

not clear enough, press <<PGUP>> to expand the window.

Click the <<LEFT MOUSE>> or press <<ENTER>> to delete the via.  Press

<<ESC>> or click <<RIGHT MOUSE>> when you have finished deleting vias.

CHANGING PART OF THE PCB

The EDIT menu makes it possible for you to make changes to items that

you have already placed on the board.  You can edit components, pads, tracks,

strings, and vias.

EDIT

Component

Pad

Track

String

Via

CHANGING A COMPONENT

When you choose EDIT COMPONENT, a message appears on the prompt line

asking you to select a component.  Move the cursor to the component you to

change then click <<LEFT MOUSE>> or press <<ENTER>>.

If you are working on a tight board and you have overlapped components,

a window opens displaying the designators of the components which are over-

lapping.  Select the designator of the component you want to edit by moving

the selection bar to it, and then pressing <<ENTER>> or clicking <<LEFT

MOUSE>>.

A window opens showing the designator, pattern, and comment for the

selected component.

EDIT COMPONENT

Designator            :               U1

Pattern               :               DIP14

Comment               :               74L504

You can change each of these by moving the selection bar to the required

option and pressing <<ENTER>> or clicking <<LEFT MOUSE>>.

If you choose to edit the component designator, a window opens displaying

three options:

COMPONENT DESIGNATOR

Text                  :               U1

Size                  :               60

Hide                  :               Display

You can change the text (e.g. change U3 to C4) or the size of the text.

The final option allows you to hide or display the designator.  Normally, you

would choose to display the designator, but if you are laying down a cluster

of components which are the same, you may only want to display the designator

and comment for one.

When you have finished editing the designator, click <<RIGHT MOUSE>> or

press <<ESC>> to return to the EDIT COMPONENT menu.

You can change the pattern name by moving the selection bar to highlight

PATTERN and then pressing <<ENTER>> or clicking <<LEFT MOUSE>>.  A window

opens and you can enter the new name, then press <<ENTER>> or click <<LEFT

MOUSE>>.  You are returned to the EDIT COMPONENT menu.

Changing the COMMENT field is done in the same way as the designator.

You should note that it is possible to choose a text height which causes the

comment string to overwrite the designator string.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

CHANGING A PAD

If you need to change the designator, shape or connection of a pad then

choose EDIT PAD.  A prompt appears asking you to select the pad.  Move the

cursor to the pad then press <<ENTER>> or click <<LEFT MOUSE>>.  A window

opens showing detailed information about the selected pad.

EDIT PAD

Designator            :       1

Shape                 :       Square62

Power/Gnd             :       No Plane Connection

Rotate

The value next to the designator shows the pin number assigned to the

pad.  If you need to change the pin number, move the selection bar to high-

light the DESIGNATOR option, then press <<ENTER>> or click <<LEFT MOUSE>>.

Now enter the new pin value and once again press <<ENTER>> or click <<LEFT

MOUSE>>.

To change the type of pad, move the selection bar to SHAPE then press

<<ENTER>> or click <<LEFT MOUSE>>.  A window opens listing all the available

pad types.  Choose the new pad type then press <<ENTER>> or click <<LEFT

MOUSE>>.

The POWER/GND option can be used to indicate a connection to either the

ground plane or the power plane.  You can also decide if the connection is to

be direct or relief.  Choose the plane that the pad is to be connected to,

then press <<ENTER>> or click <<LEFT MOUSE>>.  Then choose the type of

connection and press <<ENTER>> or click <<LEFT MOUSE>>.

The ROTATE option can be used to rotate non-circular and non-square pads

through 90 degrees.  This has the effect of providing a wider range of pad

types.

CHANGING A TRACK

You can change the layer a track is on or you can change its width.

Choose EDIT TRACK.  You are prompted to select the track you want to

edit.  Move the cursor to the track, then press <<ENTER>> or click <<LEFT

MOUSE>>.  Remember that a track is just one segment of an overall connection,

so choose the track carefully.

A window opens allowing you to change the track layer or width.

To change the layer, choose LAYER.  A window opens displaying all the

layers which are available.  Choose the layer to which you want to transfer

the track, then press <<ENTER>> or click <<LEFT MOUSE>>.

To change the track width, choose WIDTH, select a track and then select

the new width (remember that you must be on the same layer as the track you

are trying to select).

CHANGING A STRING

You can change the style of a string or the characters which comprise

the string.

Choose EDIT STRING.  You are prompted to select the string you want to

edit.  Move the cursor to the string, then press <<ENTER>> or click <<LEFT

MOUSE>>.

Choose SIZE to change the string size (this refers to the height of the

text).  A window opens listing the sizes which are available.  They range from

48 mils to 560 mils.

Choose TEXT to change the string text.  Remember that all the line

editing features described in the Line Editor Tutorial are available - there

is no need to retype the whole line!

CHANGING A VIA

Since vias belong to every layer, the only parameter you can change is

the via size.

Choose EDIT VIA.  Select a via by moving the cursor to it, then pressing

<<ENTER>> or clicking <<LEFT MOUSE>>.

A window opens listing the via sizes that you can choose.  Select a value

from 40 mils to 150 mils by moving the selection bar to the required value

then pressing <<ENTER>> or clicking <<LEFT MOUSE>>.

Next, you are prompted to select another via and so on.  Finish by

clicking <<RIGHT MOUSE>> or pressing <<ESC>>.

MANAGING FILES

The FILE option of the MAIN MENU lets you carry out many of the

"housekeeping" tasks associated with managing files and the work space.

FILE

Clear

Dos

Files

Path

Quit

Save

It allows you to save and retrieve files, to set the path to a directory,

to inspect a directory for the names of .PCB files, and to clear the work

space.  It also allows you to carry out a range of DOS functions without

losing your current work sheet and the associated settings.

CLEARING THE MEMORY

FILE CLEAR allows you to clear the work space when you want to start

working on a new board.

Before you invoke this option, you must make sure that you have saved all

changes to your current work sheet.

The CLEAR option gives you a chance to change your mind by asking you to

confirm the fact that you want to clear the current work space.  Choose YES

if you want to clear the work space.  Choose NO if you do not want to clear it.

ISSUING DOS COMMANDS

If you are working in the PROTEL-EASYTRAX program and need to carry out

some DOS command that is not available through one of the menus, choose FILE

DOS to exit the program temporarily.

You are strongly advised to save your current work before invoking this

option in case you cannot return to the program for some reason.

A window with the message:

CONFIRM SUSPEND TO DOS

appears.  Choose YES to confirm the fact that you want to quit to DOS.

If you choose YES, the DOS prompt appears and you can carry out most of

the normal DOS commands such as changing drives, inspecting directory lists,

copying files, and changing directories.

There are no restrictions on the DOS commands that you can issue.  One

thing that you must remember, however, is not to type EASYEDIT to start the

program again as this reloads the program and your previous work may be lost.

Another restriction is that the overlay file called EASYEDIT.OVR must

not be moved to a different directory at this stage.

To return to the program, type:

EXIT

at the DOS prompt and your session is resumed at the point at which you

exited.

CHECKING FILES STORED ON THE DISK

You often need to know the names of .PCB files which are already stored

on your disc.  Choosing FILE FILES allows you to check the names of all files

or of selected files.

When you choose this option an input window opens prompting you to enter

a "mask" for the type of files you want to know about.

The default mask of *.* has been chosen so that all files in the default

directory are listed.

FILE MASK

*.*----------------------------

Press <<ENTER>> or click <<LEFT MOUSE>> to list the files.  If you enter

a more restrictive mask such as *.NET or *.PCB, then only files with the

extension .NET or .PCB are listed.

If you enter a specification such as:

CHIP?.PCB

then files with names such as CHIP1.PCB, CHIP2.PCB, and CHIPA.PCB appear.

The selection bar can be moved to highlight a particular filename but no

action can be carried out on the file.

Press either mouse button or the <<ENTER>> key or the <<ESC>> key to

close the window.

LOADING A FILE

This is an option that you need often.  It allows you to load a PROTEL-

EASYTRAX file from the hard disk or the floppy disk into the microcomputer

so that you can view or edit it.

Choose FILE LOAD.  You are asked to specify a file name.  If you cannot

remember the name of the file, you can simply click the <<LEFT MOUSE>> or

press <<ENTER>> and a list of files in the currently selected directory is

displayed.  Move the selection bar to the file you want to load and click

<<LEFT MOUSE>> or press <<ENTER>>.

A window opens asking you to confirm that this is the file you want to

load.  If you choose YES, then the file is loaded and the previous file is

erased from the microcomputer's memory.  For this reason you should ensure

that you have saved your current work before loading another file.

If you want to load a file from a directory which is different to the one

set by FILE PATH, then you should type in the file's full path name.

You can also load a .PCB file when you start the PROTEL-EASYTRAX program.

If you want to load the file called BOARD1.PCB, then at the DOS prompt type:

EASYEDIT BOARD1 and press <<ENTER>>.

The first prompt is bypassed, and the .PCB file is then loaded

automatically.

SPECIFYING A PATH NAME FOR FILES

It is a good idea to store your .PCB files in a different directory to

the PROTEL-EASYTRAX programs and files.

The path name of the directory in which you are going to store you files

can be specified with the PATH option.  When you choose FILE PATH a window

opens asking you to specify a new path.  Enter the specification for the new

path, then press <<ENTER>> or click <<LEFT MOUSE>>.  This directory

specification is now displayed as the currently selected path when you choose

INFORMATION STATUS.

If you have created a directory for your files such as \FILES, then that

is the path name you should enter.  If you have decided that the files should

be stored in a subdirectory of the PCB directory, then you should enter a

path name such as \PCB\FILES.  If you want to store your files on a floppy

disk, then enter a path name such as A:\, or if the files are to be stored

in the second drive, B:\.

LEAVING THE PROTEL-EASYTRAX PROGRAM

When you have finished a PROTEL-EASYTRAX session and are ready to leave

the program, choose FILE QUIT.

You are asked to confirm the fact that you want to quit.  If you respond

with NO, then you can continue with the session.  If you respond with YES,

then you are returned to the DOS prompt.

If the board has been changed in any way since the last save operation,

another window opens asking you if you want to save the file before you are

returned to the DOS prompt.  Choose YES to save the file before you exit and

NO to exit without saving the file.

SAVE FILE BEFORE EXIT

No

Yes

Before turning the computer off, remember that is very important to back

up your files by copying them to a floppy disk and then storing the floppy

disk in a different, and safe, location.

SAVING YOUR FILES

It is a good idea to save the file you are working on as often as

possible.  The time between "saves" varies depending on the size of the board

you are working on.

If you are working on a very large board, it may take a considerable

time to save the file and, therefore, it becomes annoying to save every five

minutes.  However, you do not want to run the risk of having, say, half an

hour's work lost when someone trips over your power cord!

Remember that the Protel-Easytrax program saves your work every thirty

minutes unless you change the backup time in SETUP OPTIONS BACKUP.

Choose FILE SAVE.  You are asked to supply a file name, or else a file

name is specified in the input window.  If you want to give the file a new

name simply type in the name.  If you want to give it the same name as a file

which is already stored on the disk, then you are asked to confirm the fact

that you want to overwrite the file.  Choose YES, then click <<LEFT MOUSE>>

or press <<ENTER>>.

The file is saved under the .PCB extension, but the original disk file is

renamed with a .BAK extension in case you decide that you do not want the

latest changes and would prefer to revert to the previous version.  If you

want to do this, delete the .PCB file and then use the DOS RENAME command to

name the .BAK file as the .PCB file.

Once the file has been saved then you can continue working.

You should also be aware that the backup version of the file also

requires space on the disk.  For this reason, you must ensure that there is

always twice the disk space available than that needed just for the .PCB file.

SETTING GRID SIZES

There are two grids which are active in the PROTEL-EASYTRAX program.

The first is an invisible "snap" grid and the second is a visible grid guide

which takes the form of an array of dots on the screen.

The distance between points in either grid can be varied by choosing the

GRID option from the MAIN MENU.

GRIDS

Snap Grid             :       25

Visible Grid          :       1000

Note that you can change the grid size within some operations such as

Place Track and Place Component.

CHANGING THE SNAP GIRD

The "snap" grid is an invisible grid which the cursor follows (or snaps

to) whenever the cursor keys or the mouse move the cursor around the screen.

As the cursor moves it "snaps" from one grid position to the next.

The normal value is 25 mils but this value can be increased or decreased

depending on your preference.  The current value of the snap grid is always

shown on the status line at the bottom of the screen.

One example of where this is useful is when you want to work with edge

connector pads which are usually either 125 or 156 mils apart.  An example

might be if you were going to connect some of the pads of, say, a XTEDGE or

S100 connector to the ground or power planes.

Move to the point where you will start connecting the pads, choose GRID

SNAP then change the value to, say, 125.  Now each time you move the mouse or

cursor keys, the cursor will snap to the next pad position.

When a grid size is selected, the grid points originate at X:0 Y:0

regardless of whether there is a floating origin.  Thus, if the cursor is at

X:100 Y:100 and you change the snap grid to 30, the cursor would immediately

snap to X:90 Y:90 as that is the nearest grid point.

CHANGING THE VISIBLE GRID

The visible grid helps you to lay components and tracks in a careful and

systematic way.

It is usually set to 1000 mils (i.e. one inch) but the value you will

need to work with varies greatly depending on the level of magnification you

choose.  The higher the magnification, the lower the value of the visible

grid you need.

To change the value, select GRID VISIBLE GRID and enter the new value.

Press <<ENTER>> or click <<LEFT MOUSE>> to exit from the option.

HIGHLIGHTING NETS

The HIGHLIGHT option lets you carry out two main functions.  First you

can highlight a net or a single connection or copy a net and second you can

create a netlist based on the existing set of connections.

The highlight option is used widely to verify nets.  It helps you make

sure that all the connections that need to be made, have been made.  It is

also useful for copying similar connections such as memory routines.

HIGHLIGHT

Connection

Duplicate

Net

Make NetList

Reset Highlight

HIGHLIGHTING A CONNECTION

Choose HIGHLIGHT CONNECTION to highlight a connection between two pads.

The prompt:

SELECT TRACK/PAD TO HIGHLIGHT

appears on the prompt line.  Move the cursor to a track or pad in the

connection you want to highlight, then press <<ENTER>> or click <<LEFT

MOUSE>>.  The message:

SORTING TRACKS.PLEASE WAIT.

appears at the top of the screen, then the connection is highlighted.  The

progress of the highlighting is shown on the status line at this stage.

The highlight remains until you either highlight another connection or

net or until you select HIGHLIGHT CONNECTION or HIGHLIGHT NET and highlight

a blank area of the board.

DUPLICATING NETS

The HIGHLIGHT DUPLICATE option provides you with a simple means of

duplicating nets such as memory routes.

To duplicate a net, highlight it first using HIGHLIGHT NET.  Choose

HIGHLIGHT DUPLICATE and the message:

SELECT COPY SOURCE

appears on the prompt line.  Select the point on the highlighted net that

you want to act as the reference point then press <<ENTER>> or click <<LEFT

MOUSE>>.  Now the message:

SELECT COPY DESTINATION

appears on the prompt line.  Move the cursor to the point you want to copy

the net to and press <<ENTER>> or click <<LEFT MOUSE>>.  You are asked to

confirm that you want to copy the highlighted connection.  Choose YES to copy

and NO to cancel the copy.

The net is duplicated and the prompt line indicates that you can copy the

net again.  You can continue making duplicate copies of the net by moving the

cursor to a new position then confirming the copy until you press <<ESC>> or

click <<RIGHT MOUSE>>.

If no net is highlighted, nothing is duplicated.

An example of the use of the HIGHLIGHT DUPLICATE option is the

duplication of memory routes.  A memory route is usually a short pad to pad

connection which is only on one layer and which may comprise two or three

short track segments.  It is not possible to duplicate such a pattern using

the REPEAT option as that only repeats the placing of the last track segment.

HIGHLIGHTING A NET

The HIGHLIGHT NET option is similar to the HIGHLIGHT CONNECTION option.

The only difference is that the HIGHLIGHT NET highlights a complete net,

whereas HIGHLIGHT CONNECTION only highlights one connection.

Choose HIGHLIGHT NET, then move the cursor to point to a part of the net

you want to highlight.  The progress of the highlighting is shown on the

status line.

CREATING A NETLIST

If you have manually routed a board and want to create a netlist, use

the HIGHLIGHT MAKE NETLIST option.

One reason why you might want to produce a netlist after you have made

all connections is to obtain a printed copy of the net list for manual

checking of connections.  Another reason might be that you do not have the

PROTEL-SCHEMATIC program which produces netlists and have had to make all

connections manually.

When you choose HIGHLIGHT MAKE NETLIST, an input window with the heading

"NETLIST FILENAME" opens.  At this stage you should enter the name for the

netlist file which is to be created.

You are now asked to confirm that you want the netlist generation to

proceed.  Choose YES and the nets are highlighted one by one as they are

stored in the file.  You will notice that on the right hand side of the status

line a RECURSION DEPTH COUNT is provided.  This simply indicates how hard the

program is "thinking" in working out the nets!

The netlist generation continues until all nets have been stored in the

file.  At this stage the status line is restored and the cursor reappears on

the board.

The file that is saved has the extension .NET.

RESETTING A HIGHLIGHT

If you have a connection or net highlighted and you no longer want it to

be highlighted, choose HIGHLIGHT RESET HIGHLIGHT.

Choosing this option simply removes the highlight from the connection or

net.  No message is displayed.

DELETING FREE PRIMITIVES

Sometimes when you are laying tracks, you might realize that there is a

better way to do it.  You, therefore, want to delete all the tracks, vias,

and free pads on the board and start again.  Or you might want to delete the

tracks, pads, and vias on one part of the board.

To delete free primitives you can place an area fill over the required

area and then highlight the net.  Since the tracks, pads, and vias are now

"connected" to the fill, all primitives are highlighted.  Now simply delete

the highlight and all the free primitives are removed from the board.  You

might need to redraw the screen to see the components again.

The steps required are:

Select PLACE FILL

Select the corners to cover all primitives

Press <<ESC>> to exit the PLACE FILL option

Choose HIGHLIGHT NET and select the fill

Wait while the fill is highlighted

Choose DELETE HIGHLIGHT

Redraw the screen with the <<END>> key

GETTING INFORMATION ABOUT THE BOARD

The INFORMATION menu is extremely useful in providing information about

the state of the board on which you are currently working.

Essential facts such as the number of arcs, components, pads, holes, and

vias as well as the amount of memory which is available can be determined

with this option.

INFORMATION

Board Dimensions

Components

Highlighted Pins

Library Components

Pwr/Gnd Pins

Status

For each of the options in the INFORMATION menu, a "dump" file can be

generated which contains the information displayed in the window.  When a

window is displayed, pressing <<ESC>> or clicking <<RIGHT MOUSE>> simply exits

you from the menu.

If, however, you press <<ENTER>> or click <<LEFT MOUSE>>, a window opens

asking you to specify a filename for a dump file.  Enter a name, then press

<<ENTER>> or click <<LEFT MOUSE>>.  The file is given a .DMP extension.  It

is a standard text file which can be output using the DOS TYPE command.

DETERMINING THE BOARD'S DIMENSIONS

Choose INFORMATION BOARD DIMENSIONS to view an information window which

displays the current size of the board.

The minimum and maximum values of the X and Y coordinates are displayed.

It is possible that the minimum values can be negative.  The reason is that

if an item such as a pad is placed at, say, X:0 Y:500, the left hand side of

the pad spills over into the negative area.

If the diameter of the pad is 62 mils, then you have extended the board

in the negative direction by 31 mils.

BOARD DIMENSIONS

Minimum X             :               -0.03   Inches

Minimum Y             :                0.02   Inches

X Size                :                5.96   Inches

Y Size                :                5.01   Inches

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.  Press

<<ENTER>> or click <<LEFT MOUSE>> to specify the name of the dump file.

LISTING THE COMPONENTS ALREADY PLACED

To obtain a list of all the components you have already placed on the

board choose INFORMATION COMPONENTS.  PROTEL-EASYTRAX scans the components

already placed and lists them in a information window.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.  Press

<<ENTER>> or click <<LEFT MOUSE>> to specify the name of the dump file.

CHECKING HIGHLIGHTED PINS

If you highlighted a connection or a net using HIGHLIGHT NET or

HIGHLIGHT CONNECTION, then INFORMATION HIGHLIGHTED PINS can be used to provide

you with a list of the pins in that net.  This is useful if you want to check

to make sure you have connected all the pins in a particular net.

Choose INFORMATION HIGHLIGHTED PINS and a window opens listing all the

pins in that net.

If you choose INFORMATION HIGHLIGHTED PINS when no pins have been

highlighted, the message:

NO PINS HAVE BEEN HIGHLIGHTED

is displayed.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.  Press

<<ENTER>> or click <<LEFT MOUSE>> to specify a dump file.

CHECKING THE NAMES OF LIBRARY COMPONENTS

To find the names of components stored in the currently selected library,

choose INFORMATION LIBRARY COMPONENTS.

A window opens showing the names of all components in the library.  Use

<<PGDN>> to move to the next screen if there are too many names.

If there is no currently selected library, the message:

NO LIBRARY HAS BEEN OPENED

is displayed.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.  Press

<<ENTER>> or click <<LEFT MOUSE>> to specify a dump file.

LISTING POWER AND GROUND PLANE NETS

To obtain a list of all the power nets or ground nets, choose INFORMATION

PWR/GND PINS.  A menu appears which allows you to choose the ground plane or

the power plane.

Choose GROUND PLANE to list all connections to the ground plane.  Choose

POWER PLANE to list all connections to the power plane.

If there are no pins, the message:

NO PINS ON THAT PLANE

is displayed.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.  Press

<<ENTER>> or click <<LEFT MOUSE>> to specify a dump file.

ESSENTIAL INFORMATION ABOUT THE BOARD

The STATUS option allows you to check information about the board.

Choose INFORMATION STATUS.  An information window opens which shows the

following details:

STATUS

Free Memory                   :       28559           Bytes

Free Ems Memory               :       0               Bytes

Arcs                          :       15

Components                    :       36

Fills                         :       0

Holes                         :       520

Pads                          :       435

Strings                       :       5

Tracks                        :       1554

Vias                          :       85

Work File Name                :       DEMO.PCB

Library                       :       EASYSTD.LIB

Current Path                  :       C:\EASYTRAX

Disk Size                     :       21309440        Bytes

Disk Free                     :       180224          Bytes

The display shows the number of bytes of free memory and expanded (EMS)

memory.  When you are creating a large board and are worried about the amount

of free memory, it is a good idea to check this status screen from time to

time.

It also displays the number of arcs, components, fills, holes, pads,

strings, tracks, and vias on the board as well as the board's filename and

the name of the current library.

This information can be quite useful.  If you are trying to minimize the

number of vias, for example, you can check this screen to see how many are

currently on the board.

The bottom of this window shows the currently active path as well as the

disk size and the number of free bytes of memory available on the disk.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.  Press

<<ENTER>> or click <<LEFT MOUSE>> to specify a dump file.

USING THE JUMP OPTION

The JUMP option is particularly useful on large boards when you need to

jump to a specific component, locate a string or pad, or move to a particular

location.

Remember that the position of the cursor on the board is defined by its

distance in mils from the origin in both the X and Y directions.  Looking at

the screen, the X direction is horizontally towards the right and the Y

direction is vertically upwards.

This may be different, however, if the origin has been changed in the

CURRENT menu.  In this case any X values to the left of the origin are

negative and any Y values below the origin are also negative.

JUMP

Component

Origin

Pad

String

JUMPING TO A COMPONENT

You will often find it necessary to jump to a specified component on

board.  The JUMP COMPONENT option uses the component designators as the key

to locating components.

Choose JUMP COMPONENT.  An input window opens asking for the component

designator.

If you can't remember the designator, simply click <<LEFT MOUSE>> or

press <<ENTER>> and an information window opens showing all component

designators on the board.

Move the selection bar to the designator required and click <<LEFT

MOUSE>> or press <<ENTER>>.  The cursor jumps to pin 1 of the component you

requested.

Try this with the DEMO.PCB file loaded.  Choose JUMP COMPONENT, then

input U5 which is the designator for the Z80 microprocessor.  Notice that

the cursor jumps to the square pad which specifies pin 1 on the chip.

After the cursor has been relocated on the component you requested, the

program returns to edit mode and does not stay in the JUMP menu.

JUMPING TO A LOCATION

This option allows you to specify a location on the board for the cursor

to jump to.  Choose JUMP LOCATION.  A window opens asking you to input the X

location.  The value in the window shows the current position of the cursor.

Type in the new value then press <<ENTER>> or click <<LEFT MOUSE>>.  A window

now opens asking for the Y location.  Type in the Y value and press <<ENTER>>

or click <<LEFT MOUSE>>.

The cursor immediately relocates at the new coordinate and you are

exited from the JUMP menu.

It may appear to be strange that the default value given in the input

window is the current cursor position.  If you are already at that position

why would you want to jump there?  The reason is that in specifying a

coordinate to jump to, you may make a mistake in typing the numbers.  If you

do then you can easily edit the incorrect number rather than typing both

coordinates in again.

JUMPING TO THE ORIGIN

This option simply relocates the cursor at the absolute origin in the

bottom left hand corner of the board, regardless of whether a floating origin

has been set.

Choose JUMP ORIGIN and the cursor immediately relocates at the origin and

you are exited from the JUMP menu.

JUMPING TO A PAD

This option allows you to nominate the pin on a given component to jump

to.  Choose JUMP PAD.  Select the component by moving to it with the mouse

or the cursor keys.  Press <<ENTER>> or click <<LEFT MOUSE>>.

An input window then opens asking you to enter a pin number.  Type in a

pin number then press <<ENTER>> or click <<LEFT MOUSE>>.  The cursor jumps to

that pad.

The input window remains open, with the cursor flashing under the first

character of the existing pin number.  You can now enter another pin number

and press <<ENTER>> or click <<LEFT MOUSE>>, or you can press <<ESC>> or

click <<RIGHT MOUSE>> to exit from the JUMP option.

This is useful when connecting pads if you need to remember the order in

which the pins are numbered.  This is not usually a problem with ICs, but it

is possible to forget the numbering system for connectors such as DB25s.

If the pin number that you specify does not exist (e.g. if you input pin

17 for a DIP16 pattern), then the message:

PAD NOT FOUND

is displayed.  Press <<ENTER>> or click <<LEFT MOUSE>> to enter another pin

number, or press <<ESC>> or click <<RIGHT MOUSE>> to exit from the option.

JUMPING TO A STRING

This option allows you to jump to a string that you specify.  Choose

JUMP STRING.  An input window opens asking you to enter the string to jump to.

If there is more than one matching string on the board PROTEL-EASYTRAX

does an exact match first.  Therefore, if you type in Pro, it firstly finds

the string Pro, then PRO, then pro then any other beginning with Pro... and

so on.

If a matching string is found, the cursor is relocated over the first

character and you are exited from the JUMP option.

If you attempt to jump to a string which does not exist, the message:

STRING NOT FOUND

is displayed.  Press <<ENTER>> or click <<LEFT MOUSE>> to enter another string

or, press <<ESC>> or click <<RIGHT MOUSE>> to exit from the option.

MANAGING LIBRARIES

The LIBRARY menu provides the options you need to create new library

components, manage existing libraries of components, and create new libraries.

If you have opened a library (called the currently selected library) the

name of that library appears in the INFORMATION STATUS window.

LIBRARY

Browse

Compact

Delete

Explode

File

List

Merge

New Library

Rename

ADDING COMPONENTS TO THE LIBRARY

Before you can add a component to the library you must first create it

with the normal tools available through the PROTEL-EASYTRAX program.  Most

components can be created with tracks, arcs, and pads.  The procedure for

doing this is described in detail in the Tutorial:  Creating New Components.

When you have created the component, mark it as a block using the BLOCK

DEFINE option.

Then choose LIBRARY ADD.  Enter a name for the new component, press

<<ENTER>> or click <<LEFT MOUSE>>, then confirm that you want to save the

component by choosing Yes.  The message:

CREATING LIBRARY COMPONENT.PLEASE WAIT.

is displayed briefly as the component is added to the current library.

If the name you choose is already being used for a library component,

the message:

LIBRARY NAME IS ALREADY USED

is displayed.

BROWSING THROUGH LIBRARY COMPONENTS

The LIBRARY BROWSE option allows you to inspect the patterns of

components stored in the library.  You can either search backwards or forwards

through the library or else inspect a particular component by nominating the

pattern.

Choose LIBRARY BROWSE.  The screen is cleared (don't worry you haven't

lost your work!) and the following menu appears.

LIBRARY BROWSE

Next

Previous

Select

Choose LIBRARY BROWSE NEXT to display the next component in the library.

You may need to expand the screen using <<PGUP>> to be able to see it

properly.  While you are in the browse option you can use the <<PGUP>>,

<<PGDN>>, and cursor keys.

When the component is displayed, the prompt line contains the message

"LIBRARY BROWSE".  This is to remind you that to return to the menu you

should press <<ENTER>> or click <<LEFT MOUSE>>.

Choose LIBRARY BROWSE PREVIOUS to step backwards through the components

in the library.

Choose LIBRARY BROWSE SELECT to display a particular component.  An

input window opens prompting you to enter the name of a component.  If you

cannot remember the name, enter a <<?>> and an information window listing all

the names opens.  Select the component by moving the selection bar to the

correct name then clicking <<LEFT MOUSE>> or pressing <<ENTER>>.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.  When

you do, the board you are working on reappears.

COMPACTING THE LIBRARY

If you add and delete a number of components, then it is possible that

the library file will become fragmented.  This may be a problem as it will

take up more space than necessary on the disk.

To compact the library, choose LIBRARY COMPACT and a window opens asking

you to confirm that you want to compact the library.  If you choose Yes,

another window opens which lists each of the components found during the

compacting process.  When the process is complete a message similar to the

following appears:

COMPONENTS FOUND = 169

Press <<ENTER>> or click <<LEFT MOUSE>> to finish the compacting.

DELETING A COMPONENT FROM THE LIBRARY

To delete a component from the library, choose Library Delete and enter

the name of the component you want to delete.  If you do not know the name

of the component, simply press <<ENTER>> or click <<LEFT MOUSE>> when you see

the ? in the input window.  Select the component you want to delete.

You are asked to confirm the fact that you want to delete the component.

Choose Yes to delete it.

"EXPLODING" A COMPONENT

Often, when you want to design a new component there is already a

component in the library which is similar and may only need minor

modifications.  If that is the case, you can place the existing library

component on the board and explode it.  Having "exploded" the component, you

can edit it and then save the edited version as a new library component.

Choose LIBRARY EXPLODE and you are asked to select the component you

want to "explode".  Move to the component and press <<ENTER>> or click <<LEFT

MOUSE>>.

The component appears to be basically unaltered (except for the

designator and comment which are removed) but each element of the component

is now in its primitive state.  That is, instead of a single component, it

is now a collection of tracks, arcs, and pads.

To convince yourself of this, move to any other component and select the

overlay (by pressing <<+>> or <<->> on the keypad).  Now choose EDIT TRACK

and move the cursor over the track portion of a component.  You are not

allowed to edit the track!  Try the same thing with a component which you

have exploded - you can now change the track.

Once you have changed the pattern you must then define it as a block

and add it to the library as a new component as described in the Tutorial:

Creating New Components.

This option is also useful if you want to make minor adjustments to

existing library patterns.

CHOOSING A DIFFERENT LIBRARY FILE

Choose the LIBRARY FILE option to change the currently selected library

file.  A different directory or disk drive can be specified if required.

A window opens with the prompt "LIBRARY FILE NAME".

Enter the name of the library file you want to make active.  If you

cannot remember the name, enter <<?>> and a list of the .LIB files is

displayed.  Move the selection bar to the one you want to open then press

<<ENTER>> or click <<LEFT MOUSE>>.

Remember that you can check the name of the currently active library by

choosing INFORMATION STATUS.

LISTING THE LIBRARY COMPONENTS

Choose LIBRARY LIST to list the names of components in the currently

selected library.  The message:

WORKING.PLEASE WAIT.

is displayed at the top of the screen and then an information window opens

displaying the names of the components in the library.

If you need to check the names of the components in other libraries, you

must first change the currently selected library with LIBRARY FILES as

described above.

MERGING LIBRARIES

This option allows you to move a component from another library into the

existing library.

Choose LIBRARY MERGE.  You are asked to enter the name of the library to

merge from.  Enter the name of the library from which you want to move the

component.  You are then asked to enter the name of the component you want to

merge.  If the component exists then it is added to the currently selected

library.

CREATING A NEW LIBRARY

This option allows you to create a new library.  Choose LIBRARY NEW

LIBRARY.  You are prompted to supply the name for the new library.  A new

library file is created on the disk with the extension .LIB.

If you try to specify a name for the new library which is the same as a

library which already exists, then the message:

LIBRARY FILE ALREADY EXISTS

is displayed.

This action removes the existing library from the memory (but not from

the disk) and allows you to create a library with a new set of components.

You can, of course, use LIBRARY MERGE to move components from other libraries

into the new library.

RENAMING A COMPONENT

This option allows you to rename a component.  Choose LIBRARY RENAME.

You are asked to rename a component in the currently selected library.  The

message:

NAME IN LIBRARY

appears.

Enter the name of the component whose name you want to change.  The

message:

NEW LIBRARY COMPONENT NAME

then appears.  Enter the new name for the component.  You are asked to confirm

that you want to rename the component.  If you choose YES, it is renamed.

If the component does not exist, the message:

COMPONENT NOT FOUND IN LIBRARY

is displayed.

MOVING ITEMS ON THE BOARD

This is particularly important for "fine tuning" a board layout.  Items

such as pads, vias, components, fills, arcs, and strings can be moved to

another place on the board.  But it is the flexibility for controlling tracks

that you will find most useful.  You can move a track, break a track at a

particular place, drag a track from one of its endpoints or re-route a track.

MOVE

Arc

Break

Component

Drag End

Fill

Pad

Re-Route

String

Track

Via

MOVING AN ARC

Choose MOVE ARC to move an arc.  The message "SELECT ARC" appears on the

prompt line.  Move the cursor to the arc you are going to move, then press

<<ENTER>> or click <<LEFT MOUSE>>.

Use the mouse or the cursor keys to move the outline of the arc to its

new position.  Fix the position by pressing <<ENTER>> or clicking <<LEFT

MOUSE>>.

You are now prompted to select another arc.  If you have finished moving

arcs, press <<ESC>> or click <<RIGHT MOUSE>>.  Otherwise you can select

another arc and move it in the same way.

BREAKING A TRACK

Sometimes you may want to make a track shorter or break a track into two

sections.  To do this choose MOVE BREAK.  The prompt "SELECT TRACK" appears.

Move the cursor to the exact spot on the track where you want to break it.

The track now acts like a "rubber band".  If you move the cursor, the

track stretches in two sections as it were a rubber band held at the

cursor position.  The message:

DRAGGING BROKEN TRACK

is displayed on the prompt line.

When you have positioned the tracks, press <<ENTER>> or click <<LEFT

MOUSE>>.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

The important thing to note about this option is that it "breaks" the

track into two tracks.  Each part will now be recognized as a track so one

section can be deleted if required.  This is a simple way of making a track

shorter.

MOVING A COMPONENT

Choose MOVE COMPONENT to move a component to a new position on the board.

The message "SELECT COMPONENT" on the prompt line indicates that you

should move the cursor to the component that you want to move.  Press

<<ENTER>> or click <<LEFT MOUSE>> to select the component.

The message on the prompt line is replaced with "MOVING COMPONENT".  As

you move the cursor, an outline of the component also moves.

Place the component in its new position by moving it with the mouse or

the cursor keys.  Refer to the table below for details of how to rotate the

component.

<<SPACE>>          Rotates the block 90 degrees.  Pressing it twice

rotates the block 180 degrees and so on.

<<X>>              Pressing <<X>> flips the block in the X direction.

<<Y>>              Pressing <<Y>> flips the block in the Y direction.

<<LEFT>>           Pressing the left cursor key moves the block left.

<<RIGHT>>          Pressing the right cursor key moves the block right.

<<UP>>             Pressing the cursor key moves the block upwards.

<<DOWN>>           Pressing the cursor key moves the block down.

Mouse              Moving the mouse in any direction also moves the

block in that direction

<<ENTER>>          Pressing the <<ENTER>> key fixes the block in its

current position.  Pressing <<LEFT MOUSE>> has the

same effect.

A window opens asking if you want to "DRAG TRACKS".

If you want the tracks to retain their "connectivity" then select Yes.

At this stage all tracks are stretched to reach the new position of the

component.  Don't worry about the fact that the tracks are distorted.  You

can re-route them later.  If you choose No, the component is moved but the

tracks remain in position, thus breaking any connections.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

DRAGGING THE END OF A TRACK

This option allows you to drag the end of a track to another place on the

board while maintaining its connection to other tracks.

Choose MOVE DRAG END and you are prompted to select a track.  Move to

the track you want to drag and click <<LEFT MOUSE>> or press <<ENTER>>.  The

cursor jumps to the closest track end and the message:

DRAGGING TRACK END

appears on the prompt line.  You can now drag the track to the new position.

Any connected track is also dragged and vias are repositioned.

Note that the track you select must be on the currently selected layer.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

MOVING A FILL

Choose MOVE FILL to move a fill.  The message "SELECT FILL" appears on

the prompt line.  Move the cursor to the fill you are going to move then

press <<ENTER>> or click <<LEFT MOUSE>>.

The message "MOVING FILL" on the prompt line indicates that you should

use the mouse or the cursor keys to move the outline of the fill to its new

position.  Fix the position by pressing <<ENTER>> or clicking <<LEFT MOUSE>>.

You are prompted to select another fill.  If you have finished moving

fills, press <<ESC>> or click <<RIGHT MOUSE>>.

MOVING A PAD

Choose MOVE PAD if you want to move a pad.  The message "SELECT PAD"

appears on the prompt line.  Move the cursor to the pad you are going to

move then press <<ENTER>> or click <<LEFT MOUSE>>.

The message:

MOVING PAD

appears on the prompt line.  Use the mouse or the cursor keys to move the

outline of the pad to its new position.  Fix the position by pressing <<ENTER>>

or clicking <<LEFT MOUSE>>.

You are now prompted to select another pad.  If you have finished moving

pads, press <<ESC>> or click <<RIGHT MOUSE>>.  Otherwise you can select

another pad and move it in the same way.

RE-ROUTING A TRACK

The MOVE RE-ROUTE option allows you to manually route a connection by

breaking the track at a number of points.  The sections remain anchored to

the pads or vias at either end.

You break the tracks and choose the new route in the same way as that

described in the Move Break section above.

MOVING A STRING

Choose MOVE STRING if you want to move a string.  The message "SELECT

STRING" appears on the prompt line.  Move the cursor to the string you are

going to move then press <<ENTER>> or click <<LEFT MOUSE>>.  Note that you

can position the cursor anywhere on the string.

The message:

MOVING STRING

appears on the prompt line.  Use the mouse or the cursor keys to move the

outline of the string to its new position.  Fix the position by pressing

<<ENTER>> or clicking <<LEFT MOUSE>>.

You are now prompted to select another string.  If you have finished

moving strings, press <<ESC>> or click <<RIGHT MOUSE>>.  Otherwise you can

select another string and move it in the same way.

MOVING A TRACK

When you want to move a track, choose MOVE TRACK and then, in response

to the:

SELECT TRACK

prompt, move the cursor to the track you are going to move.  Remember that a

track is a single line segment, not the entire connection between two pads.

Select the track you want to move by pressing <<ENTER>> or clicking

<<LEFT MOUSE>>.  The message:

MOVING TRACK

appears on the prompt line.  At this stage you can move the track to the

required position.

When you move the track, connectively with vias and related tracks is

maintained, so connected tracks and vias are dragged with the track.  Pads

are not moved with the track however.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

MOVING A VIA

Choose MOVE VIA if you want to move a via.  The message:

SELECT VIA

appears on the prompt line.  Move the cursor to the via you are going to

move then press <<ENTER>> or click <<LEFT MOUSE>>.

Use the mouse or the cursor keys to move the outline of the via to its

new position.  Fix the position by pressing <<ENTER>> or clicking <<LEFT

MOUSE>>.

You are now prompted to select another via.  If you have finished

moving vias, press <<ESC>> or click <<RIGHT MOUSE>>.  Otherwise you can

select another via and move it in the same way.

Note that the tracks which are connected to the via are stretched to

maintain their connectivity.

PLACING ITEMS ON A BOARD

The PLACE option allows you to place items such as arcs, components,

tracks on a board.  As a general rule, you should ensure that you are on the

correct layer before you place items using this option.

PLACE

Arc

Component

Fill

Pad

String

Track

Via

PLACING AN ARC

You may need to place an arc if you are creating your own component or

if you want to produce an arc-shaped outline on the board.

Choose PLACE ARC.  A prompt appears on the status line asking you to

place the arc.  Move the cursor to the position you want the arc to appear at

and click <<LEFT MOUSE>> or press <<ENTER>>.

A white circle appears and you are asked to "SELECT ARC RADIUS".  To do

this, use the <<RIGHT>> cursor key to increase the radius and the <<LEFT>>

cursor key to decrease it.  If you need to change the radius significantly,

then hold the <<SHIFT>> key down and use the <<RIGHT>> and/or <<LEFT>> cursor

keys.  The change in radius will be ten times greater.

When you have the radius you require, press <<ENTER>> or click <<LEFT

MOUSE>>, and you are then prompted to select the arc segments.  The circle is

divided into four segments.  One is from 12 o'clock round to 3 o'clock and is

under the control of the <<UP>> cursor key.  The next is from 3 o'clock to

6 o'clock and is under the control of the <<RIGHT>> cursor key.  The 6 to 9

segment is controlled by the <<DOWN>> key and the remaining segment is

controlled by the <<LEFT>> key.  Pressing any of these keys results in that

segment of the arc being deleted.  Pressing the same key again restores the

deleted segment.

As an example, if you want an arc which extends from 3 o'clock around to

9 o'clock in the clockwise direction, press the <<LEFT>> key and the <<UP>>

key.

Press <<ENTER>> or click <<LEFT MOUSE>> and the arc is fixed in position.

You are now given the opportunity to place another arc.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

PLACING A COMPONENT

When you choose PLACE COMPONENT you are prompted to indicate where you

want to place the component.  Move the cursor to the place required and click

<<LEFT MOUSE>> or press <<ENTER>>.  An input window opens asking you to enter

the name of the component.  If you have not yet placed a component, the

window contains a question mark.

You can now type in the component name or else press <<ENTER>> or click

<<LEFT MOUSE>> to see the list of components which are available.

Move the selection bar to the required component.  If the component does

not appear in the window press <<PGDN>> to see more.  Press <<ENTER>> or

click <<LEFT MOUSE>> to select the component.

The prompt:

COMPONENT DESIGNATOR

appears.

You must now supply a component designator.  A component designator

usually consists of a letter such as C or J or R followed by a number.  Thus

you may decide to call an edge connector J3 or a capacitor C4.  If you do not

supply a designator PROTEL-EASYTRAX assigns one starting at A1 then A2 and so

on.  If you specify a designator which already exists, the message:

COMPONENT DESIGNATOR IS ALREADY USED

appears.

After you supply the designator you are asked to input a comment.

Remember that any comments appear on the layer selected in SETUP COMPONENT

TEXT.  It is a good idea to keep them as short as possible.

Once you have entered the comment the prompt:

MOVING COMPONENT

appears.  The component can now be moved to another position if required.

If you need to rotate the component or flip it, refer to the table below to

determine the keys you need to press.  When it is in the correct position

press <<ENTER>> or click <<LEFT MOUSE>>.

<<SPACE>>          Rotates the block 90 degrees.  Pressing it twice

rotates the block 180 degrees and so on.

<<X>>              Pressing <<X>> flips the block in the X direction.

<<Y>>              Pressing <<Y>> flips the block in the Y direction.

<<LEFT>>           Pressing the left cursor key moves the block left.

<<RIGHT>>          Pressing the right cursor key moves the block right.

<<UP>>             Pressing the up cursor key moves the block upwards.

<<DOWN>>           Pressing the down cursor key moves the block down.

Mouse              Moving the mouse in any direction also moves the

block in that direction.

<<ENTER>>          Pressing the <<ENTER>> key fixes the block in its

current position.  Press <<LEFT MOUSE>> has the same

effect.

Note that when you have placed the component, you are prompted to place

the next one.  If you do not want to place another component press <<ESC>>

or click <<RIGHT MOUSE>> to exit from this option.

PLACING A FILL

If you need to place a rectangular area of copper then choose the PLACE

FILL option.

The message:

SELECT FIRST CORNER

appears on the prompt line at the bottom of the screen.  After you select the

first corner, an outline of the rectangle appears to help you place it

accurately.  Click <<LEFT MOUSE>> or press <<ENTER>>.  Now the message:

SELECT SECOND CORNER

is displayed.  Move the cursor to the second corner and click <<LEFT MOUSE>>

or press <<ENTER>>.  As soon as you select the second corner the rectangle

area is filled.

If you select the second corner in the same place as the first corner

you hear a beep and are asked to re-specify both the first and second corners

again.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

You can use the PLACE FILL option to delete all the free primitives on

a board.

PLACING A PAD

To place a pad, select the PLACE PAD option and move the cursor to the

required position on the board then click <<LEFT MOUSE>> or press <<ENTER>>.

The shape of the pad which is placed on the board is determined by the

setting in the CURRENT menu.  The default type is called Round62.  If you

require a pad of a different shape (say, a square pad), then change it in the

CURRENT menu and then go back to the PLACE menu.  You can also choose Moire

or Cross-Hair targets to help you line up plots.

Remember that pads belong to every layer, therefore, you do not have to

worry about which layer you are on before placing a pad.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

PLACING A STRING

To place a string, choose the PLACE STRING option.  A window opens which

allows you to enter the string to be placed.  Enter the string then press

<<ENTER>> or click <<LEFT MOUSE>>.

The string appears on the board in inverse along with the message:

MOVING STRING

on the prompt line.  You can now move the string to the place required.  Use

the cursor keys or the mouse.

The size of the string depends on the settings in the Current menu.

After the string is placed you can specify another.  When you have

finished placing strings, click <<RIGHT MOUSE>> or press <<ESC>>.

PLACING TRACKS

To route the board, you need to lay tracks to connect each of the pads

and vias that are to form part of each "net".  To do this choose PLACE TRACK.

The message:

SELECT TRACK START POINT

is displayed on the prompt line.  Move to the point where you want the track

to begin and press <<ENTER>> or click <<LEFT MOUSE>>.  Now when you move the

mouse or use the cursor keys a thin line is drawn outlining where the track

will be laid.  When the cursor is at the point that you want to lay the track

press <<ENTER>> or click <<LEFT MOUSE>>.  The track is drawn at the width

specified in the Current menu.

If you now move the cursor to a different position the track is continued

and so on.  If you click <<RIGHT MOUSE>> or press <<ESC>> then you can move

to a different spot on the board and lay another track.

To stop laying tracks, press <<ESC>> or click <<RIGHT MOUSE>> when the

prompt "SELECT TRACK START POINT" appears.

Tracks can only be drawn as vertical, horizontal, or 45 degree diagonal

lines when SETUP OPTIONS ORTHO MODE is set to "ON".  This has been set as the

default because it is regarded as good practice in the industry to only draw

tracks this way.

If SETUP OPTIONS ORTHO MODE is set to "OFF" tracks can be drawn at any

angle.

If SETUP OPTIONS VIA MODE is set to "ON", when you lay a track on one

layer then switch to another layer to continue the track, a via is placed

automatically.  If the option is set to "OFF", you must place the via

manually.

PLACING A VIA

The process for placing a via is identical to that for placing a pad.

Choose PLACE VIA and move to the place where you want to place the via then

press <<ENTER>> or click <<LEFT MOUSE>>.  The size of the via is determined

by the setting in the Current menu.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

REPEATING ACTIONS

The REPEAT option lets you automatically place a component, track, pad,

or via a number of times at predetermined places.  This is useful if you

have a number of items which you want to place in a regular pattern.

REPEAT PLACEMENT

Execute Repeat

Count Default         :       6

X-Offset (Mils)       :       0

Y-Offset (Mils)       :       100

When you choose REPEAT, the window shown above opens allowing you to

change the count default, the X-Offset, and the Y-Offset.  If you choose

COUNT DEFAULT, you can change the number of times the previous placement is

repeated.  The X-Offset and Y-Offset determine the direction in which the

placements are made.

When the option is executed, the previous placement is duplicated in a

regular pattern from its original position.  Thus, if you place a pad at

X:1000 Y:800 and set a COUNT DEFAULT of 3 and an X-Offset of 200 and a

Y-Offset of 50, the three pads are placed at X:1200 Y:850, X:1400 Y:900,

and X:1600 Y:950 when you execute it.  The cursor remains at X:1000 Y:800.

The offsets can also be negative if you want to duplicate down the

board (make the Y-offset negative) or towards the left edge of the board

(make the X-offset negative).

To implement a repeat operation, choose REPEAT EXECUTE REPEAT.  You are

prompted to enter the COUNT DEFAULT again just to make sure that you really

want to repeat the operation the number of times you have specified.  You

are then prompted to:

CONFIRM MULTIPLE REPEAT PLACE

Choose YES to execute the repeat and No to return to the REPEAT menu.

If you use the REPEAT option to copy a track, you should be aware that

only the last track placed is copied.  The option does not allow you to copy

an entire connection.  If you want to copy a set of tracks, use the HIGHLIGHT

DUPLICATE option.

CHANGING THE SETUP OPTIONS

The SETUP menu allows you to change many of the "default" parameters

which exist each time you start PROTEL-EASYTRAX.  If you change them, they

are saved when you exit from the program and subsequently become the new

default parameters.

These settings are saved in a file called EASYEDIT.DFT.  Each time you

start PROTEL-EASYTRAX, this file is loaded so that your setup preferences are

automatically re-assigned.  If you want to return to using the default setup

parameters, simply delete the .DFT file.

SETUP

Component Text

Layer Colours

Menu Colours

Keys

Options

Redraw

Strings

Toggle Layers

CONTROLLING COMPONENT TEXT

SETUP COMPONENT TEXT lets you choose the layer on which the component

text is placed.  It also lets you hide the component text designators and

comments if required.  When you choose the option the following menu appears.

COMPONENT TEXT DEFAULTS

Comments              :       Show

Designators           :       Show

Layer                 :       Overlay

The SETUP COMPONENTS TEXT COMMENTS option lets you choose to Hide or

Show the designator comment.  The SETUP COMPONENT TEXT DESIGNATORS option lets

you Hide or Show the designator of the component.  In each case, move the

selection bar to highlight the option you want, then press <<ENTER>> or

<<LEFT MOUSE>>.

When you choose SETUP COMPONENT TEXT LAYER, a window opens giving you the

choice of putting the text on the copper layer or the overlay.  The text is

either placed on the top layer or the overlay depending on whether you choose

COPPER or OVERLAY respectively.

Press <<ESC>> or <<RIGHT MOUSE>> to exit from this option.

CHOOSING LAYER COLOURS

Choose SETUP LAYER COLOURS to change the layer colours if the colours on

your screen are not suitable.  There is some variation in the quality and

adjustment of screens, so you may need to adjust the colours to provide

adequate contrast between the background layer and the various other layers

you might be using.

SETUP LAYERS

Top Layer             :       3

1 Mid Layer           :       Off

2 Mid Layer           :       Off

3 Mid Layer           :       Off

4 Mid Layer           :       Off

Bottom Layer          :       2

Overlay               :       1

Ground Plane          :       Off

Power Plane           :       Off

Board Layer           :       Off

Multi-Layer           :       3

Background            :       0

Highlight             :       2

Palette Number        :

The number of colours available for each layer varies depending on the

type of monitor you are using.  If you have a EGA screen there are sixteen

colours number 0 to 15.  If you have a CGA screen there are only four colours

numbered 0 to 3.  If a layer has been turned "off" in SETUP TOGGLE LAYERS,

the word "OFF" appears next to the layer in the menu.  You cannot assign a

colour to a layer which is turned off.

The layers are explained in more detail in the section called "Turning

layers on and off".

If you are using a CGA monitor, you can change the colours used for the

display by choosing PALETTE NUMBER from this menu.  The option is inoperative

if you are using an EGA monitor.

Move the selection bar to the layer you want to change then click <<LEFT

MOUSE>> or press <<ENTER>> until the colour you want is displayed.

Exit from this menu by pressing <<ESC>> or clicking <<RIGHT MOUSE>>.

SELECTING MENU COLOURS

You can also change the colours in the pull-down menus.  The background

colour, the colour of the text and lines, and the colour of the shadow around

the menu can all be altered.

SETUP MENU COLOURS

Background            :       0

Text                  :       1

Lines                 :       1

Shadow                :       1

Choose SETUP MENU COLOURS.  The colours can then be changed in the same

way as for the layer colours.  Move the selection bar to the one you want to

change, then click <<LEFT MOUSE>> or press <<ENTER>> until the required

colour is displayed.

Be careful to ensure that you do not choose a colour for the menu which

is the same as the background layer colour.  PROTEL-EASYTRAX does not do any

cross-checking with the layer colours to stop this happening.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

ASSIGNING FUNCTIONS TO KEYS

PROTEL-EASYTRAX allows you to assign sequences of commonly used

keystrokes to function keys to speed up operations.  If you regularly repeat

a sequence of keystrokes, such as when you are placing pads, then they can

be defined as a macro and assigned to a particular function key.

Once assigned, the macro can be invoked simply by pressing one key or

a simple key combination such as <<SHIFT>> or <<CTRL>> or <<ALT>> followed

by another key.

In the menus, a function key labelled F1 is designated by <F1>.

Pressing the <<SHIFT>>, <<ALT>>, and <<CTRL>> keys while holding down the <F1>

key is designated by <SHIFT-F1>, <ALT-1>, and <^F1> respectively.

PROTEL-EASYTRAX is supplied with a default set of macros which you may

want to use sometimes.

KEYS

Assign

Clear

Delete

Edit

List

Load

Merge

New

Record

Rename

Save

There are a number of menu options available to help you define your own

macros.  Each of these options is now explained briefly although the Tutorial

Assigning Key Macros contains a detailed tutorial explaining how to define

macros.

ASSIGN

Choose SETUP KEYS ASSIGN to reassign a macro from one function key to

another.  You may need to do this if you find you are using a macro often,

yet have assigned it to a two-key combination such as <<CTRL>> <<F5>>.

You are asked to select the macro by name or by key.  When you choose one

of these options a window opens listing either the names of all macros

currently assigned to keys or the keys which currently have macros assigned

to them.  Select the macro to be reassigned.  An input window opens asking

you to select the function key to which the macro is to be assigned.

Press the function key or function key combination that you want to

assign the macro to.

You are returned to the selection window.  If you have finished assigning

macros, press <<ESC>> or click <<RIGHT MOUSE>> to return to the KEYS menu.

CLEAR

Choose the SETUP KEYS CLEAR option to clear all macros from the memory.

Although macros are cleared from the memory, the disk file is not affected.

You are asked to confirm that you want to clear all key macros.  Choose

Yes to clear the macros and No to leave them intact.

DELETE

Choose DELETE to delete one or more macros from the memory.  The macros

are not deleted from the .KEY file unless you choose to save the new set of

macros.

You are asked to select by name or by key.  Once you have selected a

macro that you want to delete, you are asked to confirm the deletion.  Choose

Yes to proceed and No to stop the macro being deleted.

EDIT

Once you have defined a macro you can add to it or delete  parts of it.

Select the macro that you want to change and an edit window opens.  The name

of the macro and the function key it is assigned to is displayed at the top

of this window.

The window displays the representations of each of the keystrokes that

comprise the macro.  If you press the <<BACKSPACE>> key you will delete items

in the macro.  If you press any other key (including a function key containing

a macro), you add that keystroke to the macro.

Press <<ENTER>> or click <<LEFT MOUSE>> to finish editing and return to

the KEYS menu.

LIST

You can list the current macros by name or by key.  The macro names or

key names are displayed in an information window.  Press <<ENTER>> or click

<<LEFT MOUSE>> to return to the KEYS menu.

LOAD

The LOAD option allows you to load a key macro file from the disk.  Any

macros which are currently in the memory are overwritten.  Remember that the

file called EASYEDIT.KEY is loaded when you start PROTEL-EASYTRAX.

Choose this option by moving the selection bar to highlight the word

"Load" then pressing <<ENTER>> or clicking <<LEFT MOUSE>>.  The reason is

that the LIST option also begins with L and is before LOAD in the list.

An input window opens with the default value of "*.KEY".  To load a new

key file either type in the name of the file, or press <<ENTER>> or click

<<LEFT MOUSE>> to see a list of key files.

Select a file then a window opens asking you to confirm that you want to

load it.  Choose YES to load and NO to return to the KEY menu without loading

the file.

MERGE

This option allows you to merge a macro from another file.  You are

asked to nominate the file to merge from, then you are asked to name the

macro you want to merge.  The new macro is added to the macros currently in

memory.

NEW

There are two methods you can use to define a new key macro.  The first

is by choosing SETUP KEYS NEW.  An input window opens and you can enter the

name you want to give the new key macro.  Another window then opens asking

you to choose the key to which the macro will be assigned.  A default value

(which is the next available key) appears.  If the default value is not

acceptable, press another function key combination.

A macro edit window then opens allowing you to type in the keystrokes

that you want to assign.  If you want to include keystrokes such as <<ESC>>

and <<ENTER>>, press the <<SCROLL LOCK>> key to switch to "literal mode",

then press the required key.  Press the <<SCROLL LOCK>> key again to change

back to "command mode".  Press <<ENTER>> or click <<LEFT MOUSE>> to exit from

this window.  You are then prompted to enter another new key macro name.  If

you have finished defining macros, press <<ESC>> or click <<RIGHT MOUSE>>.

RECORD

The second method of defining a key macro is by recording it.  This

option allows you to work through the macro while seeing the effect each

keystroke has on the board.  Mouse movements are not recorded.

Choose SETUP KEYS RECORD.  You are prompted to supply a name and a key

as in the NEW option.  You then hear a beep and you are returned to the

board where you can then simulate the set of keystrokes that you want to store

in the macro.  When you have completed the keystrokes press <<CTRL>> <<END>>

to turn the "recorder" off.  You are then returned to the KEYS menu.

RENAME

Choose SETUP KEYS RENAME to change the name of a macro, rather than the

key it is assigned to.  You are prompted to supply the name which you want to

change and then the name you want to change it to .  If the name you try to

change to already exists, the message "KEY MACRO NAME ALREADY USED" is

displayed.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

SAVE

Once you have changed some macros or created one or more new macros you

will want to save them in a file so that you can use them again.  You can

either save them under the default name of EASYEDIT.KEY or choose a name of

your own.

If you try to save them under a name that already exists, the warning:

KEY MACRO FILE EXISTS.OVERWRITE?

is displayed.  Choose YES to delete the existing macro file.  Otherwise choose

NO.  You can then enter another name.

Press <<ESC>> or click <<RIGHT MOUSE>> to exit from this option.

SETTING OPTIONS

SETUP OPTIONS

Auto Pan              :       On

Backup                :       30

Cursor Type           :       Small Cross

Drag                  :       All Tracks

Ortho Mode            :       On

Via Mode Auto         :       Off

Flash Cursor          :       Off

Question Delete       :       On

The SETUP OPTIONS menu allows a number of important features of the

program to be changed.

The AUTO PAN option can either be "on" or "off".  If it is "on" then

moving the cursor to the edge of the screen moves you to a new section of the

board.  This is a nice feature if you have a powerful microcomputer which can

redraw the screen quickly.

Sometimes, however, you may want to do a lot of work on one section of

the board.  Turning AUTO PAN "off" means that bumping the mouse or inadvertently

moving the mouse to the edge of the screen does not cause it to be

redrawn.

The BACKUP feature is vital if you want to avoid losing important work.

It is set to do an automatic backup every 30 minutes.  If you are working with

a small board you may wish to decrease this to, say, 10 minutes.

Choose SETUP OPTIONS BACKUP.  An input window opens.  The default name

of:

EASYEDIT.ABK

is displayed.  If you want to change the name which is assigned to the backup

file, then enter it in this window.  The current time between backups is also

displayed.  Type in the new value, then click <<LEFT MOUSE>> or press <<ENTER>>

to close the window.

If you lose your work you can rename the backup file with the DOS RENAME

command.  If the backup file is called EASYEDIT.ABK and is stored in the

\FILES directory and you need to rename it to be, say, BOARD.PCB, type the

following command at the DOS prompt:

RENAME\FILES\EASYEDIT.ABK\FILES\BOARD.PCB

The CURSOR TYPE option lets you change the cursor from a small cross to

a large cross or to an arrow.  Many users prefer the arrow because it is

easier to locate on a board with many components and tracks.  When you choose

this option a menu appears with the three choices.  Move the selection bar to

the one you require and click <<LEFT MOUSE>> or press <<ENTER>>.

The DRAG option lets you choose what happens to tracks when a component

is moved from its original position on the board.  A menu provides three

options.

You can decide that all tracks should be affected, no tracks at all or

else the final option causes a question to be asked each time so that you can

make a decision based on the type of component and the number of tracks that

are likely to be involved.

PROTEL-EASYTRAX normally allows you to produce tracks in orthogonal

mode.  This means that they can only be laid horizontally, vertically, or at

45 degrees.  Such an approach is in keeping with good practice in the

industry.

Nevertheless, if you decide to do so, the ORTHO MODE option allows you

to turn this option "off" and hence provide you with the ability to lay

tracks at any angle.

The VIA MODE AUTO option provides for automatic via placement when

placing tracks.  The options are "on" and "off".  If the option is "on", then

when you lay a track and switch to another layer, then lay another track, a

via is automatically placed.  If the option is set to "off", then you must

place your own vias.

The FLASH CURSOR option allows you to have the cursor flashing (by

setting it to "on") or not flashing (by setting it to "off").  The default

value is "Off".  If you "lose" the cursor on the board, turning the FLASH

CURSOR option "On" makes it easier to locate it.

The QUESTION DELETE option allows you to decide whether you want

PROTEL-EASYTRAX to ask you for confirmation before you delete anything on the

board or not.  The default value it "ON".

CHOOSING THE REDRAW QUALITY

SETUP REDRAW allows you to choose the quality of each redraw for arcs,

fills, pads, strings, tracks, and vias.  Each one can be set to either draft

or final.

If, for example, you decide to set the REDRAW quality to "Draft" for

tracks and pads, then only the outline of these items is redrawn.  If you

set it to "Final", they are "filled in" when they are drawn.

SETTING STRING DEFAULTS

SETUP STRINGS allows you to change the default size of strings which are

placed on the board.  The term string refers to the characters that are used

as comments and component designators as well as the free text that can be

placed on a board.

The default value for each is 60 mils, but it is possible to change each

by selecting from the list provided when you choose any of these options.

TURNING LAYERS ON AND OFF

TOGGLE LAYER DISPLAY

Top Layer             :       On

1 Mid Layer           :       Off

2 Mid Layer           :       Off

3 Mid Layer           :       Off

4 Mid Layer           :       Off

Bottom Layer          :       On

Overlay               :       On

Ground Plane          :       Off

Power Plane           :       Off

Board Layer           :       Off

Multi-Layer           :       On

The SETUP TOGGLE LAYERS option allows you to decide whether a layer is

displayed or not.  As there are 11 choices available it is not a good idea to

have them all turned on at the same time.  Simply select the layer and change

it from "on" to "off" or "off" to "on".

A brief description of each of the layers is provided below.

TOP LAYER

This layer is sometimes known as the component side and is the layer on

which you place components and tracks which cannot be routed on the bottom

layer.

MID LAYER 1,2,3,4

These are signal layers used in multi-layer boards.  You can route tracks

on these layers.

BOTTOM LAYER

This layer is sometimes known as the solder side.  It is used for routing

tracks.

OVERLAY

This layer is often used for silk screening so that text can be overlayed

on the board.

GROUND PLANE

The ground plane is the plane which is used to shield sensitive sections

of the circuit and to minimize radiated energy.  It is a separate plane of

copper sandwiched between the other layers.  There are holes in the copper

wherever leads pass through to other layers so that they do not contact with

the copper.

POWER PLANE

The power plane is very similar to the ground plane.  It is used when

large currents are needed.

BOARD LAYER

The board layer can be used, for example, when you want to show the

edges of the board in a plot.  Simply draw the board outline on this layer

then ask for it to be plotted.

MULTI-LAYER

With this layer you can define the colour for multi-layer items such as

pads and vias.

USING THE UN-DELETE OPTION

The UN-DELETE option is your safeguard against accidental deletion of

items you have placed.

When you choose UN-DELETE, a window opens asking you to specify the

"UN-DELETE COUNT".  This is the number of items you have deleted that you

want to restore to the board.

UN-DELETE COUNT

1_______

PROTEL-EASYTRAX starts restoring deleted items, starting from the most

recently deleted item.

A SIMPLE EXAMPLE

You may gain a clearer understanding of how this option operates by

working through the following example.

Move to a blank area on the board and choose PLACE COMPONENT.  Place, in

order, a DIP4, DIP6, DIP8, DIP16, and DIP40.  Arrange them so that they are

side by side on the board.  Don't worry about the designators or comments.

Now, exit the PLACE option and choose DELETE COMPONENT.  Delete each of

the components you have just placed starting with the DIP40 pattern and

working back to the DIP4 pattern.

Now choose UN-DELETE.  Input the value 2 for the "UN-DELETE COUNT" and

press <<ENTER>> or click <<LEFT MOUSE>>.

You will see that the DIP4 and DIP6 patterns have been restored because

these were the two that were deleted most recently.

Now choose UN-DELETE again.  This time specify an "UN-DELETE COUNT" of 1.

The DIP8 pattern will now be restored.

The "stack" of deleted items is capable of storing up to 5000 items.

EXPANDING AND CONTRACTING THE DISPLAY

The ZOOM option in the MAIN MENU can be used to expand and contract the

display.  Sometimes you will want to view the entire board (e.g. when you are

highlighting nets) and other times you will need to magnify a section to make

minor placement adjustments (e.g. accurate placing of pads).

Note that this menu can be selected within some operations such as

PLACE TRACK and PLACE COMPONENT.

ZOOM

Redraw

Pan

Expand

Contract

All

Select

Window

REDRAWING THE SCREEN

Choose ZOOM REDRAW to redraw the current screen at the current

magnification.  This is useful if you have just deleted, say, some tracks or

components because some parts of the display may be missing.

An alternative is to press the <<END>> key at any stage.

MOVING AROUND THE BOARD

If you have the AUTO PAN option set to "on", then every time the cursor

reaches the edge of the screen it "pans" to the next screen.

Choosing ZOOM PAN makes the current cursor position the center of the

screen.  This particularly useful if you have turned the auto pan option "off"

by choosing SETUP OPTIONS AUTO PAN OFF.

EXPANDING THE DISPLAY

If you need to work at a higher magnification, then choose ZOOM EXPAND.

The display will be momentarily blanked and then redrawn showing more detail

of the features on the board.  The expanded display is centered around the

current cursor position unless it is on the very edge of the board.

Repeat the process and you will notice that the display is expanded

further.  There are six zoom levels available.  At the lowest level one

screen pixel represents 100 mils whereas at the highest level one pixel

represents 1 mil.

Note that pressing the <<PGUP>> key zooms the display up one level.

Pressing <<PGUP>> a number of times quickly causes it to zoom straight to the

higher level without drawing the intermediate levels.

When the highest level of magnification has been reached, pressing

<<PGUP>> or choosing EXPAND no longer has any effect on the screen.

CONTRACTING THE DISPLAY

This option provides the opposite facility to the EXPAND option.  Each

time CONTRACT is chosen the screen is redrawn but contracted one level.

Note that pressing the <<PGDN>> key also contracts the display one level,

and pressing it a number of times quickly has the same effect as with the

<<PGUP>> key.

FITTING THE BOARD ON THE SCREEN

Choose ZOOM ALL to redraw the display so that the entire board can be

seen on the screen.

If the board size has been changed since you started the editing session

then you should first choose INFORMATION BOARD so that PROTEL-EASYTRAX can

work out the scaling necessary for it to fit exactly onto the screen.

SELECTING A ZOOM VALUE

ZOOM SELECT allows you to select from one of the seven pre-set zoom

values.

ZOOM SELECT

1

3

5

10

20

50

100

Move the selection bar to the magnification you require and click <<LEFT

MOUSE>> or press <<ENTER>>.  The screen is redrawn at the zoom level you

selected.

SPECIFYING A ZOOM WINDOW

If you want to inspect a particular section of the board in more detail,

choose the ZOOM WINDOW option.

The prompt:

SELECT FIRST ZOOM WINDOW CORNER

indicates that you should select the first corner of the window.  Click the

<<LEFT MOUSE>> or press <<ENTER>> when the cursor is at the corner of the

area you want to select.  Move the cursor to the opposite corner of the

rectangular area you want to inspect.  A box appears outlining the window.

The prompt now reads:

SELECT SECOND ZOOM WINDOW CORNER

Click the <<LEFT MOUSE>> or press <<ENTER>> to select the second corner.

The window now zooms to fill the screen.

USING THE EASYPLOT PROGRAM

The EASYPLOT program has been designed to allow you to produce quality

artwork using a convenient pen plotter or photoplotter and to produce draft

quality check plots.

One of the features of this program is that the external protection

device is not required.  This allows you to use the program on a low-cost PC

connected to a printer or plotter, thus freeing up more expensive equipment

for the task of designing printed circuit boards.

Remember that there are three tutorials at the front of this manual which

explain how to plot a board using a step-by-step approach.

STARTING THE EASYPLOT PROGRAM

If you using a hard disk drive, you should make sure that you are in a

directory from which the EASYPLOT program is accessible.

Type:  EASYPLOT then press <<ENTER>>.

When the program is activated, a menu called the MAIN PLOT MENU appears

in the top left hand corner of the screen and a status window appears towards

the bottom of the screen.

The menu is similar to the MAIN MENU in the PROTEL-EASYTRAX program.

The status window displays the current status of some parameters relating to

the plot.

The MAIN PLOT MENU displays a number of options which you need to set up

a plot.   Each of these are now discussed in detail.

MAIN PLOT MENU

File

Information

Options

Setup

Plot

Print

Gerber Plot

To select an option from the MAIN PLOT MENU, move the selection bar to

highlight the option you require, then press <<ENTER>> or click <<LEFT MOUSE>>.

THE FILE MENU

When you select FILE, the following menu is displayed.

FILE

Dos

Load

Path

Files

Quit

There are five options which you can select from the FILE menu.  These

options allow you to manipulate files, inspect lists of filenames or exit to

DOS.

THE DOS OPTION

This option is identical to the DOS option in the PROTEL-EASYTRAX

program.  It allows you to exit to DOS to carry out "housekeeping" operations

such a copying files, deleting files, creating directories and making backups.

When you choose DOS, you are asked to confirm the fact that you want to

exit to DOS.  If you choose YES, you can then carry out all the normal DOS

functions.

The same restrictions as those mentioned in the PROTEL-EASYTRAX program

apply.

When you are ready to continue using the EASYPLOT program, simply type:

EXIT and press <<ENTER>>

at the DOS prompt and you will be returned to the program.

LOADING A FILE

The LOAD option allows you to load the .PCB file that you want to print

or plot.  When you choose the option, an input window opens asking you to

enter the name of the file you want to load.  Enter the name then press

<<ENTER>> or click <<LEFT MOUSE>>.

The default value of *.PCB in the window indicates that if you do not

remember the name of the file you can press <<ENTER>> to call up the

information window which lists the names of all the .PCB files in the

currently selected directory.

When you choose a file and the load begins, a load window opens showing

the items being loaded into memory.

LOADING PCB FILE

Checking

-----------------

Arcs                  15

Fills                  0

Pads                 435

Tracks              1554

Strings               78

Vias                  85

Components            36

------------------

2203

PRESS ANY KEY TO ABORT

This window gives you an indication of the number of components and other

items that will be plotted or printed.

When the file has been loaded, the information in the status window is

updated to show the name of the new file.

Path:           C:\FILES

Layout File:    \FILES\DEMO

Layout Size:    5900x5900

Free Memory:    114654

If there are no files available for printing or plotting, the message:

NO FILES FOUND

is displayed.

CHANGING THE PATH NAME

If you choose PATH, you can change the path to the directory containing

the .PCB files you want to print or plot.

An input window opens asking for the name of the new path.  Type in the

path name, then press <<ENTER>> or click <<LEFT MOUSE>>.

If the path doesn't exist then the error "PATH NOT FOUND" is displayed.

Once you have specified a path, this will automatically be appended to

the front of any file specification that you enter in the LOAD option.

LISTING FILES

When you choose the FILES option, a window opens asking you to enter a

mask for the filenames that you want to list.  If you enter:

*.*

then all the files in the currently selected directory are displayed.  If you

enter something like:

*.PCB

then only the .PCB files are listed.

QUITTING

This option allows you to quit the program.  Choose QUIT and a window

opens asking you to confirm the fact that you want to quit to DOS.  If you

choose NO then you can continue using the program.  If you choose YES, you

are returned to DOS.

DISPLAYING INFORMATION ABOUT THE BOARD

If you choose INFORMATION, a window which displays information relating

to the plot or print is displayed.  The information in the window is shown

below.

INFORMATION

Plot Type               :       Check Plot

Plot Scale              :             2.00

Print Scale             :             1.00

Holes in Layout         :              520

LowX                    :               25

LowY                    :               25

HighX                   :             5925

HighY                   :             5025

PLOT TYPE shows the currently selected plot type such as Ground Plane or

Overlay or Top Layer.  It could also be a Check Plot in which all the

currently selected layers are plotted.  The type of plot is chosen through

OPTIONS TYPE OF PLOT.

PLOT SCALE and PRINT SCALE show the scaling factor to be used in the

print or plot.  The default value for each is 1.  If the scale is changed to

2, then the board is plotted at twice the size.  The scaling factors can be

changed by choosing PRINTER SETUP SCALE or PLOTTER SETUP SCALE.

On the next line, the number of holes in the board is displayed.  This

value is very useful because it tells you how many holes need to be drilled.

Remember that the greater the number of holes, the more expensive the board

is to manufacture.  If the number is higher than you want, then you may need

to redesign the board or change the way you have routed the tracks to reduce

the number of vias.

The final values define the X and Y coordinates of the bottom left hand

corner and the top right hand corner of the board.  These values give you an

exact measurement of the size of the board to be plotted or printed.

Notice that you cannot alter any of the values in the INFORMATION screen.

Pressing any key our mouse button will return you to the MAIN PLOT MENU.

The information displayed in this screen is also displayed when plotting

or printing commences.

CHOOSING THE PLOT OPTIONS

There are a number of options that you can choose to affect the plot or

print.  Choose OPTION from the MAIN PLOT MENU and the following menu appears.

SETUP OPTIONS

Type of Plot          :       Check Plot

Board Layer           :               On

Pads                  :               On

Vias                  :               On

Strings               :               On

Title Block           :              Off

Pad Hole Guide Size   :                0

Solder Mask Enlargement:               0

PWR/GND Enlargement   :                0

When you choose the TYPE OF PLOT option a window opens which lists the

fourteen plot options.

SELECT PLOT TYPE

Check Plot                    Top Layer

1 Mid Layer                   2 Mid Layer

3 Mid Layer                   4 Mid Layer

Bottom Layer                  Overlay

Ground Plane                  Power Plane

Top Solder Mask               Bottom Solder Mask

Pad Master                    Drill Guide

The CHECK PLOT option allows you to produce a multi-layer check plot or

check print.  If you use a single pen plotter, you may be asked to change the

pen after each layer has been plotted.  This could happen if you have assigned

different pens to different layers in CHECK PLOT PENS, to ensure that each

layer is plotted in a different colour.

You can choose to print or plot each of the single layers and the Overlay.

You can choose to print or plot the ground plane, but remember that since

these planes consist mainly of copper, they will appear in the negative.  You

can also choose to plot solder masks or Pad Master from this menu.

When you choose any of the options from this menu you will notice that

the information is updated in the status window.

The next four options in the OPTIONS menu allow you to decide whether

the border, pads, strings, and title block will be printed or not.  Choose

the option you want to change and switch it from "On" to "Off" or vice versa.

You might, for example, decide to turn pads "On" when plotting the Top

Layer, but "Off" for all other layers.  This is because most pads belong to

every layer, hence only need to be plotted once.  You can save valuable

plotting time by adopting this procedure.

The pad hole size for the plot is automatically set to zero.  If you want

to change this value choose OPTIONS PAD HOLE GUIDE SIZE.  An input window

opens and you can type in the value you require then press <<ENTER>> or click

<<LEFT MOUSE>>.

You can also specify an enlargement factor for the solder mask and the

holes for the power and ground planes in the same way.

THE SETUP OPTION

The SETUP option allows you to choose many of the characteristics of the

plot you want to undertake.

SETUP

Serial Ports

Check Plot

Plotter

Printer

Gerber

Pens

Menu Colours

SERIAL PORTS

When you choose SETUP SERIAL PORTS, a window opens allowing you to choose

one of two serial ports.  Choose either port 1 or port 2 depending on which

one your plotter is connected to, then press <<ENTER>> or click <<LEFT

MOUSE>>.

SERIAL PORT SETUP

Serial Port 1

Serial Port 2

You can now change some of the serial port parameters such as the baud

rate, the number of data bits, the type of handshaking, the parity type and

the number of stop bits.

SERIAL PORT 1 SETUP

Baud                  :               2400

Data Bits             :                  8

Handshake             :           Hardwire

Parity                :               None

Stop Bits             :                  1

There is a detailed description of these features in Appendix B of this

manual.

Press <<ESC>> or click <<RIGHT MOUSE>> twice to return to the SETUP

menu.

CHECK PLOT

If you decide to produce a Check Plot, all layers which are currently

turned on are plotted.  If you produce a "Draft" Check Plot, then only an

outline of the tracks appears.  This means that you can see any pads which

lie underneath tracks on the plot.

Choose SETUP CHECK PLOT to select the layers you want to be output in the

check plot.

A window listing each of the layers opens.  You can then toggle each

layer "On" or "Off" as required.  If you are designing a simple two-sided

board you should turn the following layers on:

Top Layer

Bottom Layer

Overlay

Multi Layer Pads

If you do not turn on the last of these options, the pads (which belong

to every layer) are not plotted.

PLOTTER SETUP

When you choose the PLOTTER option, a window opens allowing you to change

a number of parameters relating to the plot.

PLOTTER SETUP

Type                  :               DMPL

Device                :       Serial Port 1

Scale                 :              1.000

X Offset              :              0.000

Y Offset              :              0.000

X Correction          :              1.000

Y Correction          :              1.000

Orientation           :             Normal

Quality               :              Draft

Software Arc          :                 On

Arc Quality           :                  5

Options               :

Pens                  :

If you choose TYPE, the message:

Searching for plotter drivers

is displayed briefly, then a window opens listing the types of plotter for

which drivers are available.  Select one by highlighting it then pressing

<<ENTER>> or clicking <<LEFT MOUSE>>.

Choose DEVICE to select a serial port, parallel port or an output file

as the device to which the output will be sent.

The SCALE option lets you enlarge or reduce a plot and to correct for

scaling errors inherent in your plotter.

If you are producing final artwork, you may want the plots to be twice

the size of the normal output.  To do this, change the SCALE to 2.  If you

want to reduce the artwork to half the size, change the SCALE factor to 0.5.

The X and Y offsets are used to start the plot at a point other than the

normal origin.  This may be because you want to plot more than one board on

the same piece of film, for example.  Enter a new value for both the X and

Y offsets and then press <<ENTER>> or click <<LEFT MOUSE>>.

The correction values change the scaling in either the X direction or

the Y direction.  You may need to use these correction factors if your plotter

is inaccurate in either the X or Y direction.  To check for errors, draw a

vertical and horizontal track of exactly the same length.  When you have

plotted the tracks, measure them carefully.  If they are not the same you

need to correct either the X or the Y value.

Choose ORIENTATION to select "Normal" or "Rotated" for the final plot.

Choose QUALITY to select "Draft" or "Final" for the plot.  If you choose

"Draft" then tracks only appear in outline.

Some plotters have the ability to draw an arc if the parameters are

supplied.  If the plotter you are using cannot draw arcs, then the plot

program must ask the plotter to draw a number of short line segments.  First,

you must set the SOFTWARE ARC option to "On".  The ARC QUALITY option allows

you to set the length of the line segment which is used to draw the arc.  A

value of 5 mils is a reasonable compromise for most jobs.  The greater the

number of line segments used to draw the arc, the longer it takes.

PRINTER SETUP

The procedure for setting up a printer is similar to the procedure for

setting up a plotter.

You should first select TYPE to choose the printer driver.  You can

choose from Epson, Quiet Jet, or HP Laser drivers.  With the HP laser printer

you can choose a resolution of 100, 150, or 300 dots per inch.  It is

recommended that you only use the 300 dots per inch resolution for final

prints.

The ORIENTATION option has the extra option of AUTO CENTER.  This

provides for a print to be centered on the printed pages.  Therefore, if the

print takes four pages, the output appears in the appropriate corners of each

of the four pages so that they form one image when placed together.  Note

that this option will override any offsets which have been set up.

Another variation is that you can choose between continuous paper and

single sheet paper.

You may have to use the X & Y CORRECTION options if you are using a

printer with slightly different characteristics to the Epson FX series.

The Epson FX series has a vertical step size of 1/72 inch.   The Epson

LQ series, however, has a vertical step size of 1/60 inch.  This means that

720 dots is 10 inches ling on an FX series printer and 12 inches long on an

LQ series printer.  The output will, therefore, be longer in the X direction

on an LQ series printer.  To make it the same distance in the X direction

it must be reduced by a factor of 1.2.  Therefore, you will need to make the

X-correction factor 0.833.

If you send the output to a file, the message:

PRINT FILE GENERATED

is displayed.  Press <<ENTER>> or click <<LEFT MOUSE>> and you are returned

to the MAIN PLOT MENU.

GERBER PLOTTER SETUP

This option allows you to set up the parameters for photoplotter output.

The information is sent to a file since it is likely that you will take a

disk to a photoplotter bureau to produce the final artwork.  Before producing

a Gerber file you should read the manual associated with the photoplotter to

learn how to set up the required parameters.

GERBER SETUP

Output File           :       \FILES\DEMO

X Offset              :             0.000

Y Offset              :             0.000

Aperture Table         :           RCS.APT

Match Tolerance       :                5%

G54                   :               Off

Arc Quality           :                 5

Options               :

The X and Y Offsets can be used in exactly the same way as they are for

a normal plot.  By changing these offsets you can choose to start plotting

at a different spot on the film.  This allows more than one plot to be

performed on one film, thus saving on the cost of film.

Next you should choose the Aperture Table which matches that required by

the photoplotter you are going to use.  While most photoplotters use the

Gerber standard for selecting apertures and for moving to a particular spot

on the film, they vary considerably in the apertures which are available.  To

overcome this problem, there are a number of standard Aperture Tables supplied

with PROTEL-EASYTRAX.  Select the aperture table you require by choosing

SETUP GERBER APERTURE TABLE.  A list of the available aperture tables is

displayed.  Highlight the one you require then press <<ENTER>> or click

<<LEFT MOUSE>>.  The name is now displayed in the GERBER SETUP menu.

Next you should set the MATCH TOLERANCE you require.  An example might

help here.  If you have a large number of circular 62 mil pads on your board,

these can be plotted quickly by selecting a 62 mil aperture and then "flashing"

a light onto the film.

But what happens if the photoplotter does not have a 62 mil aperture?

Some have a 60 mil round aperture and for most plotting tasks you are likely

to require, this is close enough.  The MATCH TOLERANCE is the figure you set

to say what variation in area you can tolerate.  If the program is able to

match an aperture with a pad which is within the tolerance you set, then it

does so automatically.

The area of a 62 mil pad is 3.1415 x 31 = 3019 square mils.  The area

"flashed" by a 60 mil aperture is 3.1415 x 30 = 2827 square mils.  The

percentage variation is calculated as (3019 - 2827) x 100/3019 = 6.35.

Therefore, if you set the MATCH TOLERANCE to 8%, say, the 60 mil aperture

is selected automatically.

Don't set the tolerance too high (e.g. 30%) otherwise there may be

problems fitting tracks between pads in some cases.

Some older Gerber plotters need to be sent the code G54 before an

aperture change.  Thus, if the aperture D10 is to be selected the software must

send a G54 code then a D10 code.  If the plotter requires this code to be

sent, set the G54 option to "On".  If it does not need this code, set it to

"Off".

Some plotters have the ability to draw an arc if the parameters are

supplied.  This is not the case with some Gerber photoplotters.  In order to

draw an arc, the plot program must ask the photoplotter to draw a number of

short line segments.  The greater the number of line segments used to draw

the arc, the longer it takes.  The ARC QUALITY option allows you to set the

length of the line segment which is used to draw the arc.  A value of 5 mils

is a reasonable compromise for most jobs.

SETTING THE PEN PARAMETERS

Choosing SETUP PENS allows you to set the speed at which the pens are to

be driven and the size of pen that you have in the plotter.  When you choose

the option the following menu is displayed.

SETUP PENS

Check Plot Pens

Plotter Pens

If you are doing a Check Plot, the CHECK PLOT PENS options allows you to

allocate different pens to different layers as well as specifying the size of

each pen and setting the pen speed.  In the PLOTTER PENS option you can choose

which pen is to be regarded as the main pen as well as specifying the size of

each pen and setting the pen speed.

HINT

You can set the pen speed to a high value, but the ink in the pen won't

run quickly enough to form a clear line on the paper.  If you set the value

too low, then you may get ink "blotches" on the paper - it also takes a long

time to plot.  Experiment with different speeds until you are happy with the

result.

To input the pen speed, highlight the PEN SPEED option and press <<ENTER>>

or click <<LEFT MOUSE>>.  Enter the value in the input window, then press

<<ENTER>> or click <<LEFT MOUSE>>.  You are returned to the CHECK PLOT PENS

or the PLOTTER PENS menu.

To specify the pen size, choose PEN SIZES from either menu.  Enter the

value of the diameter of the pen in mils and then press <<ENTER>> or click

<<LEFT MOUSE>>.  You are returned to the CHECK PLOT PENS or the PLOTTER PENS

menu.  This is vital so that the printer driver can draw the tracks, arcs,

vias, and pads accurately.  It is obvious that is you have a 26 mil diameter

pen then it is impossible to draw a 12 mil track at normal scale!  If you do

not set the pen size, it defaults to 26 mils.

Press <<ESC>> or click <<RIGHT MOUSE>> four times to return to the

MAIN PLOT MENU.

CHANGING THE MENU COLOURS

Each of the menu colours can be changed by choosing SETUP MENU COLOURS.

One nice feature of the program is that you can choose different colours for

different levels of menu.  Thus you can choose one set of colours for the

main menu another set for, say, the SETUP menu, another for the PLOTTER

SETUP menu and another for the PRINTER DEVICE menu.

Notice that after setting up the menu colours for the first level, you

can copy the previous setup down to the next level of menu.

PRINTING OR PLOTTING A PCB

The final action you need to take is the actual print or plot.  Make sure

that your printer or plotter is turned on and is online.  Choose PLOT from

the MAIN MENU after making sure that all SETUP options are correct and that

you have loaded a file.  You can check all this in the status window.

The program checks the file and assembles the information relating to

the layer or layers to be output.  The information window will keep you

informed of the progress of the plot.

If the output does not appear within a few minutes, check that the

printer is online and plugged into the correct port.

A similar process occurs if you choose PRINT from the MAIN PLOT MENU.

If you have directed the output to a file, the extension given to the

file depends on the layer you are plotting.  The extensions are:

Check Plot                    .?CK

Top Layer                     .?TL

1 Mid Layer                   .?M1

2 Mid Layer                   .?M2

3 Mid Layer                   .?M3

4 Mid Layer                   .?M4

Bottom Layer                  .?BL

Overlay                       .?OV

Ground Plane                  .?GD

Power Plane                   .?PW

Top Solder Mask               .?TS

Bottom Solder Mask            .?BS

Pad Master                    .?PM

Drill Guide                   .?DG

where the question mark at the start of the extension is P for a plot file

and R for a print file.

CREATING A GERBER FILE

When you are ready to produce a Gerber file, return to the MAIN PLOT

MENU and select GERBER PLOT.  The plotting screen appears with the information

window on the right hand side of the screen.

You are asked to confirm that you want to proceed with the Gerber plot.

This gives you the opportunity to inspect the parameters you have chosen for

the plot.  If any of them are incorrect, select NO and change them.  If you

are ready to proceed, select YES.

The program now attempts to match every item on the board with an

aperture on the photoplotter that you are going to use.  For each item, it

looks for an aperture in the Aperture Table you selected.  If it cannot produce

a match within the tolerance you specified, it asks you to select from a

table of the closest apertures or enables you to step through the apertures

to manually select one.

You see a display similar to the one below.

MATCHING DRAFT CODES

ASSIGNING:                    Circular (28x28)

----------------------------------------------

Match Method                  Code  Shape Size

Flash Method:                  D19  Circular (40x40)

Stroke Method:                 D15  Circular (25x25)

Fill Match:                    D13  Circular (20x20)

A window then opens allowing you to select which method you want the

photoplotter to use to make the shape.

SELECT PLOT METHOD

Flash Method

Stroke Method

Fill Method

Manual Assignment

Make sure that you do not choose the stroke method to create a pad.  If

you attempt to, the message:

Cannot use line aperture for flash

is displayed.

It is a good idea to choose as few Fill Match options as possible because

these usually take longer to complete.

When all apertures have been matched, the message:

MATCH FILE GENERATED

appears indicating that a file has been created which matches each shape to be

generated with an aperture which is available on the chose photoplotter.

The match file shows the shapes which the program has tried to assign

apertures to and the aperture and exposure method which has been chosen-

either by the program or by you.  A sample from a match file is shown below.

ASSIGNING     :               Circular        (28 x 28)

STROKE        :   D15         Circular        (25 x 25)

ASSIGNING     :               Circular        (40 x 40)

FLASH         :   D19         Circular        (40 x 40)

ASSIGNING     :               Circular        (60 x 60)

FLASH         :   D28         Circular        (62 x 62)

Press any key and the final Gerber output file is then generated.  The

Gerber file in this case is assigned the name DEMO.GTL.  The extension

indicates that is a Gerber output file and that is a plot of the Top

Layer.

The extensions for the files produced by the Gerber plot are as follows:

Check Plot                    .GCK

Top Layer                     .GTL

1 Mid Layer                   .GM1

2 Mid Layer                   .GM2

3 Mid Layer                   .GM3

4 Mid Layer                   .GM4

Bottom Layer                  .GBL

Overlay                       .GOV

Ground Plane                  .GGD

Power Plane                   .GPP

Top Solder Mask               .GTS

Bottom Solder Mask            .GBS

Pad Master                    .GPM

Drill Guide                   .GDG

HINTS FOR PRODUCING GOOD PLOTS

There are a number of simple rules you can follow to make sure that your

plot is of a high quality.

Perhaps the most important factor is the quality of the paper and the

pens that you use.  You might be able to save money and use a slightly

cheaper type of paper or plotting pen, but the result is usually a poor

quality plot.  It is best to keep cheap paper and pens for the draft quality

plots, but to use the best pens and paper for the final plot.  Some of the

main factors in ensuring good quality output are now discussed.

Laser Printers

Take the laser printer for example.  PROTEL-EASYTRAX supports output to

an HP Laserjet printer.  You can produce output at 100, 150, or 300 dots per

inch.  For the draft plot it is a good idea to only use 150 dots per inch and

to use regular 80 gsm paper (the same paper that is used in photocopiers).

When you are sure that you are ready to produce the final output,

however, you should change to 300 dots per inch and also change to a clay-

coated paper.  This paper is slightly heavier and has a shiny surface.  Our

experiments have shown that it feeds through the printer accurately and the

toner does not "bleed" into the paper, hence making the outline sharper.

Most printers are able to supply clay-coated paper on request.

If you printer allows you to vary the density of the toner, turn it up a

little prior to carrying out the final plot, but remember to turn it down

when you have finished otherwise you will waste toner.

Plotter Pens

There are a wide range of plotting pens on the market.  Felt-tipped pens

are suitable for draft plots, but do not produce accurate final plots.

A pen which is suitable for final plots must be capable of providing a

consistent ink flow, must not dry out when the pen is lifted off the film

for short periods and must be of the correct diameter to draw tracks

accurately.

The pens that have been found to be the most suitable are those with

tungsten carbide, cross-grooved points.  Two brands which meet these

requirements are:

Staedtler 757 PL6 CS Plotter Point (for 2:1 plots)

Staedtler 757 PL3 CS Plotter Point (for 1:1 plots)

Koh-I-Noor 63VTB-2/.60 Plotter Point (2:1 plots)

Koh-I-Noor 63VTB-00/.30 Plotter Point (1:1 plots)

You should use a latex-based ink in these pens to make sure that you get

a dense plot without the ink running or drying out in the pen.  Two good

inks are:

Koh-I-Noor 3084-F RAPIDRAW Ink

Graphics Resources Intense Black Plotter Ink

Both of these inks have produced excellent results when used with the

correct plotting film with the pen speed set correctly.

Drafting Film

Your choice of drafting film is not as critical as the choice of ink and

pen.  Nevertheless, you should make sure that you buy a reasonable film.  Use

single-matte or double-matte polyester film of around 3 mil thickness.  A

recommended film is:

Dura-lene polyester matte drafting film

There are many other types of film, so talk to your local plotter

supplier about the type of film they recommend.

Choosing the Pen Speed

The pen speed is a critical factor which is often overlooked.  If you set

the pen speed to high, insufficient ink flows on to the paper resulting in

"streaky" tracks.  If you set it too low, too much ink may flow out thus

producing "blotches" on the film.

Try plotting a number of thick tracks side by side on a piece of film.

Make each track a different plot and plot each one at a different pen speed,

then decide which speed suits your plotter and pens the best.

GERBER PHOTOPLOTTER FILE FORMAT

Your local bureau may need to know the format of the Gerber file that

you supply.  For this reason, the format is now described in detail.

Each Gerber file is divided into blocks which are exactly 512 characters

long.  There are 510 characters of data followed by a carriage return

character then a line feed character.

Each block consists of a number of short Gerber records, each of which

is terminated by the character *.  A block contains as many complete records

as will fit into 510 characters.  The remaining characters in each block

(including the final block) are spaces (hex 20).

The records may refer to an absolute location or a draft code which

changes apertures.  Thus a record might be:

X800Y775*             which instructs the plotter to move to

a particular location;

or

D16*                  which instructs the plotter to select a

different aperture.

The particular draft codes vary according to the photoplotter being used,

but usually the following codes always have the same effect.  Some modern

plotters have a greater range of apertures available.

D01           Turn the light source on.

D02           Turn the light source off.

D03           Flash the light source.

D10-D19       Select apertures 1 - 10.

D70-D71       Select apertures 11 - 12.

D20-D29       Select apertures 13 - 22.

D72-D73       Select apertures 23 - 24.

On some older plotters the special code G54 needs to be sent before each

change of aperture code.

The last Gerber record is terminated by the special record M02*.  After

this block, there is one more block which contains the character hex 08, then

509 spaces, then a carriage return and a line feed.

The following extract from a Gerber file shows the first and last blocks.

Notice that on the first line, the draft code D16 selects a particular

aperture and then D01 and D02 codes are then used to turn the light on and

off at various locations on the film.  Because there are no D03 records in

the first block it is likely that the aperture is a line aperture used for

drawing tracks.

Notice also that the final record is the special code M02*.

X0Y375*D16*X0Y125*D01*X60Y125*D02*X80Y145*D01*X60Y125*D02*X8

0Y225*D01*X80Y145*D02*X60Y245*D01*X80Y225*D02*X0Y245*D01*X60

Y245*D02*X20Y245*D01*X360Y125*D02*

D03*X3075Y3950*D03*X3075Y4050*D03*X3075Y4150*D03*X3075Y4250*

D03*X5125Y1350*D03*X5125Y1250*D03*X5125Y1150*D03*X5125Y1050*

D03*X5575Y1750*D03*D02*M02*

The  last  Gerber record is terminated by the special record M02*.

After  this  block,  there  is  one  more block which contains the

character  hex  08,  then 509 spaces, then a carriage return and a

line feed.

A  Gerber  file  may  be inspected by displaying them with the DOS

TYPE  command.    The  file extensions for each of the permissible

plots is as follows:

Top Layer               .GTL

Mid Layer 1             .GM1

Mid Layer 2             .GM2

Mid Layer 3             .GM3

Mid Layer 4             .GM4

Bottom Layer            .GBL

Top Overlay             .GTO

Bottom Overlay          .GBO

Power Plane             .GPW

Ground Plane            .GGD

Top Solder Mask         .GTS

Bottom Solder Mask      .GBS

Top Paste Mask          .GTP

Bottom Paste Mask       .GBP

Pad Master              .GPM

Keep Out Layer          .GKO

Drill Drawing           .GDD

Drill Guide             .GDG

You can inspect any of the Gerber files by returning to DOS and displaying

them on the DOS TYPE command.

APERTURE FILE FORMAT

You might need to create your own Aperture File if the bureau you are

going to send the file to is unable to use one of the standard Aperture Files

supplied with PROTEL-EASYTRAX.

The following is an example of an Aperture File.  The first part of the

file contains an explanation of the file format.

; APERTURE FILE FORMAT

;

; <draft code> <shape> <xsize> <ysize> <hole size> <use>

;

; EXPLANATION OF PARAMETERS

;

; <draft code>   Code assigned to this aperture e.g. D14

;                Note that this code must start with 'D'

;                followed by the code number.

;

; <shape>        Can be any of the following shapes:

;                         CIRCULAR

;                         RECTANGULAR

;                         SQUARE

;                         OCTAGONAL

;                         ROUNDRECT

;                         CROSSHAIR

;                         MOIRE

;

; <xsize>        size of aperture in the x direction in

;                mils (thous).

;

; <ysize>        size of aperture in the y direction in

;                mils (thous).

;

; <hole size>    size of hole in aperture in mils (thous).

;                zero if no hole in aperture.

;

; <use>          specifies what the aperture can be used for.

;                There are three possible settings:

;                   LINE   can only be used to draw lines

;                   FLASH  can only be used to flash pads

;                   MULTI  can be used for either

;                   blank  defaults to MULTI setting

;

D11 CIRCULAR     40  40  0

D12 SQUARE       40  40  0

D13 CIRCULAR     10  10  0

D14 CIRCULAR     12  12  0

D15 CIRCULAR     15  15  0

D16 SQUARE       20  20  0

D17 CIRCULAR     72  72  0

D18 CIRCULAR     25  25  0

D19 CIRCULAR     30  30  0

D20 SQUARE       50  50  0

D25 CIRCULAR      8   8  0

D71 SQUARE       62  62  0

D72 CIRCULAR     62  62  0

D73 CIRCULAR    250 250  0

Notice that the final line refers to the draft code D73 which calls a

circular aperture which flashes a pad of diameter 250 mils.  A few lines

above it there is a circular aperture with draft code D25 with a diameter of

8 mils which is used for drawing lines.

CREATING AN APERTURE FILE

You can now create your own Aperture File using a word processing

package or a simple text editor.  Make sure that each line in the file has

the correct structure.  There must be six entries in each line, starting with

the draft code, although the last entry can be left blank.  These are:

<draft code> <shape> <x size> <y size> <hole size> <use>

If you are using a word processing package to create the file, make sure

that you write it to the disk as an ASCII (or text) file.  You must give the

filename the extension .APT for it to be recognized by EASYPLOT as an

Aperture File.

THE PLOT LAYERS

This section contains a brief description of each of the plots you can

request with the EASYPLOT program.  For each of the options there is a brief

description, as well as an example of the output which is generated using the

DEMO.PCB file which is supplied with the PROTEL-EASYTRAX.

Some of the important factors to be considered with each plot are also

mentioned briefly.

CHECK PLOT

A check plot shows all items on each of the layers that are currently

selected.  Select the layers you want plotted by choosing SETUP CHECK PLOT

and then switching the required layers "On".

If you have a multi-pen plotter and want each layer to be plotted with a

different pen, specify the pen numbers with SETUP CHECK PLOT PENS.  This can

mean that each of the layers are plotted in a different colour, for example.

For the DEMO board, turn on the following layers:

Top Layer

Bottom Layer

Overlay

Multi Layer Pads

A Check Plot can be in either draft or final artwork modes.  If it is in

draft mode only, the outlines of tracks and pads are plotted.  If you choose

final artwork, all the items on the board are filled in during the plot.

TOP LAYER

This is a plot of the top layer of the board.  This layer is also known

as the component side.  You see all the tracks that have been placed on this

layer as well as pads and vias and edge connectors.  Components are usually

placed on this layer.

MID LAYERS

There are four mid layers available in PROTEL-EASYTRAX.  Tracks can be

placed on each of these layers.  Multi layer pads and vias are also shown in

the plot if requested.

These layers are not used in the DEMO board hence no plots are included.

BOTTOM LAYER

This layer is used to route tracks on a double-sided board.  It is

sometimes used to place components as well.  Tracks on this layer are

normally placed horizontally, although vertical placements are sometimes

necessary.

OVERLAY

This is the layer which is normally used for silk screening text and

other information which is to be displayed on the board.  If you are producing

an overlay plot for silk screening purposes, then leave pads out of the plot.

You can do this by choosing OPTIONS PADS OFF from the MAIN PLOT MENU.

GROUND PLANE

This is a plot of the ground plane.  Since this plane consists mainly of

copper, it is usually plotted in the negative.  If you want to make sure that

there is sufficient clearance for pins passing through this plane which are

not to be connected to it, you can specify an enlargement factor by choosing

SETUP PWR/GND ENLARGEMENT.  You are given the opportunity to specify the

number of mils increase in diameter for the hole you require.

POWER PLANE

The description of the ground plane in the preceding section is similar

to that for a power plane plot.

SOLDER MASKS

The solder masks are plots of all pads with the holes filled in.  No

strings or tracks are plotted.

Most manufacturers of printed circuit boards require that the solder mask

be slightly larger than the actual pad size to ensure that the mask adequately

covers the pad area.  For this reason, you are able to specify an enlargement

factor by choosing OPTIONS SOLDER MASK ENLARGEMENT.

Note that are solder masks for both the top layer and the bottom layer.

PAD MASTER

This plot shows each of the pads and vias on the board.  It also helps

you check the placement of components and can act as a guide for the

programming of drill machines during the manufacture of the PCB.

DRILL GUIDE

This plot gives your PCB manufacturer a guide to the holes that must be

drilled through the board.

UTILITY PROGRAMS

The program disk contains three utilities.  Two of them allow you to

convert files created with PROTEL-PCB 3.0 so that they are compatible with

PROTEL-EASYTRAX.  The other one allows you to produce a Bill of Materials

file for a board.

THE PCB3CON PROGRAM

This program will convert PCB files created with PROTEL-PCB 3.0 so that

they can be loaded into PROTEL-EASYTRAX.  You should be aware that because

netlists cannot be used with PROTEL-EASYTRAX this information is lost when

the files are converted.

The simplest way to convert a file called, say, BOARD.PCB is to return

to the DOS prompt and type:

PCB3CON BOARD.PCB NEWBOARD.PCB

Thus, the name of the PROTEL-EASYTRAX file is now NEWBOARD.PCB.

If you do not specify the .PCB extension, that extension will be forced

by the program.

If you just type:

PCB3CON and press <<ENTER>>

then you are prompted to supply the old file name and the new PROTEL-EASYTRAX

file name.

The program also asks you whether you want the component designator text

and the comment text hidden and which pad shape you want.

You can specify complete path names for the old file and the new file.

THE PLIB3CON PROGRAM

This program has exactly the same structure as the PCB3CON program.  It

is used to convert libraries that you might have created with PROTEL-PCB

3.0 so that they are compatible with PROTEL-EASYTRAX.

If you want to convert a library called OLDLIB.LIB to be called

NEWLIB.LIB then type:

PLIB3CON OLDLIB.LIB NEWLIB.LIB

All the other features of the program are the same as for PCB3CON except

that you are only prompted to supply the pad shape.

You need to be aware that libraries created with PROTEL-PCB 3.0 may

increase in size significantly when converted.  The reason is that there is

more information stored about each pad in PROTEL-EASYTRAX libraries.

THE BOM PROGRAM

The BOM program produces a Bill of Materials file which lists the

components you need to construct the board.  You can ask for the output to be

produced in either a detailed form or a summarized form.

To produce the Bill of Materials file, return to DOS prompt and type:

BOM then press <<ENTER>>

You are asked to input the name of the PROTEL-EASYTRAX .PCB file.  Type

in the name, then press <<ENTER>>.  You are then asked to specify the name

of the output file.  Once again type in the name, then press <<ENTER>>.  It

is automatically supplied with a .BOM extension.

You can inspect the file by using the TYPE command at the DOS prompt.

APPENDIX "A" FEATURES OF PROTEL-EASYTRAX

This Appendix outlines the technical features of PROTEL-EASYTRAX which

you may need to refer to from time to time.

IMPORTANT FILES

The files that are supplied with the package are:

EASYEDIT.EXE          This is one of the main program files.  It

must be stored in the same directory as the

file EASYEDIT.OVR.

EASYEDIT.OVR          This is the "overlay" part of the program.

While this is part of the main program, it

resides on the disk and is only loaded into

memory as required.  This helps free up

memory space for your boards.  This file can

be stored in any directory, but it must be in

the same directory as the EASYEDIT.EXE file.

If you have EMS memory installed, the .OVR file

is loaded into that portion of memory, thus

making calls to the overlay very fast.

EASYSTD.LIB           This is the main library file which contains

the patterns for the components you are likely

to place on your boards.  There are 169

components in the standard library.

PCB3CON.EXE           This program is used to convert .PCB files

created with the PROTEL-PCB program to a format

which can be read by PROTEL-EASYTRAX.

PLIB3CON.EXE          This program is used to convert library files

created with the PROTEL-PCB program to a format

which can be read by PROTEL-EASYTRAX.

BOM.EXE               This program produces a bill of materials for

the file you specify.

EASYEDIT.MNU          This file holds the menu messages and the error

messages used by PROTEL-EASYTRAX and must always

be available to the program at start up.

EASYEDIT.KEY          This file holds the default key macros which are

automatically loaded when you start PROTEL-

EASYTRAX.

DEMO.PCB              This is one of the demonstration PCB files which

is referred to in the manual.  All PCB files

generated with the PROTEL-EASYTRAX program have

a .PCB extension.  This board has already been

routed.

***.BAT               There are a number of graphics drivers supplied

with PROTEL-EASYTRAX.  A batch file is supplied

for each which copies the specified graphics

driver to the file called GRAPH.DRV.

GRAPH.DRV             The currently selected graphics driver.

The files that you might create have the following extensions:

.PCB                  This extension will be attached to any PCB

files that you create.

.NET                  This extension is attached to the netlist files

you create with the HIGHLIGHT MAKE NETLIST

option in PROTEL-EASYTRAX or those that you

have created with the PROTEL-SCHEMATIC program.

.KEY                  These are the files you have created which

contain the keyboard macros hat you have defined.

.BOM                  The Bill of Materials file which contains a list

of components on the board.

Other files that might be created are:

???.BAK              This file is created when you save a PCB file.

It is a copy of the previous save of the file.

If you realize that you have made a major error

in the current editing session, you can restore

your file to it previous state by renaming the

.BAK file.

EASYEDIT.ABK         This is the file which is automatically created

every 30 minutes (or whatever you have set the

backup time to be in the Current Backup option).

If your system crashes and you need to restore

your file from the latest backup, simply rename

this file with the name of your current .PCB file.

EASYEDIT.DFT         This is the file which contains the setup

parameters which you have chosen.  This file is

created if you change any of the default

parameters.

HARDWARE AND SOFTWARE CHECK LIST

In order to use the PROTEL-EASYTRAX software you will need the

following:

An IBM-PC or PS/2 microcomputer or compatible

2 disk drives (2 floppy drives or one floppy and one hard drive)

Graphics card (any card for which a driver is supplied)

Colour Monitor

One parallel printer port

Microsoft Mouse or compatible (recommended)

Draft printer

MS-DOS or PC-DOS (Version 2.0 or greater)

HARDWARE REQUIREMENTS

To use the PROTEL-EASYTRAX software you will need an IBM-PC/XT/AT or

IBM PS/2 microcomputer (or compatible).

The microcomputer should have at least 640K RAM and the software will

support expanded memory boards which conform to the L.I.M. standard.

The software will perform adequately on a machine with an 8088

microprocessor running at 4.77 MHz, but you will notice a significant

increase in speed if a higher clock speed or a more advanced processor (such

as an 80286 or 80386) is used.

Installing a numeric co-processor will not lead to an increase in speed

since PROTEL-EASYTRAX software only uses integer calculations.

The system should have at least two floppy disk drives or one floppy and

one hard disk drive.  A single drive system is not suitable.

A range of colour cards is supported.  The software will work with a

Colour Graphics Adaptor (CGA) card, an Enhanced Graphics Adaptor (EGA) card,

Hercules card, VGA card, the Vega Deluxe card, three Genoa cards and the

Paradise 800 card.  Monochrome cards are not supported.

The higher resolution of the EGA, VGA, Genoa, Paradise, and Vega Deluxe

cards make them preferable for intricate design work, however, many users

have achieved excellent results with CGA systems.

New drivers will be added to this set in the future.

While the software has been designed to work with keyboard-only systems

it is strongly recommended that either a serial mouse or a bus mouse be

installed.  The software works with the Microsoft Mouse or any mouse that

is 100% compatible with it.

A dot matrix printer or laser printer using 100 dot per inch resolution

is recommended for draft prints.  The software supports the Epson FX and LQ

series, the Quiet Jet and the HP Laserjet II.

PROTEL-EASYTRAX supports the popular HPGL, DMPL, and Roland DXY 800

plotter languages.  There are numerous plotters available which support

these plotting languages, and they span wide price/performance ranges.  The

following plotters are just a few which have been used successfully by

PROTEL-EASYTRAX customers.

Protel PT101 Bench Top Photoplotter

HP-7440A (ColourPro), 7475A, 7550A, 7570A, (DraftPro),

7580B, 7585B, 7586B series.

Houston Instruments DMP-40, 41/42, 51/52, 56 series.

Roland DXY 800, DXY 880, 980 series.

Enter Computer Sweet-P Plotters.

CALCOMP 104X Series

The PROTEL-EASYTRAX software supports Gerber photoplotters and photo-

plotters that emulate the Gerber series.  These photoplotters produce high

quality PCB artwork and are accurate to the PROTEL-EASYTRAX 1 mil resolution.

SOFTWARE REQUIREMENTS

In addition to the PROTEL-EASYTRAX software, you will need a copy of

either MS-DOS or PC-DOS operation system.  The version number should be

2.0 or greater.

The PROTEL-EASYTRAX software provides automatic backups of your files,

but it is strongly recommended that a regular backup of .PCB files is made

onto a different disk.

You should use the DOS COPY command to copy your .PCB files to a

different disk, then store that disk in a different location.  It is also a

good idea to back up your .KEY and .LIB files from time to time as well.

PARAMETERS

The following list gives the values of many of the PROTEL-EASYTRAX parameters.

Max board size             32 x 32 inches

Board Layers               6 single layers plus power and ground planes

1 overlay

Grid Resolution            Visual and snap grid adjustable from 1 mil

to 1000 mils

Zoom Factor                Variable from 1:1 to 1:100

Track Width                Seven widths:10,12,15,30,50,70, and 100 mils

Via Size                   Six via sizes: 40,50,62,70,100, and 150 mils

Pad Shapes                 Circular, rectangular, rounded-rectangular,

square, edge, cross hair and Moire

Pad Orientation            Normal and rotated

Targets                    Moire and cross hair targets

Cursor Shape               Arrow, small cross and large cross

Filename specs             8 characters

Pads                       48 pad types

Key Macros                 48 key macros (definable)

Library Components         200 components maximum in any one library

String Sizes               Eight sizes:  48,60,84, 120, 160, 280, 420

and 560 mils

APPENDIX "B" RS232-C CONSIDERATIONS

Many plotters come equipped with a RS232-C interface, and a cable is

required between the plotter and computer to provide data transmission and

handshaking.  Correctly configuring this cable can be an exasperating task -

even for experienced engineers.  This appendix explains the relevant RS232-C

conventions for cables that have been tested on a wide variety of plotters

and computers with 100% success.

It is natural to ask why parallel interfaces seem to work every time

with no problems, whereas serial interfaces can take great amounts of time

and patience to set up.

SERIAL VERSUS PARALLEL INTERFACES

Parallel interfaces provide one-way transmission only, from computer to

a peripheral device, such as a printer or a plotter.  Since data is only

transmitted in one direction, the data and handshaking connections and their

direction of flow are straight-forward.  All data bits (8) originate from the

computer and are received by the peripheral.  The data strobe signal obviously

originates at the computer as well, since it indicates that valid data is on

the bus.  The acknowledge and busy lines clearly originate from the peripheral

and so on with all of the signals.  A standard cable has been defined for all

these signals, and the computer and peripheral are intrinsically configured

to originate or receive these signals as appropriate.

However, the RS232-C standard defines the signals for bidirectional

communication between two pieces of equipment, known as Data Terminal

Equipment (DTE).A DTE is defined as:

1.  The point of origin for any data to be transmitted, and

2.  the final destination for transmitted data.

Thus, the computer and plotter are both defined as DTE.  This is where

problems begin.  From an RS232-C point of view, both the computer and the

plotter look the same, and signals such as Transmitted Data (from the DTE)

are assigned to the same pins in both devices.  Contrast this with the

parallel interface in which the interpretation of a signal pin is intrinsic

to whether the device originates data or receives it.

Thus, interfacing two DTEs needs an intermediary device or devices to

connect the transmitted data pin of one DTE to the received data pin of the

other, and vice versa, and to also correctly configure the handshaking

signals.  The RS232-C protocol defines Data Communications Equipment (DCE),

which connects directly to DCE and transmits and receives the data over a

channel - a DCE does not originate data, nor is it the final resting place

for data.  A modem is a DCE that modulates the data for transmission over a

voice channel (a telephone line) and demodulates it at the receiving end back

to digital data.

CABLE CONNECTIONS

In the case of connecting a computer to a nearby peripheral such as a

plotter, the cable is configured to take the place of the DCE connecting

two DETs.  Because it replaces the modems described previously, such a cable

is known as a Null Modem.  The IBM-AT uses a DB9 connector for its serial

port, whereas other machines use the familiar DB25 connector.

When the plotter is ready to receive data it asserts the Data Terminal

Ready (DTR) signal.  This is received at the computer as Clear to Send (CTS)

and the computer commences transmission.  When the plotter buffer is full,

the DTR line goes low, and the computer sees that it is no longer clear to

send data.  As the plotter continues plotting and empties the buffer, its

DTR signal again goes high and the computer recommences data transmission.

From the computer's point of view, it requires that CTS is high as well

as Data Set Ready (DSR), Data Carrier Detect (DCD), and (in some computers)

Ring Indicator (RI), before it will transmit data.  When the computer is

turned on, its serial port is initialized and its DTR is asserted, since the

computer DTR, DSR, DCD, and RI lines within the shell of the connector, the

computer can sense that a valid connector is plugged into its serial port.

The cable is then made symmetrical so that either end of the cable can

be plugged into the computer or plotter.  Users of Houston Instruments DMP

plotters will find this handy, since the plotter uses the same male DB25

connector as the IBM-PC/XT.  With HP plotters this symmetry isn't as

important, since the HP plotters use female DB25 connectors and the cable

ends cannot be swapped.  Use the correct connectors to plug into your plotter

and computer.

If you are using the IBM-AT (or compatible) computer, either obtain a

DB9 to DB25 adapter and use the cable to connect directly to the DB9

connector on the computer.

This discussion should allow you to connect your computer and plotter

without problems, and should enable you to interface with other serial

devices as well.  Refer to the manual that comes with your plotter for

further information on its interfacing and handshaking.

BAUD RATE, DATA BITS, ETC.

The other important consideration when using a serial interface is to

match the baud rate (110-9600), the number of data bits (7 or 8), stop bits

(1 or 2) and the parity bit (Odd, Even, or None) of both the computer and

plotter.  Your plotter manual should indicate the default settings of the

plotter upon being powered up, and will contain information on changing the

communications setup.  Some plotters do not have default settings but have

DIP switch settings for the communications parameters.

Set up your plotter according to the manual and then be sure to match

these settings with those in the EASYPLOT program.  These settings are

stored when the plot finishes, so that once they are set correctly they will

not have to be changed while using the same plotter.

For those of you with no idea of how you should initially set your

plotter, use the following values to begin with:

Baud Rate:            2400

Data Bits:            8

Stop Bits:            2

Parity:               N

This is an intermediate baud rate and will yield error-free data

transmissions for reasonably long cables.

WHAT TO DO IN CASE OF PROBLEMS

1.  Don't panic!  First check your cable connections, make sure no wires

have broken, and check that your EASYPLOT setting matches the baud

rate, parity, etc. of your plotter.

2.  Make sure you are using the correct serial port.  If EASYPLOT

specified serial port 1 (COM1:), make sure your plotter is

connected to that port and not the parallel port or another serial

port.

3.  If your plot goes all right for a while and then starts putting

stray lines and arcs all over the layout, this generally indicates

improper handshaking.  Check that your cable is properly configured

and that all communications settings match.  This problem can also

occur if the data is being corrupted.  Keep the plotter cable as

short as possible and keep it away from power cords, etc.  If you

are using a long cable, you may have to reduce the baud rate to

obtain error-free transmission.  Due to the distributed resistance

and capacitance of cables, there is a trade-off between cable

length and baud rate for reliable data transmission.

4.  Make sure you have specified the correct plotter type in the EASYPLOT

program.  Is your plotter a true Hewlett-Packard, Houston Instruments,

or Roland DXY 800 plotter, or one that emulates one of them?  Some

plotters, such as the Roland DXY-880, have their own language as well

as emulating HPGL.  Make sure that if you are using a dual-language

plotter you set the plotter DIP switches for the correct language.

APPENDIX C ERROR MESSAGES

This appendix provides a brief explanation of each of the error

messages which occur while using the PROTEL-EASYTRAX and EASYPLOT programs.

They are arranged in alphabetical order of the first character in the

message to make them easy to look up.

NON-FATAL ERRORS

BAD FORMAT IN KEY MACRO FILE

The key macro file which has an extension of .KEY has been

corrupted.  This may have occurred during editing with a word

processor.

To overcome the problem, delete the key macro file and copy the

backup key macro file into that directory.  This remedy assumes

that you have made regular backups of your important files.

BAD LIBRARY FILE FORMAT

The library file which has an extension of .LIB has been

corrupted.

The remedy is the same as that for the key macro and pad type files.

BAD PCB FILE FORMAT

The PCB file with an extension of .PCB has been corrupted.

The remedy is to delete the file and then copy the latest backup

into the directory, or to rename the automatic backup file which

is created every 30 minutes under the .ABK extension.

BLOCK NOT MARKED

You have attempted to move, copy or delete a block without first

defining a block.

Choose BLOCK DEFINE to define the block and then carry out the

required operation.

COMPONENT DESIGNATOR IS ALREADY USED

You have attempted to assign a component designator that has

already been used for another component on the board.

When you place a component the program will automatically

supply the next available designator if you type, say, U? instead

of, say, U3.  If you are placing a number of components of the

same type, the designator which appears in the input window will

automatically be the next available.

COMPONENT NOT FOUND

This error will occur if you attempt a JUMP COMPONENT operation

to a non-existent component.

Choose INFORMATION COMPONENTS to check which components are

currently placed.

COMPONENT NOT FOUND IN LIBRARY

If you are placing a component or renaming a component and you

enter a component name which does not exist in the component

library, then this error message will appear.

It is possible that you may not have specified the name correctly.

To rectify the problem, enter a <<?>> and press <<ENTER>> or click

<<LEFT MOUSE>>.  Then select the component you require from the

list provided.  If the component name does not appear in the list

you may need to select another library.

CURRENT LAYER MUST BE TURNED ON

You have attempted to turn a layer "off" (in Setup Toggle Layer)

when it is the currently selected layer.

To overcome the problem exit from this menu and select another

layer to be the current layer, then try again.

FILE NOT FOUND

You have attempted to load a file which is not in the currently

selected library.  This may occur if you do not specify the name

of the file correctly.  Alternatively, you may have entered the

name of a file which does not exist.

To overcome the problem, check the name of the file with FILE FILES

or change the path with FILE PATH.

KEY MACRO FILE NOT FOUND

You have specified a key macro file which does not exist.

Choose FILE FILES and enter the mask *.KEY, then inspect the

names of the key macro files on your disk.

KEY MACRO KEY ALREADY USED

You have attempted to assign a key macro to a function key which

already has one assigned to it.  This may occur in SETUP KEYS

ASSIGN or SETUP KEYS NEW.

If you still want to assign the macro to that key, you must first

clear the key by choosing SETUP KEYS DELETE.

KEY MACRO NAME ALREADY USED

You have attempted to assign a name to  a key macro which is

already in use.  This may occur in SETUP KEYS RENAME OR SETUP

KEYS NEW.

To overcome the problem, simply enter a different name.

KEY MACRO NOT FOUND

You have specified a key macro which does not exist.

LIBRARY FILE ALREADY EXISTS

This error will occur if you choose LIBRARY NEW LIBRARY and try

to specify a name for the new library which already exists.

To overcome the problem, change the name you want to give to

the new library.

LIBRARY NOT FOUND

You have tried to load a library which has either not yet been

created or it not in the directory specified.

LIBRARY FULL

You have attempted to add a component to a library which already

has 200 components in it.

To overcome the problem, either delete a component from the

library first, or create a new library.

LIBRARY NAME IS ALREADY USED

This error occurs during the LIBRARY ADD option if you try to add

a new component to a library and assign a name which already exists.

Check the names of the components in the library with LIBRARY LIST

and then change the name of the new component so that is does not

clash with an existing name.

NO COMPONENTS IN LIBRARY

You have just created a new library file and then tried to LIBRARY

LIST, LIBRARY BROWSE, or PLACE COMPONENT or a similar option.

You will need to add some components to the current library or

change to a different library.

NO COMPONENTS PLACED

You have attempted to select a component when there are no

components placed on the board.

NO KEY MACROS LOADED

An attempt has been made to refer to a key macro when no macros

have been loaded.  Key macros are stored in a file called

EASYEDIT.KEY.  This file is automatically loaded when you start

the EASYEDIT program.

This error will occur when you try to carry out many of the

functions in SETUP KEYS if you have cleared the key macros from

the memory.

To overcome the problem, choose SETUP KEYS LOAD and load in a key

macro file.

NO LIBRARY HAS BEEN OPENED

You have tried to load a component or save a component without

first having specified a library.

Normally, the default library is opened when you start PROTEL-

EASYTRAX.  If no library is open, check your disk or directory

to make sure a library file exists.

NO PINS HAVE BEEN HIGHLIGHTED

You have chosen the INFORMATION HIGHLIGHTED PINS option without

having first highlighted any pins.

To rectify the problem, choose HIGHLIGHT NET or HIGHLIGHT

CONNECTION and highlight some pins.

NO PINS ON THAT PLANE

You have tried to list the nets on the power or ground plane when

no connections have been made.

Check that you really want the connections to be on that plane.  If

you do, then turn that plane on with SETUP TOGGLE LAYERS and make

the required connections.

NO PRIMITIVES FOR LIBRARY COMPONENT

You have tried to save a component to a library but have not yet

defined any primitives such as pads or tracks for that component.

NOT ENOUGH MEMORY FOR COMPONENT

You have tried to create a component or merge a component when

there is not enough memory available.

If you need the new component, then delete one of the existing

components before loading the new one.

NOT ENOUGH MEMORY TO SUSPEND TO DOS

There is not enough memory left to exit to DOS.

You should either choose FILE QUIT or continue working without

exiting to DOS.

ONLY 48 KEY MACROS AT ANY TIME

An attempt has been made to define a 49th key macro.  PROTEL-

EASYTRAX only allows 48 key macros to be defined at any time.

To overcome the problem, choose SETUP KEYS DELETE and delete

a macro you no longer need before defining the new macro.

OUT OF MEMORY

There is not enough memory for the operation you have attempted.

This is likely to happen if you attempt to load a large .PCB file.

PAD NOT FOUND

You have chosen the JUMP PAD option and specified a pad number

which does not exist on the chosen component.  An example would

be selecting a DIP16 pattern and then specifying pin 17 for the

jump.

Check the number of pins in the component and specify a pin which

exists.

PATH NOT FOUND

You have attempted to specify a non-existent drive or directory

in the FILE PATH option.

Check the path specification to ensure that both the drive and

the directory actually exist.

PCB FILE NOT FOUND

An attempt has been made in FILE LOAD to load a PCB file which

does not exist or which cannot be found.

Check the names of the PCB files with the FILE FILES option.

If the filename is not displayed, then check FILE PATH to see

if the correct pathname has been specified.  If you are using

a floppy drive system, check to see that the correct drive is

specified and that the right disk has been inserted.

STRING NOT FOUND

You have specified a string in the JUMP STRING option which

is not on the board.

This may be because the string you specified is not present

or because you have specified it incorrectly or incompletely.

FATAL ERRORS

In each case, these errors cause the program to stop operating.  You

will need to start the program again and retrieve the latest version of your

.PCB file from the backup version.

If you generate a fatal error and cannot work out the reason for it, we

would appreciate it if you contacted us.

Division By Zero Error.EASYEDIT Program Aborted.

The program has attempted to carry out a fatal division by zero

operation.  DOS traps such an error and causes the program to

stop operating.

Stack Overflow Error.EASYEDIT Program Aborted

The stack has overflowed for some reason.  This is a rare error.

Not Enough Memory.EASYEDIT Program Aborted

You have run out of memory, possibly because your PCB file has

become too large.

Floating Point Overflow Error.EASYEDIT Program Aborted

Another rare error over which you have no control.

Floating Point Underflow Error.EASYEDIT Program Aborted.

Same as for Overflow error.

Not Enough Memory to Start EASYEDIT.

This suggests that one or more resident program are already in

the memory.  You are likely to generate this error if you suspend

to DOS by choosing FILE DOS and then try to start the program by

typing EASYEDIT instead of EXIT.

EASYEDIT.MNU Not Found. EASYEDIT Program Aborted.

All of the menu descriptions and messages are held in a file called

EASYEDIT.MNU.  This file must be in the currently selected

directory when the program is started.

Security Device Not Found, Program Aborted

You have attempted to run the PROTEL-EASYTRAX program without having

first connected the external security device supplied with the

program.