.N:1

.H:...Page F-$$$...

FIRST.DOC

Copyright 1986, by Thomas F. Droege, All Rights Reserved.

DROEGE

Design Robot for Origination of Exacting Graphic Engineering

DROEGE is a general purpose program for the design of printed circuit

photomasters.  Is was created as a demonstration program for our low cost

precision photoplotter.  As it was developed it became more and more useful as

a design tool, so eventually the circuit boards and the mechanical drawings for

our photoplotter were done using this program.

This is a manual CAD system.  That is, it does nothing automatically

like schematic capture, wire listing, routing, parts lists, etc..  It does

provide a computer aided equivalent of a taped master or drawing board with the

advantage that things are always in perfect registration and one can work on

selected layers.

General features are a 65 x 65 inch working area 12 layers deep.

Resolution over this area is 0.001 inch.  At least this is the way it is

labeled - the output can be scaled to anything so what is really provided is a

65000 X 65000 point working space.  The layers can be turned on and off and

assigned to any available color.  Lines, Pads (something a photoplotter can do

- a small object drawn all at once), Symbols (a named collection of other

objects), Arcs, and Labels can be entered.  The Symbol feature is particularly

powerful since Symbols can be nested within Symbols to many levels (20)

allowing construction of very complex objects with a few keystrokes.

The program being distributed through free channels is a limited

version.  It has most programming features, however storage space is

restricted.  Depending on how clever one is in conserving space, circuits can

be designed with ten to twenty chips or about 100 components.  As can be seen

from the demonstration files,  fairly elaborate designs can be created with the

"free" version.  The program with full storage allocation will accommodate

designs of about 300 chip equivalents.

You will find no appeal to guilt to get you to buy the registered

version.  If you really use it, it will be in your interest to buy a license.

If you try this demonstration version  on something small to test its features

you are earning what you get by your work,  and helping us sell the product - a

fair exchange we think.  Print FEATURES.DOC to learn the advantages of buying a

licensed copy and to print an order form.  Print PLOTTER.DOC to learn about our

low cost photoplotter.

Print DEMO.DOC for instructions for a quick tour around the facilities.

TUTOR.DOC directs you through a few simple tasks.  Do not underestimate the

effort required to learn to use this or any other CAD system.  As a member of a

CAD selection committee for a large laboratory the author has reviewed many

systems, it is an awful job.  My current view is that one just begins to know

the features needed after six months use.

***** SPECIAL OFFER *****

For a limited time, if you follow the procedure in PHOTOOL.DOC we will

produce a phototooling set for your job.  We will plot anything you can get on

8" x 10" film, for $25.00 per layer (Since we don't know when you will read

this, this offer is subject to confirmation, see PHOTOOL.DOC for the

procedure).

The following lists the documentation on this disk with a brief

description of what each file contains.  We suggest you do DEMO.DOC and

TUTOR.DOC and don't bother to look at much else until you have tried to do a

few things.  KEYTREE.DOC will help show you where you are going in the program:

FIRST.DOC

This document.

MANUAL.BAT - PRINTS A MANUAL CONSISTING OF THE FOLLOWING SECTIONS:

(You may print individual sections if you wish)

INTRO.DOC

Introduction and design philosophy.

DEMO.DOC

This leads you through a demonstration of what the system will do.  You

do not get to do anything, just look.  This is our "gee whiz" demo.

TUTOR.DOC

This leads you step by step through a few simple tasks.  We have tried

to start with excruciating detail and gradually tell you less and less

as you become familiar with our general approach.

QUICK.DOC

Quick summaries of how to perform the most common tasks.

HOWTO.DOC

How to do some more complex operations.

APPENDIX.BAT - PRINTS AN APPENDIX TO THE MANUAL CONSISTING OF THE FOLLOWING

SECTIONS (Or you can print them separately):

COMMANDS.DOC

An alphabetical listing of the command keys.  This is the primary

reference document for all the operations that are available.

KEYTREE.DOC

Operation of this program is by use of the function keys.  Each key

press typically reassigns the keys to a new set of values.  KEYTREE.DOC

is a "map of the cave" showing the route through the menu keys with

very brief summaries of key operation.  The key menu is presently

fourteen layers deep and has hundreds of entries, many of them

repeated.  A number of the key operations are reentrant so that much

deeper entry is possible.  The design allows quick movement to a menu

that will do what you want.  It may be useful to print this out and

hang it on the wall when you first start using this program to remind

you of what is available and as an index for COMMANDS.DOC.  We thought

this would be very useful.  We find it almost impossible to read, but

we include it because we went to the work to prepare it and it might

fit someone's taste.

ERRORS.DOC

Error messages and their meaning.

STANDARD.DOC

This document suggests a set of standards to get you started.  It pays

to approach a job in a consistent manner.  This file gives a set of

standard procedures for producing a design for a two sided plated

through hole board.  If you follow this standard we will be able to

plot your job for you with minimal hassle.

PLOTOUT.DOC

How to generate a plot output file with sample file output.

THE FOLLOWING MISCELLANEOUS DOCUMENTS CAN ALL BE PRINTED WITH MISC.BAT

OR THEY MAY BE PRINTED SEPARATELY:

PHOTOOL.DOC

How to get a set of phototooling prepared from your design.  Read this

before you prepare a job for us to plot.

UPGRADE.DOC

A list of the planned upgrades.

WISH.DOC

A form to submit requests for improvements and upgrades.

USERS.DOC

We will collect and distribute useful libraries.  This is a form to

make a submission.

BUG.DOC

The bug reporting form and procedure.  Serious bug finders may write

for a version containing bug identification tools.

FEATURES.DOC

Features of the full version of this program, and an order form.

PLOTTER.DOC

Features of our Photoplotter.

UPDATE.DOC

Changes and bug fixes by date.

NEWBUG.DOC

Known Unfixed Bugs

#