Page A-1

COMMANDS.DOC

The keys are listed in alphabetical order with their key number.  All key name

and number combinations are unique.  Since some of the commands are deep into

the key tree, we will list after some of them the path in the form used in

QUICK.DOC

5ALL

Resets the screen parameters so that subsequent redraws will display all (well

most) of the work area.  The cursor moves to its present x,y position in the

new space.  If data is entered before a redraw, it is entered in the new space

so beware!  Best to redraw immediately after a change of screen area.  We don't

immediately redraw because redraw takes a long time and you might change your

mind.

2ARC

Arc requests entry of three points counter clockwise.  An arc is generated

which starts at the first point, passes counterclockwise through the second

point, and ends at the third point.  The algorithm used is to draw

perpendiculars to chords between point 1 and 2 and between 2 and 3.  The center

is placed at the intersection and the arc starts at point 1 and ends at point

3.  The center is placed on the current grid at the expense of having the arc

pass through the points.  This is because the find operation searches only on

the current grid.  If the center was placed exactly, you would need to search

on a one-mil grid to find arcs and circles.  If you want to place an accurate

arc, place it on a one-mil grid.  But be warned, it could take thirty years to

find a worst placed object searching on a one-mil grid.  So get very close

before starting such a search.  Arc line width is determined by the current

setup.

3AUTO

Auto allows setting up the window seen by the screen of the whole working

space.  It is usually used to go from a big view of the job to select a smaller

area.  For use the cursor is placed at the upper left hand corner of the

desired viewing area.  When AUTO is pressed, the screen prompt asks for a

screen width.  When entered, The cursor immediately moves to its new position

in the altered view, but the screen is not redrawn.  Beware! Data entered after

a view change and before a redraw will be entered in its position on the new

screen view.  It probably will not be where you want it.  We do not redraw

automatically to give you a chance to change your mind.

9BACK

The F9 key backs up one menu.  It is one of the primary means of getting around

the menus.  Except for two places where it produces a prompt, it terminates

the present operation and backs up one menu.  In the two cases, leaving a

symbol and leaving a job, where work or time would be lost a prompt asks "are

you sure?".  Answering anything but "y" will stop the backup.  Backup (F9)

stops most long operations.  In particular it stops a search operation and a

redraw.  If a line has been started but not ended it ends the line.

4CENTER

Select lines drawn in the dot - long dash format used by draftsman to indicate

center lines.  Centerlines are not presently supported on anything but the

Page A-2

screen and the DOS screen dump.  No effect on pads, symbols, lettering, or

arcs.

5CHANGE

Change the entry in the redraw menu.  When change is pressed, the current

screen view parameters are loaded into the redraw table at the currently

selected number.  A prompt asks for the name for this screen view which it

enters.  F10 will cause redraw at the just entered view, or UP DWN may be used

to select another view.  4SETUP can be used to get to the area selection menu

without redrawing each time if you can use the cursor position to select views.

1CIRCLE

The prompt asks for a radius.  After the radius is entered, the FIND, DELETE,

Circles can also now be found, deleted, etc..  When a circle is found with a

different radius, the radius is changed to the found radius.  The last radius

is remembered until arc entry is left.  Circle line width, color, and level are

determined by the current setup.

3CREATE

Do something creative to a symbol.  This path takes you to the creation of a

new symbol or the the addition or deletion of material from an old one.  This

is also the path to kill (delete) a symbol and the path to make a symbol a

library.

3DASH

Select lines drawn dashed.  Dashed lines are not supported presently on

anything but the screen and the DOS screen dump.  No effect on pads, symbols,

lettering, or arcs.

2DELETE

Deletes a found object.  No effect if nothing is currently found.  The entire

object is redrawn in the background color.  Note that this may black out data

that is still retained if two objects intersect.

2DOT

Lines are drawn dotted.  No effect on pads, symbols, lettering, or arcs.

2DOWN

Decrements to the next lower numbered selection.  Numbers wrap around after

reaching one.  The selected number is displayed on the UP and DWN keys.

KP2 DOWN ARROW

Moves the cursor toward the bottom of the screen.  When the cursor reaches the

screen bottom, it hangs there although the virtual cursor position as indicated

by the X= and Y= indications at the top of the screen continues to move.  Data

is entered and found based on the X= , Y= cursor positions.  Thus it is

possible, and sometimes desirable to enter items off the screen.  This is

particularly handy for long interconnections where the position of the remote

end is known.

1DR@L

Page A-3

1DR@W

Pressing this key causes it to alternate between DR@W and DR@L. DR@L

causes all lines to be drawn at the minimum width regardless of the specified

line width.  This results in a significantly faster redraw particularly when at

high magnification.  DR@W causes the lines to be drawn at the specified width.

3EDARC

Moves immediately to the menu:

1FIND   2DELETE 3*MODFY 4SETUP  5REJECT

without selection of CIRCLE or ARC.  This allows find and delete operations on

arcs without setting up one.

4EDAPER

Selecting 4EDAPER displays the following menu:

1 UP    2 DWN   3       4       5*APER

The key 2EDAPER gets an UP DOWN menu with key 5 showing *APER to

indicate that aperture is being selected.  As the aperture is cycled it

changes the table entry on the selected setup and level.  See 1EDSET.

Pushing the UP and DWN keys causes the aperture to cycle through the available

selection.  The selection wraps around.  The top of the screen displays the

current SETUP, LEVEL, COLOR, and APER; and their current values.  Note that

SETUP and LEVEL only select table entry positions while COLOR and APER change

them.  Since this table has so many entries, it is too big to allow full

disclosure of the present entry status without filling the screen and needing

constant redraws.  We recommend that you list the setup table with the LSTSET

command and post it near your terminal.  Most of the entries should be reserved

for frequently used operations.  We recommend that you reserve one for special

purposes and never count on it being set for any particular combination.

A photoplotter can contain multiple apertures.  An example might be a square

62 mil pad with rounded corners and a 15 mil drill spot in its center.  This

item can be drawn all at once by positioning the aperture at a selected

location and flashing a light behind it.  We call such an operation a Pad.  The

same object might be moved while the light is on, causing a line to be drawn on

the film.  We provide a selection of fifteen apertures most of which can be

used as either lines or pads.  We would not recommend drawing a line with the

target aperture which is intended for registration marks, but we allow it.  For

those not having access to our (or someone else's) photoplotter, one can think

of aperture changes as pen changes on a pen plotter.  Since there is access to

the plot files it should be easy to write editing programs to edit the

aperture changes to whatever commands the available plotting device requires.

As inspiration and information is made available these conversion programs will

be included.  The present aperture list includes round apertures from 5 mils

to 125 mils, several square apertures and a target.  Subsequent plotting

programs allow one for one exchange of apertures so we would have no problem

(except cost) changing apertures if requested at plot time.  Write us if you

have a special problem (like surface mount devices) and need custom apertures.

3EDCOLR

Page A-4

Selecting 4EDCOLR displays the following menu:

1 UP    2 DWN   3       4       5*EDCOLR

The key 2EDCOLR gets an UP DOWN menu with key 5 showing *COLOR to

indicate that color is being selected.  As it is cycled, color changes the

table entry.  Color is changed only on the selected setup and level.  See

1EDSET.

Pushing the UP and DWN keys causes the color to cycle through the available

selection.  The selection wraps around.  The top of the screen displays the

current SETUP, LEVEL, COLOR, and APER; and their current values.  Note that

SETUP and LEVEL only select table entry positions while COLOR and APER change

them.  Since this table has so many entries, it is too big to allow full

disclosure of the present entry status without filling the screen and needing

constant redraws.  We recommend that you list the setup table with the LSTSET

command and post it near your terminal.  Most of the entries should be reserved

for frequently used operations.  We recommend that you reserve one for special

purposes and never count on it being set for any particular combination.

1EDLAB

Go to the enter data menu to enter the label that has been set up.

2EDLEVL

Selecting 4EDLEVL displays the following menu:

1 UP    2 DWN   3       4       5*LEVEL

The key 2EDLEVL gets an UP DOWN menu with key 5 showing *LEVEL to

indicate that level is being selected.  Level does not change the setup.  It

only selects the level on the selected setup which will be changed by the color

and aperture selection.  See 1EDSET.

Pushing the UP and DWN keys causes the level to cycle through the available

selection.  The selection wraps around.  The top of the screen displays the

current SETUP, LEVEL, COLOR, and APER; and their current values.  Note that

SETUP and LEVEL only select table entry positions while COLOR and APER change

them.  Since this table has so many entries, it is too big to allow full

disclosure of the present entry status without filling the screen and needing

constant redraws.  We recomment that you list the setup table with the LSTSET

command and post it near your terminal.  Most of the entries should be reserved

for frequently used operations.  We recommend that you reserve one for special

purposes and never count on it being set for any particular combination.

It is convenient to think of this system as consisting of 12

transparent layers each 65536 points square stacked on a drawing board.  A

point could mean anything, but in the USA it is convenient to think of points as

thousandths, at least for printed circuit layout purposes.  Since scale factors

can be applied during the plotting process it is easy to consider the points as

fractional millimeters feet or even light years.  The system allows display of

as few or as many layers as desired.  In addition, one can draw objects on any

number of selected layers at once.  Drawn objects can be different sizes on

different layers.  One of the available colors can be attached to a layer

causing it to be shown.  Drawing and showing a layer are independent

operations.  We can draw on a layer that is not shown and vise versa.  For

example we could draw a red 10 mil line on layer one and a green 15 mil line on

layer two at the same time.  The result will be a white 15 mil line on the

Page A-5

display, however later redraws of single layers would show the lines in their

proper widths and colors.

1EDSET

Selecting 4EDSET displays the following menu:

1 UP    2 DWN   3       4       5*SETUP

The key 2EDSET gets an UP DOWN menu with key 5 showing *SETUP to

indicate that setup is being selected.  Setup does not change anything.  It

only selects the setup that will have its entries changed by subsequent level,

color, and aperture selections.

Pushing the UP and DWN keys causes the setup to cycle through the available

selection.  The selection wraps around.  The top of the screen displays the

current SETUP, LEVEL, COLOR, and APER; and their current values.  Note that

SETUP and LEVEL only select table entry positions while COLOR and APER change

them.  Since this table has so many entries, it is too big to allow full

disclosure of the present entry status without filling the screen and needing

constant redraws.  We recommend that you list the setup table with the LSTSET

command and post it near your terminal.  Most of the entries should be reserved

for frequently used operations.  We recommend that you reserve one for special

purposes and never count on it being set for any particular combination.

8END

Ends the current operation.  Usually a line.  A number of other operations do

the equivalent of a line end if their operation would result in some confusion

about the data.

1ENTER

Go toward entering data.  This is on the first menu and is the choice between

non data entering operations and data creation operations.

2FILE

Go toward some file operation.  This is on the first menu and selects the file

system to load or to store data.

1FIND

Find the current object type.  Only lines are found in line mode, pads in pad

mode, etc..  A square search is performed on the current grid, starting with

the present location of the cursor.  Lines are found by any point on the line.

When a point on a line is found, the cursor moves to the line starting  point

and the message displays "line found at " current x,y.  The line is drawn

dashed.  Pads are found by their center.  Symbols are found by their tag.  The

tag is the position of the cursor when the symbol origin was located when the

symbol was created.  Symbol data is located relative to the tag position.

Labels are located at the lower left hand corner of their first letter.  Arcs

are located at their center.

The search for an object starts at the current cursor location and continues in

a square search on the current grid.  Note the difference between the current

grid and the M indication at the top of the screen.  The M indication shows the

current step taken by the cursor when an arrow key is pressed or held.  M is

always some multiple of the grid as selected by the 6SPEED and KP5 SPEED SHIFT

Page A-6

keys.  If an object is not on the current grid it will not be found.

When an object is found you may then 2DELETE it or 5REJECT it to continue the

search.   9BACK will abort the search next time the search reaches the upper

left hand corner of its square search pattern.

2GETFIL

Get a file from the file system.  You may load a job or a library.  The major

difference between jobs and libraries is that the job carries with it all the

screen setup parameters, and data located in the job area.  Libraries are

single symbols or collections of symbols.  Note that it is usually advisable to

put your entire job in a symbol sine it is then more portable.

5GRID

Selects the current grid.  Any integral grid is allowed.  How about a

sixteenth (.0625) you ask?  Sorry, to get a sixteenth you need to multiply

everything by two in your head or something.  Nice for laying out connectors

with 0.156" spacing.  But WARNING, CAUTION, LOOK OUT!!!!!  Search only finds

things on the current grid.  If you have used a funny grid and can't find

something, look for it on a 1 mil grid.  Better get close to it.  A search on a

1 mil grid to find an object in the least fortunate position could take

thirteen years if the data base is not too full.

4JOBFIL

Lists on the screen the job files on the default disk.

4KILL

Removes a symbol from the data base.  When a symbol kill is attempted, the

system searches through the entire data base for uses of the symbol.  This may

take a while.  If any uses are found you are told which symbol they are used in

so that you can find and delete their use.  If any symbol uses are found, kill

is not performed.  You must go through and delete every use before a kill is

executed.  This is necessary because nesting is allowed and deleting a symbol

which calls other symbols would create a mess.

4LABEL

Go to enter lettering.  Be sure to see NOLAB/SHOLAB.  You must have loaded an

alphabet library.  If you don't have the alphabet library in place lettering

will be drawn as dots.  If you have only a partial library in place,

unavailable letters will be drawn as a dot.  If you are not going to do

lettering everything goes faster without the alphabet library in place,

particularly loads and saves.  It is best to save the lettering for the end of

the job.  Labels are also drawn slowly.  This is because they are drawn line

segment by line segment.  While this is slow, it has some advantages.  See the

3MAKLAB command for some astounding things that can be done with labels.

KP4 LEFT ARROW

Moves the cursor toward the left of the screen.  When the cursor reaches the

screen left side, it hangs there although the virtual cursor position as

indicated by the X= and Y= indications at the top of the screen continues to

move.  Data is entered and found based on the X= , Y= cursor positions.  Thus

it is possible, and sometimes desirable to enter items off the screen.  This is

particularly handy for long interconnections where the position of the remote

end is known.

Page A-7

1LINE

Select line entry and edit.

3LIBFIL

Lists on the screen the library files available on the default disk.

2LIBSYM

Make a library from the open symbol.  This is actually a very complicated

process.  The symbol may call other symbols, and they in turn still others.

When 3OPRSYM is selected a redraw of the symbol is initiated, during which a

list is kept of all the symbols used in the open symbol.  This enables creation

of a library containing all the symbols used in the open symbol.

1LIST

Lists the symbols used in a symbol.  The symbol itself is included and is

considered to be on level zero.  All the symbols used are named and the number

of times they are used at level one, level two, and levels greater than two are

listed in a table.  If you use distinctive symbols for each type of part this

is a good way to get a parts count.  Likewise if you use a symbol for component

pads, you will also be able to get a hole count.

If, as we do, you use many layers of symbols; it is easy to loose track of

where symbols are.  Since you can only find symbols that are one level down

from the present, a table like this is useful for complicated jobs.

7LOCATE

Enter the currently selected type of object into the data base at the current

cursor position.  For a line this produces a line start if none has been

started, or a line mid point otherwise.  Line starts only generate a tick sound

to indicate that the line has started.  Lines are not drawn until two points

are accumulated.  For other objects the entry is made and the object is drawn.

Locate does nothing if you are not properly set up to make an entry.  You are

not always told that nothing has happened though if nothing is drawn you might

suspect a problem.

1LOCLIN

Select entry of lines.  This gets the menu that allows find, delete, etc..

2LOCPT

Select point mode.  This gets the menu that allows find, delete, etc.. For the

time being this is only useful for the deletion of single points from an

existing line.  Later we will make this menu more useful by allowing the

insertion of points in an existing line.  Users will quickly discover why we

want to be able to do this.

1LOCSYM

Select entry of symbols.  This gets the menu that allows find, delete, etc..

If nothing happens when you locate a symbol you have probably forgotten to name

one.

Page A-8

1LODJOB

Load a job from the default disk.  This can take a while.  About 3 minutes for

a full sized job with this demo program on a PC, much less with an AT as the

loading process is mostly compute limited.  If you plan to work with really big

jobs on our full sized program you will want an AT.  During the loading process

the job is assembled from scratch and everything is linked and relocated.

After a load operation "what you see is what you have".  If the job loads

without complaint you can be pretty sure that everything is OK.  Only one job

can be loaded and it must be loaded first.  The setup files on the job

overwrite those of the setup files.  This means that the aperture and view

tables on the saved job are restored with it, as well as the cursor position

and a few other things.  If you want to preserve the setup parameters, save

your entire job as a symbol (you can only do this if you have created it as a

symbol - which we recommend) and load it as a library.

2LODLIB

Load a symbol from a library.  If you are going to load a job you must do that

first.  There is no limit to the number of libraries that can be loaded except

by the length of the symbol table which is 300 symbols in this demonstration

version.  The first symbol in memory wins, and other symbols with the same name

are ignored.  See "HOW TO REPLACE A JOB SYMBOL WITH A LIBRARY SYMBOL" in the

HOW TO  section for a way around this.  The system ignores duplicate names

since otherwise many replications of common symbols would occur.

2LROT=

Rotates individual letters in a string.  Each time hit it steps by one wrapping

around to zero and rotates the letters 90 degrees counter-clockwise.  By using

LROT=1 and SROT=3  you can produce an "hotel" sign type label.

5LSTKEY

Lists the labels assigned to the keyboard along with their sequential and hash

positions.  Nothing prevents you from editing the keyboard labels.  If you

don't like the way we draw a "G" for example then just open the symbol G$BOLD

and have it your way.

We hope to get wonderful alphabets to distribute to our users.  How about

something in Gothic?

4LSTLAB

Lists the labels that you have created.  These all start with the entered

string and end with $LABEL.

4LSTSET

Lists the sixteen setups.  It takes two pages but we find it necessary to print

this and post it by our terminal.   We can never remember if we have used odd

or even setups for level 1 or where we put the green 15 mil lines.

3LSTSYM

Lists the symbols.  The labels and keyboard symbols are listed separately, see

above.  You are given a rough measure of their size and their sequential and

hash positions.

Page A-9

3MAKLAB

When 3MAKLAB is pressed you are asked to enter a label.  You may then type an

arbitrary string from the keyboard.  This string is then taken apart character

by character and a search is made of the symbol table for (character)$BOLD.  If

such a symbol is found, it is scaled and rotated according to the parameters

set by 2SETLAB and assembled into a new symbol named (arbitrary string)$LABEL

which is again rotated and assigned color and level from the current setup.  If

some of the keystrokes in the arbitrary string entry are not found in the

symbol table, they are replaced by a dot.  After this process is complete your

are put into the  1FIND  2DELETE ... menu where you can locate the string as

often as you want.  A future entry of the same string will find it in the

symbol table and thus conserve space.

This scheme allows interesting translation possibilities.  When "A" is entered

in a string, the system looks for the symbol A$BOLD.  The system has no way of

knowing that A$BOLD represents an "A".  There is nothing to prevent you from

editing the A$BOLD symbol to make it, for example, a Chinese character.  Or a

small circuit board, or the drawing of a house or ...

We will welcome font libraries of other designs.

1MIR=

Mirrors labels.  Cycles between MIR X, MIR Y, and MIROFF.  You want to mirror

some of the labels so that when you plot the back side of the board with the

emulsion side up, the labels will be "right reading" when viewed through the

emulsion from the back side.  STANDARD.DOC lists one way to do everything so it

will come out right.  It is easy to find a combination which gets you a mirror

image of the board you want.

2NAME

Accepts a symbol name for later symbol entry.  The name stays in memory as the

current symbol unless some other symbol is found.  Then the found symbol

becomes the current symbol for entry.  This is so a symbol can be moved by the

find, delete, locate sequence.

1NEW

Initiates creation of a new symbol.  You will be asked for a symbol name.

Because of the lettering features, symbol names are different if the case of

any of their letters is different.  If a symbol with the same name (and

identical case) is found in memory you get the "symbol already created must

open to edit" message.

2NOLAB

2SHOLAB

Since labels are drawn out of individual lines they can take a long time to

draw.  Usually the labeling is not interesting on a drawing except when being

modified.  This key alternates as pressed an causes subsequent redraws to show

or not show the labels as indicated.

5OPEN

Prompt asks for the symbol name.  Remember case must match.  If the symbol is

found in the symbol table, it is drown on a scale that attempts to fill the

screen.  You may now select the type of entry to be made.

Page A-10

3OPRSYM

Redraws the open symbol while keeping track of the nested symbols used in the

open symbol.  You are then given the opportunity to make a library of the

symbol, to list the symbol and its nestlings, or to change its name or replace

it with another symbol.  Note that while you are allowed to abort the OPRSYM

redraw, the only way that you can get to the next menu is to allow the redraw

to run to completion.  This is necessary to allow proper generation of the

symbol lists.

3ORIGIN

When a new symbol is named, you are asked to locate the origin.  The origin can

be located anywhere in the work space.  Data entered into a symbol is located

relative to the origin.

5OTHER

Select arc entry and edit.  Later we will add other things through this menu,

thus the name.

2PAD

Select pad entry and edit.

5PALET

Pushing this key causes the display to alternate between the two available

medium resolution palettes.

3PLOT

Select creation of plotter output.

3PLTFIL

Create a plotter output file.

1PUTFIL

Select creation of a job or library save file.

10REDRAW

Function key 10 always initiates a redraw sequence.  It always destroys

the current screen display, first by writing a menu over it, and then by the

redraw operation.  Redraw causes the key display below and puts up on the

screen 15 preselected display areas.  The distribution copy has some of these

labeled while others are blank.  You will want to customize this menu to suit

your style and your job.  The storage area contains +- 32,768 data points in X

and Y.  It is usually desirable to display only a fraction of this space.  By

using the SETUP key, which shows up on many of the menus, it is possible to

specify the x and y coordinates of the upper left hand screen corner and the

screen width.  See "HOW TO SET UP REDRAWS" in the "HOW TO" section.  Once you

have done this, REDRAW allows saving this setup with the name of your selection

- like "memory section".  It is then possible to quickly switch from one

section of your layout to another by pushing redraw (F10) using the UP - DWN

keys to select a pre-stored selection then pushing SELECT (F3) to initiate a

Page A-11

redraw of the selected area.

Pushing redraw a second time after the selection menu is presented

causes a redraw at the current screen layout.

Redraw can take a long time.

4REGION

This key allows setting up the area of the 65536 x 65536 space that

will be displayed.  Region allows selection of x and y offsets and the screen

width.  ALL selects nearly the entire screen.  AUTO asks for a screen width,

then scales the display for the entered width with the present cursor position

at the top left corner.  SPACE asks you to specify the top left hand corner and

a screen width.

5REJECT

Rejects the result of a search operation.  The search continues from the point

of the found object.  F9 will stop the search and back up the menu.

4RENAME

Renames a symbol.  See "HOW TO REPLACE A JOB SYMBOL WITH A LIBRARY SYMBOL" in

the HOW TO section.

3REPLAC

Replace a symbol with a different symbol.  This is a global edit so be careful.

Remember "replace A with B whoops-replace B with A" can create an unsolvable

mess.  See HOW TO above.

KP6 RIGHT ARROW

Moves the cursor toward the right of the screen.  When the cursor reaches the

screen right side, it hangs there although the virtual cursor position as

indicated by the X= and Y= indications at the top of the screen continues to

move.  Data is entered and found based on the X= , Y= cursor positions.  Thus

it is possible, and sometimes desirable to enter items off the screen.  This is

particularly handy for long interconnections where the position of the remote

end is known.

5ROT=

Pushing this key causes it to gain a label of 0 to 3.  It is at zero initially.

Symbols entered with ROT= other than zero will be rotated in 90 degree

increments with 1 being 90 degrees counterclockwise.

1SAVJOB

Saves the entire data base as a job.  You are asked to enter a job name.  Two

files are created.  (Jobname).PCD and (Jobname).PCS.  Names are restricted to 8

characters per DOS rules.  (Jobname).PCD contains all the setup parameters.

(Jobname).PCS contains all the job symbols, and one special symbol which is the

data in the job itself.

This is the normal way to save your work when you wish to continue and restore

everything like it was when you discontinued work.  Everything is saved, all

the job date, the current screen layout, and even the cursor position.

Page A-12

2SAVLIB

Saves all the job symbols as a library.  Note the difference between SAVLIB and

LIBSYM.  SAVLIB saves all the symbols on the job as a library.  Even unused and

abandoned ones.  LIBSYM saves all the symbols in one given symbol as a library.

Only those used are saved.  If you make the whole working job a symbol, LIBSYM

provides a nice way to condense the job removing all the accumulated junk.

5SCALE

Provides for scaling labels variable symbols.  The alphabet we provide has been

drawn 0.125 high.  SCALE allows multiplying this size over a range of 100 to

0.01.

4SCREN

SCREN is accessible through SETUP which appears many places on the key

tree.  It provides a means for setting how and where items are displayed on the

screen.  It introduces two layers of menus.

3SELECT

Causes a redraw with the area parameters that were in place when the

numbered selection was stored.  When the redraw is complete, the menu is

restored to the menu in place when redraw was pressed.

2SETLAB

Accesses a menu which allows rotating letters, rotating an mirroring strings,

and scaling labels.

4SETUP

SETUP appears many places on the screen.  It provides access to menues which

control data level entry, object display color and size, and screen size and

position in the data space.

1SET=

2SET=

3SET=

4SET=

Display the four preset SETUPs when in the STITCH mode.  The keys display the

preset SETUPs.  After a line is started in stitch mode, pushing a SET= key

which has a setup different from the current one will cause the via symbol to

be located and the line continued with the new setup parameters.

2SHOLAB

2NOLAB

Since labels are drawn out of individual lines they can take a long time to

draw.  Usually the labeling is not interesting on a drawing except when being

modified.  This key alternates as pressed an causes subsequent redraws to show

or not show the labels as indicated.

1SOLID

Pressing this key produces a beep to indicate something has happened

and subsequent lines are drawn solid.

Page A-13

1SPACE

Allows selection of the redraw display by manual input of the x,y coordinates

of the upper left hand screen corner and the screen width.

6SPEED

Selects cursor movement as the number of times the grid spacing that it moves

for each key press.  The speed cycles in a 1,2,4,5,10,20,40,1... sequence.

KP5 SPEED SHIFT

Speed shift multiplies the current speed by 10.  We operate with the SPEED set

to the current grid we are using for locating objects.  Usually this is 0.025

or 0.100.  This works well in a small area.  When we have to move a big

distance we hit SPEED SHIFT and move until we get close, then we shift back.

We find it convenient to operate  with one hand on the function keys for

operation selection, and one hand on the keypad for movement.   Caps lock must

be on and num lock and scroll lock off for SPEED SHIFT to work.  We have

resisted making monitor calls to find the state of these keys till they are

provided by the system.

3SROT=

Rotates an entire string (or variable symbol).  When the key is pressed it

displays its value from an initial value of zero.  Rotations are in 90 degree

steps from horizontal to the right.

5STITCH

Move to stitch mode.  Stitch allows the entry of printed circuit traces while

moving between layers of a multi-layer board without the need to switch between

lines on different layers and pads.  You are allowed to preset four setups and

a via symbol which are presumably used to switch between traces on a four

level board.  These setups appear on function keys 1-4.  Pushing any of these

four keys instantly switches to the indicated setup.  You may now start and end

a line in the conventional way.  In addition, pushing a setup key that is not

the currently active one, ends the current line, locates the via symbol at the

current cursor location, switches the setup to that of the key pressed, and

starts a line with the new setup.   Note that in the usual generality of

symbols in this program, anything is allowed as a via symbol.

This allows fast insertion of traces which must move from layer to layer over

their length.

Note that there is no rule that stitch must be used to stitch.  The via symbol

is only inserted if a setup is changed while a line is in progress.  Thus

stitch can also be used for four quick setup changes while in line entry mode

if care is taken to only change setups when a line is not in progress.

5STSET

Switch to stitch setup mode.  Tells you that pushing function keys 1-4 will

allow assigning setups to those keys.  Push as many as you will need and enter

a setup.  Then push VIASYM to assign a symbol to the via between board layers.

When you have things the way you want them use back (F9) to go back to the menu

where stitch is active.  Stitch stays setup until you change it, though

presently the stitch setups are not saved from session to session.

Page A-14

3STYLE

Style presents a menu which allows selection of the way in which an

entered line is shown.  This is intended for mechanical drawings reproduced on

the dot matrix printer.  All photo and pen plotters may not respond to this

option.

3SYMBOL

Select entry of symbols or other symbol operations.  Symbols may be entered

into symbols, but not into themselves.  We do not yet protect for all possible

loop conditions, i.e. A calls B which calls C which calls A, but we do protect

for the more common cases.  If you do succeed in creating such a loop we die

gracefully at level 20 and put you back into the main menu, where you should be

able to rescue your work.

5SYMLST

Lists the symbol contents as stored in memory.  For guru's only.

1UP

Advances to the next higher numbered selection.  Numbers wrap around on

reaching maximum.  The selected position is displayed on the UP and DWN keys.

KP8 UP ARROW

Moves the cursor toward the top of the screen.  When the cursor reaches the

screen top, it hangs there although the virtual cursor position as indicated by

the X= and Y= indications at the top of the screen continues to move.  Data is

entered and found based on the X= , Y= cursor positions.  Thus it is possible,

and sometimes desirable to enter items off the screen.  This is particularly

handy for long interconnections where the position of the remote end is known.

5VARSYM

VARSYM asks for a symbol name then gives you the edit menu.  Note that variable

symbols are "labels" and are found as such.

Variable symbols are an essentially useless but fun adjunct to the way we do

labels in this program.  Keyboard label characters used to make labels are just

symbols, with all the characteristics of regular symbols.  When you enter a

label, you are allowed to assign an arbitrary level, color, scale, and aperture

to it.  The way we handle this is to separate the form of the label symbol from

the level, scale, aperture, and color that happened to be assigned to it when

it was created.

Any symbol can be used as a variable symbol.  When entered, the symbol form is

scaled, rotated, colored, and drawn with the assigned apertures on the assigned

levels.  Note that only its form is used.  If the original symbol was a printed

circuit with different apertures on different levels, all this is ignored.

Obviously this is nonsense for printed circuits.  It would, for example, allow

scaling a standard screw drawing which could then be placed on the desired

level in the desired color.  We have used this for mechanical assembly drawings

where the ability to change a symbol's color allows distinguishing it for study

of clearances.

Page A-15

5VIASYM

Pushing this key allows entry of the name of the symbol to be used as a via

when in stitch mode.

5*APER

5*COLOR

5*LEVEL

5*SETUP

These labels appear in the setup menu to indicate the item that is being

changed.

2*MAKE

Nothing implemented.  Here we plan to allow making some area of a job into a symbol.

3*MODFY

Nothing implemented.  This appears on the edit menu.  We plan to allow

insertion of line points and move and copy of lines and symbols.

1*PLOT

Nothing implemented.  Provision for direct output to a plotter.

2*PRINT

Nothing implemented.  We plan for output to a dot matrix printer.  For the time

being use the shift-print screen function of DOS 2.0 and higher.

Page A-16