ALGR2APR   Version 1.2

Copyright (C) GraphiCode Inc 1993  All Rights Reserved.

This program may be distributed as shareware.  No user fee or registration

is required.

This converter will convert from an Allegro .LST aperture file to a

GraphiCode .APR aperture file.  The converter can be used to convert single

files or groups of files.

usage: ALGR2APR [-Metric] <aperture file> [<outfile>]

Example:

To convert the aperture file "ALIST1.LST" for use with GraphiCode

products, type:

ALGR2APR ALIST1

This creates the GraphiCode aperture wheel file "ALIST1.APR".

To convert "ALIST1.LST" to "NEWALIST.APR" type:

ALGR2APR ALIST1 NEWALIST

To convert several aperture files type:

ALGR2APR *.LST

This is the Allegro format supported by this converter:

Date: 04-21-93

SpaceLabs Medical,Inc.

PCB,  AC BRICK

BOARD NO.:  388-0475-01

FILM STATUS:  A

PHOTOPLOT FILE DESCRIPTIONS:

l1.art:  COMPONENT SIDE (LAYER1)

l2.art:  GND PLANE (LAYER 2)

l3.art:  INTERNAL TRACES (LAYER 3)

l4.art:  INTERNAL TRACES (LAYER 4)

l5.art:  GND PLANE (LAYER 5)

l6.art:  CIRCUIT SIDE (LAYER 6)

sst.art: SILKSCREEN TOP

ssb.art: SILKSCREEN BOTTOM

smt.art: SOLDERMASK TOP (spread .004)

smb.art: SOLDERMASK BOTTOM (spread .004)

spt.art: PASTEMASK TOP

spb.art: PASTEMASK BOTTOM

aat.art: ASSEMBLY AID TOP

aab.art: ASSEMBLY AID BTM

Format:

Input code: ASCII

English format: 2.3

Coordinates: absolute

Leading Zero Suppression

Equal Coordinate Suppression

Definitions:

CIRCLE:  Round aperture for pads and/or line draws.

SQUARE:  Square aperture for pads and/or line draws.

THERMAL:  Thermal pads- Inside diameter is shown, airgap to outside

diameter is .015, 4 "X" ties of .015 thickness each.

RECTANGLE:  Rectangular pads (XXX)X(YYY), in mils, with squared corners.

(X dimensions are first, Y dimensions are second)

OBLONG:  Oval shaped pads (XXX)X(YYY), in mils, with full rounded ends.

(X dimensions are first, Y dimensions are second)

X :  Left to right dimensions. (positive direction)

Y :  Bottom to top dimensions. (positive direction)

APERTURE DESCRIPTIONS:

SHAPE            SIZE          DCODE

------------------------------------

CIRCLE             7            D12

SQUARE            20            D18

THERMAL          40             D83

RECTANGLE         60        65           D96

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.