ORBO2APR   Version 1.2

This converter will convert from a Orbotech .APR aperture file to

GraphiCode's .APR aperture file (although the files have the same extension,

they have very different formats).  The converter can be used to convert

single files or groups of files.  Some Orbotech aperture files use the .APT

extension, the name isn't really important - the file contents are.

usage: ORBO2APR <aperture file> [<outfile>]

Example:

To convert "ALIST1.APR" to "NEWALIST.APR" type:

ORBO2APR ALIST1 NEWALIST

To convert several files at once type:

ORBO2APR *.APR or

GraphiCode aperture files will then be produced with the extension

.AP_  to distinguish them from the original Orbotech .APR files.

This is a sample Orbotech aperture file (another format sample follows):

**** WHEEL  =   tmp  ****

UNIT IN : MILS

-------------------------------

D-Code   0010 : R100

D-Code   0011 : S100

D-Code   0012 : R050

D-Code   0013 : S050

D-Code   0014 : smd200-100

D-Code   0015 : smd200-100-f

D-Code   0016 : smd200-100-20

D-Code   0017 : smd200-100-50

D-Code   0018 : smd200-100-80

D-Code   0019 : rp100-050

-------------------------------

---------------------------------

| DATA VERIFIED, PROCEED TO USE  |

|--------------------------------|

|------------| SIGNATURE | DATE  |

|------------|-------------------|

|VERIFIED BY |           |       |

|------------|-----------|-------|

|CONFIRMED BY|           |       |

----------------------------------

Here's another supported format (beginning with ORBO2APR v1.1):

Design Output Job Report

File Type      :  PCB

File Name      :  hh-5033-sp

Output Mode    :  BOTH

No. Layers     :  5

Output Files

layer100       :  pat-a           (PAINT)

layer200       :  pat-b           (PAINT)

layer300       :  res-a           (PAINT)

layer400       :  res-b           (PAINT)

layer500       :  mask            (PAINT)

Coordinates    :  ABSOLUTE

Units          :  INCH

Zeros          :  LEADING

Code           :  ASCII

Number Format  :  2.3

DCODE 10  :    R100

DCODE 11  :    S050

DCODE 12  :  smd100-70ff

DCODE 13  :  smd100-70

DCODE 14  :  rp100-50

Version 1.2 adds support for decimal values (R100.125, for example).

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.