APT2APR   Version 1.9

Copyright (C) GraphiCode Inc 1992,1993  All Rights Reserved.

This program may be distributed as shareware.  No user fee or registration

is required.

This converter will convert from a Mentor Graphics .APT aperture file to a

GraphiCode .APR aperture file.  The converter can be used to convert single

files or groups of files.  Five different formats from Mentor are currently

supported - see the examples at the end of this file.  Version 1.3 adds the

ability to convert APT files written in mm (if metric is not specified, inch

units are assumed).

usage: APT2APR [-Metric] <aperture file> [<outfile>]

Example:

To convert the aperture file "ALIST1.APT" for use with GraphiCode

products, type:

APT2APR ALIST1

This creates the GraphiCode aperture wheel file "ALIST1.APR".

To convert "ALIST1.APT" to "NEWALIST.APR" type:

APT2APR ALIST1 NEWALIST

To convert several aperture files type:

APT2APR *.APT

For .APT files with values in mm, use the -M option:

APT2APR -M ALIST1 NEWALIST

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.

Example of Type 1:

Aperture Position    D_Code  Shape        Type         Power    Height(Y)/Diameter  Width(X)     Used Last Run

1                 10   circle        trace                  0.005000           0.000000            yes

2                 11   circle        trace                  0.010000           0.000000            yes

3                 12   circle        trace                  0.012500           0.000000            yes

4                 13   circle        flash                  0.075000           0.000000            yes

Example of Type 2:

# DEFine APerture Position  Shape       Type   Height(Y)/Diameter  Width(X)

DEFine APerture    1    Circle          Trace          0.015000      0.000000

DEFine APerture    2    Circle          Trace          0.050000      0.000000

DEFine APerture    3    Circle          Trace          0.400000      0.000000

Example of Type 3:

Pos.  D_Code    Shape       Type    Power   Width(X)/Dia         Height(Y)

1     10      circle        trace                                0.0050

2     11      circle        trace                                0.0300

3     12      donut         flash           0.1250 O.D.          0.0470 I.D.

4     13      circle        trace                                0.0150

Example of Type 4:

Aperture Position    D_Code  Shape        Type         Power    Width(X)     Height(Y)/Diameter       Used Last Run

1                 10   circle        trace                0.0                0.01                  yes

2                 11   circle        trace                0.0                0.06                  yes

3                 12   circle        trace                0.0                0.03                  yes

4                 13   circle        trace                0.0                0.025                 yes

5                 14   circle        trace                0.0                0.05                  yes

6                 15   circle        trace                0.0                0.08                  yes

7                 16   circle        flash                0.0                0.1                   yes

8                 17   rectangle     flash                0.1                0.1                   yes

Example of Type 5:

# Aperture nr. D_Code Shape      Type   Power  Diam./Width(X)  Height(Y)

1         14    circle     flash            0.0157        0.0000

2         17    circle     flash            0.0276        0.0000

3         18    circle     flash            0.0472        0.0000

4         19    circle     flash            0.0591        0.0000

5         20    circle     flash            0.0748        0.0000

6         21    circle     flash            0.0985        0.0000

7         23    circle     flash            0.1969        0.0000

8         24    circle     flash            0.0020        0.0000

Example of Type 6:

APERTURE POSITION  D_CODE  SHAPE        TYPE       SPECIAL    DIAMETER/HEIGHT(Y)  WIDTH(X)

1              10   CIRCLE        TRACE                  0.007

2              11   CIRCLE        FLASH                  0.032

3              12   CIRCLE        FLASH                  0.058

4              13   RECTANGULAR   FLASH                  0.058              0.058

5              14   CIRCLE        FLASH                  0.042

6              15   RECTANGULAR   FLASH                  0.042              0.042

7              16   RECTANGULAR   FLASH                  0.032              0.032

8              17   CIRCLE        FLASH                  0.033

Example of Type 7:

---------------------------------------------------------------------

MENTOR APERTURE WHEEL

D-CODE  DESCRIPTION

---------------------------------------------------------------------

10      5  MIL ROUND

11      6  MIL ROUND

12      7.75 MIL ROUND

13      10 MIL ROUND

14      12 MIL ROUND

15      15 MIL ROUND

16      17 MIL ROUND

Example of Type 8:

// Wednesday August 4, 1993; 16:56:49

//

$$define_aperture(  1 , @Circle      , @Trace,    0.002000 ,     0.000000 ,  10 );

$$define_aperture(  2 , @Circle      , @Flash,    0.005000 ,     0.000000 ,  11 );

$$define_aperture(  3 , @Circle      , @Flash,    0.008000 ,     0.000000 ,  12 );

$$define_aperture(  4 , @Rectangle   , @Flash,    0.012000 ,     0.070000 ,  13 );