# Release Notes for Full Rate E1 LEDR Update

Microwave Data Systems Inc.

GENERAL OVERVIEW:

This code release is the result of a complete redesign of the clocking scheme for the full rate E1 payload interfaces on the LEDR platform. It is intended to increase the overall robustness of the payload interface and address any outstanding issues from the previous design. Plesiochronous clocking is now fully supported and provides the ability to vary any of the tributary clocks independently of one another. By nature of this design there is no longer a need to operate the ports from the same derived clock. Please note that G.703 limits the acceptable data rate for an individual E1 tributary to +/- 50ppm.

APPLICATION NOTES:

- The full rate E1 update is intended for full-rate links ONLY and should not be installed on T1 or sub-rate radios.

- This update is incompatible with previous releases for 1E1 and higher data rates. Both ends of the link need to be upgraded to the same E1 revision.

- The software package contains images for option, FPGA, and script regions only. These images can be applied to any existing firmware release after version 5.1.7.

REVISION HISTORY:

Revision E1.1.03							December 9, 2004

Release E1.1.03 contains the following versions of firmware:

FPGA:		E1.1.03

Modem Scripts:	E1.1.03

Option FPGA:	E1.1.03

## OVERVIEW

- Initial release. 1E1, 2E1, 3E1, and 4E1 functionality.

## RESOLVED ISSUES

- Not Applicable

KNOWN ISSUES

- None