--------------------------------------------------------------------------

BTC3 250 KV Tesla Coil / Lightning Generator

First Published by Information Unlimited

Transcribed to the electronic media by Thallion of Swedish Infomania

--------------------------------------------------------------------------

A tesla coil is one of the most fascinating electrical display

devices to see in operation. A large unit can produce a continuous spark of

a length exceeding the height of the coil. Electric discharges simulating

lightning bolts will produce cracks of noise louder than a rifle shot.

These sparks as well as being highly impressive and attention getting also

can produce bizarre effects to occur in most common materials. For example,

wood may explode into splinters or made to glow with an errie reddish light

from WITHIN. Insulating materials seem to be useless against this energy.

Lights light without wires, sparks and corona in the form of St. Elmo's

fire occurs within proximity of the device. High energy electric and

magnetic fields render electronic equipment useless. Phenomena not normally

associated with standard HV electricity becomes apparent in the form of

many weird and bizarre effects.

GENERAL THEORY

A tesla coil is a high frequency resonant transformer. It differs

from a conventional transformer in that the voltage and current

relationships between primary and secondary are independent of turns ratio.

A working apparatus basically consists of a secondary (LS1) and a primary

(LP1). It is obvious that the primary circuit is capacitance dominant and

tuning the primary circuit via taps along the primary coil does not alter

the frequency by as much as one would expect. However, this relatively fine

tuning of the primary circuit to the secondary is mandatory for proper

operation. Force driving (untuned) the secondary coil will produce hot

spots and interwinding breakdown.

INTRODUCTION TO YOUR EASY BUILD TESLA COIL

This project shows how to construct a device using a step up

transformer producing 6000 volts at 20 ma from the 115 ac line. This

voltage current combination can produce a painful shock and all attempts

should be made to construct this device with some know how or experienced

help. Safety rules should be followed at all times. The device also

produces ozone - therefore, use in well ventilated area. Do not use for

prolonged periods of time, 30 seconds on at any one time is ample for any

demonstration. Avoid eye exposure to spark gap without ample protection

such as safety glasses, shielding, etc. [ Transcriber's note: This is due

to emission of invisible, high energy, ultra violet rays, that in a very

painful way can damage your eyes. ]

The unit when constructed as shown can develop voltages up to and

in excess of 250,000 volts. It will cause a discharge lamp such as a

regular household fluorescent lamp to glow up to a distance of several feet

from the unit. The high voltage center coil terminal can actually be

touched with a piece of metal held securely in the demonstrators hand,

creating quite a conversation piece.

The unit can be used for exciting all types of gas discharge tubes,

x-ray tubes, plucker tubes etc. It demonstrates corona, St. Elmo's Fire and

the effect of high voltage on many different types of materials. On a good

dry day, with the device properly adjusted, it is possible to produce

sparks 10" to 12" in length. It is designed for the experimenter who wishes

to explore the effects of high frequency, high voltage energy.

CIRCUIT THEORY (Ref FIG 13-1)

The device consists of a secondary coil LS1 containing

approximately 500 turns wound on a PVC form 12" long. This coil possesses

an inherent resonant frequency determined by its inductance and capacity. A

primary circuit consisting of a drive coil LP1 and capacitor C1 are impulse

driven by a spark gap SG1. This primary circuit should also have an

resonant frequency equal to that of the secondary coil for maximum

performance. It is possible to force the secondary coil with less results.

The output voltage of the device is dependant on the ratio of Q between

these two coils.

The primary coil has an adjustable tap that allows for fine tuning.

It should be noted that it doesn't take much in the way of added

capacitance to the secondary to alter its resonance pint. Even a change in

the output terminal TER1 may require readjustment of the tap.

Transformer T1 supplies the necessary high voltage. It is rated at

6000 volts about 23 ma. A larger capacity transformer will produce more

output but may stress the other circuit components. This voltage charges

the primary storage capacitor (C1) to a voltage where it fires the spark

gap (SG1) producing an impulse of current through the primary inductance

(LP1) where oscillations take place. The frequency is determined by the

inductance and capacity values of the primary circuit. Voltage output of

the secondary coil (LS1) is usually approximately to V2=C1*V1/C2 where C1 =

primary storage capacity, V1 = the spark gap discharge voltage, and C2 =

the secondary coil capacitance (usually relatively small). Another way of

expressing this relation is that the output volts are dependent on input

drive voltage times the ratio of primary "Q" to secondary "Q". There are

several texts describing some of TESLA'S works and these are available

through your library.

CONSTRUCTION STEPS (ref. FIG 13-2)

PLEASE NOTE THAT SUBSTITUTION OF CERTAIN PARTS MAY BE MADE BY THE BUILDER,

HOWEVER, THIS MAY INCREASE OR DECREASE PERFORMANCE.

1. Wind 11" of #26 magnet wire (WR13) on coil form (CTL1). Leave 3/4" to 1"

on each end. Fasten wires by threading them through small holes in coil

form. Leave 12" leads. Coil form must be dry and clean. Varnish, dope or

coat finished winding with paraffin wax several times to seal against

moisture and hold in place. Winding must be neat and tight. Avoid kinks

and overlaps as this will seriously affect coil performance.

The best way to wind the coil is with a buddy using a broom stick as a

shaft and carefully dispensing the wire as needed. Keep wire taut.

Winding should be approximately 50 turns per inch for 10" using less

than 600 feet of wire. Final coil is secured to mounting plate (MP1) via

plastic cap (CA1).

2. Obtain some #12 wire and form into a coil of 8 turns with a 6" diameter

as shown in FIG 13-2. Hold together with tape. Fabricate 4 pieces of dry

wood or plastic for SP1 1-1/4 x 3/4 x 1" for mounting and securing coil

primary (LP1). Use tye wraps to secure (SP1) blocks and coil

simultaneously to (MP1) mounting plate as shown.

3. To construct spark gap (see FIG 13-3) fabricate a 3" teflon block (BL1)

as shown. Note angle brackets (BR1) and (BR2) for attaching (SW10) and

(SW20) spark gap screws. These brackets should be tapped for 1/4-20

screws on one end. Leave enough space between bottom bracket and chassis

to prevent breakdown from occurring. Note flattened shield section with

attaching to screws. Note that the bottom screw has nut tightened

against the bottom bracket for locking. The top screw is allowed to turn

allowing for spark gap adjustment. Attach an insulating shaft of plastic

tubing by pressing on to the stud end of this screw. It may be desired

to cut out a window for observing the gap. Cover hole with glass,

plastic, etc., to protect your eyes from UV. For continued use, it is

advised to lock SW10 via lock nut to prevent overheating.

4. (C1) capacitor is mounted via bracket as shown on the grounded end.

FIG 13-4. The other end is supported via several pieces of double sided

tape built up to the correct thickness for proper securing. Only one

capacitor is shown. The remaining assembly is illustrated in the

drawings with attention being given to proper positioning and spacing of

the HV components. MP1 mounting base must be non-metallic, dimensions of

metal enclosing are not critical as long as there is ample spacing. Use

sheet metal screws, rivets, etc., for putting enclosure together if home

fabricated. Use standard wiring and assembly techniques.

5. A convenient output terminal for the secondary can be a smooth door knob

attached via a bolt thru CA2. This also looks good and gives the coil a

finished look.

6. When firing up your coil, start with the spark gap set at 2 turns from

"closed", and adjustment tap at maximal inductance. Note that this lead

also contributes to total inductance when routed as shown. (Same

direction as turns of primary coil). It should be possible with this

coil to easily obtain 10 to 12 inch streamers when properly adjusted.

Careful adjustment of the tap location and gap adjustment will greatly

enhance performance. When operating for a prolonged period always note

potential breakdown points occurring indicated by heavy corona or

premature sparking. These points must be corrected or a burning tracking

condition will be the result.

EXPERIMENTS WITH YOUR COIL

1. ADJUSTMENT - Suspend a grounded metallic object above the device. Start

at about 3" separation and make adjustments increasing separation until

the optimum spark length is obtained. (Note - grounding means connection

to metal base).

2. EFFECT ON HUMAN BODY - Use caution as this may cause a reflex secondary

reaction even though it is painless. Hold a metal object TIGHTLY and

advance to coil terminal. Note the painless, tingling sensation. Fake

out your pals by letting them think it is really painful. This

demonstrates the skin or surface effect of high frequency electricity.

3. EFFECT ON INSULATORS - Place various objects on the top of the coil and

note the effect of the high frequency electricity. The sparks are not

stopped by glass or other usual insulators. Experiment using objects

such as light bulbs, bottles, glass, etc.

4. EFFECT ON PARTIAL INSULATORS - Place wood pieces about 12" x 1" x 3" and

note red streaks and other bizarre weird phenomena occurring from within

the piece.

5. IONIZATION OF GASES - Obtain a fluorescent lamp and allow it to come

within several feet of the device. It will glow and produce light

without direct connection, clearly demonstrating the effects of the

electric an magnetic fields of the gas. Note the distance from the coil

that the lamp will glow. Experiment using a neon lamp in place. Obtain

other lamps and note the colors, distances and other phenomena.

6. INDUCTION FIELDS - This is demonstrated by obtaining a small filament

type lamp such as a flashlight bulb or similar and connecting between a

large 1-1/2 to 2' diameter metal or wire loop. The lamp will now light

due to energy coupled by induction. You will note that current is

required to light this type of lamp and is entirely different than the

radiation field that ionizes and causes the gas lamps to glow.

7. CREATE SPECIAL EFFECTS ON PINWHEELS, COLOR FIRES, ETC. - By connecting

pieces of nichrome wire as shown in sketches. Note - rotor rotates

creating a ring of fire. Try different types of rotors.

8. ION MOTOR - Fab and carefully balance rotor as shown in FIG 13-5. Use

piece of #16 or #18 copper buss wire. Rotor will spin at high speeds if

carefully balanced demonstrating ion propulsion.

SPECIAL NOTE: - Your tesla coil is hungry for "capacitance". It is

suggested to obtain a 2" to 3" aluminum funnel and place

in atop of the coil. Connection can be made simply by

placing on top of the output wire. Our sample lab unit at

6-1/2 turns with this setup. This does not mean yours will

be identical as there are many variables that fill effect

this parameter. It is important to have this top

capacitance to prevent voltage breakdown along side of

coil.

SPECIAL NOTE: - The tap wire of your coil will add or subtract up to 1/2

turn depending on direction routed around coil. Keep wire

flat and away from coil to prevent damaging arcover.

SPECIAL NOTE: - Optimum output requires proper selection of the correct tap

position on the primary coil. Start at 5 turns from inside

and tap into wire using a metal thumb tack pushed through

the insulation. Connect alligator clip to thumb tack head.

When proper tap is found remove insulation & solder into

place. Ref step 6 of plans. See above note.

SPECIAL NOTE: - DANGER! DANGER! Do not use this device near people with

pacemakers or computerized equipment.

Component and part listing:

�������������������������������������������������������������������������Ŀ

�Comp. ID � Amount  � Description / Value                                 �

�������������������������������������������������������������������������Ĵ

� T1      �   1     � 6000 volts 23 ma ignition transformer (IU: $44)     �

� C1      �   1     � 0.005 uF 6000 volt capacitor special pulse (IU:$34) �

� CTL1    �   1     � 13" wound coil with #26 (0.4mm) magnet wire (IU:$40)�

� R1/NE1  �   1/1   � 39K 1/4 W Resistor / Neon lamp with leads           �

� S1      �   1     � Medium toggle switch                                �

� RFC1    �   1     � 2.5 mH RF choke, should have HV rating              �

� C??     �   1     � 3 wire #18 (1.0mm) power chord                      �

� CL1     �   1     � Alligator clip                                      �

� WR12    �   15'   � #12 (2.0mm) covered wire for forming primary        �

� WR7     �   6"    � #16 (1.3mm) buss wire                               �

� WR8     �   2'    � #12 covered wire, same as WR12                      �

� WR17    �   1     � 3" #24 precut lead                                  �

� BU1     �   1     � 1/2" plastic bushing or clamp for line cord         �

� BU2     �   1     � 3/8" plastic bushing                                �

� WN1     �   1     � Small wire nuts                                     �

� FT1     �   6     � Rubber feet stick-on                                �

� B??2    �   2     � Small 3/4 x 3/4 brass brackets (FIG 13-3)           �

� TY1     �   4     � 8" nylon tyewraps for securing primary to MP1       �

� FUN1    �   1     � Small Al funnel FIG 10                              �

� CA?1    �   1     � Hollow 1" brass ball FIG 10                         �

� LAB1    �   1     � Danger HV warning label                             �

� LAB2    �   1     � Tuning label                                        �

� TA1     �   2"    � 1" two-side tape                                    �

� SID1,2  �   2     � 9" x 11" #22 gal fab per FIG 13-4 as 9x11x5-1/2 box �

� BOT1    �   1     � 9" x 11" gal fab per FIG 13-4                       �

� BR3     �   1     � 1" x 2" #22 gal bracket for holding C1              �

� MP1     �   1     � 9" x 11" finished masonite fab per FIG 13-4         �

� BL1     �   1     � 3/8 x 3/8 x 3" teflon or equiv. fab per FIG 13-3    �

� SF1-4   �   4     � 1-1/4 x 3/4 x 1" insulating blocks fab per fig 13-2 �

� TUBE    �   1     � 3" x 3/16" ID poly tube                             �

�������������������������������������������������������������������������͵

� HARDWARE:                                                               �

�������������������������������������������������������������������������͵

� SW3     �   4     � 6-32 x 1" PH screws & nuts                          �

� SW2     �   5     � 6-32 x 1/2" PH screws                               �

� SW4     �   1     � 8 x 3/8 sheet metal screw                           �

� SW?     �   14    � 6 x 3/8 self tap type F or sheet metal screws       �

� SW10,20 �   1,1   � 1/4 20 x 2",1/2" carriage bolts (smooth brass heads)�

� NU1/WA1 �   9/5   � 6-32 kep nuts / flat washers #6                     �

� CA1,2   �   2     � 3 1/2" plastic caps                                 �

� NU4     �   2     � 1/4 20 hex nuts                                     �

���������������������������������������������������������������������������

--------------------------------------------------------------------------

Transcriber's notes:

Finally, the address for IU, where you can get their excellent catalog for

free, containing over a hundred books, kits and ready-to-use HV projects:

Information Unlimited

Box 716 AMHERST

N.H. 03031, USA

--------------------------------------------------------------------------